3GPP TSG RAN WG1 Meeting #100bis-E					     R1-200xxxx

e-Meeting, April 20th – 30th, 2020

Source:	Moderator (Intel Corporation)

Title:	TP Email Discussion #5 [100b-e-NR-5G_V2X_NRSL-Mode-2-05]

Agenda item:	7.2.4.2.2

Document for:	Discussion and Decision

Introduction

This document provides discussion on TP as per the fifth email discussion on V2X Mode-2 during RAN1#100bis-e.

[100b-e-NR-5G_V2X_NRSL-Mode-2-05] Email approval of TP capturing previous meeting’s agreement regarding Time and frequency resource indication based on the submitted contributions till 4/22 (Intel, Sergey)

The intention of the TP is to implement the following agreements made in RAN1#100e:

Agreements: - Time resource assignment in SCI uses an extended time domain RIV mechanism as follows: if N=1 TRIV=0 elseif N=2 TRIV=T1 else if T2-T1-1≤15 TRIV=30T2-T1-1+T1+31 else TRIV=3031-T2+T1+62-T1 end if end if where - N denotes the actual number of resources indicated - Ti denotes i-th resource time offset - for N=2, 1≤T1≤31 - for N=3, 1≤T1≤30, T1<T2≤31 Agreements: - For frequency resource indication, the following resource index calculation is used - For Nmax = 2, - r=f2+i=1m-1N subchannel SL+1-i - For Nmax = 3, - r=f2+f3⋅N subchannel SL+1-m+i=1m-1N subchannel SL+1-i2 - where - f2 denotes lowest sub-channel index for the second resource, if any - f3 denotes lowest sub-channel index for the third resource, if any - m denotes number of sub-channels in a frequency resource allocation - If time domain allocation indicates N < Nmax, the decoded lowest sub-channel indexes corresponding to Nmax minus N last resources are not used

The draft TPs were identified in contributions [1], [7] and [13]. In the next section, the TP based on these contributions is provided.

Proposed draft TP to TS 38.214

--------------------------------------------- TP to 38.214, section 8.1.5 starts ---------------------------------------------------

### 8.1.5	UE procedure for determining slots and resource blocks for PSSCH transmission associated with an SCI format 0-1

The set of slots and resource blocks for PSSCH transmission is determined by the resource used for the PSCCH transmission containing the associated SCI format 0-1, and fields "Frequency resource assignment", "Time resource assignment" of the associated SCI format 1 as described below.

"Time resource assignment" carries logical slot offset indication of N = 1 or 2 actual resources when sl-MaxNumPerPreserve is 2, and N = 1 or 2 or 3 actual resources when sl_MaxNumPerPreserve is 3, in a form of time RIV (TRIV) field which is determined as follows:

if N=1

TRIV=0

elseif N=2

TRIV=tT1

else

if tT2-tT1-1≤15

TRIV=30tT2-tT1-1+tT1+31

else

TRIV=3031-tT2+tT1+62-tT1

end if

end if

where tTi denotes i-th resource time offset in logical slots of a resource pool with respect to the first resource where for N = 2, 1≤tT1≤31; and for N = 3, 1≤tT1≤30, tT1<tT2≤31.

The starting sub-channel of the PSSCH associated with the slot where SCI format 0-1 received, is determined according to clause 8.1.2.2. The number of contiguously allocated sub-channels LsubCH≥1 and the starting sub-channel indexes of resources reserved by the received SCI format 0-1 are determined from "Frequency resource assignment" which is equal to an extended frequency RIV r(FRIV) where.

If sl-MaxNumPerReserve is 2 then

rFRIV=nsubCH,1startf2+i=1LsubCH-1N subchannel SL+1-i

If sl-MaxNumPerReserve is 3 then

rFRIV=nsubCH,1startf2+nsubCH,2startf3⋅N subchannel SL+1-LsubCH+i=1LsubCH-1N subchannel SL+1-i2

where

-	nsubCH,1startf2 denotes the starting sub-channel index for the second resource

-	nsubCH,2startf3 denotes the starting sub-channel index for the third resource

-	N subchannel SL is the number of sub-channels in a resource pool provided according to the higher layer parameter numSubchannel

If TRIV indicates N < sl-MaxNumPerReserve, the decoded starting sub-channel indexes corresponding to sl-MaxNumPerReserve minus N last resources are not used.

--------------------------------------------- TP to 38.214, section 8.1.5 ends ---------------------------------------------------

| Source | Comments | FL comments |
| --- | --- | --- |
| Qualcomm | To clarify that N is bounded by N_max: - "Time resource assignment" carries logical slot index indication of N = 1 or 2 or, when sl-MaxNumPerPreserve is 3, 3 actual resources in a form of time RIV (TRIV) field which is determined as follows Editorial comments: - where Ti denotes i-th resource time offset in logical slots of a resource pool with respect to the first resource where for N = 2, 1≤T1≤31,; and for N = 3, 1≤T1≤30, T1<T2≤31. - Could you replace T1,T2 with different variable names to avoid confusion with the selection window limits, which are also used in 38.214? For example, S1,S2 (or X1,X2), … | Tried to accept all the proposed changes |
| Apple | We prefer to denote f2 and f3 as “the starting sub-channel index….”, as defined in LTE V2X (TS36.213). Similar to TRIV, we could use the term FRIV, rather than “r”, for the frequency resource index. | Accepted the proposed changes |
| Sharp | - Similar to Qualcomm’s first proposed change, but it would be clearer if we say “N=1 or 2 when sl-MaxNumPerPreserve is 2, and N=1 or 2 or 3 when sl_MaxNumPerPreserve is 3”. - Not sure if it is clear enough to say “other PSSCH”. It could happen that a resource is reserved but eventually not used, in which case there is no “other PSSCH” transmission. It may be better to just use “resource” as already used in other places of the TP. - Could we add a reference to the definition of N subchannel SL? - Agree with Apple that to be consistent, it would be better to use FRIV rather than “r”. | Accepted the proposed changes |
| CATT | Agree with the first change of Qualcomm, and N is bounded by N_max. Agree with Apple that FRIV rather than “r” should be used for the frequency resource index. | Accepted the changes |
| Huawei/HiSilicon | The description that “The lowest sub-channel index of the first PSSCH associated with the received SCI format 0-1 is determined from the sub-channel index where PSCCH carrying the SCI format 0-1 is received” is inaccurate, where the PSSCH associated the received SCI does not have to be “the first PSSCH” in time among the all resources indicated by the SCI, the restriction on the sequence in time should not be introduced. The lowest sub-channel for PSSCH associated the received SCI has been captured in the existing section 8.1.2.2 of TS 38.214 as follows: The lowest sub-channel for sidelink transmission is the sub-channel on which the lowest PRB of the associated PSCCH is transmitted. Similarly, the notations that “the lowest sub-channel index for the second/third resource” for f2 and f3 should be removed. f2 and f3 should be denoted as the first sub-channel index nsubCHstart1 and the second sub-channel index nsubCHstart2, The frequency resource index “r” should be replaced by frequency resource indication valued FRIV. | I think with current agreements (w/o backward indication) it is OK to use “first”, and modify later if needed. But I removed “first” and clarified that this is for the resource where current SCI 0-1 received. My understanding of 8.1.2.2 is to define a PSSCH resource for demodulation purpose mainly, and this section 8.1.5 is mainly for resource reservation purpose. I’m open to simplify, if needed. I replaced f2/f3 with nsubCHstart, although there is no definition of them previously, so that we are open to use any notations. Replaced ‘r’ by FRIV |
| Huawei/HiSilicon (v7) | - Since backward indication is still under discussion, and the word “extended” seems unclear and unnecessary, the following changes are suggested:: “…The number of contiguously allocated sub-channels LsubCH≥1 and the starting sub-channel indexes of resources reserved indicated by the received SCI format 0-1 are determined from "Frequency resource assignment" which is equal to an extended frequency RIV (FRIV) where.” - Since the FL has already removed “first” (see the comment above), now it’s unclear and unnecessary to say “the second/third resource”, so the following changes are suggested: “- nsubCH,1start denotes the first starting sub-channel index for the second resource - nsubCH,2start denotes the second starting sub-channel index for the third resource” |  |
| Sharp | Regarding Huawei’s second comment, since for TRIV the TP uses “N resources”, “the first resource”, “the i-th resource”, we should do the same for FRIV, i.e. there is no problem in saying “second resource” and “third resource” in the original wording in the TP. Regarding Huawei’s first comment, it is fine to replace “reserved” with “indicated”, but I think the FL’s original intention was to use “reserved” to make sure the first resource is not included here, so if “indicated” is used, then we should say “…The number of contiguously allocated sub-channels LsubCH≥1 for the N resources and the starting sub-channel indexes of the second resource (when sl-MaxNumPerPreserve is 2 or 3) and third resources (when sl-MaxNumPerPreserve is 3) reserved indicated by the received SCI format 0-1 …”. In fact, there has not been any text to explain the time/frequency of the “first resource”, so in the last paragraph for TRIV we may say “… where the first resource is in the slot where SCI format 0-1 was received, and tTi denotes i-th resource time offset in logical slots of a resource pool with respect to the first resource …The starting sub-channel of the first resource PSSCH associated with the slot where SCI format 0-1 received, is …”. |  |
| Huawei/HiSilicon (v9) | In TRIV, the “first/second/third” represents the sequence of resource in time, but the “first/second/third” for FRIV in the previous TP does not correspond to the sequence of resources in time. Thus, the “first/second/third” is needed for TRIV, but not needed for FRIV. And since we have already deleted “the first”, maybe we need to avoid using “the second/third resource”, as appeared in Sharp’s comment above. In summary, we still suggest our changes in v7, and we think the changes proposed by Sharp above is not necessary. |  |

- References

- R1-2001552	Remaining details of sidelink resource allocation mode 2	Huawei, HiSilicon

- R1-2001661	Remaining issues on mode 2 resource allocation mechanism	vivo

- R1-2001749	Discussion on remaining open issue for mode 2	OPPO

- R1-2001793	Remaining Issues on Sidelink Mode 2 Resource Allocation	Panasonic Corporation

- R1-2001805	Remaining details of Resource allocation for sidelink - Mode 2	Nokia, Nokia Shanghai Bell

- R1-2001877	Remaining details on mode 2 resource allocation for NR V2X	Fujitsu

- R1-2001886	Discussion on resource allocation for Mode 2	LG Electronics

- R1-2001896	Remaining issues of mode 2 operation on sidelink	ZTE, Sanechips

- R1-2001907	Sidelink mode-2 resource allocation	MediaTek Inc.

- R1-2001964	Resource allocation for NR sidelink Mode 2	TCL Communication Ltd.

- R1-2001969	Discussion on resource allocation for NR sidelink Mode 2	Lenovo, Motorola Mobility

- R1-2001978	Remaining Issues in Resource Allocation for Mode 2 NR V2X	Fraunhofer HHI, Fraunhofer IIS

- R1-2001994	Solutions to remaining opens of resource allocation mode-2 for NR V2X sidelink design	Intel Corporation

- R1-2002041	Remianing details on mode-2 resource allocation	Futurewei

- R1-2002078	Remaining issues on Mode 2 resource allocation in NR V2X	CATT

- R1-2002126	On Mode 2 for NR Sidelink	Samsung

- R1-2002234	Resource allocation Mode 2 for NR SL	Ericsson

- R1-2002267	Remaining issues in NR sidelink mode 2 resource allocation	Spreadtrum Communications

- R1-2002301	Remaining Issues on NR Sidelink Mode 2 Resource Allocation	InterDigital, Inc.

- R1-2002325	On Remaining Details of Mode 2 Resource Allocation	Apple

- R1-2002362	Remaining issues on resource allocation Mode 2	NEC

- R1-2002388	Remaining issues on resource allocation mode 2 for NR sidelink	Sharp

- R1-2002402	On resource reservation in Mode 2 resource allocation	Xiaomi Communications

- R1-2002439	Remaining issues on resource allocation mechanism mode 2	NTT DOCOMO, INC.

- R1-2002487	Remain details on mode-2 resource allocation for NR V2X	ITL

- R1-2002489	Remaining issue for Mode 2 resource allocation in NR V2X	ASUSTeK

- R1-2002539	Sidelink Resource Allocation Mechanism for NR V2X	Qualcomm Incorporated