3GPP TSG RAN WG1 Meeting #100bis-E					     R1-200xxxx

e-Meeting, April 20th – 30th, 2020

Source:	Moderator (Intel Corporation)

Title:	TP Email Discussion #6 [100b-e-NR-5G_V2X_NRSL-Mode-2-06]

Agenda item:	7.2.4.2.2

Document for:	Discussion and Decision

Introduction

This document provides discussion on TP as per the sixth email discussion on V2X Mode-2 during RAN1#100bis-e.

[100b-e-NR-5G_V2X_NRSL-Mode-2-06] Email approval of TPs to fix PSSCH RSRP and capture exclusion of TTIs in the same period / aperiodic reservations

till 4/23 (Intel, Sergey)

TP on PSSCH DMRS

In [8], the following TP is provided:

#### 8.4.2.1 RSRP for resource selection in sidelink resource allocation mode 2 In sidelink resource allocation mode 2, the UE measures RSRP for resource selection as follows: - PSSCH-RSRP over the DM-RS resource elements for the PSCCH carrying the received SCI format 0-1 the PSSCH according to the received SCI format 0-1 if higher layer parameter RSforSensing is set to “PSSCH DM RS”, and - PSCCH-RSRP over the DM-RS resource elements for the PSSCH according to the received SCI format 0-1 the PSCCH carrying the received SCI format 0-1 if higher layer parameter RSforSensing is set to “PSCCH DM RS”.

In [21], the following TP is provided:

===============start================ 8.4.2.1 RSRP for resource selection in sidelink resource allocation mode 2 In sidelink resource allocation mode 2, the UE measures RSRP for resource selection as follows: - PSSCH-RSRP over the DM-RS resource elements for the PSSCH according to the received SCI format 0-1 if higher layer parameter sl-RS-ForSensing is set to " pssch ", and - PSCCH-RSRP over the DM-RS resource elements for the PSCCH carrying the received SCI format 0-1 if higher layer parameter sl-RS-ForSensing is set to " pscch ". ===============end================

It seems both TPs fix the same issue in the same manner, while the TP from [21] also aligns the RRC parameter name to the latest RAN2 CR. Thus, this TP version is proposed to be captured:

--------------------------------------------- TP to 38.214, section 8.4.2.1 starts ---------------------------------------------------

8.4.2.1	RSRP for resource selection in sidelink resource allocation mode 2

In sidelink resource allocation mode 2, the UE measures RSRP for resource selection as follows:

-	PSSCH-RSRP over the DM-RS resource elements for the PSSCH according to PSCCH carrying the received SCI format 0-1 if higher layer parameter sl-RS-ForSensingRSforSensing is set to "psschPSSCH DM RS", and

-	PSCCH-RSRP over the DM-RS resource elements for the PSSCH according to PSCCH carrying the received SCI format 0-1 if higher layer parameter sl-RS-ForSensingRSforSensing is set to "pscchPSCCH DM RS".

--------------------------------------------- TP to 38.214, section 8.4.2.1 ends ---------------------------------------------------

| Source | Comments |
| --- | --- |

TP on Exclusion of Same Period TTIs / Aperiodic Exclusion

There is one identified gap in current implementation of specification. It seems current specification may not clearly capture the case of aperiodic reservations.

In [24], the following TP is provided to fix the issue:

### 8.1.4 UE procedure for determining the subset of resources to be reported to higher layers in PSSCH resource selection in sidelink resource allocation mode 2 […] The following steps are used: […] 5) The UE shall exclude any candidate single-slot resource Rx,y from the set SA if it meets all the following conditions: - the UE has not monitored slot tmSL in Step 2. - for any periodicity value allowed by the higher layer parameter reservationPeriodAllowed and a hypothetical SCI format 0-1 received in slot tmSL with "Resource reservation period" field set to that periodicity value and indicating all subchannels of the resource pool in this slot, condition c in step 6 would be met. 6) The UE shall exclude any candidate single-slot resource Rx,y from the set SA if it meets all the following conditions: a) the UE receives an SCI format 0-1 in slot tmSL, and "Resource reservation period" field, if present, and "Priority" field in the received SCI format 0-1 indicate the values Prsvp_RX and prioRX, respectively according to Clause [TBD] in [6, TS 38.213]; b) the RSRP measurement performed, according to clause 8.4.2.1 for the received SCI format 0-1, is higher than ThprioRX; c) the SCI format received in slot tmSLor the same SCI format which, if and only if the "Resource reservation period" field is present in the received SCI format 0-1, is assumed to be received in slot(s) tm+q×Prsvp_RX'SL determines according to clause [TBD] in [6, TS 38.213] the set of resource blocks and slots which overlaps with Rx,y+j×Prsvp_TX' for q=1, 2, …, Q and j=0, 1, …, Cresel-1. Here, Prsvp_RX' is Prsvp_RX converted to units of logical slots, Q=TscalPrsvp_RX if Prsvp_RX< Tscal and n'-m≤Prsvp_RX', where tn'SL = n if slot n belongs to the set t0SL,t1SL,...,tTmaxSL, otherwise slot tn'SL is the first slot after slot n belonging to the set t0SL,t1SL,...,tTmaxSL; otherwise Q=1. Tscal is FFS 7) The UE shall exclude any candidate single-slot resource Rx,y from the set SA if it meets all the following conditions: a) the UE receives an SCI format 0-1 in slot tmSL, and "Priority" field in the received SCI format 0-1 indicate the values Prsvp_RX and prioRX, respectively according to Clause [TBD] in [6, TS 38.213]; b) the RSRP measurement performed, according to clause 8.4.2.1 for the received SCI format 0-1, is higher than ThprioRX; c) the SCI format received in slot tmSLor the same SCI format is assumed to be received in slot(s) tm+y'SL determines according to clause [8.1.5] the set of resource blocks and slots which overlaps with Rx',y+y'. Here, x' and y' are indicated by "Time resource assignment" field and "Frequency resource assignment" field in the SCI format, respectively. 78) If the number of candidate single-slot resources remaining in the set SA is smaller than 0.2⋅Mtotal, then Th(pi) is increased by 3 dB for each priority value Th(pi) and the procedure continues with step 4. The UE shall report set SA to higher layers.

In [27], the following TP is provided to fix the issue:

----------------------------------------------------begin text proposal for 38.214---------------------------------------------------- ### 8.1.4 UE procedure for determining the subset of resources to be reported to higher layers in PSSCH resource selection in sidelink resource allocation mode 2 <<<unchanged text omitted>>> b) the RSRP measurement performed, according to clause 8.4.2.1 for the received SCI format 0-1, is higher than ThprioRX; c) the SCI format received in slot tmSLor the same SCI format which, if and only if the "Resource reservation period" field is present in the received SCI format 0-1, is assumed to be received in slot(s) tm+q×Prsvp_RX'SL determines according to clause [TBD] in [6, TS 38.213] the set of resource blocks and slots which overlaps with Rx,y+j×Prsvp_TX' for q=0,1, 2, …, Q and j=0, 1, …, Cresel-1. Here, Prsvp_RX' is Prsvp_RX converted to units of logical slots, Q=TscalPrsvp_RX if Prsvp_RX< Tscal and n'-m≤Prsvp_RX', where tn'SL = n if slot n belongs to the set t0SL,t1SL,...,tTmaxSL, otherwise slot tn'SL is the first slot after slot n belonging to the set t0SL,t1SL,...,tTmaxSL; otherwise Q=1. Tscal is FFS. -----------------------------------------------------end text proposal for 38.214-----------------------------------------------------

In FL understanding, TP in [24] may repeat already captured procedures, if the “[TBD] in [6, TS 38.213]” part in step 6c is updated to the actual clause 8.1.5 which instructs how time and frequency resources indicated in SCI are determined.

In FL understanding, TP in [27] may not fully solve the mentioned problem. It is preferred to clearly define that q and P_rsrp_RX is only applied in case of periodic indication in SCI 0-1. This version is proposed to be discussed/approved:

-----------------------------------------------------start text proposal for 38.214-----------------------------------------------------

### 8.1.4	UE procedure for determining the subset of resources to be reported to higher layers in PSSCH resource selection in sidelink resource allocation mode 2

<<<unchanged text omitted>>>

b) the RSRP measurement performed, according to clause 8.4.2.1 for the received SCI format 0-1, is higher than  ThprioRX;

c) the SCI format received in slot tmSLor the same SCI format which, if and only if the "Resource reservation period" field is present in the received SCI format 0-1, is assumed to be received in slot(s) tm+q×Prsvp_RX'SL determines according to clause 8.1.5 the set of resource blocks and slots which overlaps with Rx,y+j×Prsvp_TX' for j=0, 1, …, Cresel-1. If and only if the "Resource reservation period" field is present in the received SCI format 0-1,q=1, 2, …, Q Cresel-1. Here, Prsvp_RX' is Prsvp_RX converted to units of logical slots,  Q=TscalPrsvp_RX  if Prsvp_RX< Tscal and  n'-m≤Prsvp_RX', where tn'SL = n if slot n belongs to the set t0SL,t1SL,...,tTmaxSL, otherwise slot tn'SL is the first slot after slot n belonging to the set t0SL,t1SL,...,tTmaxSL; otherwise Q=1. Tscal is FFS.

-----------------------------------------------------end text proposal for 38.214-----------------------------------------------------

| Source | Comments |
| --- | --- |

- References

- R1-2001552	Remaining details of sidelink resource allocation mode 2	Huawei, HiSilicon

- R1-2001661	Remaining issues on mode 2 resource allocation mechanism	vivo

- R1-2001749	Discussion on remaining open issue for mode 2	OPPO

- R1-2001793	Remaining Issues on Sidelink Mode 2 Resource Allocation	Panasonic Corporation

- R1-2001805	Remaining details of Resource allocation for sidelink - Mode 2	Nokia, Nokia Shanghai Bell

- R1-2001877	Remaining details on mode 2 resource allocation for NR V2X	Fujitsu

- R1-2001886	Discussion on resource allocation for Mode 2	LG Electronics

- R1-2001896	Remaining issues of mode 2 operation on sidelink	ZTE, Sanechips

- R1-2001907	Sidelink mode-2 resource allocation	MediaTek Inc.

- R1-2001964	Resource allocation for NR sidelink Mode 2	TCL Communication Ltd.

- R1-2001969	Discussion on resource allocation for NR sidelink Mode 2	Lenovo, Motorola Mobility

- R1-2001978	Remaining Issues in Resource Allocation for Mode 2 NR V2X	Fraunhofer HHI, Fraunhofer IIS

- R1-2001994	Solutions to remaining opens of resource allocation mode-2 for NR V2X sidelink design	Intel Corporation

- R1-2002041	Remianing details on mode-2 resource allocation	Futurewei

- R1-2002078	Remaining issues on Mode 2 resource allocation in NR V2X	CATT

- R1-2002126	On Mode 2 for NR Sidelink	Samsung

- R1-2002234	Resource allocation Mode 2 for NR SL	Ericsson

- R1-2002267	Remaining issues in NR sidelink mode 2 resource allocation	Spreadtrum Communications

- R1-2002301	Remaining Issues on NR Sidelink Mode 2 Resource Allocation	InterDigital, Inc.

- R1-2002325	On Remaining Details of Mode 2 Resource Allocation	Apple

- R1-2002362	Remaining issues on resource allocation Mode 2	NEC

- R1-2002388	Remaining issues on resource allocation mode 2 for NR sidelink	Sharp

- R1-2002402	On resource reservation in Mode 2 resource allocation	Xiaomi Communications

- R1-2002439	Remaining issues on resource allocation mechanism mode 2	NTT DOCOMO, INC.

- R1-2002487	Remain details on mode-2 resource allocation for NR V2X	ITL

- R1-2002489	Remaining issue for Mode 2 resource allocation in NR V2X	ASUSTeK

- R1-2002539	Sidelink Resource Allocation Mechanism for NR V2X	Qualcomm Incorporated