3GPP TSG RAN WG1 #100bis	R1-200xxxx

e-Meeting, April 20th – 30th, 2020

Source:	moderator (vivo)

Title:	Feature lead summary on ULFPTx-03

Agenda Item:	7.2.6.4

Document for:	Discussion and Decision

Summary

Following email thread is assigned for discussion:

[100b-e-NR-eMIMO-ULFPTx-03] Email discussion on Issue #4 in R1-2002746: TP for power scaling for Mode2. By 4/24 and corresponding TP (if any) by 4/30 – Rakesh (vivo)

Discussion on issue 4[1]

Issue 4: TP for power scaling for Mode2

    - TP1

TS 38.213

-------------------------------------------------------------------------------------------

7.1	Physical uplink shared channel

For a PUSCH transmission on active UL BWP , as described in Clause 12, of carrier  of serving cell , a UE first calculates a linear value  of the transmit power , with parameters as defined in Clause 7.1.1. For a PUSCH transmission scheduled by a DCI format or configured by ConfiguredGrantConfig or semiPersistentOnPUSCH, if txConfig in PUSCH-Config is set to 'codebook',

-	….. set to 'nonCoherent' or 'partialAndNonCoherent', the UE scales PPUSCH,b,f,c(i,j,qd,l) by s where:

-	….

-	….. s=1 for full power TPMIs reported by the UE [16, TS 38.306] corresponding to the value of codebookSubset, and s is the ratio of a number of antenna ports with non-zero PUSCH transmission power over a number of SRS ports for the remaining TPMIs and corresponding codebookSubset, where the number of SRS ports is associated with the a SRS resource indicated by SRI if more than one SRS resources are is configured in the …. set to 'codebook', or the number of SRS ports is associated with the SRS resource if only one SRS resource is configured in the …. set to 'codebook', and

-	if ….is not provided, s=1

-	else, if each SRS resource in the …set to 'codebook' has more than one SRS port, the UE scales the linear value by the ratio of the number of antenna ports with a non-zero PUSCH transmission power to the maximum number of SRS ports supported by the UE in one SRS resource.

The UE splits the power equally across the antenna ports on which the UE transmits the PUSCH with non-zero power.

----------------------------------------------------------------------------------------

Please provide your views/comments in the table below

| Company/organization | comments |
| --- | --- |
| OPPO | Not needed. For 4 Tx ports, a non-coherent UE will report UE capability via 2 bits and a partial-coherent UE will use 4 bits. In the tdoc proposing the above-mentioned TP, UE may use 6 bits, where 2 of which report capability for non-coherent operation and the other 4 bits report capability for partial coherent operation. From our understanding, the tdoc misunderstood the agreement. |
| CMCC | It seems that companies have different understandings regarding the following agreements. In our understanding, for 2Tx UE reporting ‘non-coherent’ capability in 2-13, UE can report 2-port TPMI(s) in codebookSubset = nonCoherent that can support full power transmission. For 4Tx UE reporting ‘non-coherent’ capability in 2-13, UE can report 2-port and/or 4-port TPMI(s) in codebookSubset = nonCoherent that can support full power transmission. For 4Tx UE reporting ‘partial/non-coherent’ capability in 2-13, UE can report 2-port TPMI(s) in codebookSubset = nonCoherent and/or 4-port TPMI(s) in codebookSubset = nonCoherent and/or 4-port TPMI(s) in codebookSubset = partialandNonCoherent that can support full power transmission. We think we should first clarify whether this understanding is correct or not, then we can decide whether this TP is needed or not. Agreement For 2 ports, number of bits to indicate TPMI(s) which can deliver UL full power: - 2 bits (bitmap) - Whether is this capability reporting is optional or not will be discussed as part of UE capability discussions Agreement For 4 ports, number of bits to indicate TPMI(s) which can deliver UL full power: - Non Coherent 2 bits - Partial coherent 4 bits - Additional entries on top of existing entries may be added to table 1 and table 2 - Whether is this capability reporting is optional or not will be discussed as part of UE capability discussions |
| Samsung | Not needed. In our view, a UE reports only one TPMI group (not multiple) depending on number of ports and its capability (non-coherent or partial-coherent). Based on this understanding, we also think this TP is not needed. |

    - TP2

-----------------------------------------------------------------------------------

-	if ULFPTxModes in PUSCH-Config is set to Mode1, and each SRS resource in the SRS-ResourceSet with usage set to 'codebook' has more than one SRS port', s is the ratio of a number of antenna ports with non-zero PUSCH transmission power over the maximum number of SRS ports supported by the UE in one SRS resource, or

-	if ULFPTxModes in PUSCH-Config is set to Mode2, when s=1 for full power TPMIs reported by the UE [16, TS 38.306], and s is the ratio of a number of antenna ports with non-zero PUSCH transmission power over a number of SRS ports for remaining TPMIs, where the number of SRS ports is associated with a SRS resource indicated by SRI if more than one SRS resources are configured in the SRS-ResourceSet with usage set to 'codebook', or the number of SRS ports is associated with the SRS resource if only one SRS resource is configured in the SRS-ResourceSet with usage set to 'codebook', or when full power TPMIs are not reported by the UE, s is the ratio of the number of antenna ports with a non-zero PUSCH transmission power to the number of SRS ports is associated with a SRS resource indicated by SRI if more than one SRS resources are configured in the SRS-ResourceSet with usage set to ‘codebook’ and the indicated SRS resource has more than one SRS port, or

-	if ULFPTxModes in PUSCH-Config is not provided, s=1

------------------------------------------------------------------------------------

Please provide your views/comments in the table below

| Company/organization | comments |
| --- | --- |
| OPPO | Not needed as there is no issue. The wording “for remaining TPMIs” is very clear. |
| CMCC | We also think the part added can be covered by the remaining TMPIs case. |
| Samsung | Not needed, same view as OPPO, since the text “the remaining TPMIs” is equivalent to “TPMIs are not reported by the UE” |

    - TP3

------------------------------------------------------------------------------------

if ULFPTxModes in PUSCH-Config is set to Mode2, s=1 for full power TPMIs reported by the UE [16, TS 38.306] corresponding to the value of codebookSubset, and s is the ratio of a number of antenna ports with non-zero PUSCH transmission power over a number of SRS ports for remaining TPMIs corresponding to the value of codebookSubset, where the number of SRS ports is associated with a SRS resource indicated by SRI or srs-ResourceIndicator for type 1 configured grant if more than one SRS resources are configured in the SRS-ResourceSet with usage set to 'codebook', or the number of SRS ports is associated with the SRS resource if only one SRS resource is configured in the SRS-ResourceSet with usage set to 'codebook', and

----------------------------------------------------------------------------------

Please provide your views/comments in the table below

| Company/organization | comments |
| --- | --- |
| OPPO | There are two part for this TP - The first part “corresponding to the value of codebookSubset” is similar to TP1. Thus we suggest to split TP3, the first part should be merged to TP1 and only discuss the second part in TP3 - The second part “or srs-ResourceIndicator for type 1 configured grant”. We support it. |
| CMCC | We support this TP. For ease of discussion, we also think this TP could be split into two parts. The first part could be discussed together with TP1. |
| Samsung | Agree with OPPO, the first part is included in TP1 and hence not needed. We are also with the second part. |

# References

[1] R1-2002746, Summary of prep email discussion on ULFPTx, RAN1#100b-e