3GPP TSG RAN WG1 Meeting #101                                                               	           R1-2003588

e-Meeting, May 25th - June 5th, 2020

Title:	[Draft] LS reply on checking the view on sidelink

Response to: 	R1-2003256 (R2-2004084)

Release:	Rel-16

Work Item:	5G_V2X_NRSL-Core

Source:	RAN1

To:	RAN2

CC:

Contact Person:

Name:	Teng Ma

E-mail Address:	mateng@catt.com

1. Overall Description:

RAN1 would like to thank RAN2 for the LS R2-2004084 on checking the views of sidelink.

In R2-2004084, RAN2 asked RAN1 to check the following three aspects on sidelink. After discussion in RAN1, the answer can be found below each question.

- RAN2 respectfully asks RAN1 to indicate whether the IIoT equation in point 1 above can be used for NR sidelink.

RAN1 Answer:

From RAN1 perspective, IIoT equation can be used for NR sidelink with a changing from symbol level to slot level. The HARQ process ID equation can be updated as

HARQ Process ID = [floor(CURRENT_slot / periodicity)] modulo nrofHARQ-Processes + harq-ProcID-Offset

where CURRENT_slot = (SFN × numberOfSlotsPerFrame + slot number in the frame), numberOfSlotsPerFrame refers to the number of consecutive slots per frame as specified in TS 38.211, and periodicity refers to the periodicity of the configured grant.

- RAN2 respectfully asks RAN1 to confirm whether there is any concern on the working assumption made in point 2.

RAN1 Answer:

Based on RAN1’s agreements, more than one MCS tables are configured/used to sidelink transmission. The configured MCS range should related to each MCS table, and the MCS ranges can be different among different MCS tables. Therefore, the working assumption can be further clarified that:

- “Only one MCS range is configured with each supported MCS table and applied to both dynamic grant and configured grant type 1/2; no configured grant type 1/2 specific MCS range is further needed”.

- RAN2 respectfully asks RAN1 to take point 3 into account and provide feedback, if any.

RAN1 Answer:

From RAN1 perspective, this agreement in RAN2 is agreeable.

2   Actions:

To RAN2

ACTION: RAN1 would like to ask RAN2 to take the answers above of the three aspects into account.

3   Date of Next RAN WG1 Meetings:

TSG-RAN WG1 Meeting #102	24 - 28 Aug. 2020, 		Online

TSG-RAN WG1 Meeting #102bis	12 - 16 Oct. 2020, 		China