3GPP TSG RAN WG1 #101                                                                                       R1-2003822

E-meeting, May 25th – June 5th, 2020

Agenda Item:	7.2.2.1.3

Source:	Lenovo, Motorola Mobility

Title:	Text proposals for UL transmission for NR-U

Document for:	Discussion & Decision

# Introduction

The contribution is focused on remaining issues for UL transmission in Rel-16 NR-U.

In RAN1#100bis-e meeting, regarding PUSCH scheduled by fallback DCI, i.e., DCI format 0-0, transmitted in either CSS or USS, the corresponding agreement on resource allocation in term of assigned RB set is listed below:

Agreement:

- For PUSCH scheduled by DCI 0_0 received in a CSS when UL resource allocation Type 2 is configured, PUSCH is allocated to the RB set of the active UL BWP that intersects the RB set of the active DL BWP in which DCI 0_0 is received. If there is no intersection, PUSCH is allocated to RB Set 0 of the active UL BWP.

- FFS1: PUSCH allocation within the active UL BWP corresponding to an UL carrier without intra-cell guard bands

- FFS2: Whether or not the first bullet is modified to “…the active DL BWP in which the first REG of the received DCI 0_0 is located,” in order to facilitate a CORESET not confined to a single RB set.

Agreement:

- For PUSCH scheduled by DCI 0_0 received in a USS when UL resource allocation Type 2 is configured, PUSCH is allocated to the RB set(s) of the active UL bandwidth part indicated by the Y bits in the FDRA field of DCI 0_0.

Agreement:

Adopt TP#1 in R1-2002913 for 38.213 Section 8.3 and TP#2 for 38.214 Section 6.1.2.2.3 where the last bullet of TP#1 is modified as follows:

- The UE assumes the RB set allocation in the active UL BWP for a PUSCH scheduled by the RAR UL grant is given by [FFS: rule for for RB set allocation]

- FFS: PUSCH allocation rule within the interlaces indicated by the frequency domain resource allocation field

Agreement:

To resolve the FFS in TP#1, support one of the following two alternatives for the PUSCH allocation rule.

- When UL resource allocation Type 2 is configured, the UE assumes that PUSCH is allocated as follows:

  - Alt-1: PUSCH is allocated to the RB set of the active UL BWP that intersects the RB set of the active DL BWP in which the DCI 0_1 that schedules the PDSCH containing the RAR UL grant is received. If there is no intersection, PUSCH is allocated to RB Set 0 of the active UL BWP.

  - Alt-2: PUSCH is allocated to the initial UL BWP if the active UL BWP fully overlaps the initial UL BWP, otherwise PUSCH is allocated to RB Set 0 of the active UL BWP.

- FFS: Rule for PUSCH allocation for an UL carrier without intra-cell guard bands.

As shown in above, there are still some remaining issues on UL resource allocation. In this contribution, our views are present.

# Discussion

## UL resource allocation type 2

Regarding UL resource allocation for PUSCH scheduled by DCI format 0-0 in CSS, there are two remaining issues: (1) how to determine the assigned RB set when there is no intra-cell guard band configured for the carrier; (2) how to determine the first RB set contained in the current active UL BWP when the DCI format 0-0 is spanned in more than one RB set of the current active DL BWP.

For the 1st issue, in case that there is no intra-cell guard band configured for the UL carrier, when the current active UL BWP is assumed for the scheduled PUSCH, then the UE shall determine the resource allocation in the frequency domain as an intersection of the resource blocks of the indicated interlaces and the active uplink BWP. However, this solution has two drawbacks: (1) maybe too much resource is scheduled for PUSCH with a small packet size; (2) high risk on PUSCH transmission failure since UE can’t transmit the PUSCH as long as one of the LBTs for the RB sets is failing. Furthermore, since we have agreed that for PUSCH scheduled by DCI format 0_0 received in a CSS, PUSCH is allocated to the RB set of the active UL BWP that intersects the RB set of the active DL BWP in which the DCI format 0_0 is received; PUSCH is allocated to RB Set 0 of the active UL BWP if there is no intersection. A unified solution between the case where the intra-cell guard band is configured and the case where the intra-cell guard band is not configured is preferred.

Proposal 1: For PUSCH allocation within the active UL BWP on an UL carrier without intra-cell guard band configured, when the PUSCH is scheduled by DCI format 0-0 in a CSS, the PUSCH is allocated to the RB set of the active UL BWP that intersects the RB set of the active DL BWP where the DCI format 0-0 is received; the PUSCH is allocated to RB set 0 of the active UL BWP if there is no intersection.

For the 2nd issue, from a UE’s perspective, when DCI format 0-0 is received in CSS and spanned on one or more RB sets within the active DL BWP, correspondingly, there may be zero, one or more RB sets in the active UL BWP that intersect the one or more RB sets within the active DL BWP where the DCI format 0-0 is received. One simple solution is to determine the resource allocation for the PUSCH on the lowest RB set of the one or more RB sets within the active UL BWP. Of course, if there is no intersection, the PUSCH is allocated to RB set 0 of the active UL BWP.

Proposal 2: For PUSCH scheduled by DCI format 0-0 in a CSS, the PUSCH is allocated to the lowest RB set of the active UL BWP that intersects one or more RB sets of the active DL BWP where the DCI format 0-0 is received; the PUSCH is allocated to RB set 0 of the active UL BWP if there is no intersection.

Based on above two proposals, we have a draft TP to capture that in standard.

Proposal 3: Adopt below TP in TS38.214 in order to capture above proposal.

-----------------------------------------------< BEGIN TEXT PROPOSAL >-------------------------------------------------

##### TS38.214

##### 6.1.2.2.3	Uplink resource allocation type 2

In uplink resource allocation of type 2, the resource block assignment information defined in [5, TS 38.212] indicates to a UE a set of up to M interlace indices, and for DCI 0_0 monitored in a UE-specific search space and DCI 0_1 a set of up to  NRB-set,ULBWP  contiguous RB sets, where M and interlace indexing are defined in Clause 4.4.4.6 in [4, TS 38.211]. For DCI 0_0 monitored in a UE-specific search space and DCI 0_1, the UE shall determine the resource allocation in frequency domain as an intersection of the resource blocks of the indicated interlaces and the indicated set of RB sets and intra-cell guard bands defined in Clause 7 between the indicated RB sets, if any. For DCI 0_0 monitored in a common search space, the UE shall determine the resource allocation in frequency domain as an intersection of the resource blocks of the indicated interlaces and a single uplink RB set of the active UL BWP. The uplink RB set is the lowest RB set that intersects with the downlink RB set(s) of the active downlink BWP in which the UE detects the DCI 0_0. If there is no intersection, the uplink RB set is RB set 0 in the active uplink BWP.

For µ=0, the X=6 MSBs of the resource block assignment information indicates to a UE a set of allocated interlace indices m0+l, where the indication consists of a resource indication value (RIV). For 0≤RIV<M(M+1)/2 , l=0,1,⋯L-1 the resource indication value corresponds to the starting interlace index m0 and the number of contiguous interlace indices (). The resource indication value is defined by:

if (L-1)≤M/2 then

RIV=M(L-1)+m0

else

RIV=M(M-L+1)+(M-1-m0)

For RIV≥M(M+1)/2 , the resource indication value corresponds to the starting interlace index m0 and the set of values  according to Table 6.1.2.2.3-1.

Table 6.1.2.2.3-1: m0  and  for RIV≥M(M+1)/2.

| RIV-M(M+1)/2 | m0 |  |
| --- | --- | --- |
| 0 | 0 | {0, 5} |
| 1 | 0 | {0, 1, 5, 6} |
| 2 | 1 | {0, 5} |
| 3 | 1 | {0, 1, 2, 3, 5, 6, 7, 8} |
| 4 | 2 | {0, 5} |
| 5 | 2 | {0, 1, 2, 5, 6, 7} |
| 6 | 3 | {0, 5} |
| 7 | 4 | {0, 5} |

For µ=1, the X=5 MSBs of the resource block assignment information comprise a bitmap indicating the interlaces that are allocated to the scheduled UE. The bitmap is of size M bits with one bitmap bit per interlace such that each interlace is addressable, where M and interlace indexing is defined in Section 4.4.4.6 in [4, TS 38.211]. The order of interlace bitmap is such that interlace 0 to interlace M-1 are mapped from MSB to LSB of the bitmap. An interlace is allocated to the UE if the corresponding bit value in the bitmap is 1; otherwise the interlace is not allocated to the UE.

For DCI 0_0 monitored in a UE-specific search space and  DC 0_1 for both µ=0 and µ=1, the Y=log2NRB-set,ULBWPNRB-set,ULBWP+12LSBs of  the resource block assignment information indicate to a UE a set of contiguously allocated RB sets for PUSCH scheduled by DCI 0_0 monitored in a UE-specific search space, DCI 0_1 and Type 1 and Type 2 configured grant. The resource allocation field consists of a resource indication value (RIVRBset). For 0≤RIVRBset<NRB-set,ULBWP(NRB-set,ULBWP+1)/2 , l=0,1,⋯LRBset-1 the resource indication value corresponds to the starting RB set (RBsetSTART) and the number of contiguous RB sets LRBset. The resource indication value is defined by;

if (LRBset-1)≤NRB-set,ULBWP/2 then

RIVRBset=NRB-set,ULBWP(LRBset-1)+RBsetSTART

else

RIVRBset=NRB-set,ULBWP(NRB-set,ULBWP-LRBset+1)+(NRB-set,ULBWP-1-RBsetSTART)

where  RBsetSTART=0,1,⋯NRB-set,ULBWP-1, LRBset≥1 and shall not exceed NRB-set,ULBWP-RBsetSTART

If transform precoding is enabled according to the procedure in Clause 6.1.3, then the UE transmits PUSCH on the lowest-indexed MRBPUSCH PRBs indicated by the frequency domain resource assignment information. MRBPUSCH is the largest integer not greater than the number of RBs indicated by the frequency domain resource assignment information that fulfils the conditions in [4, TS 38.211 Clause 6.3.1.4].

<omitted text>

-----------------------------------------------< END TEXT PROPOSAL >-------------------------------------------------

# Conclusion

In this contribution, we focus on the PUSCH transmission issues and have below proposals:

Proposal 1: For PUSCH allocation within the active UL BWP on an UL carrier without intra-cell guard band configured, when the PUSCH is scheduled by DCI format 0-0 in a CSS, the PUSCH is allocation to the RB set of the active UL BWP that intersects the RB set of the active DL BWP where the DCI format 0-0 is received; the PUSCH is allocated to RB set 0 of the active UL BWP if there is no intersection.

Proposal 2: For PUSCH scheduled by DCI format 0-0 in a CSS, the PUSCH is allocated to the lowest RB set of the active UL BWP that intersects one or more RB sets of the active DL BWP where the DCI format 0-0 is received; the PUSCH is allocated to RB set 0 of the active UL BWP if there is no intersection.

Proposal 3: Adopt below TP in TS38.214 in order to capture above proposal.

# References

Chairman’s notes on RAN1#100bis-e meeting

TS38.211-g10

TS38.212-g10

TS38.213-g10

TS38.214-g10