3GPP TSG RAN WG1 #102	R1-2005324

e-Meeting, August 17th – 28th, 2020

Title:	DRAFT Reply on LS on Detail MIMO MAC CE operations

Response to:       R1-2005203 (R2-2005811)

Release:	Rel-16

Work Item:	NR_eMIMO-Core

Source:	vivo [RAN1]

To:	RAN2

CC:

# Contact Person:

# Name:	Peng SUN

E-mail Address:	sunpeng@vivo.com

# 1. Overall Description:

RAN1 would like to thank RAN2 for the LS in R1-2005203 (R2-2005811) on detail MIMO MAC CE operation.

Regarding the mentioned two issues and three questions, RAN1 has the corresponding responses as following.

Question 1: Could RAN1 specify how the TCI state codepoints are mapped when the number of codepoints in the TCI field of DCI format 1_2 is less than the maximum number of codepoints in the TCI field of DCI format 1_1 taking into account that the MAC CE may signal TCI states corresponding to maximum amount of DCI codepoints.?

Answer 1: RAN1 achieved the following agreement regarding the mapping of the TCI states to the codepoints for DCI format 1-2.

Agreements: Support additional 1 or 2 bits for “Transmission configuration indication” in DCI format 1_2 • The candidate value of range for tci-PresentInDCI-ForDCIFormat1_2 is {1, 2, 3} • 0 bit for “Transmission configuration indication” in DCI format 1_2 if tci-PresentInDCI-ForDCIFormat1_2 is not configured • Same RRC configuration on the TCI state list for DCI format 1_1 and DCI format 1_2 • Same TCI states for DCI format 1_1 and DCI format 1_2 are activated by MAC CE • If 1 bit is configured, then the first two TCI states activated by MAC CE can be dynamically indicated by this 1 bit for single TRP • If 2 bits is configured, then the first four TCI states activated by MAC CE can be dynamically indicated by this 2 bits for single TRP

Regarding the following comment from RAN2, RAN1 would like to point out that it may not always the case that “MAC CE has the TCI states corresponding to maximum amount of DCI codepoints” even UE is only configured with DCI format 1-1. For the case when MAC CE does not have the TCI states corresponding to maximum amount of DCI codepoints, there is not explicit specification in RAN1 that states the mapping relationship between codepoints and TCI states.

Comment from RAN2 “It should be further understood that the MAC CE has the TCI states corresponding to maximum amount of DCI codepoints regardless if UE is configured with DCI format 1_2 with less DCI codepoints.”

Question 2: Is RAN2 understanding correct that the intended functionality of the Serving cell set based SRS spatial relation indication MAC CE is to only indicate the SRS spatial relations for SRS resource(s) which are applied for all configured serving cell set? (i.e. no need to activate/deactivate transmission of SRS resources(s) for all configured cells in the serving cell set)

Answer 2: Yes.

Question 3: Is RAN2 understanding correct that the Serving cell set based SRS spatial relation indication MAC CE can be used to indicate the SRS spatial relations in SUL configuration?

Answer 3: No. The behaviour is not discussed when the agreement is achieved. The SUL configuration is not applicable for this feature.

# 2. Actions:

To RAN2

ACTION:

RAN1 would like to respectfully request RAN2 to take the above RAN1 feedback into account.

# 3. Date of Next TSG-RAN WG1 Meetings:

TSG-RAN WG1 Meeting #103-e     26th Oct. – 13th Nov. 2020                             Online

TSG-RAN WG1 Meeting #104       1st – 5th Mar. 2021                                Athens, Greece