3GPP TSG RAN WG1 #102-e                                             R1-2005410

e-Meeting, August 17th – 28th, 2020

Source:	vivo

Title:	Discussion on joint scheduling

Agenda Item:	8.13.2

Document for:	Discussion and Decision

- Introduction

In R17 DSS WID [1], the following objectives are agreed:

- PDCCH enhancements for cross-carrier scheduling including [RAN1, RAN2]

  - PDCCH of SCell scheduling PDSCH or PUSCH on P(S)Cell

  - Study, and if agreed specify PDCCH of P(S)Cell/SCell scheduling PDSCH on multiple cells using a single DCI

    - The number of cells can be scheduled at once is limited to 2

    - The increase in DCI size should be minimized

- Note: The total PDCCH blind decoding budget should not be changed as a result of this work

- Note: These enhancements are not specific to DSS and are generally applicable to cross-carrier scheduling in carrier aggregation

In the contribution, we provide our views on scheduling PDSCH on two cells with a single DCI.

- Scenario and use case

Dynamic spectrum sharing (DSS) provides a very useful migration path from LTE to NR by allowing LTE and NR to operate on the same frequency, it is important to ensure sufficient scheduling capacity for NR UEs on the shared carriers.

However, some operators are allocated with several narrow spectrums scattered across different frequencies. For example, Table 1 from [2] has listed some narrow channel bandwidths that have been requested so far. With such narrow bandwidth, PDCCH capacity becomes a major bottleneck to achieve the best performance.

Table 1. requested narrow channel bandwidth

| Band (s) | Channel Bandwidth(s) |
| --- | --- |
| n5 | 7, 11 MHz |
| n12 | 12 MHz |
| n26 | 7 MHz |
| n28 | 13, 33 MHz |
| n29 | 6, 7, 8, 11 MHz |

Supporting joint scheduling is a possible solution to fully exploit these scattered frequency resources while still ensuring a relatively low PDCCH overhead. Specifically, the number of DCIs required for joint scheduling is halved compared to single scheduling, in which case, PDCCH overhead may significantly decrease. Hence, joint scheduling is expected to provide more opportunities for L1 scheduling/data transmission and higher throughput can be achieved. From the UE side, this solution requires less PDCCH processing compared with the case where two DCI are required for scheduling two PDSCHs as illustrated in Figure 1.

Figure 1. Example of single scheduling and joint scheduling

Nonetheless, to ensure sufficient scheduling capacity on the NR Pcell even when the Pcell shares the same frequency with LTE, it may be beneficial to consider joint scheduling in the following scenarios:

- Scenario1: Pcell schedules two cells by joint scheduling

Compared with using two DCIs on Pcell to schedule two cells separately, jointly scheduling on Pcell allows gNB to use fewer DCI to schedule the same number of transmissions, which reduces the amount of control signaling required on Pcell and possibly increases the capacity.

- Scenario2: Scell#1 schedules Scell#1+Pcell by joint scheduling

In this case, Scell#1 schedules Pcell and itself through a single DCI. It can offload to the Scell#1 some of the PDCCH for scheduling Pcell that was originally required to be transmitted on the Pcell, hence joint scheduling may increase the scheduling capacity on Pcell.

Proposal 1. Joint scheduling should be studied in following scenarios

- Pcell jointly schedules two cells;

- A Scell jointly schedules Pcell and the Scell itself;

However, several aspects are not clear according to the current WID and need further clarification.

First of all, if a single DCI schedules PDSCH on multiple cells, there can be different understandings on the number of TBs to be transmitted in the scheduled PDSCHs. We would like to clarify that PDSCHs jointly scheduled by a single DCI should correspond to different TBs, there would be a significant impact on RAN2 otherwise. Specifically, if the PDSCHs scheduled jointly correspond to single TB or duplications of the same TB, in addition to the necessary work in the physical layer that needs to be done, high layer spec also needs to be heavily modified to support this feature if the duplicated TBs need to be soft combined. For example, RAN2 may need to change their protocol architecture to allow sharing a HARQ entity between two cells as well as joint buffer maintenance of two cells, which is complicated and may not be feasible.

Proposal 2. The study focuses on the case that PDSCH jointly scheduled by a single DCI correspond to different TBs.

Secondly, it is noted that there are a lot of commonalities between the design of UL grant and DL grant in R15 and R16. However, the WID only mentions DL joint scheduling for enhancement. Thus, it is beneficial to clarify whether or not joint scheduling UL needs to be studied.

Proposal 3. Clarify whether PUSCH joint scheduling should be studied.

- Potential issues

One of the key challenges of joint scheduling is how to control the DCI payload size for joint scheduling of different TBs. On one side, in R15 and R16, most of the information fields in a DCI depend on the configurations of the scheduled cell, so an information field cannot be directly shared between the cells, especially if there is a big difference between the configuration of the two cells. An example is in Figure 2, CC#1 and CC#2 have different SCS, TDD configuration, and bandwidth, it would be hard for the two cells to share the same all fields. On the other side, if each functionality has two separate information fields for the cells in the DCI of joint scheduling, the DCI payload size, as well as the number of CCE consumed, may double, which does not reduce the PDCCH overhead overall and PDCCH detection reliability is also impacted.

Figure 2. Example of single scheduling and joint scheduling

It is preferable not to share the fields that heavily depend on the configuration of the scheduled cell and the fields that directly affect the transmission mechanism such as MCS, because field sharing in this case may degrade the scheduling performance. For fields that do not depend on the scheduled cells, such as PUCCH-related information that is mainly associated with PUCCH Pcell, can be considered to be shared between two cells.

In the following table, we list some major information fields and provides a preliminary analysis of the field type.

Table 2. DCI information field type

| Information field | Separate field or shared field |
| --- | --- |
| Identifier for DCI formats – 1 bit | Shared field |
| Carrier indicator – 0 or 3 bits | Shared field |
| Bandwidth part indicator – 0, 1 or 2 bits | Shared field |
| PRB bundling size indicator – 0 or 1 bit | Separate field |
| Rate matching indicator – 0, 1, or 2 bits | Separate field |
| ZP CSI-RS trigger – 0, 1, or 2 bits | Separate field |
| FDRA – 13 bits for 106 RB in 20MHz | Separate field |
| TDRA – 0, 1, 2, 3 or 4 bits | Separate field |
| Modulation and coding scheme – 5 bits | Separate field |
| New data indicator – 1 bit | Separate field |
| Redundancy version – 2 bits | Separate fields |
| HARQ process number – 4 bits | Separate field |
| VRB-to-PRB mapping – 1 bit | Shared field |
| Downlink assignment index – 0,2,4 bits | Shared field |
| TPC command for scheduled PUCCH – 2 bits | Shared field |
| PUCCH resource indicator – 3 bits | Shared field |
| PDSCH-to-HARQ feedback timing indicator – 3 bits | Shared field |
| Antenna port(s) – 4, 5, or 6 bits | Separate field |
| SRS request – 2, 3 bits | Separate field |
| DMRS sequence initialization – 1 bit. | Separate field |
| Transmission configuration indication – 0, 3 bits | Separate field |
| CBG transmission information (CBGTI) – 0 , 2, 4, 6, or 8 bits | Separate field |
| CBG flushing out information (CBGFI) – 0 ,1 bit | Separate field |
| CRC – 24 bits | Shared field |

Proposal 4. The number of bits required for joint DCI needs to be investigated to see if supporting joint scheduling brings performance gains.

In addition to the DCI size and field design mentioned above, the following issues need to be resolved as well.

- Configuration of cell pairs for joint scheduling

It is necessary to further to clarify that how to configure the cell pairs for joint scheduling and whether there are any restrictions on choosing cell pairs for joint scheduling.

- The framework of joint scheduling

In cross-carrier scheduling, UE is indicated by RRC IE CrossCarrierSchedulingConfig that the data on this cell is scheduled by PDCCH in its own cell (i.e. ‘own’ in schedulingCellInfo) or another cell (i.e. ‘other’ in schedulingCellInfo). Support of joint scheduling may require some changes to this framework.

- DCI format for joint scheduling

If an existing DCI format such as DCI format 1_1 is used for joint scheduling purpose, then the DCI for single scheduling case should always perform zero-padding to keep alignment with the DCI size of the case where joint scheduling is enabled, which implies that a large number of redundant bits will be inserted into the DCI format even for single-cell scheduling. In this case, it not only leads to a higher DCI code rate but also leads to worse coverage and performance for single-cell scheduling. By contrast, introducing a new DCI format for joint scheduling can ensure a high degree of scheduling flexibility while avoiding negative impacts on the performance of the existing DCIs. However, it raises some other issues, such as how to maintain the DCI size budget.

- PDCCH/CCE BD budget maintenance for joint scheduling

In R15, the PDCCH/CCE BD budget is maintained for each scheduled cell. Since there are two scheduled cells in the joint scheduling case, it should be discussed how to maintain the PDCCH/CCE BD budget across the cell pair.

- HARQ-ACK reporting for joint scheduling

For each single TB on a PDSCH occasion involved in a HARQ-ACK codebook in R15, there is only 1 HARQ-ACK bit generated. However, if joint scheduling is supported, some enhancements for the HARQ-ACK codebook determination are needed.

Observation 1. To support joint scheduling, the following issues need to be resolved.

- DCI field design

- Any restrictions on the scheduled cells to be paired for joint scheduling

- Framework of joint scheduling

- Whether to introduce a new DCI format

- PDCCH BD budget maintenance if joint scheduling is enabled

- HARQ-ACK codebook determination if joint scheduling is enabled

- Conclusion

In this contribution, we discuss some issues on joint scheduling and have the following observation and proposals:

Observation 1. To support joint scheduling, the following issues need to be resolved.

- DCI field design

- Any restrictions on the scheduled cells to be paired for joint scheduling

- Framework of joint scheduling

- Whether to introduce a new DCI format

- PDCCH BD budget maintenance if joint scheduling is enabled

- HARQ-ACK codebook determination if joint scheduling is enabled

Proposal 1. Joint scheduling should be studied in following scenarios

- Pcell jointly schedules two cells;

- A Scell jointly schedules Pcell and the Scell itself;

Proposal 2. The study focuses on the case that PDSCH jointly scheduled by a single DCI correspond to different TBs.

Proposal 3. Clarify whether PUSCH joint scheduling should be studied.

Proposal 4. The number of bits required for joint DCI needs to be investigated to see if supporting joint scheduling brings performance gains.

- Reference

- RP-193260, “New WID on NR Dynamic spectrum sharing (DSS)” , Ericsson, RAN #86, Dec 2019.

- RP-201352, “New SID: Efficient utilization of licensed spectrum that is not aligned with existing NR channel bandwidths”, T-Mobile USA, Ericsson, RAN#88-E, Jun 2020.