3GPP TSG RAN WG1 Meeting #102-e	R1-2005549

e-Meeting, August 17th – 28th, 2020

Agenda item:		7.2.5.2

Source:		Nokia, Nokia Shanghai Bell

Title:	Maintenance of Rel-16 URLLC UCI enhancements

Document for:		Discussion and Decision

# 1	Introduction

In this contribution, we summarize our input to the maintenance discussions on UCI (i.e. HARQ-Ack) enhancements of Rel-16 URLLC & IIoT enhancements.

Based on the guidance from RAN1 chairman, issues related to intra-UE prioritization/multiplexing previously discussed in RAN1 as part of AI 7.2.5.2 are now part of AI 7.2.5.4 and discussed in our companion contribution in R1-2005550.

In this contribution, Section 2 includes identified issues related to HARQ-ACK enhancements and Section 3 clarifies the issue on sub-slot PUCCH repetition.

# 2	HARQ-ACK CB with the new DCI formats 0_2 and 1_2 (TS 38.213, Sec. 9.1.2, 9.1.2.1, 9.1.2.2, 9.1.3.2)

In the CR phase, we introduced in some places DCI formats 0_2 and 1_2 by e.g. specifically mentioning 1_2 or 0_2 where applicable, or only referring to ‘a DCI format’ (if all UL grant / DL assignment formats are applicable). But it seems such changes are still missing in various places in Sec. 9.1.X.X – and the following logic is used here in the changes:

- 0_2 with / without DAI: DCI format 0_1  a DCI format that includes / does not include a DAI field (wording used in Sec. 9)

- 1_2 with /without total DAI: DCI format 1_1  a DCI format that includes / does not include a total DAI field (modification from the wording above for DL assignment)

- 1_2 & CBG: DCI format 1_0  a DCI format that does not support CBG-based PDSCH receptions (wording reused from Sec. 9.1.3.1)

Proposal 1: Introduce the interaction of HARQ-Ack CB generation and the new DCI formats 0_2 & 1_2 in sections 9.1.2, 9.1.2.1, 9.1.2.2 and 9.1.3.2 of TS 38.213 by adopting the following TPs with changes in red:

TP to TS 38.213, Sec. 9.1.2, 9.1.2.1, 9.1.2.2 and 9.1.3.2 on DCI formats 0_2 & 1_2 and HARQ-Ack procedures ### 9.1.2 Type-1 HARQ-ACK codebook determination This clause applies if the UE is configured with pdsch-HARQ-ACK-Codebook = semi-static. A UE reports HARQ-ACK information for a corresponding PDSCH reception or SPS PDSCH release only in a HARQ-ACK codebook that the UE transmits in a slot indicated by a value of a PDSCH-to-HARQ_feedback timing indicator field in a corresponding DCI format 1_0 or DCI format 1_1. The UE reports NACK value(s) for HARQ-ACK information bit(s) in a HARQ-ACK codebook that the UE transmits in a slot not indicated by a value of a PDSCH-to-HARQ_feedback timing indicator field in a corresponding DCI format 1_0 or DCI format 1_1. ## <Unchanged text is omitted> TP to TS 38.213, Sec. 9.1.2.1 on DCI formats 0_2 & 1_2 and HARQ-Ack procedures #### 9.1.2.1 Type-1 HARQ-ACK codebook in physical uplink control channel ## <Unchanged text is omitted> If a UE receives a SPS PDSCH, or a SPS PDSCH release, or a PDSCH that is scheduled by a DCI format 1_0 that does not support CBG-based PDSCH receptions and if - the UE is configured with one serving cell, and - , and - PDSCH-CodeBlockGroupTransmission is provided to the UE the UE generates HARQ-ACK information only for the transport block in the PDSCH or only for the SPS PDSCH release. If a UE receives a SPS PDSCH, or a SPS PDSCH release, or a PDSCH that is scheduled by a DCI format 1_0 that does not support CBG-based PDSCH receptions and if - the UE is configured with more than one serving cells, or - , and - PDSCH-CodeBlockGroupTransmission is provided to the UE the UE repeats times the HARQ-ACK information for the transport block in the PDSCH or for the SPS PDSCH release. ## <Unchanged text is omitted> A UE determines HARQ-ACK information bits, for a total number of HARQ-ACK information bits, of a HARQ-ACK codebook for transmission in a PUCCH according to the following pseudo-code. In the following pseudo-code, if the UE does not receive a transport block or a CBG, due to the UE not detecting a corresponding DCI format 1_0 or DCI format 1_1, the UE generates a NACK value for the transport block or the CBG. The cardinality of the set defines a total number of occasions for PDSCH reception or SPS PDSCH release for serving cell corresponding to the HARQ-ACK information bits. ## <Unchanged text is omitted> If , the UE determines a number of HARQ-ACK information bits for obtaining a transmission power for a PUCCH, as described in Clause 7.2.1, as where - is the number of transport blocks the UE receives in PDSCH reception occasion for serving cell if harq-ACK-SpatialBundlingPUCCH and PDSCH-CodeBlockGroupTransmission are not provided, or the number of transport blocks the UE receives in PDSCH reception occasion for serving cell if PDSCH-CodeBlockGroupTransmission is provided and the PDSCH reception is scheduled by a DCI format 1_0 that does not support CBG-based PDSCH receptions, or the number of PDSCH receptions if harq-ACK-SpatialBundlingPUCCH is provided or SPS PDSCH release in PDSCH reception occasion for serving cell and the UE reports corresponding HARQ-ACK information in the PUCCH. - is the number of CBGs the UE receives in a PDSCH reception occasion for serving cell if PDSCH-CodeBlockGroupTransmission is provided and the PDSCH reception is scheduled by a DCI format 1_1 that supports CBG-based PDSCH reception and the UE reports corresponding HARQ-ACK information in the PUCCH. ## <Unchanged text is omitted> TP to TS 38.213, Sec. 9.1.2.2 on DCI formats 0_2 & 1_2 and HARQ-Ack procedures #### 9.1.2.2 Type-1 HARQ-ACK codebook in physical uplink shared channel If a UE would multiplex HARQ-ACK information in a PUSCH transmission that is not scheduled by a DCI format or is scheduled by a DCI format 0_0 that does not include a DAI field, then - if the UE has not received any PDSCH or SPS PDSCH release that the UE transmits corresponding HARQ-ACK information in the PUSCH, based on a value of a respective PDSCH-to-HARQ_feedback timing indicator field in a DCI format scheduling the PDSCH reception or the SPS PDSCH release or on the value of dl-DataToUL-ACK if the PDSCH-to-HARQ_feedback timing indicator field is not present in the DCI format, in any of the occasions for candidate PDSCH receptions by a DCI format 1_0 or DCI format 1_1 or SPS PDSCH on any serving cell , as described in Clause 9.1.2.1, the UE does not multiplex HARQ-ACK information in the PUSCH transmission; - else the UE generates the HARQ-ACK codebook as described in Clause 9.1.2.1, except that harq-ACK-SpatialBundlingPUCCH is replaced by harq-ACK-SpatialBundlingPUSCH, unless the UE receives only a SPS PDSCH release, or only SPS PDSCH reception, or only a PDSCH that is scheduled by DCI format 1_0 with a counter DAI field value of 1 on the PCell in the occasions for candidate PDSCH receptions in which case the UE generates HARQ-ACK information only for the SPS PDSCH release or only for the PDSCH reception as described in Clause 9.1.2. A UE sets to NACK value in the HARQ-ACK codebook any HARQ-ACK information corresponding to PDSCH reception or SPS PDSCH release that the UE detects in a PDCCH monitoring occasion that starts after a PDCCH monitoring occasion where the UE detects a DCI format 0_0 or a DCI format 0_1 scheduling the PUSCH transmission. A UE does not expect to detect a DCI format switching a DL BWP within symbols prior to a first symbol of a PUSCH transmission where the UE multiplexes HARQ-ACK information, where is defined in [6, TS 38.214]. If a UE multiplexes HARQ-ACK information in a PUSCH transmission that is scheduled by DCI format 0_1 that includes a DAI field, the UE generates the HARQ-ACK codebook as described in Clause 9.1.2.1 when a value of the DAI field in the DCI format 0_1 is except that harq-ACK-SpatialBundlingPUCCH is replaced by harq-ACK-SpatialBundlingPUSCH. The UE does not generate a HARQ-ACK codebook for multiplexing in the PUSCH transmission when unless the UE receives only a SPS PDSCH release, or only SPS PDSCH(s), or only a PDSCH that is scheduled by DCI format 1_0 with a counter DAI field value of 1 on the PCell in the occasions for candidate PDSCH receptions in which case the UE generates HARQ-ACK information only for the SPS PDSCH release or only for the PDSCH reception as described in Clause 9.1.2. if the PUSCH is scheduled by a DCI format that includes a DAI field and the DAI field in the DCI format 0_1 is set to '0'; otherwise, . ## <Unchanged text is omitted> TP to TS 38.213, Sec. 9.1.3.2 on DCI formats 0_2 & 1_2 and HARQ-Ack procedures #### 9.1.3.2 Type-2 HARQ-ACK codebook in physical uplink shared channel If a UE would multiplex HARQ-ACK information in a PUSCH transmission that is not scheduled by a DCI format or is scheduled by a DCI format that does not include a DAI field, then - if the UE has not received any PDCCH within the monitoring occasions for DCI formats scheduling PDSCH receptions, or SPS PDSCH release, or DCI format 1_1 indicating SCell dormancy on any serving cell c and the UE does not have HARQ-ACK information in response to a SPS PDSCH reception, or in response to a detection of a DCI format 1_1 indicating SCell dormancy, to multiplex in the PUSCH, as described in Clause 9.1.3.1, the UE does not multiplex HARQ-ACK information in the PUSCH transmission; - else, the UE generates the HARQ-ACK codebook as described in Clause 9.1.3.1, except that harq-ACK-SpatialBundlingPUCCH is replaced by harq-ACK-SpatialBundlingPUSCH. If a UE multiplexes HARQ-ACK information in a PUSCH transmission that is scheduled by DCI format 0_1 that includes a DAI field, the UE generates the HARQ-ACK codebook as described in Clause 9.1.3.1, with the following modifications: - For the pseudo-code for the HARQ-ACK codebook generation in Clause 9.1.3.1, after the completion of the c and loops, the UE sets where is the value of the DAI field in the DCI format 0_1 scheduling the PUSCH transmission according to Table 9.1.3-2 - For the case of first and second HARQ-ACK sub-codebooks, DCI format 0_1 and DCI format 0_2 includes a first DAI field corresponding to the first HARQ-ACK sub-codebook and a second DAI field corresponding to the second HARQ-ACK sub-codebook - harq-ACK-SpatialBundlingPUCCH is replaced by harq-ACK-SpatialBundlingPUSCH. If a UE is not provided PDSCH-CodeBlockGroupTransmission and the UE is scheduled for a PUSCH transmission by DCI format 0_1 that includes a DAI field with DAI field value and the UE has not received any PDCCH within the monitoring occasions for PDCCH with a DCI format 1_0 or DCI format 1_1 for scheduling PDSCH receptions or SPS PDSCH release on any serving cell c and the UE does not have HARQ-ACK information in response to a SPS PDSCH reception to multiplex in the PUSCH, as described in Clause 9.1.3.1, the UE does not multiplex HARQ-ACK information in the PUSCH transmission. If a UE is provided PDSCH-CodeBlockGroupTransmission and the UE is scheduled for a PUSCH transmission by DCI format 0_1that includes a DAI field with first DAI field value or with second DAI field value and the UE has not received any PDCCH within the monitoring occasions for PDCCH with a DCI format 1_0 or with DCI format 1_1, respectively, for scheduling PDSCH receptions or SPS PDSCH release, or DCI format 1_1 indicating SCell dormancy, on any serving cell c and the UE does not have HARQ-ACK information in response to a SPS PDSCH reception to multiplex in the PUSCH, as described in Clause 9.1.3.1, the UE does not multiplex HARQ-ACK information for the first sub-codebook or for the second sub-codebook, respectively, in the PUSCH transmission. ## <Unchanged text is omitted>

# 3 Clarification on not supporting sub-slot PUCCH repetition

In the FL summary R1-2002695, some companies raised the issue that a clarification is needed on the fact that sub-slot PUCCH repetition is not supported in Rel-16. Our understanding is that this operation was not discussed/agreed, and thus we also believe that sub-slot PUCCH repetition should not be supported.

Proposal 2: Adopt the following TP to TS 38.213 Sec. 9 to clarify that sub-slot PUCCH repetition operation is not supported:

TP to TS 38.213, Section 9 9 UE procedure for reporting control information <Unchanged text is omitted> In the remaining of this Clause except for sub-Clause 9.2.6, if a UE is provided subslotLength-ForPUCCH, a slot for an associated PUCCH transmission includes a number of symbols indicated by subslotLength-ForPUCCH. <Unchanged text is omitted>

# 4	Conclusions

In this contribution, we discuss the identified clarifications and additions on UCI with respect to the Rel-16 URLLC & IIoT WIs. The related discussions can be summarized as:

Proposal 1: Introduce the interaction of HARQ-Ack CB generation and the new DCI formats 0_2 & 1_2 in sections 9.1.2, 9.1.2.1, 9.1.2.2 and 9.1.3.2 of TS 38.213 by adopting the proposed TPs in Section 2.

Proposal 2: Adopt the proposed TP in Section 3 to TS 38.213 Sec. 9 to clarify that sub-slot PUCCH repetition operation is not supported.

Based on the guidance by RAN1 chairman, issues related to intra-UE prioritization/multiplexing previously discussed in RAN1 as part of AI 7.2.5.2 are now part of AI 7.2.5.4 and discussed in our companion contribution in R1-2005550. In case the issues would be still discussed under AI 7.2.5.2, the interested reader is referred to the related discussions and proposals on the following identified issues related to intra-UE prioritization/multiplexing:

- Priority determination with DCI formats 0_1/1_1 and 0_2/1_2 is discussed in Sec. 2 of R1-2005550;

- Re-transmission of CBG based PUSCH with cancellation is discussed in Sec. 3 of R1-2005550;

- Handling overlapping between high-priority PUSCH without UL-SCH and SR is discussed in Sec. 4 of R1-2005550.