3GPP TSG RAN WG1 #102-e			R1-2005578

e-Meeting, August 17th – 28th, 2020

Agenda Item: 	8.5.2

Source:	Sony

Title:	Initial Views on Evaluation of Positioning Accuracy and Latency

Document for:	Discussion and Decision

# Introduction

# A study item on NR positioning enhancements in Rel-17 was approved [1]. One of the objectives is to study enhancements and solutions necessary to support the high accuracy (horizontal and vertical), low latency, network efficiency (scalability, RS overhead, etc.), and device efficiency (power consumption, complexity, etc.) requirements for commercial uses cases:

- Define additional scenarios (e.g. (I)IoT) based on TR 38.901 to evaluate the performance for the use cases (e.g. (I)IoT). [RAN1]

- Evaluate the achievable positioning accuracy and latency with the Rel-16 positioning solutions in (I)IoT scenarios and identify any performance gaps. [RAN1]

- Identify and evaluate positioning techniques, DL/UL positioning reference signals, signalling and procedures for improved accuracy, reduced latency, network efficiency, and device efficiency.
Enhancements to Rel-16 positioning techniques, if they meet the requirements, will be prioritized, and new techniques will not be considered in this case. [RAN1, RAN2]

In RAN1 #101-e, the following agreements related to positioning requirements were made [2]:

Agreement:

- In Rel-17 target positioning requirements for commercial use cases are defined as follows:

  - Horizontal position accuracy (< 1 m) for [90%] of UEs

  - Vertical position accuracy (< [2 or 3] m) for [90%] of UEs

  - End-to-end latency for position estimation of UE (< [100 ms])

  - FFS: Physical layer latency for position estimation of UE (< [10 ms])

- In Rel-17 target positioning requirements for IIoT use cases are defined as follows:

  - Horizontal position accuracy (< X m) for [90%] of UEs

    - X = [0.2 or 0.5] m

  - Vertical position accuracy (< Y m) for [90%] of UEs

    - Y = [0.2 or 1] m

  - End-to-end latency for position estimation of UE (< [10ms, 20ms, or 100ms])

  - FFS: Physical layer latency for position estimation of UE (< [10ms])

- Note: Target positioning requirements may not necessarily be reached for all scenarios

In this contribution, we provide our initial simulation results on positioning accuracy based on the agreed simulation scenarios and parameters in RAN1#101e and also our view on achieving positioning latency for commercial use-cases.

# Discussions

## Evaluation of Positioning Accuracy

The system level simulation was performed based on the agreed simulation assumptions and parameters in RAN1 101e meeting. We were evaluating both commercial use cases and IIoT use cases. For commercial use cases, we used Indoor Open Office (IOO) scenario. For IIoT use cases, we used Indoor Factory - Sparse Clutter High base-station placement (InF-SH) and Indoor Factory - Dense Clutter High base-station placement (InF-DH). We evaluate the operation in both FR1 & FR2. All the details on simulation scenarios and its parameters are given in Tables 1 to 3.

The gNB uses 8 Tx analog beam in which one OFDM symbol is occupied per PRS resource. We consider to use PRS with 1 symbol in order to also reduce the latency as Rel-17 require a stringent latency requirement as shown in the previous section. Furthermore, gNB performs Tx beam-sweeping for the PRS transmission. In our simulation, we use PRS configuration with Comb-12. The details of PRS configuration used in the simulation are shown in Table 1.

The UE performs both DL-TDOA and AoD estimation. As part of DL-TDOA operation, the UE uses adaptive threshold-based first peak detection for TOA estimate. The power delay profiles are calculated and all the peaks (local maxima) are detected. A predefined threshold which is adaptive to the estimated noise is used to filter out unqualified peaks. Among the rest of the peaks, the first one is selected as the TOA estimate and further used/reported as RSTD to LS. The location server applies the AOD estimation based on the UE measurement report together with some other prior information related to TRP (beam configuration, TRP bearing, etc.) In case of FR2, the UE performs Rx beam-forming in by combining the signals received from multiple antenna elements. To obtain the maximum beamforming gain in UE side, we first apply MUSIC algorithm to estimate the AOA and subsequently apply the corresponding combining vector to the signals.

On the location server operation (LS), we used maximum likelihood TDOA+ AOD algorithm for location estimate (MLE). Here, it is assumed that the TOA error and the AOD error in the LOS link are Gaussian distributed. A prior information of the standard deviation of the distribution is known. Then, the conditional probability of the UE location given by TOAs and AODs is derived in the location server. The MLE will search for the best UE estimate that can maximize the probability. We also introduce a method to enhance the positioning estimation accuracy by using a NLOS detection estimator (NLOS). This can be facilitated by utilizing UE report on the complex value of the first path in the cross-correlation operation in DL-TDOA. Based on N TRPs, there is always a probability that a RSTD measurement is from NLOS link. In total, there are  possible combinations which is denoted as ‘State’. The NLOS detection algorithm will calculate all the UE location candidates and select one as the best estimate.

Table 1: PRS configuration

| Parameter | [Case1, InF-SH, FR2], [Case2, InF-DH, FR2], [Case3, InF-OO, FR2] |
| --- | --- |
| Channel model (baseline, otherwise state any modifications) | Baseline (See Table 3) |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) | Comb-12 |
| Reference signal (type of sequence, number of ports, …) | Pseudo-random sequence 1 antenna port |
| Number of sites | 12 |
| Number of symbols used per slot per positioning estimate | 8 |
| Number of slots per positioning estimate | 1 |
| Power-boosting level | 0 dB |
| Uplink power control (applied/not applied) | Not applicable |
| Interference modelling (ideal muting, or other) | Ideal muting |
| Description of Measurement Algorithm (e.g. super resolution, interference cancellation, ….) | Adaptive threshold-based first peak detection for TOA measurement |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, taylor series, etc) | Maximum likelihood algorithm |
| Network synchronization assumptions | Perfect synchronization |
| Beam-related assumption (beam sweeping / alignment assumptions at the tx and rx sides) | Beam sweeping at Tx |
| Precoding assumptions (codebook, number of antenna elements used, etc.) | DFT codebook |

The simulation results based on the aforementioned simulation scenarios and parameters are shown below. Firstly, the simulation results in InF-SH scenarios using FR2 and bandwidth 400 MHz are shown in Figure 1. Using the known technique, DL-TDoA and AoD as introduced in NR Rel-16 cannot meet the target positioning accuracy requirements 0.2 m at 90% CDF. The result can be improved by operating NLOS detection together with DL-TDOA and AoD. Here, we can achieve 0.4 m at 90% CDF.

Figure 1: Simulation results in Indoor Factory Sparse High (InF-SH) using FR2 with BW 400 MHz.

The simulation results for the case of FR1 is shown in Figure 2. Here our proposed enhancement scheme (TDOA-AoD with NLOS detection) can still improve the horizontal accuracy performance. However, the performance target cannot be met using the present configuration.

Figure 2: Simulation results in Indoor Factory Sparse High (InF-SH) using FR1 with BW 100 MHz.

Observation 1: In InF-SH scenario, the target of horizontal positioning accuracy in FR2 is nearly achieved by using positioning technique enhancements, i.e. incorporating legacy DL-TDOA and AoD with NLOS detection.

The simulation results for InF-SH in FR2 with various bandwidth configuration is shown in Figure 3. Here, we consider the positioning technique: DL-TDoA+AoD with NLOS detection. It can be observed that the PRS bandwidth configuration is heavily affecting the positioning accuracy performance. The performance accuracy target cannot be made especially with the usage of PRS in smaller bandwidth configuration. This basically reflect the real deployment as the operator is expected to utilize a portion of its cell bandwidth for PRS allocation.

Figure 3: Simulation results in Indoor Factory Sparse High (InF-SH) using FR2 with various BW size.

Observation 2: The usage of PRS configuration in a smaller bandwidth degrades the positioning accuracy performance.

In Figure 4, the simulation results in InF-DH is shown. Here, we consider this as one of the most challenging scenarios. It can be observed that none of the positioning techniques considered (DL-TDoA+AOD, and DL-TDoA+AoD with NLOS detection can meet the target of horizontal positioning accuracy requirements. Further investigations are required in order to improve the positioning accuracy result.

Observation 3: In InF-DH scenario, the target of horizontal positioning accuracy cannot be met by using positioning technique enhancements, i.e. incorporating legacy DL-TDOA and AoD with NLOS detection.

Figure 4: Simulation results in Indoor Factory Dense High (InF-DH) using FR2 with BW 400 MHz.

Furthermore, we have also evaluated the positioning techniques in one of the commercial use-cases, that is for Indoor Open Office scenario (InH-OO) and shown in Figure 5. The result of using the legacy positioning technique (DL-TDOA + AoD) cannot met the target of horizontal positioning accuracy. Incorporating NLOS detection together with the legacy positioning technique improve the performance and obtaining 0.7m horizontal positioning accuracy at 90% CDF.

Figure 5: Simulation results in Indoor Open Office (InH-OO) using FR2 with BW 400 MHz.

Observation 4: In InH-OO scenario, the target of horizontal positioning accuracy can be met by using positioning technique enhancements, i.e. incorporating legacy DL-TDOA and AoD with NLOS detection.

## Positioning Latency Requirement Aspects

According to [3], the definition of positioning service latency is the time elapsed between the event that triggers the determination of the position-related data and the availability of the position-related data at the system interface. We can consider this as the end-to-end positioning latency.

One example of the process in obtaining positioning measurement in DL-TDoA is elaborated here. The signalling of positioning in the legacy radio access network can be illustrated in Figure 6. LS sends positioning a measurement request to the UE via LPP protocol (and transparent to gNB). In case of inter-frequency measurement, the UE performs measurements within a measurement gap, particularly when PRS from multiple gNB(s) are scheduled. Once the UE has obtained the positioning measurement data, the UE sends an uplink scheduling request (SR) to the serving gNB. The serving gNB sends the corresponding UL grant in downlink control channel (PDCCH). The UE then finally can transmit the positioning measurement reports in uplink data channel (PUSCH) to the LS. Here, we can consider that the positioning latency involves both higher layer signalling and layer-1 signalling.

Figure 6: Signalling mechanism in DL-TDOA positioning technique

PRS configuration

PRS configuration

SR request

Meas GAP

PRS transmission

Positioning meas request

UE

gNB

LS

POS meas

UL grant

POS meas_rep

Observation 5: Multiple and consecutive signallings (i.e. higher layer signalling and layer 1 signalling) increase the end-to-end positioning latency.

Considering that NR Rel-17 is targeting significantly lower end-to-end latency, any attempt to reduce the signalling can be beneficial in order to reduce the latency and meet the latency requirement. Based on the above descriptions, we can consider the following layer-1 operation that may introduce high latency:

- Large PRS periodicity. Hence, it would be beneficial to study the transmission of aperiodic PRS in an attempt to reduce latency.

- Multiple signalling in positioning measurement report mechanism. It would be beneficial to study a fast positioning measurement report performed by the UE.

Proposal 1: RAN1 to study the operation of aperiodic PRS and fast positioning measurement report in order to meet positioning latency requirements.

# Conclusion

Our views and considerations on evaluation of positioning accuracy and latency are discussed in this contribution. We have made the following observation and proposals:

Observation 1: In InF-SH scenario, the target of horizontal positioning accuracy in FR2 is nearly achieved by using positioning technique enhancements, i.e. incorporating legacy DL-TDOA and AoD with NLOS detection.

Observation 2: The usage of PRS configuration in a smaller bandwidth degrades the positioning accuracy performance.

Observation 3: In InF-DH scenario, the target of horizontal positioning accuracy cannot be met by using positioning technique enhancements, i.e. incorporating legacy DL-TDOA and AoD with NLOS detection.

Observation 4: In InH-OO scenario, the target of horizontal positioning accuracy can be met by using positioning technique enhancements, i.e. incorporating legacy DL-TDOA and AoD with NLOS detection.

Observation 5: Multiple and consecutive signallings (i.e. higher layer signalling and layer 1 signalling) increase the end-to-end positioning latency.

Proposal 1: RAN1 to study the operation of aperiodic PRS and fast positioning measurement report in order to meet positioning latency requirements.

# References

- RP-193237 SID: NR Positioning Enhancements

- TR 38.855 Study on NR positioning support

- TS 22.261 “Service requirements for the 5G system”

- TR 22.804 “Study on Communication for Automation in Vertical Domains”

- https://sites.google.com/a/co911rc.org/co911rc/resources/wireless-9-1-1

- https://en.wikipedia.org/wiki/Time_to_first_fix

# Appendix A

Table 2: Common simulation parameters

|  | FR1 Specific Values | FR2 Specific Values |
| --- | --- | --- |
| Carrier frequency, GHz | 3.5GHz | 28GHz |
| Bandwidth, MHz | 100MHz | 400MHz |
| Subcarrier spacing, kHz | 30kHz | 120kHz |
| gNB model parameters |  |  |
| gNB noise figure, dB | 5dB | 7dB |
| UE model parameters |  |  |
| UE noise figure, dB | 9dB – Note 1 | 13dB – Note 1 |
| UE max. TX power, dBm | 23dBm – Note 1 | 23dBm – Note 1 EIRP should not exceed 43 dBm. |
| UE antenna configuration | Panel model 1 – Note 1 Mg = 1, Ng = 1, P = 2, dH = 0.5λ, (M, N, P, Mg, Ng) = (1, 2, 2, 1, 1) | Baseline: Multi-panel Configuration 1 and Panel Configuration a – Note 1 - Multi-panel Configuration 1: (Mg, Ng) = (1, 2); Θmg,ng=90°; Ω0,1=Ω0,0+180°; (dg,H, dg,V)=(0,0) - Panel Configuration a: - Each antenna array has shape dH=dV=0.5λ - Config a: (M, N, P) = (2, 4, 2), - the polarization angles are 0° and 90° - The antenna elements of the same polarization of the same panel is virtualized into one TXRU |
| UE antenna radiation pattern | Omni, 0dBi | Antenna model according to Table 6.1.1-2 in TR 38.855 |
| PHY/link level abstraction | Explicit simulation of all links, individual parameters estimation is applied. Description of applied algorithms for estimation of signal location parameters (See Table 1). |  |
| Network synchronization | Perfect Synchronization |  |
|  | Note 1: According to 3GPP TR 38.802 Note 2: According to 3GPP TR 38.901 |  |

Table 3: Assumptions of InF scenario

|  |  | FR1 Specific Values |  | FR2 Specific Values |
| --- | --- | --- | --- | --- |
| Channel model |  | InF-SH, InF-DH |  |  |
| Layout | Hall size | InF-SH: (baseline) 300x150 m InF-DH: (baseline) 120x60 m |  |  |
|  | BS locations | 18 BSs on a square lattice with spacing D, located D/2 from the walls. - for the small hall (L=120m x W=60m): D=20m - for the big hall (L=300m x W=150m): D=50m |  |  |
|  | Room height | 10m |  |  |
| Total gNB TX power, dBm |  | 24dBm | 24dBm EIRP should not exceed 58 dBm |  |
| gNB antenna configuration |  | (M, N, P, Mg, Ng) = (4, 4, 2, 1, 1), dH=dV=0.5λ – Note 1 | (M, N, P, Mg, Ng) = (4, 8, 2, 1, 1), dH=dV=0.5λ – Note 1 One TXRU per polarization per panel is assumed |  |
| gNB antenna radiation pattern |  | Single sector – Note 1 | 3-sector antenna configuration – Note 1 |  |
| Peneteration loss |  | 0dB |  |  |
| Number of floors |  | 1 |  |  |
| UE horizontal drop procedure |  | Uniformly distributed over the horizontal evaluation area for obtaining the CDF values for positioning accuracy, The evaluation area should be at least the convex hull of the horizontal BS deployment. It can also be the whole hall area if the CDF values for positioning accuracy is obtained from whole hall area. |  |  |
| UE antenna height |  | Baseline: 1.5m |  |  |
| UE mobility |  | 3km/h |  |  |
| Min gNB-UE distance (2D), m |  | 0m |  |  |
| gNB antenna height |  | Baseline: 8m |  |  |
| Clutter parameters: {density , height ,size } |  | Low clutter density: {20%, 2m, 10m} High clutter density: {40%, 2m, 2m} |  |  |
| Note 1: According to Table A.2.1-7 in 3GPP TR 38.802 |  |  |  |  |