3GPP TSG RAN WG1 Meeting #102-e	R1-2005817

E-meeting, August 17-28, 2020

Agenda Item:	6.2.3

Source:	Huawei, HiSilicon

Title:	Corrections on additional SRS symbols

Document for:	Discussion and Decision

# Introduction

In RAN#88, the CRs on LTE DL MIMO enhancement in [1] have been approved. In this contribution, discussions and text proposals are given to correct the specifications.

# Discussion

## Spec alignment

When capturing the additional SRS symbols transmitted in uplink normal subframes, the term “additional sounding reference signal” is used in TS 36.211 as below:

### 5.5.3	Sounding reference signal

Two types of sounding reference signals can be configured:

-	basic sounding reference signal, supporting periodic or aperiodic transmission

-	additional sounding reference signal, supporting aperiodic transmission only

While in TS 36.213, it is referred to as “triggering type 2”:

A UE shall transmit Sounding Reference Symbol (SRS) on per serving cell SRS resources based on three trigger types:

-	trigger type 0: higher layer signalling

-	trigger type 1: DCI formats 0/0A/0B/4/4A/4B/1A/6-0A/6-1A for FDD, TDD, and frame structure type 3 and DCI formats 2B/2C/2D/3B for TDD, and frame structure type 3, and DCI format 7-0A/7-0B/7-1E/7-1F/7-1G for TDD if the UE is configured by higher layers for SRS triggering via DCI format 7-0A and has indicated the capability srs-DCI7-Triggering-FS2-r15/ srs-DCI7-Triggering-FS2-r16 and the UE is configured for SRS triggering with srs-DCI7-TriggeringConfig-r15/ srs-DCI7-Triggering-FS2-r16.

-	trigger type 2: DCI formats 0/4/1A/6-0A/6-1A for FDD and TDD, and DCI formats 2B/2C/2D/3B for TDD, and DCI format 7-0A/7-0B/7-1E/7-1F/7-1G for TDD if the UE is configured by higher layers for SRS triggering via DCI format 7-0A and has indicated the capability srs-DCI7-Triggering-FS2-r16 and the UE is configured for SRS triggering with srs-DCI7-TriggeringConfig-r16.

Therefore, the terms between the specs needs to be aligned. As the essential difference between the legacy SRS and additional SRS is that the additional SRS is configured to the first 13 symbols in normal uplink subframes, therefore, they can be distinguished by the different mapping in 36.211. However, the related section in 36.213 is by the trigger type, and the difference between trigger type 1 and trigger type 2 is not sufficient to distinguish legacy and additional SRS symbols. Therefore, it is proposed as below to align the specs:

Proposal 1: Endorse the text proposal to 36.213 in section 2.1.

=========================Start of text proposal 1 to 36.213===========================

<Unchanged parts are omitted>

## 8.2	UE sounding procedure

If the UE is configured with a PUCCH-SCell, the UE shall apply the procedures described in this clause for both primary PUCCH group and secondary PUCCH group unless stated otherwise

- When the procedures are applied for the primary PUCCH group, the terms 'secondary cell', 'secondary cells', 'serving cell', and 'serving cells' in this clause refer to secondary cell, secondary cells, serving cell or serving cells belonging to the primary PUCCH group respectively unless stated otherwise.

- When the procedures are applied for secondary PUCCH group, the terms 'secondary cell', 'secondary cells', 'serving cell' and 'serving cells' in this clause refer to secondary cell, secondary cells (not including the PUCCH-SCell), serving cell, serving cells belonging to the secondary PUCCH group respectively unless stated otherwise. The term 'primary cell' in this clause refers to the PUCCH-SCell of the secondary PUCCH group.

A UE shall transmit Sounding Reference Symbol (SRS) on per serving cell SRS resources based on three trigger types:

-	trigger type 0: higher layer signalling

-	trigger type 1: triggering of basic sounding reference signals as defined in subclause 5.5.3 of [3], by DCI formats 0/0A/0B/4/4A/4B/1A/6-0A/6-1A for FDD, TDD, and frame structure type 3 and DCI formats 2B/2C/2D/3B for TDD, and frame structure type 3, and DCI format 7-0A/7-0B/7-1E/7-1F/7-1G for TDD if the UE is configured by higher layers for SRS triggering via DCI format 7-0A and has indicated the capability srs-DCI7-Triggering-FS2  and the UE is configured for SRS triggering with srs-DCI7-TriggeringConfig .

- trigger type 2: triggering of additional sounding reference signals as defined in subclause 5.5.3 of [3], by DCI formats 0/4/1A/6-0A/6-1A for FDD and TDD, and DCI formats 2B/2C/2D/3B for TDD, and DCI format 7-0A/7-0B/7-1E/7-1F/7-1G for TDD if the UE is configured by higher layers for SRS triggering via DCI format 7-0A and has indicated the capability srs-DCI7-Triggering-FS2 and the UE is configured for SRS triggering with srs-DCI7-TriggeringConfig.

<Unchanged parts are omitted>

=========================End of text proposal to 36.213===========================

# Conclusions

In this contribution, remaining issues on additional SRS symbols are discussed with following proposal:

Proposal 1: Endorse the text proposal to 36.213 in section 2.1.

# References

3GPP, RP-200700 Corrections to DL MIMO efficiency enhancements for LTE, RAN#88e, RAN1 .