3GPP TSG RAN WG1 #102-e                                                                                R1-2005982

e-Meeting, August 17th – 28th, 2020

Source:	OPPO

Title:	Discussion on the introduction of new bandwidths of CSI-RS for tracking

Agenda Item:	7.2.12

Document for:	Discussion and Decision

Introduction

In RAN#88e meeting, there was a discussion on the support of flexible bandwidth operation. As a result, new bandwidths of CSI-RS for tracking were proposed. The final outcomes of the discussion were agreed as in RP-201333 [1]:

- Task RAN1 (cc: RAN2) to define TRS bandwidth sizes of 28, 32, 36, 40, 44, 48 RBs. - All TRS configured for a given BWP with the newly defined TRS bandwidth sizes for a UE span the same set of RBs. - All allocated PDSCH RBs are confined within the bandwidth spanned by TRS + up to 3RBs beyond either/both of the highest RB and lowest RB of the TRS. - Only supported for 10MHz UE channel bandwidth, 52 RB BWP size, and 15kHz SCS, in FDD bands. - Note: No new performance requirement on UE is introduced here. - A “per-band” UE capability is to be defined for this optional UE feature, that indicates per band support for one of: - “All newly defined TRS bandwidth sizes” - “All newly defined TRS bandwidth sizes except 28 RB size” - Introduce from Release 16 as part of TEI16

In this contribution, we present our views on new bandwidths of CSI-RS for tracking and propose some text proposals to update the specification.

Discussion

As agreed in RP-201333 [1], the new bandwidths of CSI-RS are with 4-PRB steps from 28 to 48 RBs. If a PDSCH has more 4 PRBs than TRS, then gNB should configure larger TRS bandwidth for better UE performance. Thus we have the following the proposal:

Proposal 1: Clarify that the allocated PDSCH RBs are confined within the bandwidth spanned by TRS + up to a total number of 3RBs beyond both of the highest RB and lowest RB of the TRS

Based on RP-201333 and Proposal 1, we give the following text proposal

In TS 38. 214 Section 5.1.6.1.1 <omitted text> A UE does not expect to be configured with a NZP-CSI-RS-ResourceSet configured both with trs-Info and repetition. Each CSI-RS resource, defined in Clause 7.4.1.5.3 of [4, TS 38.211], is configured by the higher layer parameter NZP-CSI-RS-Resource with the following restrictions: - the time-domain locations of the two CSI-RS resources in a slot, or of the four CSI-RS resources in two consecutive slots (which are the same across two consecutive slots), as defined by higher layer parameter CSI-RS-resourceMapping, is given by one of - , , or for frequency range 1 and frequency range 2, - , , , , , or for frequency range 2. - a single port CSI-RS resource with density given by Table 7.4.1.5.3-1 from [4, TS 38.211] and higher layer parameter density configured by CSI-RS-ResourceMapping. - the bandwidth of the CSI-RS resource, as given by the higher layer parameter freqBand configured by CSI-RS-ResourceMapping, is the minimum of 52 and NBWP,isize resource blocks, or is equal to NBWP,isize resource blocks. For operation with shared spectrum channel access, freqBand configured by CSI-RS-ResourceMapping, is the minimum of 48 and NBWP,isize resource blocks, or is equal to NBWP,isize resource blocks. For operation with the FDD bands where subcarrier spacing configuration μ is 0 and the maximum transmission bandwidth configuration NRB for UE channel bandwidth is 52 RBs [8, TS 38.101-1], freqBand configured by CSI-RS-ResourceMapping can be one of {28, 32, 36, 40, 44, 48}. - the UE is not expected to be configured with the periodicity of slots if the bandwidth of CSI-RS resource is larger than 52 resource blocks. - the periodicity and slot offset for periodic NZP CSI-RS resources, as given by the higher layer parameter periodicityAndOffset configured by NZP-CSI-RS-Resource, is one of slots where 10, 20, 40, or 80 and where µ is defined in Clause 4.3 of [4, TS 38.211]. - same powerControlOffset and powerControlOffsetSS given by NZP-CSI-RS-Resource value across all resources. If freqBand configured by CSI-RS-ResourceMapping is one of {28, 32, 36, 40, 44, 48}, UE does not expect that the total number of PRBs allocated for PDSCH but not overlapped with the PRBs carrying CSI-RS for tracking is more than 3. <omitted text>

Conclusion

In this contribution, we present our view on flexible bandwidth operation. The following proposal and the corresponding text proposal are provided.

Proposal 1: Clarify that the allocated PDSCH RBs are confined within the bandwidth spanned by TRS + up to a total number of 3RBs beyond both of the highest RB and lowest RB of the TRS

References

- RP-201333, Final Proposal for Flex_BW

- TS 38.213, NR; Physical layer procedures for control

- TS 38.331, NR; Radio Resource Control (RRC) protocol specification

- TS 38.306, NR; User Equipment (UE) radio access capabilities

- R1-2003262, LS reply on uplink power control for NR-NR Dual-Connectivity