3GPP TSG RAN WG1 #102-e		             R1-2006170

e-Meeting, August 17th – 28th, 2020

Agenda item:	8.11.2.1

Source: 	Samsung

Title: 	On resource allocation for power saving

Document for:	Discussion and Decision

# Introduction

UE-to-UE direct communication is one important scenario developed by 3GPP with a series of techniques including LTE D2D, LTE V2X and NR sidelink. The first standard of NR sidelink was completed in Rel-16 Work Item “5G V2X with NR sidelink”, in which the sidelink technique focusing on vehicle-to-everything (V2X) was specified. In Rel-17, a wider scope of sidelink applications was expected, and a new Rel-17 Work Item on “NR sidelink enhancement” was approved in RAN#86 [1] to support various use cases in both public safety and commercial scenarios. The WID was further updated in RAN#88-e [2] with modification on timeline.

Rel-16 NR sidelink mainly focus on vehicle UEs which have sufficient battery supply. In Rel-17, power sensitive UEs such as pedestrian UE and UE in public safety and commercial use cases are involved in the typical use cases. Therefore, power saving for NR sidelink was introduced in Rel-17 WI to allow SL UE operates in a power efficient manner with the following objective:

- Specify resource allocation to reduce power consumption of the UEs [RAN1, RAN2] - Baseline is to introduce the principle of Rel-14 LTE sidelink random resource selection and partial sensing to Rel-16 NR sidelink resource allocation mode 2. - Note: Taking Rel-14 as the baseline does not preclude introducing a new solution to reduce power consumption for the cases where the baseline cannot work properly.

In this contribution, a preliminary consideration on possible solutions for NR sidelink power saving and potential issues of reusing LTE technique are provided.

# Discussion

In Rel-16 NR sidelink mode 2 operation, long term sensing window is specified to detect resource reservations and avoid collision as much as possible. However, the sensing procedure can hardly be supported by power sensitive UEs with limited battery capacity. LTE Rel-14 V2X introduces random selection and partial sensing as a trade-off between power consumption and reliability. The basic rule of random selection and partial sensing can be reused in NR sidelink with essential enhancements.

## Random selection

Random selection significantly reduces power consumption by skipping sensing procedure, but cannot avoid collision by selecting resource randomly, thus introduce a degradation on PRR performance of legacy sidelink UE. In addition, the collision between Rel-17 UEs using random selection may become non-negligible when the density of UE using random selection increases. Therefore, some evaluation on the performance of random selection is necessary to estimate the feasibility of random selection and suitable scenarios.

In LTE V2X, the principle of random selection is that UE determines resource selection window as a candidate resource set, and randomly select among all resources within candidate resource set unless the resource are unavailable e.g. restricted by half-duplex or simultaneous transmission. Enhancements on LTE random selection should also be considered in Rel-17 to improve the PRR performance of random selection and reduce its impact on sidelink UEs using full/partial sensing. Considering the PRR performance of random selection in different scenarios will change, the conditions of using random selection for a given transmission can be further discussed to refine the scenarios appropriate for random selection. The generation of candidate resource set during random selection can also be improved according to L1 priority or sidelink service type to satisfy the various requirement of different sidelink service types.

Proposal 1: Enhancement of random resource selection is studied to reduce collision probability among UEs.

## Partial sensing

In Rel-14 LTE V2X, partial sensing is an effective way to balance power consumption and collision avoidance for pedestrian UEs. The principle of partial sensing is that UE determine a set of subframe with consists of at least Y subframe in resource selection window as a candidate resource set, then for each subframe  within the candidate resource set, UE senses on a subset of previous subframes , on which other sidelink UEs may reserve resource on subframe  for periodical traffics. Then UE performs RSRP-based resource exclusion and RSSI-based ranking within the candidate resource set according to sensing result.

With limited sensing window length by monitoring periodical subframes corresponding resource selection window, LTE pedestrian UEs can detect potential collisions and estimate the quality of each resource within resource selection window. The principle can be similarly reused in Rel-17 NR sidelink. NR sidelink UE can generate the candidate resource set, then for each slot  within the candidate resource set, NR sidelink UE further determine a set of previous slots on which other sidelink UE may reserve resource on slot . UE perform sensing on each set and then exclude candidate resources according to the SCI decoded in the sensing slot sets.

The feasibility of full or partial sensing procedure of LTE V2X is based on a precondition that the most traffics in LTE sidelink are periodical, and the impact of event-triggered traffics was evaluated as acceptable. UE could predict when the packet arrives from higher layer to be transmitted, and after receiving one transmission from other sidelink UE, UE could also predict its following transmissions in subsequent periods. Therefore, a simple back tracking solution with periodicity configured by higher layer is good enough to protect the PRR performance in Mode 2.

However, NR sidelink considers scenarios of aperiodic transmission importantly and sensing mechanism is targeting both periodic and aperiodic traffic types. Therefore, the performance of LTE-like partial sensing needs further evaluation and may not satisfy reliability requirement of some NR traffics.

It is beneficial to consider enhancement on partial sensing to allow it estimating interference from not only periodic traffic but also aperiodic traffic. For example, per slot  within candidate resource corresponds to a set of slots that may reserve resources on slot  for aperiodic traffic, and the set can be determined by the range of SCI reservation, such as up to 32 slots before slot . It will sacrifice power consumption since the number of slots within sensing window will increase. In addition, we can consider introducing re-evaluation check and/or pre-emption check procedure in NR partial sensing to avoid collision on the SL and improve PRR. However, trade-off between performance gain and power consumption should be carefully analyzed for partial sensing enhancement in NR sidelink.

Proposal 2: Reusing the principle of partial sensing in LTE sidelink, and consider the feasibility for partial sensing corresponding to both aperiodic and periodic traffics.

In addition, partial sensing for NR sidelink may also needs to consider Rel-17 sidelink enhancements such as inter-UE coordination and sidelink DRX. For example, the information acquired via inter-UE coordination may be utilized in partial sensing procedure, and sidelink DRX configuration may have impact on the determination of partial sensing window. Therefore, enhancement on LTE partial sensing is needed to cover relevant Rel-17 sidelink features.

Proposal 3: Enhancement of partial sensing is studied to consider new aspects introduced in Rel-17 NR sidelink such as inter-UE coordination, sidelink DRX and so on.

# Conclusions

In this contribution, the issues in random selection and partial sensing procedures are discussed with the following proposals:

Proposal 1: Enhancement of random resource selection is studied to reduce collision probability among UEs.

Proposal 2: Reusing the principle of partial sensing in LTE sidelink, and consider the feasibility for partial sensing corresponding to both aperiodic and periodic traffics.

Proposal 3: Enhancement of partial sensing is studied to consider new aspects introduced in Rel-17 NR sidelink such as inter-UE coordination, sidelink DRX and so on.

# References

[1] RP‑193231, “NR Sidelink enhancement,” RAN#86, Stiges, Spain, December, 2019.

[2] RP-201385, “NR Sidelink enhancement,” RAN#88-e, e-meeting, June, 2020.