3GPP TSG RAN WG1#102-e		R1-2006256

e-Meeting, August 17th – 28th, 2020

Agenda Item:     7.2.6

Source:	Spreadtrum Communications

Title:	Discussion on remaining issues on UL full power transmission

Document for:	Discussion and decision

Introduction

In this contribution, we present our opinions on the remaining issues for UL full power transmission, and provide related text proposals.

Discussion

Last meeting, we have spent some time on discussing about whether UE could be configured with single port SRS resource with usage set to be ‘codebook’ and fullpowerMode1 simultaneously. From the perspective of technical aspects, definitely it could be configured. But on the other hand, when single port SRS resource is configured, in some degree, it could not actually reflect the definition of fullpowerMode1 where UE uses a subset of TPMIs that combine ports in a layer to produce full power transmission. Thus, after email discussion and compromise, it seems to reach a consensus that UE shall not expect to be configured with single port SRS resource with usage as codebook and fullpowerMode1 simultaneously. Given that the current TS38.214 specification still allows UE to be configured with single port SRS resource with usage as codebook and fullpowerMode1 simultaneously, in our opinion, it is necessary to make one official conclusion or one CR to make the specification more clear.

Proposal 1: For the purpose of making specification more clear, we support the following conclusion

- Conclusion: A UE shall not expect to be configured with higher layer parameter ul-FullPowerTransmission-r16 set to ‘fullpowerMode1’ and SRS-ResourceSet with usage set to ‘codebook’ with single port SRS resource simultaneously.

Based on the above conclusion, we proposed one corresponding text proposal as below:

Proposal 2: Based on the above, we have the following text proposal#1 for TS38.214.

---------------------------------------Start of Text proposal#1 for TS 38.214-----------------------------------------

6.1.1.1	Codebook based UL transmission

--------------------------------------------------Unchanged text omitted------------------------------------------------

A UE shall not expect to be configured with higher layer parameter ul-FullPowerTransmission set to 'fullpowerMode1' and codebookSubset or codebookSubset-ForDCIFormat0_2 set to 'fullAndPartialAndNonCoherent' simultaneously.

A UE shall not expect to be configured with higher layer parameter ul-FullPowerTransmission-r16 set to ‘fullpowerMode1’ and SRS-ResourceSet with usage set to ‘codebook’ with single port SRS resource simultaneously.

The UE shall transmit PUSCH using the same antenna port(s) as the SRS port(s) in the SRS resource indicated by the DCI format 0_1 or 0_2 or by configuredGrantConfig according to clause 6.1.2.3.

---------------------------------------End of Text proposal#1 for TS 38.214------------------------------------------

According to the latest TS38.331 specification, UL full power transmission schemes related parameter naming in PUSCH-Config is as the following shows. Thus, RAN1 specification, e.g., TS38.214, TS38.213, TS38.212, should be updated to keep consistent with TS38.331.

ul-FullPowerTransmission-r16 ENUMERATED {fullpower, fullpowerMode1, fullpoweMode2} OPTIONAL, -- Need R

Proposal 3: Based on the above, we have the following text proposal#2 for TS38.213, text proposal#3 for TS38.214, and text proposal#4 for TS38.212.

TP#2:

------------------------------------------Start of Text Proposal#2 for TS 38.213--------------------------------------

7.1	Physical uplink shared channel

For a PUSCH transmission on active UL BWP , as described in Clause 12, of carrier  of serving cell , a UE first calculates a linear value  of the transmit power , with parameters as defined in Clause 7.1.1. For a PUSCH transmission scheduled by a DCI format other than DCI format 0_0, or configured by ConfiguredGrantConfig or semiPersistentOnPUSCH, if txConfig in PUSCH-Config is set to 'codebook',

-	if ul-FullPowerTransmission-r16 in PUSCH-Config is provided, the UE scales PPUSCH,b,f,c(i,j,qd,l) by s where:

-	if ul-FullPowerTransmission-r16 in PUSCH-Config is set to fullpowerMode1, and each SRS resource in the SRS-ResourceSet with usage set to 'codebook' has more than one SRS port, s is the ratio of a number of antenna ports with non-zero PUSCH transmission power over the maximum number of SRS ports supported by the UE in one SRS resource

-	if ul-FullPowerTransmission-r16 in PUSCH-Config is set to fullpowerMode2,

-	s=1 for full power TPMIs reported by the UE [16, TS 38.306], and s is the ratio of a number of antenna ports with non-zero PUSCH transmission power over a number of SRS ports for remaining TPMIs, where the number of SRS ports is associated with a SRS resource indicated by a SRI field in a DCI format scheduling the PUSCH transmission if more than one SRS resource is configured in the SRS-ResourceSet with usage set to 'codebook', or indicated by Type 1 configured grant, or the number of SRS ports is associated with the SRS resource if only one SRS resource is configured in the SRS-ResourceSet with usage set to 'codebook',

-	s=1, if a SRS resource with a single port is indicated by a SRI field in a DCI format scheduling the PUSCH transmission when more than one SRS resource is provided in the SRS-ResourceSet with usage set to 'codebook', or indicated by Type 1 configured grant, or if only one SRS resource with a single port is provided in the SRS-ResourceSet with usage set to 'codebook', and

-	if ul-FullPowerTransmission-r16 in PUSCH-Config is set to fullpower, s=1

------------------------------------------End of Text Proposal#2 for TS 38.213--------------------------------------

TP#3:

---------------------------------------Start of Text Proposal#3 for TS 38.214-----------------------------------------

6.1.1.1	Codebook based UL transmission

---------------------------------------------------Unchanged text omitted-------------------------------------------

For codebook based transmission, the UE determines its codebook subsets based on TPMI and upon the reception of higher layer parameter codebookSubset in pusch-Config for PUSCH associated with DCI format 0_1 and codebookSubset-ForDCIFormat0_2 in pusch-Config for PUSCH associated with DCI format 0_2 which may be configured with 'fullyAndPartialAndNonCoherent', or 'partialAndNonCoherent', or 'nonCoherent' depending on the UE capability. When higher layer parameter ul-FullPowerTransmission-r16 is set to 'fullpowerMode2' and the higher layer parameter codebookSubset or the higher layer parameter codebookSubset-ForDCIFormat0_2 is set to 'partialAndNonCoherent', and when the SRS-resourceSet with usage set to "codebook" includes at least one SRS resource with 4 ports and one SRS resource with 2 ports, the codebookSubset associated with the 2-port SRS resource is 'nonCoherent'. The maximum transmission rank may be configured by the higher layer parameter maxRank in pusch-Config for PUSCH scheduled with DCI format 0_1 and maxRank-ForDCIFormat0_2 for PUSCH scheduled with DCI format 0_2.

---------------------------------------------------Unchanged text omitted-------------------------------------------

For codebook based transmission, the UE may be configured with a single SRS-ResourceSet with usage set to 'codebook' and only one SRS resource can be indicated based on the SRI from within the SRS resource set. Except when higher layer parameter ul-FullPowerTransmission-r16 is set to 'fullpowerMode2', the maximum number of configured SRS resources for codebook based transmission is 2. If aperiodic SRS is configured for a UE, the SRS request field in DCI triggers the transmission of aperiodic SRS resources.

A UE shall not expect to be configured with higher layer parameter ul-FullPowerTransmission-r16 set to 'fullpowerMode1' and codebookSubset or codebookSubset-ForDCIFormat0_2 set to 'fullAndPartialAndNonCoherent' simultaneously.

The UE shall transmit PUSCH using the same antenna port(s) as the SRS port(s) in the SRS resource indicated by the DCI format 0_1 or 0_2 or by configuredGrantConfig according to clause 6.1.2.3.

The DM-RS antenna ports  in Clause 6.4.1.1.3 of [4, TS38.211] are determined according to the ordering of DM-RS port(s) given by Tables 7.3.1.1.2-6 to 7.3.1.1.2-23 in Clause 7.3.1.1.2 of [5, TS 38.212].

Except when higher layer parameter ul-FullPowerTransmission-r16 is set to 'fullpowerMode2', when multiple SRS resources are configured by SRS-ResourceSet with usage set to 'codebook', the UE shall expect that higher layer parameters nrofSRS-Ports in SRS-Resource in SRS-ResourceSet shall be configured with the same value for all these SRS resources.

When higher layer parameter ul-FullPowerTransmission-r16 is set to 'fullpowerMode2',

-	the UE can be configured with one SRS resource or multiple SRS resources with same or different number of SRS ports within an SRS resource set with usage set to 'codebook'.

-	up to 2 different spatial relations can be configured for all SRS resources in the SRS resource set with usage set to 'codebook' when multiple SRS resources are configured in the SRS resource set.

-	subject to UE capability, a maximum of 2 or 4 SRS resources are supported in an SRS resource set with usage set to 'codebook'

---------------------------------------End of Text Proposal#3 for TS 38.214------------------------------------------

TP#4:

---------------------------------------Start of Text Proposal#4 for TS 38.212-----------------------------------------

7.3.1.1.2	Format 0_1

---------------------------------------------------Unchanged text omitted-------------------------------------------

- Precoding information and number of layers – number of bits determined by the following:

-	0 bits if the higher layer parameter txConfig = nonCodeBook;

-	0 bits for 1 antenna port and if the higher layer parameter txConfig = codebook;

-	4, 5, or 6 bits according to Table 7.3.1.1.2-2 for 4 antenna ports, if txConfig = codebook, ul-FullPowerTransmission-r16 is not configured or configured to fullpowerMode2 or configured to fullpower, and according to whether transform precoder is enabled or disabled, and the values of higher layer parameters maxRank, and codebookSubset;

-	4 or 5 bits according to Table 7.3.1.1.2-2A for 4 antenna ports, if txConfig = codebook, ul-FullPowerTransmission-r16= fullpowerMode1, maxRank=2, transform precoder is disabled, and according to the values of higher layer parameter codebookSubset;

-	4 or 6 bits according to Table 7.3.1.1.2-2B for 4 antenna ports, if txConfig = codebook, ul-FullPowerTransmission-r16= fullpowerMode1, maxRank=3 or 4, transform precoder is disabled, and according to the values of higher layer parameter codebookSubset;

-	2, 4, or 5 bits according to Table 7.3.1.1.2-3 for 4 antenna ports, if txConfig = codebook, ul-FullPowerTransmission-r16 is not configured or configured to fullpowerMode2 or configured to fullpower, and according to whether transform precoder is enabled or disabled, and the values of higher layer parameters maxRank, and codebookSubset;

-	3 or 4 bits according to Table 7.3.1.1.2-3A for 4 antenna ports, if txConfig = codebook, ul-FullPowerTransmission-r16= fullpowerMode1, maxRank=1, and according to whether transform precoder is enabled or disabled, and the values of higher layer parameter codebookSubset;

-	2 or 4 bits according to Table7.3.1.1.2-4 for 2 antenna ports, if txConfig = codebook, ul-FullPowerTransmission-r16 is not configured or configured to fullpowerMode2 or configured to fullpower, and according to whether transform precoder is enabled or disabled, and the values of higher layer parameters maxRank and codebookSubset;

-	2 bits according to Table 7.3.1.1.2-4A for 2 antenna ports, if txConfig = codebook, ul-FullPowerTransmission-r16= fullpowerMode1, transform precoder is disabled, maxRank=2, and codebookSubset=nonCoherent;

-	1 or 3 bits according to Table7.3.1.1.2-5 for 2 antenna ports, if txConfig = codebook, ul-FullPowerTransmission-r16 is not configured or configured to fullpowerMode2 or configured to fullpower, and according to whether transform precoder is enabled or disabled, and the values of higher layer parameters maxRank and codebookSubset;

-	2 bits according to Table 7.3.1.1.2-5A for 2 antenna ports, if txConfig = codebook, ul-FullPowerTransmission-r16= fullpowerMode1, maxRank=1, and according to whether transform precoder is enabled or disabled, and the values of higher layer parameter codebookSubset;

For the higher layer parameter txConfig=codebook, if ul-FullPowerTransmission-r16 is configured to fullpowerMode2, maxRank is configured to be larger than 2, and at least one SRS resource with 4 antenna ports is configured in an SRS resource set with usage set to 'codebook' and an SRS resource with 2 antenna ports is indicated via SRI in the same SRS resource set, then Table 7.3.1.1.2-4 is used.

---------------------------------------------------Unchanged text omitted-------------------------------------------

Table 7.3.1.1.2-2: Precoding information and number of layers, for 4 antenna ports, if transform precoder is disabled, maxRank = 2 or 3 or 4, and ul-FullPowerTransmission-r16 is not configured or configured to fullpowerMode2 or configured to fullpower

---------------------------------------------------Unchanged text omitted-------------------------------------------

Table 7.3.1.1.2-2A: Precoding information and number of layers for 4 antenna ports, if transform precoder is disabled, maxRank = 2, and ul-FullPowerTransmission-r16 = fullpowerMode1

---------------------------------------------------Unchanged text omitted-------------------------------------------

Table 7.3.1.1.2-2B: Precoding information and number of layers for 4 antenna ports, if transform precoder is disabled, maxRank = 3 or 4, and ul-FullPowerTransmission-r16 = fullpowerMode1

---------------------------------------------------Unchanged text omitted-------------------------------------------

Table 7.3.1.1.2-3: Precoding information and number of layers for 4 antenna ports, if transform precoder is enabled and ul-FullPowerTransmission-r16 is either not configured or configured to fullpowerMode2, or if transform precoder is disabled, maxRank = 1, and ul-FullPowerTransmission-r16 is not configured or configured to fullpowerMode2 or configured to fullpower

---------------------------------------------------Unchanged text omitted-------------------------------------------

Table 7.3.1.1.2-3A: Precoding information and number of layers for 4 antenna ports, if transform precoder is enabled and ul-FullPowerTransmission-r16 = fullpowerMode1, or if transform precoder is disabled, maxRank = 1, and ul-FullPowerTransmission-r16 = fullpowerMode1

---------------------------------------------------Unchanged text omitted-------------------------------------------

Table 7.3.1.1.2-4: Precoding information and number of layers, for 2 antenna ports, if transform precoder is disabled, maxRank = 2, and ul-FullPowerTransmission-r16 is not configured or configured to fullpowerMode2 or configured to fullpower

---------------------------------------------------Unchanged text omitted-------------------------------------------

Table 7.3.1.1.2-4A: Precoding information and number of layers, for 2 antenna ports, if transform precoder is disabled, maxRank = 2, and ul-FullPowerTransmission-r16 = fullpowerMode1

---------------------------------------------------Unchanged text omitted-------------------------------------------

Table 7.3.1.1.2-5: Precoding information and number of layers, for 2 antenna ports, if transform precoder is enabled and ul-FullPowerTransmission-r16 is not configured or configured to fullpowerMode2 or configured to fullpower, or if transform precoder is disabled, maxRank = 1, and and ul-FullPowerTransmission-r16 is not configured or configured to fullpowerMode2 or configured to fullpower

---------------------------------------------------Unchanged text omitted-------------------------------------------

Table 7.3.1.1.2-5A: Precoding information and number of layers, for 2 antenna ports, if transform precoder is enabled and ul-FullPowerTransmission-r16 = fullpowerMode1, or if transform precoder is disabled, maxRank = 1, and ul-FullPowerTransmission-r16 = fullpowerMode1

---------------------------------------------------Unchanged text omitted-------------------------------------------

Table 7.3.1.1.2-32: SRI indication for codebook based PUSCH transmission, if ul-FullPowerTransmission-r16 is not configured, or ul-FullPowerTransmission-r16 = fullpowerMode1, or ul-FullPowerTransmission-r16 = fullpowerMode2, or ul-FullPowerTransmission-r16 = fullpower and NSRS=2

---------------------------------------------------Unchanged text omitted-------------------------------------------

Table 7.3.1.1.2-32A: SRI indication for codebook based PUSCH transmission, if ul-FullPowerTransmission-r16 = fullpowerMode2 and NSRS=3

---------------------------------------------------Unchanged text omitted-------------------------------------------

Table 7.3.1.1.2-32B: SRI indication for codebook based PUSCH transmission, if ul-FullPowerTransmission-r16 = fullpowerMode2 and NSRS=4

---------------------------------------------------Unchanged text omitted-------------------------------------------

7.3.1.1.3	Format 0_2

---------------------------------------------------Unchanged text omitted-------------------------------------------

Precoding information and number of layers – number of bits determined by the following:

-	0 bits if the higher layer parameter txConfig = nonCodeBook;

-	0 bits for 1 antenna port and if the higher layer parameter txConfig = codebook;

-	4, 5, or 6 bits according to Table 7.3.1.1.2-2 for 4 antenna ports, if txConfig = codebook, ul-FullPowerTransmission-r16 is not configured or configured to fullpowerMode2 or configured to fullpower, and according to whether transform precoder is enabled or disabled, and the values of higher layer parameters maxRank-ForDCIFormat0_2, and codebookSubset-ForDCIFormat0_2;

-	4 or 5 bits according to Table 7.3.1.1.2-2A for 4 antenna ports, if txConfig = codebook, ul-FullPowerTransmission-r16=fullpowerMode1, the values of higher layer parameters maxRankForDCI-Format0-2=2, transform precoder is disabled, and according to the value of higher layer parameter codebookSubsetForDCI-Format0-2;

-	4 or 6 bits according to Table 7.3.1.1.2-2B for 4 antenna ports, if txConfig = codebook, ul-FullPowerTransmission-r16=fullpowerMode1, the values of higher layer parameters maxRankForDCI-Format0-2=3 or 4, transform precoder is disabled, and according to the value of higher layer parameter codebookSubsetForDCI-Format0-2;

-	2, 4, or 5 bits according to Table 7.3.1.1.2-3 for 4 antenna ports, if txConfig = codebook, ul-FullPowerTransmission-r16 is not configured or configured to fullpowerMode2 or configured to fullpower, and according to whether transform precoder is enabled or disabled, and the values of higher layer parameters maxRank-ForDCIFormat0_2, and codebookSubset-ForDCIFormat0_2;

-	3 or 4 bits according to Table 7.3.1.1.2-3A for 4 antenna ports, if txConfig = codebook, ul-FullPowerTransmission-r16=fullpowerMode1, maxRankForDCI-Format0-2=1, and according to whether transform precoder is enabled or disabled, and the value of higher layer parameter codebookSubsetForDCI-Format0-2;

-	2 or 4 bits according to Table7.3.1.1.2-4 for 2 antenna ports, if txConfig = codebook, ul-FullPowerTransmission-r16 is not configured or configured to fullpowerMode2 or configured to fullpower, and according to whether transform precoder is enabled or disabled, and the values of higher layer parameters maxRank-ForDCIFormat0_2 and codebookSubset-ForDCIFormat0_2;

-	2 bits according to Table 7.3.1.1.2-4A for 2 antenna ports, if txConfig = codebook, ul-FullPowerTransmission-r16=fullpowerMode1, transform precoder is disabled, the maxRankForDCI-Format0-2=2, and codebookSubsetForDCI-Format0-2=nonCoherent;

-	1 or 3 bits according to Table7.3.1.1.2-5 for 2 antenna ports, if txConfig = codebook, ul-FullPowerTransmission-r16 is not configured or configured to fullpowerMode2 or configured to fullpower, and according to whether transform precoder is enabled or disabled, and the values of higher layer parameters maxRank-ForDCIFormat0_2 and codebookSubset-ForDCIFormat0_2;

-	2 bits according to Table 7.3.1.1.2-5A for 2 antenna ports, if txConfig = codebook, ul-FullPowerTransmission-r16=fullpowerMode1, maxRankForDCI-Format0-2=1, and according to whether transform precoder is enabled or disabled, and the value of higher layer parameter codebookSubsetForDCI-Format0-2.

For the higher layer parameter txConfig=codebook, if ul-FullPowerTransmission-r16 is configured to fullpowerMode2, the values of higher layer parameters maxRankForDCI-Format0-2 is configured to be larger than 2, and at least one SRS resource with 4 antenna ports is configured in an SRS resource set with usage set to 'codebook' and an SRS resource with 2 antenna ports is indicated via SRI in the same SRS resource set, then Table 7.3.1.1.2-4 is used.

---------------------------------------End of Text Proposal#4 for TS 38.212------------------------------------------

Conclusion

We have the following proposals regarding to Rel-16 full TX power transmission:

Proposal 1: For the purpose of making specification more clear, we support the following conclusion

- Conclusion: A UE shall not expect to be configured with higher layer parameter ul-FullPowerTransmission-r16 set to ‘fullpowerMode1’ and SRS-ResourceSet with usage set to ‘codebook’ with single port SRS resource simultaneously.

Proposal 2: Based on the above, we have the following text proposal#1 for TS38.214.

Proposal 3: Based on the above, we have the following text proposal#2 for TS38.213, text proposal#3 for TS38.214, and text proposal#4 for TS38.212.

Reference

- 3GPP TS 38.214: "NR; Physical layer procedures for data", g20

- 3GPP TS 38.213: "NR; Physical layer procedures for control", g20

- 3GPP TS 38.331: "NR; Radio Resource Control (RRC); Protocol specification",g10

- 3GPP TS 38.212: "NR; Multiplexing and channel coding",g20