3GPP TSG RAN WG1 #102-e			R1-2006278

e-Meeting, August 17th – 28th, 2020

Agenda item:	7.2.5.1

Source: 	Spreadtrum Communications

Title: 	Remaining issues of PDCCH enhancements for URLLC

Document for:	Discussion and decision

Introduction

This contribution gives some considerations for the remaining issues of PDCCH design on URLLC [1].

Discussion

Rel-16 PDCCH monitoring for M-TRP

In the Rel-16 work item of MIMO enhancement, multi-DCI based multi-TRP transmission can be supported by configuring two CORESETPoolIndex. Meanwhile, single-DCI based Multi-TRP URLLC enhancements including SDM, FDM and TDM schemes for scheduled PDSCH were also defined.

One remaining issues is whether multi-DCI based multi-TRP URLLC should be supported. We think this feature is not essential. Because there are already 5 different schemes to increase the reliability of URLLC traffic based on single-DCI scheduling. For multi-DCI based multi-TRP, when two PDCCHs schedule two PDSCHs/PUSCHs across TRPs, i.e. PDCCHs are associated with different values of CORESETpoolIndex, there is nothing help to improve reliability or latency. Even worse, this may reduce the performance of the PDSCH receptions. So we do not find any benefits of supporting multi-DCI based multi-TRP URLLC, and propose do not support simultaneous configurations of Rel-16 PDCCH monitoring and multi-DCI based multi-TRP.

- Not support simultaneous configurations of Rel-16 PDCCH monitoring and multi-DCI based multi-TRP.

BDs/non-overlapped CCEs limits

For the limits of Rel-16 PDCCH monitoring BDs/non-overlapped CCEs, there is only the definition of the limits for the condition of configured carrier number is more than UE capability. However, it misses the condition of configured carrier number is equal to or less than UE capability. So the following text proposal for 38.213 section 10.1 should be adopted.

- Adopted the text proposal for BDs/non-overlapped CCEs limits.

**********************************************************************************************

10.1	UE procedure for determining physical downlink control channel assignment

<Text omitted>

If a UE is configured only with Ncells,r16DL,μ downlink cells for which the UE is provided monitoringCapabilityConfig-r16 = r16monitoringcapability and with associated PDCCH candidates monitored in the active DL BWPs of the scheduling cell(s) using SCS configuration μ, and with Ncells,r16DL,(X,Y),μ of the Ncells,r16DL,μ downlink cells using combination X,Y for PDCCH monitoring, where μ=01Ncells,r16DL,μ≤Ncellscap-r16, the UE is not required to monitor, on the active DL BWP of the scheduling cell more than MPDCCHtotal,(X,Y),μ=MPDCCHmax,(X,Y),μ PDCCH candidates or more than CPDCCHtotal,(X,Y),μ=CPDCCHmax,(X,Y),μ non-overlapped CCEs per slot for each scheduled cell.

If a UE is configured only with Ncells,r16DL,μ downlink cells for which the UE is provided monitoringCapabilityConfig-r16 = r16monitoringcapability and with associated PDCCH candidates monitored in the active DL BWPs of the scheduling cell(s) using SCS configuration μ, and with Ncells,r16DL,(X,Y),μ of the Ncells,r16DL,μ downlink cells using combination X,Y for PDCCH monitoring, where μ=01Ncells,r16DL,μ>Ncellscap-r16, a DL BWP of an activated cell is the active DL BWP of the activated cell, and a DL BWP of a deactivated cell is the DL BWP with index provided by firstActiveDownlinkBWP-Id for the deactivated cell, the UE is not required to monitor more than MPDCCHtotal,(X,Y),μ=Ncellscap-r16⋅MPDCCHmax,(X,Y),μ⋅Ncells,r16DL,(X,Y),μj=01Ncells,r16DL,j PDCCH candidates or more than CPDCCHtotal,(X,Y),μ=Ncellscap-r16⋅CPDCCHmax,(X,Y),μ⋅Ncells,r16DL,(X,Y),μj=01Ncells,r16DL,j non-overlapped CCEs

-	per set of spans on the active DL BWP(s) of all scheduling cell(s) from the Ncells,r16DL,(X,Y),μ downlink cells, if the union of PDCCH monitoring occasions on all scheduling cells from the Ncells,r16DL,(X,Y),μ downlink cells results to PDCCH monitoring according to the combination X,Y and any pair of spans in the set is within Y symbols, where first X symbols start at a first symbol with a PDCCH monitoring occasion and next X symbols start at a first symbol with a PDCCH monitoring occasion that is not included in the first X symbols

-	per set of spans across the active DL BWP(s) of all scheduling cells from the Ncells,r16DL,(X,Y),μ downlink cells, with at most one span per scheduling cell for each set of spans, otherwise

where Ncells,r16DL,j is a number of configured cells with SCS configuration j. If a UE is configured with downlink cells for which the UE is provided both monitoringCapabilityConfig-r16 = r15monitoringcapability and monitoringCapabilityConfig-r16 = r16monitoringcapability, Ncellscap-r16 is replaced by Ncells,r16cap-r16.

For each scheduled cell, the UE is not required to monitor on the active DL BWP with SCS configuration μ of the scheduling cell, from the Ncells,r16DL,(X,Y),μ downlink cells using combination X,Y, more than minMPDCCHmax,(X,Y),μ,MPDCCHtotal,(X,Y),μ PDCCH candidates or more than minCPDCCHmax,(X,Y),μ,CPDCCHtotal,(X,Y),μ non-overlapped CCEs per span.

<Text omitted>

**********************************************************************************************

Conclusion

In this contribution, we have the following observations and proposals.

- Not support simultaneous configurations of Rel-16 PDCCH monitoring and multi-DCI based multi-TRP.

- Adopted the text proposal for BDs/non-overlapped CCEs limits.

References

3GPP, “Chairman's Notes RAN1 101-e”.