3GPP TSG RAN WG1 #102-e			R1-2006282

e-Meeting, August 17th – 28th, 2020

Agenda item:	8.13.2

Source: 	Spreadtrum Communications

Title: 	Discussion on multi-cell PDSCH scheduling via a single DCI

Document for:	Discussion and decision

Introduction

During RAN 86 meeting, a new WID on NR Dynamic spectrum sharing (DSS) was approved [1]. The objectives are listed below.

- PDCCH enhancements for cross-carrier scheduling including [RAN1, RAN2]

  - PDCCH of SCell scheduling PDSCH or PUSCH on P(S)Cell

  - Study, and if agreed specify PDCCH of P(S)Cell/SCell scheduling PDSCH on multiple cells using a single DCI

    - The number of cells can be scheduled at once is limited to 2

    - The increase in DCI size should be minimized

- Note: The total PDCCH blind decoding budget should not be changed as a result of this work

- Note: These enhancements are not specific to DSS and are generally applicable to cross-carrier scheduling in carrier aggregation

This contribution provides some initial considerations on multi-cell PDSCH scheduling via a single DCI.

Discussion

Regarding multi-cell PDSCH scheduling via a single DCI, two schemes can be discussed.

- Scheme 1: multi-cell PDSCHs scheduling via a single DCI

- Scheme 2: one multi-cell PDSCH scheduling via a single DCI

For Scheme 1, a single DCI can schedule two PDSCHs for two cells. An example is shown in Figure 1 (a).

For Scheme 2, a single DCI can schedule one PDSCHs mapping to two cells. An example is shown in Figure 1 (b).

Obviously, Scheme 1 and Scheme 2 can reduce UE’s PDCCH monitoring and save power consumption. Furthermore, they can decrease the PDCCH overload from the gNB side. So multi-cell PDSCH scheduling via a single DCI is a deserved to define and specify for CA deployments.

- (b)

Figure 1

Scheme 1 is easier than Scheme 2. It only needs to study the DCI format design, including two separate fields for 2 cells or a common field shared by 2 cells. Some HARQ/Scheduling issues may be required to discussion, for example the relationship of the two PDSCHs on the two cells, the timing of HARQ-ACK feedback etc. Meanwhile, Scheme 2 is more complex. From our understanding, one channel coding and rate matching are done for the TB, and then it is mapped to the resources across two cells, in the order of frequency first and time second. May be some works are related with RAN2. So some clarifications are needed for which scheme is in the scope of this item, one scheme or two schemes.

- Support multi-cell PDSCH scheduling via a single DCI.

- Clarify which the scheme is in the scope.

Conclusion

In this contribution, we made the following observations and proposals.

- Support multi-cell PDSCH scheduling via a single DCI.

- Clarify which the scheme is in the scope.

References

RP-193260, 3GPP RAN, New WID on NR Dynamic spectrum sharing (DSS)