3GPP TSG RAN WG1 Meeting #102-e 	R1-2006398

E-meeting, August 17th – 28th, 2020

Agenda Item:	8.5.4

Source:	Huawei, HiSilicon

Title:	Analysis of positioning service latency

Document for:	Discussion and Decision

Introduction

At the RAN#86 meeting, the study item on NR Positioning Enhancements [1] was approved, which includes the following objective to evaluate the positioning latency:

- Study enhancements and solutions necessary to support the high accuracy (horizontal and vertical), low latency, network efficiency (scalability, RS overhead, etc.), and device efficiency (power consumption, complexity, etc.) requirements for commercial uses cases (incl. general commercial use cases and specifically (I)IoT use cases as exemplified in section 3 above (Justification)):

  - Evaluate the achievable positioning accuracy and latency with the Rel-16 positioning solutions in (I)IoT scenarios and identify any performance gaps. [RAN1]

In RAN1#101-e, agreements, including the following two shown below, have been achieved to evaluate positioning latency:

Agreement:

Physical layer latency can be evaluated through analysis and, optionally, numerical evaluation.

Agreement:

Higher layer positioning latency can be evaluated in this SI.

- FFS: how to evaluate higher-layer positioning latency

- FFS: which higher-layers should be included in the evaluation

In this contribution, we present our results of the achievable positioning latency and describe the corresponding methods we applied for the evaluation.

Discussion

Rel-16 supports 5GS-based LCS service with MT-LR, MO-LR, and NI-LR, among which MT-LR and MO-LR are generalized assumed as commercial use cases.

In the following sections, the service latency for MT-LR and MO-LR using NR positioning is analyzed for  the ticked positioning modes shown in Table 1 below.

Table 1 Positioning modes latency analyzed in this discussion paper

|  | MT-LR | MO-LR |
| --- | --- | --- |
| UE-assisted, LMF based (DL-TDOA, DL-AoD) | √ | √ |
| Network assisted, LMF based (UL-TDOA, UL-AoA) | √ |  |
| UE-based (DL-TDOA, DL-AoD) |  | √ |

As illustrated in [2], positioning service latency is defined by the time elapsed between the event that triggers the determination of the position-related data and the availability of the position-related data at the system interface. The overall service latency depends on the delay on the radio interface, transmission within the 5G system, transmission to a server which may be outside the 5G system, and data processing.

For the sake of brevity, procedures external to the PLMN (between LCS client and GMLC), procedures related to roaming case (between V-PLMN and H-PLMN), and privacy verification procedures (e.g. between GMLC and UDM) are not included in the latency evaluation.

The simplified procedures are based on the SA2 stage-2 specification [1], in the respective clauses 6.1.2 and 6.2.

MT-LR with UE assisted, LMF based positioning

The procedure uses DL-TDOA and/or DL-AoD positioning as the exemplary positioning method, as the two methods share similar mechanisms.

The overall delay and signaling flow are summarized in Table 2 and Figure 1, respectively.

Table 2 Overall delay analysis

| Step No. | Procedure | Delay: Tntw is the delay for signaling between network entities TUL-RRC is the delay for UL RRC signaling between UE and RAN TDL-RRC is the delay for DL RRC signaling between RAN and UE TPhy is the delay for physical layer delay |
| --- | --- | --- |
| 1 | AMF Location request (GMLC AMF) | Tntw |
| 2 | LMF Location request (AMF LMF) | Tntw |
| 3 | LPP Provide assistance data | 2Tntw+TDL-RRC |
| 4 | LPP Request location information | 2Tntw+TDL-RRC |
| 5 | RRC Gap request | TUL-RRC |
| 6 | RRC Gap configuration | TDL-RRC |
| 7 | Physical layer measurement | TPhy |
| 8 | LPP Provide location information | 2Tntw+TUL-RRC |
| 9 | LMF Location calculation | TlocCal |
| 10 | LMF Location response (LMF AMF) | Tntw |
| 11 | AMF Location response (AMF GMLC) | Tntw |
| Total |  | 10Tntw+3TDL-RRC+2TUL-RRC+TlocCal+TPhy |

Figure 1 MT-LR with UE-assisted, LMF based positioning (DL-TDOA/DL-AoD)

MT-LR with network assisted, LMF based positioning

The procedure uses UL-TDOA and/or UL-AoA positioning as the exemplary positioning method, as the two methods share similar mechanisms.

The overall delay and signaling flow are summarized in Table 3 and Figure 2, respectively.

Table 3 Overall delay analysis

| Step No. | Procedure | Delay: Tntw is the delay for signaling between network entities TPhy is the delay for physical layer delay |
| --- | --- | --- |
| 1 | AMF Location request (GMLC AMF) | Tntw |
| 2 | LMF Location request (AMF LMF) | Tntw |
| 3 | NRPPa POSITIONING INFORMATION REQUEST | 2Tntw |
| 4 | RRC SRS configuration | 0 (Assuming existing SRS is used) |
| 5 | NRPPa POSITIONING INFORMATION RESPONSE | 2Tntw |
| 6 | NRPPa MEASUREMENT REQUEST | 2Tntw |
| 7 | Physical layer measurement | TPhy |
| 8 | NRPPa MEASUREMENT RESPONSE | 2Tntw |
| 9 | LMF Location calculation | TlocCal |
| 10 | LMF Location response (LMF AMF) | Tntw |
| 11 | AMF Location response (AMF GMLC) | Tntw |
| Total |  | 12Tntw+TlocCal+TPhy |

Figure 2 MT-LR with network-assisted, LMF based positioning (UL-TDOA/UL-AoA)

MO-LR with UE assisted, LMF based positioning

The procedure uses DL-TDOA and/or DL-AoD positioning as the exemplary positioning method, as the two methods share similar mechanisms.

The overall delay and signaling flow are summarized in Table 4 and Figure 3, respectively.

Table 4 Overall delay analysis

| Step No. | Procedure | Delay: - Tntw is the delay for signaling between network entities - TUL-RRC is the delay for UL RRC signaling between RAN and UE - TDL-RRC is the delay for DL RRC signaling between RAN and UE - TPhy is the delay for physical layer delay |
| --- | --- | --- |
| 1 | NAS Location request (UE AMF) | Tntw+TUL-RRC |
| 2 | LMF Location request (AMF LMF) | Tntw |
| 3 | LPP Provide assistance data | 2Tntw+TDL-RRC |
| 4 | LPP Request location information | 2Tntw+TDL-RRC |
| 5 | RRC Gap request | TUL-RRC |
| 6 | RRC Gap configuration | TDL-RRC |
| 7 | Physical layer measurement | TPhy |
| 8 | LPP Provide location information | 2Tntw+TUL-RRC |
| 9 | LMF Location calculation | TlocCal |
| 10 | LMF Location response (LMF AMF) | Tntw |
| 11 | NAS Location response (AMF UE) | Tntw+TDL-RRC |
| Total |  | 10Tntw+4TDL-RRC+3TUL-RRC+TlocCal+TPhy |

Figure 3 MO-LR with UE-assisted, LMF based positioning (DL-TDOA/DL-AoD)

MO-LR with UE based positioning

The procedure uses DL-TDOA and/or DL-AoD positioning as the exemplary positioning method, as the two methods share similar mechanisms.

The overall delay and signaling flow are summarized in Table 5 and Figure 4, respectively.

Table 5 Overall delay analysis

| Step No. | Procedure | Delay: Tntw is the delay for signaling between network entities TUL-RRC is the delay for UL RRC signaling between RAN and UE TDL-RRC is the delay for DL RRC signaling between RAN and UE TPhy is the delay for physical layer delay |
| --- | --- | --- |
| 1 | NAS Location request (UE AMF) | Tntw+TUL-RRC |
| 2 | LMF Location request (AMF LMF) | Tntw |
| 3 | LPP Provide assistance data | 2Tntw+TDL-RRC |
| 4 | RRC Gap request | TUL-RRC |
| 5 | RRC Gap configuration | TDL-RRC |
| 6 | Physical layer measurement | TPhy |
| 7 | UE Location calculation | TlocCal |
| Total |  | 4Tntw+2TDL-RRC+2TUL-RRC+TPhy+TlocCal |

Figure 4 MO-LR with UE-based positioning (DL-TDOA/DL-AoD)

Summary

The overall latency analysis is given in Table 6. The typical value assumes the following individual components.

- Tntw=0.5ms

TDL-RRC=5ms

TUL-RRC=2ms

TlocCal=5ms

TPhy=20ms for SRS and DL PRS

Table 6 Summary of positioning service delay

| Case | Overall delay | Typical value |
| --- | --- | --- |
| MT-LR (UE assisted, LMF based) | 10Tntw+3TDL-RRC+2TUL-RRC+TlocCal+TPhy | 49ms |
| MT-LR (Network assisted, LMF based) | 12Tntw+TlocCal+TPhy | 31ms |
| MO-LR (UE assisted, LMF based) | 10Tntw+4TDL-RRC+3TUL-RRC+TlocCal+TPhy | 56ms |
| MO-LR (UE based) | 4Tntw+2TDL-RRC+2TUL-RRC+TPhy+TlocCal | 41ms |

Based on the analysis, we have the following observations.

- Observation 1: Physical measurement delay is the main contributor to the overall positioning service delay.

Observation 2: The typical positioning service end-to-end delay is between 30ms to 50ms.

Conclusion

In this contribution, we have the following proposals.

- Observation 1: Physical measurement delay is the main contributor to the overall positioning service delay.

Observation 2: The typical positioning service end-to-end delay is between 30ms to 50ms.

- Reference

3GPP RP-193237, “New SID: Study on NR positioning support”, Sitges, Spain, December 9th – 12th, 2019.

3GPP TS 22.261, Service requirements for the 5G system; Stage 1.

3GPP TS 23.273, 5G System (5GS) Location Services (LCS); Stage 2.