3GPP TSG RAN WG1 Meeting #102-e	R1-2006419

E-meeting, August 17–28, 2020

Agenda Item:	6.2.2

Source:	Huawei, HiSilicon

Title:	Corrections on transmission in preconfigured UL resources for NB-IoT

Document for:	Discussion and Decision

# Introduction

In NB-IoT Rel-16, the agreements related to PUR are summarized in [1]. This paper discusses some remaining issues of PUR.

# Sequence-group hopping

As shown in TS 36.211 (see below), in legacy NB-IoT, sequence-group hopping for NPUSCH can be disabled for a certain UE through the higher-layer parameter groupHoppingDisabled despite being enabled on a cell basis unless the NPUSCH transmission corresponds to a Random Access Response Grant or a retransmission of the same transport block as part of the contention based random access procedure.

However, since groupHoppingDisabled is not included in PUR-Config-NB, the current specification is incorrect. Generally, there are two alternatives to solve this issue:

Alt 1: Send LS to RAN2 to include groupHoppingDisabled in PUR-Config-NB, and the RAN1 specfication does not need to be updated.

Alt 2: Modify RAN1 specification (see TP below) so that PUR is treated similar to RACH.

Proposal 1: RAN1 needs to decide how to address sequence-group hopping for PUR NPUSCH.

------------------------------------- Start of Text Proposal for TS 36.211------------------------------- ------------------------------------------- Unchanged parts omitted -------------------------------------- ##### 10.1.4.1.3 Group hopping … Sequence-group hopping can be enabled or disabled by means of the cell-specific parameter groupHoppingEnabled provided by higher layers. Sequence-group hopping for NPUSCH can be disabled for a certain UE through the higher-layer parameter groupHoppingDisabled despite being enabled on a cell basis unless the NPUSCH transmission corresponds to a Random Access Response Grant or a retransmission of the same transport block as part of the contention based random access procedure, or NPUSCH transmission corresponds to preconfigured uplink resources. -------------------------------------------- Unchanged parts omitted ------------------------------------- ---------------------------------------------- End of Text Proposal ----------------------------------------

##### (copied from TS 36.331) NPUSCH-ConfigDedicated-NB-r13 ::= SEQUENCE { ack-NACK-NumRepetitions-r13 ACK-NACK-NumRepetitions-NB-r13 OPTIONAL, -- Need ON npusch-AllSymbols-r13 BOOLEAN OPTIONAL, -- Cond SRS groupHoppingDisabled-r13 ENUMERATED {true} OPTIONAL -- Need OR }

# NPDSCH/NPDCCH interference randomization

In the discussion of the WI, it was assumed to re-use legacy NPDCCH/NPDSCH transmission schemes. However, some places in the spec are overlooked with the introduction of PUR C-RNTI, such as the interference randomization. Therefore, the following TP is proposed to complete the spec. Note that some line breaks are added in the proposed TP for better readability.

Proposal 2: Endorse the text proposal in Section 3 and capture its contents in TS 36.211.

------------------------------------- Start of Text Proposal for TS 36.211------------------------------- ------------------------------------------- Unchanged parts omitted -------------------------------------- ### 10.2.3 Narrowband physical downlink shared channel #### 10.2.3.4 Mapping to resource elements … For frame structure type 1, - for NPDSCH associated with C-RNTI when interferenceRandomisationConfig is used according to [11], - or NPDSCH associated with RA-RNTI, TC-RNTI or P-RNTI and transmitted in an NB-IoT carrier configured by SystemInformationBlockType22-NB, - or NPDSCH associated with C-RNTI in an NB-IoT carrier configured by SystemInformationBlockType22-NB when RadioResourceConfigDedicted-NB is not configured by higher layer, - or NPDSCH associated with PUR C-RNTI or G-RNTI or SC-RNTI, or for frame structure type 2, - for NPDSCH not carrying the BCCH, define as the block of complex-valued symbols mapped to subframe number ns/2 and radio frame number . … ### 10.2.5 Narrowband physical downlink control channel #### 10.2.5.5 Mapping to resource elements … For frame structure type 1, - for NPDCCH associated with RA-RNTI, TC-RNTI or P-RNTI and transmitted in an NB-IoT carrier configured by SystemInformationBlockType22-NB, - or NPDCCH associated with C-RNTI in an NB-IoT carrier configured by SystemInformationBlockType22-NB when RadioResourceConfigDedicted-NB is not configured by higher layer, - or NPDCCH associated with PUR C-RNTI or G-RNTI or SC-RNTI, - or for NPDCCH associated with C-RNTI when interferenceRandomisationConfig is used according to [11], or for frame structure type 2, each complex-valued symbol , shall be multiplied with ,where -------------------------------------------- Unchanged parts omitted ------------------------------------- ---------------------------------------------- End of Text Proposal ----------------------------------------

# Conclusion

The contribution discusses some remaining issues of PUR. The proposals based on the analysis in the contribution are as following:

Proposal 1: RAN1 needs to decide how to address sequence-group hopping for PUR NPUSCH.

Proposal 2: Endorse the text proposal in Section 3 and capture its contents in TS 36.211.

# References

- R1-1913595, “RAN1 agreements for Rel-16 Additional Enhancements for NB-IoT”, Futurewei, RAN1#99, Reno, USA, 18th – 22nd November 2019