3GPP TSG-RAN WG1 Meeting #102-e	R1-2006440

e-Meeting, August 17th – 28th, 2020

Agenda Item:	7.2.4.6

Source:	Ericsson

Title:	Correction for SL QoS management

Document for:	Discussion, Decision

# 1	Discussion

In RAN1#101-e it was agreed that the future granted resources which have been released due to HARQ feedback are not counted in the evaluation of CR. Accordingly, a TP to TS 38.215 was approved to capture this conclusion, which manifested in the following note in the definition of CR in TS 38.215:

NOTE 6: A resource is considered granted if it is a member of a configured sidelink grant as defined in TS 38.321 [7].

The part in TS 38.321 V16.0.0 (2020-03) which was referred to by this note is the following:

5.22.1.1 SL Grant reception and SCI transmission … If the MAC entity has been configured by RRC to transmit using pool(s) of resources in a carrier as indicated in TS 38.331 [5] or TS 36.331 [21] based on sensing or random selection, the MAC entity shall for each Sidelink process: … 1> if a configured sidelink grant is available for retransmission(s) of a MAC PDU which has been positively acknowledged as specified in clause 5.22.1.3.3: 2> clear the PSCCH duration(s) and PSSCH duration(s) corresponding to retransmission(s) of the MAC PDU from the configured sidelink grant.

However, in the TS 38.321 V16.1.0 (2020-07), the term “configured sidelink grant” was replaced by “selected sidelink grant” as follows:

5.22.1.1 SL Grant reception and SCI transmission … If the MAC entity has been configured with Sidelink resource allocation mode 2 to transmit using pool(s) of resources in a carrier as indicated in TS 38.331 [5] or TS 36.331 [21] based on sensing or random selection, the MAC entity shall for each Sidelink process: … 1> if a selected sidelink grant is available for retransmission(s) of a MAC PDU which has been positively acknowledged as specified in clause 5.22.1.3.3: 2> clear the PSCCH duration(s) and PSSCH duration(s) corresponding to retransmission(s) of the MAC PDU from the selected sidelink grant.

As a result, it is necessary to update NOTE 6 in TS 38.215 as in the following TP:

----------------------------------------------------begin text proposal for 38.215----------------------------------------------- ### 5.1.26 Sidelink channel occupancy ratio (SL CR) <<<unchanged text omitted>>> NOTE 6: A resource is considered granted if it is a member of a selected sidelink grant as defined in TS 38.321 [7]. ----------------------------------------------------end text proposal for 38.215-----------------------------------------------