3GPP TSG RAN WG1 #102-e	R1-2006503

e-Meeting, August 17th – 28th, 2020

Agenda Item:	8.1.2.4

Source:	Apple Inc.

Title:	Views on Rel-17 HST enhancement

Document for:	Discussion/Decision

# Introduction

In RAN#86 meeting, Rel-17 FeMIMO WI was approved in RP-193133 [1]. Four main areas haven been identified for the Rel-17 FeMIMO, namely

- Enhancement on multi-beam operation, mainly targeting FR2 while also applicable to FR1

- Enhancement on the support for multi-TRP deployment, targeting both FR1 and FR2

- Enhancement on SRS, targeting both FR1 and FR2

- Enhancement on CSI measurement and reporting

Within the scope of multi-TRP enhancement, one sub-objective is the High-Speed Train (HST) enhancement as highlighted as quoted below from [1]

- Enhancement on the support for multi-TRP deployment, targeting both FR1 and FR2: - Identify and specify features to improve reliability and robustness for channels other than PDSCH (that is, PDCCH, PUSCH, and PUCCH) using multi-TRP and/or multi-panel, with Rel.16 reliability features as the baseline - Identify and specify QCL/TCI-related enhancements to enable inter-cell multi-TRP operations, assuming multi-DCI based multi-PDSCH reception - Evaluate and, if needed, specify beam-management-related enhancements for simultaneous multi-TRP transmission with multi-panel reception - Enhancement to support HST-SFN deployment scenario: - Identify and specify solution(s) on QCL assumption for DMRS, e.g. multiple QCL assumptions for the same DMRS port(s), targeting DL-only transmission - Evaluate and, if the benefit over Rel.16 HST enhancement baseline is demonstrated, specify QCL/QCL-like relation (including applicable type(s) and the associated requirement) between DL and UL signal by reusing the unified TCI framework

In this contribution, we provide our views on the Rel-17 FeMIMO HST enhancement.

# High Doppler shift in HST deployment

The major difficulty or challenge in HST deployment comes from handling of high Doppler shift due to the combination of high device movement speed and potential deployment at high band. When a UE in the high-speed train travels at very high speed between two TRPs, UE would observe a high positive Doppler shift to one of the TRP, and high negative Doppler shift to the other TRP.

We use an example to demonstrate the issues that we need to solve for HST deployment

- Train Speed: 400km/hr

- Carrier frequency: 5GHz

- Location of the UE: equal distance between two TRP

- 30kHz SCS

Based on the above assumptions, the device will observe around +1850Hz Doppler shift from one TRP and -1850Hz Doppler shift from the other TRP. As results, when two TRPs perform SFN transmission, the composite signal at the UE side will experience 3700Hz sinusoid fading as illustrated in Figure 1.

From Figure 1., it is easy to observe that the channel experiences a very fast fading, i.e., the channel completes a complete sinusoid cycle within about half a slot, i.e., ~ 8 symbols.

Figure 1 Amplitude of composite SFN channel

However, after further looking at the HST problem, we have the following two observations which lead to two directions for the HST enhancement

- Even though the composite channel appears to fade very fast, it is primarily due to the high Doppler shift instead of Doppler spread. Therefore, if UE can estimate the Doppler shift, UE can improve its channel estimate by performing parametric enhanced channel estimate

  - This leads to the DMRS QCL enchantment

- Even with perfect channel estimation, the channel capability still suffers due to the channel variation. If gNB can pre-compensate for the Doppler shift and perform the right phase compensation, gNB can ensure the composite channel to always have the strongest amplitude without Doppler shift

  - This leads to the gNB doppler pre-compensation enhancement

We will discussion the two directions at high level in the following two sub-sections

## DMRS QCL enhancement

To assist the UE channel estimation, gNB needs to transmit two TRS, one TRS from each TRP. Then, UE can estimate the individual Doppler shift from each TRP which can be used in the parametric channel estimation to improve the channel estimate quality. The main issue we need to resolve is how to indicate that DMRS is QCL to two TRS.

In Rel-16, for Single-DCI based Multi-TRP solution, we already introduced TCI codepoint enhancement to allow a single TCI point to contain up to 2 TCI states. Furthermore, TCI codepoint is used to enable 5 different Single-DCI Multi-TRP schemes, including (1) SDM (2) FDMSchemeA (3) FDMSchemeB (4) TDMSchemeA (5) Scheme 4. Similarly, we can consider DMRS QCL enhancement based on the Single-DCH Multi-TRP solutions. Therefore, we have the following proposal

Proposal 1: For HST DMRS QCL enhancement, we can reuse the Rel-16 TCI codepoint enhancement and enhance HST DMRS QCL based on Rel-16 Single-DCI Multi-TRP solutions

## gNB Doppler shift pre-compensation

For gNB to perform efficient Doppler shift pre-compensation, gNB needs to know the Doppler shift between the UE and each TRP. To obtain Doppler shift, there are two directions

- gNB acquires the Doppler shift based on the UE SRS sounding

- gNB acquires the Doppler shift based on the UE measurement reporting

As results, we have the following proposal

Proposal 2: For HST enhancement with gNB Doppler shift pre-compensation, we can consider two solutions (1) gNB acquires the Doppler shift based on the UE SRS sounding (2) gNB acquires the Doppler shift based on the UE measurement reporting.

# Conclusion

In RAN#86 meeting, Rel-17 FeMIMO WI was approved in RP-193133 [1]. Four main areas haven been identified for the Rel-17 FeMIMO, namely

- Enhancement on multi-beam operation, mainly targeting FR2 while also applicable to FR1

- Enhancement on the support for multi-TRP deployment, targeting both FR1 and FR2

- Enhancement on SRS, targeting both FR1 and FR2

- Enhancement on CSI measurement and reporting

Within the scope of multi-TRP enhancement, one sub-objective is the High-Speed Train (HST) enhancement. In this contribution, we provide our views on the Rel-17 FeMIMO HST enhancement with the following proposals

Proposal 1: For HST DMRS QCL enhancement, we can reuse the Rel-16 TCI codepoint enhancement and enhance HST DMRS QCL based on Rel-16 Single-DCI Multi-TRP solutions

Proposal 2: For HST enhancement with gNB Doppler shift pre-compensation, we can consider two solutions (1) gNB acquires the Doppler shift based on the UE SRS sounding (2) gNB acquires the Doppler shift based on the UE measurement reporting.

# Reference

- RP-193133, "New WID: Further enhancements on MIMO for NR", 3GPP TSG RAN Meeting #86, Sitges, Spain, December 9-12, 2019