3GPP TSG RAN WG1 #102-e	R1-2006509

e-Meeting, August 17th – 28th, 2020

Agenda Item:	8.13.1

Source:	Apple Inc.

Title:	Views on Rel-17 DSS SCell scheduling PCell

Document for:	Discussion/Decision

# Introduction

Dynamic spectrum sharing allows NR to be co-channel deployed with LTE, i.e., NR and LTE share the same spectrum, to address the spectrum availability issue from the operator. However, from performance or user experience perspective, it is very hard for DSS deployment to deliver better performance with NR compared to LTE. In Rel-15/16, multiple solutions are adopted to allow DSS in many areas such as (1) symbol level rate matching, including DMRS, mini-slot scheduling, flexible CORESET starting symbol within a slot, etc. (2) RB level rate matching via RRC configuration (3) RE level PDSCH rate matching to LTE.

In the Rel-17 “New WID on NR Dynamic spectrum sharing (DSS)”, the following objective was approved to address the PCell scheduling restriction with DSS

This work item is limited to FR1, and includes the following objectives for NR Dynamic Spectrum Sharing (DSS): - PDCCH enhancements for cross-carrier scheduling including [RAN1, RAN2] - PDCCH of SCell scheduling PDSCH or PUSCH on P(S)Cell - Study, and if agreed specify PDCCH of P(S)Cell/SCell scheduling PDSCH on multiple cells using a single DCI - The number of cells can be scheduled at once is limited to 2 - The increase in DCI size should be minimized - Note: The total PDCCH blind decoding budget should not be changed as a result of this work - Note: These enhancements are not specific to DSS and are generally applicable to cross-carrier scheduling in carrier aggregation

In this contribution, we provide our views on the objective of PDCCH of SCell scheduling PDSCH or PUSCH on P(S)Cell.

# Existing PCell scheduling restrictions

## Cross carrier scheduling (CCS) restrictions

In Rel-15 and Rel-16, cross carrier scheduling (CCS) is allowed for both PDSCH and PUSCH, with the following restrictions

- No cross CG cross carrier scheduling

- PCell can only be scheduled by itself

- For each scheduled cell, only one scheduling cell can be RRC configured to schedule it

Below is the corresponding specification in 38.331 and 38.213

## PCell PDCCH configurations

In the current specification, there are three ways to configure the PDCCH

- pdcch-ConfigSIB1 in MIB

- PDCCH-ConfigCommon

- PDCCH-Config

In terms of the cross-carrier scheduling

- Fall back DCI, i.e. Format 0_0 and Format 1_0, do not support cross carrier scheduling

- DCI Format 0_1, Format 0_2, Format 1_1 and Format 1_2, support cross carrier scheduling

In terms of the search space configuration, there are the following possible configurations

- Type0-PDCCH CSS: searchSpaceSIB1, searchSpaceZero

  - Only fall back DCI is allowed

- Type0A-PDCCH CSS: searchSpaceOtherSystemInformation

  - Only fall back DCI is allowed

- Type1-PDCCH CSS: pagingSearchSpace

  - Only fall back DCI is allowed

- Type2-PDCCH CSS: ra-SearchSpace

  - Only fall back DCI is allowed

- Type3-PDCCH CSS

  - Only fall back DCI is allowed

- USS

It is also important to note that if PCell is required to support initial acquisition, it becomes almost impossible to move all the PDSCH/PUSCH scheduling to SCell due to the lack of RRC connection

## NR CORESET configuration

In DSS, NR CORESET is expected to be configured in the symbols without LTE CRS.

- When LTE has 1 or 2 port CRS, CRS occupies symbol {0, 4, 7, 11}

- When LTE has 3 or 4 port CRS, CRS occupies symbol {0, 1, 4, 7, 8, 11}

In Rel-15, FG3-2 was defined to enhance DSS support, i.e., “PDCCH monitoring on any span of up to 3 consecutive OFDM symbols of a slot”. For a UE that supports FG3-2, a DSS NW can configure up to 3 symbol CORESET when LTE has 1or 2 port CRS, or, up to 2 symbol CORESET when LTE has 3 or 4 port CRS

Observation 1: For a UE that supports FG3-2, a DSS NW can configure up to 3 symbol CORESET when LTE has 1 or 2 port CRS, or, up to 2 symbol CORESET when LTE has 3 or 4 port CRS.

# PDCCH of SCell scheduling PDSCH or PUSCH on P(S)Cell

Based on the above discussion, it is not straightforward and easy to move the PCell scheduling to SCell especially when PCell is required for initial acquisition. As results, we have the following proposals

In Rel-15, FG3-2 was defined to enhance DSS support, i.e., “PDCCH monitoring on any span of up to 3 consecutive OFDM symbols of a slot”. For a UE that supports FG3-2, a DSS NW can configure up to 3 symbol CORESET when LTE has 2 or 2 port CRS, or, up to 2 symbol CORESET when LTE has 3 or 4 port CRS

Proposal 1: We do not observe strong motivation to allow SCell to schedule PCell for DSS enhancement.

Proposal 2: For DCI enhancement, if we have to consider solution to allow SCell to schedule PCell, we should strive to keep the same design principle that for each scheduled cell, only one scheduling cell can be configured

# Conclusion

Dynamic spectrum sharing allows NR to be co-channel deployed with LTE, i.e., NR and LTE share the same spectrum, to address the spectrum availability issue from the operator. However, from performance or user experience perspective, it is very hard for DSS deployment to deliver better performance with NR compared to LTE. In the Rel-17 “New WID on NR Dynamic spectrum sharing (DSS)”, one objective was approved to consider PDCCH of SCell scheduling PDSCH or PUSCH on P(S)Cell.

We have the following observation and proposals

Observation 1: For a UE that supports FG3-2, a DSS NW can configure up to 3 symbol CORESET when LTE has 1 or 2 port CRS, or, up to 2 symbol CORESET when LTE has 3 or 4 port CRS.

Proposal 1: We do not observe strong motivation to allow SCell to schedule PCell for DSS enhancement.

Proposal 2: For DCI enhancement, if we have to consider solution to allow SCell to schedule PCell, we should strive to keep the same design principle that for each scheduled cell, only one scheduling cell can be configured

# Reference

- RP-193260, “New WID on NR Dynamic spectrum sharing (DSS)”, 3GPP TSG RAN Meeting #86, Sitges, Spain, December 9-12, 2019