3GPP TSG RAN WG1 Meeting #102-e	R1-2006560

e-Meeting, August 17th – 28th, 2020

Source:	Sharp

Title:	Remaining issues on synchronization mechanism for NR sidelink

Agenda Item:	7.2.4.3

Document for:	Discussion and Decision

Introduction

In this document, we share our views on one remaining issue on synchronization mechanism for NR sidelink.

Discussion

Indication of the non-TDD case in sl-TDD-Config

The following changes regarding indication of TDD in sl-TDD-Config in SidelinkMasterInformationBlock were endorsed in RAN1#101-e.

It is however unclear what should be indicated in sl-TDD-Config in case tdd-UL-DL-ConfigurationCommon is not provided. It is proposed to use one reserved code point for (a1, a2, a3,a4) to indicate this case.

## 16.1 Synchronization procedures […] For transmission of an S-SS/PSBCH block, a UE includes a bit sequence a0, a1, a2, a3, …, a11 in the PSBCH payload to indicate sl-TDD-Config and provide a slot format over a number of slots, where - a0=0 if pattern1 is provided by tdd-UL-DL-ConfigurationCommon; a0=1 if both pattern1 and pattern2 are provided by tdd-UL-DL-ConfigurationCommon as described in Clause 11.1 - a1, a2, a3,a4 are determined based on - P in pattern1 as described in Table 16.1-1 for a0=0 - P in pattern1 and P2 in pattern2 as described in Table 16.1-2 for a0=1 where P and P2 are as described in Clause 11.1 - a5, a6, a7,a8, a9, a10, a11 are the 7th to 1st LSBs of uslotsSL, respectively - for a0=0, uslotsSL=uslots*2μ-μref+usym*2μ-μrefL+I1 - for a0=1, uslotsSL=uslots,2*2μ-μref+usym,2*2μ-μrefL+I2w*P*2μ+1w+uslots*2μ-μref+usym*2μ-μrefL+I1w where - L is the number of symbols in a slot: L=12 if cyclicPrefix-SL = “ECP”; else, L=14 - I1 is 1 if usym*2μ-μref mod L≥ L-Y, else I1 is 0 - I2 is 1 if usym,2*2μ-μref mod L≥L-Y, else I2 is 0 - Y is the sidelink starting symbol index provided by sl-StartSymbol - w is the granularity of slots indication as described in Table 16.1-2 - μref, uslots, usym, uslots,2, usym,2 are described in Clause 11.1 - μ=0, 1, 2, 3 corresponds to SL SCS as defined in [4, TS 38.211] Table 16.1-1: Slot configuration period when one pattern is indicated a1, a2, a3,a4; Slot configuration period of pattern1 P (msec). 0, 0, 0, 0; 0.5. 0, 0, 0, 1; 0.625. 0, 0, 1, 0; 1. 0, 0, 1, 1; 1.25. 0, 1, 0, 0; 2. 0, 1, 0, 1; 2.5. 0, 1, 1, 0; 4. 0, 1, 1, 1; 5. 1, 0, 0, 0; 10. Reserved; Reserved. Table 16.1-2: Slot configuration period and granularity when two patterns are indicated a1, a2, a3,a4; Slot configuration period of pattern1 P (msec); Slot configuration period of pattern2 P2 (msec); Granularity w in slots with different SCS. 15kHz; 30 kHz; 60 kHz; 120 kHz. 0, 0, 0, 0; 0.5; 0.5; 1. 0, 0, 0, 1; 0.625; 0.625. 0, 0, 1, 0; 1; 1. 0, 0, 1, 1; 0.5; 2. 0, 1, 0, 0; 1.25; 1.25. 0, 1, 0, 1; 2; 0.5. 0, 1, 1, 0; 1; 3; 1; 2. 0, 1, 1, 1; 2; 2. 1, 0, 0, 0; 3; 1. 1, 0, 0, 1; 1; 4. 1, 0, 1, 0; 2; 3. 1, 0, 1, 1; 2.5; 2.5. 1, 1, 0, 0; 3; 2. 1, 1, 0, 1; 4; 1. 1, 1, 1, 0; 5; 5; 1; 2; 4. 1, 1, 1, 1; 10; 10; 1; 2; 4; 8.

Proposal 1: In case tdd-UL-DL-ConfigurationCommon is not provided, the sl-TDD-Config bits are set as follows,

a0=0, a1, a2, a3,a4=(1, 1, 1, 1), a5, a6, a7,a8, a9, a10, a11=(1, 1, 1, 1, 1, 1, 1)

Conclusion

In this contribution, we discuss one remaining issue on synchronization for NR sidelink, and make the following proposal:

Proposal 1: In case tdd-UL-DL-ConfigurationCommon is not provided, the sl-TDD-Config bits are set as follows,

a0=0, a1, a2, a3,a4=(1, 1, 1, 1), a5, a6, a7,a8, a9, a10, a11=(1, 1, 1, 1, 1, 1, 1)

References

RP-190766, “New WID on 5G V2X with NR sidelink”, LG Electronics, Huawei, 3GPP RAN#83.