3GPP TSG-RAN WG1 Meeting #102-e	R1-2006615

e-Meeting, August 17th – 28th, 2020

Agenda Item:	8.8.2.3

Source:	Ericsson

Title:	Coverage enhancement for channels other than PUSCH and PUCCH

# Introduction

In this contribution, we consider potential areas of enhancement to remediate coverage bottlenecks found in [1] and [2]. DL coverage solutions specifically targeting Msg4 PDSCH are discussed. Mechanisms to improve downlink coverage without increasing downlink resource usage or latency are considered.

# Discussion

As observed in [1] and [2], for the uplink, PUSCH data, and perhaps uplink control for FR1 may be coverage bottlenecks. It was also found that downlink channels bearing control information were not coverage bottlenecks, although for FR2 10 Mbps PDSCH data may be more limiting than some uplink data. While coverage may not be limited for lower data rate PDSCH, it may be inefficient particularly when accurate CSI is not available, requiring large amounts of resource for a relatively small TBS.

Observation:

- Maintaining PDSCH coverage without accurate CSI may require relatively high amounts of downlink resource

Proposal:

- Further study downlink enhancements exploiting the early availability of CSI

## Potential areas of enhancement for spectrally efficient downlink coverage

Based on simulation results in [1] and [2], DL channel coverage is generally better than UL channel, partially because of more transmit antennas and more transmission power of gNB, more accurate channel estimation based on reciprocity and CSI report. But among DL channels, Msg4 contention resolution on PDCCH and PDSCH is a little problematic. It may happen that after UE sends out Msg3, it can’t successfully decode Msg4. Then UE sends out HARQ for PUCCH and triggers a Msg4 retransmission. The Msg4 failure is partially because it has much less coverage enhancement features available than PDSCH for UE in RRC_Connected mode. In NR up to Rel-16, Msg4 PDSCH doesn’t support beam management or PDSCH slot aggregation since RRC connection is not established yet, nor does it support TBS scaling which is appliable for Msg2 PDSCH scheduled by RA-RNTI.

Observation:

- Msg4 PDSCH has worse coverage than other DL PDSCH.

One simple approach to providing coverage is to use a sufficiently low coding rate on the downlink, which increases both the amount of energy delivered to the UE as well as maximizing the coding gain. However, this is a relatively expensive way to achieve downlink coverage.

NR can provide large amounts of array gain via digital or analog beamforming without increasing the amount of downlink resource, and so is a spectrally efficient way to achieve coverage. Array gain brought by proper beamforming leads to higher received signal power and higher SNR at UE, which can improve coverage. NR R15/16 supports CSI reporting after an RRC connection is established, which can be used for beamforming. During the RACH procedure, a Tx and Rx beam pair between UE and network is maintained as the one used for Msg1 transmission and reception. If an early CSI report is available during random access, array gain can improve coverage of downlink channels during random access without the increased overhead needed by low code rate PDSCH transmission.

To achieve the array gain, the network must have sufficient channel state information. If the PDSCH packet size is large enough, there will be adequate time for the UE to signal the CSI to the network, and the initially inefficient transmissions will not overly degrade the net spectral efficiency. If PDSCH packet sizes are smaller, then there may not be enough time to obtain the CSI without excessive latency or RRC signalling overhead. Therefore, an early CSI report especially benefits small PDSCH packets. In this case, beamforming configuration based on early CSI report is effective starting from Msg4 PDSCH till following DL transmission and the UE doesn't need to report CSI or be configured otherwise.

The UE configurations used early in radio link setups will generally need to be quite simple, since RRC signalling will take some time to provide to the UE and the network may have limited knowledge of the UE’s capabilities. Therefore, the use cases envisioned here are for basic CSI reporting, such as CSI-RSRP based on a small number of CSI-RS ports, SS-RSRP and/or SSBRI. Therefore, we expect that early CSI reporting should require minimal calculation effort for the UE.

Observation:

- Early CSI availability can provide downlink coverage for small PDSCH packet sizes before normal CSI is available without excessive latency, RRC signalling overhead or excessive downlink resource.

- Early CSI calculation use cases imply minimal calculation effort for the UE.

Proposal:

- Study techniques to provide CSI during random access

# Summary

In this contribution, we consider potential classes of coverage enhancement techniques. DL coverage solutions specifically targeting Msg4 PDSCH are discussed. Mechanisms to improve downlink coverage without increasing downlink resource usage or latency were also considered.

We have following observations based on the discussions.

Observations:

- Maintaining PDSCH coverage without accurate CSI may require relatively high amounts of downlink resource

- Msg4 PDSCH has worse coverage than other DL PDSCH.

- Early CSI availability can provide downlink coverage for small PDSCH packet sizes before normal CSI is available without excessive latency, RRC signalling overhead or excessive downlink resource.

- Early CSI calculation use cases imply minimal calculation effort for the UE.

Based on the observations and discussions, we have following proposals.

Proposals:

- Further study downlink enhancements exploiting the early availability of CSI

- Study techniques to provide CSI during random access

# References

R1-2006611, “Simulation Parameters and Initial Results for FR1”, Ericsson, RAN1 #102-e, August 2020.

R1-2006612, “Simulation Parameters and Initial Results for FR2”, Ericsson, RAN1 #102-e, August 2020.