3GPP TSG RAN WG1 #102-e		      R1-2006657

e-Meeting, August 17th – 28th, 2020

Source:	vivo

Title:	Corrections for configured grant PUSCH

Agenda Item:	7.2.5.6

Document for:	Discussion and Decision

- Introduction

In this contribution, we propose some corrections for RRC parameter names used in RAN1 specification to align them with RAN2 specification.

- Corrections

In current TS 38.331-g20 [1], sps-ConfigDeactivationStateList is used to indicate a list of the deactivation states in which each state can be mapped to a single or multiple SPS configurations to be deactivated. Similarly, for Configured Grant type 2 configurations, the RRC parameter name is ConfiguredGrantConfigType2DeactivationStateList. Therefore, following is proposed to align the RRC parameters between RAN1 and RAN2 specifications.

Proposal 1: Adopt following text proposal in TS 28.213.

TS 38.213 ## 10.2 PDCCH validation for DL SPS and UL grant Type 2 A UE validates, for scheduling activation or scheduling release, a DL SPS assignment PDCCH or a configured UL grant Type 2 PDCCH if - the CRC of a corresponding DCI format is scrambled with a CS-RNTI provided by cs-RNTI, and - the new data indicator field in the DCI format for the enabled transport block is set to '0', and - the DFI flag field, if present, in the DCI format is set to '0', and - if validation is for scheduling activation and if the PDSCH-to-HARQ_feedback timing indicator field in the DCI format is present, the PDSCH-to-HARQ_feedback timing indicator field does not provide an inapplicable value from dl-DataToUL-ACK. If a UE is provided a single configuration for UL grant Type 2 PUSCH or for SPS PDSCH, validation of the DCI format is achieved if all fields for the DCI format are set according to Table 10.2-1 or Table 10.2-2. If a UE is provided more than one configurations for UL grant Type 2 PUSCH or for SPS PDSCH, a value of the HARQ process number field in a DCI format indicates an activation for a corresponding UL grant Type 2 PUSCH or for a SPS PDSCH configuration with a same value as provided by Configuredgrantconfig-index or by SPSconfig-index, respectively. Validation of the DCI format is achieved if the RV field for the DCI format is set as in Table 10.2-3. If a UE is provided more than one configuration for UL grant Type 2 PUSCH or for SPS PDSCH - if the UE is provided ConfiguredGrantConfigType2DeactivationStateListType2Configuredgrantconfig-ReleaseStateList or sps-ConfigDeactivationStateList SPS-ReleaseStateList, a value of the HARQ process number field in a DCI format indicates a corresponding entry for scheduling release of one or more UL grant Type 2 PUSCH or SPS PDSCH configurations - if the UE is not provided ConfiguredGrantConfigType2DeactivationStateList Type2Configuredgrantconfig-ReleaseStateList or sps-ConfigDeactivationStateList SPS-ReleaseStateList, a value of the HARQ process number field in a DCI format indicates a release for a corresponding UL grant Type 2 PUSCH or for a SPS PDSCH configuration with a same value as provided by Configuredgrantconfig-index or by SPSconfig-index, respectively [omit the unchanged part]

- Conclusion

In the contribution, we correct some RRC parameters in RAN1 specifications. Following is the proposal:

Proposal 1: Adopt following text proposal in TS 28.213.

TS 38.213 ## 10.2 PDCCH validation for DL SPS and UL grant Type 2 A UE validates, for scheduling activation or scheduling release, a DL SPS assignment PDCCH or a configured UL grant Type 2 PDCCH if - the CRC of a corresponding DCI format is scrambled with a CS-RNTI provided by cs-RNTI, and - the new data indicator field in the DCI format for the enabled transport block is set to '0', and - the DFI flag field, if present, in the DCI format is set to '0', and - if validation is for scheduling activation and if the PDSCH-to-HARQ_feedback timing indicator field in the DCI format is present, the PDSCH-to-HARQ_feedback timing indicator field does not provide an inapplicable value from dl-DataToUL-ACK. If a UE is provided a single configuration for UL grant Type 2 PUSCH or for SPS PDSCH, validation of the DCI format is achieved if all fields for the DCI format are set according to Table 10.2-1 or Table 10.2-2. If a UE is provided more than one configurations for UL grant Type 2 PUSCH or for SPS PDSCH, a value of the HARQ process number field in a DCI format indicates an activation for a corresponding UL grant Type 2 PUSCH or for a SPS PDSCH configuration with a same value as provided by Configuredgrantconfig-index or by SPSconfig-index, respectively. Validation of the DCI format is achieved if the RV field for the DCI format is set as in Table 10.2-3. If a UE is provided more than one configuration for UL grant Type 2 PUSCH or for SPS PDSCH - if the UE is provided ConfiguredGrantConfigType2DeactivationStateListType2Configuredgrantconfig-ReleaseStateList or sps-ConfigDeactivationStateList SPS-ReleaseStateList, a value of the HARQ process number field in a DCI format indicates a corresponding entry for scheduling release of one or more UL grant Type 2 PUSCH or SPS PDSCH configurations - if the UE is not provided ConfiguredGrantConfigType2DeactivationStateList Type2Configuredgrantconfig-ReleaseStateList or sps-ConfigDeactivationStateList SPS-ReleaseStateList, a value of the HARQ process number field in a DCI format indicates a release for a corresponding UL grant Type 2 PUSCH or for a SPS PDSCH configuration with a same value as provided by Configuredgrantconfig-index or by SPSconfig-index, respectively [omit the unchanged part]

References

[1] 3GPP TS 38.331 V16.1.0, “Radio Resource Control (RRC) protocol specification,” July 2020