3GPP TSG RAN WG1 Meeting #102-e	R1- 2006769

Aug. 17th – 28th, 2020

Agenda item:	7.2.4.2.1

Source: 	Qualcomm Incorporated

Title: 	Sidelink Resource Allocation Mode 1

Document for:	Discussion/Decision

# Introduction

In this contribution, we discuss open issues for NR V2X specifications regarding Mode 1 resource allocation.

# DCI Size Alignment

The following agreement and working assumption were made in RAN1 99 [5]:

Agreements:

- Existing DCI size budget is maintained when the UE is configured with SL

- (working assumption): The size of the new DCI format and the size of one of the existing NR DCI formats are aligned.

In the NR Uu controlling LTE sidelink, the following agreement was made [6]:

Agreements

- If UE is configured to monitor DCI 3_0, the sizes of DCI 3_1 and DCI 3_0 are is aligned by zero padding

- If UE is not configured to monitor DCI 3_0, the mechanism for size alignment between DCI 3_0 and Uu DCI is reused for size alignment between DCI 3_1 and Uu DCI

To implement the above, it is simplest to align DCI 3-0 (DCI 3-1) size with DCI 0-1 or 1-1. If neither DCI 0-1 nor DCI 1-1 is configured, and the size budget can still be maintained, DCI 3-0 size is not aligned. Otherwise, DCI 3-0 is size aligned with the largest Uu DCI format configured for the UE.

Proposal 1: DCI 3-0 size is aligned with the larger of DCI 0-1 or 1-1 if either of the latter two is configured.

Proposal 2: DCI 3-0 size is aligned with the largest DCI format that the UE monitors if neither DCI 0-1 nor 1-1 is configured and the DCI size budget would be exceeded otherwise.

# Scheduling from PSCell and SCell

One remaining open issue is whether the UE monitors DCI formats 3-0 and 3-1 on PSCell and SCell. Such a requirement increases UE complexity while its benefits have not been studied for sidelink. Therefore, we propose to have the UE only monitor DCI 3-0 and 3-1 on PCell.

Proposal 3: DCI formats 3-0 and 3-1 are only monitored on PCell

# Conclusion

Proposal 1: DCI 3-0 size is aligned with the larger of DCI 0-1 or 1-1 if either of the latter two is configured.

Proposal 2: DCI 3-0 size is aligned with the largest DCI format that the UE monitors if neither DCI 0-1 nor 1-1 is configured and the DCI size budget would be exceeded otherwise.

Proposal 3: DCI formats 3-0 and 3-1 are only monitored on PCell

# References

- 3GPP TS 38.211 V16.1.0, NR, Physical Channels and Modulation (Release 16).

- 3GPP TS 38.212 V16.1.0, NR, Multiplexing and Channel Coding (Release 16).

- 3GPP TS 38.213 V16.1.0, NR, Physical Layer Procedures for Control (Release 16).

- 3GPP TS 38.214 V16.1.0, NR, Physical Layer Procedures for Data (Release 16).

- Chairman’s Notes, RAN1 #99, November 2019, Reno, USA.

- Chairman’s Notes, RAN1 #100e, March 2020.

- Chairman’s Notes, RAN1 #100bis-e, April 2020.