3GPP TSG RAN WG1 #102e 	R1- 2006778

August 17th – 28th, 2020

Agenda item:	7.2.5.5

Source: 	Qualcomm

Title: 	Remaining issues on uplink inter-UE Tx multiplexing and prioritization

Document for:	Discussion and Decision

# 1	Introduction

In this document, the proposals for the following topics are presented. Each corresponding section also includes a reason for the proposed addition or modification.

| Section 2 | Earlier cancellation relaxation for ULCI |
| --- | --- |
| Section 3 | A 2-D bit-map for ULCI |
| Section 4 | Enhanced power control for PUSCH with repetitions |

End

# 2	Earlier cancellation relaxation for ULCI

As has been discussed for cancellation under intra-UE prioritization, enforcing an exact cancellation time incurs significant implementation complexity for the UE. Instead, if an uplink transmission needs to be cancelled, the UE should be allowed to start the cancellation even before the first indicated symbol.

Proposal 1: If a UE is indicated by DCI 2_4 to cancel a PUSCH transmission, the UE can cancel the transmission before the first indicated symbol group. In other words, the first indicated symbol using DCI format 2_4 for cancellation is the latest time that the UE should start the cancellation of PUSCH.

# 3		A 2D bitmap for ULCI

In RAN1 #99, it was agreed that the time domain granularity for ULCI is defined as number of partitions within the time region denoted by GCI, and that the number of symbols indicated by ULCI, denoted by TCI, shall exclude cell-specific semi-static downlink symbols. However, since the ULCI may need to be monitored with small periodicity, it may happen that the number of symbols covered by the ULCI after removing the semi-static DL symbols is smaller than GCI. In this case, the current 2D bitmap method will resolve in unnecessarily wasted bits. Instead, these bits could be reused to indicate a finer resolution in the frequency domain.

Consider for example, GCI=7, NCI=14 and TCI=5. The 2D mapping rule in the current spec will have 2 frequency-region in each symbol group, and 4 bits will be wasted. Instead, we could reuse the additional four bits to achieve a finer granularity in the indicate 3 frequency domain on the first four symbol groups, as shown in the following figure.

Fig 1. 2D big map for ULCI

Modified clause (Section 11.2A of TS 38.213)

For a serving cell having an associated field in DCI format 2_4, for the field denote by

-	NCI a number of bits provided by CI-PayloadSize

-	BCI a number of PRBs provided by frequencyRegionforCI in timeFrequencyRegion

-	TCI a number of symbols, excluding symbols for reception of SS/PBCH blocks and DL symbols indicated by tdd-UL-DL-ConfigurationCommon, provided by timeDurationforCI in timeFrequencyRegion

-	GCI a number of partitions for the TCI symbols provided by timeGranularityforCI in timeFrequencyRegion

If GCI≤TCI,

- GCI sets of bits from the NCI bits have a one-to-one mapping with GCI groups of symbols where each of the first GCI-TCI+TCIGCI⋅GCI groups includes TCIGCI symbols and each of the remaining TCI-TCIGCI⋅GCI groups includes TCIGCI symbols.

- For a group of symbols, NBI=NCIGCI bits from each set of bits have a one-to-one mapping with NBI groups of PRBs where each of the first NBI-BCI+BCINBI⋅NBI groups includes BCINBI PRBs and each of the remaining BCI-BCINBI⋅NBI groups includes BCINBI PRBs.

If GCI>TCI,

- TCI sets of bits from the NCI bits have a one-to-one mapping with the TCI symbols, where each of the first NCI-NCITCI⋅TBI  set of bits includes NBI=NCITCI bits, and each of the remaining TCI-NCI+NCITCI⋅TBI   sets include NBI=NCITCI bits

- For a symbol, the corresponding set of bits has a one-to-one mapping with NBI RB groups (RBGs) of PRBs, where each of the first NBI-BCI+BCINBI⋅NBI groups includes BCINBI PRBs and each of the remaining BCI-BCINBI⋅NBI groups includes BCINBI PRBs.

A UE determines a symbol duration with respect to a SCS configuration of an active DL BWP where the UE monitors PDCCH for DCI format 2_4 detection.

A UE determines a first PRB index as NRFRstart=Ocarrier+RBstart and a number of contiguous RBs as NRFRsize=LRB from frequencyRegionforCI that indicates an offset RBstart and a length LRB as RIV according to [6, TS 38.214], and from offsetToCarrier that indicates Ocarrier for a SCS configuration of an active DL BWP where the UE monitors PDCCH for DCI format 2_4 detection.

End

# 4   An enhanced power control for PUSCH with repetitions

For Rel-16 enhanced uplink power control (i.e., dynamic open-loop power control), a UE may be dynamically indicated the P0 values in DCI via the open-loop power control field. At the same time, the UE may also be configured or dynamically scheduled with PUSCHs with repetitions that occurs in multiple slots. When PUSCH are scheduled with repetitions (either repetition Type A or Type B) and is indicated by DCI to do power boosting (i.e., if the OLPC field in the scheduling DCI indicates 1 or 01 or 10), shall the UE perform power-boosting on all repetitions, or on a subset of repetitions? For simplicity, we propose that the power boosting is applied to all PUSCH repetitions scheduled by the DCI.

Proposal 2: for Rel-16 PUSCH, clarify that the open-loop power control parameter indicated in DCI format 0_1/0_2 applies to all PUSCH repetitions scheduled by the DCI.