3GPP TSG RAN WG1 #102	R1-2006862
e-Meeting, August 17th – 28th, 2020

Source:				Panasonic

Title:	Maintenance for DL signals and channels for NR-U

Agenda item:	7.2.2.1.2

Document for:			Discussion and Decision

Introduction

This document proposes to clarify the condition of DMRS for 10 symbol PDSCH. This is the resubmission of the previous one [1].

Discussion

PDSCH DMRS shift of 10 symbol PDSCH

In Rel-16, each duration of {2, …, 13} is supported for PDSCH mapping type B (normal CP) [2]. Consequently, DMRS for those durations are also specified.

In DSS operation with numerology μ=0, PDSCH mapping type B can be on the symbols #4-13. DMRS for this PDSCH mapping is specified on the symbols #4, 8, 11 and then it can collide with the CRS for 4 CRS ports configuration in LTE non-MBSFN subframe. Figure 1 shows the DMRS and CRS position.

Figure 1: DMRS and CRS position

In order to avoid the collision between NR DMRS and LTE CRS, the procedure to shift DMRS is specified. When the collision happens, DMRS is shifted to the next symbol according to the current description.

TS 38.211 V16.0.0 [3]

7.4.1.1.2 Mapping to physical resources … For PDSCH mapping type B … - if the higher-layer parameter lte-CRS-ToMatchAround or additionalLTE-CRS-ToMatchAroundList is configured, the PDSCH duration ld=10 symbols for normal cyclic prefix, the subcarrier spacing configuration μ=0, single-symbol DM-RS is configured, and at least one PDSCH DM-RS symbol in the PDSCH allocation collides with a symbol containing resource elements as indicated by the higher-layer parameter lte-CRS-ToMatchAround or additionalLTE-CRS-ToMatchAroundList, then l shall be incremented by one in all slots. …

The implementation is simplified to shift DMRS regardless of MBSFN or non-MBSFN subframe although there is no collision in MBSFN subframe. That’s why the agreement to shift DMRS in all slots was made in RAN1 #99.

Agreement: Resolve the FFS from the agreement in RAN1#98 as follows - If the PDSCH duration ld is 10 OFDM symbols for normal cyclic prefix, and if at least one of the DM-RS symbols corresponding to the PDSCH allocation collides with resources reserved for at least one of the symbols of a LTE CRS transmission, shall be incremented by 1 - The increment is applicable to all slots

However, the current specification has an inconsistency on the condition of the shift. As the description is based on "at least one PDSCH DM-RS symbol in the PDSCH allocation collides with a symbol containing resource elements...", when PDSCH allocation is no collision with CRS transmission, UE assumes DMRS is not shifted. But once the collision is found in PDSCH allocation, the UE has to assume DMRS in all slots is shifted regardless whether there is the collision or not. Besides, it is not described for the UE when DMRS shift stops as once collision with CRS transmission, the increment is applicable to all slots. It can be interpreted that this DMRS shift applied forever.

In order to resolve above issue, the description of collision should be judged by the semi-static parameters as it is applied to all slots. Specifically, we propose that the DMRS is shifted when all the conditions below are fulfilled:

- Higher layer parameter related to LTE CRS is configured

- PDSCH duration: 10

- Numerology: μ=0

- Single symbol DMRS is configured

- PDSCH starting position: 4 (Note: this removal is the update reflecting the views from companies to avoid complexity increase).

Proposal 1: adopt the following TP for TS 38.211

Text Proposal for PDSCH DMRS shift of 10 symbol PDSCH to TS 38.211 in Rel-16:

7.4.1.1.2 Mapping to physical resources … For PDSCH mapping type B … - if the higher-layer parameter lte-CRS-ToMatchAround or additionalLTE-CRS-ToMatchAroundList is configured, the PDSCH duration ld=10 symbols for normal cyclic prefix, the subcarrier spacing configuration μ=0, single-symbol DM-RS is configured at least one PDSCH DM-RS symbol in the PDSCH allocation collides with a symbol containing resource elements as indicated by the higher-layer parameter lte-CRS-ToMatchAround or additionalLTE-CRS-ToMatchAroundList, then l shall be incremented by one in all slots. …

- 3. Conclusion

Proposal 1: adopt the following TP for TS 38.211

Text Proposal for PDSCH DMRS shift of 10 symbol PDSCH to TS 38.211 in Rel-16:

7.4.1.1.2 Mapping to physical resources … For PDSCH mapping type B … - if the higher-layer parameter lte-CRS-ToMatchAround or additionalLTE-CRS-ToMatchAroundList is configured, the PDSCH duration ld=10 symbols for normal cyclic prefix, the subcarrier spacing configuration μ=0, single-symbol DM-RS is configured at least one PDSCH DM-RS symbol in the PDSCH allocation collides with a symbol containing resource elements as indicated by the higher-layer parameter lte-CRS-ToMatchAround or additionalLTE-CRS-ToMatchAroundList, then l shall be incremented by one in all slots. …

- 4. Reference

R1-2002054, Panasonic, RAN1 #100b, e-Meeting, April 2020

R1-2002786, Feature lead summary for NR-U DL Signals and Channels, Moderator (Lenovo), RAN1 #100b, e-Meeting, April 2020

3GPP TS 38.211 V16.0.0