3GPP TSG RAN WG1 #102-e                                                                                            R1-2006876

E-Meeting, August 17th – 28th 2020

Agenda item:	8.11.2.2

Source:	Robert Bosch GmbH

Title:	Sidelink Resource Allocation Enhancements

Document for:	Discussion

# Introduction

In RAN#86 meeting, a new Rel-17 WI for sidelink enhancements was approved with two main motivations, power saving and enhanced reliability/reduced latency. The objectives of the WID were further adapted in RAN#88e in order to meet the shifted timeline due to current pandemic, where the following is stated:

In Rel-17 WI sidelink enhancements, the following objectives are approved [1]:

2. Resource allocation enhancement:

- Study the feasibility and benefit of the enhancement(s) in mode 2 for enhanced reliability and reduced latency in consideration of both PRR and PIR defined in TR37.885 (by RAN#91), and specify the identified solution if deemed feasible and beneficial [RAN1, RAN2]

  - Inter-UE coordination with the following until RAN#90.

    - A set of resources is determined at UE-A. This set is sent to UE-B in mode 2, and UE-B takes this into account in the resource selection for its own transmission.

  - Note: The study scope after RAN#90 is to be decided in RAN#90.

  - Note: The solution should be able to operate in-coverage, partial coverage, and out-of-coverage and to address consecutive packet loss in all coverage scenarios.

  - Note: RAN2 work will start after [RAN#89].

This contribution provides high-level discussions and motivation for resource allocation enhancement considering inter-UE coordination. More specifically, we are considering the automotive community concerns regarding UE assisting information and possible reliability enhancements and latency reduction in this case.

# Resource allocation enhancement

In NR-V2X Rel-16, several enhancements are introduced including advanced control channel design, physical layer structure, physical layer procedure (e.g., HARQ/CSI feedbacks), and multiple resource allocation enhancements for both periodic and aperiodic traffic. During Rel-16 SI phase, two sub-modes of Mode 2 resource allocation were proposed to allow UEs assisting each other enhancing reliability and reducing latency. These two sub-modes were:

- Mode 2b: where one UE assists another UE for resource allocation, and

- Mode 2d: where one UE schedules specific resources for another UE.

Both sub-modes were not considered in Rel-16 WI phase. However, Rel-17 considers studying and specifying, if deemed feasible, resource allocation enhancements considering inter-UE coordination. In our understanding, this may require exchanging assisting information.

## Inter-UE coordination enhancing reliability and reducing latency

In our understanding, inter-UE coordination may help UEs reducing latency (by avoiding long sensing operation) or avoiding resource conflicts, e.g.,

- Resource collisions between UEs transmission in the same slot

- Half-duplex problem

- Hidden node problem

Failure to decoding resource reservations, either due to half-duplex problem and/or hidden node problem, may result in performance degradation. Hence, inter-UE coordination may support enhancing the performance in this case. Inter-UE coordination may also support achieving power saving by, e.g., avoiding long/always-on sensing operation. Additionally, if inter-UE coordination would avoid excessive retransmission due to resource conflict/transmission failures, it may then reduce power consumption in such a way.

Observation 1: Inter-UE coordination can

- enhance reliability by resolving resource allocation conflict like resource collision, failure to decoding resource reservation due to hidden node or half-duplex problems, etc.

- support power saving by avoiding always-on sensing

- reduce latency by avoiding long sensing operation

In automotive domain, inter-UE coordination can be efficiently utilized in some use cases, e.g., platooning, group-start, and vulnerable road-user (VRU) communication. In Platooning use case, a leader vehicle/truck is envisaged to coordinate communication between group members. Hence, selecting a set of common free resources may be exchanged between the group-lead and other group members. Similarly, group start needs a fast and reliable communication resources between the instantaneously established vehicle group members. For VRU, pedestrian UEs may expect to receive specific resources for transmission from assisting UE (vehicle or infra-structure), in which these resources can guarantee high transmission reliability. In general, also other advanced V2X use cases can benefit from inter-UE coordination exploiting the additional latency reduction and reliability enhancements.

Observation 2: Inter-UE coordination is beneficial for various V2X advanced use cases and other use cases like commercial and public safety use cases

Figure 1 depicts possible resource conflict (e.g., between UE-B and UE-B’) where resources may be colliding or facing a half-duplex problem. Another issue may result in resource conflict is having the two UEs (UE-B/B’) hidden from each other. In these two cases, a third UE (e.g., UE-A) may assist any of the other UEs (UE-B and/or UE-B’) to resolve their conflicting transmission. Hence, the assisting UE-A can send an assisting information including the detected resource conflicts. Detection of the conflicting resources may, e.g., rely on decoding sidelink control information (SCI) or decoding the DMRS signals.

Figure 1: Resource allocation conflict due to half-duplex problem and/or hidden node problem

Additionally, a UE may assist another UE by sending a set of possibly free/available (recommended resources). In this case, other UEs can use the proposed resources with no or limited sensing. This can be beneficial to assist UEs with limited capabilities, e.g., pedestrian UEs, or UEs within the same group communication. Moreover, the assisting information may assist UEs selecting their feedback resources, e.g., PSFCH, possible power control values, DRX cycles coordination, and/or recommending certain cast types.

Proposal 1: A UE may send an assisting information including:

- possible conflicting resources due to collision, half-duplex, or hidden-node problem

- possible available/recommended resources for other UEs

- possible feedback scheduling information, power control, common DRX information, and/or recommended cast type

## Inter-UE coordination assisting information

A UE may detect confecting resources via sensing/decoding SCI information and/or detecting DMRS signals. Once resource conflict is detected, a UE may send the corresponding resource information (using an indication, resource patterns or compressed/simplified resource sets) in a specific assisting message. Such an assisting information may be sent as a MAC control element (CE) and or in a physical sidelink control channel (SCI), e.g., using one or more of reserved bits in the SCI combined with the information in the SCI reservations. For physical layer messages, the limited number of bits (in either first or second stage SCI) can be used to indicate the presence of the resources conflict.

Additionally, it was also proposed during Rel-16 SI phase to use even higher-layer messages, e.g., RRC-S messages. In this case, RRC-S messages may indicate possible resource conflicts, available resources, or other assisting information. However, this can be only possible when a unicast session is established.

Proposal 2: A UE may send assisting information using physical-layer or higher-layer signaling

In order to trigger sending of assisting information, a UE (e.g., assisting UE-A) may evaluate the conflicting resources after the problem is identified, e.g.,:

- consecutively for several time slots in the past

- for a certain high priority transmission (threshold)

- for conflicting transmission within a certain communication range

Once coordination is triggered as mentioned above, a UE (e.g., UE-A) may send the assisting information considering possible processing time, see Figure 2 for more details. The UE may decide to send assisting information for multiple times (e.g., using retransmission) allowing enough time for the other UEs to resolve the conflict. The number of repetitions, K, of the assisting information may depend on the severity of the resource conflict or the transmission priority of the colliding UEs. In order to preserve congestion control levels, a UE may decide on the previous metrics considering the CBR values. Additionally, the frequency and repetitions of assisting information messages may depend on the measured CBR/CR values or any other congestion metrics.

Figure 2: Illustration of timeline of assisting information

Proposal 3: A UE may be triggered to send an inter-UE coordination using certain triggering conditions, e.g.,

- priority of transmission indicated in the received SCI,

- communication range of the other UEs

- the persistence/duration of the existing resource conflict,

- severity of resource collision depending on RSRP threshold

Proposal 4: Repetition of inter-UE coordination messages depends on the severity of the resource collision or transmission priority of the assisted UEs

Proposal 5: Congestion control needs to be preserved while performing inter-UE coordination

A UE may send other assisting information for UEs in vicinity, e.g., recommended free resources. Such an information may be used by other UEs for limiting their sensing and reduce power dissipation. Yet, the UE may also send such available resources along with conflicting resources. Additionally, other useful information may be studied, e.g., feedback scheduling information, power control information, DRX information, and recommended cast type.

# Conclusions

In this contribution the following proposals have been made:

Proposal 1: A UE may send an assisting information including:

- possible conflicting resources due to collision, half-duplex, or hidden-node problem

- possible available/recommended resources for other UEs

- possible feedback scheduling information, power control, common DRX information, and/or recommended cast type

Proposal 2: A UE may send assisting an information using physical-layer or higher-layer signaling

Proposal 3: A UE may be triggered to send an inter-UE coordination using certain triggering conditions, e.g.,

- priority of transmission indicated in the received SCI,

- communication range of the other UEs

- the persistence/duration of the existing resource conflict,

- severity of resource collision depending on RSRP threshold

Proposal 4: Repetition of inter-UE coordination messages depends on the severity of the resource collision or transmission priority of the assisted UEs

Proposal 5: Congestion control needs to be preserved while performing inter-UE coordination

# References

- RP-2013805, “NR Sidelink enhancement”, Work Item Description, Electronic Meeting, June 29 - July 3, 2020.