| 3GPP TSG-RAN WG1 Meeting #102 e-Meeting, August 17th – 28th, 2020 |  | R1-2006977 |
| --- | --- | --- |

Agenda item:	8.8.2.1

Source: 	Qualcomm Incorporated

Title: 	Potential coverage enhancement techniques for PUSCH

Document for:	Discussion/Decision

# Introduction

In this contribution, we discuss some potential techniques to enhance PUSCH coverage. More specifically, we discuss MPR reduction, DMRS enhancements, TBS optimization for small payload, and Msg3 repetition.

# Techniques to reduce MPR in uplink transmissions

Based on our initial link budget analysis for rural and urban scenarios [1][2], uplink channels are a bottleneck for coverage and are a limiting factor when determining 5G NR cell coverage. This situation primarily arises due to fact that the BS and the UE differ significantly in the total transmitted power. This draws attention to techniques that let the UE transmit at a higher power, while being subject to the restrictions imposed by its power class. These observations motivate us to take a closer look at any unused transmit power at the UE and draws attention to the MPR table in 38.101-1.

We use a power class 3 UE as a motivating example, but the ideas presented here are applicable to UEs of any power class. Table 6.2.2-1 of 38.101-1 as provided in Table 1 specifies a set of power reduction values dependent on RB allocation and modulation order for power class 3 UEs. The power back off values are then used by the UE to calculate the lower bound on its PCMAX,f,c value. The set of power reduction values may sometimes be band-dependent, but for the sake of motivating the current proposal, we use band n78 as an exemplary band, and note that for this band this table applies as it is without any further consideration for band-specific A-MPR. The values in this table are chosen such that a UE’s transmissions meet the various RAN4 restrictions on ACLR, IBE, EVM and SEM.

Table 1 MPR Table from 38.101-1

While it is seen that for DFT-S-OFDM with inner RB allocations it is required to transmit the lower order modulations such as pi/2 BPSK and QPSK at full power without any power back off, a back off of up to 1 dB may be applied to outer and edge RB allocations. As one proceeds to higher order modulations, significant power back offs may be applied, with DFT-S-OFDM with 64 QAM being allowed up to 2.5 dB back off. Similarly, for CP-OFDM, power back off of up to 3 dB may be applied to outer RB allocations with QPSK modulation, with progressively increasing back offs for higher order constellations.

Note that DFT-S-OFDM with pi/2 BPSK has two sets of values defined, one for the case where the 0 dB MPR is in reference to 23 dBm and another where the 0 dB MPR is in reference to 26 dBm. This change in reference power to 26 dBm is permitted when UE is operating in TDD mode with less than 40% of the slots in a radio frame being used for uplink transmission. This particular amendment is based on the observation that the power class of a UE is defined on the basis of average transmit power and not based on instantaneous power used in a given slot, thus allowing a UE to transmit at a power above the value indicated by its power class. UEs that support this additional 3 dB boost are identified by the powerBoosting-pi2BPSK capability.

It is thus clear that for both lower and higher order constellations there is unused uplink power at the UE transmitter than can potentially be unlocked by considering advanced waveform design ideas that help the UE meet the various RAN4 requirements without having to back off on maximum transmit power. In particular, we believe it is beneficial to study advanced waveform design techniques that allow reducing the MPR values for waveforms of all modulation orders.

Reducing the PAPR (Peak to Average Power Ratio) and CM (Cubic Metric) of the waveform is a common method for achieving higher transmission powers leading to lower MPRs. The input back off required for meeting the RAN4 restrictions is generally smaller for the reduced-PAPR waveform. This translates to a higher transmission power as well as higher PA efficiency. Among the large number of known PAPR reduction techniques, “tone reservation” stands out due to its simplicity and efficient usage of the available resources. This technique dedicates a subset of available sub-carriers to reducing the PAPR of the waveform. These subcarriers are referred to as Peak Reduction Tones (PRTs). To preserve the EVM, there is no overlap between the PRTs and the subcarriers used for transmitting the data.

The principle of tone reservation aims to use the PRTs to construct a compensating waveform, which when added to the raw waveform results in a lower-PAPR waveform. This compensating waveform is constructed by identifying peaks to cancel in the original waveform. The compensating waveform only uses the PRTs and thus leaves the data tones unaltered. Several efficient algorithms exist for this purpose. For example, one well known algorithm is by J. Tellado [4] that iteratively cancels the peaks in the original waveform.

While there are many options for the location of PRTs in reference to the data tones, for the DFT-s-OFDM waveform, reserving tones immediately adjacent to the RBs allocated to a UE is a reasonable choice. This is illustrated in the figure below where two UEs are allocated dedicated data tones for their uplink transmission and in addition have some tones on either side of their allocation reserved as PRTs. Note that when using algorithms such as those in [4], the power expended by a UE within the PRTs is quite negligible in comparison to the overall transmit power.

Figure 1 Tone reservation

On the receiver side, the gNB can discard the PRTs and process only the data tones allocated to a UE to recover the uplink payload. Thus, this approach to increasing uplink tx power has very minimal impact on gNB receiver design.

These ideas are applicable to any modulation order and also to CP-OFDM waveforms, thus providing a common framework to boost uplink transmit power across a wide range of scenarios.

To illustrate the effectiveness of these principles, we consider a UE that is allocated 8 RBs for PUSCH transmission. In addition to these 8 RBs, the UE is also provided with 1 RB of PRTs on either side of its allocation, as shown in the figure below. Thus, the UE is in all allocated a total of 10 RBs out of which 8 are used as data tones, and 2 RBs used for waveform shaping.

Figure 2 Tone reservation with 20% Sideband PRTs

Focusing on such an allocation we consider a DFT-S-OFDM with QPSK modulation and highlight the significant improvements in PAPR and Raw Cubic Metric (RCM) improvements due to tone reservation in Fig 3, the left and right figures respectively represent the CCDF of  PAPR and RCM. The PAPR improvements are as large as 3.3 dB at 10-6 CCDF level and 2.2 dB at 10-4. The RCM improvements are 1.2 dB and 1.0 dB at 10-3 and 10-2 CCDF levels respectively. The Power Spectral Density (PSD) of the PRTs is 15 dB lower than the PSD of the data tones. Therefore, the observed PAPR and RCM gains come at a negligible price in terms of energy and complexity.

Given these encouraging results and the promise of a common unified framework to increase the uplink transmit powers across all waveform types and modulation orders, we make the following proposal:

Proposal 1: Techniques for UE transmit waveform design that allow further reduction in the MPR values for DFT-S-OFDM and CP-OFDM waveforms should be studied for coverage enhancement.  In particular, consider tone reservation principle for DFT-s-OFDM and CP-OFDM waveforms to further reduce PAPR.

Fig 3: CCDF of PAPR and RCM of the DFT-s-OFDM QPSK waveform before and after PAPR reduction

# PUSCH DMRS Enhancements

In this section, we discuss two possible approaches namely DMRS bundling and adaptive DMRS to improve channel estimation accuracy for PUSCH, and hence to extend the coverage of PUSCH.

## DMRS bundling

In Rel-15/16, channel estimation for data demodulation is based on DMRS symbols within a slot (or a mini-slot). There is no coherence requirement on the DMRS transmissions across different slots/repetitions.  At the cell edge, the performance degradation due to channel estimation error may be significant, and methods to improve the channel estimation quality may be beneficial. One way to improve the channel estimation accuracy in this scenario is to let the receiver jointly processing DMRS symbols in multiple slots. To this end, the transmitter needs to coherently transmit DMRS symbols over multiple slots/repetitions, i.e., the transmitter should maintain phase continuity across DMRS symbols in different slots/repetitions. This technique is also known as DMRS bundling.

Fig 4: DMRS bundling performance with 1 DMRS symbol per slot

To show how much performance gain can be offered by using DMRS bundling, we provide some simulation results below. Fig 4, we compare the performance of DMRS transmission with and without bundling for PUSCH. In the simulations, we assume that the number of DMRS symbols is fixed to 1 symbol, and data communication occupies 13 OFDM symbols. For DMRS bundling, we consider both the case in which the same TBs are transmitted over different slot (i.e., slot aggregation), as well as the case in which different TBs are transmitted over different slots. In all the simulations, the same MCS value is used. The red curves illustrate the performance without DMRS bundling. The blue curves show the performance with DMRS bundling over two slots. The green curves show the performance with DMRS bundling over four slots. As can be seen from the figure, DMRS bundling offers more than 0.8~1.5 dB gain in the considered scenarios.

Fig 5: DMRS bundling performance with fixed TBS per slot

In Fig 5, we consider a scenario where a PUSCH is scheduled with 1 RB and 14 OFDM symbols. We change the number of DMRS symbols within the slot but kept the transport block size (TBS) fixed. It is appreciated that there is a trade off on how many DMRS symbols are used for the transmission. On the one hand, with more DMRS symbols, the channel estimation quality is better. On the other hand, using more DMRS symbols means higher coding rate (i.e., less resources for transmitting data). As can be seen from the figure, the performance of using 1 DMRS symbol and 2 DMRS symbols provide similar performance, which are uniformly better than using 4 DMRS symbols. However, by using 1 DMRS per slot and bundle across 2 slots, we may achieve around 0.5 dB gain compared to the case of no DMRS bundling with 1 or 2 DMRS symbols per slot. Further gains may be achieved by bundling across more slots.

Based on the above analysis, we make the following proposal:

Proposal 2: Consider DMRS bundling technique for coverage enhancement in NR Rel-17.

## Adaptive DMRS

Current specification assumes usage of fixed pilot patterns which are RRC configured per UE. Consequently, it is not possible to reconfigure DMRS configurations since RRC based reconfiguration is a non-synchronous process that also involves a high latency in practice. There is some flexibility with a limited set of DMRS patterns that are implicitly configured based on a static DMRS configuration and some PDSCH allocation parameters, but it is introduced in order to ease NW scheduling procedures rather than to allow DMRS configuration adaptation “on the fly”.

Channel estimation accuracy depends on the level of correlation of the channel in time and frequency, working SNR point of a UE and on the used for chest 2D pilot grid option. Channel parameters and SNR conditions are different for different UEs and are also varying in time. Different channel and SNR conditions require different pilot configuration option to maximize spectral efficiency of a link per UE. Hence, using a fixed pilot configuration in the NW requires to trade off in advance between average pilots overhead and UEs performance. As a result, in some cases extra resources are wasted for unnecessary pilots while in other cases UE performance is floored because of non-appropriate pilot configuration.

### Adaptive DMRS configuration

Using adaptive DMRS configuration per UE per slot (or some number of slots) to keep it aligned with a varying channel and reception conditions can provide a significant link performance improvement. For cell edge UEs link performance is limited by channel estimation processing gain which can be improved if denser DMRS patterns are used. Correspondingly, DMRS configuration adaptation can improve both coverage and rate over range characteristics in the network. Relative comparison for maximal achievable TPUT with different tested DMRS configurations is provided below for few scenarios (TPUT per each SNR point is normalized by the max TPUT achievable with the best DMRS option among the tested options)

Figure 6 Performance of different DMRS configurations at low doppler

Figure 7 Performance of different DMRS configurations at medium doppler

(*) - Jitter of the curves is related to simulated SNR and MCS resolution

It can be seen that, for different scenarios and SNR ranges, a different DMRS configuration option (color) hits 100% of the max relative TPUT (becomes to be the best option among the 4 tested options). Relative degradations of different DMRS configuration options for every SNR point can be also observed. This clearly shows a significant value in DMRS adaptation for a wide range of scenarios.

We also provide results focused on the possible cell edge scenarios in Figure 8 with the following assumptions:

- RI=1

- MCS adaptation per DMRS option, MCSs from range 0-9

- Same allocation size (time, frequency) for the different tested DMRS configurations

Figure 8 Performance of PUSCH with different DMRS configurations for cell edge scenario

Additional comparison for the same scenarios in Figure 8 but with fixed MCS is provided in Figure 9 and Figure 10 for MCS = 0 and MCS = 2, respectively.

Figure 9 Performance of PUSCH with different DMRS configurations  RI=1, MCS=0

Figure 10 Performance of PUSCH with different DMRS configurations RI=1, MCS=2

As can be observed from the above results, usage of the most appropriate DMRS configuration has a potential to provide gain of up to 1.7dB per fixed MCS and tens of percent of TPUT increase at cell edge scenarios for PUSCH.

Proposal 3: Consider adaptive DMRS configuration for PUSCH to improve both coverage and link efficiency and introduce signaling mechanisms for dynamic DMRS configuration change.

An example of signaling mechanism is that a set of DMRS patterns is RRC-configured. Then a combination of MAC-CE and DCI signaling might be used to indicate a suitable DMRS pattern that the UE shall use for PUSCH transmission.

### Additional RS

The uplink channel estimation could be improved by adaptively activating additional reference signal (RS). The activation mechanism may be similar to the one discussed in Section 3.2.1. An example of such mechanism is that the UE receives DMRS pattern via RRC and possibly receives an indication in DCI to activate additional RS to support channel estimation at gNB. In other word, the gNB could use both DMRS indicated in RRC and additional RS for channel estimation. A new DMRS pattern including RRC-configured DMRS and additional RS might be introduced with the introduction of additional RS. Based on the above analysis, we make the following proposal:

Proposal 4: Additional RS should be studied for improving channel estimation to extend PUSCH coverage.

# TBS Scaling and Optimization for PUSCH

In Rel-15/16, according to section 6.1.4.2 and 5.1.3.2 of TS38.214. TBS is determined based on the scheduled modulation order Qm, coding rate R, number of MIMO layers ν and number of available REs in the scheduled slot (denoted by NRE). To deliver a packet to the gNB, the UE may be scheduled with higher MCS and less resource if channel condition is good or lower MCS and larger resource if the channel condition is bad. For UE at cell-edge, larger resource allocation does not seem to be an ideal option due to the limited power budget. Thus, cell-edge UEs are most likely to engage in narrow band transmissions with low MCS (QPSK and lower coding rate). Such a configuration can often require the UE to divide a higher-layer packet into multiple segments and transmit the packets over multiple small TBs in multiple UL grants.

As illustrated in the figure below, if a UE has a packet with 1280 bits (e.g., a video call), the UE may transmit the packet in one slot using 16RBs, or divide it into 4 parts each with 320 bits and transmit them in 4 slots using 4RBs to have better coverage.

Figure 11 Illustration for TB segmentation and TBS scaling/optimization.

Another example would be VoIP service in which UE needs to deliver 328 bits per 20ms. UE may transmit it in one slot with 4RBs or divide it into 4 parts each with 82 bits and transmit them in 4 slots using 1RB/slot to have better coverage.

While the second option is clearly the better choice for a cell-edge UE, the segmentation of the original packet incurs two kinds of penalties --- (a) upper-layer headers get appended to each segment, there by increasing the total number of bits to transmit and (b) coding gain tends to decrease as the block size decreases, i.e., it is not very power efficient to transmit multiple TBs of a few 100 bits as opposed to a single TB of size 1000 bits.

In order to overcome these issues, and to let a cell-edge UE enjoy all the gains of being able to transmit a packet over multiple slots without excessive segmentation, we propose a simple bundling of multiple slots by appropriately scaling the TB size so that the entire payload can be encoded as a single TB. That is the UE may be scheduled to transmit a larger TBS (without segmentation) using the REs across multiple slots, as shown in the bottom figure of Error! Reference source not found.. Since more parity bits would be transmitted for a larger TBS, it can be expected that the MCL could be improved with larger coding gain. This option can be applied to both eMBB and VoNR services.

Figure 12 illustrates the gains of letting a TB span multiple slots. The figure considers a UE with a 1 RB, MCS 0 allocation with TB size scaled to span a single slot, two slots, or up to 4 slots. It is seen that when the TB is allowed to span upto 4 slots, up to 2 dB gain is observed at 0.1 BLER.

Figure 12 PUSCH Performance enhancement using TB size scaling

Hence, based on the above discussion, we make the following proposal:

Proposal 5: Consider TBS scaling and optimization across multiple slots for coverage enhancement for eMBB and VoNR in Rel-17.

# Msg3 PUSCH Repetition for Msg3 Coverage Enhancement

In Rel-15/16 4-step RACH procedure, the UE transmits an initial Msg3 based on the resource allocation in RAR. If the initial Msg3 transmission fails, the UE monitors the PDCCH with CRC scrambled by TC-RNTI to obtain the resource allocation for Msg3 retransmission. For both initial Msg3 transmission and retransmission, the UE is only configured to transmit Msg3 without a repetition.

The link budget analysis showed that the coverage of Msg3 in 4-step RACH could be quite limited. For example, in the rural scenarios at 700MHz carrier frequency, the Msg3 has approximately the same coverage as the unicast PUSCH at 100kbps without repetition. The coverage for the unicast PUSCH when the repetition is enabled is expected to be better than that of Msg3. Furthermore, having multiple Msg3 retransmissions would increase the RACH procedure latency. Also, it would require higher PDCCH overhead since the UE would need to acquire DCI with CRC scrambled by RA-RNTI for receiving RAR and DCI with CRC scrambled by TC-RNTI to obtain the grant for initial transmission and retransmission. Hence, Msg3 should be improved for better coverage, lower RACH procedure latency and efficient PDCCH overhead. On approach to improve coverage of Msg3 is to support Msg3 PUSCH repetition which may be enabled for Msg3 retransmissions only.

Proposal 6: Consider Msg3 PUSCH repetition as an enhancement technique to extend coverage of Msg3 in Rel-17.

# Conclusion

This contribution discusses some potential techniques for enhancing coverage in both FR1 and FR2. In particular, the following proposals have been made:

On techniques to reduce MPR for uplink transmissions

Proposal 1: Techniques for UE transmit waveform design that allow further reduction in the MPR values for DFT-S-OFDM and CP-OFDM waveforms should be studied for coverage enhancement.  In particular, consider tone reservation principle for DFT-s-OFDM and CP-OFDM waveforms to further reduce PAPR.

On DMRS enhancements

Proposal 2: Consider DMRS bundling technique for coverage enhancement in NR Rel-17.

Proposal 3: Consider adaptive DMRS configuration for PUSCH to improve both coverage and link efficiency and introduce signaling mechanisms for dynamic DMRS configuration change.

Proposal 4: Additional RS should be studied for improving channel estimation to extend PUSCH coverage.

On PUSCH enhancements for small payload

Proposal 5: Consider TBS scaling and optimization across multiple slots for PUSCH coverage enhancement for eMBB and VoNR.

On Msg3 enhancements

Proposal 6: Consider Msg3 PUSCH repetition as an enhancement technique to extend coverage of Msg3 in Rel-17.

# Reference

- R1-2006818, Baseline FR1 coverage performance, 3GPP TSG RAN1 WG1, Meeting 102-e, August 17th – 28th, 2020.

- R1-2006819, Baseline FR2 coverage performance, 3GPP TSG RAN1 WG1, Meeting 102-e, August 17th – 28th, 2020.

- RP-193240, New SID on NR coverage enhancement, 3GPP TSG RAN Meeting #86, Sitges, Spain, Dec. 9-12, 2019.

- J. Tellado, Multicarrier Modulation With Low PAR: Applications to DSL and Wireless, 2000