3GPP TSG RAN WG1 Meeting #102-e R1-2007170 e-Meeting, August 17th – 28th, 2020 CR-Form-v12.0. CHANGE REQUEST. 38.213; CR; 0122; rev; -; Current version:; 15.10.0. For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests.. Proposed change affects:; UICC apps; ME; X; Radio Access Network; X; Core Network. Title:; CR on determining P0 for a PUSCH retransmission corresponding to a RAR UL grant in TS 38.213. Source to WG:; ZTE. Source to TSG:; R1. Work item code:; NR_newRAT-Core; Date:; 2020-08-24. Category:; F; Release:; Rel-15. Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900.; Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16). Reason for change:; In TS38.213, there is no specific description for PUSCH retransmission corresponding to a RAR UL grant regarding PO_UE_PUSCH. PUSCH retransmission corresponding to the RAR UL grant may have different PO_UE_PUSCH value from initial PUSCH transmission corresponding to the RAR UL grant in RRC CONNECTED mode. If PO_UE_PUSCH value for PUSCH retransmission is lower than for the initial PUSCH transmission, it may impact PUSCH retransmission. For instance, if the difference of two PO_UE_PUSCH values is large, it may break the power continuity for a plurality of PUSCH (re)transmission corresponding to the RAR UL grant and one TPC command may be not enough to compensate the gap. Therefore, we suggest to clarify in the spec that there is the same PO_UE_PUSCH value between the initial PUSCH transmission and the PUSCH retransmission.. -. Summary of change:; Modifiy the j value for PUSCH re-transmission to be the same as that for the corresponding RAR scheduled PUSCH transmission, i.e., j=0.. Consequences if not approved:; This is wrongly configured power setting parameters will let the actual power level of PUSCH re-transmission may be much lower than the power level of initial transmission. It will cause the frequent random access failure in the system.. Clauses affected:; 7.1.1. Y; N. Other specs; X; Other core specifications; TS/TR ... CR .... affected:; X; Test specifications; TS/TR ... CR .... (show related CRs); X; O&M Specifications; TS/TR ... CR .... Other comments:; Isolated impact analysis: The CR is aiming to clarify the UE action for a PUSCH retransmission corresponding to the RAR UL grant when determining PO_UE_PUSCH value. It is expected UEs and networks are implemented with the same understanding and therefore no change is required on UE and network implementation.. This CR's revision history:; This is the first version for this CR..

<Unchanged part omitted>

### 7.1.1 UE behaviour

If a UE transmits a PUSCH on active UL BWP  of carrier  of serving cell  using parameter set configuration with index  and PUSCH power control adjustment state with index , the UE determines the PUSCH transmission power  in PUSCH transmission occasion  as

[dBm]

where,

-	is the UE configured maximum output power defined in [8-1, TS 38.101-1], [8-2, TS38.101-2] and [8-3, TS38.101-3] for carrier  of serving cell  in PUSCH transmission occasion .

-	 is a parameter composed of the sum of a component  and a component  where .

-	If a UE is not provided P0-PUSCH-AlphaSet or for a PUSCH (re)transmission corresponding to a RAR UL grant as described in Clause 8.3, , , and , where the parameter preambleReceivedTargetPower [11, TS 38.321] (for ) and msg3-DeltaPreamble (for ) are provided by higher layers, or  dB if msg3-DeltaPreamble is not provided, for carrier  of serving cell

-	For a PUSCH (re)transmission configured by ConfiguredGrantConfig, ,  is provided by p0-NominalWithoutGrant, or  if p0-NominalWithoutGrant is not provided, and  is provided by p0 obtained from p0-PUSCH-Alpha in ConfiguredGrantConfig that provides an index P0-PUSCH-AlphaSetId to a set of P0-PUSCH-AlphaSet for active UL BWP  of carrier  of serving cell

-	For , a  value, applicable for all , is provided by p0-NominalWithGrant, or  if p0-NominalWithGrant is not provided, for each carrier  of serving cell  and a set of values are provided by a set of p0 in P0-PUSCH-AlphaSet indicated by a respective set of p0-PUSCH-AlphaSetId for active UL BWP  of carrier  of serving cell

-	If the UE is provided by SRI-PUSCH-PowerControl more than one values of p0-PUSCH-AlphaSetId and if DCI format 0_1 includes a SRI field, the UE obtains a mapping from sri-PUSCH-PowerControlId in SRI-PUSCH-PowerControl between a set of values for the SRI field in DCI format 0_1 [5, TS 38.212] and a set of indexes provided by p0-PUSCH-AlphaSetId that map to a set of P0-PUSCH-AlphaSet values. If the PUSCH transmission is scheduled by a DCI format 0_1 that includes a SRI field, the UE determines the value of  from the p0-PUSCH-AlphaSetId value that is mapped to the SRI field value

-	If the PUSCH transmission except for the PUSCH retransmission corresponding to a RAR UL grant is scheduled by a DCI format 0_0 or by a DCI format 0_1 that does not include a SRI field, or if SRI-PUSCHPowerControl is not provided to the UE, , and the UE determines  from the value of the first P0-PUSCH-AlphaSet in p0-AlphaSets

-	For

-	For ,  is a value of msg3-Alpha, when provided; otherwise,

-	For ,  is provided by alpha obtained from p0-PUSCH-Alpha in ConfiguredGrantConfig providing an index P0-PUSCH-AlphaSetId to a set of P0-PUSCH-AlphaSet for active UL BWP  of carrier  of serving cell

-	For , a set of  values are provided by a set of alpha in P0-PUSCH-AlphaSet indicated by a respective set of p0-PUSCH-AlphaSetId for active UL BWP  of carrier  of serving cell

-	If the UE is provided SRI-PUSCH-PowerControl and more than one values of p0-PUSCH-AlphaSetId, and if DCI format 0_1 includes a SRI field, the UE obtains a mapping from sri-PUSCH-PowerControlId in SRI-PUSCH-PowerControl between a set of values for the SRI field in DCI format 0_1 [5, TS 38.212] and a set of indexes provided by p0-PUSCH-AlphaSetId that map to a set of P0-PUSCH-AlphaSet values. If the PUSCH transmission is scheduled by a DCI format 0_1 that includes a SRI field, the UE determines the values of  from the p0-PUSCH-AlphaSetId value that is mapped to the SRI field value

-	If the PUSCH transmission except for the PUSCH retransmission corresponding to a RAR UL grant is scheduled by a DCI format 0_0 or by a DCI format 0_1 that does not include a SRI field, or if SRI-PUSCH-PowerControl is not provided to the UE, , and the UE determines  from the value of the first P0-PUSCH-AlphaSet in p0-AlphaSets

<Unchanged part omitted>