3GPP TSG RAN WG1 Meeting #102-e	R1-2007273

E-Meeting, August 17 – 28, 2020

Agenda Item:	6.2.3

Source:	Moderator (Huawei)

Title:	Feature summary on [102-e-LTE-LTE_DL_MIMO_EE-01]

Document for:	Discussion and Decision

Introduction

Agreements and conclusions in previous meeting for the LTE DL MIMO efficiency enhancements (WI code LTE_DL_MIMO_EE-Core; WID in RP-182901) are summarized in [1].

This documents provides the proposals and summary of discussions of the following second phase email discussion for identified issues of LTE downlink MIMO efficiency enhancement [5].

[102-e-LTE-LTE_DL_MIMO_EE-01] Terminology of additional SRS and trigger type 2 SRS – Yubo (Huawei)

- Issue #1 from FL prep phase summary on LTE DL MIMO efficiency enhancement

- Discussion and agreement by 8/19, TPs by 8/21

Discussion

### Issue 1: Terminology of additional SRS and trigger type 2 SRS.

- As explained in [2][3][4], the term used in 36.211 is additional SRS symbol while in 36.213 it’s trigger type 2. But it is not specified that trigger type 2 SRS corresponds to additional SRS, and it should be clarified that trigger type 0 SRS and trigger type 1 SRS correspond to periodic SRS and aperiodic SRS of basic SRS respectively.

- Proposal: Endorse one of the following text proposal

Text proposal to 36.213 <Unchanged parts are omitted> ## 8.2 UE sounding procedure If the UE is configured with a PUCCH-SCell, the UE shall apply the procedures described in this clause for both primary PUCCH group and secondary PUCCH group unless stated otherwise - When the procedures are applied for the primary PUCCH group, the terms 'secondary cell', 'secondary cells', 'serving cell', and 'serving cells' in this clause refer to secondary cell, secondary cells, serving cell or serving cells belonging to the primary PUCCH group respectively unless stated otherwise. - When the procedures are applied for secondary PUCCH group, the terms 'secondary cell', 'secondary cells', 'serving cell' and 'serving cells' in this clause refer to secondary cell, secondary cells (not including the PUCCH-SCell), serving cell, serving cells belonging to the secondary PUCCH group respectively unless stated otherwise. The term 'primary cell' in this clause refers to the PUCCH-SCell of the secondary PUCCH group. A UE shall transmit Sounding Reference Symbol (SRS) on per serving cell SRS resources based on three trigger types: - trigger type 0: higher layer signalling - trigger type 1: triggering of basic sounding reference signals as defined in subclause 5.5.3 of [3], by DCI formats 0/0A/0B/4/4A/4B/1A/6-0A/6-1A for FDD, TDD, and frame structure type 3 and DCI formats 2B/2C/2D/3B for TDD, and frame structure type 3, and DCI format 7-0A/7-0B/7-1E/7-1F/7-1G for TDD if the UE is configured by higher layers for SRS triggering via DCI format 7-0A and has indicated the capability srs-DCI7-Triggering-FS2 and the UE is configured for SRS triggering with srs-DCI7-TriggeringConfig . - trigger type 2: triggering of additional sounding reference signals as defined in subclause 5.5.3 of [3], by DCI formats 0/4/1A/6-0A/6-1A for FDD and TDD, and DCI formats 2B/2C/2D/3B for TDD, and DCI format 7-0A/7-0B/7-1E/7-1F/7-1G for TDD if the UE is configured by higher layers for SRS triggering via DCI format 7-0A and has indicated the capability srs-DCI7-Triggering-FS2 and the UE is configured for SRS triggering with srs-DCI7-TriggeringConfig. <Unchanged parts are omitted>

Or

Text proposal to 36.211 ---- Unchanged parts are omitted ---- ### 5.5.3 Sounding reference signal Two types of sounding reference signals can be configured: - basic sounding reference signal, supporting periodic or aperiodic transmission - additional sounding reference signal, supporting aperiodic transmission only Basic SRS corresponds to either SRS trigger type 0 or type 1 in 8.2 of [4]. Additional SRS corresponds to SRS trigger type 2 in 8.2 of [4]. ---- Unchanged parts are omitted ----

Or

-------------------------- Start of proposed TP1 for TS 36.211 ------------------------------------ #### 5.5.1.5 Determining virtual cell identity for sequence generation The definition of depends on the type of transmission. Transmissions associated with PUSCH: - if no value for is configured by higher layers or if the PUSCH transmission corresponds to a Random Access Response Grant or a retransmission of the same transport block as part of the contention based random access procedure, - otherwise. Transmissions associated with SPUCCH/PUCCH: - if no value for is configured by higher layers, - otherwise. Basic sSounding reference signals transmissions of types 0 and 1 as defined in 8.2 of [4]: - nIDRS=nIDSRS if the higher-layer parameters nSRS-Identity-Legacy and nSRS-Identity both are configured, where nIDSRS equals the higher-layer parameter nSRS-Identity - nIDRS=NIDcell otherwise. Additional sSounding reference signals transmissions of type 2 as defined in 8.2 of [4]: - nIDRS=NIDcell if no value for nIDSRS is configured by the higher-layer parameter nSRS-Identity - nIDRS=nIDSRS otherwise. ----------------------------------------------- Unchanged text omitted ------------------------------------------- ### 5.5.3 Sounding reference signal Three types of sounding reference signals can be configured as defined in 8.2 of [4]: SRS trigger type 0, SRS trigger type 1, and SRS trigger type 2. Two types of sounding reference signals can be configured: - basic sounding reference signal, supporting periodic or aperiodic transmission - additional sounding reference signal, supporting aperiodic transmission only #### 5.5.3.1 Sequence generation ##### 5.5.3.1.1 Sequence generation for SRS trigger types 0 and 1 basic SRS The sounding reference signal sequence is defined by clause 5.5.1, where is the sequence-group number defined in clause 5.5.1.3, is the base sequence number defined in clause 5.5.1.4, and . The cyclic shift of the sounding reference signal is given as , where is configured separately for periodic and each configuration of aperiodic sounding by the higher-layer parameters cyclicShift and cyclicShift-ap, respectively, for each UE and is the number of antenna ports used for sounding reference signal transmission. The parameter if , otherwise . The parameter is given by the higher layer parameter transmissionCombNum if configured, otherwise . ##### 5.5.3.1.2 Sequence generation for SRS trigger type 2 additional SRS The sounding reference signal rSRS(p)n is defined by clause 5.5.3.1.1 with the following exceptions - nSRScs is given by the higher-layer parameter additionalSRS-cyclicShift - Nap is given by the higher-layer parameter additionalSRS-AntennaPort - KTC is given by the higher-layer parameter additionalSRS-transmissionComb - the function fgh in clause 5.5.1.3 is given by fghns,l=0if group hopping is disabledi=072ic8nsNsymbUL+l+i mod 30if group hopping is enabled where l is the OFDM symbol index within the slot ns and NsymbUL is the number of OFDM symbols per slot - the function v in clause 5.5.1.4 is given by v=cnsNsymbUL+lif group hopping is disabled and sequence hopping is enabled0otherwise #### 5.5.3.2 Mapping to physical resources ##### 5.5.3.2.1 Mapping to physical resources for SRS trigger types 0 and 1 basic SRS The sequence shall be multiplied with the amplitude scaling factor in order to conform to the transmit power specified in clause 5.1.3.1 in 3GPP TS 36.213 [4], and mapped in sequence starting with to resource elements on antenna port according to ----------------------------------------------- Unchanged text omitted ------------------------------------------- ##### 5.5.3.2.2 Mapping to physical resources for SRS trigger type 2 additional SRS An additional SRS of trigger type 2 spans one or more OFDM symbols in the time domain, where - the starting OFDM symbol l0 within the subframe is given by the higher-layer parameter additionalSRS-startPos; - the duration N in number of OFDM symbols, including potential guard symbols, is given by the higher-layer parameter additionalSRS-duration; Mapping to physical resources shall be done according to clause 5.5.3.2.1 with the following exceptions: - frequency hopping between OFDM symbols is supported and if a UE is configured by higher layer parameter additionalSRS-GuardSymbolFH, a guard symbol is added between every frequency hop; - nSRS=lR where l is the index of the OFDM symbol number carrying additional SRS of trigger type 2 within the subframe not counting guard symbol(s), and R∈1,2,3,4,6,7,8,9,12,13 is the repetition factor given by the higher-layer parameter additionalSRS-RepNum; - BSRS is given by the higher-layer parameter additionalSRS-Bandwidth; - bhop is given by the higher-layer parameter additionalSRS-HoppingBandwidth; - NFH is the number of frequency hops for additional SRS of trigger type 2, derived from N=RNFH+NFH-1GFH if antenna switching is not configured for additional SRS of trigger type 2, and from N=RNASNFH+NAS-1GAS+NFH-1GFH if antenna switching is configured for additional SRS of trigger type 2, where R is the repetition factor given by the higher-layer parameter additionalSRS-RepNum, NAS is the number of antenna switches for additional SRS of trigger type 2 defined in 8.2 of [4], GAS∈0, 1 is the guard-symbol configuration for antenna switching given by the higher-layer parameter additionalSRS-GuardSymbolAS, GFH∈0, 1 is the guard symbol configuration for frequency hopping given by the higher-layer parameter additionalSRS-GuardSymbolFH, and N is given by the higher-layer parameter additionalSRS-duration; - nRRC is given by the higher-layer parameter freqDomainPosition-additionalSRS; - Nap is given by the higher-layer parameter additionalSRS-AntennaPort; - nSRScs is given by the higher-layer parameter additionalSRS-cyclicShift; - KTC is given by the higher-layer parameter additionalSRS-transmissionCombNum; - kTC is given by the higher-layer parameter additionalSRS-transmissionComb. ------------------------------------------- End of proposed TP 1 ----------------------------------------------------

Please input your comments to the above options for the alignment of terminologies:

| Companies | Comments |
| --- | --- |
| Qualcomm | We slightly prefer the 2nd TP [3] because: - We prefer not to delete ‘additional SRS’ defined in TS36.211 because TS36.331 refers to ‘additional SRS’ instead of ‘SRS trigger type 2’. - In the 1st TP, ‘trigger type 0’ is not clarified to be the basic SRS with periodic transmission. |
| Nokia, Nokia Shanghai Bell | We agree that alignment of terminology as proposed in the TPs above is needed. Our slight preference is option 2. |
| LGE | Agree with Qualcomm and Nokia. |
| ZTE | Considering the change brought by second TP is minimal, we slightly prefer the second TP. |
| Huawei, HiSilicon | We can be fine with the second TP. |

Summary

To be added.

- References

- R1-1913596, “RAN1 agreements for DL MIMO efficiency enhancements for LTE”, Huawei, Reno, USA, November 2019.

- R1-2005817	Corrections on additional SRS symbols	Huawei, HiSilicon

- R1-2006592	Text proposals on additional SRS symbols	LG Electronics

- R1-2006690	Maintenance on additional SRS symbols	Ericsson

- R1-2007028	Feature summary on LTE DL MIMO efficiency enhancement	Moderator (Huawei)