3GPP TSG RAN WG1 Meeting #102-e	R1-2007421

eMeeting, August 19-28, 2020

Agenda Item:	7.2.4.7

Source:	Moderator (FUTUREWEI)

Title:	FL summary #1 on NR Uu scheduling LTE sidelink

Document for:	Discussion and decision

# Introduction

A WID on NR-V2X was approved at RAN#83, with one of the objectives as follows:

2. Specify support for NR Uu to provide control for LTE sidelink

- Sidelink mode 4 as per the study outcome [RAN2, RAN1]; and

- Sidelink mode 3-like RRC-configured SPS scheduling with either RRC-based activation/deactivation as per the study outcome or DCI-based activation/deactivation [RAN1, RAN2].

  - RAN1 to make a decision on which option is supported until RAN#84.

- 6 companies submitted contributions for this AI ([1]-[6]).

# Issues listed by companies

The list of issues brought by companies is summarized here. The issues are summarized in Table 1, along with the proposals for each of them. Then, in following subsections, each issue is discussed in more detail.

Table 1. Summary of issues.

| Issue | Description | FL Proposal |
| --- | --- | --- |
| Issue 1 | Power control: whether open loop power control should be supported on the sidelink when NR Uu schedules LTE sidelink in the case of a shared carrier | No email discussion |
| Issue 2 | Whether a parameter to indicate LTE carrier frequency is needed in NR SIB 13 | Discuss in RAN2 |

## Issue 1: Power control

This issue is brought in 5 contributions ([1]-[5]). It was discussed at RAN1#100b-e, with no consensus achieved, and further discussed at RAN1#101-e, with no consensus either. The issue is whether on a shared Uu/SL carrier, open loop power control should be applied to protect the eNB/gNB, as it is for LTE. 5 companies are in favor of solving the issue, ([1], [2], [5]) 2 advocate no change ([3], [4]).

Given the extensive amount of discussion on this topic with no agreement reached, and the fact that from contributions submitted at this meeting, nothing indicates any change in position one way or another, this topic should not be discussed any more.

FL’s proposal:

- No discussion on open loop power control for the shared carrier at this meeting

## Issue 2: SL index of DCI field

This issue in brought up in [6], where it is argued that in the IE SL-InterFreqInfoListV2X, it is necessary to indicate the LTE carrier frequency.

Without any judgment on the merit of the proposal, all the discussions in [6] are related to RRC signaling. Besides, the issue is clearly within the expertise of RAN2. Thus, it is proposed not to discuss this issue in RAN1, but to bring the discussion in RAN2.

FL’s proposal:

- No discussion on whether indicating the LTE carrier frequency in needed in NR SIB 13. Companies are encouraged to discuss the issue in RAN2.

# Conclusion

Remaining issues on cross-RAT scheduling were listed and discussed.

# Decisions at previous meetings

## RAN1#96

Agreements:

- Scheduling by gNB using RRC for LTE sidelink scheduled mode is supported from RAN1 perspective under the premise that there is sufficient time for coordination between the NR and LTE modules. No DCI to activate/release

  - RRC message delivers the SPS grant configuration and releases the SPS configuration.

  - Support of this scheduling mode is subject to UE capability (may or may not have capability for both LTE & NR)

  - Note: some specification LTE change is needed to support the reception of a grant through RRC

    - RRC message contains mode 3 grant content and timing

    - Up to the Editor to capture it as mode 3 or new LTE sidelink mode

  - No intention to have additional NR & LTE specification change (other than those described above) for this function in Rel-16

- RAN1 studied the feasibility of SPS scheduling by gNB for LTE sidelink with DCI activation/release, but there is no consensus to support it

## RAN1#96bis

Agreements:

Regarding RRC-based versus DCI-based activation/release of LTE sidelink SPS, RAN1 agrees to make the choice on the basis of at least:

- Spec impact

- Flexibility

- Performance, including latency

- Implementation complexity

- Timing of the activation/deactivation

## RAN2#105bis

Agreements:

For scheduling LTE SL UEs, the gNB uses RRC messages to deliver the SPS grant configuration.

Separate system information block should be designed to support LTE resource pool configuration via NR Uu. It will be defined as a container (OCTET STRING) and actual information follows what defined in LTE RRC.

gNB should be able to configure the LTE V2X mode 4 sidelink resource pool via dedicated signalling. In addition, gNB should be able to configure mode3 SL resources via dedicated signaling. It will be defined as a container (OCTET STRING) and actual information follows what defined in LTE RRC.

## RAN1#97

Agreements:

- DCI-based activation/deactivation is supported

  - Support of LTE PC5 scheduling by NR Uu (mode 3-like ) is based on UE capability

  - NR DCI provides the fields of DCI 5A in LTE-V that are related to SPS scheduling

  - The size of DCI for activation/deactivation is one of the DCI size(s) that will be defined for NR Uu scheduling NR V2V

    - FFS whether the DCI format is the same as one of the DCI formats that will be defined for NR Uu scheduling NR V2V

  - Activation/deactivation applies to the first LTE subframe after Z+X ms after receiving the DCI

    - Z is the same timing offset in current LTE V2X specs

    - X>0. FFS value(s) of X, and if one or multiple values of X are possible

## RAN1#98

Agreements:

- A new RNTI is introduced to scramble the NR DCI used for scheduling LTE PC5.

- X is dynamically indicated using a field in the DCI

  - FFS whether the DCI field provides an index to a table or the value of X

  - The minimum value of X is subject to UE capability

    - UE reports a single value subject to UE capability

## RAN1#98b

Agreements:

- The NR DCI field to indicate X provides an index to a table of values

  - The table of values is configurable, and has 8 values

  - The size of the DCI field is fixed at 3 bits

## RAN1#99

Agreements:

- Use a separate PDCCH monitoring configuration (as configured in Rel-15) for NR DCI scheduling LTE SL

  - The per-CC and across-CC blind decoding budget and the maximum number of non-overlapped CCEs for channel estimation are not increased.

  - The per-CC and across-CC maximum number of search spaces is not increased.

  - The per-CC and across-CC maximum number of CORESETs is not increased

    - When in the same slot, there is both PDCCH monitoring for Uu SL and PDCCH monitoring for SL for the same CC, the search space(s) for LTE SL is configured to be the same or a subset of those for Uu for the same CC or vice versa

Agreements:

- The minimum value of X signalled in the UE capability is one of the values (excluding spare values) that can be signalled in DCI 3_1

Agreements:

The supported values of X signaled in the DCI are:

- 0.75ms, 1ms, [1.25ms], [1.5ms], 2ms, 4ms, 5ms, 8ms, 10ms, 20 ms

  - Additional value(s) can be discussed during the Feb. meeting

- Spare values are reserved for future deployments

## RAN1#100-e

Agreements

- For NR Uu scheduling LTE sidelink, the subframe of the first sidelink transmission is the first SL subframe of the corresponding resource pool that starts not earlier than , where TDCI is the start timing of the slot carrying DCI format 3_1, NTA and Tc are defined in TS 38.211, X is the value indicated by Timing offset field in DCI format 3_1, m is the value indicated by SL index in DCI format 3_1 if SL index is present, otherwise m=0.

Agreements

- If UE is configured to monitor DCI 3_0, the sizes of DCI 3_1 and DCI 3_0 are aligned by zero padding.

- If UE is not configured to monitor DCI 3_0, the mechanism for size alignment between DCI 3_0 and Uu DCI is reused for size alignment between DCI 3_1 and Uu DCI

Agreements:

- The set of possible values for X is: 0ms, 0.25ms, 0.5ms, 0.625ms, 0.75ms, 1ms, 1.25ms, 1.5ms,1.75ms, 2ms, 2.5ms, 3ms, 4ms, 5ms, 6ms, 8ms, 10ms, 20 ms

## RAN1#101-e

Conclusion:

- SL index is present in DCI 3_1 only for cases with TDD configurations 0-6

- The fields SL SPS configuration index and activation/release in DCI 3_1 are always present

- Capture in the LTE 36.212 specification that for the SPS activation/release field, in DCI 5A ‘0’ means release and ‘1’ means activation

# References

| No. | Tdoc | Type | Source | Title | Status |
| --- | --- | --- | --- | --- | --- |
| 1 | R1-2005322 | discussion | ZTE, sanechips | Remaining issues on NR Uu control LTE sidelink | present |
| 2 | R1-2005803 | discussion | Huawei, HiSilicon | Remaining details of NR Uu control for LTE sidelink | present |
| 3 | R1-2006007 | discussion | OPPO | Remaining open issues in NR Uu controlling LTE sidelink | present |
| 4 | R1-2006106 | discussion | Samsung | On NR Uu controling for LTE sidelink | present |
| 5 | R1-2006441 | discussion | Ericsson | Open issue for NR Uu controlling LTE sidelink transmissions | present |
| 6 | R1-2006562 | discussion | Sharp | Remaining issues on support of NR Uu controlling LTE sidelink | present |