3GPP TSG RAN WG1#102-e		R1-200xxxx

e-Meeting, August 17th – 28th, 2020

Title:	[DRAFT] LS reply on NR SRS carrier switching

Response to:	R1-2005209 / R2-2006362 LS on NR SRS carrier switching

Release:	Release 15

Work Item:	NR_newRAT-Core

Source:	Qualcomm Incorporated [RAN1]

To:	RAN2

Cc:

Contact Person:

#### Name: 	Alberto Rico

#### E-mail Address:	albertor@qti.qualcomm.com

1. Overall Description:

RAN1 discussed the LS R1-2005209 / R2-2006362, where RAN2 requested feedback on whether the following two cases are allowed:

[…] for typeA RAN2 didn’t achieve consensus on whether the following 2 cases on NR SRS carrier switching are allowed: - Case 1: SRS carrier switching to a SUL without PUCCH/PUSCH while PUCCH/PUSCH is configured in NUL; - Case 2: SRS carrier switching to both NUL and SUL, neither of which are configured with PUCCH/PUSCH.

RAN1 concluded that neither of the cases are allowed from RAN1 specification point of view for the following reasons:

- For Case 1, SRS carrier switching is only allowed to carriers that have both uplink and downlink slots. Therefore, SRS carrier switching to SUL carrier is not supported.

- For Case 2, configuring PUCCH/PUSCH in neither NUL nor SUL is not supported.

2. Actions:

To RAN2:

ACTION: 	RAN1 respectfully requests RAN2 to take the above into account.

3. Date of Next TSG-RAN WG1 Meetings:

TSG-RAN WG1 Meeting #103-e	2020-10-26 / 2020-11-13