3GPP TSG RAN WG1 Meeting #102-e	R1-2007300

# E-meeting, August 17–28, 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.213 | CR | 1354 | rev | - | Current version: | V16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarifications for PUR Repetition Adjustment |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Moderator (Sierra Wireless) |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_eMTC5 -Core |  |  |  |  |  | Date: |  |  | 2020-08-26 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | For PUR, the currently specified procedure to update the PUR repetitions via an L1 ACK/Fallback DCI does not support updating of the RRC PUR configuration and does not match RAN2’s requirements. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Implemented RAN2’s requirements: - The adjustment on the PUSCH repetition number provided with L1 ACK / fallback indicator updates the repetition number configuration in PUR configuration in RRC layer. - The PHY layer provides a 3-bit repetition adjustment index to higher layers so that the value can be stored in the PUR configuration in RRC. - The format of the 3-bit information is same as RRC parameter numRepetitions-r16 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The L1 ACK/Fall DCI repetition adjustment does not upate the RRC PUR configuration and the RAN1 and RAN2 specification would not be aligned. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.0 and 9.1.5.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## 8.0	UE procedure for transmitting the physical uplink shared channel

<Unchanged parts are omitted>

For a PUSCH transmission using preconfigured uplink resource, the UE shall use the repetition number configured by higher layers.

<Unchanged parts are omitted>

#### 9.1.5.3	Preconfigured Uplink Resource ACK/fallback procedure

If a UE has initiated a PUSCH transmission using preconfigured uplink resource on a given serving cell, and upon detection of a MPDCCH with DCI format 6-0A/6-0B with CRC scrambled by PUR C-RNTI intended for the UE within the PUR search space window as defined in Subclause 9.1.5, and the corresponding DCI is for PUR ACK/fallback indication (as defined in [4]):

-	the UE shall deliver the PUR ACK/fallback indication, as signalled on the MPDCCH, to the higher layers, and

-	the UE shall deliver to higher layers a 3-bit PUSCH repetition adjustment according to Table 8-2b for CEModeA or Table 8-2c for CEModeB as signalled on the MPDCCH, where a bit with a value of 0 shall be prepended to the DCI field if the DCI field has a size of 2 bits.

<Unchanged parts are omitted>