3GPP TSG-RAN WG1 Meeting #102-e	R1- 20xxxxx

e-Meeting, August 17th – 28th, 2020

Agenda Item:	6.2.1

Source:	Moderator (Ericsson)

Title:	FL summary #1 for Resource reservation field in SPS release DCI in
LTE-MTC and NB-IoT

Document for:	Discussion, Decision

Introduction

This document provides a summary of the following RAN1 email discussion.

- [102-e-LTE-eMTC5-05] Resource reservation field in SPS release DCI – Johan (Ericsson) - Resource reservation field in SPS release DCI (R1-2005503 proposal 1) - Common email discussion for LTE-MTC (R1-2005503) and NB-IoT (R1-2005504) - Discussions/Agreements by 8/19, TPs by 8/24

Issue #1: LTE-MTC case

Contribution [1] proposes that if the Rel-16 LTE-MTC resource reservation feature is configured, then the ‘Resource reservation’ field in a DCI for SPS release is set to a default value for SPS release MPDCCH validation. The following TP for 36.213 is provided.

## 9.2 PDCCH/EPDCCH/MPDCCH/SPDCCH validation for semi-persistent scheduling <Unchanged parts are omitted> Table 9.2-1C: Special fields for Semi-Persistent Scheduling Release MPDCCH Validation DCI format 6-0A; DCI format 6-1A. HARQ process number; set to '000'; FDD: set to '000' TDD: set to '0000. Redundancy version; set to '00'; set to '00'. Repetition number; set to '00'; set to '00'. Modulation and coding scheme; set to '1111'; set to '1111'. TPC command for scheduled PUSCH; set to '00'; N/A. Resource block assignment; Set to all '1's; Set to all '1's. Resource reservation; Set to ‘1’ if present; Set to ‘1’ if present. <Unchanged parts are omitted>

- Companies are invited to provide their comments on the text proposal.

| Company | Comments |
| --- | --- |
| Qualcomm | We do not think this is essential – there are many other fields (including optional fields) in 6-0A and 6-1A that are just ignored for validation. Actually, this would be the first optional field in DCI used for validation. Just checking 6-0A, the following fields are ignored: - SRS request - Modulation order override. -DAI - UL index Given that this is a single bit, the gain is very small in false alarm. Having said this, if there is a clear majority that want to add this bit to validation, we will not object. |
| Lenovo&MotoM | Not an essential issue. Hope to keep the legacy validation |
| Nokia | We do not see this as being essential, but we will not oppose if there is a clear majority. |
| Ericsson | It does not seem like an essential correction. |
| ZTE | We respect majority view if companies think it is not an essential issue. Then, the Resource reservation field in DCI for SPS release is ignored by the UE. |
| LG | Not essential. The performance improvement would be marginal. |

Issue #2: NB-IoT case

Contribution [2] proposes that if the Rel-16 NB-IoT resource reservation feature is configured, then the ‘Resource reservation’ field in a DCI for SPS release is set to a default value for SPS release NPDCCH validation. The following TP for 36.213 is provided.

### 16.6.3 NPDCCH validation for semi-persistent scheduling <Unchanged parts are omitted> A UE shall validate a Semi-Persistent Scheduling assignment NPDCCH only if all the following conditions are met: - the CRC parity bits obtained for the NPDCCH payload are scrambled with the Semi-Persistent Scheduling C-RNTI - the new data indicator field is set to '0'. Validation is achieved if all the fields for the used DCI format N0 are set according to Table 16.6.3-1 or Table 16.6.3-2. If validation is achieved, the UE shall consider the received DCI information accordingly as a valid semi-persistent activation or release. If validation is not achieved, the received DCI format shall be considered by the UE as having been received with a non-matching CRC. Table 16.6.3-1: Special fields for Semi-Persistent Scheduling Activation NPDCCH Validation DCI format N0. HARQ process number (present if UE is configured with 2 uplink HARQ processes); set to '0'. Redundancy version; set to '0'. Modulation and coding scheme; set to '0000'. Resource assignment; set to '000'. Table 16.6.3-2: Special fields for Semi-Persistent Scheduling Release NPDCCH Validation DCI format N0. HARQ process number (present if UE is configured with 2 uplink HARQ processes); set to '0'. Redundancy version; set to '0'. Repetition number; set to '000'. Modulation and coding scheme; set to '1111'. Subcarrier indication; Set to all '1's. Resource reservation (present if UE is configured with higher layer parameter ul-ResourceReservationConfig); Set to ‘1’. <Unchanged parts are omitted>

- Companies are invited to provide their comments on the text proposal.

| Company | Comments |
| --- | --- |
| Qualcomm | Similar view as issue #1. |
| Nokia | We do not see this as being essential. |
| Ericsson | It does not seem like an essential correction. |
| ZTE | We respect majority view if companies think it is not an essential issue. Then, the Resource reservation field in DCI for SPS release is ignored by the UE. |
| LG | Not essential. The performance improvement would be marginal. |

References

- R1-2005503, “Clarification on Resource reservation filed in LTE-MTC SPS Release DCI”, ZTE

- R1-2005504, “Clarification on Resource reservation filed in NB-IoT SPS Release DCI”, ZTE