3GPP TSG RAN WG1 Meeting #102-e	R1-200xxxx

e-Meeting, August 17th – 28th, 2020

Source: 	Moderator (ZTE)

Title: 	Email Discussion Summary of [102-e-NR-7.1CRs-01]

Agenda item:	7.1

Document for:	Discussion/Decision

# Introduction

During RAN1#102-e, six contributions were submitted to discuss and clarify the potential ambiguity issue for Msg3 PUSCH retransmission power control [1-6]. During the preparation phase, companies agreed to discuss this issue in RAN1#102-e meeting.

- [102-e-NR-7.1CRs-01] Determination of P0 for a Msg3 PUSCH retransmission by 8/24 – Bo (ZTE)  For Rel-15, Issue#1 in R1-2006958  Discussion/Agreements by 8/19, TPs by 8/24

This summary is trying to collect/summarize companies’ input and draw potential TP based on companies’ input.

# Discussion

## Background introduction

In LTE, Msg3 PUSCH first transmission and its retransmission have the same PO_UE_PUSCH. But in NR, there is no specific description for Msg3 PUSCH retransmission regarding PO_UE_PUSCH, as follows.

- is a parameter composed of the sum of a component and a component where . - If a UE is not provided P0-PUSCH-AlphaSet or for a PUSCH transmission scheduled by a RAR UL grant as described in Clause 8.3, , , and , where the parameter preambleReceivedTargetPower [11, TS 38.321] (for ) and msg3-DeltaPreamble (for ) are provided by higher layers, or dB if msg3-DeltaPreamble is not provided, for carrier of serving cell - For a PUSCH (re)transmission configured by ConfiguredGrantConfig, , is provided by p0-NominalWithoutGrant, or if p0-NominalWithoutGrant is not provided, and is provided by p0 obtained from p0-PUSCH-Alpha in ConfiguredGrantConfig that provides an index P0-PUSCH-AlphaSetId to a set of P0-PUSCH-AlphaSet for active UL BWP of carrier of serving cell - For , a value, applicable for all , is provided by p0-NominalWithGrant, or if p0-NominalWithGrant is not provided, for each carrier of serving cell and a set of values are provided by a set of p0 in P0-PUSCH-AlphaSet indicated by a respective set of p0-PUSCH-AlphaSetId for active UL BWP of carrier of serving cell - If the UE is provided by SRI-PUSCH-PowerControl more than one values of p0-PUSCH-AlphaSetId and if DCI format 0_1 includes a SRI field, the UE obtains a mapping from sri-PUSCH-PowerControlId in SRI-PUSCH-PowerControl between a set of values for the SRI field in DCI format 0_1 [5, TS 38.212] and a set of indexes provided by p0-PUSCH-AlphaSetId that map to a set of P0-PUSCH-AlphaSet values. If the PUSCH transmission is scheduled by a DCI format 0_1 that includes a SRI field, the UE determines the value of from the p0-PUSCH-AlphaSetId value that is mapped to the SRI field value - If the PUSCH transmission is scheduled by a DCI format 0_0 or by a DCI format 0_1 that does not include a SRI field, or if SRI-PUSCHPowerControl is not provided to the UE, , and the UE determines from the value of the first P0-PUSCH-AlphaSet in p0-AlphaSets - For - For , is a value of msg3-Alpha, when provided; otherwise, - For , is provided by alpha obtained from p0-PUSCH-Alpha in ConfiguredGrantConfig providing an index P0-PUSCH-AlphaSetId to a set of P0-PUSCH-AlphaSet for active UL BWP of carrier of serving cell - For , a set of values are provided by a set of alpha in P0-PUSCH-AlphaSet indicated by a respective set of p0-PUSCH-AlphaSetId for active UL BWP of carrier of serving cell - If the UE is provided SRI-PUSCH-PowerControl and more than one values of p0-PUSCH-AlphaSetId, and if DCI format 0_1 includes a SRI field, the UE obtains a mapping from sri-PUSCH-PowerControlId in SRI-PUSCH-PowerControl between a set of values for the SRI field in DCI format 0_1 [5, TS 38.212] and a set of indexes provided by p0-PUSCH-AlphaSetId that map to a set of P0-PUSCH-AlphaSet values. If the PUSCH transmission is scheduled by a DCI format 0_1 that includes a SRI field, the UE determines the values of from the p0-PUSCH-AlphaSetId value that is mapped to the SRI field value - If the PUSCH transmission is scheduled by a DCI format 0_0 or by a DCI format 0_1 that does not include a SRI field, or if SRI-PUSCH-PowerControl is not provided to the UE, , and the UE determines from the value of the first P0-PUSCH-AlphaSet in p0-AlphaSets

According to the current NR specification, Msg3 PUSCH first transmission and its retransmission have the same PO_UE_PUSCH before RRC CONNECTED mode. However, if Msg3 PUSCH retransmission occurs in RRC CONNECTED mode, when there is a P0-PUSCH-AlphaSet configured, the first P0-PUSCH-AlphaSet in p0-AlphaSets should be considered for the Msg3 PUSCH retransmission. That means that Msg3 PUSCH retransmission may have different value from Msg3 PUSCH first transmission in RRC CONNECTED mode.

It seems no specific benefit to have different PO_UE_PUSCH values for Msg3 PUSCH retransmission and Msg3 PUSCH first transmission. If PO_UE_PUSCH value for Msg3 PUSCH retransmission is lower than for Msg3 PUSCH first transmission, it may impact the performance of Msg3 PUSCH retransmission. Furthermore, if the difference of two PO_UE_PUSCH values is large, it may break the power continuity for a plurality of Msg3 PUSCH transmissions and one TPC command may be not enough to compensate the gap.

## Companies’ input

According to the input from contributions [1-6], unifying PO_UE_PUSCH value for Msg3 PUSCH retransmission and Msg3 PUSCH first transmission is recommended. In technical, this is also aligned with the already UE implementation. The potential TP for Rel-15 is provided as follows. It is noted that the terminology of “Msg3 PUSCH retransmission” has been used in Section 8.3	PUSCH scheduled by RAR UL grant in TS 38.213.

Proposed TP: {38.213: 7.1.1	UE behaviour}

### 7.1.1 UE behaviour If a UE transmits a PUSCH on active UL BWP of carrier of serving cell using parameter set configuration with index and PUSCH power control adjustment state with index , the UE determines the PUSCH transmission power in PUSCH transmission occasion as [dBm] where, - is the UE configured maximum output power defined in [8-1, TS 38.101-1], [8-2, TS38.101-2] and [8-3, TS38.101-3] for carrier of serving cell in PUSCH transmission occasion . - is a parameter composed of the sum of a component and a component where . - If a UE is not provided P0-PUSCH-AlphaSet or for a PUSCH transmission scheduled by a RAR UL grant as described in Clause 8.3 or for a Msg3 PUSCH retransmission, , , and , where the parameter preambleReceivedTargetPower [11, TS 38.321] (for ) and msg3-DeltaPreamble (for ) are provided by higher layers, or dB if msg3-DeltaPreamble is not provided, for carrier of serving cell - For a PUSCH (re)transmission configured by ConfiguredGrantConfig, , is provided by p0-NominalWithoutGrant, or if p0-NominalWithoutGrant is not provided, and is provided by p0 obtained from p0-PUSCH-Alpha in ConfiguredGrantConfig that provides an index P0-PUSCH-AlphaSetId to a set of P0-PUSCH-AlphaSet for active UL BWP of carrier of serving cell - For , a value, applicable for all , is provided by p0-NominalWithGrant, or if p0-NominalWithGrant is not provided, for each carrier of serving cell and a set of values are provided by a set of p0 in P0-PUSCH-AlphaSet indicated by a respective set of p0-PUSCH-AlphaSetId for active UL BWP of carrier of serving cell - If the UE is provided by SRI-PUSCH-PowerControl more than one values of p0-PUSCH-AlphaSetId and if DCI format 0_1 includes a SRI field, the UE obtains a mapping from sri-PUSCH-PowerControlId in SRI-PUSCH-PowerControl between a set of values for the SRI field in DCI format 0_1 [5, TS 38.212] and a set of indexes provided by p0-PUSCH-AlphaSetId that map to a set of P0-PUSCH-AlphaSet values. If the PUSCH transmission is scheduled by a DCI format 0_1 that includes a SRI field, the UE determines the value of from the p0-PUSCH-AlphaSetId value that is mapped to the SRI field value - If the PUSCH transmission except for the Msg3 PUSCH retransmission is scheduled by a DCI format 0_0 or by a DCI format 0_1 that does not include a SRI field, or if SRI-PUSCHPowerControl is not provided to the UE, , and the UE determines from the value of the first P0-PUSCH-AlphaSet in p0-AlphaSets - For - For , is a value of msg3-Alpha, when provided; otherwise, - For , is provided by alpha obtained from p0-PUSCH-Alpha in ConfiguredGrantConfig providing an index P0-PUSCH-AlphaSetId to a set of P0-PUSCH-AlphaSet for active UL BWP of carrier of serving cell - For , a set of values are provided by a set of alpha in P0-PUSCH-AlphaSet indicated by a respective set of p0-PUSCH-AlphaSetId for active UL BWP of carrier of serving cell - If the UE is provided SRI-PUSCH-PowerControl and more than one values of p0-PUSCH-AlphaSetId, and if DCI format 0_1 includes a SRI field, the UE obtains a mapping from sri-PUSCH-PowerControlId in SRI-PUSCH-PowerControl between a set of values for the SRI field in DCI format 0_1 [5, TS 38.212] and a set of indexes provided by p0-PUSCH-AlphaSetId that map to a set of P0-PUSCH-AlphaSet values. If the PUSCH transmission is scheduled by a DCI format 0_1 that includes a SRI field, the UE determines the values of from the p0-PUSCH-AlphaSetId value that is mapped to the SRI field value - If the PUSCH transmission except for the Msg3 PUSCH retransmission is scheduled by a DCI format 0_0 or by a DCI format 0_1 that does not include a SRI field, or if SRI-PUSCH-PowerControl is not provided to the UE, , and the UE determines from the value of the first P0-PUSCH-AlphaSet in p0-AlphaSets

Please provide company’s view in the table below.

- Note that the discussion for corresponding TP for Rel-16 will be made after this Rel-15 TP is stable.

| - Company | - Comment |
| --- | --- |
| - Samsung | - We are supportive to make the change, however, we have identified only mentioned the msg.3 is not enough, since the issue exists for both msg.3 (in contention based RACH) and the PUSCH retransmission (in contention free RACH), that’s why in our proposed TP, we used the PUSCH (re-)transmission corresponding to the RAR UL grant, which is also the language used in LTE spec, we think it’s accurate enough. - Thus, we suggest for the TP, we should use “the PUSCH (re-)transmission corresponding to the RAR UL grant” to cover both CFRA and CBRA. |
| - MediaTek | - We also think the change is needed. Otherwise, the performance of PUSCH retransmission for RAR UL grant may be degraded. - We agree that the TP proposed by Samsung can cover both CBRA & CFRA. So, we are fine with Samsung’s proposal. |
| - Huawei, HiSilicon | - Samsung’s wording is better. Therefore, in order to get a full picture of compact changes, we suggest the changes, - Change1: “If a UE is not provided P0-PUSCH-AlphaSet or for a PUSCH (re-)transmission corresponding to a RAR UL grant as described in Clause 8.3, ,” - Change2&3: “If the PUSCH transmission except for the PUSCH retransmission corresponding to a RAR UL grant is scheduled by a DCI format 0_0 or by a DCI format 0_1 that does not include a SRI field,” |
| - Nokia/NSB | - We support the change. We slightly prefer Huawei’s words. |

# Summary

The following potential TP is updated based on the companies’ input.

Draft TP

# Reference

[1] R1-2005446, Discussion on determining P0 for a Msg3 PUSCH retransmission, ZTE

[2] R1-2005447, Draft CR on determining P0 for a Msg3 PUSCH retransmission in TS 38.213, ZTE

[3] R1-2005630, Draft CR on correction for Msg3 PUSCH retransmission power control, MediaTek Inc.

[4] R1-2006085, 38.213 DRAFT CR (Rel-15, F) on msg3 power determination, Samsung

[5] R1-2006086, 38.213 DRAFT CR (Rel-16, A) on msg3 power determination, Samsung

[6] R1-2006087, Discussion on msg3 power determination, Samsung