3GPP TSG RAN WG1 #102-e	                                                                 R1-200XXXX

# e-Meeting, August 17th – 28th, 2020

Agenda item:	7.1

Source:	Moderator (Nokia)

Title:	Summary for [102-e-NR-7.1CRs-18] “Clarification on which UE capability component indicates the number of supported simultaneous CSI calculations NCPU” (Issue #16)

Document for:	Discussion and Decision

# Introduction

This document is created to facilitate the email discussion [102-e-NR-7.1CRs-18] “Clarification on which UE capability component indicates the number of supported simultaneous CSI calculations N_CPU”. This thread is triggered by Issue #16 of [1] and originates from a draft CR to TS38.214 in [2].

| Issue# | Tdoc# | Source | Issue description |
| --- | --- | --- | --- |
| 16 | R1-2006851 | Nokia, NSB | Clarification on which UE capability component indicates the number of supported simultaneous CSI calculations N_CPU. Carry over from previous meeting. |

# Company views

Please provide company’s view in the table below:

| Company | View |
| --- | --- |
| Nokia | In subclause 5.2.1.6 of TS38.214, the parameter NCPU is defined as the number of simultaneous CSI calculations supported by a UE. The text specifies that this parameter is indicated by the UE, but it is not clear which UE capability component indicates it and that the indication is given per-CC as well as across all active CCs. Without this clarification, a UE may report the maximum supported number of simultaneous CSI reports in simultaneousCSI-ReportsPerCC and simultaneousCSI-ReportsAllCC, instead of the maximum number of CPUs, which creates an underreporting problem because one CSI report can occupy multiple CPUs. |

# Conclusion

To be added after the discussion.

# Reference

- R1-2006958 RAN1#102-e preparation phase on NR Rel-15 CRs, Ad-hoc chair (Samsung)

- R1-2006851 38.214CRdraft (Rel-15, F) Clarification on which UE capability component indicates the number of supported simultaneous CSI calculations NCPU, Nokia, Nokia Shanghai Bell