3GPP TSG-RAN WG1 Meeting #102-e											R1-200xxxx

e-Meeting, August 17th – 28th, 2020

Agenda Item:	7.2.1

Source:			Moderator (ZTE)

Title:			Email discussion #1 for 2-step RACH

Document for:	Discussion

# Introduction

This document is intended to address the following remaining issues by email discussion.

[102-e-NR-2step-RACH-01] Email discussion/approval of potential CR(s) addressing issues #1, #3, #7.1 as in the summary:

- Alignment on RRC parameters

- Capture the condition in the previous agreement for subset RO sharing

- Editorial issues related to the TDRA

By 8/21 – Li (ZTE)

# Alignment on RRC parameters (issue #1)

There are multiple contributions proposed to align the RRC parameter names for 2-step RACH between the RAN1 specs and RRC spec, including the changes on 38.211 (R1-2006284, R1-2006407 and R1-2006609), 38.213 (R1-2005664 and R1-2006284). The proposed TPs are merged into proposal 1 for 38.211 and proposal 2 for 38.213 as follows.

Proposal 1:

- Adopt the TP#1 in 38.211, to align the RRC parameters names with RRC specs.

Reasons for change To align the RRC parameter names for 2-step RACH between the RAN1 specs and RRC spec Summary of changes Implement the above update Specs/Sections impacted TS 38.211, Section 5.3.2, 6.3.3.1, 6.3.3.2 -------------------------Text proposal #1 starts for TS 38.211 ---------------------------- 5.3.2 OFDM baseband signal generation for PRACH The time-continuous signal on antenna port p for PRACH is defined by sl(p,μ)t=k=0LRA-1ak(p,RA)ej2πk+Kk1+kΔfRAt-NCP,lRATc-tstartRA K=ΔfΔfRA k1=k0μ+NBWP,istart-Ngridstart,μNscRB+nRAstartNscRB+nRANRBRANscRB-Ngridsize,μNscRB2 k0μ=Ngridstart,μ+Ngridsize,μ2NscRB-Ngridstart,μ0+Ngridsize,μ02NscRB2μ0-μ where and - is given by clause 6.3.3; - is the subcarrier spacing of the initial uplink bandwidth part during initial access. Otherwise, is the subcarrier spacing of the active uplink bandwidth part; - μ0 is the largest μ value among the subcarrier spacing configurations by the higher-layer parameter scs-SpecificCarrierList; - is the lowest numbered resource block of the initial uplink bandwidth part and is derived by the higher-layer parameter initialUplinkBWP during initial access. Otherwise, is the lowest numbered resource block of the active uplink bandwidth part and is derived by the higher-layer parameter BWP-Uplink; - nRAstart is the frequency offset of the lowest PRACH transmission occasion in frequency domain with respect to physical resource block 0 of the active uplink bandwidth part. The quantity nRAstart is given by the higher-layer parameter msgA-RO-FrequencyStart-r16 if configured and a type-2 random-access procedure is initiated as described in clause 8.1 of [5, TS 38.213], otherwise by msg1-FrequencyStart as described in clause 8.1 of [5 TS 38.213]; <Unchanged Text Omitted> #### 6.3.3.1 Sequence generation <Unchanged Text Omitted> The cyclic shift is given by where is given by Tables 6.3.3.1-5 to 6.3.3.1-7, the higher-layer parameter restrictedSetConfig determines the type of restricted sets (unrestricted, restricted type A, restricted type B), or the higher-layer parameter msgA-RestrictedSetConfig-r16, if provided, determines the type of restricted sets (unrestricted, restricted type A, restricted type B) if a type-2 random access procedure is initiated as described in clause 8.1 of [TS 38.213], and Tables 6.3.3.1-1 and 6.3.3.1-2 indicate the type of restricted sets supported for the different preamble formats. <Unchanged Text Omitted> #### 6.3.3.2 Mapping to physical resources The preamble sequence shall be mapped to physical resources according to where is an amplitude scaling factor in order to conform to the transmit power specified in [5, TS38.213], and is the antenna port. Baseband signal generation shall be done according to clause 5.3 using the parameters in Table 6.3.3.1-1 or Table 6.3.3.1-2 with given by Table 6.3.3.2-1. Random access preambles can only be transmitted in the time resources obtained from Tables 6.3.3.2-2 to 6.3.3.2-4 and depends on FR1 or FR2 and the spectrum type as defined in [8, TS38.104]. The PRACH configuration index in Tables 6.3.3.2-2 to 6.3.3.2-4 is - for Table 6.3.3.2-3 given by the higher-layer parameter prach-ConfigurationIndex-v1610 if configured, otherwise by the higher-layer parameter prach-ConfigurationIndex, or by msgA-PRACH-ConfigurationIndex-r16 if configured; and <Unchanged Text Omitted> Random access preambles can only be transmitted in the frequency resources given by either the higher-layer parameter msg1-FrequencyStart or msgA-RO-FrequencyStart-r16 if configured as described in clause 8.1 of [5 TS 38.213]. The PRACH frequency resources nRA∈0,1,…,M-1, where M equals the higher-layer parameter msg1-FDM or msgA-RO-FDM if configured, are numbered in increasing order within the initial uplink bandwidth part during initial access, starting from the lowest frequency. Otherwise, nRA are numbered in increasing order within the active uplink bandwidth part, starting from the lowest frequency. <Unchanged Text Omitted> ------------------------- Text proposal #1 ends for TS 38.211 -------------------------------

Any comments to Proposal 1?

| Company | Comments |
| --- | --- |
| Nokia | We agree to the intention of the proposed changes. In our opinion, the TP in 6284 is simpler, and could be used as reference for updating. Suggestion for update: “the higher-layer parameter restrictedSetConfig for Type-1 random access procedure and the higher-layer parameter msgA-RestrictedSetConfig, if configured, for Type-2 random access procedure determine…”. In this way, the text becomes simpler and more compact. One thing to note is that when referencing higher layer parameters in RAN1 specifications, we should not have the release tag from the ASN.1 appended. That is, msgA-RestrictedSetConfig-r16 should rather be msgA-RestrictedSetConfig. This comment applies to all changes in the TP. |

Proposal 2:

- Adopt the TP#2 in 38.213, to align the RRC parameters names with RRC specs.

Reasons for change To align the RRC parameter names for 2-step RACH between the RAN1 specs and RRC spec Summary of changes Implement the above update Specs/Sections impacted TS 38.213, Section 7.1.1, 8.1 -------------------------Text proposal #2 starts for TS 38.213 ------------------------------ ### 7.1.1 UE behavior <Unchanged Text Omitted> - If a UE established dedicated RRC connection using a Type-2 random access procedure, as described in Clause 8, and is not provided P0-PUSCH-AlphaSet, or for a PUSCH transmission for Type-2 random access procedure as described in Clause 8.1A, j=0, PO_UE_PUSCH,b,f,c(0)=0, and PO_NOMINAL_PUSCH,f,c(0)=PO_PRE+ΔMsgA_PUSCH, where PO_PRE is provided by msgA-preambleReceivedTargetPower, or by preambleReceivedTargetPower if msgA-preambleReceivedTargetPoweris not provided, and ΔMsgA_PUSCH is provided by msgADeltaPreamble, or ΔMsgA_PUSCH=ΔPREAMBLE_Msg3 dB if msgADeltaPreamble is not provided, for carrier f of serving cell c <Unchanged Text Omitted> ## 8.1 Random access preamble <Unchanged Text Omitted> For Type-2 random access procedure with common configuration of PRACH occasions with Type-1 random access procedure, a UE is provided a number N of SS/PBCH block indexes associated with one PRACH occasion by ssb-perRACH-OccasionAndCB-PreamblesPerSSB and a number Q of contention based preambles per SS/PBCH block index per valid PRACH occasion by msgA-CB-PreamblesPerSSB-PerSharedRO. The PRACH transmission can be on a subset of PRACH occasions associated with a same SS/PBCH block index for a UE provided with a PRACH mask index by msgA-SSB-SharedRO-MaskIndex according to [11, TS 38.321]. For Type-2 random access procedure with separate configuration of PRACH occasions with Type-1 random access procedure, a UE is provided a number N of SS/PBCH block indexes associated with one PRACH occasion and a number R of contention based preambles per SS/PBCH block index per valid PRACH occasion by msgA-SSB-PerRACH-OccasionAndCB-PreamblesPerSSB when provided; otherwise, by ssb-perRACH-OccasionAndCB-PreamblesPerSSB. For Type-1 random access procedure, or for Type-2 random access procedure with separate configuration of PRACH occasions from Type 1 random access procedure, if N<1, one SS/PBCH block index is mapped to 1N consecutive valid PRACH occasions and R contention based preambles with consecutive indexes associated with the SS/PBCH block index per valid PRACH occasion start from preamble index 0. If N≥1, R contention based preambles with consecutive indexes associated with SS/PBCH block index n, 0≤n≤N-1, per valid PRACH occasion start from preamble index n⋅NpreambletotalN where Npreambletotal is provided by totalNumberOfRA-Preambles for Type-1 random access procedure, or by msgA-TotalNumberOfRA-Preambles for Type-2 random access procedure with separate configuration of PRACH occasions from a Type 1 random access procedure, and is an integer multiple of N. <Unchanged Text Omitted> A UE determines a first interlace or first RB for a first PUSCH occasion in an active UL BWP respectively from interlaceIndexFirstPO-MsgA-PUSCH or from frequencyStartMsgA-PUSCH that provides an offset, in number of RBs in the active UL BWP, from a first RB of the active UL BWP. A PUSCH occasion includes a number of interlaces or a number of RBs provided by nrofInterlacesPerMsgA-PO or by nrofPRBs-perMsgA-PO, respectively. Consecutive PUSCH occasions in the frequency domain of an UL BWP are separated by a number of RBs provided by guardBandMsgA-PUSCH. A number Nf of PUSCH occasions in the frequency domain of an UL BWP is provided by nrofMsgA-PO-FDM. <Unchanged Text Omitted> Consecutive PUSCH occasions within each slot are separated by guardPeriodMsgA-PUSCH symbols and have same duration. A number Nt of time domain PUSCH occasions in each slot is provided by nrofMsgA-PO-perSlot and a number Ns of consecutive slots that include PUSCH occasions is provided by nrofSlotsMsgA-PUSCH. A UE is provided a DMRS configuration for a PUSCH transmission in a PUSCH occasion in an active UL BWP by msgA-DMRS-Config. <Unchanged Text Omitted> A PUSCH occasion for PUSCH transmission is defined by a frequency resource and a time resource, and is associated with a DMRS resource. The DMRS resources are provided by msgA-DMRS-Config. <Unchanged Text Omitted> where Npreamble=ceilTpreambleTPUSCH, Tpreamble is a total number of valid PRACH occasions per association pattern period multiplied by the number of preambles per valid PRACH occasion provided by rach-ConfigCommonTwoStepRA, and TPUSCH is a total number of valid PUSCH occasions per PUSCH configuration per association pattern period multiplied by the number of DMRS resource indexes per valid PUSCH occasion provided by msgA-DMRS-Config. <Unchanged Text Omitted> ------------------------- Text proposal #2 ends for TS 38.213 -------------------------------

Any comments to Proposal 2?

| Company | Comments |
| --- | --- |
| Nokia | Agree with the TP#2. One editorial: It seems that “msgA-preambleReceivedTargetPoweris” from first part of the TP is missing a space. |

# Capture the condition in the previous agreement for subset RO sharing (issue #3)

In R1-2006091, it was pointed out that one constraint is missing in the description of subset RO sharing in 38.213 according to our previous agreement

Proposal 1:

- Adopt the TP#3 in 38.213, to capture the missing condition in the description of subset RO sharing.

Reasons for change

To capture the missing condition in the description of subset RO sharing

Summary of changes

Implement the above update

Specs/Sections impacted

TS 38.213, Section 8.1

-------------------------Text proposal #3 starts for TS 38.213 ----------------------------

## 8.1	Random access preamble

<Unchanged Text Omitted>

For Type-2 random access procedure with common configuration of PRACH occasions with Type-1 random access procedure, a UE is provided a number N of SS/PBCH block indexes associated with one PRACH occasion by ssb-perRACH-OccasionAndCB-PreamblesPerSSB and a number Q of contention based preambles per SS/PBCH block index per valid PRACH occasion by msgA-CB-PreamblesPerSSB. The PRACH transmission can be on a subset of PRACH occasions associated with a same SS/PBCH block index within an SSB-RO mapping cycle for a UE provided with a PRACH mask index by msgA-ssb-sharedRO-MaskIndex according to [11, TS 38.321].

<Unchanged Text Omitted>

------------------------- Text proposal #3 ends for TS 38.213 -------------------------------

Any comments to Proposal 3?

| Company | Comments |
| --- | --- |
| Nokia | Agree with the TP#3 |

# Editorial issues related to the TDRA (issue #7.1)

R1-2006407 (proposal 4) and R1-2006609 (proposal 3) pointed out the following editorial issue related to the TDRA for MsgA PUSCH.

Proposal 4:

- Adopt the TP#4 in 38.213, to correct the description of TDRA for MsgA PUSCH.

Reasons for change

To correct the description of TDRA for MsgA PUSCH

Summary of changes

Implement the above update

Specs/Sections impacted

TS 38.213, Section 8.1A

-------------------------Text proposal #4 starts for TS 38.213 ----------------------------

8.1A	PUSCH for Type-2 random access procedure

<Unchanged Text Omitted>

If a UE does not have dedicated RRC configuration, or has an initial UL BWP as an active UL BWP, or is not provided startSymbolAndLengthMsgA-PO, msgA-PUSCH-timeDomainAllocation provides a SLIV and a PUSCH mapping type for a PUSCH transmission by indicating

-	one of the first maxNrofUL-Allocations values from PUSCH-TimeDomainResourceAllocationList, if PUSCH-TimeDomainResourceAllocationList is provided in PUSCH-ConfigCommon

-	one of the entries from table 6.1.2.1.1-2 in [6, TS 38.214], if PUSCH-TimeDomainResourceAllocationList is not provided in PUSCH-ConfigCommon

else, the UE is provided with a SLIV by startSymbolAndLengthMsgA-PO, and a PUSCH mapping type by mappingTypeMsgA-PUSCH for a PUSCH transmission.

For mapping one or multiple preambles of a PRACH slot to a PUSCH occasion associated with a DMRS resource, a UE determines a first slot for a first PUSCH occasion in an active UL BWP from msgA-PUSCH-TimeDomainOffset that provides an offset, in number of slots in the active UL BWP, relative to the start of a PUSCH slot including the start of each PRACH slot. The UE does not expect to have a PRACH preamble transmission and a PUSCH transmission with a msgA in a PRACH slot or in a PUSCH slot, or to have overlapping msgA PUSCH occasions for a MsgA PUSCH configuration. The UE expects that a first PUSCH occasion in each slot has a same SLIV [6, TS 38.214] for a PUSCH transmission that is provided by startSymbolAndLengthMsgA-PO or by msgA-PUSCH-timeDomainAllocation.

<Unchanged Text Omitted>

------------------------- Text proposal #4 ends for TS 38.213 -------------------------------

Any comments to Proposal 4?

| Company | Comments |
| --- | --- |
| Nokia | We are not OK with this proposal. The specifications are not broken, and we do not see a need for introducing an optimization. Basically, this proposal will introduce a change of the baseline functionality by introducing a different mapping. |

# Summary

The final proposals and the potential CRs are to be updated…