3GPP TSG RAN WG1 #102-e	         	 R1- 200XXXX

e-Meeting, August 17th – August 28th, 2020

Agenda item:		7.2.2.2.1

Source:	Moderator (Nokia)

Title:	Summary of [102-e-NR-unlic-NRU-ChAcc-01] Email discussion/approval on XXXX

Document for:		Discussion and Decision

# 1	Introduction

This document captures the discussion in the following RAN1#102-e email thread:

[102-e-NR-unlic-NRU-ChAcc-01] Email discussion/approval on the following from R1-2006675 by 8/20; if necessary, endorse associated TPs by 8/26 – Timo (Nokia)

- Issue#1: Indication of LBT type, CP extension and CAPC; N1 timeline for UL transmissions with CP extension

- Issue#4: Clarifications to channel access for semi-static channel occupancy

- Issue#5: DL and UL Channel Access related

- Issue#6: Multi-channel Channel Access

This contribution summarizes the discussion and collects companies views on each question and is organized as follows:

1	Introduction	1

2. Issue #1	2

2.1 LBT type for non-contiguous SRS and PUSCH/PUCCH	2

2.2 CP extension and LBT type for semi-static channel access	4

2.3 Other CP extension / LBT type indication related issues	6

2.4 CAPC of fallback UL grants	6

3. Issue #4	7

3.1 Deployment scenario for semi-static channel access	8

3.2 Editorial corrections related to semi-static channel access	9

3.3 Clarification of the initiating node for FFPs	10

3.4 Other clarifications related to semi-static channel access	11

4. Issue #5	13

4.1 Clarifications to restrictions for Type 1 DL channel access / DRS	13

4.2 Clarifications to DL CWS adjustment	14

4.3 Clarifications to UL CWS adjustment	16

4.4 CWS for channels without explicit feedback	19

5. Issue #6	20

5.1 Clarifications to DL Multi-channel access procedures	20

5.2 Clarifications to UL Multi-channel access procedures	21

6. Editorial Issues	23

7. Conclusions	25

References	25

# 2. Issue #1

Issue #1 Indication of LBT type, CP extension and CAPC; N1 timeline for UL transmissions with CP extension

| LBT type for non-contiguous SRS and PUSCH/PUCCH | R1-2006020 (p3) R1-2006095 (p5) R1-2006301 (p1, p2) R1- 2006370 (p2) |
| --- | --- |
| CP extension and LBT type for semi-static channel access | R1-2005600 (p1) R1-2006763 (section 3) |
| other CP extension related | R1-2006301 (p5) |
| CAPC of fallback UL grants | R1-2006763 (section 2) R1-2005600 (p7) |

## 2.1 LBT type for non-contiguous SRS and PUSCH/PUCCH

Four contributions discuss the determination of LBT type and other related parameters for non-contiguous SRS and PUSCH/PUCCH transmissions, that are triggered with a single DCI.

R1-2006020 (p3)

Proposal 3: for cases where one DCI schedules/triggers two UL transmissions, select the following one option - Option 1: LBT type and CP extension indication are used for the first UL transmission, default LBT type and CP extension are used for the second UL transmission. - Option 2: LBT type and CP extension indication are used for both UL transmissions. - Option 3: LBT type and CP extension indication are used only for PUSCH (if DCI 0_1) or for PUCCH (if DCI - Note: the indicated CAPC in DCI 0_1 is always used for PUSCH.

R1-2006095 (p5)

“applying the indicated CP extension and channel access only for first UL signal/channel is sufficient.”

================================= Start of TP for TS 38.211 ================================ 5.3.1 OFDM baseband signal generation for all channels except PRACH and RIM-RS ================================ Unchanged Texts Omitted ================================= - for dynamically scheduled PUSCH, SRS, and PUCCH transmissions Text=minmaxText',0, Tsymb,(l-1)mod7∙2μμ Text'=k=1CiTsymb, l-kmod 7∙2μ μ-Δi where Δi is given by Table 5.3.1-1 with C1=1 for μ∈0,1, C1=2 for μ=2, and C2 and C3 given by the higher-layer parameters cp-ExtensionC2-r16 and cp-ExtensionC3-r16, respectively, and TTA given by clause 4.3.1. For contention-based random access, or in absence of higher-layer configuration of C2 and C3, the value of Cishall be set to the largest integer fulfilling Text'<Tsymb, (l-1)mod7∙2μ μ for each of the values of i∈2,3. Text is applied to the first UL transmission scheduled by the scheduling DCI. ================================ Unchanged Texts Omitted ================================= ================================= End of TP for TS 38.211 =================================

R1-2006301 (p1, p2)

Proposal #1: For the transmission of SRS that is scheduled without PUSCH or PUCCH, UE shall perform channel access type and apply CP extension indicated by the corresponding DCI. Proposal #2: For the transmission of SRS that is scheduled with PUSCH or PUCCH, UE shall apply indicated channel access type and CP extension to SRS if the SRS is transmitted prior to the PUSCH or PUCCH, otherwise, apply to PUSCH or PUCCH.

R1-2006370 (p2)

Proposal 2: When discontinuous Aperiodic SRS and PUSCH/PUCCH are scheduled with a single DCI: - the LBT type and the CP extension for the first one of the scheduled transmissions is as indicated in the DCI - the LBT type and the CP extension for the second one of the transmissions is determined as in the two tables below. LBT type in the DCI (used also for the first transmission); LBT Type for the second transmission. Type 1 (Cat 4); Outside of a gNB COT Type 1 (Cat 4) Within a gNB or UE COT Type 2A (Cat2 25 us). Type 2A (Cat2 25 us); Type 2A (Cat2 25 us). Type 2B (Cat2 16 us); Type 2A (Cat2 25 us). Type 2C (no LBT); Type 2A (Cat2 25 us). CP extension in the DCI (used also for the first transmission); CP extension for the second transmission. 0 (i.e. no CP extension); 0. C1*symbol length – 25 us; 0 OR C1*symbol length – 25 us. C2*symbol length – 16 us – TA; 0 OR C1*symbol length – 25 us. C3*symbol length – 25 us – TA; 0 OR C1*symbol length – 25 us.

Summary:

All TDocs seem to be ok with applying the indicated CP extension and LBT Type for the first of the discontinuous UL transmissions that is scheduled with DCI 0_1 or 1_1, irrespective of whether that is SRS or PUSCH/PUCCH.

For the second UL transmissions, that is not contiguous with the first one, the options are:

- Alt 1: a default LBT type and CP extension are used for the second UL transmission.

  - one example of default values is in R1-2006370

- Alt 2: the indicated LBT type and CP extension are used for both first and 2nd UL transmissions

- Alt 3: a zero-CP extension is used for the 2nd transmissions, irrespective of what was indicated for the first UL transmission

  - (how to determine the LBT type in this case?)

Companies are asked to provide their views related to the above proposals with the table below:

| Company | Comment |
| --- | --- |

## 2.2 CP extension and LBT type for semi-static channel access

At RAN1#101e there was some discussion related to how to determine the CP extension and especially LBT type with semi-static channel access, where the LBT definition is not 100% the same as in the case of LBE.

A related proposal in R1-2005600 is:

R1-2005600

Proposal 1: The agreement on CP extension and LBT can be achieved by gNB implementation without modification the current spec.

R1-2006763 also discusses the same issue and concludes:

To simply the change to the current version of the spec, we propose to make these fields available for semi-static channel access as well, while introduce the following special handling: - UE ignores CAPC value - UE does not expected to be configured entries with cat 4 LBT - UE does not expect to be configured with entries with CP extension values other than C2*symbol length – 16 us – TA or 0 These can be captured in 37.213 4.3 and 38.212 7.3.1.1.2. ============TP for 38.212================================== 7.3.1.1.2 Format 0_1 ----------Unchanged text omitted----------------------- - ChannelAccess-CPext-CAPC – 0, 1, 2, 3, 4, 5 or 6 bits. The bitwidth for this field is determined as log2(I) bits, where I is the number of entries in the higher layer parameter ULDCI-trigerred-UL-ChannelAccess-CPext-CAPC-List-r16 for operation in a cell with shared spectrum channel; otherwise 0 bit. One or more entries from Table 7.3.1.1.2-35 are configured by the higher layer parameter ULDCI-trigerred-UL-ChannelAccess-CPext-CAPC-List-r16. When ChannelAccessMode-r16=”semi-static”, UE will ignore the CAPC value indicated by ChannelAccess-CPext-CAPC. ----------Unchanged text omitted----------------------- ======================================================= ============TP for 37.213 4.3================================== 4.3 Channel access procedures for semi-static channel occupancy If the absence of any other technology sharing a channel can be guaranteed on a long-term basis (e.g. by level of regulation) and if a gNB provides UE(s) with higher layer parameters ChannelAccessMode-r16 ='semistatic' by SIB1 or dedicated configuration, a periodic channel occupancy can be initiated every Tx within every two consecutive radio frames, starting from the even indexed radio frame at x∙Tx with a maximum channel occupancy timeTy=0.95TxTY=min0.95Tx,Tx-0.1, where Tx=Period in ms, is a higher layer parameter provided in semiStaticChannelAccessConfig-r16 and x∈0,1,…,20Tx-1. In the following procedures in this subclause, when a gNB or UE performs sensing for evaluating a channel availability, the sensing is performed at least during a sensing slot duration Tsl=9us. The corresponding XThresh adjustment for performing sensing by a gNB or a UE is described in subclauses 4.1.5 and 4.2.3, respectively. A channel occupancy initiated by a gNB and shared with UE(s) shall satisfy the following: - The gNB shall transmit a DL transmission burst(s) starting at the beginning of the channel occupancy time immediately after sensing the channel to be idle for at least a sensing slot duration Tsl=9us. If the channel is sensed to be busy, the gNB shall not perform any transmission during the current channel occupancy time. - The gNB may transmit a DL transmission burst(s) within the channel occupancy time immediately after sensing the channel to be idle for at least a sensing slot duration Tsl=9us if the gap between the DL transmission burst(s) and any previous transmission burst is more than 16us. - The gNB may transmit DL transmission burst(s) after UL transmission burst(s) within the channel occupancy time without sensing the channel if the gap between the DL and UL transmission bursts is at most 16us. - A UE may transmit UL transmission burst(s) after detection of a DL transmission burst(s) within the channel occupancy time as follows: - 16usIf the UL transmission is indicated by DCI format 0_1 or DCI format 1_1 to use Type 2C channel access, the UE may transmit UL transmission burst(s) after a DL transmission burst(s) within the channel occupancy time without sensing the channel. - 16us If the UL transmission is indicated by DCI format 0_0 or DCI format 1_0 or RAR UL grant to use Type 1 channel access or Type 2A channel access, or if the UL transmission is indicated by DCI format 1_1 or DCI format 0_1 to use Type 2A channel access, the UE may transmit UL transmission burst(s) after a DL transmission burst(s) within the channel occupancy time after sensing the channel to be idle for at least a sensing slot duration Tsl=9us before transmission. - The gNB and UEs shall not transmit any transmissions in a set of consecutive symbols for a duration of at least Tz=max0.05Tx , 100us before the start of the next channel occupancy time. =======================================================

Companies are asked to provide their views related to the above proposals with the table below:

| Company | Comment |
| --- | --- |

## 2.3 Other CP extension / LBT type indication related issues

One Tdoc addressed the issue of the duration of CP extension being capped to 1 symbol, e.g. in the case of misaligned assumption of the TA value at the UE and gNB. The related proposal is listed below:

R1- 2006301

Proposal #5: If Text'>Tsymb,(l-1)mod7∙2μμ, the UL transmission performed with Cat-1 or Cat-2 LBT should be dropped, and the UL transmission performed with Cat-4 LBT can be transmitted with CP extension length of Text=minmaxText',0, Tsymb,(l-1)mod7∙2μμ.

Companies are asked to provide their views related to the proposal above with the table below:

| Company | Comment |
| --- | --- |

## 2.4 CAPC of fallback UL grants

Two contributions discuss the determination of CAPC of fallback UL grants. The related proposals are as follows:

R1-2005600

------------------------------------------------------- Start of TP #6----------------------------------------------------------------- 4.1.3 DL channel access procedures in a shared channel occupancy For the case where a gNB uses channel access procedures as described in clause 4.1.1 to initiate a transmission and shares the corresponding channel occupancy with a UE that transmits a transmission as described in clause 4.2.1.2, the gNB may transmit a transmission within its channel occupancy that follows the UE's transmission if any gap between any two transmissions in the gNB channel occupancy is at most 25us. In this case the following applies: - If the gap is 25us or16us, the gNB can transmit the transmission on the channel after performing Type 2A or 2B DL channel access procedures as described in clause 4.1.2.1 and 4.1.2.2, respectively. - If the gap is up to 16us, the gNB can transmit the transmission on the channel after performing Type 2C DL channel access as described in clause 4.1.2.3. For the case where a gNB acquires a channel occupancy time (COT) using channel access procedures as described in subclause 4.1.1 and shares the corresponding channel occupancy time (COT) with a UE, the UE assumes channel access priority class (CAPC) value p=4 used by gNB to obtain channel occupancy time (COT) if channel access priority class (CAPC) used by gNB is not indicated explicitly in UL grant of RAR. -------------------------------------------------------- End of TP #6---------------------------------------------------------------- The reference to the modification for Section 4.1.3 of TS 37.213 is based on the agreement of RAN1 #99 meeting in Part3 of the Appendix. Proposal 7: It is proposed to capture a missing agreement that “In RAR, the UE assumes CAPC=4 was used by the gNB to acquire the CO when CAPC is not indicated explicitly” in Section 4.1.3 of the latest version of TS 37.213.

R1-2006763

==TP for 37.213 4.2.1================= 4.2.1 Channel access procedures for uplink transmission(s) --unchanged text omitted---- When a UE uses Type 1 channel access procedures for PUSCH transmissions on configured resource, the UE determines the corresponding UL channel access priority p in Table 4.2.1-1 following the procedures described in Subclause 5.6.2 in [9]. When a UE uses Type 1 channel access procedures for PUSCH transmissions with user plane data indicated by a UL grant or related to random access procedure where the corresponding UL channel access priority p is not indicated, the UE determines p in Table 4.2.1-1 following the same procedures as for PUSCH transmission on configured resources using Type 1 channel access procedures. When a UE uses Type 2A, Type 2B, or Type 2C UL channel access procedures for PUSCH transmissions indicated by a fallback UL grant or related to random access procedure where the corresponding UL channel access priority p is not indicated, the UE assumes gNB uses channel access priority class p=4 for the Channel Occupancy Time. A UE shall not transmit on a channel for a Channel Occupancy Time that exceeds Tulmcot,p where the channel access procedure is performed based on the channel access priority class p associated with the UE transmissions, as given in Table 4.2.1-1. The total Channel Occupancy Time of autonomous uplink transmission(s) obtained by the channel access procedure in this clause, including the following DL transmission if the UE sets 'COT sharing indication' in AUL-UCI to '1' in a subframe within the autonomous uplink transmission(s) as described in Subclause 4.1.3, shall not exceed Tulmcot,p, where Tulmcot,p is given in Table 4.2.1-1. --unchanged text omitted---- =================================

Companies are asked to provide their views related to the above proposals with the table below:

| Company | Comment |
| --- | --- |

# 3. Issue #4

Issue #4 Clarifications to channel access for semi-static channel occupancy

| Clarifications to channel access for semi-static channel occupancy | R1-2005600 (p3, p4, p5, p6) R1-2005809 (p10) R1-2005914 (p1) R1-2006351 (p1, p2, p3, p4, p5) R1- 2006370 (p1) |
| --- | --- |

## 3.1 Deployment scenario for semi-static channel access

Two TDocs addressed the issue in Section 4.3 of 37.213, where use of semis-static channel access is limited to the case when other technologies are not-present on a long-term basis.

R1-2005914:

- Proposal 1	Remove the condition on presence or absence of other technologies in 37.213 for semi-static channel access procedures.

  - Adopt the following TP1 for clause 4.3 of TS37.213:

----------------------------------------------- Beginning of Text Proposal1 (TS 37.213)---------------------------------------------------- ## 4.3 Channel access procedures for semi-static channel occupancy If the absence of any other technology sharing a channel can be guaranteed on a long-term basis (e.g. by level of regulation) and if a gNB provides UE(s) with higher layer parameters ChannelAccessMode-r16 ='semistatic' by SIB1 or dedicated configuration, a periodic channel occupancy can be initiated every Tx within every two consecutive radio frames, starting from the even indexed radio frame at x∙Tx with a maximum channel occupancy time Ty=0.95Tx, where Tx=Period in ms, is a higher layer parameter provided in semiStaticChannelAccessConfig-r16 and x∈0,1,…,20Tx-1. ----------------------------------------------- End of Text Proposal ----------------------------------------------------

R1-2006730

Proposal 1: remove the limitation in 37.213 on semi-static channel access being applicable only in absence of other technologies. -------- Beginning of Text Proposal (TS 37.213) ------------ ## 4.3 Channel access procedures for semi-static channel occupancy If the absence of any other technology sharing a channel can be guaranteed on a long-term basis (e.g. by level of regulation) and if a gNB provides UE(s) with higher layer parameters ChannelAccessMode-r16 ='semistatic' by SIB1 or dedicated configuration, a periodic channel occupancy can be initiated every Tx within every two consecutive radio frames, starting from the even indexed radio frame at x∙Tx with a maximum channel occupancy time Ty=0.95Tx, where Tx=Period in ms, is a higher layer parameter provided in semiStaticChannelAccessConfig-r16 and x∈0,1,…,20Tx-1. ---------- End of Text Proposal -------------

A related proposal in R1-2005600 proposes a different clarification to the same section:

R1-2005600

------------------------------------------------------------- Start of TP #2-------------------------------------------------------- 4.3 Channel access procedures for semi-static channel occupancy If the absence of any other technology and other nodes with ChannelAccessMode-r16 = "dynamic" or ChannelAccessMode-r16 is absent sharing a channel can be guaranteed on a long-term basis (e.g. by level of regulation) and if a gNB provides UE(s) with higher layer parameters ChannelAccessMode-r16 ='semistatic' by SIB1 or dedicated configuration, a periodic channel occupancy can be initiated every Tx within every two consecutive radio frames, starting from the even indexed radio frame at x∙Txwith a maximum channel occupancy time Ty=0.95Tx, where Tx=Period in ms, is a higher layer parameter provided in semiStaticChannelAccessConfig-r16 and x∈0,1,…,20Tx-1 . <unchanged part omitted> --------------------------------------------------------- End of TP #2---------------------------------------------------------------- As shown in Part 1 in the Appendix, FBE operation for the scenario where it is guaranteed that LBE nodes are absent on a long term basis (e.g., by level of regulation) in TS 38.889. Therefore, it is proposed to capture the assumption for the description of FBE. Proposal 3: It is proposed to add “and other nodes with ChannelAccessMode-r16 = "dynamic"or ChannelAccessMode-r16 is absent” after “any other technology”, to clarify the FBE scenario in Section 4.3 of the latest version of TS 37.213.

Companies are asked to provide their views related to the above proposals with the table below:

| Company | Comment |
| --- | --- |

## 3.2 Editorial corrections related to semi-static channel access

R1-2005600 proposes a few editorial corrections to Section 4.3 of 37.213:

----------------------------------------------------------------- Start of TP #3-------------------------------------------------------- If the absence of any other technology sharing a channel can be guaranteed on a long-term basis (e.g. by level of regulation) and if a gNB provides UE(s) with higher layer parameters ChannelAccessMode-r16 ='semistatic' by SIB1 or dedicated configuration, a periodic channel occupancy can be initiated every Tx within every two consecutive radio frames, starting from the even indexed radio frame at x∙Txwith a maximum channel occupancy time Ty=0.95Tx, where Tx=Period in ms, is a higher layer parameter provided by SIB1 or dedicated configuration in semiStaticChannelAccessConfig-r16 and . <unchanged part omitted> --------------------------------------------------------- End of TP #3---------------------------------------------------------------- Proposal 4: It is proposed to remove “by SIB1 or dedicated configuration” after “ChannelAccessMode-r16 ='semistatic'” and add the words in front of “in SemiStaticChannelAccessConfig” in Section 4.3 of the latest version of TS 37.213.

----------------------------------------------------------------- Start of TP #4-------------------------------------------------------- If the absence of any other technology sharing a channel can be guaranteed on a long-term basis (e.g. by level of regulation) and if a gNB provides UE(s) with higher layer parameters ChannelAccessMode-r16 ='semistatic' by SIB1 or dedicated configuration, a periodic channel occupancy can be initiated every Tx within every two consecutive radio frames, starting from the even indexed radio frame atx∙iTx with a maximum channel occupancy time Ty=0.95Tx, where Tx=Period in ms, is a higher layer parameter provided in semiStaticChannelAccessConfig-r16 and xi∈0,1,…,20Tx-1. <unchanged part omitted> --------------------------------------------------------- End of TP #4---------------------------------------------------------------- Proposal 5: It is proposed to change the parameter form “” to “” in Section 4.3 of the latest version of TS 37.213.

----------------------------------------------------------------- Start of TP #5-------------------------------------------------------- If the absence of any other technology sharing a channel can be guaranteed on a long-term basis (e.g. by level of regulation) and if a gNB provides UE(s) with higher layer parameters ChannelAccessMode-r16 ='semistatic' by SIB1 or dedicated configuration, a periodic channel occupancy can be initiated every Tx within every two consecutive radio frames, starting from the even indexed radio frame at x?Tx with a maximum channel occupancy time Ty=0.95Tx, where Tx=Period Tx=Period Tx= period in ms, is a higher layer parameter provided in sSemiStaticChannelAccessConfig-r16 and . <unchanged part omitted> --------------------------------------------------------- End of TP #5---------------------------------------------------------------- Proposal 6: It is proposed to change the higher layer parameters from “Period” to “period” and “semiStaticChannelAccessConfig-r16” to “SemiStaticChannelAccessConfig” in Section 4.3 of the latest version of TS 37.213 .

Companies are asked to provide their views related to the three proposals above with the table below:

| Company | Comment |
| --- | --- |

## 3.3 Clarification of the initiating node for FFPs

R1-2005809 proposes clarifications to Section 4.3 based on the fact that UE-initiated FFPs are not supported in Rel-16, as well as some editorial modifications.

R1-2005809

## *** <Beginning of Text Proposal 7> *** ## 4.3 Channel access procedures for Semi-static channel occupancy If the absence of any other technology sharing a channel can be guaranteed on a long-term basis (e.g. by level of regulation) and if a gNB provides UE(s) with higher layer parameters ChannelAccessMode-r16 ='semistatic' by SIB1 or dedicated configuration, a periodic channel occupancy can be initiated by the gNB only every Tx within every two consecutive radio frames, starting from the even indexed radio frame at x∙Tx with a maximum channel occupancy time Ty=0.95Tx, where Tx=Period in ms, is a higher layer parameter provided in semiStaticChannelAccessConfig-r16 and x∈0,1,…,20Tx-1. In the following procedures in this subclause, when a gNB or a UE scheduled/configured by a gNB performs sensing for evaluating a channel availability, the sensing is performed at least during a sensing slot duration Tsl=9us. The corresponding XThresh adjustment for performing sensing by a gNB or a UE is described in subclauses 4.1.5 and 4.2.3, respectively. A channel occupancy initiated by a gNB and shared with UE(s) shall satisfy the following: - The gNB shall transmit a DL transmission burst starting at the beginning of the channel occupancy time immediately after sensing the channel to be idle for at least a sensing slot duration Tsl=9us. If the channel is sensed to be busy, the gNB shall not perform any transmission during the current period. ## *** Unchanged text is omitted *** - The gNB and UEs shall not transmit any transmissions in a set of consecutive symbols for a duration of at least Tz=max0.05Tx , 100us before the start of the next period. ## *** <End of Text Proposal 7> *** Proposal 10：Adopt TP7 into section 4.3 of TS 37.213 as it would also impact developing the specifications for UE-initiated semi-static CO under the Rel-17 IIoT/URLLC work item

Companies are asked to provide their views related to the TP above with the table below:

| Company | Comment |
| --- | --- |

## 3.4 Other clarifications related to semi-static channel access

R1-2006351 discusses a few further issues related to semi-static channel access-

Proposal 1: Update TS 37.213 Clause 4.3 based on TP #1. -------------------------------------------------------- Start of TP #1 ---------------------------------------------------- If the absence of any other technology sharing a channel can be guaranteed on a long-term basis (e.g. by level of regulation) and if a gNB provides UE(s) with higher layer parameters ChannelAccessMode-r16 ='semistatic' by SIB1 or dedicated configuration, a periodic channel occupancy can be initiated every Tx within every two consecutive radio frames, starting from the even indexed radio frame at x∙Tx with a maximum channel occupancy time Ty=0.95Tx, where Tx=Period in ms, is a higher layer parameter provided in semiStaticChannelAccessConfig-r16 and x∈0,1,…,20Tx-1. Each Tx comprises a channel occupancy time Ty=Tx-Tz from the beginning and an idle period Tz=max⁡(0.05Tx, 100us) from the end. -------------------------------------------------------- End of TP #1 -----------------------------------------------------

Proposal 2: In FBE, when DCI 2_0 does not contain SFI, UE cancels the reception of periodic or semi-persistent CSI-RS configured by higher layers on downlink and flexible symbols (including the case where no semi-static TDD configuration is provided to the UE) if the periodic or semi-persistent CSI-RS location is outside the acquired CO duration, where the CO is acquired by a detection of DCI 2_0.

Proposal 3: In FBE, UE performs UL transmission validation (as well as CSI-RS reception) on reception of DCI format 2_0. Update TS 37.213 Clause 4.3 based on TP #2. -------------------------------------------------------- Start of TP #2 ---------------------------------------------------- - A UE may transmit UL transmission burst(s) after detection of a DCI format 2_0 DL transmission burst(s) within the channel occupancy time as follows: - If the gap between the UL and DL transmission bursts is at most 16us, the UE may transmit UL transmission burst(s) after a DL transmission burst(s) within the channel occupancy time without sensing the channel. - If the gap between the UL and DL transmission bursts is more than 16us, the UE may transmit UL transmission burst(s) after a DL transmission burst(s) within the channel occupancy time after sensing the channel to be idle for at least a sensing slot duration Tsl=9us within a 25us interval ending immediately before transmission. -------------------------------------------------------- End of TP #2 -----------------------------------------------------

Proposal 4: Update TS 37.213 Clause 4.3 based on TP #3. -------------------------------------------------------- Start of TP #3 ---------------------------------------------------- - The gNB and UEs shall not transmit any transmissions in a set of consecutive symbols overlapping with for a duration of at least Tz=max0.05Tx , 100us before the start of the next channel occupancy time. -------------------------------------------------------- End of TP #3 -----------------------------------------------------

Proposal 5: Discuss for FBE the necessity of a short DRS transmission in a FFP in which the CCA failed.

Companies are asked to provide their views related to the TPs and proposals above with the table below:

| Company | Comment |
| --- | --- |

# 4. Issue #5

Issue #5: DL and UL Channel Access related

| Clarifications to restrictions for Type 1 DL channel access / DRS | R1-2006095 (p1) R1-2006351 (p6) |
| --- | --- |
| Clarifications to DL CWS adjustment | R1-2005809 (p1) R1-2006881 (p2, p3) |
| Clarifications to UL CWS adjustment | R1-2005809 (p2, p3, p4) R1-2006095 (p2, p3, p4) R1-2006301 (p6, p8) |
| CWS for channels without explicit feedback | R1-2006301 (p7) R1-2005809 (p3, p4) |

## 4.1 Clarifications to restrictions for Type 1 DL channel access / DRS

R1-2006095 proposes a clarification to the definition of the duty cycle for DRS transmissions.

================================ Start of TP for TS 37.213 ================================= 4.0 General ================================ Unchanged Texts Omitted ================================= - A discovery burst refers to a DL transmission burst including a set of signal(s) and/or channel(s) confined within a window and associated with a duty cycle. The duty cycle is defined as DDB/PDB, wherein DDB is the duration of the discovery burst and determined according to the transmission starting instance of the discovery burst, and PDB is determined as the periodicity of the discovery burst transmission window, as described in subclause 4.1 of [7]. The discovery burst can be any of the following: - Transmission(s) initiated by an eNB that includes a primary synchronization signal (PSS), secondary synchronization signal (SSS) and cell-specific reference signal(s)(CRS) and may include non-zero power CSI reference signals (CSI-RS). - Transmission(s) initiated by a gNB that includes at least an SS/PBCH block consisting of a primary synchronization signal (PSS), secondary synchronization signal (SSS), physical broadcast channel (PBCH) with associated demodulation reference signal (DM-RS) and may also include CORESET for PDCCH scheduling PDSCH with SIB1, and PDSCH carrying SIB1 and/or non-zero power CSI reference signals (CSI-RS). ================================= End of TP for TS 37.213 =================================

R1-2006351 proposes another clarification to the condition for choosing LBT type DRS transmission in Section 4.1.1.

-------------------------------------------------------- Start of TP #4 ---------------------------------------------------- This clause describes channel access procedures to be performed by an eNB/gNB where the time duration spanned by the sensing slots that are sensed to be idle before a downlink transmission(s) is random. The clause is applicable to the following transmissions: - Transmission(s) initiated by an eNB including PDSCH/PDCCH/EPDCCH, or - Transmission(s) initiated by a gNB including unicast PDSCH with user plane data, or unicast PDSCH with user plane data and unicast PDCCH scheduling user plane data, or - Transmission(s) initiated by a gNB including PDSCH/PDCCH not multiplexed with discovery burst, or - Transmission(s) initiated by a gNB including discovery burst multiplexed with unicast information, or - Transmission(s) initiated by a gNB with including only discovery burst or with including discovery burst multiplexed with non-unicast information, where the transmission(s) duration is larger than 1ms or the transmission causes the discovery burst duty cycle to exceed 1/20. <Unchanged part omitted> A gNB may use any channel access priority class for performing the procedures above to transmit transmission(s) including discovery burst(s) satisfying the conditions described in this subclause. A gNB shall use a channel access priority class applicable to the unicast user plane data multiplexed in PDSCH for performing the procedures above to transmit transmission(s) including unicast PDSCH with user plane data. A gNB may use any channel access priority class for performing the procedures above to transmit other transmission(s) described in this subclause. -------------------------------------------------------- End of TP #4 -----------------------------------------------------

Companies are asked to provide their views related to the TPs and proposals above with the table below:

| Company | Comment |
| --- | --- |

## 4.2 Clarifications to DL CWS adjustment

R1-2005809 proposes clarifications to Section 4.1.4.2:

## *** <Beginning of Text Proposal 1> *** #### 4.1.4.2 Contention window adjustment procedures for DL transmissions by gNB If a gNB transmits transmissions including PDSCH that are associated with channel access priority class p on a channel, the gNB maintains the contention window value CWp and adjusts CWp before step 1 of the procedure described in subclause 4.1.1 for those transmissions using the following steps: - For every priority class p∈1,2,3,4, set CWp=CWmin,p. - If HARQ-ACK feedback is available after the last update of Wp , go to step 3. Otherwise, if the gNB transmission after procedure described in subclause 4.1.1 does not include a retransmission or is transmitted within a duration Tw from the end of the reference duration corresponding to the earliest DL channel occupancy after the last update of CWp , go to step 5; otherwise go to step 4. - The HARQ-ACK feedback(s) corresponding to PDSCH(s) in the reference duration for the latest DL channel occupancy for which HARQ-ACK feedback is available is used as follows: - If at least one HARQ-ACK feedback is ‘ACK’ for PDSCH(s) with transport block based feedback or at least 10% of HARQ-ACK feedbacks is ‘ACK’ for CBGs overlapping with the channel and in PDSCH(s) with code block group based feedback, go to step 1; otherwise go to step 4. - Increase CWp for every priority class p∈1,2,3,4 to the next higher allowed value. - For every priority class p∈1,2,3,4, maintain CWp as it is; go to step 2. The reference duration and duration Tw in the procedure above are defined as follows: - The reference duration corresponding to a channel occupancy initiated by the gNB including transmission of PDSCH(s) is defined in this subclause as a duration starting from the beginning of the channel occupancy until the end of the first slot where at least one unicast PDSCH is transmitted over all the resources allocated for the PDSCH, or until the end of the first transmission burst by the gNB that contains unicast PDSCH(s) transmitted over all the resources allocated for the PDSCH, whichever occurs earlier. If the channel occupancy includes a unicast PDSCH, but it does not include any unicast PDSCH transmitted over all the resources allocated for that PDSCH, then, the duration of the first transmission burst by the gNB within the channel occupancy that contains unicast PDSCH(s) is the reference duration for CWS adjustment. - Tw=max⁡(TA,TB+1ms) where TB is the duration of the transmission burst from start of the reference duration in ms and TA=5ms if the absence of any other technology sharing the channel can not be guaranteed on a long-term basis (e.g. by level of regulation), and TA=10ms otherwise. ## *** Unchanged text is omitted *** *** <End of Text Proposal 2> ***

R1-2006881 raises another issue, related to CWS reset for CBG case.

- Proposal 3: We propose to reset CW as a minimum under CWS adjustment procedure if all CBG-NACKs generated by being not correctly detecting TB at the UE are received at the gNB when a part of total CBGs is retransmitted by the gNB. The text proposal for TS 37.213 is as follows: ===========================Start of Text Proposal for TS37.213============================ #### 4.1.4.2 Contention window adjustment procedures for DL transmissions by gNB If a gNB transmits transmissions including PDSCH that are associated with channel access priority class p on a channel, the gNB maintains the contention window value CWp and adjusts CWp before step 1 of the procedure described in clause 4.1.1 for those transmissions using the following steps: 1) For every priority class p∈1,2,3,4, set CWp=CWmin,p. 2) If HARQ-ACK feedback is available after the last update of Wp , go to step 3. Otherwise, if the gNB transmission after procedure described in clause 4.1.1 does not include a retransmission or is transmitted within a duration Tw from the end of the reference duration corresponding to the earliest DL transmission burst after the last update of CWp transmitted after the procedures described in clause 4.1.1, go to step 5; otherwise go to step 4. 3) The HARQ-ACK feedback(s) corresponding to PDSCH(s) in the reference duration for the latest DL transmission burst for which HARQ-ACK feedback is available is used as follows: a. If at least one HARQ-ACK feedback is 'ACK' for PDSCH(s) with transport block based transmissions or at least 10% of HARQ-ACK feedbacks is 'ACK' for PDSCH(s) with code block group based transmissions or if all of HARQ-ACK feedbacks are ‘NACK’ for PDSCH(s) with code block group based transmissions which is generated by being not correctly detecting transport block at the UE when a part of total code block group based transmission is retransmitted by the gNB, go to step 1; otherwise go to step 4. 4) Increase CWp for every priority class p∈1,2,3,4 to the next higher allowed value. 5) For every priority class p∈1,2,3,4, maintain CWp as it is; go to step 2. The reference duration and duration Tw in the procedure above are defined as follows: - The reference duration corresponding to a channel occupancy initiated by the gNB including transmission of PDSCH(s) is defined in this clause as a duration starting from the beginning of the channel occupancy until the end of the first slot where at least one unicast PDSCH is transmitted over all the resources allocated for the PDSCH, or until the end of the first transmission burst by the gNB that contains unicast PDSCH(s) transmitted over all the resources allocated for the PDSCH, whichever occurs earlier. If the channel occupancy includes a unicast PDSCH, but it does not include any unicast PDSCH transmitted over all the resources allocated for that PDSCH, then, the duration of the first transmission burst by the gNB within the channel occupancy that contains unicast PDSCH(s) is the reference duration for CWS adjustment. - Tw=max⁡(TA,TB+1ms) where TB is the duration of the transmission burst from start of the reference duration in ms and TA=5ms if the absence of any other technology sharing the channel can not be guaranteed on a long-term basis (e.g. by level of regulation), and TA=10ms otherwise. =========================== End of Text Proposal for TS37.213============================.

Companies are asked to provide their views related to the TPs and proposals above with the table below:

| Company | Comment |
| --- | --- |

## 4.3 Clarifications to UL CWS adjustment

R1-2005809 discusses issues related to determination of the reference duration for CWS update, and in particular use of “transmission burst” and “channel occupancy. Related TP is below:

## *** <Beginning of Text Proposal 2> *** #### 4.2.2.2 Contention window adjustment procedures for UL transmissions scheduled/configured by gNB If a UE transmits transmissions using Type 1 channel access procedures that are associated with channel access priority class p on a channel, the UE maintains the contention window value CWp and adjusts CWp for those transmissions before step 1 of the procedure described in subclause 4.2.1.1, using the following steps: - For every priority class p∈1,2,3,4, set CWp=CWmin,p; - If HARQ-ACK feedback is available after the last update of CWp, go to step 3. Otherwise, if the UE transmission after procedure described in subclause 4.2.1.1 does not include a retransmission or is transmitted within a duration Tw from the end of the reference duration corresponding to the earliest UL channel occupancy after the last update of CWp, go to step 5; otherwise go to step 4. - The HARQ-ACK feedback(s) corresponding to PUSCH(s) in the reference duration for the latest UL channel occupancy for which HARQ-ACK feedback is available is used as follows: - If at least one HARQ-ACK feedback is ‘ACK’ for PUSCH(s) with transport block (TB) based feedback or at least 10% of HARQ-ACK feedbacks is ‘ACK’ for CBGs overlapping with the channel and in PUSCH(s) with code block group (CBG) based feedback go to step 1; otherwise go to step 4. - Increase CWp for every priority class p∈1,2,3,4 to the next higher allowed value; - For every priority class p∈1,2,3,4, maintain CWp as it is; go to step 2. ## *** Unchanged text is omitted *** ## *** <End of Text Proposal 2> ***

Companies are asked to provide their views related to the TPs and proposals above with the table below:

| Company | Comment |
| --- | --- |

R1-2006095 discusses also CWS adjustment and makes following proposals, accompanied by a TP:

Proposal 2: Reuse the RRC configured minimum duration cg-minDFIDelay-r16 for the minimum latency between reference duration and the following UL grant or CG-DFI for CWS adjustment. Proposal 3: Specify CWS adjustment based on valid HARQ-ACK defined in TS 38.213. ================================= Start of TP for TS 37.213 ================================ 4.2.2.2 Contention window adjustment procedures for UL transmissions scheduled/configured by gNB ================================ Unchanged Texts Omitted ================================= The HARQ-ACK feedback, reference duration and duration Tw in the procedure above are defined as the following: - HARQ-ACK feedback for PUSCH(s) transmissions are expected to be provided to UE(s) explicitly or implicitly where implicit HARQ-ACK feedback for the purpose of contention window adjustment in this subclause, is determined based on the indication for a new transmission or retransmission in the DCI scheduling PUSCH(s) as follows: - If a new transmission is indicated, 'ACK' is assumed for the transport blocks or code block groups in the corresponding PUSCH(s) for the TB-based and CBG-based transmission, respectively. - If a retransmission is indicated for TB-based transmissions, 'NACK' is assumed for the transport blocks in the corresponding PUSCH(s). - If a retransmission is indicated for CBG-based transmissions, if a bit value in the code block group transmission information (CBGTI) field is '0' or '1' as described in subclause 5.1.7.2 in [8], 'ACK' or 'NACK' is assumed for the corresponding CBG in the corresponding PUSCH(s), respectively. Explicit HARQ-ACK feedback for the purpose of contention window adjustment in this subclause is determined based on valid HARQ-ACK in the CG-DFI as described in subclause 10.5 in [7]. - The reference duration corresponding to a channel occupancy initiated by the UE including transmission of PUSCH(s) is defined in this subclause as a duration starting from the beginning of the channel occupancy until the end of the first slot where at least one unicast PUSCH is transmitted over all the resources allocated for the PDSCH, or until the end of the first transmission burst by the gNB that contains unicast PUSCH(s) transmitted over all the resources allocated for the PDSCH, whichever occurs earlier, and the duration starting no later than a number of symbols provided by cg-minDFIDelay-r1 before an UL grant or a CG-DFI. If the channel occupancy includes a unicast PDSCH, but it does not include any unicast PDSCH transmitted over all the resources allocated for that PUSCH, and the duration starting no later than a number of symbols provided by cg-minDFIDelay-r1 before an UL grant or a CG-DFI, then, the duration of the first transmission burst by the UE within the channel occupancy that contains PUSCH(s) is the reference duration for CWS adjustment. - Tw=max⁡(TA,TB+1ms) where TB is the duraon of the transmission burst from start of the reference duration in ms and TA=5ms if the absence of any other technology sharing the channel cannot be guaranteed on a long-term basis (e.g. by level of regulation), and TA=10ms otherwise. If a UE transmits transmissions using Type 1 channel access procedures associated with the channel access priority class p on a channel and the transmissions are not associated with explicit or implicit HARQ-ACK feedbacks as described above in this subclause, the UE adjusts CWp before step 1 in the procedures described in subclause 4.2.1.1, using the latest CWp used for any UL transmissions on the channel using Type 1 channel access procedures associated with the channel access priority class p. If the corresponding channel access priority class p has not been for any UL transmission on the channel, CWp=CWmin,p is used. ================================ Unchanged Texts Omitted ================================= ================================= End of TP for TS 37.213 =================================

R1-2006301 also discusses similar issue, and proposes:

Proposal #8: The reference duration for UL CWS adjustment can be defined in the recent UL burst starting before n-X, where n and X correspond to the starting time of UL grant and the minimum time between UL grant and the end of reference duration, respectively, and X is configured by RRC signalling or is set to the same value with cg-minDFIDelay-r16.

Companies are asked to provide their views related to the TPs and proposals above with the table below:

| Company | Comment |
| --- | --- |

## 4.4 CWS for channels without explicit feedback

R1-2006301 discusses CWS adjustment for RACH procedure and makes a proposal along with a TP:

Proposal #6: The CWS for Msg3 can be adjusted based on the reception of Msg4. ================================ Start of TP#4 for TS 37.213 ================================ 4.2.2.2 Contention window adjustment procedures for UL transmissions scheduled/configured by gNB ================================ Unchanged Texts Omitted ================================= If a UE transmits transmissions using Type 1 channel access procedures associated with the channel access priority class on a channel and the transmissions are not associated with explicit or implicit HARQ-ACK feedbacks as described above in this subclause, the UE adjusts before step 1 in the procedures described in subclause 4.2.1.1, using the latest used for any UL transmissions associated with explicit or implicit HARQ-ACK feedbacks on the channel using Type 1 channel access procedures associated with the channel access priority class . If the corresponding channel access priority class has not been for any UL transmission on the channel, is used. ================================ Unchanged Texts Omitted ================================= ================================= End of TP#4 for TS 37.213 ================================

R1-2005809 makes also a similar proposal:

Proposal 3：Since Msg 3 PUSCH satisfies the subclause in section 4.2.2.2 of TS 37.213 as a UL channel with implicit HARQ feedback NACK, receiving Msg 4 in response should be considered as an implicit HARQ feedback ACK for the purpose of CWS adjustment. ## *** <Beginning of Text Proposal 3> *** #### 4.2.2.2 Contention window adjustment procedures for UL transmissions scheduled/configured by gNB ## *** Unchanged text is omitted *** The HARQ-ACK feedback, reference duration and duration Tw in the procedure above are defined as the following: - HARQ-ACK feedback for PUSCH(s) transmissions are expected to be provided to UE(s) explicitly or implicitly where implicit HARQ-ACK feedback for the purpose of contention window adjustment in this subclause, is determined based on either the indication for a new transmission or retransmission in the DCI scheduling PUSCH(s) or the success or failure of receiving of an associated channel/signal in response as follows: - If a new transmission is indicated, ‘ACK’ is assumed for the transport blocks or code block groups in the corresponding PUSCH(s) for the TB-based and CBG-based transmission, respectively. - If a retransmission is indicated for TB-based transmissions, ‘NACK’ is assumed for the transport blocks in the corresponding PUSCH(s). - If a retransmission is indicated for CBG-based transmissions, if a bit value in the code block group transmission information (CBGTI) field is ‘0’ or ‘1’ as described in subclause 5.1.7.2 in [8], ‘ACK’ or ‘NACK’ is assumed for the corresponding CBG in the corresponding PUSCH(s), respectively. - If the UE successfully receives Msg 4 in response to its transmission of Msg 3 PUSCH, ‘ACK’ is assumed for the corresponding TB in the corresponding PUSCH. - The reference duration corresponding to a channel occupancy initiated by the UE including transmission of PUSCH(s) is defined in this subclause as a duration starting from the beginning of the channel occupancy until the end of the first slot where at least one PUSCH is transmitted over all the resources allocated for the PUSCH, or until the end of the first transmission burst by the UE that contains PUSCH(s) transmitted over all the resources allocated for the PUSCH, whichever occurs earlier. If the channel occupancy includes a PUSCH, but it does not include any PUSCH transmitted over all the resources allocated for that PUSCH, then, the duration of the first transmission burst by the UE within the channel occupancy that contains PUSCH(s) is the reference duration for CWS adjustment. ## *** Unchanged text is omitted *** ## *** <End of Text Proposal 3> ***

Companies are asked to provide their views related to the TPs and proposals above with the table below:

| Company | Comment |
| --- | --- |

# 5. Issue #6

Issue #6: Multi-channel Channel Access:

| Clarifications to DL Multi-channel access procedures | R1-2005809 (p5) |
| --- | --- |
| Clarifications to UL Multi-channel access procedures | R1-2005809 (p6, p7, p8, p9) R1-2006301 (p3) |

## 5.1 Clarifications to DL Multi-channel access procedures

R1-2005809 proposes a clarification to DL Multi-channel access procedures related to transmission overlapping multiple channels:

## *** <Beginning of Text Proposal 4> *** ##### 4.1.6.1.1 Type A1 multi-channel access procedures Counter N as described in subclause 4.1.1 is independently determined for each channel ci and is denoted as Nci. If the absence of any other technology sharing the channel cannot be guaranteed on a long term basis (e.g. by level of regulation), when the eNB/gNB ceases transmission on any one channel cj∈C, for each channel ci≠cj, the eNB/gNB can resume decrementing Nci when idle sensing slots are detected either after waiting for a duration of 4⋅Tsl, or after reinitialising Nci. For determining CWp for channel ci, any PDSCH that fully or partially overlaps with channel ci, is used in the procedure described in subclause 4.1.4.2. ##### 4.1.6.1.2 Type A2 multi-channel access procedures Counter N is determined as described in subclause 4.1.1 for channel cj∈C, and is denoted as Ncj, where cj is the channel that has the largest CWp value. For each channel ci, Nci=Ncj. When the eNB/gNB ceases transmission on any one channel for which Nci is determined, the eNB/gNB shall reinitialise Nci for all channels. For determining CWp for channel ci, any PDSCH that fully or partially overlaps with channel ci, is used in the procedure described in subclause 4.1.4.2. *** <End of Text Proposal 4> ***

Companies are asked to provide their views related to the TPs and proposals above with the table below:

| Company | Comment |
| --- | --- |

## 5.2 Clarifications to UL Multi-channel access procedures

R1-2005809 discusses UL multi-channel access and makes following proposals:

Proposal 6: UE should be allowed to switch CAT4 LBT to CAT2 LBT on the same RB set(s) where DCI format 2_0 carrying available RB set indicator is detected when the available RB set indicators indicate all RB sets are not available for reception. Proposal 7：Adopt TP5 into section 4.2.1.0.0 of TS 37.213. ## *** <Beginning of Text Proposal 5> *** ##### 4.2.1.0.0 Channel access procedures upon detection of a common DCI *** Unchanged text is omitted *** If a UE determines the duration in time domain and the location in frequency domain of a remaining channel occupancy initiated by the gNB from a DCI format 2_0 as described in subclause 11.1.1 of [7], the following is applicable: - The UE may switch from Type 1 channel access procedures as described in subclause 4.2.1.1 to Type 2A channel access procedures as described in subclause 4.2.1.2.1 for its corresponding UL transmissions within the determined duration in time and location in frequency domain of the remaining channel occupancy. In this case, if the UL transmissions are PUSCH transmissions on configured resources, the UE may assume any priority class for the channel occupancy shared with the gNB. - If the UE determines from a DCI format 2_0 that all RB sets are not available, the UE may switch from Type 1 channel access procedures to Type 2A channel access procedures on the same RB set(s) where the DCI format 2_0 was detected. ## *** Unchanged text is omitted *** ## *** <End of Text Proposal 5> ***

Proposal 8: For an NR-U UE scheduled with a PUSCH on multiple RB sets and indicated with type 1 channel access, if the channel frequencies of the corresponding set of channels C is NOT a subset of one of the sets of channel frequencies defined in subclause 5.7.4 in [2], the UE performs type 1 channel access procedure on each channel individually. Proposal 9：Adopt TP6 into section 4.2.1.0.4 of TS 37.213. ## *** <Beginning of Text Proposal 6> *** ##### 4.2.1.0.4 Channel access procedures for UL multi-channel transmission(s) If a UE - is scheduled to transmit on a set of channels C, and if Type 1 channel access procedure is indicated by the UL scheduling grant for the UL transmission on the set of channels C, or - is scheduled to transmit on a set of channels C, and if Type 1 channel access procedure is indicated by the UL scheduling grants for the UL transmissions on the set of channels C, and if the UL transmissions are scheduled to start transmissions at the same time on all channels in the set of channels C , or - intends to perform an uplink transmission on a configured resources on the set of channels C with Type 1 channel access procedure, or - intends to perform an uplink transmission on configured resources on the set of channels C with Type 1 channel access procedure, and if UL transmissions are configured to start transmissions on the same time all channels in the set of channels C, and if the channel frequencies of set of channels C is a subset of one of the sets of channel frequencies defined in subclause 5.7.4 in [2] - the UE may transmit on channel ci∈C using Type 2 channel access procedure as described in subclause 4.2.1.2, - if Type 2 channel access procedure is performed on channel ci immediately before the UE transmission on channel cj∈C, i≠j, and - if the UE has accessed channel cj using Type 1 channel access procedure as described in subclause 4.2.1.1, - where channel cj is selected by the UE uniformly randomly from the set of channels C before performing Type 1 channel access procedure on any channel in the set of channels C. - the UE may not transmit on channel ci∈C within the bandwidth of a carrier, if the UE fails to access any of the channels, of the carrier bandwidth, on which the UE is scheduled or configured by UL resources. if the channel frequencies of set of channels C is not a subset of one of the sets of channel frequencies defined in subclause 5.7.4 in [2], the UE may transmit on channel ci∈C using type 1 channel access procedure as described in subclause 4.2.1.1. ## *** Unchanged text is omitted *** ## *** <End of Text Proposal 6> ***

R1-2006301 discusses also related aspects and proposes.

Proposal #3: Reflect the followings in TS 37.213: - For UL active BWP configured with no intra-cell guard band, a UE is allowed to transmit UL transmission only if the UE succeeds LBT for all RB set(s) corresponding to the UL BWP. - For DL, if gNB transmits DL transmission to a UE configured with DL active BWP where intraCellGuardBandDL-r16 for the corresponding serving cell indicates to the UE that no intra-cell guard-bands are configured, gNB is allowed to transmit DL transmission to the UE only if gNB succeeds LBT for the whole DL BWP.

Companies are asked to provide their views related to the TPs and proposals above with the table below:

| Company | Comment |
| --- | --- |

# 6. Editorial Issues

Editorial Issues:

| #1 | order of ChannelAccess-CPext field in DCI 0_0 | R1- 2005333 (p2) |
| --- | --- | --- |

R1- 2005333 raises the issue of the order of DCI fields in DCI 0_0.

In DCI format 0_0, the “UL/SUL indicator” is meant to be located in the last bit position of DCI format 0_0. However, in current spec, the “ChannelAccess-CPext” is added after the “UL/SUL indicator”. Therefore, the “ChannelAccess-CPext” should be moved to the front of “UL/SUL indicator” as shown in text proposal 2. ----------------------------------------------- Start of text proposal 2 ------------------------------------------------------ TS 38.212 ##### 7.3.1.1.1 Format 0_0 <unchanged text omitted> The following information is transmitted by means of the DCI format 0_0 with CRC scrambled by C-RNTI or CS-RNTI or MCS-C-RNTI: …… - ChannelAccess-CPext – 2 bits indicating combinations of channel access type and CP extension as defined in Table 7.3.1.1.1-4 for operation in a cell with shared spectrum channel access; 0 bit otherwise. - UL/SUL indicator – 1 bit for UEs configured with supplementaryUplink in ServingCellConfig in the cell as defined in Table 7.3.1.1.1-1 and the number of bits for DCI format 1_0 before padding is larger than the number of bits for DCI format 0_0 before padding; 0 bit otherwise. The UL/SUL indicator, if present, locates in the last bit position of DCI format 0_0, after the padding bit(s). - If the UL/SUL indicator is present in DCI format 0_0 and the higher layer parameter pusch-Config is not configured on both UL and SUL the UE ignores the UL/SUL indicator field in DCI format 0_0, and the corresponding PUSCH scheduled by the DCI format 0_0 is for the UL or SUL for which high layer parameter pucch-Config is configured; - If the UL/SUL indicator is not present in DCI format 0_0 and pucch-Config is configured, the corresponding PUSCH scheduled by the DCI format 0_0 is for the UL or SUL for which high layer parameter pucch-Config is configured. - If the UL/SUL indicator is not present in DCI format 0_0 and pucch-Config is not configured, the corresponding PUSCH scheduled by the DCI format 0_0 is for the uplink on which the latest PRACH is transmitted. …… The following information is transmitted by means of the DCI format 0_0 with CRC scrambled by TC-RNTI: - ChannelAccess-CPext – 2 bits indicating combinations of channel access type and CP extension as defined in Table 7.3.1.1.1-4 for operation in a cell with shared spectrum channel access; 0 bit otherwise - UL/SUL indicator – 1 bit if the cell has two ULs and the number of bits for DCI format 1_0 before padding is larger than the number of bits for DCI format 0_0 before padding; 0 bit otherwise. The UL/SUL indicator, if present, locates in the last bit position of DCI format 0_0, after the padding bit(s). - If 1 bit, reserved, and the corresponding PUSCH is always on the same UL carrier as the previous transmission of the same TB <unchanged text omitted> ----------------------------------------------- End of text proposal 2 --------------------------------------------------------

Companies are asked to provide their views related to the TPs and proposals above with the table below:

| Company | Comment |
| --- | --- |

# 7. Conclusions

TBA

# References

| 1 | R1-2005333 | Remaining issues on the channel access procedures | vivo |
| --- | --- | --- | --- |
| 2 | R1-2005600 | Remaining issues on the channel access procedure for NR-U | ZTE, Sanechips |
| 3 | R1-2005809 | Maintenance on channel access procedures | Huawei, HiSilicon |
| 4 | R1-2005914 | Channel access procedures | Ericsson |
| 5 | R1-2006020 | Discussion on the remaining issues of channel access procedure | OPPO |
| 6 | R1-2006095 | Channel access procedures for NR-U | Samsung |
| 7 | R1-2006301 | Remaining issues of channel access procedure for NR-U | LG Electronics |
| 8 | R1-2006351 | Remaining issues on channel access procedures for NR-U | ETRI |
| 9 | R1-2006370 | Remaining Issues on Channel Access Procedures for NR-U | Nokia, Nokia Shanghai Bell |
| 10 | R1-2006763 | TP for Channel access procedures for NR unlicensed | Qualcomm Incorporated |
| 11 | R1-2006881 | Remaining issues on channel access procedure for NR-U | WILUS Inc. |