[102-e-NR-5G_V2X_NRSL-PHYstructure-02] Email discussion/approval w.r.t. “max data rate for SL and TBS”: 2A, 3A, 3B (as in the summary) by 08/20, with potential TPs by 8/25 – Jeongho (Samsung)

This document has the following questions.

A. Max data rate definition for SL

B. 2nd SCI overhead for TBS determination

C. MCS restriction for TBS determination

A. Max data rate definition for SL

This is to handle the LS from RAN2, which is found in R1-2003258. The related contributions submitted to RAN1#102-e are [vivo], [LGE], [Huawei, HiSilicon], [Samsung], [Apple], [OPPO, R1-2006001].

In TS38,306, the max data rate for NR is defined. For the max data rate for sidelink, can the existing data rate formula be reused? That is to reuse the following formula and modification:

data rate (in Mbps)=10-6⋅vLayers⋅Qm⋅f⋅Rmax⋅NPRBBW,μ⋅12Tsμ⋅1-OH	(2)

wherein

Rmax = 948/1024

vLayers is the maximum number of supported layers by higher layer parameter.

Qm is the maximum supported modulation order by higher layer parameter.

f is the scaling factor, which is 1.

μ is the numerology (as defined in TS 38.211).

Tsμ is the average OFDM symbol duration in a subframe for numerology μ, i.e. Tsμ=10-314⋅2μ.

Note that normal cyclic prefix is assumed.

NPRBBW,μ is the maximum RB allocation in bandwidth BW with numerology μ, as defined in TS 38.101-1 and TS 38.101-2, where BW is the UE supported maximum bandwidth for a given band.

OH is the overhead and takes the following values

x, for frequency range FR1 for SL

y, for frequency range FR2 for SL

The modifications from NR Uu are as below:

- The max number of layers is depending on UE capability.

- Qm is depending on UE capability.

- f is fixed as 1.

- OH is as x for FR1 and y for FR2.

Please provide the above definition for the max data rate for SL and also provide the values of x and y for OH in sidelink.

If further modification is needed, please provide the views and reason.

If other alternative is needed, please provide the views and reason.

| Company | Views |
| --- | --- |

B. 2nd SCI overhead for TBS determination

From the agreement in RAN1#101-e, the actual number of REs for 2nd SCI is considered for TBS determination with having the following:

-	NRESCI,2 is the number of coded modulation symbols generated for 2nd-stage SCI transmission (prior to duplication for the 2nd layer, if present) according to Clause 8.4.4 of [5, TS 38.212].

A number of contributions [ZTE, Sanechips], [vivo], [CATT], [Huawei, HiSilicon], [Intel], [InterDigital], [Samsung], [Ericsson], [Apple], [Panasonic], [NTT DCM] proposed some modifications for the overhead consideration of 2nd SCI.

Please provide your views on the following alternatives:

- Alt 1. No changes

- Alt 2. Assuming the gamma as zero for the 2nd SCI

- Alt 3. Other modification(s), if any.

| Company | Views |
| --- | --- |

C. MCS restriction for TBS determination

There are two contributions [Intel], [Sharp] for MCS restriction: one is proposing not to use the code point only indicating Qm, and another is proposing not to change MCS table between initial transmission and retransmission.

Please provide your views on the following alternatives:

Alt 1. No need to restrict

Alt 2. Not use the MCS code point only indicating the modulation order

Alt 3. Not change MCS table between initial transmission and retransmission.

Alt 4. Other restriction(s), if any.

| Company | Views |
| --- | --- |

# Reference

- R1-2005292	Remaining details on physical layer structure for the sidelink	FUTUREWEI

- R1-2005307	Remaining issues of NR sidelink physical layer structure	ZTE, Sanechips

- R1-2005338	Remaining issues on physical layer structure for NR sidelink	vivo

- R1-2005646	Discussion on sidelink physical layer structure	MediaTek Inc.

- R1-2005667	Remaining issues on physical layer structure for NR sidelink	CATT

- R1-2005740	Discussion on essential corrections in physical layer structure	LG Electronics

- R1-2005761	TP on 1st symbol duplication for AGC	NEC

- R1-2005786	Remaining issues of V2X PHY layer structure	Mitsubishi Electric RCE

- R1-2005796	Remaining details of sidelink physical layer structure	Huawei, HiSilicon

- R1-2005846	Remaining opens for NR-V2X sidelink physical layer structure	Intel Corporation

- R1-2005997	Remaining issues on physical structure for NR sidelink	OPPO

- R1-2006074	Remaining issues on PHY structure for NR V2X	InterDigital, Inc.

- R1-2006099	On Physical Layer Structures for NR Sidelink	Samsung

- R1-2006254	Remaining issues for sidelink physical layer structure	Spreadtrum Communications

- R1-2006433	TPs related to PHY structures	Ericsson

- R1-2006484	On Remaining Issues of Sidelink Physical Layer Structure	Apple

- R1-2006535	Remaining issue on physical layer structure for sidelink in NR V2X	Panasonic Corporation

- R1-2006557	Remaining issues on physical layer structure for NR sidelink	Sharp

- R1-2006584	Remaining issues on sidelink physical layer structure on NR V2X	ASUSTeK

- R1-2006693	Maintenance for sidelink physical layer structure	NTT DOCOMO, INC.

- R1-2006768	Sidelink Physical Layer Structure	Qualcomm Incorporated