3GPP TSG RAN WG1 #102-e Meeting	DRAFT_R1-200xxxx

e-Meeting, August 17th – 28th, 2020

Agenda item:	7.2.4.1

Source: 	Moderator (Samsung)

Title: 	Text Proposals for PSFCH sequence related

Document for:	Endorsement

Introduction

During WI and RAN1#102-e meeting, the following were agreed.

Agreements: For group and sequence hopping for PSFCH, the following is used. - u = n_ID mod 30 and v = 0, where - n_ID is given by hoppingID_PSFCH when (pre-)configured; if not (pre-)configured, n_ID = 0 Agreements: For PSFCH sequence generation, m_int=0 is used. Agreements: For the initialization of c(i) for the calculation of n_cs for PSFCH sequence, the following is used. - C_init = hoppingID_PSFCH when (pre)-configured; if not (pre-)configured, c_init=0

Text Proposal for TS38.211

----------------------------------Begin of text proposal for 38.212 Section 8.3.1.1---------------------------------

- 8.3.4.2.1	Sequence generation

The sequence xn shall be generated according to

xn=ru,vα,δn

n=0,1,…,NscRB-1

where ru,vα,δ(n) is given by clause 6.3.2.2 with the following exceptions:

-	mcs is given by clause 16.3  of [5, TS 38.213];

-	m0 is given by clause 16.3 of [5, TS 38.213];

-	l is the OFDM symbol number in the PSFCH transmission where l=0 corresponds to the first OFDM symbol of the PSFCH transmission;

-	l' is the index of the OFDM symbol in the slot that corresponds to the first OFDM symbol of the PSFCH transmission in the slot given by [5, TS 38.213]

-	u=nID mod 30 and ν=0, where nID is given by higher layer parameter sl-PSFCH-HopID when (pre-)configured; if not (pre-)configured, nID=0.

-	cinit=nID, where nID is given by higher layer parameter sl-PSFCH-HopID when (pre-)configured; if not (pre-)configured, cinit=0.

-	mint=0

----------------------------------End of text proposal for 38.214 Section 8.3.1.1---------------------------------