[102-e-NR-5G_V2X_NRSL-PHYstructure-03] Email discussion/approval w.r.t. “PSFCH sequence related”: 4A (as in the summary) by 08/20, with potential TPs by 8/25 – Jeongho (Samsung)

The current text for PSFCH in TS38.211 is as below.

#### 8.3.4.2 PSFCH format 0 ##### 8.3.4.2.1 Sequence generation The sequence xn shall be generated according to xn=ru,vα,δn n=0,1,…,NscRB-1 where ru,vα,δ(n) is given by clause 6.3.2.2 with the following exceptions: - mcs is given by clause 16.3 of [5, TS 38.213]; - m0 is given by clause 16.3 of [5, TS 38.213]; - l is the OFDM symbol number in the PSFCH transmission where l=0 corresponds to the first OFDM symbol of the PSFCH transmission; - l' is the index of the OFDM symbol in the slot that corresponds to the first OFDM symbol of the PSFCH transmission in the slot given by [5, TS 38.213]

In RAN1#102-e, the five contributions [CATT], [Huawei, HiSilicon], [OPPO], [Sharp], [NTT DCM] raised the issues related to PSFCH sequence.

With the current specifications, it is necessary to define u, v, and α. In order to define those parameters, the followings are to be discussed in this thread.

- Group/sequence hopping

  - A1. Depending on whether to be supported or not, how to define the hopping parameter u and v

- For alpha,

  - B1. How to define m_init

  - B2 How to define c_init

A1. How to define u and v

Two contributions propose to use u = n_ID mod 30, where n_ID is given by hoppingID_PSFCH and v=0.

(This is the same as replacing hoppingId with hoppingID_PSFCH and pucch_GroupHopping=’neither’.)

Please provide your views and reason.

| Company | Views |
| --- | --- |

B1. How to define m_init

minit is introduced for interlaced PUCCH format. Therefore, minit=0 can be used for PSFCH.

Please provide your views and reason.

| Company | Views |
| --- | --- |

B2. How to define c_init

The remaining issue is how to define the pseudo-random sequence  used for .

Three contributions propose to the following alternative to define c(i).

- Alt 1. C_init = hoppingID_PSFCH

- Alt 2. Depending on the source ID

- Alt 3. Depending on the CRC of the PSSCH

Please provide your views and reason.

| Company | Views |
| --- | --- |

# Reference

- R1-2005292	Remaining details on physical layer structure for the sidelink	FUTUREWEI

- R1-2005307	Remaining issues of NR sidelink physical layer structure	ZTE, Sanechips

- R1-2005338	Remaining issues on physical layer structure for NR sidelink	vivo

- R1-2005646	Discussion on sidelink physical layer structure	MediaTek Inc.

- R1-2005667	Remaining issues on physical layer structure for NR sidelink	CATT

- R1-2005740	Discussion on essential corrections in physical layer structure	LG Electronics

- R1-2005761	TP on 1st symbol duplication for AGC	NEC

- R1-2005786	Remaining issues of V2X PHY layer structure	Mitsubishi Electric RCE

- R1-2005796	Remaining details of sidelink physical layer structure	Huawei, HiSilicon

- R1-2005846	Remaining opens for NR-V2X sidelink physical layer structure	Intel Corporation

- R1-2005997	Remaining issues on physical structure for NR sidelink	OPPO

- R1-2006074	Remaining issues on PHY structure for NR V2X	InterDigital, Inc.

- R1-2006099	On Physical Layer Structures for NR Sidelink	Samsung

- R1-2006254	Remaining issues for sidelink physical layer structure	Spreadtrum Communications

- R1-2006433	TPs related to PHY structures	Ericsson

- R1-2006484	On Remaining Issues of Sidelink Physical Layer Structure	Apple

- R1-2006535	Remaining issue on physical layer structure for sidelink in NR V2X	Panasonic Corporation

- R1-2006557	Remaining issues on physical layer structure for NR sidelink	Sharp

- R1-2006584	Remaining issues on sidelink physical layer structure on NR V2X	ASUSTeK

- R1-2006693	Maintenance for sidelink physical layer structure	NTT DOCOMO, INC.

- R1-2006768	Sidelink Physical Layer Structure	Qualcomm Incorporated

# Appendix. Sequence and CS hopping for PUCCH in TS38.211

#### 6.3.2.2 Sequence and cyclic shift hopping PUCCH formats 0, 1, 3, and 4 use sequences ru,vα,δ(n) given by clause 5.2.2 with δ=0 where the sequence group u and the sequence number v depend on the sequence hopping in clause 6.3.2.2.1 and the cyclic shift α depends on the cyclic shift hopping in clause 6.3.2.2.2. ##### 6.3.2.2.1 Group and sequence hopping The sequence group u=fgh+fss mod 30 and the sequence number v within the group depends on the higher-layer parameter pucch-GroupHopping: - if pucch-GroupHopping equals 'neither' where is given by the higher-layer parameter hoppingId if configured, otherwise nID=NIDcell. - if pucch-GroupHopping equals 'enable' where the pseudo-random sequence is defined by clause 5.2.1 and shall be initialized at the beginning of each radio frame with where is given by the higher-layer parameter hoppingId if configured, otherwise nID=NIDcell. - if pucch-GroupHopping equals 'disable' where the pseudo-random sequence is defined by clause 5.2.1 and shall be initialized at the beginning of each radio frame with cinit=25nID30+nID mod 30 where is given by the higher-layer parameter hoppingId if configured, otherwise nID=NIDcell. The frequency hopping index nhop=0 if intra-slot frequency hopping is disabled by the higher-layer parameter intraSlotFrequencyHopping. If frequency hopping is enabled by the higher-layer parameter intraSlotFrequencyHopping, for the first hop and for the second hop. ##### 6.3.2.2.2 Cyclic shift hopping The cyclic shift α varies as a function of the symbol and slot number according to αl=2πNscRBm0+mcs+mint+ncsns,fμ,l+l' mod NscRB where - ns,fμ is the slot number in the radio frame - l is the OFDM symbol number in the PUCCH transmission where l=0 corresponds to the first OFDM symbol of the PUCCH transmission, - l' is the index of the OFDM symbol in the slot that corresponds to the first OFDM symbol of the PUCCH transmission in the slot given by [5, TS 38.213] - is given by [5, TS 38.213] for PUCCH format 0 and 1 while for PUCCH format 3 and 4 is defined in clause 6.4.1.3.3.1 - except for PUCCH format 0 when it depends on the information to be transmitted according to clause 9.2 of [5, TS 38.213]. - mint is given by - mint=5nIRBμ for PUCCH formats 0 and 1 if PUCCH shall use interlaced mapping according to any of the higher-layer parameter useInterlacePUCCH-Common-r16 or useInterlacePUCCH-Dedicated-r16, where nIRBμ is the resource block number within the interlace; - mint=0 otherwise The function is given by ncsns,fμ,l=m=072mc8Nsymbslotns,fμ+8l+m where the pseudo-random sequence is defined by clause 5.2.1. The pseudo-random sequence generator shall be initialized with , where is given by the higher-layer parameter hoppingId if configured, otherwise nID=NIDcell.