[102-e-NR-5G_V2X_NRSL-PHYstructure-03] Email discussion/approval w.r.t. “PSFCH sequence related”: 4A (as in the summary) by 08/20, with potential TPs by 8/25 – Jeongho (Samsung)

The current text for PSFCH in TS38.211 is as below.

#### 8.3.4.2 PSFCH format 0 ##### 8.3.4.2.1 Sequence generation The sequence xn shall be generated according to xn=ru,vα,δn n=0,1,…,NscRB-1 where ru,vα,δ(n) is given by clause 6.3.2.2 with the following exceptions: - mcs is given by clause 16.3 of [5, TS 38.213]; - m0 is given by clause 16.3 of [5, TS 38.213]; - l is the OFDM symbol number in the PSFCH transmission where l=0 corresponds to the first OFDM symbol of the PSFCH transmission; - l' is the index of the OFDM symbol in the slot that corresponds to the first OFDM symbol of the PSFCH transmission in the slot given by [5, TS 38.213]

In RAN1#102-e, the five contributions [CATT], [Huawei, HiSilicon], [OPPO], [Sharp], [NTT DCM] raised the issues related to PSFCH sequence.

With the current specifications, it is necessary to define u, v, and α. In order to define those parameters, the followings are to be discussed in this thread.

- Group/sequence hopping

  - A1. Depending on whether to be supported or not, how to define the hopping parameter u and v

- For alpha,

  - B1. How to define m_init

  - B2 How to define c_init

A1. How to define u and v

Two contributions propose to use u = n_ID mod 30, where n_ID is given by hoppingID_PSFCH and v=0.

(This is the same as replacing hoppingId with hoppingID_PSFCH and pucch_GroupHopping=’neither’.)

Please provide your views and reason.

| Company | Views |
| --- | --- |
| NTT DOCOMO | Support; i.e. hoppingId is replaced with hoppingID_PSFCH and pucch_GroupHopping = neither. Performance gain of sequence/group hopping has not identified so far. neither should be OK. |
| OPPO | Agree. |
| Qualcomm | Agree with the proposal from Docomo. The benefit of sequence or group hopping in the context of PSFCH has not been shown. |
| Sharp | Support in principle. In TS38.211, for the case when hoppingId is absent, n_ID=cell ID. While a cell ID is not always available, e.g. for an OOC SL UE. Thus, besides replacing hoppingId with hoppingID_PSFCH(and v=0), we propose that if hoppingID_PSFCH is not configured, n_ID=source ID. |
| LG Electronics | We are supportive that hoppingID_PSFCH replace hoppingId. According to the following agreement, we do not need to define default value for u. Agreements made in RAN1#99: - CDM between PSFCH transmissions from different UEs in the same PRB is supported as follows: - Cyclic shift can be selected based on - the L1 source ID of TX UE for unicast and groupcast option 1. - the L1 source ID of TX UE and the member ID of RX UE in groupcast option 2. - FFS whether or not to use additional parameter(s) for the selection (e.g., using PSCCH DM-RS, etc) to conclude this week - Base sequence is - (Pre-)configured per resource pool Since we do not have replacement of pucch_GroupHopping, we are supportive that pucch_GroupHopping=’neither’ for PSFCH. In our understanding, the important thing is that the same base sequence will be used by all the UEs in a resource pool. |
| ZTE, Sanechips | Support. No need to support sequence/group hopping given no discussion/consensus on the gain. |
| vivo | Agreed in principle, with the following comments: - It seems RAN2 define it in a slightly different name: SL-PSFCH-Config-r16 ::= SEQUENCE { sl-PSFCH-Period-r16 ENUMERATED {sl0, sl1, sl2, sl4} OPTIONAL, -- Need M sl-PSFCH-RB-Set-r16 BIT STRING (SIZE (10..275)) OPTIONAL, -- Need M sl-NumMuxCS-Pair-r16 ENUMERATED {n1, n2, n3, n6} OPTIONAL, -- Need M sl-MinTimeGapPSFCH-r16 ENUMERATED {sl2, sl3} OPTIONAL, -- Need M sl-PSFCH-HopID-r16 INTEGER (0..1023) OPTIONAL, -- Need M sl-PSFCH-CandidateResourceType-r16 ENUMERATED {startSubCH, allocSubCH} OPTIONAL, -- Need M ... } - As commented by Sharp, this parameter sl-PSFCH-HopID is optional without default value in RRC. Then RAN1 may have to handle the case of sl-PSFCH-HopID is not configured. But the proposal from Sharp is a little confusing to us on what the “source ID” is. Does it refer to the source ID of the UE transmitting the PSFCH? An alternative is to define a default value for n_ID. |
| Huawei, HiSilicon | We are supportive for the proposal in principle, n_ID can be given by hoppingID_PSFCH and v=0, but if the hoppingID_PSFCH is not configured, CRC of associated PSSCH can be used. This is because the cell ID, which is used when hoppingId is absent in NR Uu, is not always available in sidelink. |
| Ericsson | We are supportive of the proposal from DCM. Moreover, for the case whenhoppingID_PSFCH is not (pre)configured, we believe n_ID=source ID can be used, where source ID is the ID of UE transmitting PSSCH. |
| CATT | Agree |
| Samsung | Agree. Also, as the agreement captured by LGE, the base sequence is (pre-)configured per resource pool. So, there is no case such that hoppingID_PSFCH is not configured. |
| Intel | Agree w/ proposal |
| Futurewei | Agree with the proposal. When whenhoppingID_PSFCH is not (pre)configure, a value different for each PSFCH needs to be used in order to reduce interference. We do not have strong view on which value to use (source, ID, etc) |
| Apple | Agree with the proposal. We do not need to apply group and sequence hopping for PSFCH. |
| Nokia, NSB | Agree. Sequence/group hopping has not been discussed. |

Summary

Based on the majority, the higher layer parameter hoppingID_PSFCH can be used for u. Then the remaining issue is the case that the higher layer parameter hoppingID_PSFCH is not configured for the resource pool. In that case, RAN1 needs further discussion on the alternatives.

Proposal 1

For group and sequence hopping for PSFCH, the following is used.

- u = n_ID mod 30 and v = 0, where

  - n_ID is given by hoppingID_PSFCH when configured.

- RAN1 further discusses for the case that hoppingID_PSFCH is not configured with

  - Alt 1. Its error case. (i.e., hoppingID_PSFCH should be configured.)

  - Alt 2. n_ID = fixed value (e.g., 1010, 0)

  - Alt 3. n_ID = source ID (which is the source ID of UE transmitting the associated PSSCH)

  - Alt 4. n_ID = CRC of the associated PSSCH

B1. How to define m_init

minit is introduced for interlaced PUCCH format. Therefore, minit=0 can be used for PSFCH.

Please provide your views and reason.

| Company | Views |
| --- | --- |
| NTT DOCOMO | Support; i.e. minit=0 No spec update is assumed. |
| OPPO | minit=0 |
| Qualcomm | Agree with the proposal |
| Sharp | Support |
| LG Electronics | We support minit=0. This parameter is used for unlicensed band operation. |
| ZTE, Sanechips | Support |
| vivo | Agree with the proposal. |
| Huawei, HiSilicon | Agree. |
| Ericsson | Support |
| CATT | Agree |
| Samsung | Agree |
| Intel | Agree w/ proposal |
| Futurewei | Support |
| Apple | Agree |
| Nokia, NSB | Agree |

Summary

Based on the majority views, the following is proposed.

Proposal 2

For PSFCH sequence generation, mint=0 is used.

B2. How to define c_init

The remaining issue is how to define the pseudo-random sequence  used for .

Three contributions propose to the following alternative to define c(i).

- Alt 1. C_init = hoppingID_PSFCH

- Alt 2. Depending on the source ID

- Alt 3. Depending on the CRC of the PSSCH

Please provide your views and reason.

| Company | Views |  |
| --- | --- | --- |
| NTT DOCOMO | Alt 1. Reuse NR-Uu. |  |
| OPPO | Alt 1 |  |
| Qualcomm | Alt 1. The benefit of other alternatives has not been demonstrated. |  |
| Sharp | Alt 1 for hoppingID_PSFCH is configured, otherwise Alt2. For presence of hoppingID_PSFCH, we support to reuse NR Uu, i.e. Alt1, for absence of hoppingID_PSFCH, Alt 2 is preferred. |  |
| LG Electronics | We are supportive of Alt 1 with default value of 1010. Randomization across different UEs shall not be applied to c(i) for n_CS. When different n_CS values are used across different UEs, CDM between PSFCH with different m_0 is no longer valid. To be specific, it can make the gap between alpha values of different PSFCH resources smaller than target delay spread, and the orthogonality coming from CS will be lost. If the default value of n_ID is needed, we can consider that 1010 replace NIDcell as in other sequence generation discussion. |  |
| ZTE, Sanechips | Alt 1 |  |
| vivo | Alt 1 if sl-PSFCH-HopID is configured. In the case of sl-PSFCH-HopID is not configured, we prefer to use a default value instead, e.g., n_ID = 0. Comment to Alt 2: Does the “source ID” refer to the source ID of the UE transmitting the PSFCH? |  |
| Huawei, HiSilicon | Alt. 3. The pseudo-random sequence generator shall be initialized with, where is the CRC of the associated PSSCH. |
| Ericsson | We support Alt 1 if hoppingID_PSFCH is configured, otherwise Alt2 (C_init = source ID of UE transmitting PSSCH). |
| CATT | Alt 1 |
| Samsung | Alt 1. No need to support further randomization methods without the verification on performance gain. |
| Intel | Support Alt.1 |
| Futurewei | Alt. 1 |
| Apple | Alt. 1 |
| Nokia, NSB | Alt 1 |

Summary

Based on the majority, the higher layer parameter hoppingID_PSFCH can be used. Then, similar to A1, the remaining issue is the case that the higher layer parameter hoppingID_PSFCH is not configured for the resource pool. In that case, RAN1 needs further discussion on the alternatives.

Proposal 3

For the initialization of c(i) for the calculation of n_cs for PSFCH sequence, the following is used.

- C_init = hoppingID_PSFCH when configured.

- RAN1 further discusses for the case that hoppingID_PSFCH is not configured with

  - Alt 1. Its error case. (i.e., hoppingID_PSFCH should be configured.)

  - Alt 2. C_init = fixed value (e.g., 1010, 0)

  - Alt 3. C_init = source ID (which is the source ID of UE transmitting the associated PSSCH)

  - Alt 4. C_init = CRC of the associated PSSCH

# Reference

- R1-2005292	Remaining details on physical layer structure for the sidelink	FUTUREWEI

- R1-2005307	Remaining issues of NR sidelink physical layer structure	ZTE, Sanechips

- R1-2005338	Remaining issues on physical layer structure for NR sidelink	vivo

- R1-2005646	Discussion on sidelink physical layer structure	MediaTek Inc.

- R1-2005667	Remaining issues on physical layer structure for NR sidelink	CATT

- R1-2005740	Discussion on essential corrections in physical layer structure	LG Electronics

- R1-2005761	TP on 1st symbol duplication for AGC	NEC

- R1-2005786	Remaining issues of V2X PHY layer structure	Mitsubishi Electric RCE

- R1-2005796	Remaining details of sidelink physical layer structure	Huawei, HiSilicon

- R1-2005846	Remaining opens for NR-V2X sidelink physical layer structure	Intel Corporation

- R1-2005997	Remaining issues on physical structure for NR sidelink	OPPO

- R1-2006074	Remaining issues on PHY structure for NR V2X	InterDigital, Inc.

- R1-2006099	On Physical Layer Structures for NR Sidelink	Samsung

- R1-2006254	Remaining issues for sidelink physical layer structure	Spreadtrum Communications

- R1-2006433	TPs related to PHY structures	Ericsson

- R1-2006484	On Remaining Issues of Sidelink Physical Layer Structure	Apple

- R1-2006535	Remaining issue on physical layer structure for sidelink in NR V2X	Panasonic Corporation

- R1-2006557	Remaining issues on physical layer structure for NR sidelink	Sharp

- R1-2006584	Remaining issues on sidelink physical layer structure on NR V2X	ASUSTeK

- R1-2006693	Maintenance for sidelink physical layer structure	NTT DOCOMO, INC.

- R1-2006768	Sidelink Physical Layer Structure	Qualcomm Incorporated

# Appendix. Sequence and CS hopping for PUCCH in TS38.211

#### 6.3.2.2 Sequence and cyclic shift hopping PUCCH formats 0, 1, 3, and 4 use sequences ru,vα,δ(n) given by clause 5.2.2 with δ=0 where the sequence group u and the sequence number v depend on the sequence hopping in clause 6.3.2.2.1 and the cyclic shift α depends on the cyclic shift hopping in clause 6.3.2.2.2. ##### 6.3.2.2.1 Group and sequence hopping The sequence group u=fgh+fss mod 30 and the sequence number v within the group depends on the higher-layer parameter pucch-GroupHopping: - if pucch-GroupHopping equals 'neither' where is given by the higher-layer parameter hoppingId if configured, otherwise nID=NIDcell. - if pucch-GroupHopping equals 'enable' where the pseudo-random sequence is defined by clause 5.2.1 and shall be initialized at the beginning of each radio frame with where is given by the higher-layer parameter hoppingId if configured, otherwise nID=NIDcell. - if pucch-GroupHopping equals 'disable' where the pseudo-random sequence is defined by clause 5.2.1 and shall be initialized at the beginning of each radio frame with cinit=25nID30+nID mod 30 where is given by the higher-layer parameter hoppingId if configured, otherwise nID=NIDcell. The frequency hopping index nhop=0 if intra-slot frequency hopping is disabled by the higher-layer parameter intraSlotFrequencyHopping. If frequency hopping is enabled by the higher-layer parameter intraSlotFrequencyHopping, for the first hop and for the second hop. ##### 6.3.2.2.2 Cyclic shift hopping The cyclic shift α varies as a function of the symbol and slot number according to αl=2πNscRBm0+mcs+mint+ncsns,fμ,l+l' mod NscRB where - ns,fμ is the slot number in the radio frame - l is the OFDM symbol number in the PUCCH transmission where l=0 corresponds to the first OFDM symbol of the PUCCH transmission, - l' is the index of the OFDM symbol in the slot that corresponds to the first OFDM symbol of the PUCCH transmission in the slot given by [5, TS 38.213] - is given by [5, TS 38.213] for PUCCH format 0 and 1 while for PUCCH format 3 and 4 is defined in clause 6.4.1.3.3.1 - except for PUCCH format 0 when it depends on the information to be transmitted according to clause 9.2 of [5, TS 38.213]. - mint is given by - mint=5nIRBμ for PUCCH formats 0 and 1 if PUCCH shall use interlaced mapping according to any of the higher-layer parameter useInterlacePUCCH-Common-r16 or useInterlacePUCCH-Dedicated-r16, where nIRBμ is the resource block number within the interlace; - mint=0 otherwise The function is given by ncsns,fμ,l=m=072mc8Nsymbslotns,fμ+8l+m where the pseudo-random sequence is defined by clause 5.2.1. The pseudo-random sequence generator shall be initialized with , where is given by the higher-layer parameter hoppingId if configured, otherwise nID=NIDcell.