3GPP TSG RAN WG1 Meeting #102-e	  R1-2xxxxxx

E-meeting, August 17-28, 2020

Agenda Item:	7.2.5.1

Source:	Moderator (Huawei)

Title:	Email discussion/approval [102-e-NR-L1enh-URLLC-PDCCH enhancements-01] on remaining issues on DCI format design

Document for:	Discussion and Decision

Introduction

The email discussion is to discuss the remaining issues on DCI format design.

[102-e-NR-L1enh-URLLC-PDCCH enhancements-01] Email discussion/approval on remaining issues on DCI format design – Chengyan (Huawei)

- Issue A-1: Remaining issue on DCI size alignment in TS 38.212

- Issue A-2: Type2 HARQ-ACK codebook construction related to DAI bit width

- Discussion/Agreement by 8/21 and TPs by 8/28

This document summarizes the above issue and provide some initial proposals for discussion. Companies are encouraged to provide the first round views by 8/18, then we can adjust the proposals and prepare the TPs for the next step discussions.

DCI format scheduling Rel-16 URLLC

Based on the contributions from companies, the following issues related to DCI format design are discussed.

- Issue A-1: Remaining issue on DCI size alignment due to the introduction of DCI format 0_2/1_2

In RAN1#101-e meeting, DCI size alignment was discussed and the following agreements were made:

Agreement

- A UE is not expected to monitor a first decoding candidate with DCI format 0_0/1_0  and a second candidate with DCI format 0_2/1_2, where the two decoding candidates are mapped to the same resource and the DCI formats 0_0/1_0 and 0_2/1_2 have the same size.

- A UE is not expected to monitor a first decoding candidate with DCI format 0_1/1_1  and a second candidate with DCI format 0_2/1_2, where the two decoding candidates are mapped to the same resource and the DCI formats 0_1/1_1 and 0_2/1_2 have the same size.

However, the specification is not complete for the above agreement, since there is no consensus on the following two bullets:

- There is no need for gNB to ensure different DCI size for DCI format 0_1 and DCI format 1_2

- There is no need for gNB to ensure different DCI size for DCI format 1_1 and DCI format 0_2

Some companies provide views on the above issue in the contribution and the position is summarized as below:

  - Support: (i.e. there is no need for gNB to ensure different DCI size for DCI format 0_1 and DCI format 1_2, and there is no need for gNB to ensure different DCI size for DCI format 1_1 and DCI format 0_2)

    - Ericsson, Intel, ZTE, Huawei, HiSilicon, CATT

  - Reasons

    - The “Identifier for DCI formats” field in DCI formats can always be used to differentiate UL DCI format and DL DCI format, therefore no need for gNB to ensure different DCI size.

    - If we force different sizes for DL and UL, it will introduce draw backs like increasing the DCI size unnecessary, increasing more difficulty at gNB side to ensure different size unnecessary, etc.

    - The current agreement exactly means that the DCI size alignment is only performed between DL DCI formats or UL DCI formats. The text in the RAN1 agreement is consistent only if “0_x/1_x” is interpreted as “0_x and 1_x, respectively”, and not if interpreted as “x_0 and x_1”. This is because a decoding candidate cannot correspond to more than one DCI format, unless they are of the same size, and if they are of the same size, then it the issue is moot.

  - Note support: any DCI format of 0_1 and 1_1 cannot be size-aligned with any DCI of 0_2 and 1_2

    - Qualcomm

  - Reasons

    - Simplifies the UE implementation, as it allows the UE to determine the DCI format (between 0_1/1_1 and 0_2/1_2) prior to decoding the PDCCH.

Feature lead view #1: The above issue was discussed a lot last meeting, and there is very strong majority view. Therefore, it is recommended to go to the majority view.

In addition, Ericsson (R1-2005506) additionally pointed that if the CORESET and search space configurations do not lead to any PDCCH candidates of different DCI formats having the same CCE mapping, it is not necessary that the sizes of those DCI formats need to be different.

- RAN1#101e agreement covers the cases where a UE is not expected to monitor DCI formats with same size only when the PDCCH candidates of corresponding DCI formats are mapped to the same resource.

  - Support: Ericsson

Feature lead view #2: In theory it is true that there is no need to ensure different size when there is no overlap. However, it can be expected that more complexity will be increased at both gNB and UE sides. Therefore, it is recommended to keep what given in the current specification.

Based on the situation for the above two issues, it is recommended to go with the proposal below:

Proposal 2-1: Endorse the text proposal in R1-2xxxxxx for TS 38.212 Section 7.3.1.0.

- 7.3.1.0 DCI size alignment *** Unchanged text is omitted *** The UE is not expected to handle a configuration that, after applying the above steps, results in - the total number of different DCI sizes configured to monitor is more than 4 for the cell; or - the total number of different DCI sizes with C-RNTI configured to monitor is more than 3 for the cell; or - the size of DCI format 0_0 in a UE-specific search space is equal to DCI format 0_1 in another UE-specific search space; or - the size of DCI format 1_0 in a UE-specific search space is equal to DCI format 1_1 in another UE-specific search space; or - the size of DCI format 0_0 in a UE-specific search space is equal to DCI format 0_2 in another UE-specific search space; or - the size of DCI format 1_0 in a UE-specific search space is equal to DCI format 1_2 in another UE-specific search space ; or - the size of DCI format 0_2 in a UE-specific search space is equal to DCI format 0_1 in the same or another UE-specific search space; or - the size of DCI format 1_2 in a UE-specific search space is equal to DCI format 1_1 in the same or another UE-specific search space. *** Unchanged text is omitted ***

| Company | View |
| --- | --- |

## Issue A-2: Type2 HARQ-ACK codebook construction related to DAI bit width

In RAN1#101-e meeting, type 2 HARQ-ACK codebook construction related to DAI bit width was discussed under PDCCH enhancements, and the following agreement was achieved:

Agreement

If UE is configured to monitor DCI format 1_2/0_2, the HARQ-ACK codebook size for type-2 HARQ-ACK codebook is determined by

Further, the pseudo-code related to the agreement was also specified in section 9.1.3 of TS38.213 v16.2.0:

9.1.3.1 Type-2 HARQ-ACK codebook in physical uplink control channel […] If the UE transmits HARQ-ACK information in a PUCCH in slot n and for any PUCCH format, the UE determines the , for a total number of OACK HARQ-ACK information bits, according to the following pseudo-code: Set – PDCCH with DCI format scheduling PDSCH reception or SPS PDSCH release monitoring occasion index: lower index corresponds to earlier PDCCH monitoring occasion Set Set Set Set Set to the number of serving cells configured by higher layers for the UE […] Set to the number of PDCCH monitoring occasion(s) while […] end while if end if if harq-ACK-SpatialBundlingPUCCH is not provided to the UE and the UE is configured by maxNrofCodeWordsScheduledByDCI with reception of two transport blocks for at least one configured DL BWP of a serving cell, OACK=2⋅TD⋅j+Vtemp2-1mod TD+1 else OACK=TD⋅j+Vtemp2-1mod TD+1 end if for any

However, Huawei (R1-2005790) and WILUS (R1-2006882) pointed out that some error exists with the pseudo highlight in yellow.

Huawei R1-2005790 Take the case shown in Table 1 as an example, where the gNB sends 3 DL DCIs with 1-bit counter DAI in three monitoring occasions and one UL grant with 2-bit UL DAI=3. If there is no missed DCI in the given example, then both gNB and the UE will have the same understanding about the codebook size, i.e. OAck = 3. However, if the DL DCI in MO#3 is missed, based on the value in Table 1 and the pseudo code highlighted in yellow above, the value of j is still equal to 0, which will result in OAck = 1 according to the equation OACK=TD⋅j+Vtemp2-1mod TD+1 for type 2 HARQ-ACK codebook construction. The reason for this problem is that the yellow-marked pseudo-code “if Vtemp2<Vtemp, j=j+1, end if” will not update the value of j in this case, because Vtemp2=3 is not smaller than Vtemp=2 in this case because of the different number of bits that are used for counter DAI and total DAI. Table 1 - Last DCI is missed (2-bits UL DAI and 1-bit counter DAI) MO#1; MO#2; MO#3; UL grant. cDAI=1; cDAI=2; cDAI=1; UL DAI=3. Correct value of j assuming no missed DCI; 0; 0; 1. Value of j according to the pseudo code in the spec if DCI in MO#3 is missed; 0; 0. Note that when no DCI is missed or one DCI but not the last one is missed, then there is no problem as shown in table 2 below. Table 2 - No DCI or one DCI but not the last one is missed (2-bits UL DAI and 1-bit counter DAI) MO#1; MO#2; MO#3; UL grant. cDAI=1; cDAI=2; cDAI=1; UL DAI=3. Correct value of j assuming no missed DCI; 0; 0; 1. Value of j according to pseudo code in the spec if DCI in MO#1 is missed; 0; 1. Value of j according to pseudo code in the spec if DCI in MO#2 is missed; 0; 1. To solve this issue, we propose to change the yellow pseudo-code to “if Vtemp2-1modTD+1<Vtemp, j=j+1, end if”. Then the value of j will be updated correctly even if the DCI in MO#3 is missed. Proposal 3: Change the pseudo-code “if Vtemp2<Vtemp, j=j+1, end if” to “if Vtemp2-1modTD+1<Vtemp, j=j+1, end if” in section 9.1.3.1 of 38.213. Endorse the TP below. 9.1.3.1 Type-2 HARQ-ACK codebook in physical uplink control channel ***Unchanged text is omitted*** if Vtemp2<Vtemp Vtemp2-1modTD+1<Vtemp end if ***Unchanged text is omitted***.

WILUS R1-2006882 In the revised pseudo-code, the type-2 HARQ-ACK codebook size is determined based on Vtemp2 as well as the value of j. Also, the value of j is incremented when Vtemp2 is less than Vtemp (as shown in the yellow part in the pseudo-code). Note that Vtemp2 is coming from UL DAI value, which is one of {1, 2, 3, 4}, and Vtemp is the last counter-DAI value, which is one of {1, 2, … 2TD}, among counter-DAI values included in received PDCCHs. The range of two values such as Vtemp and Vtemp2 is not aligned due to configurable counter-DAI field size, TD. Therefore, it is necessary to align the range of two values by comparing the two values in the pseudo-code. Similarly, as in the modifications by the agreement at RAN1#101-e meeting, since the range of Vtemp2 can be re-interpreted to that of Vtemp, the comparison should be performed by the re-interpreted value of Vtemp2, i.e., Vtemp2-1mod TD+1. - Proposal 1: In case of HARQ-ACK multiplexing on PUSCH, the value of j is determined by comparing between the re-interpreted value of Vtemp2, i.e., Vtemp2-1mod TD+1, and Vtemp. - Proposal 2: Adopt the following text proposal for TS38.213

Feature lead view: The issue does exist and needs to be addressed. The proposal from Huawei and WILUS looks reasonable.

Proposal 2-2: Endorse the text proposal in R1-2xxxxxx for TS 38.213 Section 9.1.3.1.

- 9.1.3.1 Type-2 HARQ-ACK codebook in physical uplink control channel ***Unchanged text is omitted*** if Vtemp2<Vtemp Vtemp2-1modTD+1<Vtemp end if ***Unchanged text is omitted***

Please provide your views on proposal 3-2.

| Company | View |
| --- | --- |

- References

- R1-2005347	PDCCH enhancements for URLLC	vivo

- R1-2005413	Remaining issues on PDCCH enhancements for NR URLLC	ZTE

- R1-2005506	Remaining Issue of PDCCH Enhancements for NR URLLC	Ericsson

- R1-2005672	Remaining issues on PDCCH enhancements	CATT

- R1-2005790	Corrections on PDCCH enhancement for URLLC	Huawei, HiSilicon

- R1-2005850	Remaining corrections for PDCCH enhancements for URLLC	Intel Corporation

- R1-2006051	PDCCH enhancements for URLLC	OPPO

- R1-2006109	Maintenance on PDCCH enhancements	Samsung

- R1-2006278	Remaining issues of PDCCH enhancements for URLLC	Spreadtrum Communications

- R1-2006487	Remaining issues on  PDCCH enhancements	Apple

- R1-2006549	Remaining Issues on PDCCH Enhancements for Rel-16 URLLC	Quectel

- R1-2006563	Remaining issues on PDCCH enhancements for NR URLLC	Sharp

- R1-2006774	Remaining issues on PDCCH Enhancements for URLLC	Qualcomm Incorporated

- R1-2006865	Remaining issue for TCI field	ASUSTeK

- R1-2006882	Remaining issues on PDCCH for NR URLLC	WILUS Inc.