3GPP TSG-RAN WG1 Meeting #102-e	R1-200xxxx

# e-Meeting, August 17th – 28th, 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRAFT CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.212 | CR |  | rev | - | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections on 5G V2X sidelink features after RAN1#102-e |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_V2X_NRSL-Core |  |  |  |  |  | Date: |  |  | 2020-09-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Corrections based on text-proposals/agreements made at RAN1#102-e including alignment across specifications. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Correction based on email thread [102-e-NR-5G_V2X_NRSL-PHYstructure-01] (R1-2007160); - Correction based on email thread [102-e-NR- 5G_V2X_NRSL-PHYprocedure-01] (R1-2007349); - Correction based on email thread [102-e-NR-5G_V2X_NRSL-Mode-1-01] (R1-2007415); - Correction based on the RAN1#102-e agreement: “MscSCI2(l) = MscPSSCH(l) - MscDMRS(l) - MscPT-RS(l) - MscPSCCH(l)” is replaced with “MscSCI2=MscPSSCHl-MscPSCCH(l)” in clause 8.4.4, including removal of corresponding definition on MscPT-RS(l) and MscDMRS(l). - In SCI format 1-A, remove the incorrect case of sl-PSSCH-DMRS-TimePatternList being not configured. - Correction to DCI format 3_0 and SCI formats 2-A/2-B to set the HARQ process number field size to 4 bits based on the value range of sl-NrOfHARQ-Processes being always (1…16) in TS 38.331. - Reference alignment and editorial changes - For SCI format 2-B: - “Communication range requirement” is provided by higher layer parameter sl-ZoneConfigMCR-Index in TS 38.331 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete specification of 5G V2X sidelink. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3.1.0.1(new), 7.3.1.4.1, 8.3.1.1, 8.4.1.1, 8.4.1.2, 8.4.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 38.211, TS 38.213, TS 38.214 |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# 7	Downlink transport channels and control information

< Unchanged parts are omitted >

7.3.1	DCI formats

< Unchanged parts are omitted >

7.3.1.0	DCI size alignment

< Unchanged parts are omitted >

7.3.1.0.1	DCI size alignment for DCI formats for scheduling of sidelink

If DCI format 3_0 or DCI format 3_1 is monitored on a cell, DCI size alignment for DCI format 3_0 and DCI format 3_1 is performed as described in this clause after performing the DCI size alignment described in Clause 7.3.1.0. The size(s) of the DCI formats configured to monitor for a cell in this clause refers to that after performing the DCI size alignment described in Clause 7.3.1.0.

If DCI format 3_0 or DCI format 3_1 is monitored on a cell and the total number of DCI sizes of the DCI formats configured to monitor for the cell and DCI format 3_0 or DCI format 3_1 is more than 4, zeros shall be appended to DCI format 3_0 if configured and DCI format 3_1 if configured, until the payload size of DCI format 3_0 or DCI format 3_1 equals that of the smallest DCI format configured to monitor for the cell that is larger than DCI format 3_0 or DCI format 3_1.

The UE is not expected to handle a configuration that results in:

-	the total number of different DCI sizes configured to monitor for the cell and DCI format 3_0 or DCI format 3_1 is more than 4; and

-	the payload size of DCI format 3_0 or DCI format 3_1 is larger than the payload size of all other DCI formats configured to monitor for the cell.

##### 7.3.1.4.1	Format 3_0

DCI format 3_0 is used for scheduling of NR PSCCH and NR PSSCH in one cell.

The following information is transmitted by means of the DCI format 3_0 with CRC scrambled by SL-RNTI or SL-CS-RNTI:

-	Resource pool index –log2I  bits, where I is the number of resource pools for transmission configured by the higher layer parameter sl-TxPoolScheduling.

-	Time gap – 3 bits determined by higher layer parameter sl-DCI-ToSL-Trans, as defined in clause 8.1.2.1 of [6, TS 38.214]

-	HARQ process number –log2Nprocess 4 bits as defined in clause 16.4 of [5, TS 38.213]

< Unchanged parts are omitted >

8	Sidelink transport channels and control information

< Unchanged parts are omitted >

8.3.1	1st-stage SCI formats

< Unchanged parts are omitted >

8.3.1.1	SCI format 1-A

SCI format 1-A is used for the scheduling of PSSCH and 2nd-stage-SCI on PSSCH

The following information is transmitted by means of the SCI format 1-A:

-	Priority – 3 bits as defined in clause 5.4.3.3 of [12, TS 23.287].

-	Frequency resource assignment – log2(N subChannel SLN subChannel SL + 12) bits when the value of the higher layer parameter sl-MaxNumPerReserve is configured to 2; otherwise log2(N subChannel SLN subChannel SL + 12N subChannel SL + 16) bits when the value of the higher layer parameter sl-MaxNumPerReserve is configured to 3, as defined in clause 8.1.2.2 of [6, TS 38.214].

-	Time resource assignment – 5 bits when the value of the higher layer parameter sl-MaxNumPerReserve is configured to 2; otherwise 9 bits when the value of the higher layer parameter sl-MaxNumPerReserve is configured to 3, as defined in clause 8.1.2.1 of [6, TS 38.214].

-	Resource reservation period –log2Nrsv_period bits as defined in clause 8.1.4 of [6, TS 38.214], where Nrsv_period is the number of entries in the higher layer parameter sl-ResourceReservePeriodList, if higher layer parameter sl-MultiReserveResource is configured; 0 bit otherwise.

-	DMRS pattern – log2Npattern bits as defined in clause 8.4.1.1.2 of [4, TS 38.211], where Npattern is the number of DMRS patterns configured by higher layer parameter sl-PSSCH-DMRS-TimePatternList.

< Unchanged parts are omitted >

8.4.1	2nd-stage SCI formats

< Unchanged parts are omitted >

8.4.1.1	SCI format 2-A

SCI format 2-A is used for the decoding of PSSCH, with HARQ operation when HARQ-ACK information includes ACK or NACK, when HARQ-ACK information includes only NACK, or when there is no feedback of HARQ-ACK information.

The following information is transmitted by means of the SCI format 2-A:

-	HARQ process number –log2Nprocess 4 bits as defined in clause 16.4 of [5, TS 38.213].

-	New data indicator – 1 bit as defined in clause 16.4 of [5, TS 38.213].

-	Redundancy version – 2 bits as defined in clause 16.4 of [6, TS 38.214].

-	Source ID – 8 bits as defined in clause 8.1 of [6, TS 38.214].

-	Destination ID – 16 bits as defined in clause 8.1 of [6, TS 38.214].

-	HARQ feedback enabled/disabled indicator – 1 bit as defined in clause 16.3 of [5, TS 38.213].

-	Cast type indicator – 2 bits as defined in Table 8.4.1.1-1.

-	CSI request – 1 bit as defined in clause 8.2.1 of [6, TS 38.214].

Table 8.4.1.1-1: Cast type indicator

| Value of Cast type indicator | Cast type |
| --- | --- |
| 00 | Broadcast |
| 01 | Groupcast when HARQ-ACK information includes ACK or NACK |
| 10 | Unicast |
| 11 | Groupcast when HARQ-ACK information includes only NACK |

8.4.1.2	SCI format 2-B

SCI format 2-B is used for the decoding of PSSCH, with HARQ operation when HARQ-ACK information includes only NACK, or when there is no feedback of HARQ-ACK information.

The following information is transmitted by means of the SCI format 2-B:

-	HARQ process number –log2Nprocess 4 bits as defined in clause 16.4 of [5, TS 38.213].

-	New data indicator – 1 bit as defined in clause 16.4 of [5, TS 38.213].

-	Redundancy version – 2 bits as defined in clause 16.4 of [6, TS 38.214].

-	Source ID – 8 bits as defined in clause 8.1 of [6, TS 38.214].

-	Destination ID – 16 bits as defined in clause 8.1 of [6, TS 38.214].

-	HARQ feedback enabled/disabled indicator – 1 bit as defined in clause 16.3 of [5, TS 38.213].

-	Zone ID – 12 bits as defined in clause 5.8.11 of [9, TS 38.331].

-	Communication range requirement – 4 bits determined by higher layer parameter sl-ZoneConfigMCR-Index.

< Unchanged parts are omitted >

8.4.4	Rate Matching

For 2nd-stage SCI transmission on PSSCH with SL-SCH, the number of coded modulation symbols generated for 2nd-stage SCI transmission prior to duplication for the 2nd layer if present, denoted as QSCI2', is determined as follows:

QSCI2'=minOSCI2+LSCI2∙βoffsetSCI2QmSCI2∙R, αl=0NsymbolPSSCH-1MscSCI2(l)+γ

where

-	OSCI2 is the number of the 2nd-stage SCI bits

-	LSCI2 is the number of CRC bits for the 2nd-stage SCI, which is 24 bits.

-	βoffsetSCI2 is indicated in the corresponding 1st-stage SCI.

-	MscPSSCH(l) is the scheduled bandwidth of PSSCH transmission, expressed as a number of subcarriers.

-	MscPSCCH(l) is the number of subcarriers in OFDM symbol l that carry PSCCH and PSCCH DMRS associated with the PSSCH transmission.

MscDMRS(l)l

MscPT-RS(l)l

-	MscSCI2(l) is the number of resource elements that can be used for transmission of the 2nd-stage SCI in OFDM symbol l, for l=0,1,2⋯,NsymbolPSSCH-1 and for NsymbolPSSCH=Nsymbsh-NsymbPSFCH, in PSSCH transmission, where NsymbshNsymbslot = sl-lengthSymbols - 2, where sl-lengthSymbols is the number of sidelink symbols within the slot provided by higher layers as defined in [6, TS 38.214]. If higher layer parameter sl-PSFCH-Period = 2 or 4,  NsymbPSFCH = 3 if "PSFCH overhead indication" field of SCI format 1-A indicates "1", and NsymbPSFCH = 0 otherwise. If higher layer parameter sl-PSFCH-Period = 0, NsymbPSFCH=0. If higher layer parameter sl-PSFCH-Period is 1, NsymbPSFCH=3.

-	MscSCI2l=MscPSSCHl-MscPSCCH(l)MscDMRS(l)MscPT-RSl

-	γ is the number of vacant resource elements in the resource block to which the last coded symbol of the 2nd-stage SCI belongs.

-	R is the coding rate as indicated by "Modulation and coding scheme" field in SCI format 1-A.

-	α is configured by higher layer parameter sl-Scaling.

The input bit sequence to rate matching is d0, d1,d2,d3, ⋯,dN-1, where N is the number of coded bits.

Rate matching is performed according to Clause 5.4.1 by setting IBIL=1.

The output bit sequence after rate matching is denoted as g0SCI2, g1SCI2, g2SCI2, g3SCI2,⋯, gGSCI2-1SCI2, where GSCI2=QSCI2'∙ QmSCI2 and QmSCI2 is modulation order of the 2nd-stage SCI. A UE is not expected to have GSCI2>4096K.

< Unchanged parts are omitted >