3GPP TSG RAN WG1 Meeting #102-e     															          R1-2006826

17th Aug– 28th August 2020

Agenda Item:		8.10.2

Source:				Moderator (Qualcomm Incorporated)

Title: 		Summary of [102-e-NR-eIAB-02]

Document for:		Discussion and decision

Introduction

This contribution provides a summary of the following email discussion:

[102-e-NR-eIAB-02] Email discussion on other enhancements for simultaneous operation of IAB-node’s child and parent links by 8/28– Luca (Qualcomm)

- Prioritize topics to be resolved in RAN1#102-e by 8/19

Summary of discussion on prioritization of discussion topics

Based on the discussion on prioritization in the [102-e-NR-eIAB-02] email thread, it was generally agree to discuss the following topics according to the specified priority:

Timing modes: - Discussion on which additional timing modes besides Case 1 (prioritizing Case 6 and Case 7 timing and leveraging the discussion that has already taken place in SI/Rel-16 on the same) are needed / useful for which duplexing scenario under which conditions. HIGH PRIORITY - Discussion on prioritization / focus in Rel-17 for additional timing modes –MEDIUM PRIORITYand conditional on agreement on high priority portion of proposed Topic 1 and Topic 2 of email thread [102-e-NR-eIAB-01] Interference mitigation: - Discussion on which interference scenarios apply to which duplexing scenario under which conditions. HIGH PRIORITY - Discussion on available solutions (e.g. Rel-16 CLI framework) and/or need and prioritization for Rel-17 IAB specific enhancements for handling the identified interference scenarios –MEDIUM PRIORITYand conditional on agreement on high priority portion of proposed Topic 1 and Topic 2 of email thread [102-e-NR-eIAB-01] Power control: - Discussion on the need for power control for which duplexing scenario under which conditions –HIGH PRIORITY - Discussion on prioritization / focus in Rel-17 for power control enhancements –MEDIUM PRIORITYand conditional on agreement on high priority portion of proposed Topic 1 and Topic 2 of email thread [102-e-NR-eIAB-01] In the above, “conditions” is defined as the key attributes of the scenario which are relevant for RAN1 e.g. FR1 vs. FR2, in-band/out-of-band, TDD spectrum considerations such as RAN4 Scenarios 1/2, high-level antenna designs/RF architectures, deployment considerations etc.), as defined in the [102-e-NR-eIAB-01] email thread.

Discussion on timing modes

Topic 1

This topic relates to the discussion on which additional timing modes besides Case 1 (prioritizing Case 6 and Case 7 timing and leveraging the discussion that has already taken place in SI/Rel-16 on the same) are needed / useful for which duplexing scenario under which conditions.

Input from related contributions:

| Huawei, HiSilicon R1-2005261 | Observation 3: Case #6 timing mode can achieve transmission timing alignment, which facilitates joint transmission of child and parent links of IAB node and mitigates the interference between MT and DU. Observation 5: Case #7 timing mode can achieve symbol-level timing alignment, which facilitates joint reception of child and parent links of IAB node. Proposal 1: Case #6 timing should be supported to mitigate interference in MT Tx/DU Tx scenario. Proposal 2: Case #7 timing need to be supported for IAB to enabling better interference mitigation for simultaneous reception. Proposal 3: A Case #7-like timing mode can be adopted to enhance self-interference cancelation in UL full-duplex. |
| --- | --- |
| Vivo R1-2005400 | Proposal 1: To maintain simultaneous transmission between MT and DU, IAB node should support the Case #6 timing mode as defined in TR 38.874. Proposal 2: To maintain simultaneous reception between MT and DU, IAB node should support the Case #7 timing mode as defined in TR 38.874. |
| AT&T R1-2005952 | Proposal 4: New timing alignment mechanisms beyond Case 1 timing should be considered in Rel-17 for SDM/MPTR scenarios in resources which are orthogonal from those used by access or TDM-only backhaul links. |
| LG Electronics R1-2006383 | Proposal 1: - Timing alignment mechanism for ‘case #6 (MT Tx / DU Tx)’ and ‘case #7 (MT Rx / DU Rx)’ of the IAB timing mode are considered as a starting point for specification work - New cases of IAB timing mode for the other simultaneous scenarios (i.e., MT Tx / DU Rx and MT Rx / DU Tx) are identified. - Case #8: Case#1 + The UL transmission timing of an IAB-node can be aligned with the IAB-node's UL reception timing. - Case #9: Case#1 + The DL reception timing of an IAB-node can be aligned with the IAB-node's DL transmission timing. Proposal 2: - Simultaneous IAB-MT Tx and IAB-DU Tx can be operated by network configuration. Also, IAB timing mode case#6 can be operated by network configuration. - When simultaneous IAB-MT Tx and IAB-DU Tx is configured, IAB timing mode case #6 (MT UL Tx time is aligned with DU DL Tx time) can be applied according to network configuration. - When IAB timing mode case#6 is allowed, MT may apply timing advance value determined by DU DL Tx time. - If network allows both TDM and simultaneous MT Tx/DU Tx, and IAB timing mode case#6 is allowed, MT may apply one of two timing advance values depending on IAB resource multiplexing. |
| NTT DOCOMO R1-2006745 | Proposal 2: Case #6 and #7 timing modes should be considered for IAB node which has single transceiver/antenna panel. |
| Qualcomm R1-2006826 | Observation 3: The benefits of Case 6 and Case 7 timing modes may be limited in a multi-panel implementation aimed at enhanced duplexing capabilities between the IAB-MT and the IAB-DU. Case 7 timing may have a higher benefit than Case 6 timing. |
| Ericsson R1-2006904 | Observation 2 Simultaneous transmission and reception on child and parent links can be enabled by supporting Case-6 and Case-7 timing alignment configurations. Proposal 3 Case-6 OTA timing alignment should be supported, if simultaneous transmission on parent and child links is supported for Rel-17 IAB. Proposal 4 Case-7 OTA timing alignment should be supported, if simultaneous reception on parent and child links is supported for Rel-17 IAB. |

There is a majority view that Case 6 and Case 7 timing modes can provide some benefit in at least some scenarios, e.g. SDM with single panel implementation.

For reference, the four main multiplexing scenarios from the Rel-17 WID are:

Case 1: Simultaneous MT-Tx/DU-Tx

Case 2: Simultaneous MT-Rx/DU-Rx

Case 3: Simultaneous MT-Rx/DU-Tx

Case 4: Simultaneous MT-Tx/DU-Rx

In reference to the above multiplexing scenarios the following conclusion is proposed:

FL Conclusion 1:

The applicability of Case 6 and Case 7 timing to the defined multiplexing scenarios is summarized in the following table as a function of single panel vs. dual panel implementation:

| Applicability / Benefit of Case 6 and Case 7 timing |  | IAB-Node implementation |  |
| --- | --- | --- | --- |
|  |  | Single Panel | Dual Panel |
| Multiplexing scenario | Case 1: Simultaneous MT-Tx/DU-Tx | Case 6 | N/A |
|  | Case 2: Simultaneous MT-Rx/DU-Rx | Case 7 | N/A |
|  | Case 3: Simultaneous MT-Rx/DU-Tx | N/A | N/A |
|  | Case 4: Simultaneous MT-Tx/DU-Rx | N/A | N/A |

| Company | Do you agree with FL Conclusion 1? | Comments |
| --- | --- | --- |
| Qualcomm | Yes | None |
| LG Electronics | For Case 1 and Case2, Yes. But, For Case 3 and Case 4, No | For case 1 and case2 of multiplexing scenario, case 6 and case 7 can be considered. In addition, new timing alignment cases should be defined for case 3 and case 4 of multiplexing scenario if single panel based operation is assumed for the scenarios. |

Topic 2:

This topic relates to the discussion on prioritization / focus in Rel-17 for additional timing modes.

| ZTE, Sanechips R1-2005468 | - Observation 1: To support case-6 timing in Rel-17 may cause following concerns. - Misalignment of UL-Rx timing at parent for child nodes and access UEs, for which all existing solutions (TDM-based, non-TDM-based) have deficiencies. - RAN1 may need to revise or even re-design Rel-16 case-1 timing. - It is unclear whether RAN4 should re-define the UL-Tx timing requirement once the UL-Tx timing is decoupled from TA process and aligned with DL-Tx timing, and, if yes, how complicated it is. - Proposal 1: To de-prioritize case-6 timing in Rel-17. |
| --- | --- |
| Lenovo, Motorola Mobility R1- 2005928 | Proposal 1: Support both transmission timing alignment (Case-6) and reception timing alignment (Case-7) for IAB Rel-17. |
| Samsung R1-2006166 | Proposal 1: Case #6 and Case #7 timing in the TR38.874 can be a starting point for timing discussion in Rel-17 IAB. |
| CMCC R1-2006229 | Proposal 1: The case #6 and case #7 could be a starting point for the discussion for the IAB timing mode under the simultaneous operation of IAB nodes. Case#7 is slightly preferred than case#6 to ensure both network synchronization, and symbol-level alignment. |
| Qualcomm R1-2006826 | Observation 1: Operation in Case 6 timing mode of an IAB-node may cause uplink interference at the IAB-DU receiver of its parent node and/or may require special handling in the uplink scheduler of its parent node to TDM users to avoid such interference. Observation 2: Operation in Case 7 timing mode may require changes to the Rel-15 UL timing control for IAB nodes, which in turn may also impact the OTA timing mechanism defined in Rel-16 for IAB. Proposal 1: Downselect one of the following: - Alt 1: adopt Case 1 as the only timing mode. - Alt 2: quantify the benefits of Case 7 timing mode to determine whether such benefits are sufficient to justify the additional complexity. - Alt 3: quantify the benefits of Case 6 and Case 7 timing modes to determine whether such benefits are sufficient to justify the additional complexity. |
| Fujitsu R1-2005544 | Proposal 1: Consider effective negative TA for supporting simultaneous operation of MT Rx/DU Rx in Rel-17. Proposal 2: Further investigate the required control of the parent or the network for supporting simultaneous operation of MT Tx/Du Tx. |

There are different views on the prioritization of Case 6 and Case 7 timing modes. However, there seems to be a preference of Case 7 timing vs. Case 6 timing. As a resul, the following is proposed:

FL Proposal 1:

Case 7 timing is supported in Rel-17 for IAB-nodes operating in multiplexing scenario Case 2 (simultaneous MT-Rx/DU-Rx).

Case 6 timing is deprioritized in Rel-17 until the solutions for Case 7 timing are specified.

| Company | Do you agree with FL Proposal 1? | Comments |
| --- | --- | --- |
| Qualcomm | Yes | None |
| Fujitsu |  | We agree with that case-7 can have higher priority than case-6. But we are open to discuss them in parallel. |
| LG Electronics |  | It is good to discuss them in parallel. |