3GPP TSG RAN WG1 #103-e			R1-2007790

e-Meeting, October 26th – November 13th, 2020

Agenda Item:	8.2.1

Source:	InterDigital, Inc.

Title:	Consideration on supporting above 52.6GHz in NR

Document for:	Discussion and Decision

# Introduction

In RAN1#102-e [1], evaluation assumptions and technical aspects to support higher frequency (up to 71 GHz) have been agreed. In this contribution, we discuss potential issues and associated standards impacts to support higher frequencies up to 71GHz in NR.

# Bandwidth and Numerologies

## Bandwidth

Although mmW spectrum (52.6 GHz to 71 GHz) allows a bandwidth of 2.16 GHz, most of the regions impose a strict occupied bandwidth rule of 70% to 100% of system bandwidth. This restriction may cause problems especially in the uplink direction as UE may not have enough data to fulfil this requirement and/or UE would have to consume a higher RF and baseband power than needed. In addition, NR should support different maximum possible bandwidth in different bands which requires configurable bandwidth. At least for study purposes, multiples of 400 MHz which has been supported for FR2 can be considered up to 2 GHz. For example, 800 MHz, 1.6 GHz, and 2 GHz may be additionally considered for frequencies from 52.6 GHz to 71 GHz.

Proposal 1: Study multiples of 400 MHz up to 2 GHz should be considered for frequencies from 52.6 GHz to 71 GHz.

Even though choosing the smaller bandwidth will be better to fulfill the regulation requirement of occupied bandwidth and to reduce power consumption, this may cause coexistence issues with other RAT (e.g. 802.11 ad/ay) that currently exist in the mmW spectrum of 52.6 GHz to 71 GHz and use 2.16 GHz bandwidth.

Proposal 2: Study potential coexistence issue with other RAT in the spectrum of 52.6 GHz to 71 GHz.

## Subcarrier spacing

It has been agreed to study the applicable numerology to support higher frequency between 52.6GHz and 71GHz [3]. In that frequency range, phase noise and carrier frequency offset are much higher due to imperfection of PA and Local Oscillator (LO). Furthermore, the Doppler shift/spread is larger as carrier frequency goes higher. A larger subcarrier spacing may mitigate the adverse impacts from higher phase noise and carrier frequency offsets.

Figure 1 shows the BLER performance according to the subcarrier spacing with and without the presence of RF impairments using TDL channel model with 400 MHz bandwidth. Other evaluation results with 2 GHz bandwidth and CDL-B model can be found in our companion contribution [2]. As seen in the figure, the performance loss from RF impairments gets mitigated as subcarrier spacing becomes larger especially when higher modulation order is used.

(a) MCS 22, 400 MHz BW, TDL-A with 5 ns DS 		(b) MCS 22, 400 MHz BW, TDL-A with 10 ns DS

(c) MCS 22, 400 MHz BW, TDL-A with 20 ns DS 		(d) MCS 16, 400 MHz BW, TDL-A with 5 ns DS

(e) MCS 16, 400 MHz BW, TDL-A with 10 ns DS 		(f) MCS 16, 400 MHz BW, TDL-A with 20 ns DS

(a) MCS 7, 400 MHz BW, TDL-A with 5 ns DS 		(b) MCS 7, 400 MHz BW, TDL-A with 10 ns DS

(c) MCS 7, 400 MHz BW, TDL-A with 20 ns DS

Figure 1. BLER performance based on subcarrier spacing in the presence of RF impairments

From the observations, it is obvious that subcarrier spacing 480 kHz and 960 kHz should be considered as candidate subcarrier spacings to combat RF impairments as well as PAPR issues.

Observation 1: Larger subcarrier spacings such as 480 kHz and 960 kHz mitigate the RF impairments in higher frequency especially for higher modulation order.

Another factor to consider while selecting subcarrier spacings is the practical aspect of FFT sizes. The maximum FFT size supported in Rel-15/16 is 4096-point FFT. By limiting the subcarrier spacings with the maximum FFT size, the implementation impacts on both UE and gNB for re-designing the FFT engines can be avoided.

Observation 2: Limiting subcarrier spacing choices to keep the maximum FFT size as in Rel-15/16 can reduce implementation burden for redesigning FFT engine.

Additional criteria to consider while selecting subcarrier spacings is to minimize the design of physical channels such as PSS/SSS and PBCH and to limit the minimum FFT size to 512-point FFT.

Observation 3: Limiting subcarrier spacing choices to keep the minimum FFT size to 512-points can avoid redesign of SS/PBCH block.

From the discussion above, it is observed that limiting the subcarrier spacing choices to keep the minimum and maximum FFT sizes as in Rel-15 is beneficial in terms of implementation complexity as well as standards efforts.

Proposal 3: The candidate new subcarrier spacing is limited to the subcarrier spacing that is within minimum and maximum FFT sizes in Rel-15.

Table 1 summarizes candidate subcarrier spacings for a give bandwidth which is within the existing minimum and maximum FFT sizes.

Table 1 Examples of bandwidth, subcarrier spacings and FFT sizes

| Bandwidth | Subcarrier spacing | FFT size |
| --- | --- | --- |
| 400 MHz | 120 kHz | 4096 |
|  | 240 kHz | 2048 |
|  | 480 kHz | 1024 |
|  | 960 kHz | 512 |
| 800 MHz | 240 kHz | 4096 |
|  | 480 kHz | 2048 |
|  | 960 kHz | 1024 |
|  | 1.92 MHz | 512 |
| 1.6 GHz | 480 kHz | 4096 |
|  | 960 kHz | 2048 |
|  | 1.92 MHz | 1024 |
|  | 3.84 MHz | 512 |
| 2 GHz | 960 kHz | 4096 |
|  | 1.92 MHz | 2048 |
|  | 3.84 MHz | 1024 |

Based on Table 1, it is observed that 960 kHz subcarrier spacing can support up to 2 GHz bandwidth while 480 kHz subcarrier spacing can support up to 1.6 GHz. Supporting 2 GHz bandwidth is important for the coexistence between NR and 802.11 ad/ay. For example, if NR bandwidth for LBT operation is smaller than a channel (possibly 2 GHz) of 802.11 ad/ay, the portion covered by the NR LBT process may have low energy and make NR assume the channel is free when it isn’t. In that regard, considering the available spectrum in 52.6 GHz to 71 GHz, corresponding maximum channel bandwidth (i.e., 2 GHz) and the coexistence, 960 kHz should be considered for the specification support.

Proposal 4: Considering the available spectrum, corresponding maximum channel bandwidth and the coexistence, 960 kHz should be considered for the specification support.

# Delay spread

In this section, we discuss delay spread results based on the system level simulations with the agreed evaluation assumptions [1]. Figure 2 shows the CDF of RMS delay spread for Indoor Factory B, Indoor Office C and Outdoor C and Table 2 provides CP lengths of normal CP and extended CP for candidate subcarrier spacings. As seen in the figure, while each scenario experiences different amounts of RMS delay spread, regardless of scenarios, most of UEs experience smaller RMS delay spreads than normal CP of 960 kHz.

NCP for 960 kHz SCS

Figure 2. Post Beamforming RMS Delay Spread

Table 2 Examples of CP sizes for candidate subcarrier spacings

| Subcarrier spacing | Normal CP | Extended CP |
| --- | --- | --- |
| 120 kHz | 585.94 ns | 2083.34 ns |
| 240 kHz | 292.97 ns | 1041.67 ns |
| 480 kHz | 146.48 ns | 520.83 ns |
| 960 kHz | 73.24 ns | 260.42 ns |
| 1.92 MHz | 36.62 ns | 130.21 ns |
| 3.84 MHz | 18.31 ns | 65.11 ns |

From the observations, considering the different amounts of RMS delay spreads for possible scenarios, supporting multiple subcarrier spacings for higher frequencies would be beneficial.

Observation 4: While each scenario experiences different amounts of RMS delay spread, regardless of scenarios, most of UEs experience smaller RMS delay spreads than normal CP of 960 kHz.

Proposal 5: Considering the different amounts of RMS delay spreads for possible scenarios, supporting multiple subcarrier spacings for higher frequencies would be beneficial.

# Physical channels/procedures

## DM-RS enhancement

A larger subcarrier spacing could degrade channel estimation performance significantly as the frequency gap between two adjacent DM-RS REs in frequency gets effectively larger, thus resulting in poor interpolation performance. Figure 3 shows the BLER performance according to the number of DM-RS symbols (including both front-loaded and additional DM-RS symbols).

Figure 3. BLER performance based on number of DM-RS symbols in the presence of RF impairments (MCS 22, 400 MHz BW, TDL-A with 5 ns delay spread)

As shown in the figure, the performance loss from channel estimation error gets reduced as the number of DM-RS symbols increases especially when higher modulation order is used. Given that the DM-RS density degradation is mainly for frequency domain, enhanced performance may be achieved by providing enhanced DM-RS design for a larger subcarrier spacing. As PUSCH shares DM-RS design with PDSCH, the performance impact of DM-RS should be considered PUSCH as well as PDSCH.

Observation 5: The performance loss from channel estimation error gets reduced as DM-RS density increases especially when a higher modulation order is used.

Proposal 6: Study enhanced DM-RS designs for a larger subcarrier spacing for PDSCH and PUSCH.

The PDCCH DM-RS frequency density is relatively sparse as there are only 3 REs per RB, thus the DM-RS RE is located every 4 REs. Assuming that 3.84MHz subcarrier spacing (if supported) is used, there is no DM-RS over roughly 12 MHz. Considering that downlink coverage is determined based PDCCH coverage, the performance loss of PDCCH due to channel estimation should be avoided. In addition, performance loss of PUCCH needs to be studied considering limited power of UE transmission power.

Proposal 7: Study channel estimation performance impact of PDCCH and PUCCH with a larger subcarrier spacing.

## UE processing time

In NR, UE minimum processing time and switching time has been defined to guarantee the time gap for UE processing on following processes:

- PDSCH processing time (e.g., L1, N1 and d1,1)

- PUSCH processing time (e.g., L2, N2 and d2,1)

- BWP switching time (e.g., DCI/timer based and RRC based)

- TCI state switching (e.g., timeDurationForQCL, MAC CE based and RRC based)

- CSI processing time (e.g., Zref, Z’ref, Z1, Z1’, Z2, Z2’, Z3 and Z3’)

- Scell activation delay.

Considering that the OFDM symbol length gets shorter as the subcarrier spacing becomes larger, the UE processing time for new subcarrier spacings has to be also studied and specified. For the determination of UE processing time for higher frequencies, in addition to the shorter OFDM symbol length, the following aspects should be considered:

- Increased number of panels and number of antenna elements per panel

- Narrow beamwidth due to the increased number of antenna elements

- Large variations of BWP size due to large available bandwidth.

Proposal 8: Study required UE processing time for higher frequencies considering the differences on antenna/panel structure, beam width, BWP size and new subcarrier spacings.

Existing processing time determination methods until Rel-15/16 are based on fixed parameters such as subcarrier spacing and UE capabilities. The existing UE capabilities for the processing time determination methods only consider worst case scenarios to reduce UE implementation complexity. The methods based on worst case scenarios reduce UE implementation complexity, but the methods require more redundant processing time than UE implementation needs especially for higher frequencies considering increased number of antenna elements/panels and increased number of beams with narrow beam width. For higher frequencies, enhanced processing time determination methods can be studied to reduce the redundant processing time. One possible method can be applying different processing time based on parameters which contribute UE processing time. For example, a new processing time requirement can be defined for UEs which process a small packet for its transmission or reception while existing processing time requirement can be used for other packets.

Observation 6: Existing processing time determination methods are based on worst case scenarios and may require more redundant processing time for higher frequencies.

Proposal 9: Study application of different processing time requirements based on parameters which contribute UE processing time.

## Scheduling enhancement

As the slot length gets shorter due to larger subcarrier spacing, the existing single slot based scheduling method for PDSCH may lead to excessive CORESET/search space overheads and redundant UE power consumptions on blind detections. Given that, the enhancement of time domain resource allocation may be a crucial part for efficient operation in higher frequencies. However, allowing flexible time domain scheduling granularity or slot bundling may bring complex UE implementations and corresponding specification impacts. Given that, the enhanced time domain resource allocation method should be carefully studied considering the scheduling efficiency, the UE implementation impacts and the specification impacts.

Observation 7: The enhancement of time domain resource allocation may be a crucial part for efficient operation in higher frequencies.

Proposal 10: Study the enhanced time domain resource allocation method considering the scheduling efficiency, the UE implementation impacts and the specification impacts.

In contrast to time domain resource allocation, however, benefits from frequency domain resource allocation enhancement should be carefully studies. Table 3 shows number of RBs for possible combinations of bandwidth and subcarrier spacing for 52.6 – 71 GHz assuming the maximum FFT size of Rel-15/16 is kept and Table 4 shows RBG sizes based on a size of BWP in TS 38.214 [7].

Table 3 Examples of number of RBs for combinations of bandwidth and subcarrier spacing

| Bandwidth | Subcarrier spacing | Number of RBs |
| --- | --- | --- |
| 400 MHz | 120 kHz | 256 |
|  | 240 kHz | 128 |
|  | 480 kHz | 64 |
|  | 960 kHz | 32 |
| 800 MHz | 240 kHz | 256 |
|  | 480 kHz | 128 |
|  | 960 kHz | 64 |
|  | 1.92 MHz | 32 |
| 1.6 GHz | 480 kHz | 256 |
|  | 960 kHz | 128 |
|  | 1.92 MHz | 64 |
|  | 3.84 MHz | 32 |
| 2 GHz | 960 kHz | 160 |
|  | 1.92 MHz | 80 |
|  | 3.84 MHz | 40 |

Table 4 RBG size based on BWP size

| Bandwidth Part Size | Configuration 1 | Configuration 2 |
| --- | --- | --- |
| 1 – 36 | 2 | 4 |
| 37 – 72 | 4 | 8 |
| 73 – 144 | 8 | 16 |
| 145 – 275 | 16 | 16 |

As shown in Table 3 and Table 4, if the maximum FFT size of Rel-15/16 is kept, it is observed that maximum number of RBs and required payloads of DCI for frequency domain resource allocation do not increase. While introducing larger RBG size may extend a cell coverage by reducing the required payloads in DCI, it also reduces frequency domain resource allocation flexibility. Given that, the benefits from frequency domain resource allocation enhancements should be carefully studied.

Observation 8: If the maximum FFT size of Rel-15/16 is kept, it is observed that maximum number of RBs and required payloads of DCI for frequency domain resource allocation do not increase.

Proposal 11: The benefits from frequency domain resource allocation enhancements should be carefully studied.

## Beam failure recovery

In higher frequencies, the Rel-15/16 BFR operation may have following problems:

- Reduced reliability of BFR due to narrow beam width of the higher frequencies

  - Existing BFR operation detects beam failure only when all monitoring beams fail. While FR2 operation dynamically recovers the beam failure based on relatively wider beam width, UE in higher frequencies may experience out of coverage when all monitoring beams fail. For better reliability, partial beam failure detection can be considered for higher frequencies

- Reliable and efficient beam failure monitoring

  - Existing BFR operation allows up to 10 monitoring RSs and 16 candidate RSs for beam failure detection and new beam selection. If beamwidth in higher frequencies reduces N times, then the maximum number of monitoring RSs should be increased to N times to have identical coverage in both horizontal and vertical domains. In addition, efficient utilization of the monitoring RSs can be considered as the simple extension of the monitoring RSs reduces efficiency of the BFR operation.

Observation 9: Due to the narrow beamwidth in higher frequencies, UE may experience reliability issue to recover dynamic blockage via the existing BFR operation.

Proposal 12: Enhanced BFR operation to provide better reliability and efficiency should be studied for higher frequencies.

# Summary

In this contribution, we discussed the issues in extending NR FR2 operations to 71 GHz. From the discussions, we made following observations and proposals:

Observation 1: Larger subcarrier spacings such as 480 kHz and 960 kHz mitigate the RF impairments in higher frequency especially for higher modulation order.

Observation 2: Limiting subcarrier spacing choices to keep the maximum FFT size as in Rel-15/16 can reduce implementation burden for redesigning FFT engine.

Observation 3: Limiting subcarrier spacing choices to keep the minimum FFT size to 512-points can avoid redesign of SS/PBCH block.

Observation 4: While each scenario experiences different amounts of RMS delay spread, regardless of scenarios, most of UEs experience smaller RMS delay spreads than normal CP of 960 kHz.

Observation 5: The performance loss from channel estimation error gets reduced as DM-RS density increases especially when a higher modulation order is used.

Observation 6: Existing processing time determination methods are based on worst case scenarios and may require more redundant processing time for higher frequencies.

Observation 7: The enhancement of time domain resource allocation may be a crucial part for efficient operation in higher frequencies.

Observation 8: If the maximum FFT size of Rel-15/16 is kept, it is observed that maximum number of RBs and required payloads of DCI for frequency domain resource allocation do not increase.

Observation 9: Due to the narrow beamwidth in higher frequencies, UE may experience reliability issue to recover dynamic blockage via the existing BFR operation.

Proposal 1: Study multiples of 400 MHz up to 2 GHz is considered for frequencies from 52.6 GHz to 71 GHz.

Proposal 2: Study potential coexistence issue with other RAT in the spectrum of 52.6 GHz to 71 GHz.

Proposal 3: The candidate new subcarrier spacing is limited to the subcarrier spacing that is within minimum and maximum FFT sizes in Rel-15.

Proposal 4: Considering the available spectrum, corresponding maximum channel bandwidth and the coexistence, 960 kHz should be considered for the specification support.

Proposal 5: Considering the different amounts of RMS delay spreads for possible scenarios, supporting multiple subcarrier spacings for higher frequencies would be beneficial.

Proposal 6: Study enhanced DM-RS designs for a larger subcarrier spacing for PDSCH and PUSCH.

Proposal 7: Study channel estimation performance impact of PDCCH and PUCCH with a larger subcarrier spacing.

Proposal 8: Study required UE processing time for higher frequencies considering the differences on antenna/panel structure, beam width, BWP size and new subcarrier spacings.

Proposal 9: Study application of different processing time requirements based on parameters which contribute UE processing time.

Proposal 10: Study the enhanced time domain resource allocation method considering the scheduling efficiency, the UE implementation impacts and the specification impacts.

Proposal 11: The benefits from frequency domain resource allocation enhancements should be carefully studied.

Proposal 12: Enhanced BFR operation to provide better reliability and efficiency should be studied for higher frequencies.

# References

- “Final Report of 3GPP TSG RAN WG1 #102-e”, 3GPP RAN1 Meeting #102-e, Aug, 17th – 28th, 2020

- R1-2007792, “Evaluation results for above 52.6 GHz”, InterDigital, Inc., RAN1#103-e, Oct 26th – Nov 13th, 2020

- RP-193259, “Study on supporting NR from 52.6GHz to 71 GHz”

- 3GPP TR 38.807 (v16.0.0): “Study on requirements for NR beyond 52.6 GHz (Release 16)”

- 3GPP TR 38.901 (v15.0.0): “Study on channel model for frequencies from 0.5 to 100 GHz (Release 15)”

- 3GPP TR38.803 (v14.2.0), “Study on new radio access technology: Radio Frequency (RF) and co-existence Aspects (Release 14)”

- 3GPP TS38.214 (v16.2.0), “Physical layer procedures for data (Release 16)”

- Ben-Dor, Eshar, et al. "Millimeter-wave 60 GHz outdoor and vehicle AOA propagation measurements using a broadband channel sounder." 2011 IEEE Global Telecommunications Conference-GLOBECOM 2011. IEEE, 2011.

# Annex I: Link Level Simulation assumptions

Table 5 Link Level Simulation Assumptions

| Carrier frequency | 60 GHz |  |  |  |
| --- | --- | --- | --- | --- |
| Duplexing | FDD |  |  |  |
| Bandwidth | 400 MHz |  |  |  |
| Subcarrier Spacing (kHz) | 120 | 240 | 480 | 960 |
| CP Type | Normal CP for subcarrier spacing evaluations Extended CP for DM-RS evaluations |  |  |  |
| Antenna Configurations | 2x2 |  |  |  |
| Channel Model | TDL-A model (5 ns, 10ns and 20 ns Delay Spread) |  |  |  |
| UE Mobility | 3 km/hr |  |  |  |
| RF impairments | Phase Noise: Example 2 as specified in TR38.803 (sec. 6.1.11.2) PA nonlinearity: Rapp model No Frequency offset modeling |  |  |  |
| Transmission scheme | Rank 1 using precoder cycling with PRG size of 4 RBs |  |  |  |
| Channel/Noise Estimation | Realistic |  |  |  |
| PTRS | Every 2nd PRB in frequency and every OFDM symbol in time |  |  |  |
| DMRS | Release 15 Type 1 with 1 front-loaded DM-RS and 1 additional DM-RS unless specified |  |  |  |

# Annex II: System Level Simulation assumptions

Table 6 System Level Simulation Assumptions for Indoor Factory-B

| BS Height | 8 m |
| --- | --- |
| ISD | 50 m |
| Carrier Frequency | 60 GHz |
| System Bandwidth | 2000 MHz |
| Number of UEs | 10 per BS |
| UE Height | 1.5 m |
| BS Antenna Configuration | (Mg, Ng, M, N, P) = (1, 1, 4, 8, 2) Panel is ceiling mounted pointing towards ground. |
| UE Antenna Configuration | (Mg, Ng, M, N, P) = (1, 2, 2, 2, 2), Note: The 2 panels are back to back, 0°, 180° |
| BS Tx Power | 40 dBm |
| UE Noise Figure | 10 dB |

Table 7 System Level Simulation Assumptions for Indoor Office-C

| BS Height | 3 m |
| --- | --- |
| ISD | 20 m |
| Carrier Frequency | 60 GHz |
| System Bandwidth | 2000 MHz |
| Number of UEs | 10 per BS |
| UE Height | 1 m |
| UE Mobility | 3 km/h |
| BS Antenna Configuration | (Mg, Ng, M, N, P) = (1, 1, 4, 8, 2) Panel is ceiling mounted pointing towards ground. |
| UE Antenna Configuration | (Mg, Ng, M, N, P) = (1, 2, 2, 2, 2), Note: The 2 panels are back to back, 0°, 180° |
| BS Tx Power | 40 dBm |
| UE Noise Figure | 10 dB |

Table 8 System Level Simulation Assumptions for Outdoor-C

| BS Height | 10 m |
| --- | --- |
| ISD | 150 m |
| Carrier Frequency | 60 GHz |
| System Bandwidth | 2000 MHz (note: Large compared to anything we have done) |
| Number of UEs | 10 per sector, 100% outdoor |
| UE Height | 1.5 m |
| UE Mobility | 3 km/h |
| BS Antenna Configuration | (Mg, Ng, M, N, P) = (1, 1, 8, 16, 2) |
| UE Antenna Configuration | (Mg, Ng, M, N, P) = (1, 2, 2, 2, 2), Note: The 2 panels are back to back, 0°, 180° |
| BS Tx Power | 40 dBm |
| UE Noise Figure | 10 dB |