3GPP TSG RAN WG1 Meeting #103-e

e-Meeting, October 26th – November 13th, 2020

Title:	RAN1 Chairman’s Notes

1	Opening of the meeting (Day 1: UTC 9:00 AM)	3

1.1	Call for IPR	3

1.2	Competition Law Statement	3

1.3	Network Usage Conditions	3

1.4	Check-in for Registered Delegates	4

1.5	Aspects related to RAN1 Meeting Management	4

2	Approval of Agenda	4

3	Highlights from RAN plenary	4

4	Approval of Minutes from previous meetings	4

5	Incoming Liaison Statements (14+2=16)	4

5.1	RAN1 Aspects for RF requirements for NR frequency range 1 (FR1) (2)	8

5.2	RAN1 Aspects for RF Requirement Enhancements for FR2	9

6	E-UTRA (3+10=13)	9

6.1	Maintenance of E-UTRA Releases 8 – 15 (3)	9

6.2	Maintenance of E-UTRA Release 16 (3+3+1+1+1+1=10)	10

6.2.1	Maintenance of Additional MTC Enhancements (3)	10

6.2.2	Maintenance of Additional Enhancements for NB-IoT (3)	10

6.2.3	Maintenance of DL MIMO efficiency enhancements for LTE (1)	11

6.2.4	Maintenance of LTE-based 5G Terrestrial Broadcast (1)	11

6.2.5	LTE Rel-16 UE Features (1)	11

6.2.6	Others (1)	11

7	NR Maintenance (10+51=61)	12

7.1	Maintenance of Release 15 NR (10)	12

7.2	Maintenance of Release 16 NR (1+7+2+12+7+4+1+2+2+2+9+2=51)	13

7.2.1	Maintenance of Two step RACH for NR (1)	14

7.2.2	Maintenance of NR-based Access to Unlicensed Spectrum (7)	14

7.2.3	Maintenance of Integrated Access and Backhaul for NR (2)	16

7.2.4	Maintenance for 5G V2X with NR sidelink (12)	17

7.2.5	Maintenance of Physical Layer Enhancements for NR URLLC (7)	22

7.2.6	Maintenance of Enhancements on MIMO for NR (4)	24

7.2.7	Maintenance of UE Power Saving for NR (1)	25

7.2.8	Maintenance of NR positioning support (2)	25

7.2.9	Maintenance of NR Mobility Enhancements (2)	26

7.2.10	Maintenance of Multi-RAT Dual-Connectivity and Carrier Aggregation enhancements (LTE, NR) (2)	26

7.2.11	NR Rel-16 UE Features (9)	28

7.2.12	Others (2)	51

8	Release 17	52

8.1	Further enhancements on MIMO for NR	52

8.1.1	Enhancements on Multi-beam Operation	52

8.1.2	Enhancements for Multi-TRP Deployment	52

8.1.2.1	Enhancements on Multi-TRP for PDCCH, PUCCH and PUSCH	52

8.1.2.2	Enhancements on Multi-TRP inter-cell operation	53

8.1.2.3	Enhancements on beam management for multi-TRP	53

8.1.2.4	Enhancements on HST-SFN deployment	54

8.1.3	Enhancements on SRS flexibility, coverage and capacity	54

8.1.4	CSI enhancements: MTRP and FR1 FDD reciprocity	54

8.1.5	Others	55

8.2	Study on supporting NR from 52.6GHz to 71 GHz	55

8.2.1	Required changes to NR using existing DL/UL NR waveform	55

8.2.2	Channel access mechanism	62

8.2.3	Others	69

8.3	Enhanced Industrial Internet of Things (IoT) and URLLC	77

8.3.1	Necessity and Support of Physical Layer feedback enhancements	77

8.3.1.1	UE feedback enhancements for HARQ-ACK	77

8.3.1.2	CSI feedback enhancements	78

8.3.2	Enhancements for unlicensed band URLLC/IIoT	79

8.3.3	Intra-UE Multiplexing/Prioritization	81

8.3.4	Others	83

8.4	Solutions for NR to support non-terrestrial networks (NTN)	83

8.4.1	Timing relationship enhancements	84

8.4.2	Enhancements on UL time and frequency synchronization	85

8.4.3	Enhancements on HARQ	86

8.4.4	Others	87

8.5	Study on NR Positioning Enhancements	88

8.5.1	Additional scenarios for evaluation	88

8.5.2	Evaluation of achievable positioning accuracy and latency	89

8.5.3	Potential positioning enhancements	97

8.5.4	Others	101

8.6	Study on Support of Reduced Capability NR Devices	102

8.6.1	Potential UE complexity reduction features	102

8.6.2	Reduced PDCCH monitoring	106

8.6.3	Coverage recovery and capacity impact	114

8.6.4	Framework and Principles for Reduced Capability	120

8.6.5	Others	121

8.7	UE Power Saving Enhancements	125

8.7.1	Enhancements for idle/inactive-mode UE power saving	125

8.7.1.1	Potential paging enhancements	125

8.7.1.2	TRS/CSI-RS occasion(s) for idle/inactive UEs	127

8.7.1.3	Others	128

8.7.2	Potential extension(s) to Rel-16 DCI-based power saving adaptation during DRX ActiveTime	128

8.7.3	Others	130

8.8	Study on NR coverage enhancement	130

8.8.1	Baseline coverage performance using LLS	130

8.8.1.1	FR1	130

8.8.1.2	FR2	136

8.8.2	Potential techniques for coverage enhancements	139

8.8.2.1	PUSCH coverage enhancement	139

8.8.2.2	PUCCH coverage enhancement	147

8.8.2.3	Coverage enhancement for channels other than PUSCH and PUCCH	153

8.8.3	Others	157

8.9	Rel-17 enhancements for NB-IoT and LTE-MTC	157

8.9.1	Support of 16-QAM for unicast in UL and DL for NB-IoT	157

8.9.2	Support additional PDSCH scheduling delay for introduction of 14-HARQ processes in DL for eMTC	157

8.9.3	Others	157

8.10	Enhancements to Integrated Access and Backhaul	157

8.10.1	Enhancements to resource multiplexing between child and parent links of an IAB node	157

8.10.2	Other enhancements for simultaneous operation of IAB-node’s child and parent links	158

8.10.3	Others	158

8.11	NR Sidelink enhancement	158

8.11.1	Remaining issues for sidelink evaluation methodology update for power saving	158

8.11.2	Resource allocation enhancement	162

8.11.2.1	Resource allocation for power saving	162

8.11.2.2	Feasibility and benefits for mode 2 enhancements	163

8.11.3	Others	165

8.12	NR Multicast and Broadcast Services	165

8.12.1	Mechanisms to support group scheduling for RRC_CONNECTED UEs	165

8.12.2	Mechanisms to improve reliability for RRC_CONNECTED UEs	168

8.12.3	Basic functions for broadcast/multicast for RRC_IDLE/RRC_INACTIVE UEs	170

8.12.4	Others	171

8.13	NR Dynamic spectrum sharing (DSS)	171

8.13.1	Cross-carrier scheduling (from Scell to Pcell)	171

8.13.2	Multi-cell PDSCH scheduling via a single DCI	173

8.13.3	Support efficient activation/de-activation mechanism for SCells in NR CA	176

8.14	Study on XR Evaluations for NR (RAN1)	177

8.14.1	Applications, Traffic Model and Evaluation Methodology	177

8.14.2	Others	182

8.15	Study on NB-IoT/eMTC support for Non-Terrestrial Network	182

8.15.1	Scenarios applicable to NB-IoT/eMTC	182

8.15.2	Necessary changes to support NB-IoT and eMTC over satellite	184

8.15.3	Others	184

8.16	Introduction of DL 1024QAM for NR FR1	184

8.17	Others	188

9	Closing of the meeting (Day 13: UTC 5:00PM at the latest)	188

# Opening of the meeting (Day 1: UTC 9:00 AM)

## Call for IPR

I draw your attention to your obligations under the 3GPP Partner Organizations' IPR policies.  Every Individual Member organization is obliged to declare to the Partner Organization or Organizations of which it is a member any IPR owned by the Individual Member or any other organization which is or is likely to become essential to the work of 3GPP.

The attention of the delegates to this meeting was drawn to the fact that 3GPP Individual Members have the obligation under the IPR Policies of their respective Organizational Partners to inform their respective Organizational Partners of Essential IPRs they become aware of. The delegates were asked to take note that they were thereby invited: - to investigate whether their organization or any other organization owns IPRs which were, or were likely to become Essential in respect of the work of 3GPP. - to notify their respective Organizational Partners of all potential IPRs, e.g., for ETSI, by means of the IPR Statement and the Licensing declaration forms (http://www.etsi.org/WebSite/document/Legal/IPRForms.doc).

## Competition Law Statement

The attention of the delegates to this meeting was drawn to the fact that 3GPP activities are subject to antitrust and competition laws and that compliance with said laws is therefore required of any participant of this WG meeting including the Chairman and Vice Chairmen. In case of question, please contact your legal counsel. The present meeting will be conducted with strict impartiality and in the interests of 3GPP. Furthermore, delegates were reminded that timely submission of work items/contributions in advance of WG meetings is important to allow for full and fair consideration of such matters.

## Network Usage Conditions

Users shall not use the network to engage in illegal activities. This includes activities such as copyright violation, hacking, espionage or any other activity that may be prohibited by local laws. Users shall not engage in non-work related activities that consume excessive bandwidth or cause significant degradation of the performance of the network. Since the network is a shared resource, users should exercise some basic etiquette when using the 3GPP network at a meeting. It is understood that high bandwidth applications such as downloading large files or video streaming might be required for business purposes, but delegates should be strongly discouraged in performing these activities for personal use. Downloading a movie or doing something in an interactive environment for personal use essentially wastes bandwidth that others need to make the meeting effective. The meeting chairman should remind end users that the network is a shared resource; the more one user grabs, the less there is for another. Email and its attachments already take up significant bandwidth (certain email programs are not very bandwidth efficient). In case of need the chair can ask the delegates to restrict IT usage to things that are essential for the meeting itself. 1. DON’T place your WiFi device in ad-hoc mode 2. DON’T set up a personal hotspot in the meeting room 3. DO try 802.11a if your WiFi device supports it 4. DON’T manually allocate an IP address 5. DON’T be a bandwidth hog by streaming video, playing online games, or downloading huge files 6. DON’T use packet probing software which clogs the local network (e.g., packet sniffers or port scanners)

## Check-in for Registered Delegates

The attention of the delegates to this meeting was drawn to the fact that it is not permitted to check in other delegates on their behalf. In the event of technical difficulties preventing check-in, delegates should present themselves in person to the Secretary.

## Aspects related to RAN1 Meeting Management

Delegates are encouraged to check R1-1721392 for some thoughts on RAN1 meeting management.

# Approval of Agenda

R1-2007500	Draft Agenda of RAN1#103-e meeting	RAN1 Chair

No dedicated email thread – automatic approval if no comments by 10/27

R1-2009371	Additional Guidelines for RAN1#103 e-Meeting Management	RAN1 Chair

R1-2009372	RAN1#103-e Meeting Invitation, Timelines, Scope, Process	RAN1 Chair, ETSI MCC

# Highlights from RAN plenary

R1-2007502	Highlights from RAN#89-e	RAN1 Chair

# Approval of Minutes from previous meetings

R1-2007501	Report of RAN1#102-e meeting	ETSI MCC

Email approval till 10/27

Update on 10/27: approved.

# Incoming Liaison Statements (14+2=16)

R1-2009232	RAN1#103-e preparation phase on LSs	RAN1 Chair

All incoming LSs are noted. The following are to be handled in other Ais

- For LS in R1-2007512, a reply LS may be necessary – email discussion/approval till 10/29, to be handled under 7.2.12

- For LS in R1-2007521, a reply LS is necessary – target 10/30 11/2 for email approval. To be handled under 7.2.4.

- For LS in R1-2007522, a reply LS is necessary – target 10/29 for email approval. To be handled under 7.2.4.

- For LS in R1-2007523, a reply is LS – target 11/2 for email approval. To be handled under 7.2.5

- For LS in R1-2007505, a reply LS may be necessary – email discussion/approval till 10/29, to be managed under 7.2.11.

- For LS in R1-2007519, a reply LS is necessary – target 10/30 11/2 for email approval, to be handled under 7.2.11.

- For LS in R1-2007520, a reply LS is necessary – target 10/30 11/2 for email approval, to be handled under 7.2.11.

- For LS in R1-2007525, a reply LS is necessary – target 10/30 11/2 for email approval, to be handled under 7.2.11.

The following are for the next phase of email discussion/approval under AI5:

[103-e-AI5-LS-01] For LS in R1-2007518, a reply LS may be necessary – email discussion/approval till 10/29 (to be handled under AI5) – Huawei (Yubo)

Update on 10/29 – latest draft LS is approved. Final LS in R1-2009435.

[103-e-AI5-LS-02] For LS in R1-2007506,  a reply LS may be necessary – email discussion/approval till 11/2 – MTK (James Hsieh)

Update on 11/2 – to check on 11/3  11/4  draft LS text is endorsed. Final LS in R1-2009575

[103-e-AI5-LS-03] For LS in R1-2007503, a reply LS may be necessary – email discussion/approval till 10/29 – Huawei (Matthew)

Update on 10/29 – latest draft available. Check on 11/2

Update on 11/2 – latest draft is approved. Final LS in R1-2009491.

[103-e-AI5-LS-04] For LS in R1-2007514, a reply LS is necessary – target 11/2 for email approval – OPPO (Kavin)

Update on 11/2 – to check on 11/3  11/4  draft LS text is endorsed. Final LS in R1-2009621.

Email summary in R1-2009622.

[103-e-AI5-LS-05] For LS in R1-2007513, a reply LS is necessary – target 10/30 11/2 for email approval – Qualcomm (Huilin)

Update on 11/2 – to check on 11/3

Update on 11/3 – latest draft reply is approved. Final LS in R1-2009505.

[103-e-AI5-LS-06] Email discussion/approval for a potential reply LS in response to R1-2005208 by 10/29 – Samsung (Seunghoon)

Update on 10/29 – deadline extended to 11/4

Update on 11/4

Conclusion:

- Per-UE capabilities for SUL/SDL bands can be differentiated on the duplex mode(s) for Rel-15 and Rel-16.

- FFS: how to decide whether FDD or TDD capabilities apply to SUL/SDL including whether or not to add common or dedicated signaling of semi-static DL/UL configuration for SUL/SDL

Conclusion:

- Rel-16 per-UE capabilities with xDD differentiation and FRx differentiation can be differentiated for SUL/SDL bands by "per-band” capability signaling for each SUL band and SDL band.

- FFS for Rel-15 per-UE capabilities with xDD differentiation.

- FFS if two different bands are involved in the Rel-15/Rel-16 UE capability

Draft LS? 11/5  11/9, latest draft is approved. Final LS in R1-2009576.

Email summary in R1-2009591

//LTE

R1-2007518	LS on Incomplete LTE Physical Layer Capabilities	RAN2, Huawei

R1-2008342	[DRAFT] Reply LS on Incomplete LTE Physical Layer Capabilities	Huawei

//NR Rel-15 & Rel-16

DC Location

R1-2007504	LS on additional DC location reporting for intra-band UL CA	RAN4, Qualcomm

R1-2007726	[DRAFT] Reply LS on additional DC location reporting for intra-band UL CA	ZTE

R1-2007727	Discussion on additional DC location reporting for intra-band UL CA	ZTE

R1-2007795	Discussion on additional DC location reporting for intra-band UL CA	CATT

R1-2008650	Draft Reply LS on additional DC location reporting for intra-band UL CA	vivo

R1-2008708	On additional DC location reporting for intra-band UL CA	Nokia, Nokia Shanghai Bell

R1-2008773	Draft reply on additional DC location reporting for intra-band UL CA	Huawei, HiSilicon

R1-2008774	Discussion on additional DC location reporting for intra-band UL CA	Huawei, HiSilicon

R1-2008595	Draft reply LS to RAN4 LS on additional DC location reporting for intra-band UL CA	Qualcomm Incorporated

Late submission

BWP Switching

R1-2007506	LS on multiple BWP switch impact on HARQ design in dormancy SCell	RAN4, MediaTek

R1-2008208	LS on DCI-based multiple BWP switch simultaneously	Nokia, Nokia Shanghai Bell

R1-2008505	Draft LS response on DCI-based multiple BWP switch simultaneously	MediaTek Inc.

R1-2008652	Draft Reply LS on DCI-based multiple BWP switch simultaneously	vivo

R1-2008775	On DCI-based multiple BWP switch simultaneously	Huawei, HiSilicon

CSI-RS per MO

R1-2007512	LS on number of configurable CSI-RS resources per MO	RAN4, Huawei

R1-2007798	Discussion of RAN4's LS on number of configurable CSI-RS resources per MO	CATT

R1-2007932	Discussion on LS on number of configurable CSI-RS resources per MO	Intel Corporation

R1-2008649	Draft Reply LS on number of configurable CSI-RS resources per MO	vivo

R1-2008734	Maximum number of CSI-RS for L3 mobility	Nokia, Nokia Shanghai Bell

R1-2008735	[DRAFT] Response LS on the number of configurable CSI-RS resources per MO	Nokia, Nokia Shanghai Bell

V2X/PC5

R1-2007503	LS on definition of NR V2X con-current operation	RAN4, Huawei

R1-2008784	On definition of NR V2X con-current operation	Huawei, HiSilicon

R1-2007514	LS on new PQI support for PC5 communication	SA2, OPPO

R1-2008233	Discussion on reply LS on new PQI support for PC5 communication	OPPO

R1-2008645	Draft Reply LS on new PQI support for PC5 communication	vivo

R1-2008745	[Draft] LS on new PQI values for PC5 communication	Ericsson

R1-2008782	Sidelink configuration by NR Uu in PC5-only bands	Huawei, HiSilicon

R1-2008788	On new PQI support for PC5 communication	Huawei, HiSilicon

R1-2007521	LS on RAN2 agreements and RAN1 related issues	RAN2, LG Electronics

R1-2007799	Discussion on LS from RAN2 on RAN2 agreements and RAN1 related issue	CATT

R1-2007800	Discussion on the LS from RAN2 on sidelink configured grant handling	CATT

R1-2007919	[DRAFT] Reply LS on RAN2 agreements and RAN1 related issues	ZTE, Sanechips

R1-2008119	Draft reply LS on RAN2 agreements and RAN1 related issues	Samsung

R1-2008590	Draft reply LS on RAN2 agreements and RAN1 related isues	Qualcomm Incorporated

R1-2008748	Discussion on RAN2 LS on RAN2 agreements and RAN1 related issues	Ericsson

R1-2008749	[Draft] LS reply on RAN2 agreements and RAN1 related issues	Ericsson

R1-2008783	On resource reselection due to dropped retransmissions	Huawei, HiSilicon

R1-2007522	LS to RAN1 on sidelink configured grant handling	RAN2, Huawei

R1-2007920	[DRAFT] Reply LS to RAN1 on sidelink configured grant handling	ZTE, Sanechips

R1-2008120	Draft reply LS to RAN1 on sidelink configured grant handling	Samsung

R1-2008593	Draft reply LS on LS to RAN1 on sidelink configured grant handling	Qualcomm Incorporated

R1-2008648	Draft Reply LS on sidelink configured grant handling	vivo

R1-2008746	Discussion on RAN2 LS on SL CG handling	Ericsson

R1-2008747	[Draft] LS reply on SL CG handling	Ericsson

R1-2008781	On sidelink configured grant handling	Huawei, HiSilicon

NR-U

R1-2007526	LS on CAPC	RAN2, Nokia

R1-2007968	Discussion on the LS on CAPC	ZTE, Sanechips

R1-2008305	Discussion of the LS on CAPC	Ericsson Inc.

URLLC/IIoT

R1-2007523	LS on Intra UE Prioritization Scenario	RAN2, Ericsson

R1-2007702	Draft Reply LS on Intra-UE Prioritization Scenario	Ericsson

R1-2007724	[DRAFT] Reply LS on Intra UE Prioritization Scenario	ZTE

R1-2007797	Draft reply LS on intra UE prioritization scenario	CATT

R1-2008077	Discussion on RAN2 LS on Intra UE Prioritization Scenario	LG Electronics

R1-2008117	Discussion on LS on Intra UE Prioritization Scenario	Samsung

R1-2008306	[Draft] Reply LS on Intra UE Prioritization Scenario	Nokia

R1-2008866	Discussion on LS on intra-UE prioritization	Intel Corporation

Revision of R1-2008525

R1-2008592	Draft reply LS to RAN 2 LS on intra-UE prioritization	Qualcomm Incorporated

R1-2009067	Draft Reply LS on Intra UE Prioritization Scenario	vivo

Revision of R1-2008642

MR-DC/CA

R1-2007507	Reply LS on SCell Dormancy	RAN4, Huawei

CLI/RIM

R1-2007513	Full slot formats support in TDD UL-DL configuration	RAN3, Qualcomm

R1-2007796	Discussion on full slot formats support in TDD UL-DL configuration	CATT

R1-2007969	Draft reply LS on full slot formats support in TDD UL-DL configuration	ZTE

R1-2008118	Draft reply LS on Full slot formats support in TDD UL-DL configuration	Samsung

R1-2008402	Discussions on full slot formats support in TDD UL-DL configuration	LG Electronics

R1-2008586	Draft reply LS on full slot format support	Qualcomm Incorporated

R1-2008795	Discussion on the LS on Full slot formats support in TDD UL-DL configuration	Huawei, HiSilicon

UE Features

R1-2007505	LS reply to RAN1 on UE capability on wideband carrier operation for NR-U	RAN4, MediaTek

R1-2008040	Discussion on RAN4 LS on UE capability on wideband carrier operation for NR-U	LG Electronics

R1-2008207	[Draft] Reply LS on UE capability on wideband carrier operation for NR-U	Nokia, Nokia Shanghai Bell

R1-2008520	[Draft] Reply LS on UE capability on wideband carrier operation for NR-U	MediaTek Inc.

R1-2008594	Draft response LS to RAN4 on UE capability on wideband carrier operation for NR-U	Qualcomm Incorporated

R1-2008644	Draft Reply LS on UE capability on wideband carrier operation for NR-U	vivo

R1-2008780	Discussion on UE capability on wideband carrier operation for NR-U	Huawei, HiSilicon

R1-2008790	DRAFT LS reply on UE capability on wideband carrier operation for NR-U	Huawei, HiSilicon

R1-2007517	Reply LS on Rel-16 UE feature lists for NR DAPS	RAN2, Huawei

R1-2007519	LS to RAN1 on beamSwitchTiming	RAN2, vivo

R1-2007757	Draft reply LS on beamSwitchTiming	ZTE

R1-2007801	Draft reply LS on  beamSwitchTiming	CATT

R1-2008116	Draft reply LS on beamSwitchTiming	Samsung

R1-2008228	Discussion on LS on beamSwitchTiming for AP CSI-RS	OPPO

R1-2008591	Draft reply LS to RAN 2 LS on beamSwitchTiming	Qualcomm Incorporated

R1-2008651	Draft Reply LS on beamSwitchTiming	vivo

R1-2008791	[Draft] Reply LS on beamSwitchTiming	Huawei, HiSilicon

R1-2007520	LS on UE capability for V2X	RAN2, OPPO

R1-2008234	Draft reply LS on UE capability for V2X	OPPO

R1-2008589	Draft reply LS on UE capability for V2X	Qualcomm Incorporated

R1-2008646	Draft Reply LS on UE capability for V2X	vivo

R1-2008647	Draft Reply LS on RAN2 agreements for V2X	vivo

R1-2008744	[Draft] LS on UE capability for V2X	Ericsson

R1-2008794	Discussion on the LS on cell-grouping UE capability for synchronous NR-DC	Huawei, HiSilicon

R1-2007525	LS on cell-grouping UE capability for synchronous NR-DC	RAN2, Qualcomm

R1-2007730	[DRAFT] Reply LS on cell-grouping UE capability for synchronous NR-DC	ZTE

R1-2007731	Discussion on cell-grouping UE capability for synchronous NR-DC	ZTE

R1-2008506	Draft LS response on cell-grouping UE capability for synchronous NR-DC	MediaTek Inc.

R1-2008587	Discussion on cell-grouping UE capabilities for synchronous NR-DC	Qualcomm Incorporated

R1-2008588	Draft LS reply on cell-grouping UE capability for synchronous NR-DC	Qualcomm Incorporated

R1-2008643	Draft Reply LS on cell-grouping UE capability for synchronous NR-DC	vivo

//NR Rel-17

ePositioning

R1-2007524	LS on the error source for RAT-dependent positioning	RAN2, Huawei

// The following LSs are noted (RAN1 is cc-ed)

R1-2007508	Reply LS on UE capability xDD differentiation for SUL/SDL bands	RAN4, ZTE

R1-2007509	Reply LS on power control for NR-DC	RAN4, vivo

R1-2007510	LS on updated Rel-16 RAN4 UE features lists for NR and LTE	RAN4, CMCC

R1-2007511	Reply LS on positioning SRS during DRX inactive time	RAN4, Apple

R1-2007515	LS on Direct Discovery and Relay in SA2	SA2, OPPO

R1-2007516	Reply LS to 5G-ACIA-LS-2020-WI042 = RP-201279 on 3GPP NR Rel-16 URLLC and IIoT performance evaluation	RAN, Ericsson

R1-2007508	Reply LS on UE capability xDD differentiation for SUL/SDL bands	RAN4, ZTE

R1-2007527	Reply LS on exchange of information related to SRS-RSRP measurement resource configuration for UE-CLI	RAN2, ZTE

//LSs received during the e-meeting, which are noted

R1-2009358	LS on signalling of satellite backhaul connection	SA2, Samsung

R1-2009359	LS on PC5 DRX operation	SA2, LG Electronics

R1-2009489	Reply LS on maximum data rate for NR sidelink	RAN2, OPPO

R1-2009508	LS on RAN1 agreement on pre-emption	RAN2, LG Electronics

R1-2009605	Reply LS on Latency of NR Positioning Protocols	RAN2, Intel

R1-2009644	LS on per-table MCS range for mode-2	RAN2, OPPO

R1-2009645	Reply LS on definition of NR V2X con-current operation	RAN2, Huawei

R1-2009669	Reply LS on updated Rel-16 LTE and NR parameter lists	RAN2, Ericsson

## RAN1 Aspects for RF requirements for NR frequency range 1 (FR1) (2)

Refer to RP-193266 for details regarding RAN4-led WID on RF requirements for NR frequency range 1 (FR1)

Only essential corrections

Only text proposals are allowed (no individual draft CRs, please!).

- After email discussion/approval of the TPs, individual CRs are to be prepared and approved (managed by the moderator, with source as Moderator (company name))

Limit to maximum 1 contribution per company/organization/university.

- In case when the number of corrections is within the contribution limit, either a TP format with necessary justification or a draft CR format is required

- In case when the number of corrections exceeds the contribution limit, please observe the contribution limit by including two or more TPs in the contribution, where each TP is provided necessary justification

[103-e-NR-LS-TxSwitching-01] Email discussion/approval a potential CR till 10/30 11/2 – Jianchi (CT)

- Clarification on  T^mux_{proc,CSI} (R1-2007603, R1-2007725, R1-2008564)

- Clarification on the ambiguity issue on SCS and align the description on carrier1 and carrier2 with TS 38.331 (R1-2007725, R1-2008229)

Update on 11/2

- The latest TP to 38.213, Section 9.2.5 as in proposal 1 is endorsed   draft CR is approved  Final CR in R1-2009587 (38.213, CR0161).

Other proposals 11/3

Update on 11/4:

Conclusion:

- The CR in R1-2008564 is not needed.

Agreement:

- Adopt the following TP (38.214, Section 6.1.6)   alignment CR

[103-e-NR-LS-TxSwitching-02] Email discussion/approval a potential CR till 10/30 11/2 – Jianchi (CT)

- Back to back switching caused by SRS transmission (R1-2008596)

  - Note 1: no discussion on location of switching period.

  - Note 2: the previous agreements should not be overturned.

- Maximum data rate (R1-2008596)

Update on 11/2: to check on 11/3

Update on 11/4:

Agreements:

- Adopt the following TP (38.306, Section 4.1.2)

NOTE 1:  Only one of the UL or SUL carriers (the one with the higher data rate) is counted for a cell operating SUL.

NOTE 2:  For UL Tx switching between carriers in cell(s), only the supported MIMO layer combination across carriers that results in the highest combined data rate is counted for the cell(s) in the supported maximum UL data rate.

Draft LS to RAN2? 11/5  11/9 R1-2009590

R1-2007603	Discussion on the remaining problems of supporting Tx switching between two uplink carriers			Huawei, HiSilicon

R1-2007725	Remaining Maintenance Issues of UL Tx Switching	ZTE

R1-2008229	Text Proposals for Tx Switching between Two Uplink Carriers	OPPO

R1-2008564	Draft CR to 38.213 on corrections for UL Tx switching	Ericsson

R1-2008596	Remaining issues for 1Tx-2Tx switching	Qualcomm Incorporated

## RAN1 Aspects for RF Requirement Enhancements for FR2

Refer to RP-192653 for details regarding RAN4-led WID on NR RF Requirement Enhancements for FR2

Note: although contributions are allowed, RAN1’s work is “reactive” as follows:

- RAN4 will provide further details on the RAN4 agreed solution(s) to RAN1/RAN2 before RAN1/RAN2 start their work if RAN1/RAN2 help is needed.

- This objective does not aim to propose the same alternatives which were not agreed (i.e. Alt1, Alt2 and Alt3 not agreed in RAN1#98 under Rel-16 NR eMIMO work item)

R1-2007747	Enhancement on FR2 MPE mitigation	ZTE

# E-UTRA (3+10=13)

## Maintenance of E-UTRA Releases 8 – 15 (3)

R1-2008338	Corrections on preamble format indicator presence in NPDCCH order in TS 36.212	Huawei, HiSilicon

R1-2008339	Corrections on preamble format indicator presence in NPDCCH order in TS 36.213	Huawei, HiSilicon

R1-2008341	Corrections on interference randomization for NB-IoT SPS	Huawei, HiSilicon, MediaTek, Qualcomm

R1-2008582	Clarification on the subcarrier allocation for sub-PRB in CE Mode B	Ericsson

[103-e-LTE-6.1CRs-01] Email discussion/approval on R1-2008338 and R1-2008339 (NB-IOT) by 10/29 – Yubo (Huawei)

Final CRs for R1-2008338 are agreed in R1-2009438 (Rel-15, TS36.212, CR0349) and R1-2009439 (Rel-16, TS36.212, CR0350)

Final CRs for R1-2008339 are agreed in R1-2009440 (Rel-15, TS36.213, CR1363) and R1-2009441 (Rel-16, TS36.213, CR1364)

[103-e-LTE-6.1CRs-02] Email discussion/approval on R1-2008341 (NB-IOT) by 10/29 – Yubo (Huawei)

R1-2009564	Summary on [103-e-LTE-6.1CRs-02] on interference randomization for NB-IoT SPS	Moderator (Huawei)

Final CRs for R1-2008341 are agreed in R1-2009424 (Rel-15, TS36.211, CR0549) and R1-2009425 (Rel-16, TS36.211, CR0550)

[103-e-LTE-6.1CRs-03] Email discussion/approval on R1-2008582 (eMTC) by 10/29 – Gerardo (Ericsson)

R1-2009436 (Rel-15, TS36.213, CR1361)	Clarification on the subcarrier allocation for sub-PRB in CE Mode B			Ericsson

R1-2009437 (Rel-16, TS36.213, CR1362)	Clarification on the subcarrier allocation for sub-PRB in CE Mode B			Ericsson

## Maintenance of E-UTRA Release 16 (3+3+1+1+1+1=10)

Only essential corrections

Limit to maximum 1 contribution per company/organization/university for any agenda item, including “Others”, unless otherwise noted

- In case when the number of corrections is within the contribution limit, either a TP format with necessary justification or a draft CR format is required

- In case when the number of corrections exceeds the contribution limit, please observe the contribution limit by including one or more TPs in the contribution(s), where each TP is provided necessary justification

R1-2008793	Alignment corrections for Rel-16 features	FUTUREWEI

[103-e-LTE-AlignmentCR-36211] Email approval of 36.211 alignment CR – Stefan (Ericsson)

Update on 11/12 – the draft CR is approved. Final CR in R1-2009785 (36.211, CR0552)

[103-e-LTE-AlignmentCR-36212] Email approval of 36.212 alignment CR by 11/3 – Brian (Futurewei)

Update on 11/3 – the draft CR is approved. Final CR in R1-2009538 (36.212, CR0352)

[103-e-LTE-AlignmentCR-36213] Email approval of 36.213 alignment CR – Bob (Motorola)

Update on 11/12 – the draft CR is approved. Final CR in R1-2009xxx (36.213, CR0xxxx)

### Maintenance of Additional MTC Enhancements (3)

For this agenda, no more than 2 contribution per 1 company/organization/university

R1-2007713	Remaining issues on scheduling enhancement for MTC	ZTE

R1-2008340	Corrections on multi-TB scheduling for eMTC	Huawei, HiSilicon

R1-2008522	Maintenance on multi-TB scheduling	Qualcomm Incorporated

R1-2008583	PUR maintenance issues for Rel-16 LTE-MTC	Ericsson

R1-2008692	Multi-TB and resource reservation maintenance issues for Rel-16 LTE-MTC	Ericsson

R1-2008800	Corrections on transmission modes for PUR	Huawei, HiSilicon

[103-e-LTE-eMTC5-01] PUR issues – Gus (Sierra Wireless)

- Issue #1: TM1/TM2 configuration issue (R1-2008800)

- Issue #2: TM6/TM9 support issue (R1-2008583, R1-2008800)

- Issue #3: Repetition number delivery to higher layers (R1-2008583)

- Discussion and decision by 10/29, TPs by 11/5

[103-e-LTE-eMTC5-02] Multi-TB issues – Johan (Ericsson)

- Issue #1: Feedback for early termination (R1-2007713, R1-2008340, R1-2008522, R1-2008692)

- Issue #2: TPC command issue (R1-2007713)

- Issue #3: Multicast procedure text indentation issue (R1-2008692)

- Discussion and decision by 10/29, TPs by 11/5

[103-e-LTE-eMTC5-03] Parameter name corrections – Johan (Ericsson)

- Issue #1: PUR-RNTI parameter name corrections (R1-2008583)

- Issue #2: Multi-TB HARQ-ACK bundling parameter name corrections (R1-2008692)

- Issue #3: Resource reservation parameter name corrections (R1-2008692)

- Other potential parameter name issues can also be brought up in the email discussion.

- Discussion and decision by 10/29, TPs by 11/5

### Maintenance of Additional Enhancements for NB-IoT (3)

For this agenda, no more than 2 contribution per 1 company/organization/university

R1-2007712	Correction on L1 adjustment on PUR repetition number	ZTE

R1-2007714	Clarifications on scheduling enhancement for NB-IoT	ZTE

R1-2008584	PUR maintenance issues for Rel-16 NB-IoT	Ericsson

R1-2008797	Corrections on transmission in preconfigured UL resources	Huawei, HiSilicon

[103-e-LTE-NB_IoTenh3-01] PUR issues – Xiang (Huawei)

- Issue #1: UE monitoring both PUR search space and NPSS/NSSS/NPBCH (R1-2008797)

- Issue #2: L1 adjustment on PUR repetition number (R1-2007712 , R1-2008584 section 2.1)

- Discussion and decision by 10/29, TPs by 11/5

[103-e-LTE-NB_IoTenh3-02] Parameter name corrections – Xiang (Huawei)

- Issue #1: PUR-RNTI parameter name corrections (R1-2008584 section 2.2)

- Other parameter name updates if any

- Discussion and decision by 10/29, TPs by 11/5

[103-e-LTE-NB_IoTenh3-03] Multi-TB issues – Huiying (ZTE)

- Issue #1: clarification of HARQ ID assumption (section 2.1 of R1-2007714)

- Issue #2: clarification of usage of NDI (section 2.1 of R1-2007714)

- Discussion and decision by 10/29, TPs by 11/5

### Maintenance of DL MIMO efficiency enhancements for LTE (1)

R1-2008798	Corrections on additional SRS symbols	Huawei, HiSilicon

[103-e-LTE-LTE_DL_MIMO_EE-01] Corrections on additional SRS symbols – Yubo (Huawei)

- Discussion and agreement by 10/27, TPs by 10/29

### Maintenance of LTE-based 5G Terrestrial Broadcast (1)

R1-2007740	Draft 36.211 CR on LTE-based 5G Terrestrial Broadcast	ZTE

Final CR for R1-2007740 agreed in R1-2009422 (TS36.211, Rel-16, CR0547)

R1-2008523	Correction for 0.37kHz SCS	Qualcomm Incorporated

Final CR for R1-2008523 agreed in R1-2009423 (TS36.211, Rel-16, CR0548)

[103-e-LTE_TerrBcast-01] Email discussion/approval on R1-2007740 and R1-2008523 (Corrections to TS 36.211 for new numerologies) until 10/28 – Alberto (Qualcomm)

### LTE Rel-16 UE Features (1)

R1-2007912	Summary of Other UE features for LTE	Moderator (AT&T)

[103-e-LTE-UEFeatures-LS] Email approval of updated Rel-16 LTE UE features by 11/6 – Ralf (AT&T)

Note

- No dedicated email thread for LTE V2X

  - Use the customary email approval of the RAN2 LS on the updated LTE UE features to discuss anything that needs to be changed for LTE V2X

  - Scope of the LTE LS thread would be to align LTE V2X with NR V2X agreements throughout RAN1 #103-e

Update on 11/5:

- The updated RAN1 UE features list for Rel-16 LTE and the corresponding LS on updated Rel-16 RAN1 UE features lists for LTE are approved, with final documents in  R1-2009351 (feature list) and R1-2009350 (LS)

R1-2008235	Discussion on LTE Rel-16 UE features	OPPO

### Others (1)

R1-2008524	Power control for DAPS	Qualcomm Incorporated

R1-2008799	Draft CR on power sharing for LTE DAPS	Huawei, HiSilicon

[103-e-LTE-6.6CRs] Email discussion/approval on R1-2008524 and R1-2008799 (DAPS) by 10/29 – Alberto (Qualcomm)

R1-2009533	Summary of email discussion [103-e-LTE-6.6CRs]	Moderator (Qualcomm Incorporated)

R1-2009528 (Rel-16, TS36.213, CR1367)	Power sharing for LTE DAPS	Moderator (Qualcomm Incorporated), Huawei

# NR Maintenance (10+51=61)

## Maintenance of Release 15 NR (10)

Only essential corrections – a rejected draft CR will be marked in red

Including CRs rejected for Rel-15, but can be considered for Rel-16. For each of these contributions, please clearly indicate that it’s originated from Rel-15 maintenance discussion (but not adopted), and is for Rel-16 maintenance consideration only

R1-2007624	38.212CRdraft (Rel-16,F) RRC IE name fix to dynamic frequency domain resource allocation type selection (Rel-15 origin)	Nokia, Nokia Shanghai Bell

R1-2007722	Discussion on Ambiguity Issues for UE Features with Cross-Carrier Operation	ZTE

R1-2007723	Draft 38.213 CR on HARQ-ACK Determination for SPS Release	ZTE

R1-2007758	Discussion on determining PHR for PUSCH in CA	ZTE

R1-2007759	Draft CR on determining PHR for PUSCH in CA	ZTE

R1-2007760	Discussion on closed-loop power control with an absolute TPC command	ZTE

R1-2007761	Draft CR on closed-loop power control with an absolute TPC command	ZTE

R1-2007762	Draft CR on power offset between SSB and CSI-RS	ZTE

R1-2007802	Correction for UCI on Msg3 PUSCH and MsgA PUSCH	CATT

R1-2007803	Correction for type1 HARQ-ACK codebook	CATT

R1-2007804	Correction on HARQ-ACK generation for DL transmission with single TB when multi-TB is configured	CATT

R1-2007805	Correction on UE sounding procedure	CATT

R1-2008121	Draft CR on configured grant based PUSCH	Samsung

R1-2008122	38.211 DRAFT CR (Rel-16, F) on k_SSB calculation	Samsung

R1-2008123	Draft CR for 38.211 for UE receiver behavior for a RS and its QCL reference	Samsung

R1-2008423	Clarification of FG3-5b with Cross Carrier Operation	Apple

R1-2008424	Discussion on PMI/RI restrictions for Type I Single Panel Codebook with 2 Tx	Apple

R1-2008425	Draft CR on PMI/RI restrictions for Type I Single Panel Codebook with 2 Tx	Apple

R1-2008426	Discussions on PUSCH skipping	Apple

R1-2008507	Clarification on Numerology for Active DL and UL BWPs	MediaTek Inc.

R1-2008526	Rel-15 CR on uplink transmission combination with PUCCH	NTT DOCOMO, INC.

R1-2008527	Rel-16 CR on uplink transmission combination with PUCCH	NTT DOCOMO, INC.

R1-2008528	Discussion on CG skipping vs UCI multiplexing on CG PUSCH	NTT DOCOMO, INC.

R1-2008597	Draft CR on NNZC indicator of Rel-15 Type II CSI codebook	Qualcomm Incorporated

R1-2008598	Discussion on CSI-RS ambiguities	Qualcomm Incorporated

R1-2008599	Discussion on SRS carrier switching	Qualcomm Incorporated

R1-2008653	Discussion on UE FG in case of cross-carrier operation	vivo

R1-2008654	Disucssion on configured grant PUSCH with overlapping UCI on PUCCH in Rel-16	vivo

R1-2008655	Correction on 38.214 for PUSCH with UL skipping in Rel-16	vivo

R1-2008656	Draft CR on correction for Msg3 retransmission power control	vivo

R1-2008657	Corrections on aperiodic SRS triggering  minimal time interval	vivo, Nokia, Nokia Shanghai Bell

R1-2008658	Discussion on aperiodic SRS triggering  minimal time interval and zero slot offset	vivo

R1-2008659	Correction on beamswitch timing for aperiodic TRS	vivo

R1-2008683	Draft 38.214 CR (Rel-16, F, Rel-15 originating) to fix configurable xOverhead values for TBS determination	Nokia, Nokia Shanghai Bell

R1-2008767	Clarification on the PUSCH scheduling constraint in Rel-15	Apple Inc.

R1-2008776	Discussion on UL skipping for CG PUSCH	Huawei, HiSilicon

R1-2008777	Correction on supplementary uplink in 38.213	Huawei, HiSilicon

R1-2008786	Corrections on radio link monitoring in 38.213 in Rel-15	Huawei, HiSilicon

R1-2008787	Clarifications on UE features in case of cross-carrier operation	Huawei, HiSilicon

[103-e-NR-7.1CRs-01] Ambiguity Issues for UE Features with Cross-Carrier Operation (Rel-15) – Xingguang (ZTE)

- Discussion and decision by 10/29, TPs by 11/5

[103-e-NR-7.1CRs-02] Correction on HARQ-ACK Determination for SPS Release (Rel-15) – Xingguang (ZTE)

- Discussion and decision by 10/27, TPs by 10/29

[103-e-NR-7.1CRs-03] Correction on supplementary uplink in 38.213 (Rel-15) – Huawei

- Discussion and decision by 10/27, TPs by 10/29

[103-e-NR-7.1CRs-04] Correction on HARQ-ACK generation for DL transmission with single TB when multi-TB is configured (Rel-15) – CATT

- Discussion and decision by 10/27, TPs by 10/29

[103-e-NR-7.1CRs-05] RRC IE name fix to dynamic frequency domain resource allocation type selection (Rel-16) – Nokia

- Discussion and decision by 10/27, TPs by 10/29

[103-e-NR-7.1CRs-06] Correction for UCI on Msg3 PUSCH and MsgA PUSCH (Rel-16) – CATT

- Discussion and decision by 10/29, TPs by 11/5

[103-e-NR-7.1CRs-07] 38.211 DRAFT CR (Rel-16, F) on k_SSB calculation – Hongbo (Samsung)

- Discussion and decision by 10/27, TPs by 10/29

[103-e-NR-7.1CRs-08] Discussions on PUSCH skipping (Rel-16) – Xiaohang (vivo)

- Discussion and decision by 10/29, TPs by 11/5

[103-e-NR-7.1CRs-09] Draft 38.214 CR (Rel-16, F, Rel-15 originating) to fix configurable xOverhead values for TBS determination (Rel-16) – Nokia

- Discussion and decision by 10/27, TPs by 10/29

[103-e-NR-7.1CRs-10] Review of draft CRs not discussed under any of the emails threads [103-e-NR-7.1CRs-01] ~ [103-e-NR-7.1CRs-9] – Youngbum (Samsung)

- Discussion and decision by 10/29

- Decide whether to reject or continue discussions in future meetings

## Maintenance of Release 16 NR (1+7+2+12+7+4+1+2+2+2+9+2=51)

Only essential corrections

Limit to maximum 1 contribution per company/organization/university for any agenda item, including “Others”, unless otherwise noted

- In case when the number of corrections is within the contribution limit, either a TP format with necessary justification or a draft CR format is required

- In case when the number of corrections exceeds the contribution limit, please observe the contribution limit by including one or more TPs in the contribution(s), where each TP is provided necessary justification

R1-2007794	Alignment of RRC parameter names	Ericsson

R1-2008124	Alignment CR for TS 38.213	Samsung

R1-2008202	Alignment of RRC parameter names	Intel Corporation (UK) Ltd

R1-2008292	Alignment of RRC parameter names for 38.214	Nokia, Nokia Shanghai Bell

R1-2008792	Alignment of RRC parameter names for TS 38.212	Huawei

[103-e-NR-AlignmentCR-38211] Email approval of 38.211 alignement CR by 11/3 - Stefan (Ericsson)  11/5 11/11

Latest draft is approved. Final CR in R1-2009751 (38.211, CR0064)

[103-e-NR-AlignmentCR-38212] Email approval of 38.212 alignement CR by 11/3 - Zukang (Huawei)  11/511/11

Latest draft is approved. Final CR in R1-200xxxx (38.212, CR00xx)

[103-e-NR-AlignmentCR-38213] Email approval of 38.213 alignement CR b 11/3 - Aris (Samsung)  11/511/11

Latest draft is approved. Final CR in R1-2009742 (38.213, CR0176)

[103-e-NR-AlignmentCR-38214] Email approval of 38.214 alignement CR by 11/3  - Mihai (Nokia)  11/511/11

Latest draft is approved. Final CR in R1-2009778 (38.214, CR0154)

[103-e-NR-AlignmentCR-38215] Email approval of 38.215 alignement CR by 11/3 - Alexei (Intel)  11/511/11

Latest draft is approved. Final CR in R1-2009777 (38.215, CR0034)

### Maintenance of Two step RACH for NR (1)

R1-2007964	FL summary on the maintenance of Rel-16 2-step RACH	Moderator (ZTE)

[103-e-NR-2Step-01] Email discussion/approval for potential CR(s) including the following issues:

Ÿ   TP#1 in R1-2008418 (editorial)

Ÿ   TP#1 in R1-2008785 (correction)

Ÿ   TP#2 in R1-2008785 (correction/clarification)

till 10/29 – Li (ZTE)

Update on 10/29 – the 38.211 CR and the 38.213 CR (either #3 or #3+#1) are approved. Final CRs in R1-2009445 (38.211, CR0053) and R1-2009446 (38.213, CR0147)

R1-2007963	Text proposal on the MsgA configuration in unlicensed band	ZTE, Sanechips

R1-2008418	Maintenance of Two step RACH	Ericsson

R1-2008660	Miscellaneous corrections for 2-step RACH	vivo

R1-2008785	Remaining issues for 2-step RACH	Huawei, HiSilicon

### Maintenance of NR-based Access to Unlicensed Spectrum (7)

For this agenda, no more than 4 contribution per 1 company/organization/university

R1-2008693	Feature lead summary for NR-U DL Signals and Channels	Moderator (Lenovo)

R1-2008768	Feature lead summary of NR-U enhancements to initial access procedures	Moderator (Charter Communications)

R1-2008801	Feature Lead Summary on Channel Access Procedures for NR-U	Moderator (Nokia)

R1-2007606	Maintenance on initial access signals and channels	Huawei, HiSilicon

R1-2007607	Maintenance on DL signals and channels	Huawei, HiSilicon

R1-2007608	Maintenance on channel access procedures	Huawei, HiSilicon

R1-2007609	Maintenance on HARQ-ACK enhancement	Huawei, HiSilicon

R1-2007777	Clarification on restriction of initial BWP configuration	Fujitsu

R1-2007778	Clarification on resource allocation for PUSCH scheduled by RAR UL grant or DCI addressed to TC-RNTI	Fujitsu

R1-2007903	Remaining Issues and Corrections on Channel Access Procedures and Configured Grants for NR-U		Nokia, Nokia Shanghai Bell

R1-2007933	Remaining issues on NR-U	Intel Corporation

R1-2007959	Text proposals on the initial access for NR-U	ZTE, Sanechips

R1-2007960	Text proposal on the channel measurement and interference measurement	ZTE, Sanechips

R1-2007961	Text proposals on type-3 HARQ-ACK codebook and multi-PUSCH scheduling	ZTE, Sanechips

R1-2007962	Remaining issues on the configured grant for NR-U	ZTE, Sanechips

R1-2007978	Initial Access Signals and Channels	Ericsson

R1-2007979	DL Signals and Channels	Ericsson

R1-2007980	Channel Access Procedures	Ericsson

R1-2007981	HARQ Corrections	Ericsson

R1-2007985	Remaining issues on UL transmissions	ETRI

R1-2008041	Remaining issues of DL signals and channels for NR-U	LG Electronics

R1-2008042	Remaining issues of random access for NR-U	LG Electronics

R1-2008043	Remaining issues of channel access procedure and configured grant for NR-U	LG Electronics

R1-2008044	Remaining issues of HARQ procedure for NR-U	LG Electronics

R1-2008125	Remaining issues on Initial access signals and channels	Samsung

R1-2008126	Remaining issues on DL signals and channels	Samsung

R1-2008127	Remaining issues on channel access	Samsung

R1-2008128	Remaining issues on HARQ	Samsung

R1-2008204	Remaining issues on DL signals and channels	Nokia, Nokia Shanghai Bell

R1-2008205	Remaining issues on Wideband operation in NR-U	Nokia, Nokia Shanghai Bell

R1-2008206	Remaining issues on NR-U HARQ and Initial Access Procedures	Nokia, Nokia Shanghai Bell

R1-2008246	Discussion on the remaining issues of initial access signal/channel	OPPO

R1-2008247	Discussion on the remaining issues of DL signals and channels	OPPO

R1-2008248	Discussion on the remaining issues of channel access procedure	OPPO

R1-2008249	Discussion on the remaining issues of HARQ enhancements	OPPO

R1-2008383	Remaining issues on channel access procedure for NR-U	Sharp

R1-2008384	Remaining issues on DL signals/channels for NR-U	Sharp

R1-2008385	Remaining issues on initial access and UL signals/channels for NR-U	Sharp

R1-2008386	Remaining corrections for wideband operation for NR-U	Sharp

R1-2008521	Correction on SRS position	CATT

R1-2008600	PO configuration for 2-step RACH over multiple RB sets	Qualcomm Incorporated

R1-2008601	TP for Channel access procedures for NR unlicensed	Qualcomm Incorporated

R1-2008602	TP for DL signals and channels for NR-U	Qualcomm Incorporated

R1-2008603	TP for Wideband operation for NR-U Operation	Qualcomm Incorporated

R1-2008632	Remaining issues for DFI in NR-U	ASUSTeK

R1-2008661	Remaining issues on HARQ operation for NR-U	vivo

R1-2008662	Remaining issues on CG-PUSCH	vivo

R1-2008663	TPs on HARQ feedback in CG-DFI for CBG-based PUSCH transmission	vivo

R1-2008664	TPs on DL signals and channels	vivo

R1-2008724	Correction on UL channel access procedure for NR-U	WILUS Inc.

[103-e-NR-NRU-01] Email discussion/approval on issues IA-A and IA-B in R1-2008888 until 10/29 with potential CRs by 11/4 – Jing (Qualcomm)

R1-2009409	Summary of email discussion 01 for NR-U	Moderator (Qualcomm)

Agreement:

For length 139 PRACH sequence, legacy RO configuration is used and UE does not expect a configure RO to (partially) overlap with intra-cell guard band

Agreement:

For 2-step RACH, all ROs and POs are confined within a single RB set

- For RO with L139 sequence, legacy RO configuration is applied, and UE does not expect a configured RO to (partially) overlap with intra-cell guard band

- For RO with L1151 and L571 sequences, UE does not expect configured ROs to be allocated over multiple RB sets.

- For PO with non-interlaced waveform, UE does not expect a configured PO to (partially) overlap with intra-cell guard band or (partially) overlap with an RB set different from the RB set configured with the corresponding ROs

- For PO with interlaced waveform, the RB set allocation for the MsgA PUSCH transmission is the RB set corresponding to the associated MsgA PRACH transmission

The TPs in Section 3 of R1-2009409 are endorsed in R1-2009627 (Rel-16, TS38.211, CR0057)

The TPs in Section 3 of R1-2009409 are endorsed in R1-2009628 (Rel-16, TS38.213, CR0165)

[103-e-NR-NRU-02] Email discussion/approval on issues DL-B6, DL-D1 and DL-G1 in R1-2008888 until 10/29 with potential CRs by 11/5 – Alex (Lenovo)

R1-2009429	Summary of [103-e-NR-NRU-02] email discussion	Moderator (Lenovo)

R1-2009703 (38.213CR0175 Rel-16, F)	Correction on remaining channel occupancy assumption	Moderator (Lenovo)

Conclusion:

For semi-persistent CSI reporting with PUCCH, if UE cannot transmit HARQ-ACK on the MAC CE deactivation due to the UL LBT failure, the UE performs deactivation at the MAC action time based on the original scheduled HARQ-ACK transmission (the original scheduled HARQ-ACK transmission time corresponds to slot n in TS 38.214 clause 5.2.1.5.2).

R1-2009689        Reply LS on Clarification of UE behavior after receiving the MAC CE deactivation command for semi-persistent CSI reporting in NR-U

[103-e-NR-NRU-03] Email discussion/approval on issue UL-01 in R1-2008888 until 10/29 with potential CRs by 11/4 – Steve (Ericsson)

TP#1 in R1-2009357 for Clause 5.3.2 of TS38.211 is endorsed in R1-2009549 (TS38.211, CR0055, Rel-16)

[103-e-NR-NRU-04] Email discussion/approval on issues CA2.1, CA2.3, CA2.7 and CA2.8 in R1-2008888 until 10/29 with potential CRs by 11/5 – Timo (Nokia)

R1-2009698	CR to 37.213 to correct CP extension and LBT type for SRS	Moderator (Nokia), Samsung, Huawei, Broadcom, Ericsson, LGE

R1-2009699	CR to 37.213 CR to correct CAPC for RACH	Moderator (Nokia), Samsung

R1-2009700	CR to 37.213 to correct channel access for SRS	Moderator (Nokia), ETRI

R1-2009701	CR to 38.211 to correct CP extension for SRS	Moderator (Nokia), Samsung

R1-2009702	CR to 38.213 to correct references to 38.212 for RACH procedure	Moderator (Nokia), Ericsson

[103-e-NR-NRU-05] Email discussion/approval on issue HARQ-OOO, in R1-2008888 until 10/29 with potential CRs by 11/4 – David (Huawei)

R1-2009595	Feature lead summary#1 for NRU HARQ [103-e-NR-NRU-05]	Moderator (Huawei)

R1-2009596	Correction of NRU HARQ procedure in the presence of SPS PDSCH	Moderator (Huawei)

Final CR agreed in R1-2009611 (Rel-16, TS38.213, CR0163)

[103-e-NR-NRU-06] Email discussion/approval on issue CG1, CG4, CG5 and CG2, in R1-2008888 until 10/29 with potential CRs by 11/5 – Rakesh (Vivo)

R1-2009413	Summary of [103-e-NR-NRU-06] email discussion	Moderator (Vivo)

R1-2009550 (Rel-16, TS38.213, CR0159)	Miscellaneous corrections on NR unlicensed configured grant	Moderator (vivo), ASUSTeK, Intel, Huawei, HiSilicon

[103-e-NR-NRU-07] Email discussion/approval on editorial changes for issues IA-C, DL-Z1, DL-Z2, DL-Z3, UL-02, and WB04 in R1-2008888 until 10/29 – Jing (Qualcomm)

Agreement:

The following TPs in R1-2009410 are endorsed (to be forwarded to editors):

TP 2-1 in section 2.

TP 3-2a in section 3.

TP 4-1 in section 4.

TP 5-2 in section 5

TP 6-1a in section 6.

TP 7-1 in section 7.

TP 7-2b in section 7

TP 5-1a in section 5

### Maintenance of Integrated Access and Backhaul for NR (2)

R1-2008328	Corrections on PDCCH monitoring of DCI format 2_5	Huawei, HiSilicon

R1-2008409	Correction on PDCCH monitoring for IAB-MT	LG Electronics

R1-2008707	Corrections for Rel-16 IAB	Ericsson

R1-2008742	Maintenance for Rel-16 IAB	Nokia, Nokia Shanghai Bell

[103-e-NR-IAB-01] Corrections for DCI Format 2_5 monitoring – Thomas (AT&T)

- Harmonize proposals in R1-2008328, R1-2008409, R1-2008742 to approve a single CR for 38.213.

- Discussion and decision by 10/29, TPs by 11/4

[103-e-NR-IAB-02] Issue with IAB-DU “serving” cell terminology – Thomas (AT&T)

- Determine whether the term “serving cells” should be removed for IAB-DUs in 38.213

- Discussion and decision by 10/27, TPs (if needed) by 10/29

### Maintenance for 5G V2X with NR sidelink (12)

For this agenda, no more than 5 contribution per 1 company/organization/university

R1-2008134	Feature lead summary#1 for Physical layer structure for sidelink	Moderator (Samsung)

[103-e-NR-Rel-16-V2X-01] Email discussion/approval for remaining issues for PT-RS design

- Issue PS-1-1: For PT-RS sequence generation, when r(m) is defined, which DM-RS symbol is used for reference is not clear.

- Issue PS-1-2: Removal of duplication for PT-RS mapping

till 10/29, with a potential CR by 11/4 – Jeongho (Samsung)

Update on 10/29:

Agreements:

For Clause 8.4.1.2 of TS 38.211, the following TP is adopted.

-          r(m) is given by clause 8.4.1.1.1 at the position of a DM-RS symbol  the first PSSCH symbol carrying an associated DM-RS.

Second issue 11/3  made a suggestion  check 11/4

Draft CR till 11/4  the 38.211 draft CR is approved. Final CR in R1-2009662(38.211, CR0059)

Agreements:

- Endorse the TP to remove “sidelink CSI-RS”

  - To be included in editor’s alignment CR

[103-e-NR-Rel-16-V2X-02] Email discussion/approval regarding location of PSFCH/PSSCH/PSCCH/2nd SCI mapping and PSCCH precoding

- Issue PS-2-1: To clarify starting symbol and location for PSFCH

- Issue PS-2-2: To clarify time-domain location of PSSCH

- Issue PS-2-3: To clarify time-domain and frequency-domain location of PSCCH

till 10/29, with a potential CR by 11/4 – Jeongho (Samsung)

Update on 10/29:

Agreements:

- The latest TPs to 38.213/38.211 as in Proposal 2-1 are endorsed

- No change is needed for time-domain location of PSSCH as discussed in Proposal 2-2

- As in Proposal 2-3:

  - No change is needed for time-domain location of PSCCH.

  - For frequency-domain location of PSCCH, the 38.213 TP is endorsed.

Draft CR till 11/4  11/5 morning

Update on 11/5:

- Both the 38.211 draft CR and 38.213 draft CR are approved. Final CRs in R1-2009663 (38.211, CR0060) and in R1-2009674 (38.213, CR0172).

[103-e-NR-Rel-16-V2X-03] Email discussion/approval regarding toutstanding agreement from RAN1#102-e on the minimum gap between PSFCH and next SL retransmission.

- Issue M1-1-1: Conditions for applicability

- Issue M1-1-2: Value of delta

- Also include the issue of sl-DCI-ToSL-Trans for DCI 3-0

till 10/29, with a potential CR by 11/4 – Ricardo (Ericsson)

Update on 10/29:

Agreements:

- The value of delta in the agreement from RAN1#102 is:

  - 0.5 ms

Two more proposals? 11/2

Update on 11/2
Agreements:

- The agreement from RAN1#102 applies when the grant is provided for a pool configured with PSFCH resources and the UE uses it for a TB with SL HARQ-feedback enabled.

Agreements:

- The configurable values for sl-DCI-ToSL-Trans are:

  - 1-32 slots (using SL numerology)

2nd bullet of proposal 1-1? Any CRs? 11/3

Update on 11/3:

Agreements:

- If the time between PSFCH reception and next scheduled PSCCH/PSSCH retransmission is less than Tprep + delta, the UE is allowed to drop the PSCCH/PSSCH retransmission with SL HARQ feedback enabled.

  - Note: it is RAN1 understanding that the UE is allowed to drop the PSCCH/PSSCH retransmission only if the UE can not complete the PSFCH processing and the preparation of the next PSCCH/PSSCH retransmission in the time between PSFCH reception and the next scheduled PSCCH/PSSCH retransmission

Draft CRs & LS  - 11/4

- Draft CR in R1-2009547 for 38.213 is approved. Final CR in R1-2009578 (38.213, CR0160)

- Draft CR in R1-2009548 for 38.214 is approved. Final CR in R1-2009579 (38.214, CR0142)

- Draft LS (+one citation of agreements) is approved. Final LS in R1-2009577.

[103-e-NR-Rel-16-V2X-04] Email discussion/approval regarding e-evaluation procedure for periodic resource reservations

- Issue M2-1: Fix undefined UE behaviour for the case of re-evaluation performed during periodic reservation process

- Issue M2-7: Fix the issue of unreachable pre-emption event condition due to prior exclusion of slots related to non-monitored slots in the sensing window

till 10/30 11/2, with a potential CR by 11/4 – Sergey (Intel)

Update on 11/2: to check on 11/3  11/4  11/5 monring  11/9 final proposal

Update on 11/10:

Agreements:

- If periodic reservation is in use by a UE, the UE performs re-evaluation check for resources provided by MAC layer to L1, according to specified procedures

  - L1 expects that MAC layer provides resources intended for transmission of one TB, which can fit to resource selection window of current TB of the UE, and for which the relevant priority is available

  - Re-evaluation check is not applied to the resources that have been signalled in current period or previous periods as per agreements, except that it is up to UE implementation whether to apply re-evaluation check to the resource in non-initial reservation period that have not been signalled neither in the immediate last nor in the current period

  - If a resource is indicated for re-evaluation, a re-selection for the resource is performed according to the specified step 2 procedure

- NOTE: re-evaluation for the purpose of SPS period signalling in non-initial reservation period is neither supported nor precluded by this agreement

Agreements:

- When a UE checks pre-emption for a resource, the UE identifies a candidate resource set based on steps 1-7 in clause 8.1.4 TS 38.214

  - After the candidate set is identified, the UE checks SL-RSRP measurement and priority condition as per agreements, for resource(s) {r’} subject to pre-emption overlapping with received SCI 1-A and not included in the candidate set, where the RSRP threshold is the final threshold after executing steps 1-7 i.e. includes all necessary increments for reaching X%.

Email summary in R1-2009469

[103-e-NR-Rel-16-V2X-05] Email discussion/approval regarding gindication/derivation of TDD configuration.

- Issue SY-1-1: TDD configuration derivation

- Issue SY-1-2: TDD configuration for OoC UEs

till 10/29, with a potential CR by 11/4 – Teng (CATT)

Update on 10/29:

- The latest 38.213 TP (issue 1-2) is endorsed

Issue 1-1? 11/2  11/3

CR? 11/4

Update on 11/3:

Agreements:

For TDD configuration carried in PSBCH, the ambiguous issue between InC and OoC UEs can be eliminated up to network implementation. For TDD configuration for resource pool determination, sl-TDD-Config in PSBCH is used for OOC UE, if the UE selected a SyncRef UE.

-            No spec change is needed for TDD configuration ambiguity issue.

-            Minor spec change is needed for resource pool determination based on PSBCH.

The 38.213 draft CR and the 38.214 draft CR are approved. Final CRs in R1-2009531 (38.213, CR0156) and in R1-2009532 (38.214, CR0141).

Final summary in R1-2009535.

[103-e-NR-Rel-16-V2X-06]: Email discussion/approval regarding remaining issues in prioritization between SL and UL

- Issue PP-2: Prioritization rule between PSCCH/PSSCH and PUCCH without SL HARQ reports and SR/PUSCH without UL-SCH/SRS

- Issue PP-3: Prioritization rule between PSFCH/S-SSB reception(s) and UL transmission(s)

- Issue PP-4: PRACH and MsgA PUSCH are prioritized over SL transmission

till 10/30 11/2, with a potential CR by 11/4 – Hanbyul (LGE)

Update on 11/2

Agreements:

For prioritization between PSCCH/PSSCH TX and UL TX,

- When the overlapping UL TX is PUCCH with neither SL HARQ reports nor SR, PUSCH without UL-SCH, or SRS,

  - when UL TX is associated with a DCI indicating “high” in “priority field” or configured with “high priority” by higher layers (i.e., URLLC case)

    - If SL-threshold for URLLC case is configured, LTE rule is used (i.e., UL TX is down-prioritized if the priority value of SL-TX is smaller than SL-threshold, otherwise prioritized)

    - Otherwise, UL TX is prioritized

  - Otherwise, LTE rule is used with another SL-threshold configured for non-URLLC case

Agreements:

For prioritization between PSFCH/S-SSB RX and UL TX,

- The priority of PSFCH RX is the highest priority of the associated PSCCH/PSSCH

- The priority of S-SSB RX is equal to the (pre-)configured priority for the Rx UE, which is introduced for in-device coexistence.

- When the overlapping UL TX other than PUCCH carrying SL HARQ reporting, PRACH, PUSCH scheduled by RAR UL grant, and PUSCH for Type-2 random access procedure (i.e. MsgA),

  - when UL TX is associated with a DCI indicating “high” in “priority field” or configured with “high priority” by higher layers (i.e., URLLC case)

    - If SL-threshold for URLLC case is configured, LTE rule is used (i.e., UL TX is down-prioritized if the priority value of SL-TX is smaller than SL-threshold, otherwise prioritized)

    - Otherwise, UL TX is prioritized

  - Otherwise, LTE rule is used with another SL-threshold configured for non-URLLC case

Agreements:

- PRACH, PUSCH scheduled by RAR UL grant and its retransmission, PUSCH for Type-2 random access procedure (i.e. MsgA) and its retransmission are always prioritized over SL transmissions or receptions

CRs? 11/4  11/5  draft 38.213 CR is approved. Final CR in R1-2009516 (38.213, CR0153)

[103-e-NR-Rel-16-V2X-07]: Email discussion/approval regarding clarification on the sidelink slot index

- Issue PP-7: Interpretation of sidelink slot for TRIV and resource reservation period

- Issue M2-6: Interpretation of sidelink slot for sensing and resource selection procedure

till 10/29, with a potential CR by 11/4 – Hanbyul (LGE)

Update on 10/29:

Agreements:

Agreements:

Proposal 2? 11/2  11/4  11/5 morning  no conclusion.

CRs? 11/4  the 38.213 draft CR and 38.214 draft CR (without the changes in the last paragraph) are approved. Final CRs in R1-2009517(38.213, CR0154) and R1-2009518 (38.214, CR0140).

[103-e-NR-Rel-16-V2X-08]: Email discussion/approval of CRs for the agreements from previous meetings (Physical layer structure)  by 10/30 11/2 - Jeongho (Samsung)

Update on 11/2

- The latest draft 38.211/213/214 CRs are endorsed (+cover page fix). Final CRs in R1-2009640 (38.211, CR0050), R1-2009641 (38.213, CR0171), R1-2009642 (38.214, CR0145)

[103-e-NR-Rel-16-V2X-09]: Email discussion/approval of CRs for the agreements from previous meetings (Mode 1) by 10/30 11/2 – Ricardo (Ericsson)

Update on 11/2

- Draft 38.213 CR in R1-2009463 is endorsed. Final CR in R1-2009497 (38.213, CR0152)

Final summary in R1-2009511.

[103-e-NR-Rel-16-V2X-10]: Email discussion/approval of CRs for the agreements from previous meetings (Mode 2) by 10/30 11/2 – Sergey (Intel)

Update on 11/2:

- The latest 38.214 TP is endorsed.  38.214 alignment CR

- The latest 38.212 TP is endorsed  38.212 alignment CR

Draft LS (R1-2009472) is approved. Final LS in R1-2009474.

[103-e-NR-Rel-16-V2X-11]: Email discussion/approval of CRs for the agreements from previous meetings (Synchronization) by 10/30 11/2 – Teng (CATT)

Update on 11/2:

- The latest draft 38.211 and 38.213 CRs are approved. Final CR in R1-2009507 (38.211, CR0054), and R1-2009520 (38.213, CR0155).

Email summry in R1-2009535.

[103-e-NR-Rel-16-V2X-12]: Email discussion/approval of CRs for the agreements from previous meetings (Physical layer procedure) by 10/30 11/2 – Hanbyul (LGE)

Update on 11/2:

- The latest draft 38.213 CR is approved. Final CR in R1-2009402 (38.213, CR0145)

[103-e-NR-Rel-16-V2X-13]: For LS in R1-2007521, a reply LS is necessary – target 10/30 11/2 for email approval. To be handled under 7.2.4 – Sergey (Intel)

Update on 10/29:

Agreements:

- For reply to Q2 of R1-2007521, state that RAN1 has agreed to align specification(s) containing ‘SL-L-CS-RNTI’ with ‘SL Semi-Persistent Scheduling V-RNTI’.

  - The TP in Section 2.2 is endorsed – to be captured in editor’s alignment CR

Proposal 1 & proposal 3? 11/2  11/3  latest draft LS (R1-2009473) is approved. Final LS in R1-2009475.

[103-e-NR-Rel-16-V2X-14]: For LS in R1-2007522, a reply LS is necessary – target 10/29 for email approval. To be handled under 7.2.4 – Ricardo (Ericsson).

Update on 10/29:

- Draft LS in R1-2009374 is approved. Final LS in R1-2009460.

Email summary in R1-2009512

[103-e-NR-Rel-16-V2X-15]: Email discussion/approval of reply LS to R1-2009508 by 11/9 – Sergey (Intel)

11/9 – to check on 11/10 morning  latest answer+draft is approved.  to update to include the agreed proposal 1 from email thread 04  11/12  latest draft CR is approved. With final LS in R1-2009663.

R1-2007610	Correction on sidelink PT-RS sequence generation	Huawei, HiSilicon

R1-2007611	Remaining details of sidelink resource allocation mode 1	Huawei, HiSilicon

R1-2007612	Remaining details of sidelink resource allocation mode 2	Huawei, HiSilicon

R1-2007613	Remaining details of physical layer procedures for sidelink	Huawei, HiSilicon

R1-2007772	Discussion on essential corrections in physical layer structure	LG Electronics

R1-2007773	Discussion on essential corrections in resource allocation for Mode 1	LG Electronics

R1-2007774	Discussion on essential corrections in resource allocation for Mode 2	LG Electronics

R1-2007775	Discussion on essential corrections in sidelink synchronization mechanism	LG Electronics

R1-2007776	Discussion on essential corrections in physical layer procedure	LG Electronics

R1-2007779	A remaining issue on UE procedures for reporting HARQ-ACK on uplink	Fujitsu

R1-2007780	A remaining issue on simultaneous transmissions of uplink and PUSCH carrying sidelink HARQ-ACK	Fujitsu

R1-2007809	Remaining issues on physical layer structure for NR sidelink	CATT

R1-2007810	Remaining issues on Mode 1 resource allocation in NR V2X	CATT

R1-2007811	Remaining issues on Mode 2 resource allocation in NR V2X	CATT

R1-2007812	Remaining issues on sidelink synchronization mechanism in NR V2X	CATT

R1-2007813	Remaining issues on physical layer procedures for NR V2X	CATT

R1-2007921	Remaining issues of NR sidelink physical layer structure	ZTE, Sanechips

R1-2007922	Remaining issues in Mode-1	ZTE, Sanechips

R1-2007923	Remaining issues in mode 2	ZTE, Sanechips

R1-2007924	Remaining issues of synchronization	ZTE, Sanechips

R1-2007925	Remaining issues in PHY procedures for Rel-16 sidelink	ZTE, Sanechips

R1-2007934	Remaining opens of sidelink physical structure for NR V2X design	Intel Corporation

R1-2007935	Corrections related to Mode-2 resource allocation	Intel Corporation

R1-2007936	Corrections related to Mode-1 resource allocation	Intel Corporation

R1-2007986	Remaining issues on resource allocation mode 2 for NR V2X	ETRI

R1-2007987	Physical layer procedures for sidelink	ETRI

R1-2008081	Maintenance for mode 2 resource allocation	NEC

R1-2008095	Remaining issues in NR sidelink mode 1 resource allocation	Spreadtrum Communications

R1-2008096	Remaining issues in NR sidelink mode 2 resource allocation	Spreadtrum Communications

R1-2008097	Remaining issues on sidelink physical layer procedure	Spreadtrum Communications

R1-2008129	Text Proposals on Physical Layer Structures for NR Sidelink	Samsung

R1-2008130	Draft CR on PUCCH Power Control for NR Sidelink Mode 1 Scheduling	Samsung

R1-2008131	Draft CR on Mode 2 for NR Sidelink	Samsung

R1-2008132	Draft CR on Sidelink Physical Duration to Logical Slot Conversion	Samsung

R1-2008133	Draft CR on Physical Layer Procedures for NR Sidelink	Samsung

R1-2008230	Draft TP on physical structure for NR sidelink	OPPO

R1-2008231	Text proposal of mode 1 for NR sidelink	OPPO

R1-2008232	Text proposal of physical layer procedure for NR sidelink	OPPO

R1-2008236	Remaining open issues and corrections for mode 2 RA	OPPO

R1-2008237	Corrections for FDM-based semi-static power split for in-device coexistence	OPPO

R1-2008334	Correction on sidelink timing definition	Huawei, HiSilicon

R1-2008381	Remaining issue on physical layer structure and procedure for sidelink in NR V2X	Panasonic Corporation

R1-2008387	Remaining issues on physical layer structure for NR sidelink	Sharp

R1-2008388	Remaining issues on resource allocation mode 1 for NR sidelink	Sharp

R1-2008389	Remaining issues on resource allocation mode 2 for NR sidelink	Sharp

R1-2008390	Remaining issues on synchronization mechanism for NR sidelink	Sharp

R1-2008391	Remaining issues on physical layer procedures for NR sidelink	Sharp

R1-2008428	Remaining Issues of Physical Layer Procedures	Apple

R1-2008429	Remaining Issue of Sidelink Physical Layer Structure	Apple

R1-2008430	Remaining Issues of Mode 1 Resource Allocation	Apple

R1-2008431	Remaining Issues of Mode 2 Resource Allocation	Apple

R1-2008496	Maintenance for PSFCH and PSCCH symbol on NR sidelink	ASUSTeK

R1-2008497	Remaining issues on sidelink power control	ASUSTeK

R1-2008498	Miscellaneous issues of SL HARQ-ACK reporting on PUCCH	ASUSTeK

R1-2008529	Maintenance for sidelink physical layer structure	NTT DOCOMO, INC.

R1-2008530	Maintenance for resource allocation mechanism mode 1	NTT DOCOMO, INC.

R1-2008531	Maintenance for resource allocation mechanism mode 2	NTT DOCOMO, INC.

R1-2008532	Maintenance for sidelink physical layer procedure	NTT DOCOMO, INC.

R1-2008533	Maintenance for sidelink-related collision	NTT DOCOMO, INC.

R1-2008604	Remaining Issues in Physical Layer Structure	Qualcomm Incorporated

R1-2008605	Remaining Issues in Mode 1 Resource Allocation	Qualcomm Incorporated

R1-2008606	Remaining Issues in Mode 2 Resource Allocation	Qualcomm Incorporated

R1-2008633	Remaining issues for Mode 2 resource allocation in NR V2X	ASUSTeK

R1-2008665	Remaining issues on physical layer structure for NR sidelink	vivo

R1-2008666	Remaining issues on mode 1 resource allocation mechanism	vivo

R1-2008667	Remaining issues on mode 2 resource allocation mechanism	vivo

R1-2008668	Remaining issues on sidelink synchronization mechanism	vivo

R1-2008669	Remaining issues on physical layer procedure for NR sidelink	vivo

R1-2008721	Remaining issues on physical layer procedures for sidelink	KT Corp.

R1-2008750	Discussion paper on the remaining issues in Rel. 16 for NR V2X	Ericsson

R1-2008751	Draft_CR_TS38.211	Ericsson

R1-2008752	Draft_CR_TS38.212	Ericsson

R1-2008753	Draft_CR_TS38.213	Ericsson

R1-2008754	Draft_CR_TS38.214	Ericsson

### Maintenance of Physical Layer Enhancements for NR URLLC (7)

Including maintenance for IIoT

For this agenda, no more than 4 contribution per 1 company/organization/university

R1-2007634	Corrections on PDCCH enhancements	Huawei, HiSilicon

R1-2007635	Corrections on scheduling and HARQ	Huawei, HiSilicon

R1-2007636	Corrections on SPS enhancements	Huawei, HiSilicon

R1-2007703	Maintenance of PDCCH for NR URLLC	Ericsson

R1-2007704	Maintenance of UCI for NR URLLC	Ericsson

R1-2007705	Maintenance of Scheduling/HARQ for NR URLLC	Ericsson

R1-2007706	Corrections for NR URLLC/IIoT	Ericsson

R1-2007732	Discussion on extension of M-TRP operation for Rel-15 cells in CA case 3	ZTE

R1-2007733	Discussion and text proposals on UL control enhancements for NR URLLC	ZTE

R1-2007734	Text proposals for intra-UE  multiplexing and PUSCH preparation procedure time	ZTE

R1-2007735	Text proposal for UE processing order between UL CI and power scaling calculation in UL CA			ZTE

R1-2007781	A remaining issue on the reference of spatial relation for a PUSCH scheduled by DCI format 0_0			Fujitsu

R1-2007782	A remaining issue on timing for applying SCell activation or deactivation	Fujitsu

R1-2007814	Remaining issues on PDCCH enhancements and inter-UE UL multiplexing	CATT

R1-2007815	Remaining issues on UCI enhancements	CATT

R1-2007816	Remaining issues on intra-UE prioritization	CATT

R1-2007817	Discussion on overlapping between CG PUSCH and DG PUSCH	CATT

R1-2007937	Draft TP on enhanced PDCCH monitoring for eURLLC	Intel Corporation

R1-2007988	Remaining issues on URLLC	ETRI

R1-2008056	PHR issues related to URLLC/IIOT WI	LG Electronics

R1-2008109	Remaining issues of enhancements to scheduling/HARQ	Spreadtrum Communications

R1-2008135	Maintanence on PDCCH as PDSCH SLIV reference	Samsung

R1-2008136	Draft CR on Type-2 HARQ-ACK codebook	Samsung

R1-2008137	Maintanence on SPS PDSCH	Samsung

R1-2008138	Draft CR on SPS release for PDSCH with aggregation	Samsung

R1-2008276	Text proposal for PUCCH carrying HARQ-ACK	OPPO

R1-2008277	Remaining issues on  scheduling and HARQ	OPPO

R1-2008278	Remaining issues on DL SPS enhancement	OPPO

R1-2008297	Draft CRs on PUCCH repetition handling with PHY priorities and Timing of SCell activation/deactivation for sub-slot PUCCH	Nokia, Nokia Shanghai Bell

R1-2008298	Support of Type-1 HARQ-ACK Codebook with PUCCH sub-slot configurations (incl. draft CR)			Nokia, Nokia Shanghai Bell

R1-2008303	Maintenance of Rel-16 URLLC intra-UE prioritization enhancements (incl. draft CRs)	Nokia, Nokia Shanghai Bell

R1-2008304	Out-of-order CBG based PUSCH re-transmission and PHR / power scaling with UL cancelation			Nokia, Nokia Shanghai Bell

R1-2008307	Inter-UE prioritization/multiplexing	InterDigital, Inc.

R1-2008392	Maintenance on UL PDCCH search space sharing for CA for NR URLLC	Sharp

R1-2008393	Remaining issue on subselection indication for DCI format 0_1/0_2 for NR URLLC	Sharp

R1-2008432	Remaining issues on UCI and PUSCH enhancements for eURLLC	Apple

R1-2008433	PHR and Power Scaling in case of Inter-UE Cancellation for eURLLC	Apple

R1-2008434	Maintenance of configured grant design in Physical Layer Enhancements for NR URLLC	Apple

R1-2008435	Maintenance of PDCCH design in Physical Layer Enhancements for NR URLLC	Apple

R1-2008486	Maintenance of PDCCH Enhancements for Rel-16 URLLC	Quectel

R1-2008534	Corrections on UCI enhancement for Rel-16 URLLC	NTT DOCOMO, INC.

R1-2008535	Corrections on scheduling/HARQ for Rel-16 URLLC	NTT DOCOMO, INC.

R1-2008562	Remaining issue on the HARQ-ACK/PUSCH priority	ITRI

R1-2008607	Remaining issue on HARQ and scheduling for URLLC	Qualcomm Incorporated

R1-2008608	Remaining issues on UCI enhancements for URLLC	Qualcomm Incorporated

R1-2008609	Remaining issues on PUSCH enhancements for URLLC	Qualcomm Incorporated

R1-2008634	Remaining issue for TCI field	ASUSTeK

R1-2008636	Remaining issues for DL SPS enhancement	ASUSTeK

R1-2008670	PDCCH enhancements for URLLC	vivo

R1-2008671	UCI enhancements for URLLC	vivo

R1-2008672	Enhancement for Scheduling/HARQ	vivo

R1-2008673	UL inter UE Tx prioritization for URLLC	vivo

R1-2008725	Corrections on HARQ-ACK codebooks for Rel-16 URLLC	WILUS Inc.

R1-2008772	Correction on sub-slot partition	Huawei, HiSilicon

[103-e-NR-L1enh-URLLC-01] Email discussion/approval on remaining issues on enhanced PDCCH monitoring capability – Chengyan (Huawei)

- Issue B-1: Time variation of “aligned” status for PDCCH spans across DL cells

- Issue B-2: Whether to apply M-TRP on the Rel-15 cells for case 3

- Discussion and decision by 10/29, TPs by 11/5

[103-e-NR-L1enh-URLLC-02] Email discussion/approval on remaining issues on DCI format design – Chengyan (Huawei)

- Issue A-1: Type2 HARQ-ACK codebook construction related to DAI bit width

- Issue A-2: Correction on missing case of PUSCH release for search space sharing

- issue A-3: Correction on Transmission configuration indication in DCI format 1_2

- Discussion and decision by 10/29, TPs by 11/5

Note: Issue A-2 and A-3 are simple correction which are easy to fix

[103-e-NR-L1enh-URLLC-03] Email discussion/approval on remaining issues on UCI enhancements – Jia (OPPO)

- Issue 2: Intra-UE prioritization for PUCCH repetition

- Issue 3: Type-1 codebook for sub-slot based HARQ-ACK

- issue 4: Timing for secondary cell activation / deactivation

- Discussion and decision by 10/29, TPs by 11/5

[103-e-NR-L1enh-URLLC-04]  Email discussion/approval on remaining issues on scheduling/HARQ enhancements – Kianoush (Qualcomm)

- Issue 4: Intra-UE prioritization and multiplexing order

- Issue 5: Order of multiplexing and prioritization due to conflicts with semi-static DL and SSB symbols

- Discussion and decision by 10/29, TPs by 11/5

[103-e-NR-L1enh-URLLC-05] Email discussion/approval on remaining issues on PUSCH enhancements – Sigen (Apple)

- Issue 2: Maximum data rate in a slot for PUSCH repetition Type B

- Discussion and decision by 10/29, TPs by 11/5

[103-e-NR-L1enh-URLLC-06] Email discussion/approval on remaining issues on inter-UE multiplexing enhancements – Xueming (vivo)

- Issue 1: Impact to PHR calculation due to UL CI in UL CA and/or UL skipping

- Issue 2: Impact to UE power scaling due to UL CI in UL CA and/or UL skipping

- Discussion and decision by 10/29, TPs by 11/5

[103-e-NR-L1enh-URLLC-07] Email discussion/approval on eCG enhancements – Lihui (vivo)

- Reply LS to R2-2008599 on Intra UE Prioritization

- Issue 6: PUSCHs overlapping with UCI piggyback

- Discussion and decision by 10/29, TPs by 11/5

Note: Email discussion is mainly for the reply LS R2-2008599, which is out of the email discussion budget of URLLC

### Maintenance of Enhancements on MIMO for NR (4)

For this agenda, no more than 3 contribution per company/organization/university

R1-2008140	Summary for Rel.16 NR eMIMO maintenance	Moderator (Samsung)

R1-2007748	Maintenance of multi-beam operation	ZTE

R1-2007749	Draft CR on UL full power transmission Mode 1	ZTE

R1-2007750	Maintenance of Multi-TRP enhancements	ZTE

R1-2007818	Discussion on remaining issues of multi-TRP/panel transmission	CATT

R1-2007819	Correction on PTRS for UL full power transmission	CATT

R1-2007820	Remaining Issues on multi-beam operation enhancement	CATT

Withdrawn

R1-2007909	Correction on L1-SINR Resource Setting	FUTUREWEI

R1-2007938	Corrections to multi TRP	Intel Corporation

R1-2008093	Discussion on remaining issues for multi-TRP operation	Spreadtrum Communications

R1-2008094	Discussion on remaining issues on full TX power for UL transmission	Spreadtrum Communications

R1-2008139	On maintenance of Rel.16 multi-beam operation	Samsung

R1-2008141	On Rel.16 multi-TRP/panel transmission	Samsung

R1-2008142	On UL full power transmission	Samsung

R1-2008211	Text proposals for enhancements on multi-TRP and panel Transmission	OPPO

R1-2008212	Correction for default TCI state of AP CSI-RS for M-TRP	OPPO

R1-2008213	Text Proposals for Multi-beam Operation Enhancement	OPPO

R1-2008293	Maintenance on multi-TRP operation	Lenovo, Motorola Mobility

R1-2008324	Correction on L1-SINR reporting	Huawei, HiSilicon

R1-2008325	Correction on the maximum number of CORESETs for Multi-DCI Transmission	Huawei, HiSilicon

R1-2008326	Correction on the index value range of CORESET for Multi-DCI Transmission	Huawei, HiSilicon

R1-2008345	Remaining issues on multi-beam operation	Sony

Withdrawn

R1-2008436	Remaining issues on Rel-16 Multi-TRP enhancement	Apple

R1-2008437	Remaining issues on Rel-16 Beam Management	Apple

R1-2008514	Remaining issues on multi-beam operation	MediaTek Inc.

R1-2008536	Updated proposal of PUCCH spatial relation after CBRA-BFR in Rel.16	NTT DOCOMO, INC, Nokia, Nokia Shanghai Bell, InterDigital

R1-2008569	Corrections on Multi-TRP PDCCH	Quectel

R1-2008570	Draft CR on multi-TRP/panel transmission	LG Electronics

R1-2008571	Draft CR on beam management	LG Electronics

R1-2008572	Text proposals on full Tx power UL transmission	LG Electronics

R1-2008610	Remaining Issues on Multi-TRP Enhancements	Qualcomm Incorporated

R1-2008611	Remaining issue on multi-beam operation	Qualcomm Incorporated

R1-2008635	Draft CR on TCI states for Aperiodic CSI-RS	Ericsson

R1-2008637	Draft CR on DL SPS based PDSCH repetitions	Ericsson

R1-2008638	Draft CR on TCI state codepoint mapping for DCI format 1_2	Ericsson

R1-2008640	Draft CR on QCL terminology alignment	Nokia, Nokia Shanghai Bell

R1-2008641	draft CR on higher layer parameter enabling L1-SINR operation procedures	Nokia, Nokia Shanghai Bell

R1-2008674	Remaining issues and corrections on multi beam related issues	vivo

R1-2008675	Corrections on multi TRP related issues	vivo

R1-2008676	Miscellaneous corrections on power control	vivo

R1-2008723	Maintenance of Rel-16 Multi-TRP operation	Nokia, Nokia Shanghai Bell

[103-e-NR-eMIMO-01] Maintenance and TPs for Multi-beam 1: MB.1 (plus E-rated MB.3, H2-rated MB.4) in R1-2008140 – Jiwon (LGE)

- Discussion and decision by 10/29, TPs by 11/5

[103-e-NR-eMIMO-02] Maintenance and TPs for Multi-beam 1: MB.10+MB.13+MB.14 (plus E-rated MB.12, H2-rated MB.8) in R1-2008140 – Yushu (Apple)

- As MB.13 has been discussed in several meetings before, if there is no consensus reached in RAN1#103-e, this issue will not be discussed anymore in future RAN1 meetings

- Discussion and decision by 10/29, TPs by 11/5

[103-e-NR-eMIMO-03] Maintenance and TPs for Multi-TRP: MT.2 (plus H2-rated MT.1) in R1-2008140 – Li (OPPO)

- Discussion and decision by 10/29, TPs by 11/5

[103-e-NR-eMIMO-04] Maintenance and TPs for UL Full-Power: UL.2 (plus H2-rated UL.4, editorial TP on PC) in R1-2008140 – Rakesh (vivo)

- Discussion and decision by 10/29, TPs by 11/5

### Maintenance of UE Power Saving for NR (1)

[103-e-NR-Rel-16-PowSav-01] Email discussion/approvl for potential CR(s)of issues #1/#2/#3/#5 as in FL’s summary by 11/2 – Fangchen (CATT)

Update on 11/2:

- The two 38.214 TPs (for issue #1 and issue #2) are endorsed  draft 38.214 CR, to check 11/4   draft CR is approved. Final CR in R1-2009589 (38.214, CR0143).

- The RRC parameter 38.213 TP is endorsed  to forward to 38.213 alignment CR.

Final summary in R1-2009588.

R1-2007821	Remaining issues on UE Power Saving	CATT

R1-2007970	Remaining issues of Rel-16 power saving	ZTE

R1-2008104	Remaining issues on UE power saving	Spreadtrum Communications

Late submission

R1-2008143	Remaining issues for Rel-16 UE power saving	Samsung

R1-2008331	Remaining issues for Rel-16 UE power saving	Huawei, HiSilicon

R1-2008509	Remaining issues on UE power saving	MediaTek Inc.

R1-2008565	Maintenance for UE power saving	Ericsson

R1-2008677	Remaining issues on UE power saving	vivo

R1-2008732	On open issues related to Rel-16 UE power saving	Nokia, Nokia Shanghai Bell

### Maintenance of NR positioning support (2)

For this agenda, no more than 2 contribution per company/organization/university

R1-2007574	Rel-16 positioning corrections	Huawei, HiSilicon

R1-2007751	Draft CR on measurement gap configuration for DL PRS reception	ZTE

R1-2007752	Draft CR on the definition of nr-DL-PRS-expectedRSTD-r16	ZTE

R1-2007822	Discussion on configuration parameters related to SRS-Pos	CATT

R1-2007823	Discussion on linear value of SRS power split by UE	CATT

R1-2007999	Remaining issues on DL PRS	CMCC

R1-2008214	Text Proposals on NR Positioning Procedure	OPPO

R1-2008215	Text Proposals on RS for Positioning	OPPO

R1-2008414	Discussions on remaining issues on Rel-16 NR positioning	LG Electronics

R1-2008580	Editorial Corrections on Rel-16 NR positioning	LG Electronics

R1-2008678	Remaining issues on prioritization of positioning assistance data	vivo

R1-2008679	Remaining issues on TRP ID for NR positioning	vivo

R1-2008760	Corrections to 38.211 for NR positioning	Ericsson

R1-2008761	Corrections to 38.214 for NR positioning	Ericsson

R1-2008789	Correction to PRS duration calculation for PRS processing	Huawei, HiSilicon

[103-e-NR-Pos-01] Email discussion/approval on DL PRS on aspects 2, 3, 13 (to capture RAN1 agreement only), 14, 16, in the FL summary until 10/29 with potential CRs by 11/4– Alexey (Intel)

Text proposal #1 in Section 3 of R1-2009434 for Clause 5.1.6.5 in TS38.214 is endorsed in R1-2009693 (TS38.214, Rel-16, CR0147)

Text proposal #3 in Section 3 of R1-2009434 for Clause 7.4.1.7.4 in TS38.211 is endorsed in R1-2009692 (TS38.211, Rel-16, CR0062)

R1-2009694 (Rel-16, TS38.214, CR0148)	CR on DL PRS resource prioritization for UE measurements	Moderator (Intel Corporation), LGE

[103-e-NR-Pos-02] Email discussion/approval on UL SRS and procedures on aspects 6, 8, 11, 12, 15, 17 in the FL summary until 10/29 with potential CRs by 11/5 – Florent (Ericsson)

R1-2009738	CR on the configuration of spatial relation for the SRS for positioning	Moderator (Ericsson), CATT

R1-2009739	CR for replacement of cell terminology in PRS reception procedure	Moderator (Ericsson), OPPO, Nokia, Nokia Shanghai Bell

R1-2009740 (Withdrawn)	CR for correction on the QCL description between PRS and SSB	Moderator (Ericsson), OPPO, Huawei, HiSilicon

R1-2009741	CR for parameter name alignment and reference corrections in PRS reception procedure	Moderator (Ericsson), OPPO, LG Electronics

### Maintenance of NR Mobility Enhancements (2)

R1-2007593	Remaining issues on DAPS	Huawei, HiSilicon

R1-2007738	Draft CR on intra-frequency DAPS handover	ZTE

R1-2008144	Draft CR on clarification of processing capability on DAPS HO dropping timeline	Samsung

R1-2008209	Correction to DAPS HO	Ericsson

R1-2008502	Remaining issues on per CC UE capability and UL cancellation for DAPS-HO	MediaTek Inc.

R1-2008733	Remaining physical layer aspects of dual active protocol stack based HO	Nokia, Nokia Shanghai Bell

[103-e-NR-Mob-Enh-01] Email discussion/approval on the following until 10/29 with potential CRs by 11/4 – Daewon (Intel)

- Issue #2 in R1-2008871, clarification on intra-frequency DAPS definition

- Issue #3 in R1-2008871, issue on processing capability correction for Tx cancellation

R1-2009541	[103-e-NR-Mob-Enh-01] Discussions Summary #2	Moderator (Intel Corporation)

TP#2-3 in R1-2009353 and the associated CR in R1-2009527 (TS38.213, Rel-16, CR151 Rev1) are endorsed.

TP#3-3 in R1-2009353 and the associated CR in R1-2009526 (TS38.213, Rel-16, CR150 Rev1) are endorsed.

TP#3-2 in R1-2009353 and the associated CR in R1-2009525 (TS38.213, Rel-16, CR149 Rev1) are endorsed.

[103-e-NR-Mob-Enh-02] Email discussion/approval on the following until 10/29 – Daewon (Intel)

- Issue#5 in R1-2008871, issue on handling of SUL and DAPS operation

Agreement:

- In Rel-16, UE is not required to support both NUL and SUL in source cell during DAPS handover

- In Rel-16, UE is not required to support both NUL and SUL in target cell during DAPS handover

- Up to RAN2 for the solution to handle above cases

- Send LS to RAN2 to take this into consideration

R1-2009681	[Draft] LS on support of NUL and SUL during DAPS handover	Intel Corporation

Final LS approved in R1-2009682 with following modifications to draft LS in R1-2009681:

Change “RAN1 would like to kindly ask RAN2 to consider the above in their further work and provide solutions to handle not requiring supporting NUL and SUL in either source or target cell during DAPS handover” to “RAN1 would respectfully ask RAN2 to take into account the agreement made in RAN1 in their further work.”

### Maintenance of Multi-RAT Dual-Connectivity and Carrier Aggregation enhancements (LTE, NR) (2)

For this agenda, no more than 2 contribution per company/organization/university

R1-2007578	Remaining issues on CA	Huawei, HiSilicon

R1-2007736	Remaining Issues of Power Control for NR-DC and Cross-Carrier Scheduling	ZTE

R1-2007737	Remaining Issues of SCell Dormancy, Single Tx and Unaligned Frame Boundary	ZTE

R1-2007806	Remaining issues on inter-band CA with unaligned frame boundary	CATT

R1-2007807	Corrections on HARQ-ACK codebook for secondary PUCCH group	CATT

R1-2008113	Correction to dormancy indication outside Active Time	NEC

R1-2008145	Remaining issues for Scell dormancy	Samsung

R1-2008203	Remaining issues on dormancy	Nokia, Nokia Shanghai Bell

R1-2008275	BWP switching delay issues and Scell dormancy	OPPO

R1-2008503	Remaining issues on Rel-16 uplink power control for supporting NR-NR dual-connectivity			MediaTek Inc.

R1-2008504	Remaining issues on Rel-16 carrier aggregation	MediaTek Inc.

R1-2008566	Draft CR to 38.213 on corrections for SCell dormancy	Ericsson

R1-2008567	Maintenance for other MR-DC topics	Ericsson

R1-2008612	Remaining issue on NR-DC power-control	Qualcomm Incorporated

R1-2008613	Remaining issue on SCell dormancy	Qualcomm Incorporated

R1-2008680	Remaining issues on MR-DC and CA enhancements	vivo

R1-2008694	Discussion for Rel-16 DC uplink power control	Nokia, Nokia Shanghai Bell

R1-2008719	Interoperation between cross-carrier scheduling and multiple TRPs	ASUSTeK

[103-e-NR-MRDC-CA-01] Email discussion/approval on Dormancy and Unaligned CA on the following until 10/29 with potential CRs by 11/5 – Ravi (Ericsson)

- Dormancy Topic 1: Corrections to 38.213 related to SCell dormancy indication in DCI format 2_6

- Dormancy Topic 3: Corrections to 38.213 related to Case 2 dormancy indication

- Dormancy Topic 4: Corrections to 38.213 related to switching time during dormant/non-dormant BWP transition

- Dormancy Topic 5: Corrections to 38.212

- Unaligned CA Issue 1: whether the single offset duration limitation is applied to per CG or across CGs in case of NR-DC

- Unaligned CA Issue 2: Determination of Sync/Async NR-DC in the context of unaligned frame boundary CA

- Unaligned CA Issue 3: update of spec. TS 38.214 section 5.2.1.5.1 on aperiodic CSI-RS timing when the triggering PDCCH and the CSI-RS have the same numerology, to align with the agreement.

Conclusion:

In description of SCell dormancy indication in 38.212, “…MSB to LSB of the bitmap corresponding to the first to last configured SCell group...” implies that the MSB to LSB of the bitmap correspond to the first to last configured SCell group in ascending order of DormancyGroupID.

R1-2009755 (Rel-16, TS38.213, CR0177)	Corrections for SCell dormancy indication	Moderator (Ericsson), ZTE

R1-2009756 (Rel-16, TS38.214, CR0153)	Corrections for A-CSI triggering with unaligned CA	Moderator (Ericsson)

[103-e-NR-MRDC-CA-02] Email discussion/approval on PC for DC, X-CC scheduling and 1-Tx enhancements until 10/29 with potential CRs by 11/5 – Karri (Nokia)

- PC-DC Issue 2: (R1-2007736 and CR2 for power control) Clarification on the timing relation for Dynamic Power Sharing

- PC-DC Issue 3: (R1-2008503, P1 and TP) On FDD scenario for Semi-Static-mode2 power control

- PC-DC Issue 4: (R1-2008694, TP) Alignment between UE capability description on TS38.306 and UE’s behaviour on TS38.213.

- XCC A-1 (R1-2007736): Discuss if there is a need to add the PDSCH starting time to determine the last DCI in order to be able to indicate different PRIs in the same PUCCH slot for the two HARQ-ACKs in the scenario described in R1-2007736.

- XCC A-2 (R1-2007807): The clarification on the RRC parameter applicability between URLLC priority based codebook and secondary PUCCH group codebook would seem to benefit from the suggested clarification. Discuss the TP1 and TP2 to TS38.213 sections 7.2 and 9 respectively.

- XCC A-3 (R1-2007807): Interoperability of the simultaneous configuration of pdsch-HARQ-ACK-CodebookList-r16 and pdsch-HARQ-ACK-Codebook-secondaryPUCCHgroup-r16 would seem to be in a need of clarification. Discuss how to resolve the interoperability issue.

- XCC A-4 (R1-2008504): Discuss the need for introducing the additional delay ‘d’ for timeDurationForQCL in case of CCS when enableDefaultBeamForCCS is not configured as proposed in section 3 of R1-2008504 to 38.214 subclause 5.1.5

- 1TX C-1 (R1-2007737): There appears to be a discrepancy between the UE capabilities and the TS38.213 for the TDD and FDD PCell semi-static UL transmission in all subframes and a correction is needed. Discuss section 2.2 and CR4 in R1-2007737.

R1-2009630	Summary of Email discussion [103-e-NR-MRDC-CA-02]	Moderator (Nokia)

R1-2009631	LS on HARQ-ACK codebook configuration for secondary PUCCH group	RAN1, Nokia

R1-2009632	38.213 CR for NR-DC power control	Moderator (Nokia), MediaTek, Nokia, Nokia Shanghai Bell, ZTE

R1-2009633	38.213 CR Correction on HARQ-ACK codebook for secondary PUCCH group	Moderator (Nokia), CATT

R1-2009634	36.213 CR on Single UL Tx for EN-DC	Moderator (Nokia), ZTE

### NR Rel-16 UE Features (9)

R1-2007913	Summary of UE features for IAB	Moderator (AT&T)

R1-2007914	Summary of UE features for 5G V2X	Moderator (AT&T)

R1-2007915	Summary of UE features for eMIMO	Moderator (AT&T)

R1-2007916	Summary of UE features for UE power savings	Moderator (AT&T)

R1-2007917	Summary of UE features for NR mobility enhancements	Moderator (AT&T)

R1-2008538	Summary on UE features for two-step RACH	Moderator (NTT DOCOMO, INC.)

R1-2008539	Summary on UE features for NR-U	Moderator (NTT DOCOMO, INC.)

R1-2008540	Summary on UE features for URLLC/IIoT	Moderator (NTT DOCOMO, INC.)

R1-2008541	Summary on UE features for NR positioning	Moderator (NTT DOCOMO, INC.)

R1-2008542	Summary on UE features for MR-DC/CA	Moderator (NTT DOCOMO, INC.)

R1-2008543	Summary on UE features for CLI/RIM	Moderator (NTT DOCOMO, INC.)

R1-2008544	Summary on UE features for TEIs	Moderator (NTT DOCOMO, INC.)

R1-2008545	Summary on NR UE features for others	Moderator (NTT DOCOMO, INC.)

R1-2009346	Summary of email discussion/approval [103-e-NR-UEFeatures-V2X-01]	Moderator (AT&T)

R1-2009347	Summary of email discussion/approval [103-e-NR-UEFeatures-MobEnh-01]	Moderator (AT&T)

R1-2009348	Summary of email discussion/approval [103-e-NR-UEFeatures-eMIMO-01]	Moderator (AT&T)

R1-2009349	Session Notes of AI 7.2.11	Ad-Hoc Chair (AT&T)

[103-e-NR-UEFeatures-LS] Email approval of updated Rel-16 NR UE features by 11/6 – Hiroki (DCM)

Update on 11/5: check on 11/8

Update on 11/9: LS & updated feature list are approved in R1-2009586 and R1-2009585

[103-e-NR-UEFeatures-eMIMO-01] Email discussion/approval for remaining issues on UE features for NR MIMO enhancements, till 10/30 11/2 – Ralf (AT&T)

- FG 16-1a-1: resolve FFS

- FG 16-1g: resolve FFS

- FG 16-2a: how the network will interpret the signaled maximum number of CORESETs in components (1) and (2) (i.e., candidate value 5 for component (1) and candidate value 3 for component (2)) of FG 16-2a, e.g., when CORESET #0 is not configured

- FG 16-5c-2: resolve the FFS

- Ambiguity issue in case of cross-carrier operation (RP-201768, R1-2007739, R1-2008639)

- FG 16-2a-3: whether to update the note (R1-2008639)

- FGs 16-2b-3 and 16-2b-4: whether to update the note (R1-2008146, R1-2008614)

- FG 16-3a-4: whether to update the component description and consequence if feature is not supported (R1-2008737)

- FG 16-8: whether to change the note (R1-2008614)

Agreement:

| 16-1a-1 | SSB/CSI-RS for L1-SINR measurement | Per slot limitations: - The max number of SSB/CSI-RS (1Tx) for CMR - The max number of CSI-IM/NZP-IMR resources - The max number of CSI-RS (2Tx) resources for CMR Memory limitations: - The max number of SSB/CSI-RS resources as CMR - The max number of CSI-IM/NZP IMR resources Other limitations: - Supported density of CSI-RS (CMR) - The max number of aperiodic CSI-RS resources across all CCs configured to measure L1-SINR (including CMR and IMR) shall not exceed MD_1 - Supported SINR measurements | 2-21, 2-22 or 2-23, 2-23a | Yes | N/A |  | Per band | No | No |  | Component 1: Candidate values {8, 16, 32, 64} Component 2: Candidate values {8, 16, 32, 64} Component 3: Candidate values {0, 4, 8, 16, 32, 64} Component 4: Candidate values {8, 16, 32, 64 , 128} Component 5: Candidate values {8, 16, 32, 64 , 128} Component 6: Candidate values {‘1 only’, ‘3 only’, ‘1 and 3’} Component 7: Candidate values {2, 4, 8, 16, 32, 64} Component 8: Candidate values: bitmap with entries {SSB as CMR with dedicated CSI-IM, SSB as CMR with dedicated NZP IMR, CSI-RS as CMR with dedicated NZP IMR configured, CSI-RS as CMR without dedicated IMR configured} If a UE supports FG 16-1a-1 it must support CMR(CSI-RS) + dedicated CSI-IM FFS: How CSI-RS is counted when it is configured as CMR without dedicated IMR Note1: The reference slot duration is the shortest slot duration defined for the FR where the reported band belongs Note2: For component 4 and 5 the configured CSI-RS resources for both active and inactive BWPs are counted Note3: For components 1, 2 and 3, CSI-RS resources configured as CMR without dedicated IMR are counted both as CMR and IMR | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Agreement:

| 16-1g | Resources for beam management, pathloss measurement, BFD, RLM and new beam identification | - The maximum total number of SSB/CSI-RS/CSI-IM resources configured to measure within a slot across all CCs in one frequency range for any of L1-RSRP measurement, L1-SINR measurement, pathloss measurement, BFD, RLM and new beam identification - The maximum total number of SSB/CSI-RS/CSI-IM resources configured across all CCs in one frequency range for any of L1-RSRP measurement, L1-SINR measurement, pathloss measurement, BFD, RLM and new beam identification | 2-24, 2-31 | Yes | N/A |  | Per UE | No | Yes |  | Component-1: candidate value set is {2, 4, 8, 12, 16, 32, 64, 128} Component-2: candidate value set is {2, 4, 8, 12, 16, 32, 40, 48, 64, 72, 80, 96, 128, 256} FFS: how to count the RS for component (1) and (2) Note: For RS configured for new beam identification, they are always counted regardless of beam failure event Note: The “configure to measure” RS (component1) only counts those in active BWP but the configured RS (component2) counts all configured including both active and inactive BWP Note: the reference slot duration is the shortest slot duration defined for the reported FR supported by the UE | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Conclusion:

- For FGs 16-1a-1 and 16-1g, continue discussion on whether/how Section 5.2.1.6 in 38.214 applies

Agreement:

| 16-2a | Multi-DCI based multi-TRP | - The maximum number of CORESETs configured per BWP per cell in addition to CORESET 0 - The maximum number of CORESETs configured per CORESETPoolIndex (if CORESETPoolIndex is not configured, it is assumed CORESETPoolIndex = 0) per BWP per cell in addition to CORESET 0 - Support fully/partially overlapping PDSCHs in time and non-overlapping in frequency - Maximum number of unicast PDSCHs per CORESETPoolIndex per slot |  | Yes | N/A |  | Per FSPC | No | No |  | Note: A UE may assume that its maximum receive timing difference between the DL transmissions from two TRPs is within a CP Note: Processing capability 2 is not supported in any CC if at least one CC is configured with two values of CORESETPoolIndex Component 1: Candidate values {2,3,4,5} Note: 1. If UE reports value N1 for component 1, that means UE supports up to min (N1+1, 5) CORESETs in total (including CORESET#0) if there is CORESET#0, and supports maximal N1 CORESETs if there is no CORESET#0. Component 2: Candidate values {1,2,3} Note: If UE reports value N2 for component 2, that means UE supports up to min (N2+1, 3) CORESETs in total (including CORESET#0) for a TRP if there is CORESET#0, and supports maximal N2 CORESETs for another TRP if there is no CORESET#0. Component 4: Candidate values {1,2,3,4,7} Note: per SCS, similar with Rel-15 | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Agreement:

| 16-5c-2 | UL full power transmission fullpowerMode2 – SRS resources | The SRS configuration with different number of antenna ports per SRS resource for Mode 2 | 16-5c | Yes | N/A |  | Per FS | No | No |  | Component (1) candidate values: {1_2, 1_4, 1_2_4} 1st state (1_2): each SRS resource can be configured with 1 port or 2 ports 2nd state (1_4): each SRS resource can be configured with 1 port or 4 ports 3rd state (1_2_4): each SRS resource can be configured with 1 port or 2 ports or 4 ports FFS: Note: The max number of SRS resources with different ports is the same as the number of SRS resources Note: The first, second, or third state can be used if 16-5c is reported as 2 or 4. | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Conclusion:

- The UE does not expect to receive more than 1 TB if 1 TB of PDSCH is scheduled with FDMSchemeB or TDMSchemeA in a slot in a CC

Agreement:

| 16-3a-4 | CBSR | 1) CBSR with soft amplitude subset restriction | 16-3a | Yes | N/A | Only CBSR with hard amplitude subset restriction is not supported | Per Band | N/A | N/A |  |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Agreement:

| 16-8 | Active CSI-RS resources and ports for mixed codebook types in any slot | - Report a list of codebook combinations as {codebook 1, codebook 2, codebook 3} - For each codebook combination, report a list of {max number of ports per resource, max number of resources, max number of total ports} | 2-36/2-40/2-41/2-43 in Rel-15, and 16-3a, 16-3a-1, 16-3b, 16-3b-1 in Rel-16 | Yes | N/A |  | per band and per BC | N/A | N/A |  | Component-1 candidate values: Codebook 1 = {Type I SP, Type I MP} (Codebook 2, Codebook 3) = {(Type II, NULL), (Type II PS, NULL), (eType II R=1, NULL), (eType II R=2, NULL), (eType II PS R=1, NULL), (eType II PS R=2, NULL), (Type II, Type II PS)} Note 3：if a UE reports one or more codebook combinations in 16-8, then usage of active CSI-RS resources and ports for multiple codebooks in any slot is allowed only within those combinations Note 4: For coexisting of mixed codebooks in any slot, gNB need to honor 16-8 and per-codebook capability 2-36/40/41/43, 16-3a/b and 16-3a-1/16-3b-1 Note 5: Up to 4 combinations for component 1 Component-2 candidate values: - Maximum 16 triplets for each codebook combination - Max # of Tx ports in one resource: {4,8,12,16,24,32} - Max # resources: {1 to 64} - Max # total ports: {4 to 256} | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Email thread is closed.

[103-e-NR-UEFeatures-V2X-01] Email discussion/approval for remaining issues on UE features for NR V2X, till 10/30 11/2 – Ralf (AT&T)

- Finalize mode 1 scheduling when the NR Uu carrier is different from the SL carrier (cross-scheduling)

- Reply LS on UE capability for V2X (OPPO, R1-2007520)

- Whether to change FG 15-16 from per band combination to per feature set (R1-2008614)

Conclusions:

- Reply to RAN2 that cross-carrier configuration by NR Uu is not mandated by 3GPP specifications to be supported by a UE in NR sidelink for a band indicated with only the PC5 interface in 38.101-1 Table 5.2E.1-1

Draft LS is approved. Final LS in R1-2009635.

Agreement:

- Introduce a new feature group and update Component 4 in FG 15-2 to specify that it only applies to same-carrier scheduling

| 15-2 | Transmitting NR sidelink mode 1 scheduled by NR Uu | 1) UE can transmit PSCCH/PSSCH using dynamic scheduling or configured grant type 1 and 2 in NR sidelink mode 1 scheduled by NR Uu. Up to 8 configured grants can be configured for a UE. Up to C sidelink HARQ processes are supported including those for configured grants 2) UE can transmit PSSCH according to the normal 64QAM MCS OFDM table. 3) UE supports PT-RS transmission in FR2. 4) UE can monitor DCI format 3_0 for NR sidelink dynamic scheduling and configured grant type 2 on the same carrier as sidelink. 6) UE can transmit using the subcarrier spacing and CP length it reports. 8) Supports 14-symbol SL slot with all DMRS patterns corresponding to {#PSSCH symbols} = {12, 9} for slots w/wo PSFCH. If UE signals support of ECP, support 12-symbol SL slot with all DMRS patterns corresponding to {#PSSCH symbols} = {10,7} for slots w/wo PSFCH. 9) Support downlink pathloss based open loop power control 11) UE can report sidelink HARQ-ACK to gNB via PUCCH and PUSCH when it is operating in NR sidelink mode 1 |  | Yes | No |  | Per band | N.A. | N.A. | N.A. | Note: Random selection in the exceptional pool is supported. This is the basic FG for sidelink in licensed spectrum where gNB is operating on or managing that spectrum and optional FG otherwise Candidate values for C are {8,16} Component-6 candidate value set in FR1: {{15 kHz}, {30 kHz}, {60 kHz}, {15, 30 kHz}, {30, 60 kHz}, {15, 60 kHz}, {15, 30, 60 kHz}} Component-6 candidate value set in FR2: {{60 kHz}, {120 kHz}, {60, 120 kHz}} Component-6 candidate value set for CP length: {NCP,NCP and ECP} (ECP only applies to SCS of 60 kHz) Note: For Component 6, if a band is not indicated with only the PC5 interface in 38.101-1 Table 5.2E.1-1, the reported numerology shall be the same for sidelink and uplink. Component (9) is only required to be supported in a band not indicated with only the PC5 interface in 38.101-1 Table 5.2E.1-1 Note: Component 11 is not required to be supported in a band indicated with the PC5 interface in 38.101-1 Table 5.2E.1-1 In a band indicated with only the PC5 interface in 38.101-1 Table 5.2E.1-1, the UE supports at least 30 kHz with normal CP in FR1, and at least 120 kHz with normal CP in FR2 | Optional with capability signalling For UE supports NR sidelink in licensed spectrum where gNB is defined, UE must indicate this FG is supported. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 15-25 | Transmitting NR sidelink mode 1 scheduled by NR Uu on a different carrier | 1) UE can monitor DCI format 3_0 on a different carrier from sidelink for NR sidelink dynamic scheduling and configured grant type 2 | FG 15-2 | Yes | No |  | [Per FS] | N.A. | N.A. | N.A. | If the UE indicates support for FG 15-2 in a band indicated with only the PC5 interface in Table 5.2E.1-1 of 38.301-1, the UE must indicate that FG 15-25 is supported for a band combination with that band. | [Optional with capability signalling] |

Update on 11/3

Agreements:

The following changes are adopted

| 15-16 | Simultaneous transmission of uplink and sidelink | 1) UE supports simultaneous transmission of NR uplink and NR sidelink (in different bands) in a band combination for which the UE indicated simultaneous sidelink and uplink support in a band combination. | At least one of 15-2 and 15-3 | Yes | No |  | Perband combination FS | N.A. | N.A. | N.A. |  | Optional with capability signalling. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Agreements:

The following changes are adopted

| 15-25 | Transmitting NR sidelink mode 1 scheduled by NR Uu on a different carrier | 1) UE can monitor DCI format 3_0 on a different carrier from sidelink for NR sidelink dynamic scheduling and configured grant type 2 | FG 15-2 | Yes | No |  | [Per FS] | N.A. | N.A. | N.A. | If the UE indicates support for FG 15-2 in a band indicated with only the PC5 interface in Table 5.2E.1-1 of 38.301-1, the UE must indicate that FG 15-25 is supported for a band combination with that band. | [Optional with capability signalling] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

LSs? 11/4

Update on 11/5:

Agreements:

- The following changes are adopted

| 15-25 | Transmitting NR sidelink mode 1 scheduled by NR Uu on a different carrier | 1) UE can monitor DCI format 3_0 on a different carrier from sidelink for NR sidelink dynamic scheduling and configured grant type 2 | FG 15-2 | Yes | No |  | [Per FS] | N.A. | N.A. | N.A. | If the UE indicates support for FG 15-2 in a band indicated with only the PC5 interface in Table 5.2E.1-1 of 38.301-1, the UE must indicate that FG 15-25 is supported for a band combination with that band. | [Optional with capability signalling] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Another LS to RAN2, 11/11  draft LS is approved. Final LS in R1-2009643.

[103-e-NR-UEFeatures-MobEnh-01] Email discussion/approval for remaining issues on UE features for NR mobility enhancements, till 10/30 11/2 – Ralf (AT&T)

- Issue #4 in R1-2008871 on capability split between source and target cell for DAPS

Update on 10/29:

Conclusion:

- Close email discussion/approval [103-e-NR-UEFeatures-MobEnh-01] without new agreements

[103-e-NR-UEFeature-NRU-01] Email discussion/approval on UE features for NR-U (26th Oct – 3rd Nov) – Hiroki (DCM)

- Whether each of FGs10-9/9b/9c/9d/15/16/20a is applicable to licensed bands or not

- Whether/how to define basic FG(s) for each of particular NR-U deployment scenarios

- How to handle FG10-19a/b/c/d/e/f in RAN1 UE features list and how to reply to RAN4 LS

Update on 11/5 – all issues resolved using GTW.

Agreements:

- The FG10-15/16 are also applicable to licensed bands

- The FG10-20a is also applicable to licensed bands

- Note: this agreement should not cause any specification impact

Agreements:

- The FG10-9/9b/9c/9d are only applicable to unlicensed bands, and the note “the signaling is per band but is only expected for a band where shared spectrum channel access must be used” is added for the FGs.

Agreements:

- Following classification of scenarios is used to define basic FGs for NR-U according to TS38.300 B.3

  - Scenario A: Carrier aggregation between NR in licensed spectrum (PCell) and NR in shared spectrum (SCell);

    - Scenario A.1: SCell is not configured with uplink (DL only).

    - Scenario A.2: SCell is configured with uplink (DL+UL).

  - Scenario B: Dual connectivity between LTE in licensed spectrum and NR in shared spectrum (PSCell);

  - Scenario C: NR in shared spectrum (PCell);

  - Scenario D: NR cell in shared spectrum and uplink in licensed spectrum;

  - Scenario E: Dual connectivity between NR in licensed spectrum (PCell) and NR in shared spectrum (PSCell).

- Ask RAN2 to consider following TP for TS38.300 B.3

TS38.300, Annex B (informative): Deployment Scenarios B.3 NR Operation with Shared Spectrum NR Radio Access operating with shared spectrum channel access can support the following deployment scenarios: - Scenario A: Carrier aggregation between NR in licensed spectrum (PCell) and NR in shared spectrum (SCell); - Scenario A.1: SCell is not configured with uplink (DL only). - Scenario A.2: SCell is configured with uplink (DL+UL). - Scenario B: Dual connectivity between LTE in licensed spectrum and NR in shared spectrum (PSCell); - Scenario C: NR in shared spectrum (PCell); - Scenario D: NR cell in shared spectrum and uplink in licensed spectrum; - Scenario E: Dual connectivity between NR in licensed spectrum (PCell) and NR in shared spectrum (PSCell). Carrier aggregation of cells in shared spectrum is applicable to all deployment scenarios.

- Following FGs are defined as basic FGs for corresponding scenario(s) for NR-U, and associated scenario(s) for the basic FG is clarified in the note column of UE features list

  - 10-1: A2, B, C, D and E with dynamic channel access mode

  - 10-1a: A2, B, C, D and E with semi-static channel access mode

  - 10-2: A1, A2, B, C, D and E with dynamic channel access mode

  - 10-2a: A1, A2, B, C, D and E with semi-static channel access mode

  - 10-2b: B, C, D and E

  - 10-2c: B, C, D and E with dynamic channel access mode

  - 10-2d: B, C, D and E with semi-static channel access mode

  - 10-2e: C and D

- The note "This FG may be part of basic operation for a particular scenario" is removed from following FGs.

  - 10-2f, 10-3, 10-3a, 10-27, 10-29, 10-30, 10-31

- Note: There will be no more discussion on whether/how to capture the classification of scenarios in TR/TS

Agreements:

- FGs [10-19a/b/c/d/e/f] numbers are kept, and all other contents are removed from RAN1 UE features list

- Reply LS to RAN4 with following contents – agreed in R1-2009385

  - Clarify RAN1’s original intention on FG[10-19a/b/c/d/e/f] with copying current FG rows and also clarify these are just examples

    - Clarify that one of the intentions is whether or not separate UE capabilities associated with each of DL/UL modes/cases are necessary

  - Ask RAN4 to discuss and decide necessary FG(s) and clarify that RAN1 will not discuss further on these FGs that are removed from RAN1 UE features list

[103-e-NR-UEFeature-URLLCIIoT-01] Email discussion/approval on UE features for URLLC/IIoT (26th Oct – 3rd Nov) – Hiroki (DCM)

- Whether or not to confirm Working assumption on new FG for Rel-16 PDCCH monitoring capability with restriction that the same span pattern repeats in every slot for a given CC

- How to define a new FG for Out-of-order CBG-based re-transmission(s) with cancelled initial PUSCH transmission according to conclusion made at RAN#89-e

- Clarify interpretation of FGs in case of cross-carrier operation e.g., for FG 11-7a/7b/9/9a and 12-2a

- Whether or not to add components for the restriction on the number of different start symbol indices of PDCCH monitoring occasions per slot/half-slot in FG11-2

- Whether or not to add component for the supported maximum number of actual PUCCH transmissions for HARQ-ACK within a slot in FG11-3

- Whether or not to remove the note regarding relationship between FG12-1 and the feature of up to two HARQ-ACK codebooks of 11-4 and 11-4x

Extend to 11/10

Work assumption:

- The replicated FGs of 11-2a/c[d/e] with restriction for non-aligned span case are added to RAN1 UE features list

  - Component 2 of new FGs is below

    - UE supports aligned span and non-aligned span

      - In case of non-aligned span when the configured number of cells with Rel-16 PDCCH monitoring is larger than the UE reported value, PDCCH monitoring occasion(s) should be configured only on same symbol(s) every slot

Agreements:

- When UE reports 5-25, the UE supports both in-order and out-of-order CBG-based retransmission(s) (not requiring 11-12 as prerequisite even for Rel-16 UE)

- For Rel-16, new FG for UE supporting only in-order CBG-based retransmission(s) (not requiring 5-25 as prerequisite) is introduced

Agreements:

| 11. NR_L1enh_URLLC | 11-12 | CBG-based re-transmission for UL using CBGTI with only in-order CBG-based re-transmission(s) for cancelled initial PUSCH transmission | 1. Support of CBG-based PUSCH re-transmission(s) of a TB using CGBTI in case the initial PUSCH transmission was not cancelled due to gNB scheduling/indication/configuration. 2. Support of CBG-based PUSCH re-transmission(s) of a TB using CGBTI in case the initial PUSCH transmission was cancelled due to gNB scheduling/indication/configuration and the following condition is satisfied: the UE is scheduled for a re-transmission of a CBG #N in a given TB when CBG #N-1 has been transmitted before or is scheduled in the same UL grant that includes CBG#N. |  | Yes | N/A |  | Per UE | No | No |  |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Agreements:

- Regarding the interpretation of UE capabilities in case of cross-carrier operation, RAN1 clarifies that support of the following UE capability is based on the support of this capability for both the band of the scheduled/triggered/indicated cell and the band of the scheduling/triggering/indicating cell.

  - FG11-7a

Conclusion:

- UE may not do the CCE/BD counting for the purpose of dropping in spans except the first one within a slot for Rel-16 PDCCH monitoring capability on the primary cell, as defined by the following two paragraphs in TS 38.213.

A PDCCH candidate with index for a search space set using a set of CCEs in a CORESET on the active DL BWP for serving cell is not counted for monitoring if there is a PDCCH candidate with index for a search space set , or if there is a PDCCH candidate with index and , in the CORESET on the active DL BWP for serving cell using a same set of CCEs, the PDCCH candidates have identical scrambling, and the corresponding DCI formats for the PDCCH candidates have a same size; otherwise, the PDCCH candidate with index is counted for monitoring. … CCEs for PDCCH candidates are non-overlapped if they correspond to - different CORESET indexes, or - different first symbols for the reception of the respective PDCCH candidates.

[103-e-NR-UEFeature-MRDCCA-01] Email discussion/approval on UE features for MR-DC/CA (26th Oct – 3rd Nov) – Hiroki (DCM)

- Clarify interpretation of FGs in case of cross-carrier operation e.g., for FG18-5c/5d

- Whether or not to add “one of {6-2, 6-3}” as prerequisite FGs for FG18-4

- How to reply to RAN2 LS regarding cell grouping capability signaling for NR-DC

Update on 11/5 – all issues resolved using GTW.

Agreements:

- Regarding the interpretation of UE capabilities in case of cross-carrier operation, RAN1 clarifies that support of the following UE capability is based on the support of this capability for both the band of the scheduled/triggered/indicated cell and the band of the scheduling/triggering/indicating cell.

  - FG18-5c/5d

  - If reported value of X in FG18-5c/5d is different between the band of the scheduled/triggered/indicated cell and the band of the scheduling/triggering/indicating cell, the value of X reported for the scheduling/triggering/indicating cell is applied.

Agreements:

- One dormant BWP and one non-dormant BWP are UE specific BWPs even for UEs not supporting 6-2 or 6-3

Conclusion:

- FG for dormancy is not intended to increase the max possible number of UE specific BWPs that Rel-16 NR can support (i.e. 4)

Agreements:

- Send reply LS with following contents – agreed in final LS (R1-2009570)

  - RAN1 has discussed the need of PUCCH-grouping capability for NR-CA with two PUCCH groups and agreed to support the new Rel.16 capabilities copied below. From RAN1 point of view, framework of PUCCH-grouping capability for NR-CA can also be used as potential solution for synchronous NR-DC, and decision is up to RAN2.

  - Copy agreements on both PUCCH grouping and 3/4 SCSs for NR-CA

Update on 11/5: The draft CR and the draft LS are approved. Final CR in R1-2009494 (38.214, CR0138) and R1-2009496 (LS)

[103-e-NR-UEFeature-TEI-01] Email discussion/approval on UE features for NR TEIs (26th Oct – 29th Oct) – Hiroki (DCM)

- Clarify interpretation of FGs in case of cross-carrier operation e.g., for FG14-7

- How to reply to RAN2 LS on beamSwitchTiming

Agreements:

- Regarding the interpretation of UE capabilities in case of cross-carrier operation, RAN1 clarifies that support of the following UE capability is based on the support of this capability for the band of the scheduled/triggered/indicated cell only.

  - FG14-7

| 14. NR TEI | 14-7 | New capability for beamSwitchTiming values of 224 and 336 | - Indicates the minimum number of required OFDM symbols {224, 336} between the DCI triggering aperiodic CSI-RS and the corresponding aperiodic CSI-RS transmission in a CSI-RS resource set configured with repetition ‘ON’ - Candidate values: {224, 336} | 2-28 | Yes | N/A |  | Per band | N/A | N/A (FR2 only) | N/A | Agreements: ・48 is used as the beam switching threshold for Ues reporting 224 or 336 ØWhen using the higher values of the feature (sym224 and sym336), beamSwitchTiming indicates the minimum number of OFDM symbols between the DCI triggering of aperiodic CSI-RS and aperiodic CSI-RS transmission in a CSI-RS resource configured with repetition ‘ON’ to apply TCI indication in CSI-RS triggering DCI. The support of this FG is based on the support of this FG for the band of the scheduled/triggered/indicated cell only. | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Agreements:

- Send LS reply to RAN2 with following contents – final LS in R1-2009496

  - Regarding RAN2 understanding on Rel-15 UE behavior

    - In R15, if the UE indicates one value among {14, 28, 48} for beamSwitchTiming, the UE behavior for receiving AP-CSI-RS configured with repetition (set as ‘ON’ or ‘OFF’) with a scheduling offset smaller than reported beamSwitchTiming was unspecified, while the UE behavior for receiving AP-CSI-RS with a scheduling offset equal to or greater than reported beamSwitchTiming was specified.

    - In R15, when UE reports one value among {224, 336} for beamSwitchTiming, it will be used to determine UE expectation/behavior for aperiodic CSI-RS for tracking and latency requirements for L1-RSRP reporting, while UE behaviour/assumption regarding before or after beam switch timing is unspecified for measuring AP CSI-RS for CSI acquisition (without trs-Info and without repetition) and for beam management (with repetition ‘off’).

    - RAN1 listed ‘No recommendation on the desired beam switching timing’ as ‘Consequences if the feature is not supported by the UE’ in UE feature list R1-1907862.

  - Regarding Rel-16 UE behavior

    - Both beamSwitchTiming and beamSwitchTiming-r16 are optional to report

    - If the UE doesn’t include the beamSwitchTiming-r16, its behaviour is same as Rel-15

    - If the UE does include the Rel-16 capability, UE may or may not receive enableBeamSwitchTiming-r16

    - If the UE does include the Rel-16 capability but doesn’t receive enableBeamSwitchTiming-r16, its behaviour is same as Rel-15

    - If the UE receives enableBeamSwitchTiming-r16

      - For CSI-RS configured with repetition “ON”, the UE applies switch time that is the same as the signalled value from the set {224, 336}

      - For CSI-RS configured with repetition “OFF”, the UE applies switch time of 48 if beamSwitchTiming-r16 is reported

      - For CSI-RS configured without repetition and without trs-info, the UE applies switch time of 48 if beamSwitchTiming-r16 is reported

    - Note that if aperiodic CSI-RS resource set is configured with trs-info, only Rel-15 beamSwitchTiming is used according to RAN1 specification

- Prepare CR for Rel-16 TS38.214 – agreed in final CR in R1-2009494

[103-e-NR-UEFeature-Others-01] Email discussion/approval on NR UE features that are not dedicated to a specific Rel-16 work item/TEI (26th Oct – 3rd Nov) – Hiroki (DCM)

- Whether/how to define new FGs for NR-CA based on working assumption and conclusion made at RAN1#102-e

- How to define new FG(s) for requiring an offset between the end of PDCCH triggering A-SRS and the SRS transmission for CB PUSCH and antenna switching according to agreements made at RAN1#102-e

- Whether/how to define a new FG for supporting partial cancellation of configured PUCCH/PUSCH/PRACH due to dynamic SFI, dynamically granted PDSCH and CSI-RS

- Whether or not to introduce licensed/unlicensed differentiation for some Rel-15 FGs according to the proposal in R1-2008614

Extend to 11/10

Agreements:

- Define a new FG for support of up to three different numerologies for NR part of EN-DC, NGEN-DC, NE-DC and NR-CA where UE is not configured with two NR PUCCH groups

| 22. NR Others | 22-x | Support of up to three different numerologies in the same NR PUCCH group for NR part of EN-DC, NGEN-DC, NE-DC and NR-CA where UE is not configured with two NR PUCCH groups | Support of up to three different numerologies in the same NR PUCCH group for NR-CA where UE is not configured with two NR PUCCH groups 1) Which NR Carrier type(s) that can transmit NR PUCCH |  | Yes | N/A |  | Per BC | N/A | N/A | N/A | Candidate values 1) One or multiple from {FR1 licensed TDD, FR1 unlicensed TDD, FR1 licensed FDD, FR2} that can be configured with the PUCCH transmission | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

- Define a new FG for support of up to four different numerologies for NR part of EN-DC, NGEN-DC, NE-DC and NR-CA where UE is not configured with two NR PUCCH groups

| 22. NR Others | 22-x | Support of up to four different numerologies in the same NR PUCCH group for NR part of EN-DC, NGEN-DC, NE-DC and NR-CA where UE is not configured with two NR PUCCH groups | Support of up to four different numerologies in the same NR PUCCH group for NR-CA where UE is not configured with two NR PUCCH groups 1) Which NR Carrier type(s) that can transmit NR PUCCH |  | Yes | N/A |  | Per BC | N/A | N/A | N/A | Candidate values 1) One or multiple from {FR1 licensed TDD, FR1 unlicensed TDD, FR1 licensed FDD, FR2} that can be configured with the PUCCH transmission | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

- Note: These capabilities are indicated independently for each BC of EN-DC, NGEN-DC, NE-DC and NR-CA

Agreements:

- Whether to support two PUCCH groups is reported per BC for NR-CA with 3 or more bands with at least two carrier types from carrier types {FR1 licensed TDD, FR1 unlicensed TDD, FR1 licensed FDD, FR2}

  - For the BC, the UE reports one or multiple of supported configuration(s) of {primary PUCCH group config, secondary PUCCH group config} where for each supported configuration,

    - the “primary PUCCH group config” includes following information:

      - One or multiple from {FR1 licensed TDD, FR1 unlicensed TDD, FR1 licensed FDD, FR2} mapped to the primary PUCCH group

      - One or multiple from {FR1 licensed TDD, FR1 unlicensed TDD, FR1 licensed FDD, FR2} that can be configured with the PUCCH transmission in the primary PUCCH group

    - the “secondary PUCCH group config” includes following information:

      - One or multiple from {FR1 licensed TDD, FR1 unlicensed TDD, FR1 licensed FDD, FR2} mapped to the secondary PUCCH group

      - One or multiple from {FR1 licensed TDD, FR1 unlicensed TDD, FR1 licensed FDD, FR2} that can be configured with the PUCCH transmission in the secondary PUCCH group

    - Note: for each {primary PUCCH group config, secondary PUCCH group config}, each carrier type of {FR1 licensed TDD, FR1 unlicensed TDD, FR1 licensed FDD, FR2} is mapped to either or both of the primary PUCCH group config and the secondary PUCCH group config.

  - Note: RAN1 will discuss on how to handle the SDL or SUL band, for example as below

    - SDL overlapping with either TDD or FDD can follow the same principle with TDD or FDD accordingly

    - SDL having no overlapped TDD or FDD can follow the same principle with FDD

  - Note: When the carrier type of NUL is indicated for PUCCH transmission location, the SUL in the same cell as in the NUL can also be configured for PUCCH transmission

    - FFS: how to cover licensed/unlicensed and/or FR1/FR2 differentiations

  - Note: When the carrier type of NUL is indicated for one PUCCH group config, the SUL in the same cell as in the NUL can also be configured for the PUCCH group

    - FFS: SUL is counted as number of bands for the condition of this new FG reporting

Working assumption:

2nd FFS in above agreements is removed i.e., SUL is counted as number of bands for the condition of this new FG reporting.

- This note is added not only for FG22-7 but also for FG22-6/6a

Agreements:

- Define new FGs for requiring an offset between the end of PDCCH triggering A-SRS and the SRS transmission for CB PUSCH and antenna switching as below

| 22-x | For SRS for CB PUSCH and antenna switching on FR1 with symbol level offset for aperiodic SRS transmission | For SRS for CB PUSCH and antenna switching on FR1, UE requires minimum of 19 symbols offset between aperiodic SRS triggering and transmission | 2-53 | Yes | N/A |  | Per FS (applicable to FR1 only) | n/a | n/a | n/a |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 22-xa | PDCCH monitoring on any span of up to 3 consecutive OFDM symbols of a slot and constrained timeline for SRS for CB PUSCH and antenna switching on FR1 | - A UE supports FG 3-2 - For SRS for CB PUSCH and antenna switching on FR1, UE requires minimum of 19 symbols offset between aperiodic SRS triggering and transmission | 3-2, 2-53 | Yes | N/A |  | Per FS (applicable to FR1 only) | n/a | n/a | n/a |  | Optional with capability signalling |
| 22-xb | For type 1 CSS with dedicated RRC configuration, type 3 CSS, and UE-SS, monitoring occasion can be any OFDM symbol(s) of a slot for Case 2 and constrained timeline for SRS for CB PUSCH and antenna switching on FR1 | - A UE supports FG 3-5 - For SRS for CB PUSCH and antenna switching on FR1, UE requires minimum of 19 symbols offset between aperiodic SRS triggering and transmission | 3-5, 2-53 | Yes | N/A |  | Per FS (applicable to FR1 only) | n/a | n/a | n/a |  | Optional with capability signalling |
| 22-xc | For type 1 CSS with dedicated RRC configuration, type 3 CSS, and UE-SS, monitoring occasion can be any OFDM symbol(s) of a slot for Case 2 with a DCI gap and constrained timeline for SRS for CB PUSCH and antenna switching on FR1 | - A UE supports FG 3-5a - For SRS for CB PUSCH and antenna switching on FR1, UE requires minimum of 19 symbols offset between aperiodic SRS triggering and transmission | 3-5a, 2-53 | Yes | N/A |  | Per FS (applicable to FR1 only) | n/a | n/a | n/a |  | Optional with capability signalling |
| 22-xd | All PDCCH monitoring occasion can be any OFDM symbol(s) of a slot for Case 2 with a span gap and constrained timeline for SRS for CB PUSCH and antenna switching on FR1 | - A UE supports FG 3-5b - For SRS for CB PUSCH and antenna switching on FR1, UE requires minimum of 19 symbols offset between aperiodic SRS triggering and transmission | 3-5b, 2-53 | Yes | N/A |  | Per FS (applicable to FR1 only) | n/a | n/a | n/a |  | Optional with capability signalling |

Agreements:

- Clarify that partial cancelation of PUCCH/PUSCH/PRACH triggered by dynamic SFI or dynamically assigned PDSCH/CSI-RS is not supported in Rel-15

  - Prepare CR for above clarification in next meeting

- Introduce a new Rel-16 FG for partial cancelation of PUCCH/PUSCH/PRACH as below

| FG 22-x | Cancellation of PUCCH, PUSCH or PRACH with a DCI scheduling a PDSCH or CSI-RS or a DCI format 2_0 for SFI | A UE supports the partial cancellation of the SRS or PUCCH or PUSCH or PRACH configured transmission: - The UE cancels the configured PUCCH or PUSCH or PRACH in a set of symbols of a slot due to detection of a DCI format 2_0 with a slot format value other than 255 that indicates a slot format with a subset of symbols from the set of symbols as downlink or flexible - The UE cancels the configured PUCCH or PUSCH or PRACH in a set of symbols of a slot due to the detection of a DCI format 1_0, DCI format 1_1, DCI format 1_2 or DCI format 0_1 and DCI format 0_2 indicating to the UE to receive CSI-RS or PDSCH in a subset of symbols from the set of symbols. |  | Yes | N/A |  | Per FS | n/a | n/a | n/a |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

- TP for Rel-16 should also be discussed in next meeting

Agreements:

- At least for the following FGs, Rel-16 FGs can be introduced to indicate the support of the feature in unlicensed band

  - FG 1-2 (SS block based SINR measurement (SS-SINR))

  - FG 2-32a/2-32b (Semi-persistent CSI report on PUCCH/PUSCH)

  - FG 3-6 (Dynamic SFI monitoring)

  - [FG 4-19]

  - FG 4-19a/4-19b/4-19c/4-28 (HARQ-ACK multiplexing)

  - FG 4-23 (Repetitions for PUCCH format 1, 3, and 4 over multiple slots with K = 2, 4, 8)

  - FG 5-14/5-16/5-17/5-17a (PDSCH and PUSCH repetitions)

  - [FG 5-18/5-19/5-20/5-21 (SPS and configured grant)]

- Note1: for above listed FGs, indicating the support of Rel-15 FG by Rel-16 UE means support of the feature in licensed band only

- Note2: for above listed FGs, Rel-16 FGs for unlicensed band replicated from Rel-15 are “optional with capability signaling” in UE features list

  - FFS: whether each of Rel-16 versions of 4-19/4-23/4-28/5-17 is part of basic operation for corresponding scenarios of NR-U

- FFS: interpretation of support of FG in case of cross-carrier operation between licensed and unlicensed carriers

R1-2007739	Discussion on NR Rel-16 UE Features	ZTE

R1-2007808	Remaining issues on Rel-16 UE features	CATT

R1-2007939	Rel-16 UE features	Intel Corporation

R1-2008146	Remaining issues on NR Rel-16 UE features	Samsung

R1-2008216	Discussion on Rel-16 UE features	OPPO

R1-2008333	Remaining details of Rel-16 NR UE features	Huawei, HiSilicon

R1-2008427	Discussions on NR Rel-16 UE features	Apple

R1-2008490	Discussion on Rel-16 UE features for NR-U	LG Electronics

R1-2008508	Remaining issues on Rel-16 UE feature list	MediaTek Inc.

R1-2008537	Discussion on NR Rel-16 UE features	NTT DOCOMO, INC.

R1-2008631	Discussion on NR Rel-16 UE features	Qualcomm Incorporated

Revision of R1-2008614

R1-2008639	Remaining details of Rel-16 NR UE features	Ericsson

R1-2008681	Remaining issues on NR Rel-16 UE features	vivo

R1-2008737	Remaining issues on NR UE features	Nokia, Nokia Shanghai Bell

### Others (2)

Including any essential TPs related to the endorsed Rel-16 NR TEIs

R1-2008546	Summary on Rel-16 NR TEI related discussion	Moderator (NTT DOCOMO, INC.)

[103-e-NR-TEIs-01] Email discussion/approval for potential CR(s) for aperiodic TRS triggering with beam switching timing (26th - 29th Oct) – Hiroki (DCM)

- Discuss necessary update of TS38.214 5.1.6.1.1 for the case of cross carrier triggering with different numerologies (based on first part of proposed TP in R1-2008682 (part 1))

Update on 10/29:

- The latest draft CR is approved, with final CR in R1-2009447 (38.214, CR0132)

Final summary in R1-2009330

[103-e-NR-TEIs-02] Email discussion/approval for potential CR(s) and reply LS for maximum number of configurable CSI-RS resources for L3 mobility (26th - 29th Oct) – Hiroki (DCM)

- Discuss necessary update of TS38.214 5.1.6.1.3 to increase maximum number of configurable CSI-RS resources for L3 mobility according to RAN4 request

- Discuss whether/how to introduce new UE capability for the increased maximum number of configurable CSI-RS resources for L3 mobility

- Discuss reply LS to RAN4 (and RAN2)

Update on 10/29:

Agreements:

- Ask RAN2/4 to introduce a new UE capability for the increased maximum number of configurable CSI-RS resources per MO for L3 mobility in the reply LS – lated draft LS (R1-2009369) is approved, with final LS in R1-2009444 – Daewon (Intel)

  - RAN1 assumes following design for the new UE capability signaling

    - Name: increasedNumberofCSIRSPerMO-r16

    - Component description: Indicates support of up to 192 CSI-RS resource for L3 mobility configuration per measurement object configured with associatedSSB

    - Prerequisite FG: 1-4

    - Reporting type: Per UE

    - TDD/FDD differentiation: No

    - FR1/FR2 differentiation: Yes

    - Mandatory/optional: Optional with capability signaling

  - RAN1 believes up to 96 CSI-RS resource (for L3 mobility) configuration per cell and up to 192 CSI-RS resource configuration per MO (as per RAN4 request) should satisfy request from RAN4.

The draft 38.214 CR is approved, with final CR in R1-2009448 (38.214, CR0133)

Summary in R1-2009331

R1-2007911	Discussion on RIM with partially overlapping bandwidths	Ericsson

R1-2007940	Draft TPs regarding umber of configurable CSI-RS resources per MO	Intel Corporation

R1-2008210	Correction to beam switch timing	Ericsson

R1-2008682	"Correction on beamswitch timing for aperiodic TRS

and increased number of CSI-RS for mobility per MO"	vivo

R1-2008736	Increase in number of CSI-RS for L3 mobility	Nokia, Nokia Shanghai Bell

R1-2008796	Other remaining issues for NR MIMO in Rel-16	Huawei, HiSilicon

# Release 17

Limit to maximum 1 contribution per company/organization/university for any agenda item, including “Others”.

## Further enhancements on MIMO for NR

Please refer to RP-202024 for detailed scope of the WI

### Enhancements on Multi-beam Operation

Mainly targeting FR2 while also applicable to FR1

R1-2008147	Moderator summary for multi-beam enhancement	Moderator (Samsung)

R1-2007546	Enhancement on multi-beam operation	FUTUREWEI

R1-2007586	Enhancements on multi-beam operation in Rel-17	Huawei, HiSilicon

R1-2007626	Discussions on Multi-beam Enhancement	InterDigital, Inc.

R1-2007644	Further discussion on multi beam enhancement	vivo

R1-2007763	Enhancements on Multi-beam Operation	ZTE

R1-2007824	Discussion on enhancement on multi-beam operation	CATT

R1-2008000	Enhancements on multi-beam operation	CMCC

R1-2008148	Multi-Beam Enhancements	Samsung

R1-2008217	Enhancements on Multi-beam Operation	OPPO

R1-2008308	Enhancements on NR multi-beam operation	AT&T

R1-2008346	Considerations on the enhancement of multi-beam operation	Sony

R1-2008438	Views on Rel-17 Beam Management enhancement	Apple

R1-2008573	Enhancements on Multi-beam Operation	LG Electronics

R1-2008899	Enhancements on multi-beam operation	Fraunhofer IIS, Fraunhofer HHI

R1-2008903	Enhancements on Multi-beam Operation	Nokia, Nokia Shanghai Bell

R1-2008910	Enhancements on Multi-beam Operation	Lenovo, Motorola Mobility

R1-2008943	Discussion on multi-beam operation	NEC

R1-2008956	Enhancement on multi-beam operation	MediaTek Inc.

R1-2008977	Enhancements to Multi-Beam Operations	Intel Corporation

R1-2009027	Enhancements on multi-beam operation	Xiaomi

R1-2009060	Discussion on Enhancements for Multi-beam Operation	Asia Pacific Telecom co. Ltd

R1-2009129	Enhancements on multi-beam pperation	Sharp

R1-2009141	Enhancements on Multi-beam Operation	Spreadtrum Communications

R1-2009155	Discussion on multi-beam operation	ASUSTeK

R1-2009158	Multi-beam Enhancements	Convida Wireless

R1-2009174	Discussion on multi-beam operation	NTT DOCOMO, INC.

R1-2009250	Enhancements on Multi-beam Operation	Qualcomm Incorporated

R1-2009288	Enhancements on Multi-beam Operation	Ericsson

### Enhancements for Multi-TRP Deployment

#### Enhancements on Multi-TRP for PDCCH, PUCCH and PUSCH

R1-2007540	Multi-TRP/panel for non-PDSCH	FUTUREWEI

R1-2007587	Enhancements on multi-TRP for reliability and robustness in Rel-17	Huawei, HiSilicon

R1-2007627	Reliability Enhancements for PDCCH, PUCCH, and PUSCH	InterDigital, Inc.

R1-2007645	Further discussion on enhancement of MTRP operation	vivo

R1-2007764	Multi-TRP enhancements for PDCCH, PUCCH and PUSCH	ZTE

R1-2007783	Enhancements on Multi-TRP for PDCCH, PUCCH and PUSCH	Fujitsu

R1-2007793	Enhancements on Multi-TRP for PDCCH, PUCCH and PUSCH	TCL Communication Ltd.

R1-2007825	Discussion on enhancements on multi-TRP/panel for PDCCH, PUCCH and PUSCH	CATT

R1-2008001	Enhancements on Multi-TRP for PDCCH, PUCCH and PUSCH	CMCC

R1-2008149	Enhancements on Multi-TRP for PDCCH, PUCCH and PUSCH	Samsung

R1-2008218	Enhancements on Multi-TRP based enhancement for PDCCH, PUCCH and PUSCH	OPPO

R1-2008347	Considerations on Multi-TRP for PDCCH, PUCCH, PUSCH	Sony

R1-2008439	Views on Rel-17 multi-TRP reliability enhancement	Apple

R1-2008574	Enhancements on Multi-TRP for PDCCH, PUCCH and PUSCH	LG Electronics

R1-2008898	On multi-TRP enhancements for PDCCH and PUSCH	Fraunhofer IIS, Fraunhofer HHI

R1-2008904	Enhancements for Multi-TRP URLLC schemes	Nokia, Nokia Shanghai Bell

R1-2008911	Enhancements on Multi-TRP for PDCCH, PUCCH and PUSCH	Lenovo, Motorola Mobility

R1-2008944	Discussion on multi-TRP for PDCCH, PUCCH and PUSCH	NEC

R1-2008958	Enhancements on Multi-TRP for PDCCH, PUSCH and PUCCH	MediaTek Inc.

R1-2008978	Multi-TRP enhancements for PDCCH, PUCCH and PUSCH	Intel Corporation

R1-2009028	Enhancements on Multi-TRP for PDCCH, PUCCH and PUSCH	Xiaomi

R1-2009054	Discussion on enhancements on multi-TRP for uplink channels	Asia Pacific Telecom co. Ltd

R1-2009130	Enhancements on multi-TRP for PUSCH	Sharp

R1-2009142	Discussion on enhancements on Multi-TRP for PDCCHPUCCH and PUSCH	Spreadtrum Communications

R1-2009159	Multi-TRP Enhancements for PDCCH, PUCCH and PUSCH	Convida Wireless

R1-2009175	Discussion on MTRP for reliability	NTT DOCOMO, INC.

R1-2009223	On PDCCH, PUCCH and PUSCH enhancements with multiple TRPs	Ericsson

R1-2009251	Enhancements on Multi-TRP for PDCCH, PUCCH and PUSCH	Qualcomm Incorporated

R1-2009284	Enhancements on Multi-TRP for PDCCH, PUCCH and PUSCH	Qualcomm Incorporated

Withdrawn

#### Enhancements on Multi-TRP inter-cell operation

R1-2007541	Inter-cell multi-TRP operation	FUTUREWEI

R1-2007588	Enhancements on inter-cell multi-TRP operations in Rel-17	Huawei, HiSilicon

R1-2007628	Synchronization Analysis for M-TRP Inter-cell Operation	InterDigital, Inc.

R1-2007646	Further discussion on inter-cell MTRP operation	vivo

R1-2007765	Discussion on Multi-TRP inter-cell operation	ZTE

R1-2007826	Discussion on multi-TRP/panel inter-cell operation	CATT

R1-2008002	Enhancements on Multi-TRP inter-cell operation	CMCC

R1-2008150	Enhancements on Multi-TRP inter-cell operation	Samsung

R1-2008219	Enhancement on inter-cell multi-TRP operation	OPPO

R1-2008348	Considerations on inter-cell operation	Sony

R1-2008440	Views on Rel-17 Inter-cell multi-TRP operation	Apple

R1-2008575	Enhancements on Multi-TRP inter-cell operation	LG Electronics

R1-2008905	Enhancements to enable inter-cell multi-TRP operations	Nokia, Nokia Shanghai Bell

R1-2008912	Enhancements on Multi-TRP inter-cell operation	Lenovo, Motorola Mobility

R1-2008945	Discussion on multi-TRP inter-cell operation	NEC

R1-2008979	Multi-TRP enhancements for inter-cell operation	Intel Corporation

R1-2009029	Enhancement on Inter-cell Multi-TRP operations	Xiaomi

R1-2009070	Enhancement on Multi-TRP inter-cell operation	Ericsson

R1-2009143	Discussion on enhancement on multi-TRP inter-cell operation	Spreadtrum Communications

R1-2009176	Discussion on inter-cell multi-TRP operations	NTT DOCOMO, INC.

R1-2009252	Enhancements on Multi-TRP inter-cell operation	Qualcomm Incorporated

#### Enhancements on beam management for multi-TRP

R1-2007542	Beam management for simultaneous multi-TRP transmission with multi-panel reception				FUTUREWEI

R1-2007589	Discussion on multi-TRP for multi-panel reception in Rel-17	Huawei, HiSilicon

R1-2007629	Beam Management Enhancements for Multi-TRP	InterDigital, Inc.

R1-2007647	Further discussion on MTRP multibeam enhancement	vivo

R1-2007766	Enhancements on beam management for Multi-TRP	ZTE

R1-2007784	Enhancements on beam management for multi-TRP	Fujitsu

R1-2007827	Beam management enhancement for multi-TRP simultaneous reception	CATT

R1-2008003	Enhancements on beam management for multi-TRP	CMCC

R1-2008151	Enhancements on beam management for multi-TRP	Samsung

R1-2008220	Enhancements on beam management for multi-TRP	OPPO

R1-2008309	Enhancements on beam management for multi-TRP	AT&T

R1-2008349	Considerations on beam management for multi-TRP	Sony

R1-2008441	Views on Rel-17 multi-TRP BM enhancement	Apple

R1-2008576	Enhancements on beam management for multi-TRP	LG Electronics

R1-2008906	Enhancements on Beam Management for Multi-TRP/Panel Transmission	Nokia, Nokia Shanghai Bell

R1-2008913	Enhancements on beam management for multi-TRP	Lenovo, Motorola Mobility

R1-2008946	Discussion on beam management for multi-TRP	NEC

R1-2008957	Enhancements on beam management for multi-TRP	MediaTek Inc.

R1-2008980	Multi-TRP enhancements for beam management	Intel Corporation

R1-2009030	Enhancement on beam management for Multi-TRP	Xiaomi

R1-2009052	Discussion of enhancements on beam management for multi-TRP	Asia Pacific Telecom co. Ltd

R1-2009144	Discussion on enhancements on beam management for multi-TRP	Spreadtrum Communications

R1-2009156	Discussion on beam management for multi-TRP	ASUSTeK

R1-2009160	On Per-TRP Beam Failure Recovery	Convida Wireless

R1-2009177	Discussion on beam management for MTRP	NTT DOCOMO, INC.

R1-2009213	Discussion on beam management for multi-TRP	ITRI

R1-2009225	On beam management enhancements for simultaneous multi-TRP transmission with multi-panel reception	Ericsson

R1-2009253	Enhancements on beam management for multi-TRP	Qualcomm Incorporated

#### Enhancements on HST-SFN deployment

R1-2007543	Enhancement to support HST-SFN deployment scenario	FUTUREWEI

R1-2007590	Discussion on multi-TRP for high speed train in Rel-17	Huawei, HiSilicon

R1-2007630	Enhancements for M-TRP to Support HST-SFN Deployment	InterDigital, Inc.

R1-2007648	Further discussion and evaluation on HST-SFN schemes	vivo

R1-2007767	Discussion on Multi-TRP HST enhancements	ZTE

R1-2007828	On enhancements on HST-SFN deployment	CATT

R1-2008004	Enhancements on HST-SFN deployment	CMCC

R1-2008152	Enhancements on HST-SFN	Samsung

R1-2008221	Enhancements on HST-SFN deployment	OPPO

R1-2008350	Considerations on HST-SFN operation for multi-TRP	Sony

R1-2008442	Views on Rel-17 HST enhancement	Apple

R1-2008577	Enhancements on HST-SFN deployment	LG Electronics

R1-2008907	Enhancements for HST-SFN deployment	Nokia, Nokia Shanghai Bell

R1-2008947	Discussion on HST-SFN deployment	NEC

R1-2008981	Enhancements to HST-SFN deployments	Intel Corporation

R1-2009069	Enhancement on HST-SFN deployment	Ericsson

R1-2009099	Enhancements for HST-SFN deployment	Lenovo, Motorola Mobility

R1-2009145	Discussion on enhancements on HST-SFN deployment	Spreadtrum Communications

R1-2009178	Discussion on HST-SFN deployment	NTT DOCOMO, INC.

R1-2009254	Enhancements on HST-SFN deployment	Qualcomm Incorporated

### Enhancements on SRS flexibility, coverage and capacity

R1-2007544	Enhancements on SRS flexibility, coverage and capacity	FUTUREWEI

R1-2007591	Discussion on SRS enhancements for Rel-17	Huawei, HiSilicon

R1-2007631	Discussion on SRS Enhancements	InterDigital, Inc.

R1-2007649	Further discussion on SRS enhancement	vivo

R1-2007768	Enhancements on SRS flexibility, coverage and capacity	ZTE

R1-2007829	On enhancements on SRS  flexibility, coverage and capacity	CATT

R1-2008005	Enhancements on SRS flexibility, coverage and capacity	CMCC

R1-2008153	Enhancements on SRS	Samsung

R1-2008222	Enhancements on SRS flexibility, coverage and capacity	OPPO

R1-2008351	Considerations on SRS flexibility, coverage and capacity	Sony

R1-2008443	Views on Rel-17 SRS enhancement	Apple

R1-2008578	Enhancements on SRS flexibility, coverage and capacity	LG Electronics

R1-2008900	Enhancements on SRS for coverage and capacity	Fraunhofer IIS, Fraunhofer HHI

R1-2008908	Enhancements on SRS flexibility, coverage and capacity	Nokia, Nokia Shanghai Bell

R1-2008914	Enhancements on SRS	Lenovo, Motorola Mobility

R1-2008948	Discussion on SRS enhancement	NEC

R1-2008959	Enhancements on SRS flexibility, coverage and capacity	MediaTek Inc.

R1-2008982	Discussion on SRS enhancements	Intel Corporation

R1-2009031	Discussion on SRS enhancements	Xiaomi

R1-2009131	Enhancements on SRS	Sharp

R1-2009146	Considerations on SRS enhancement	Spreadtrum Communications

R1-2009179	Discussion on SRS enhancement	NTT DOCOMO, INC.

R1-2009211	SRS Performance and Potential Enhancements	Ericsson LM

R1-2009255	Enhancements on SRS flexibility, coverage and capacity	Qualcomm Incorporated

R1-2009286	Discussion on enhancement of SRS in Rel. 17 further enhanced MIMO 	CEWiT

### CSI enhancements: MTRP and FR1 FDD reciprocity

R1-2007545	CSI enhancement for multi-TRP and FDD	FUTUREWEI

R1-2007592	Discussion on CSI Enhancements for Rel-17	Huawei, HiSilicon

R1-2007632	CSI Enhancements for the Support of MTRP and FDD Reciprocity	InterDigital, Inc.

R1-2007650	Further discussion and evaluation on MTRP CSI and Partial reciprocity	vivo

R1-2007769	CSI enhancements for Multi-TRP and FR1 FDD reciprocity	ZTE

R1-2007830	CSI enhancements for MTRP and FR1 FDD with partial reciprocity	CATT

R1-2008006	Enhancements on CSI reporting for Multi-TRP	CMCC

R1-2008154	Views on Rel. 17 CSI enhancements	Samsung

R1-2008223	CSI enhancement for M-TRP and FDD reciprocity	OPPO

R1-2008352	Considerations on CSI enhancements	Sony

R1-2008444	Views on Rel-17 CSI enhancement	Apple

R1-2008579	CSI enhancements for Rel-17	LG Electronics

R1-2008901	CSI enhancements on Type II PS codebook and multi-TRP	Fraunhofer IIS, Fraunhofer HHI

R1-2008909	Enhancement on CSI measurement and reporting	Nokia, Nokia Shanghai Bell

R1-2008949	Discussion on CSI enhancement for multi-TRP	NEC

R1-2008960	CSI enhancement for NCJT	MediaTek Inc.

R1-2008983	On CSI enhancements for MTRP and FDD reciprocity	Intel Corporation

R1-2009100	CSI enhancements for mTRP and FDD reciprocity	Lenovo, Motorola Mobility

R1-2009147	Discussion on CSI enhancement for M-TRP transmission and FR1 FDD reciprocity	Spreadtrum Communications

R1-2009180	Discussion on CSI enhancements	NTT DOCOMO, INC.

R1-2009224	On CSI enhancements in Rel-17 feMIMO	Ericsson

R1-2009256	CSI enhancements - MTRP and FR1 FDD reciprocity	Qualcomm Incorporated

### Others

R1-2007547	Sounding enhancement for interference probing in TDD massive MIMO	FUTUREWEI

R1-2007633	Updated Results on Multi-TRP MIMO Deployments with Multi-Panel UEs	InterDigital, Inc.

R1-2007651	Discussion on higher layer aspects for multi-TRP enhancement	vivo

R1-2007770	Further details on Multi-beam and Multi-TRP operation	ZTE

R1-2007831	Evaluation of multi-TRP enhancement	CATT

R1-2008155	Simulation results for multi-beam enhancements	Samsung

R1-2008323	Discussion on field measurement and evaluation assumptions for FDD CSI enhancements in Rel-17		Huawei, HiSilicon

R1-2008755	Analysis of Control Signaling for multi-beam operation	Dongguan OPPO Precision Elec.

R1-2008902	On FDD channel reciprocity in real-world scenarios	Fraunhofer IIS, Fraunhofer HHI

R1-2008915	HARQ feedback of SPS PDSCH reception in multi-DCI based multiple TRPs	Lenovo, Motorola Mobility

R1-2009010	Evaluation of multi-TRP transmission with multi-panel reception	ETRI

R1-2009132	Other enhancements for beam management.	Sharp

R1-2009181	Discussion on UL dense deployment	NTT DOCOMO, INC.

R1-2009290	Additional simulation results on multi-beam operation	Ericsson

## Study on supporting NR from 52.6GHz to 71 GHz

Please refer to RP-201838 for detailed scope of the SI

R1-2007958	Draft TR 38.808 v002: Study on supporting NR from 52.6 GHz to 71 GHz	Intel Corporation

Agreement:

R1-2007958 is endorsed with the “smallest of Z_min” modifed to “smallest value of Z_max” and setting Z_min equal to 0 in Section A.3. Modifications to fix errors will be made as part of upcoming updates.

### Required changes to NR using existing DL/UL NR waveform

R1-2007549	"Further discussion on B52 numerology"	FUTUREWEI

R1-2007558	Discussion on physical layer impacts for NR beyond 52.6 GHz	Lenovo, Motorola Mobility

R1-2007604	PHY design in 52.6-71 GHz using NR waveform	Huawei, HiSilicon

R1-2007642	Physical layer design for NR 52.6-71GHz	Beijing Xiaomi Software Tech

R1-2007652	Discussion on requried changes to NR using existing DL/UL NR waveform	vivo

R1-2007785	Consideration on required changes to NR using existing NR waveform	Fujitsu

R1-2007790	Consideration on supporting above 52.6GHz in NR	InterDigital, Inc.

R1-2007847	System Analysis of NR opration in 52.6 to 71 GHz	CATT

R1-2007883	Required changes to NR using existing DL/UL NR waveform	TCL Communication Ltd.

R1-2007926	Required changes to NR using existing DL/UL NR waveform	Nokia, Nokia Shanghai Bell

R1-2007929	On phase noise compensation for NR from 52.6GHz to 71GHz	Mitsubishi Electric RCE

R1-2007941	Discussion on Required Changes to NR in 52.6 – 71 GHz	Intel Corporation

R1-2007965	On the required changes to NR for above 52.6GHz	ZTE, Sanechips

R1-2007982	On NR operations in 52.6 to 71 GHz	Ericsson

R1-2008045	Consideration on required physical layer changes to support NR above 52.6 GHz	LG Electronics

R1-2008076	Discussion on required changes to NR using existing DL/UL NR waveform in 52.6GHz ~ 71GHz			CMCC

R1-2008082	Study on the numerology to support 52.6 GHz to 71GHz	NEC

R1-2008156	Design aspects for extending NR to up to 71 GHz	Samsung

R1-2008250	Discusson on required changes to NR using DL/UL NR waveform	OPPO

R1-2008353	Considerations on required changes to NR from 52.6 GHz to 71 GHz	Sony

R1-2008457	A Discussion on Physical Layer Design for NR above 52.6GHz	Apple

R1-2008493	Discussions on required changes on supporting NR from 52.6GHz to 71 GHz	CAICT

R1-2008501	On required changes to NR using existing DL/UL NR waveform for operation in 60GHz band			MediaTek Inc.

R1-2008516	On NR operation between 52.6 GHz and 71 GHz	Convida Wireless

R1-2008547	Evaluation Methodology and Required Changes on NR from 52.6 to 71 GHz	NTT DOCOMO, INC.

R1-2008615	NR using existing DL-UL NR waveform to support operation between 52p6 GHz and 71 GHz			Qualcomm Incorporated

R1-2008726	Discussion on physical layer aspects for NR beyond 52.6GHz	WILUS Inc.

R1-2008769	Waveform considerations for NR above 52.6 GHz	Charter Communications

[103-e-NR-52-71-Waveform-Changes] Email discussion/approval on required changes to NR using existing DL/UL NR waveform and any TR updates until 11/2; address any remaining aspects by 11/10 – Daewon (Intel)

Additional checkpoints where agreements can be declared: 11/5 and 11/12 (11/12 for approval of final TR)

R1-2009313	Issue Summary for physical layer changes for supporting NR from 52.6 GHz to 71 GHz	Moderator (Intel Corporation)

R1-2009352	[103-e-NR-52-71-Waveform-Changes] Discussions Summary #1	Moderator (Intel Corporation)

R1-2009688	[103-e-NR-52-71-Waveform-Changes] Discussions Summary #4	Moderator (Intel Corporation)

R1-2009403	[103-e-NR-52-71-Waveform-Changes] Discussions Summary #1	Moderator (Intel Corporation)

R1-2009540	[103-e-NR-52-71-Waveform-Changes] Discussions Summary #2	Moderator (Intel Corporation)

R1-2009667	[103-e-NR-52-71-Waveform-Changes] Discussions Summary #3	Moderator (Intel Corporation)

R1-2009717	[103-e-NR-52-71-Waveform-Changes] Discussions Summary #5	Moderator (Intel Corporation)

R1-2009668	Summary of 38.808 TR Text Proposal Discussion	Moderator (Intel Corporation)

Agreement:

Numerologies below 120 kHz or above 960 kHz are not supported for any signal or channel.

Agreement:

For operation in 52-71 GHz:

- 120 kHz should be supported

- Up to two additional SCS may be considered and at least one should be supported

- FFS: Applicability of additional SCS to particular signals and channels

Agreement:

Capture the following observations in the TR. Editorial modifications and changes to references can be made when capturing the observations in the TR.

- It was observed that amount of specification effort increases with the number of new numerologies enabled and supported for 52.6 GHz to 71 GHz frequency.

- In order to minimize specification effort while maximizing supported use cases and deployment scenarios applicable for 52.6 GHz to 71 GHz frequency, It is recommended to support 120 kHz subcarrier spacing with normal CP length, and at least one more subcarrier spacing. It is recommended to consider supporting at most up to three subcarrier spacings, including 120 kHz subcarrier spacing. Applicability of the supported subcarrier spacing to particular signals and channels should be further discussed in the corresponding WI phase.

- It is recommended that numerologies 240 kHz, 480 kHz, and 960 kHz are considered as candidates for additional numerologies in addition to 120 kHz, and numerologies outside this range are not supported for any signals or channels.

- In order to bound implementation complexity, it is recommended to limit the maximum FFT size required to operate system in 52.6 GHz to 71 GHz frequency to 4096 and to limit the maximum of RBs per carrier to 275 RBs.

- Selection of the additional subcarrier spacing (on top of 120 kHz) should consider versatility of being able to support various applications and deployment scenarios with all the subcarrier spacings that would be supported by specification, accounting for what is already supported in Rel-15 and Rel-16 specifications.

- Some companies have noted that ability for a deployed system to operate with a single numerology for all channels and signals is beneficial, and some companies have further noted benefit remains even if SSB numerology is different. Some companies have noted mixed numerology operation is functional and is supported in Rel-15 and Rel-16 specifications (e.g. 240 kHz SSB subcarrier spacing with 120 kHz subcarrier spacing for PDCCH/PDSCH/PUSCH/PUCCH/PRACH in an initial BWP and activation of a dedicated BWP with SCS different than the initial BWP) and consideration of single numerology operation is not needed.

Agreement:

Capture the following observations in the TR. Editorial modifications and changes to references can be made when capturing the observations in the TR.

Overall implementation complexity for supporting a specific subcarrier spacing may need to consider the following, but not limited to:

- processing complexity for equalization including inter-carrier interference mitigation (if required to support higher modulation orders) and compensation, andFFT complexity per unit time for a given bandwidth,

- complexity associated with supporting multiple component carriers to reach a specific throughput

- complexity associated with supporting given reduced (in abosolute time) requirements on UE processing times (e.g. N1, N2, N3, Z1, Z2, Z3, etc) and UE PDCCH processing budget as a function of subcarrier spacing, if scheduling and monitoring unit is maintained to be one slot.

- supported features indicated by UE capability signaling or implemented by the gNB

- complexity associated with supporting required timing error tolerance which may need to considerinitial timing error, timing advance setting, TA granularity, MIMO TAE (TAE value will be defined by RAN4), multi-TRP timing alignment as a function of SCS, whether mixture or a single subcarrier spacing for signals is configured, and deployment scenarios.

- complexity associated with supporting higher sampling rates and with channel bandwidth larger than 2 GHz

Agreement:

- It is observed that for a single carrier with the same number of transmitted symbols, in general, smaller subcarrier spacing may potentially provide larger coverage due to use of smaller bandwidth and gears towards (but not limited to) coverage driven scenarios.

- It is observed that for a single carrier, in general, larger subcarrier spacing may potentially provide higher peak data rates due to use of larger bandwidth and gears towards (but not limited to) peak data-rate driven scenarios.

Agreement:

Capture the following observations in the TR. Editorial modifications and changes to references can be made when capturing the observations in the TR.

- Some companies noted that standardization effort to support 240 kHz, 480 kHz, and 960 kHz numerologies are comparable. Some companies noted that standardization effort for 240 kHz numerology could be relatively smaller compared to 480 kHz or 960 kHz numerologies.

- The following, which is not an exhaustive list, are some potential physical layer impact that are common to all numerologies:

  - supporting unlicensed operation

  - if mixed numerology is supported, supporting mixed numerology operation.

  - SSB and CORESET#0 offsets needed for supported channelization

- The following, which is not an exhaustive list, are some potential physical layer impact areas for each numerology:

  - 120 kHz:

    - Potential consideration of PTRS enhancement for CP-OFDM and DFT-s-OFDM, if needed

  - 240 kHz:

    - Potential consideration of PTRS enhancement for CP-OFDM and DFT-s-OFDM, if needed

    - If common SSB/CORESET0 numerology (240/240) is supported, SSB patterns, and CORESET#0 configuration

    - RO configuration

    - Timelines for scheduling, processing and HARQ

    - Potential enhancement to DM-RS, if needed

    - PDCCH monitoring

  - 480 kHz:

    - If 480 kHz SSB is supported, SSB patterns, and CORESET#0 configuration

    - Timelines for scheduling, processing and HARQ

    - RO configuration

    - Potential enhancement to DM-RS, if needed

    - PDCCH monitoring

    - Potential consideration of PTRS enhancement for CP-OFDM and DFT-s-OFDM, if neeeded

  - 960 kHz:

    - Potential consideration of ECP, if needed, depending on deployment scenarios

    - If 960 kHz SSB is supported, SSB patterns, and CORESET#0 configuration

    - Timelines for scheduling, processing and HARQ

    - RO configuration

    - Potential enhancement to DM-RS, if needed

    - PDCCH monitoring

    - Potential updates to smallest time unit, Tc, used in specifications depending on supported maximum carrier BW

Agreement:

Capture the following observations in the TR. Editorial modifications and changes to references can be made when capturing the observations in the TR.

Observations on the delay spread distribution:

- One source (R1-2007654, vivo) observed that for the delay spread distributions for the typical indoor scenarios evaluated, the delay spread of almost 80% of the users are less than 30 nsec.

- One source (R1-2007982, Ericsson) observed that Factory Scenario A (InF-DH) results in post-beamforming delay spreads that are a significant fraction of the CP duration for 960 kHz SCS.

- One source (R1-2007943, Intel) observed that 85% of the UE experience r.m.s delay spread small than CP length of 1.92 MHz subcarrier spacing (i.e. 36.6ns) in indoor, outdoor, and factory scenarios.

- One source (R1-2008615, Qualcomm) observed that for small range indoor hotspot deployment, the channel delay spread is not an issue with normal CP. For outdoor scenarios with larger ISD and at moderate to high SNR (this may be produced by higher EIRP or smaller BW), normal CP demonstrates SINR degradation compared to extended CP. However, for such large coverage, high EIRP, and small BW use cases, we can choose to use a small SCS, e.g., 120kHz, with NCP.

- One source (R1-2007790, Interdigital) observed that while each scenario experiences different amounts of r.m.s. delay spread, regardless of scenarios, most of UEs experience smaller r.m.s. delay spreads than normal CP of 960 kHz.

- One source (R1-2009062, Docomo) observed that the mean r.m.s. delay spread of 60 GHz system in Outdoor-B scenario is about 23 nsec and the 95%-tile delay spread value is about 80 nsec. More than half of UE experiences channels with delay larger than 20 ns, which should be referred to in the link performance evaluation with large delay configurations.

Agreement:

Capture the following observations in the TR (Editorial modifications and changes to references can be made when capturing the observations in the TR):

- Some companies have noted support of channelization that are aligned with IEEE 802.11ad and 802.11ay channelization is beneficial for coexistence. While some companies have noted alignment of channelization for coexistence is not necessary. Alignment of channelization between a NR channel and IEEE 802.11ad and 802.11ay channel in this context refers to a NR channel that is contained within one of the channels defined for IEEE 802.11ad and 802.11ay and NR channel bandwidth does not cross over channel boundaries of IEEE 802.11ad and 802.11ay.

- One company has evaluated misaligned NR wideband channels with 1.6 GHz and 2 GHz without LBT and have not identified coexistence issues between NR and NR.

- Some companies proposed that 2 GHz channel bandwidth should be supported andhave the raster points for 2 GHz channel bandwidth to be aligned with IEEE 802.11ad and 802.11ay channelization.

- Some companies proposed that 1.6 GHz should be the maximum channel bandwidth and channels do not necessarily need to be aligned with IEEE 802.11ad and 802.11ay channelizations.

- Some companies observed that support of channel bandwidth such as 200 or 400 MHz may enable efficient usage of available spectrum by 3GPP technology. Some companies observed that only supporting channelization that are alignemed with IEEE 802.11ad and 802.11ay channelization result in smaller number of supported channels for some regions of the world.

- Some companies have observed that channelization based on granularity of minimum supported channel BW would be benefitial and could provide efficient usage of available specturm. Other companies have observerd that support of channel BW such as 1.6 GHz or 2.4GHz would enable efficient usage of 5 GHz allocation in China and 5 GHz IMT allocation in Europe. Some companies have observed that smaller bandwidth (e.g. 1.6 GHz) allows for more channels (e.g., with 1.6 GHz, 3 channels instead of two) in these regions, easing frequency planning between operators at the cost of reduction in available channel bandwidth per carrier.

- Some companies proposed to support more than one channel bandwidths for a given SCS.

Agreement:

Capture the following observations in the TR (Editorial modifications and changes to references can be made when capturing the observations in the TR):

- Some companies noted SSB SCS selection should consider SCS of data/control channels and enablement of single subcarrier spacing operation.

- Some companies noted support and use of 120 kHz and/or 240 kHz SCS for SSB and 120 kHz subcarrier spacing for CORESET#0 in initial BWP and activation of dedicated BWP with an SCS for data/control different than the initial BWP  may enable re-use of existing NR specification and minimize standardization effort.

- It was identified to further investigate considerations of SSB patterns, if needed, considering:

  - Unlicensed band operation if LBT is required for SSB, e.g. SSB cycling transmission within a DRS transmission window.

  - Beam switching time between SSB,

  - Coverage of SSB

  - Multiplexing of SSB with CORESET and UL transmissions

Agreement:

Capture the following observations in the TR (Editorial modifications and changes to references can be made when capturing the observations in the TR):

- In order to benefit from higher transmit power, when maximum PSD regulatory requirements exist, RAN1 recommends support of longer PRACH sequence lengths, L=571 and L=1151, defined in Rel-16 NR specification, to be used for NR operating in 52.6 GHz to 71 GHz.

- It is recommended to not support interlace design for PRACH for NR operating in 52.6 GHz to 71 GHz.

- It is recommended to further investigate whether or not to support configurations that enable non-consecutive RACH occasions in time domainto aid LBT processes if LBT is required.

- Some companies noted that PRACH SCS selection should consider SCS of data/control channels and enablement of single subcarrier spacing operation.

- Some companies noted that 120 kHz SCS for PRACH (even if data/control channel may have different SCS) may be sufficient to support NR operating in 52.6 GHz to 71 GHz from coverage perspective.

- It was identified that potential enhancements for PRACH should consider system coverage for PRACH with subcarrier spacing larger than 120 kHz, if supported.

Agreement:

Capture the following observations in the TR (Editorial modifications and changes to references can be made when capturing the observations in the TR):

- It was identified that the potential enhancements to PDCCH monitoring including potential limitation to UE PDCCH configuration,, multiple PDSCH/PUSCH scheduling with a single DCI (using existing DCI formats or new DCI format(s)), spatial relation management for GC-PDCCH, capability related to PDCCH monitoring, and PDCCH coverage should be further investigated for higher subcarrier spacings, including the need for such enhancements.

- It was observed that PDCCH processing capabilities per multiple slots for larger SCS (e.g. 480 or 960 kHz) can maintain scheduling framework same as for smaller SCS (e.g. 120 kHz) when the UE is configured to monitor the PDCCH every multiple slots.

Agreement:

Capture the following observations in the TR (Editorial modifications and changes to references can be made when capturing the observations in the TR):

- Some companies have noted that interlace transmissions for PUSCH do not provide benefit over non-interlaced uplink allocations currently supported by NR for NR operating in 52.6 GHz to 71 GHz, while some companies have noted support of sub-PRB or PRB interlace transmissions for PUSCH may improve transmit power and possibly meets OCB requirements (some companies note OCB requirements can be met without introducing interlacing) when necessary.

- It was identified that for new subcarrier spacing, if agreed, will at least require investigation on the need for enhacnments and standardization, of the following processing timelines:

  - Processing capability for PUSCH scheduled by RAR UL grant

  - Dynamic SFI and SPS/CG cancellation timing

  - Timeline for HARQ-ACK information in response to a SPS PDSCH release/dormancy.

  - Minimum time gap for wake-up and Scell dormancy indication (DCI format 2_6)

  - BWP switch delay

  - Multi-beam operation timing (timeDurationForQCL, beamSwitchTiming, beam switch gap, beamReportTiming, etc.)

  - Timeline for multiplexing multiple UCI types

  - Minimum of P_switch for search space set group switching

  - appropriate configuration(s) of k0 (PDSCH), k1 (HARQ), k2 (PUSCH),

  - PDSCH processing time (N1), PUSCH preparation time (N2), HARQ-ACK multiplexing timeline (N3)

  - CSI processing time, Z1, Z2, and Z3, and CSI processing units

  - Any potential enhancements to CPU occupation calculation

  - Related UE capability(ies) for processing timelines

  - minimum guard period between two SRS resources of an SRS resource set for antenna switching

- It was identified that new subcarrier spacing, if agreed, may require further investigation of multi-PDSCH/PUSCH scheduling and standardization, if needed. The following aspects should be at least investigated for multi-PDSCH/PUSCH scheduling:

  - whether to support a single TB and/or multiple TBs scheduled over multiple slots

  - applicable DCI format(s) (including potential new formats, if needed) for multi-PDSCH and multi-PUSCH scheduling

  - Enhancement on multiple beam indication and association with multiple PDSCH/PUSCH scheduling

  - DM-RS enhancements such as DM-RS bundling, or changes to the time-domain pattern

  - HARQ enhancements for multi-PDSCH

  - Applicability of Rel-16 multi-PUSCH scheduling

Agreement:

Capture the following observations in the TR (Editorial modifications and changes to references can be made when capturing the observations in the TR):

It is recommended to further investigate potential enhancements to PUCCH to enable higher transmission power when regulatory limits apply. Further potential enhancements to spatial relation management for configured and/or semi-persistent UL signals/channels may be considered.

- Majority of the sources have identified PUCCH format 0, 1, and 4 as potential candidates for enahancement.

- Two sources has identified identified all PUCCH formats as potential candidates for enhancement.

Agreement:

Capture the following observations in the TR (Editorial modifications and changes to references can be made when capturing the observations in the TR):

- It is observed that in Rel-15 NR, absolute time for UE processing requirements generally decrease as subcarrier spacing increases. Some companies noted that introducing smaller UE processing time than Rel-15 and Rel-16, for larger subcarrier spacing, may lead to a more complex UE implementation. Some companies noted that  per slot level monitoring for transmission and reception may not likely be the only mode of operation for higher subcarrier spacing, while some companies noted that per slot level monitoring for transmission and reception may be used as a mode of operation in scenarios that require lower latency.

- It is observed that, in general, larger subcarrier spacing may have benefit of short symbol/slot length to support lower latency requirements compared to what was supported for Rel-15 and Rel-16 NR, assuming slot-level monitoring subject to scheduling configurations and potentially UE processing capabilities.

- It is observed that, in general, channel access with shorter symbol duration may access channel earlier when LBT is passed, assuming slot-level monitoring and potentially subject to UE processing capabilities.

- It is observed that, in general, larger subcarrier spacing has higher resilience towards phase noise. Also, in general, the performance impact from phase noise may depend on various properties of the transmission, such as modulation order and coding rate, reception processing (e.g. CPE compensation), and phase noise profile of the UE and gNB.

- It is observed that, in general, maximum delay spread supported by a SCS is proportional to its CP length and larger subcarrier spacing reduces the budget for timing errors and beam switching, if beam switching delay within CP cannot be avoided by gNB (e.g. by allocating a time gap), due to shorter CP.

  - CP needs to consider at least delay spread, timing errors (including Te), and timing alignment errors applicable for a deployment scenario.

  - Minimum requirements on timing errors for new SCS values in > 52.6 GHz should be further studied in RAN4 when specifications are developed.

- Extended CP decreases the spectrum efficiency up to 14% compared to normal CP of the same subcarrier spacing.

Agreement:

Capture the following observations in the TR (Editorial modifications and changes to references can be made when capturing the observations in the TR):

- Some companies observed that the relationship between channel bandwidth and initial access aspects should be taken into account for the supported channel bandwidth(s), especially for minimum channel bandwidth. Some companies observed that a wider minimum channel bandwidth supported for a band may help to limit the number of synchronization raster entries in the band, if the same design principle for Rel-15 licensed bands applies (Minimum channel bandwidth and synchronization raster entries will be defined by RAN4).

- Available bandwidth within a given carrier for RMSI transmission for SSB and CORESET multiplexing pattern 2 and 3 is smaller than available bandwidth for multiplexing pattern 1. Some companies observed that the channel bandwidth supported for a band should be wide enough to  enable multiplexing e.g. between SSB, CORESET0, and RMSI transmissions in multiplexing pattern 2 and 3. Some companies observed that depending on the supported carrier bandwidth and configured values of O and M, multiplexing pattern 1 can make available more time/frequency resources for RMSI PDSCH in a slot than pattern 2 and 3. Some companies observed that patterns 2 and 3 are more efficient than pattern 1 as it may potentially minimize the broadcast overhead in time.

Agreement:

Capture the following observations in the TR (Editorial modifications and changes to references can be made when capturing the observations in the TR):

- It is recommended to further investigate the need for DL and UL PT-RS enhancement for the subcarrier spacings to be supported in specifications. PT-RS enhancements, if needed, can consider the following:

  - support of high MCS values,

  - applicability of ICI compensation techniques,

  - PT-RS sequence,

  - time and frequency resources for PT-RS with OFDM and DFT-s-OFDM waveforms.

- It is recommended to further investigate the need for DL and UL DM-RS enhancements for the subcarrier spacings to be supported in specifications. DM-RS enhancements, if needed, can consider the following:

  - coherence bandwidth and its impact to orthogonal codes used for DM-RS,

  - frequency domain density and overhead,

  - maximum number of DM-RS ports.

- Some companies noted LBT failure may prevent transmission of periodic reference signals, such as P-TRS, and negatively impact performance. Some companies noted deferral of periodic reference signals may be rare and may not significantly impact system performance. Some companies noted aperiodic reference signals could be used to negate the potential impact from LBT failure.

Agreement:

Capture the following observations in the TR (Editorial modifications and changes to references can be made when capturing the observations in the TR):

It is recommended to investigate whether or not enhancements to CSI processing unit (CPU) availability check is needed when the UE is required to process CSI reports corresponding to multiple numerologies across active BWPs in different component carriers.

Agreement:

Capture the following observations in the TR (Editorial modifications and changes to references can be made when capturing the observations in the TR):

It is recommended that both single and multi-carrier operation are supported to support higher data rates.  Larger SCS may achieve larger aggregated bandwidth with multi-carrier operation given a maximum number of CCs.

Agreement:

Capture the following observations in the TR (Editorial modifications and changes to references can be made when capturing the observations in the TR):

- It is recommended to further investigate potential enhancements, if needed, to beam management at least considering one or more of potentially narrower beamwidths, CP duration, multiple beam indications for multi-PUSCH/PDSCH scheduling, triggering of reference signals for beam management, enhancements to beam management for random access procedure, intra- and/or inter-cell mobility, and adaptation to LBT failures.

- Minimum requirement on beam switching delay in > 52.6 GHz spectrum should be further studied by RAN4 when specification is further developed.

Agreement:

Capture the following for the conclusions of the TR:

------------------------------------- Begin ------------------------------------

Study of required changes to NR using existing DL/UL NR waveform to support operation between 52.6 GHz and 71 GHz was conducted. The study included study of applicable numerology including subcarrier spacing, channel BW (including maximum BW), and their impact to FR2 physical layer design to support system functionality considering practical RF impairments, and identification of potential critical problems to physical signal/channels, if any. Study of channel access mechanism, considering potential interference to/from other nodes, assuming beam-based operation, in order to comply with the regulatory requirements applicable to unlicensed spectrum for frequencies between 52.6 GHz and 71 GHz was also conducted.

As an outcome of the study, it is recommended to support 120 kHz subcarrier spacing with normal CP length, and at least one additional subcarrier spacings among 240 kHz, 480 kHz, and 960 kHz subcarrier spacing candidates. It is recommended to consider supporting at most up to three subcarrier spacings including 120 kHz. It is not recommended to consider support of only 240 kHz SCS for PDCCH/PDSCH/PUCCH/PUSCH in addition to 120 kHz. Subcarrier spacing outside 120 kHz to 960 kHz are not supported for any signals and channels. The applicability of the supported subcarrier spacing to particular signals and channels should be further discussed when specifications are developed. It is additionally recommended to limit the maximum FFT size required to 4096 and to limit the maximum of RBs per carrier to 275 RBs. The candidate supported maximum carrier bandwidth(s) for a cell should be between 400 MHz and 2160 MHz. Further investigation of the details of required changes to NR may be needed.

As an outcome of the channel access study, it is recommended to support both channel access with LBT mechanism(s) and a channel access mechanism without LBT for gNB and UE to initiate a channel occupancy. Further investigation of the details of the channel access mechanism may be needed.

---------------------------------------- End --------------------------------------------------

Agreement:

Support of only 240 kHz SCS for PDCCH/PDSCH/PUCCH/PUSCH in addition to 120 kHz should not be considered

### Channel access mechanism

R1-2007550	On channel access modes in 60GHz	FUTUREWEI

R1-2007559	Discussion on channel access for NR beyond 52.6 GHz	Lenovo, Motorola Mobility

R1-2007605	Channel access mechanism for 60 GHz unlicensed operation	Huawei, HiSilicon

R1-2007643	Channel access mechanism for NR on 52.6-71 GHz	Beijing Xiaomi Software Tech

R1-2007653	Discussion on channel access mechanism	vivo

R1-2007791	On Channel access mechanisms	InterDigital, Inc.

R1-2007848	Channel Access Mechanism in support of NR operation in 52.6 to 71 GHz	CATT

R1-2007884	Channel access mechanism	TCL Communication Ltd.

R1-2007918	Channel access mechanisms for NR from 52.6-71GHz	AT&T

R1-2007927	Design of NR channel access mechanisms for 60 GHz unlicensed band	Nokia, Nokia Shanghai Bell

R1-2007942	Channel Access Procedure for NR in 52.6 - 71 GHz	Intel Corporation

R1-2007966	On the channel access mechanism for above 52.6GHz	ZTE, Sanechips

R1-2007983	Channel Access Mechanism	Ericsson

R1-2008046	Considerations on channel access mechanism to support NR above 52.6 GHz	LG Electronics

R1-2008091	Discussion on channel access mechanism for above 52.6GHz	Spreadtrum Communications

R1-2008157	Channel access mechanism for 60 GHz unlicensed spectrum	Samsung

R1-2008251	Discussion on channel access	OPPO

R1-2008354	Channel access mechanism for 60 GHz unlicensed spectrum	Sony

R1-2008458	Views on Channel Access Mechanisms  for Unlicensed Access above 52.6 GHz	Apple

R1-2008494	Discussions on channel access mechanism on supporting NR from 52.6GHz to 71 GHz	CAICT

R1-2008517	On Channel Access Mechanism and Interference Handling for Supporting NR from 52.6 GHz to 71 GHz	Convida Wireless

R1-2008548	Channel Access Mechanism for NR in 60 GHz unlicensed spectrum	NTT DOCOMO, INC.

R1-2008563	Discussion on channel access mechanism	ITRI

R1-2008630	Channel access mechanism for NR in 52p6 to 71GHz band	Qualcomm Incorporated

Revision of R1-2008616

R1-2008717	Discussion on channel access mechanism for 52.6 to 71GHz unlicensed band	Potevio

R1-2008770	Further aspects of channel access mechanisms	Charter Communications

[103-e-NR-52-71-Channel-Access] Email discussion/approval on channel access mechanisms including aspects related to system level simulations until 11/3; address any remaining aspects by 11/11 – Jing (Qualcomm)

Additional checkpoint where agreements can be declared: 11/9

R1-2009344	FL summary for channel access mechanism for 52.6GHz-71GHz band	Moderator (Qualcomm Incorporated)

R1-2009363	FL summary for channel access mechanism for 52.6GHz-71GHz band	Moderator (Qualcomm Incorporated)

R1-2009368	FL summary for channel access mechanism for 52.6GHz-71GHz band	Moderator (Qualcomm Incorporated)

R1-2009408	FL summary for channel access mechanism for 52.6GHz-71GHz band 	Moderator (Qualcomm Incorporated)

R1-2009521	FL summary for channel access mechanism for 52.6GHz-71GHz band	Moderator (Qualcomm Incorporated)

R1-2009572	FL summary for channel access mechanism for 52.6GHz-71GHz band	Moderator (Qualcomm Incorporated)

R1-2009626	FL summary for channel access mechanism for 52.6GHz-71GHz band, ver #7	Moderator (Qualcomm Incorporated)

R1-2009724	FL summary for channel access mechanism for 52.6GHz-71GHz band, ver 09	Moderator (Qualcomm Incorporated)

R1-2009760	FL summary for channel access mechanism for 52.6GHz-71GHz band, ver 10	Moderator (Qualcomm Incorporated)

Agreement:

Capture the following observations in the TR. Editorial modifications and changes to references can be made when capturing the observations in the TR.

The following flavors of channel access schemes have been modeled.

- ‘No-LBT’:  No LBT Dynamic TDD:  NR operation with no restrictions on channel access mechanism.

- ‘TxED-omni’: Tx side ED Based LBT with Omnidirectional Sensing (‘Tx Omni LBT): Baseline LBT with sensing at the transmitter is expected to closely follow the ETSI En 302 567 based medium access procedure

- ‘TxED-Dir’, Tx Side ED Based LBT with Directional Sensing (‘Tx Directional LBT’)

- Rx Assisted LBT Flavors:  Multiple flavors of Rx Assistance have been modelled

  - RxA-1: [20, Ericsson],   Receiver assisted LBT: the LBT procedure is evaluated at the receiver instead of transmitter. The LBT result is assumed to be available instantly at the transmitter without accounting any overhead for exchanging this information between the transmitter and the receiver

  - RxA-2: [4, Huawei/HiSilicon] [40, Huawei/HiSilicon]:  Receiver performs directional LBT but transmitter performs Omni LBT. Further details for RxA-2 are as follows.  When UE is the receiver, UE receives a RTS from the gNB. Then, UE sends a “message B” to the gNB with CCA measurements results (dBm value of the measured interference) upon a successful LBT procedure. The latency from the reception of RTS to the transmission of “message B” is calculated equal to 4 slots for 120 kHz SCS and 22 slots for 960 kHz SCS. This includes the required time at the UE side for CCA. Then, gNB transmits PDSCH to the UE. The PDSCH processing time is calculated as 3 slots for 120 kHz and 13 slots for 960 kHz. A CAT4 LBT is performed at the gNB side before RTS transmission.   When gNB is the receiver, first gNB performs energy measurement at the directions of the UEs that have UL data. Then, gNB selects the UE with the lowest interference level. After, gNB sends PDCCH to schedule PUSCH transmission of that UE. Finally, PUSCH is transmitted after a successful CAT2 LBT. In our simulations, we have considered the preparation time from PDCCH reception to PUSCH transmission equal to 4 slots for 120 kHz SCS and 22 slots for 960 kHz SCS. A processing time for PUSCH at gNB is not modelled. The transmissions are restricted to Rank 1 for DL as well as UL throughout.

  - RxA-3: [4, Huawei/HiSilicon] [40, Huawei/HiSilicon]:  Only Receiver performs directional LBT procedure. The procedure is similar to RxA-2 except that gNB does not perform any LBT before RTS transmission.

  - RxA-4: [6, Vivo]: RTS and CTS type mechanism is deployed after winning contention before transmission. The RTS/CTS type exchange is between serving gNB and the served UEs. The transmitter sends a request, and the receiver feedbacks a confirmation if the request could be successfully decoded. Unlike RTS/CTS mechanism in 802.11ad, both the request and confirmation do not silence any other node. The processing delay for the RTS/CTS is assumed to be zero. There is no LBT before CTS.

  - RxA-5: [36, Qualcomm]: Rx Assistance takes the form of protecting ongoing transmissions by silencing based on sensing at the transmitters and protecting intended transmission by silencing based on sensing at the receiver.  The receiver also assists by sending silencing signals. Omni and directional sensing is applied at all nodes.  In the simulated procedure, the ECCA is performed at the gNB followed by an exchange of request/response transmissions.

- Other LBT Flavors:

  - ‘Dyn-RxA’:  Dynamic [20, Ericsson], Dynamic LBT: a node operates without LBT unless the receiver experiences a failure in reception due to a drop in SINR, which reflects a presence of interferer. Only then, the node switches to LBT. Besides, when the LBT is switched on, the RAL described in section 2.1.4 of R1-2007983 is used

Agreement:

Capture the tables in Section 3.3 of R1-2009626 in the TR with the following modifications:

- Change “DL:UL” to “DL:UL traffic ratio” in tables.

- Add “1:1” in Table 1 for vivo’s results in the “DL:UL traffic ratio” column

- Remove “No backoff” in Qualcomm’s results in Table 1

Agreement:

At least when operating with LBT, MCOT is 5ms, including all the gaps inside.

Note: Discussions related to further reductions in MCOT due to potential definition of CAPC will be handled separately.

Agreement:

Use the CCA check procedure in EN 302 567 (per RAN1 understanding as from RAN1 #102-e) as the baseline for channel access for 60GHz band when LBT is applied. The following can be discussed further during normative work.

- Whether CAPC and contention window adjustment mechanisms are introduced

- Whether ED threshold change is needed, e.g., due to changes in bandwidth, beamforming gain etc.

- Whether contention window range needs to be adjusted

Agreement:

Capture the following observations in the TR (Editorial modifications and changes to references can be made when capturing the observations in the TR):

Comparison of No-LBT (NLBT) and Tx Side ED based Omnidirectional Sensing (TxED-Omni) for Indoor Scenerio A: 6 Companies have compared No-LBT with Tx Side ED based Omni sensing LBT

- Vivo, show tail and median benefits of using TxED-Omni LBT on DL, at high loading. In other cases, including all loads for UL and other loads for DL, TdxED-Omni LBT scheme shows losses. All results are at ED threshold -47.

- Intel shows gains for DL throughput at high loads with TxED-Omni LBT for all antenna configurations when BSs are ceiling mounted, and gains for 5%ile DL throughput at high loads when the BS are not ceiling mounted. Other cases including all UL cases show losses. All results are at ED threshold of -48.

- Ericsson, HW, Nokia, Qualcomm and Samsung show loss for TxED-Omni LBT with an EDT of -47 or -48 dB for all cases.

Agreement:

Capture the following in the TR:

On the LBT bandwidth (bandwidth over which a single contiguous LBT is performed) relative to channel bandwidth (as defined in RAN4), the following alternatives have been discussed. Further down-selection of one or more of these alternatives (if needed) should be further discussed when specifications are developed.

- Alt 1: LBT bandwidth equals channel bandwidth

- Alt 2: LBT bandwidth equals the minimum of channel bandwidth and the transmission bandwidth (number of RBs for a given transmission)

- Alt 3: LBT bandwidth can be wider than channel bandwidth

- Alt 4: LBT bandwidth can be narrower than the channel bandwidth, with multiple LBT subband within a channel

- Alt 5: LBT bandwidth equals with minimum supported channel bandwidth or multiples of the minimum supported channel bandwidth

Agreement:

Capture the following in the TR:

For operation where LBT is not required, it can be further discussed when specifications are developed

- If RAN1 should introduce additional conditions/mechanisms for no-LBT to be used, or leave it for gNB implementation

- When no-LBT mode is used, if RAN1 should introduce additional restrictions, such as DFS needs to be applied, ATPC needs to be applied, long term sensing needs to be applied, certain duty cycle limitation, certain transmit power limitation, MCOT limits, etc, or leave the restriction for gNB implementation

- When no-LBT mode is used, if RAN1 should introduce mechanism for the system to fallback to LBT mode, or leave it for gNB implementation

Agreement:

Capture the following observations in the TR. Editorial modifications and changes to references can be made when capturing the observations in the TR.

The following discussion refers to 5th percentile users as ‘tail’ users and 95th percentile users as ‘upper-tail’ users. Remarks mentioning ‘all users’ are applicable to tail, median and upper tail users at once.

- Comparison of No-LBT with directional LBT (TxED-Dir) for Indoor Scenario A: Vivo,  Huawei, Nokia, Samsung, Intel, Ericsson provided results

- Vivo results show gain for directional LBT ((TxED-Dir) over No-LBT for DL, high load, for tail  , median and upper tail users, and for UL, high load for tail users. For all other cases in this comparison, TxED-Dir underperforms No-LBT. (EDT -47 dBm)

- Nokia, for 100% DL presented low, medium and high load results. For all loads, their results show significant loss for both directional and omni-directional LBT for median and high-end users. Only the tail users may have some benefit from directional LBT (as compared to No-LBT), while omni-LBT provides loss also in this case (EDT -48 dBm).

- Ericsson results show No-LBT outperforms directional LBT with (EDT -47 dBm) and directional LBT with (ED -32 dBm for gNB, ED -41 dBm for UE)

- Samsung results show gain in medium and high loads for directional LBT over No-LBT at (EDT -47 dBm) for all users for DL as well as for UL. At low loads TxED-Dir underperforms No-LBT.

- Intel shows gains for DL throughput at high loads with TxED-Dir LBT for all antenna configurations when BSs are ceiling mounted, and gains for 5%ile DL throughput at high loads when the BS are not ceiling mounted. In other cases, including all loads for UL, TdxED-Dir LBT scheme shows losses. All results are at ED threshold of -48

- Huawei largely shows loss for directional LBT over No-LBT for all loading levels and users, except DL, tail users at high loading where the results are comparable. Huawei’s TxED-Dir uses CW-Max of 127 with EDT of -47 dBm.

Agreement:

Capture the following observations in the TR (Editorial modifications and changes to references can be made when capturing the observations in the TR):

Comparison of Omni LBT (TxED-Omni) with directional LBT (TxED-Dir)  for Indoor Scenario A: Vivo, ZTE, Nokia, Samsung, Intel, Qualcomm, Ericsson, and Huawei, provided results

- For Omni LBT (TxED-Omni) with directional LBT (TxED-Dir) have been done with using the same ED Threshold. Additionally, Ericsson simulated directional LBT with adjusted thresholds (ED -32 dBm for gNB, ED -41 dBm for UE).  Multiple companies have evaluated adjustments to ED Threshold with directional sensing either implicitly or explicitly.

- Vivo results show that omni-directional is better than directional LBT in tail and median performance, and marginal difference in other cases. Both omni-directional and directional LBT use the same ED threshold of -47 dBm

- Samsung shows gain at all loading levels for directional LBT over omni-LBT (-47 dBm) for all users, for DL and UL traffic.

- Intel shows that for UL TxED-Dir LBT provides better performance relative to TxED-Omni for low ED thresholds (i.e., -55 and -65 dBm) but losses for high thresholds (i.e., -48 dBm). As for DL, TxED-Dir LBT provides consistently better performances than TxED-Omni. The gain of directionality increases with more directional UE beams.

- Qualcomm results show largely a comparable performance for omni and directional sensing using equal threshold, with small benefit of directionality under gNBs with narrower beams

- Ericsson results show that directional LBT with adjusted thresholds (ED -32 dBm for gNB, ED -41 dBm for UE) and directional LBT with ED -47 dBm, and omni-directional LBT with ED -47 dBm have comparable performance.

- For 100% DL traffic, Nokia results show that directional LBT TxED-Dir outperforms TxED-Omni at low as well as medium loads – for median, tail as well as upper tail users. The results use EDT -487 dBm

- For 100% DL traffic, ZTE shows gains in directional LBT for tail users and median users at ED thresholds -68 dBm and -62 dBm. The gains are also present in DL+UL Traffic at ED threshold -68 dBm and -62 dBm.

- Coexistence: ZTE shows that an operator using directional LBT benefits in the presence of an operator using Omni LBT, relative to a deployment where both operators use Omni-LBT. The results use ED threshold -68 dBm.

- Huawei’s results show that directional LBT (TxED-Dir) does not outperform Omni LBT (TxED-Omni)

Agreement:

Capture the following observations in the TR (Editorial modifications and changes to references can be made when capturing the observations in the TR):

Comparison of No-LBT with receiver assisted LBT for Indoor Scenario A: Ericsson, Huawei, Vivo, provided results

- Different versions of receiver assistance modelled as presented earlier

- Ericsson results uses omni-sensing at receiver. The results do not show benefit for receiver assistance over No-LBT.

- Vivo’s results use an EDT -47 dBm, in the results, RxA-4-Omni gains in both DL and UL relative to No-LBT for tail users at high loads.  RxA-4-Omni gains in DL but loses in UL relative to No-LBT for medium and high loads at all other user percentiles and mean.

- Huawei’s Receiver-only LBT (RxA-3) shows tail UPT and mean UPT gain compared to No-LBT in low, medium, and high traffic loads with InH Open Office channel model 40] and InH mixed channel model [40] in both UL and DL.

- In comparison with No-LBT, Huawei shows Receiver-assisted LBT (RxA-2) Tail UPT gain in DL with high traffic load for InH open office channel model and loss in other cases. Also, Huawei shows Receiver-assisted LBT Tail UPT gain in DL with low, moderate and high traffic load for InH mixed channel model and loss in other cases.

Agreement:

Capture the following observations in the TR (Editorial modifications and changes to references can be made when capturing the observations in the TR):

Comparison of receiver assisted LBT versions with Omni LBT (Tx-ED-omni), and directional LBT (TxED-dir) for Indoor Scenario A: Huawei, Qualcomm, Vivo and Ericsson provided results

- Ericsson results show similar performance of receiver assisted LBT (RxA-1) and omni- directional LBT (TxED-Omni). Nonetheless, the RxA-1 implementation does not model the overhead of information exchange between the transmitter and receiver. Hence, it is expected that the actual performance of RxA-1 is worse than the simulated one

- Huawei’s both flavors of receiver assistance, Rx-Assisted LBT (RxA-2), and Receiver Only LBT (RxA-3) outperform Tx-ED-Omi and Tx-ED-Dir at all loading levels and users percentiles, with larger benefits to tail users

- Qualcomm results show gains with receiver assisted LBT for DL and UL in the median as well as tail, primarily at higher loading levels. (A)  The results show receiver assisted LBT RxA-5 Omni @EDT -67dBm and RxA-5 Dir@-67dBm 67dBm outperforms TxED-Omni and TxED-Dir as loading level increases.   (B) Qualcomm results show comparable performance of RxA-5 Omni and RxA-5 Dir for the baseline gNB Antenna Configuration. (C) Further, as directionality increases at the gNB with more antenna elements, ( i.e. when  gNB Configuration (Mg,Ng,M,N,P) = (1,1,4,8,2) is replaced with  (Mg,Ng,M,N,P) = (1,1,8,16,2)) the relative benefits of Rx-Assistance are shown to be larger,. (D) Further as silencing Threshold is decreased from -67 to -72 dBm, the relative gains of Rx-Assistance increase. At 2 gHz BW, a silencing threshold of -72dBm is close to noise floor and may not be achieved as ED but may require a sequence detection mechanism.

- Vivo results show gains with receiver assisted LBT RxA-4-Omni relative to TxED-Omni primarily for uplink, at medium and high loads for all users.  For DL, the performance is comparable between RxA-4 Omni and TxED-Omni, except at high load tail, where RxA-4-Omni underperforms.

Agreement:

Capture the following observations in the TR (Editorial modifications and changes to references can be made when capturing the observations in the TR):

For Indoor scenario A:

- Huawei shows Receiver-only LBT (RxA-3) tail UPT and mean UPT gain  compared to receiver-assisted LBT (RxA-2)  in low, medium, and high traffic loads with InH Open Office channel model [40] and InH mixed channel model [40].

- Ericsson’s results in Coexistence scenario with Operator A doing No-LBT and Operator B doing TxED-Omni LBT  at -47 dBm EDT show that the operator B performance does not degrade (i.e. no losses observed) as compared to the case when Operator B coexists with another operator using LBT.

- Ericsson’s results for Dynamic LBT shows that the performance of the network can be improved when the decision to perform LBT is done dynamically per node, as compared to semi-statically operating all nodes with LBT.

Agreement:

Capture the following observations in the TR (Editorial modifications and changes to references can be made when capturing the observations in the TR):

- One Company [Ericsson] submitted results for Indoor Scenario B, which is a smaller indoor scenario with 2 operators and 1 gNB each. Their observations for this case are in line with their observations for Indoor Scenario A.

Agreement:

Capture the following observations in the TR (Editorial modifications and changes to references can be made when capturing the observations in the TR):

- Comparison of No-LBT with omnidirectional LBT (TxED-Omni) for Indoor Scenario C: Ericsson and HW show loss for TxED-Omni LBT, Charter shows roughly comparable performance

- Ericsson’s results show worse performance for TxED-Omni LBT relative to No-LBT for both threshold -47dBm and -68 dBm.  The loss is higher for EDT -68dBm.

- Charter’s low load DL:UL 50:50 results show loss for TxED-Omni LBT  over No-LBT. Their medium load DL:UL 5:2 results show gains in DL tail user and UL median user, loss in UL tail user and comparable performance for other cases.  Their high load results for DL:UL ~2:1, show small tail gain and median loss for DL and comparable performance for UL.

- Huawei’s results show loss for TxED-Omni LBT over No-LBT at -47dBm EDT for gNB and -32dBm EDT for UE.

- Comparison of omnidirectional LBT (TxED-Omni) with directional LBT (TxED-Dir) for Indoor Scenario C:

- In Huawei and Ericsson’s results, for equal ED threshold, Directional sensing, (TxED-Dir) and Omni sensing (Tx-ED-Omni) show comparable results.

- ZTE show gains for directional LBT in median users as well as tail users at -68 dBm ED threshold for 100% DL traffic

- Comparison of Rx-Assistance LBT schemes with others for Indoor scenario C

- Ericsson results show similar performance of Rx Assistance (RxA-1 -Omni) and  TxED-Omni LBT but loss relative to no-LBT at both modelled ED thresholds. There is no benefit of using RxA-1 scheme over TxED-Dir LBT scheme for ED Threshold -47dBm.

- Another form of Rx-Assistance, referred as, Dyn-RxA is shown by Ericsson to provide similar performance as No-LBT for ED Threshold -47 dBm.

- Huawei’s results show consistent loss for receiver assistance scheme RxA-2 compared to No-LBT. RxA-2 is shown to outperform TxED-Omni and TxED-Dir for this scenario.

Agreement:

Capture the following in the TR. Editorial modifications and changes to references can be made when capturing the observations in the TR.

For Outdoor scenario B:

- Ericsson results show loss of TxED-Omni LBT schemes compared to No-LBT, for two ED thresholds (-47 and -68 dBm).  TxED-Omni LBT with ED Threshold of -68 dBm dBm and -47 dBm has similar performance. HW shows loss for LBT schemes with respect to no-LBT for 1-site and 7 -site scenarios. Directional and omni LBT are comparable at -47dBm EDT for gNB and -32dBm EDT for UE.

- Huawei results show loss of TxED Omni LBT scheme compared to No-LBT for ED Threshold -47 dBm. TxED Omni and TxED-Dir are shown to have comparable performance. Receiver assisted LBT (RxA-2) is seen to improve tail performance and to a small extent median user performance at high loading levels compared to TxED-Omni, and in all other cases seen to have comparable performance. RxA-2 simulated underperforms No-LBT in all cases. These trends hold for 7-site as well as 1-site simulations.

Agreement:

It can be further discussed when specifications are developed if and how the ED threshold provided by the ETSI BRAN 302 567 should be modified to account for aspects such as transmit power, LBT bandwidth, beamforming gain, coexistence etc.

- Note: There is no consensus that all of the aspects above need to be considered

Agreement:

When LBT mode is used, it can be further discussed when specifications are developed if a responding device should use a Cat 2 LBT to share the COT, and if yes, how to define the Cat 2 LBT and if a maximum gap is to be introduced between the initiating device and responding device transmissions.

Agreement:

- Support of contention-exempt short control signalling transmission in 60GHz band for regions where LBT is required and short control signaling without LBT is allowed.

  - Note: If regulations do not allow short control signaling exemption in a region when operating with LBT, operation with LBT for these short control signals should be supported

- Restrictions to the transmission, such as, on duty cycle (airtime measured over a relatively long period of time), content, TX power, etc. can be discussed when specifications are developed.

Agreement:

It can be further discussed when specifications are developed if 3GPP specifications should define the relationship between the LBT beam and the transmission beam or leave it as implementation. If such relationship is defined, it can also be further discussed when specifications are developed if ED threshold should be adjusted by the choice of LBT beam and transmission beam.

Agreement:

When LBT mode is used, spatial domain multiplexing of different beams is supported. The LBT requirement (if any) for spatial domain multiplexing of multiple beams can be further discussed when specifications are developed. At least the following can be considered while other LBT considerations are not excluded.

- Leave the LBT behaviour for implementation

- One LBT beam covers all transmission beams

- Multiple LBT beams cover multiple transmission beams

Agreement:

When LBT mode is used, time domain multiplexing of DL/UL transmissions in different beams in the same COT is supported. The LBT requirement (if any) for time domain multiplexing of DL/UL transmissions in multiple beams can be further discussed when specifications are developed. At least the following can be considered while other LBT considerations are not excluded

- No additional LBT requirement defined and leave the LBT behaviour for implementation

- Perform directional or omni-directional LBT at the beginning of COT with sensing beam(s) that covers all TDM beams and with no LBT before each beam switching in the middle of COT.

- Perform directional or omni-directional LBT at the beginning of COT with sensing beam(s) that covers all TDM beams or the first transmission beam, and additional directional LBT with sensing beam that covers the next transmission beam for each beam switching in the middle of COT.

Agreement:

Capture the following in TR:

- The following receiver assisted channel access and interference management schemes have been considered and can be further investigated when specifications are developed

- Class A. Receiver provides assistance information (signalling) to transmitter only.  The following aspects of Class A can be further discussed when specifications are developed

  - Applicability in the following potential channel access modes:

    - LBT is performed prior to transmission

    - No LBT is performed prior to transmission

  - Details of assistance information (e.g., type, timing, content, how the assistance information is obtained etc.)

  - Whether the assistance information can be obtained by LBT performed at the receiver prior to transmission

  - Whether the assistance information can be obtained by existing layer 1 and layer 3 measurements with enhancements if needed

  - If any specification changes are needed to support Class A

- Also, the following receiver assisted channel access schemes have been considered, and considering the system performance and complexity tradeoff, these schemes will not be further investigated in Rel.17

- Class B. Receiver provides assistance information (signalling) to other NR nodes, including non-serving nodes

  - In this case, cross RAT coexistence is based on ED

  - Class B1. Intra-operator only

  - Class B2. Also including inter-operator signalling

    - In this case, cross operator coexistence is based on ED

- Class C. Receiver provides assistance information (signalling) to other NR nodes and nodes from other RAT

Agreement:

Capture observations in Section 3.4.8.4 of R1-2009760 in the TR (Section numbers and other references can be updated when incorporating into the TR)

### Others

R1-2007560	Additional evaluations for NR beyond 52.6GHz	Lenovo, Motorola Mobility

R1-2007654	Evaluation on different numerologies for NR using existing DL/UL NR waveform	vivo

R1-2007792	Evaluation results for above 52.6 GHz	InterDigital, Inc.

R1-2007928	Simulation Results for NR from 52.6 GHz to 71 GHz	Nokia, Nokia Shanghai Bell

Late submission

R1-2007943	Considerations on performance evaluation for NR in 52.6-71GHz	Intel Corporation

R1-2007967	Simulation results for NR above 52.6GHz	ZTE, Sanechips

R1-2007984	Evaluation results for NR in 52.6 - 71 GHz	Ericsson

R1-2008047	Considerations on phase noise compensation to support NR above 52.6 GHz	LG Electronics

R1-2008158	Evaluaton results for extending NR to up to 71 GHz	Samsung

R1-2008252	Discussion on other aspects	OPPO

R1-2008459	Evaluation results for Physical Layer Design for NR above 52.6GHz	Apple

R1-2008549	Potential Enhancements for NR on 52.6 to 71 GHz	NTT DOCOMO, INC.

R1-2008771	Performance evaluations for NR above 52.6 GHz	Charter Communications

R1-2008779	Link level and System level evaluation for NR system operating in 52.6GHz to 71GHz	Huawei, HiSilicon

[103-e-NR-52-71-Evaluations] Email discussion/approval on aspects related to link level evaluations until 11/4; address any remaining aspects by 11/12 – Huaming (Vivo)

Additional checkpoint where agreements can be declared: 11/9

R1-2009111	Summary of link level evaluation results and related issues on supporting NR from 52.6 GHz to 71 GHz	Moderator (Vivo)

R1-2009355	Discussion summary #1 for [103-e-NR-52-71-Evaluations]	Moderator (Vivo)

R1-2009377	Discussion summary #2 for [103-e-NR-52-71-Evaluations]	Moderator (Vivo)

R1-2009392	Discussion summary #3 for [103-e-NR-52-71-Evaluations]	Moderator (vivo)

R1-2009524	Discussion summary #4 for [103-e-NR-52-71-Evaluations]	Moderator (vivo)

R1-2009609	Discussion summary #5 for [103-e-NR-52-71-Evaluations]	Moderator (vivo)

Agreement:

Summary observations #2a in Section 2.1.1.2 of R1-2009609 are agreed to supersede the previously agreed corresponding observations.

Agreement:

Summary observations #2 in Section 2.1.3 of R1-2009609 are agreed to supersede the previously agreed corresponding observations.

Agreement:

Summary observations #2a in Section 2.1.4 of R1-2009609 are agreed to supersede the previously agreed corresponding observations.

Agreement:

Summary observations #2 in Section 2.1.5 of R1-2009609 are agreed to supersede the previously agreed corresponding observations.

Agreement:

Summary observations #2 in Section 2.3 of R1-2009609 are agreed to supersede the previously agreed corresponding observations.

Agreement:

Capture the following observations in the TR (updates to references and other editorial modifications can be made for inclusion in the TR):

7 sources ([61, Ericsson], [26, Qualcomm], [56, vivo], [64, OPPO], [21, Apple], [25, NTT DOCOMO], [12, Intel]) reported evaluation results of PSS/SSS detection performance in terms of SINR in dB achieving cell ID detection probability of 90% by one-shot detection from PSS/SSS. 4 sources ([61, Ericsson], [26, Qualcomm], [56, vivo], [21, Apple]) reported PBCH performance in terms of SINR in dB achieving PBCH BLER target of 10%. 2 sources ([5, vivo], [14, 61, Ericsson]) compared link budget of SSB for different SCS.

- For PSS and SSS detection performance, all evaluated candidate SCSs (120, 240, 480 and 960 kHz) show comparable performances with the non-optional (non-optional to be replaced by references to channel model in Tables to be added when capturing in TR) channel models and delay spread values.

  - The performance degrades as the increase of SCS.

  - Note: The following references are used to derive the observations.

  - 6 out of 7 sources reported minor performance difference (< or ~ 1 dB) between adjacent SCS for all evaluated candidate SCSs (120, 240, 480 and 960 kHz). The other source ([21, Apple]) reported more than 3 dB performance gap of 960 kHz SCS compared to other 120, 240 and 480 kHz SCS. It also reported that the gap of 960 kHz increases as the delay spread increases.

- For PBCH BLER performance, all evaluated candidate SCSs (120, 240, 480 and 960 KHz) show comparable performances with the non-optional (non-optional to be replaced by references to channel model in Tables to be added when capturing in TR) channel models and delay spread.

  - The performance degrades as the increase of SCS.

  - All 4 sources reported minor performance difference (< or ~ 1 dB) between adjacent SCS for all evaluated candidate SCSs (120, 240, 480 and 960 KHz).

  - The performance gap between 120 and 960 kHz is up to ~ 1.8 dB.

- In terms of SSB link budget, smaller SCS have better coverage than larger SCS

  - The MCL and MIL difference between 120 kHz SCS and 480 kHz SCS is about 5 dB. The MCL and MIL difference between 120 kHz SCS and 960 KHz SCS is about 8 dB.

Agreement:

Capture the following observations in the TR (updates to references and other editorial modifications can be made for inclusion in the TR):

8 sources ([61, Ericsson], [68, Huawei], [26, Qualcomm], [56, vivo], [60, ZTE], [64, OPPO], [25, NTT DOCOMO], [12, Intel]) reported evaluation results of PRACH preamble detection performance in terms of SINR in dB achieving PRACH preamble misdetection probability of 1% with evaluation assumptions and parameters as in Table A.1-1 of TR 38.808.  Two sources ([14, 61, Ericsson], [19, OPPO]) compared link budget of PRACH for different SCS.

The following are observed.

- For PRACH preamble detection performances for the same PRACH format, all evaluated candidate SCSs (120, 240, 480 and 960 kHz) show comparable performances

  - Note: The following references were used to derive the observations.

  - 7 out of 8 sources reported minor performance difference (< or ~ 1 dB) between adjacent SCS for all evaluated candidate SCSs (120, 240, 480 and 960 kHz). The other source ([64, OPPO]) reported minor performances difference among all SCS for TDL-A with 5 and 10ns DS. It reported infinite SINR for 960 kHz SCS and comparable SINR for 120, 240 and 480 kHz SCS in TDL-A with 20ns DS using the metrics of preamble miss detection probability of 1% and the estimated timing error is within [-Tcp/2, Tcp/2].

- For PRACH link budget of the same PRACH format and the same sequence length, maximum isotropic loss (MIL) and maximum coupling loss (MCL) degrade as the subcarrier spacing is increased, negatively impacting coverage.

  - Two sources ([14, 61, Ericsson], [19, OPPO]) reported that with UE power limitation of 25 dBm EIRP, the MCL/MIL difference between 120 KHz SCS and 480 KHz SCS is about 4 to 5 dB; the MCL/MIL difference between 120 KHz SCS and 960 KHz SCS is about 8 dB.

  - One source ([14, 61, Ericsson]) reported that without UE power limitation of 25 dBm EIRP (but still under regulatory limits), the MCL difference between 120 kHz SCS and 480 kHz SCS is less than 2.5 dB; the MCL difference between 120 kHz SCS and 960 kHz SCS is less than 1 dB.

  - One source ([14, 61, Ericsson]) reported that without UE power limitation of 25 dBm EIRPs (but still under regulatory limits), compared to short PRACH sequence length, longer PRACH sequence length improve MCL/MIL significantly for 120 kHz SCS due to wider bandwidth for a given SCS.

Agreement:

Capture the following observations in the TR (updates to references and other editorial modifications can be made for inclusion in the TR):

For CP-OFDM, the following are observed regarding the impact of DMRS to BLER performance.

- One source ([57, InterDigital]) reported performance improvement with increased number of DMRS symbols or increased DMRS density especially for higher modulation order for 960 kHz SCS in TDL-A (5 ns and 10 ns delay spread).

- One source ([14, Ericsson]) reported for 480 kHz SCS and below with large delay spread (TDL-A with 40 ns delay spread), the room for performance improvement with a change to the Rel-15 DMRS design is very limited.

- One source ([12, Intel]) reported a performance drop when frequency domain OCC is enabled especially for higher order modulation such as 64 QAM (MCS 22) for 960 kHz SCS in TDL-A (10ns and 20 ns delay spread) and 480 kHz SCS (20 ns delay spread). The performance gap increases when channel delay spread increases.

- One source ([26, Qualcomm]) reported performance improvement with a new DMRS pattern featured by high frequency density (i.e., every RE) and 2-FD-OCC across adjacent REs for 960 kHz SCS in TDL-A (20 ns and 40 ns delay spread)..

- One source ([10, Nokia]) reported that with Rel-15 DMRS type-1, different delay spread values (10ns and 20ns) have a negligible impact to the demodulation performance of PDSCH for a high SCS (such as 960 kHz).

Agreement:

Capture the following observations in the TR (updates to references and other editorial modifications can be made for inclusion in the TR):

7 sources ([61, Ericsson], [68, Huawei], [26, Qualcomm], [56, vivo], [64, OPPO], [10, Nokia], [21, Apple]) evaluated DFT-S-OFDM PUSCH BLER performance with different SCS.

- Compared to CP-OFDM when CPE-only compensation is enabled, DFT-s-OFDM is more robust under phase noise.

- For low and medium MCSs (QPSK and 16QAM), there’s minor performance difference among evaluated SCSs up to 960 kHz.

- With normal CP, for high MCS (64QAM), the performance improves as the increase of SCS, 120 kHz SCS shows up to ~2.0dB loss compared to other larger SCS.

  - Note: the following are references when derive the observations.

  - One source ([61, Ericsson]) reported a performance gap of 1.4~1.8 dB between 120 and 960 kHz SCS

  - One source ([68, Huawei]) reported a performance gap of 1.3~2.5 dB between 120 and 960 kHz SCS

  - One source ([26, Qualcomm]) reported a performance gap of 1.2~1.7 dB between 120 and 960 kHz SCS

  - One source ([56, vivo]) reported a performance gap of ~1.4 dB between 120 and 960 kHz SCS

  - One source ([10, Nokia]) did not report numerical SINR results in table but provided figures showing approximately similar performance difference (~ 2 dB) between 120 and 960 kHz SCS.

  - One source ([21, Apple]) reported a performance gap of more than 7 dB performance gap between 120 kHz SCS and other SCS (240, 480 and 960 kHz) at TDL-A 5 ns DS. It also reported 120 kHz SCS cannot meet the BLER target of 10% at TDL-A 10ns DS and 960 kHz SCS cannot meet the BLER target of 10% at TDL-A 20ns DS.

  - Another source ([64, OPPO]) reported 120 and 240 kHz SCS cannot meet the BLER target of 10% for all evaluated DS values.

- For high MCS (64QAM) at large delay spread (TDL-A 40ns or CDL-B 50ns DS), there’s error floor for 960 KHz SCS at least for BLER target 1%.

  - Note: the following are reference when derive the observations.

  - One source ([26, Qualcomm]) reported an error floor for 960 kHz SCS for BLER target 1%.

  - One source ([56, vivo]) reported an error floor for 960 kHz SCS for BLER target 10%

  - One source ([64, OPPO]) reported no error floor of 960 kHz SCS for the BLER target of 10% and 1% for CDL-B 50ns but an error floor for 960 kHz SCS at TDL-A 20ns for BLER target 1%

Agreement:

Capture the following observations in the TR (updates to references and other editorial modifications can be made for inclusion in the TR):

For CP-OFDM, with evaluation assumptions and parameters as in Table A.1-1 of TR 38.808, the following are observed when CPE-only compensation based on the existing Rel-15 NR PTRS structure is used for normal CP when delay spread is not large. The performance is measured in terms of SINR in dB achieving BLER target of 10% or 1%.

- For low MCS (QPSK) and medium MCS (16QAM), there is minor performance difference between different SCS values up to 960 kHz.

- For high MCS (64QAM), the performance improves in general as the increase of SCS

- For high MCS (64QAM), 13 sources ([61, Ericsson], [68, Huawei], [26, Qualcomm], [56, vivo], [60, ZTE], [64, OPPO], [10, Nokia], [2, 55, Lenovo], [21, Apple], [18, Samsung], [25, NTT DOCOMO], [12, Intel], [7, InterDigital]) compared performance of 120 and 240 kHz SCS in 400 MHz bandwidth

  - for 10% BLER target, there is a performance gap between 120kHz and 240kHz SCS where 240 kHz SCS performs better.

    - Note: the following references are used when derive the observations.

    - One source ([61, Ericsson]) reported better performance of 240 kHz SCS in CDL-D. It also reported both SCS cannot meet 10% BLER target for other evaluated channel model.

    - 3 sources ([68, Huawei], [64, OPPO], [10, Nokia]) reported both SCS cannot meet 10% BLER target

    - 4 sources ([56, vivo], [60, ZTE], [21, Apple], [7, InterDigital]) reported 120 kHz SCS cannot meet 10% BLER target while 240 kHz SCS can

    - One source ([2, 55, Lenovo]) reported better performance of 240 kHz SCS at TDL-A 5 and 10ns. It also reported that both SCS cannot meet 10% BLER target for other evaluated cases.

    - One source ([12, Intel]) reported better performance of 240 kHz SCS in CDL-D. It also reported that both SCS cannot meet 10% BLER target for other evaluated cases.

    - 2 sources ([26, Qualcomm], [18, Samsung]) reported better performance of 240 kHz SCS

    - One source ([25, NTT DOCOMO]) reported comparable performance for both SCS in CDL-D. It also reported better performance of 120 kHz SCS for other evaluated channel model.

- For high MCS (64QAM), 13 sources ([61, Ericsson], [26, Qualcomm], [56, vivo], [60, ZTE], [64, OPPO], [10, Nokia], [2, 55, Lenovo], [21, Apple], [18, Samsung], [25, NTT DOCOMO], [12, Intel], [67, Charter], [7, InterDigital]) compared performance of 240 and 480 kHz SCS in 400 MHz bandwidth

  - for 10% BLER target, there is a performance gap between 240kHz and 480kHz SCS where 480 kHz SCS performs better.

    - Note: the following references are used when derive the observations.

    - One source ([61, Ericsson]) reported better performance for 480 kHz SCS in CDL-D. It also reported 240 kHz SCS cannot meet 10% BLER target for other evaluated channel model.

    - 3 sources ([64, OPPO], [10, Nokia], [67, Charter]) reported 240 kHz SCS cannot meet 10% BLER target while 480 kHz SCS can

    - One source ([2, 55, Lenovo]) reported better performance of 480 kHz SCS at TDL-A 5 and 10ns. It also reported 240 kHz SCS cannot meet 10% BLER target for other evaluated cases.

    - One source ([12, Intel]) reported better performance of 480 kHz SCS in CDL-D. It also reported 240 kHz SCS cannot meet 10% BLER target for other evaluated cases.

    - 6 sources ([26, Qualcomm], [56, vivo], [60, ZTE], [21, Apple], [18, Samsung], [7, InterDigital]) reported better performance of 480 kHz SCS

    - One source ([25, NTT DOCOMO]) reported comparable performance for both SCS in CDL-D. It also reported better performance of 240 kHz SCS for other evaluated channel model.

- For high MCS (64QAM), 14 sources ([61, Ericsson], [68, Huawei], [26, Qualcomm], [56, vivo], [60, ZTE], [64, OPPO], [10, Nokia], [2, 55, Lenovo], [21, Apple], [18, Samsung], [25, NTT DOCOMO], [12, Intel], [67, Charter], [7, InterDigital]) compared performance of 480 and 960 kHz SCS in 400 MHz bandwidth

  - for 10% BLER target, there is a performance gap between 480kHz and 960kHz SCS where 960 KHz SCS performs better.

    - Note: the following references are used when derive the observations.

    - 7 sources ([61, Ericsson], [60, ZTE], [64, OPPO], [10, Nokia], [2, 55, Lenovo], [67, Charter], [7, InterDigital]) reported a greater than 1 dB gain of 960 kHz SCS

    - 3 sources ([26, Qualcomm], [56, vivo], [18, Samsung]) reported a smaller than 1 dB performance gain of 960 kHz SCS

    - One source ([68, Huawei]) reported better performance of 480 kHz SCS for CDL-B 50ns and better performance of 960 kHz SCS for other evaluated cases. In all comparison, the difference is greater than 1 dB.

    - Two sources ([21, Apple], [12, Intel]) reported a better performance of 480 kHz SCS than 960 kHz SCS at 20ns DS in TDL-A where 960 kHz SCS cannot meet 10% BLER target and comparable performance for both SCS in all other evaluated cases

    - One source ([25, NTT DOCOMO]) reported comparable performance for both SCS in CDL-D. It also reported better performance of 480 kHz SCS in TDL-A 5ns and better performance of 960 kHz SCS in CDL-B 20ns.

  - for 1% BLER target, the performance for 960kHz SCS is better than 480kHz SCS.

    - Among sources reported SINR values when both SCS can meet 1% BLER target, the absolute value of the performance gap between 480 kHz and 960 kHz SCS is larger than that for 10% BLER target.

- For high MCS (64QAM), 4 sources ([61, Ericsson], [56, vivo], [10, Nokia], [18, Samsung]) compared performance of 480 and 960 kHz SCS in 1600 or 2000 MHz bandwidth. 4 out of 4 sources reported performance gain around 4 ~ 5 dB of 960 kHz SCS for 10% BLER target. All 4 sources also reported that 480 kHz SCS cannot meet 1% BLER target.

Agreement:

Capture the following observations in the TR (updates to references and other editorial modifications can be made for inclusion in the TR):

For CP-OFDM, with evaluation assumptions and parameters as in Table A.1-1 of TR 38.808 (including optional delay spread value), the following are observed when CPE-only compensation based on the existing Rel-15 NR PTRS structure is used with respect to CP type and large delay spread.

- When delay spread is not large (< 40 ns in TDL-A), there is minor performance difference between normal and extended CP for SCS values up to 960 kHz when compared on the basis of equal MCS (code rate). If comparing on the basis of equal TBS (equal throughput), the performance of ECP is degraded due to higher overhead of ECP.

- Among 11 sources ([61, Ericsson], [68, Huawei], [26, Qualcomm], [56, vivo], [60, ZTE], [64, OPPO], [2, 55, Lenovo], [1, Futurewei], [25, NTT DOCOMO], [12, Intel], [7, InterDigital]) evaluated with large delay spread (i.e. 40 ns in TDL-A and/or 50ns in CDL) based on the existing Rel-15 NR PTRS structure for normal CP, 10 sources observed that for low MCS (QPSK) and medium MCS (16QAM), there is minor performance difference between different SCS values up to 960kHz for 10% BLER target

  - The other source ([1, Futurewei]) evaluated SCS 960 kHz with CPE compensation at MCS16 with normal CP in TDL-A channel with 40ns DS. It reported that the BLER for SCS 960 kHz, MCS16, and Normal CP is not acceptable (cannot meet 10% BLER target) for 40ns DS.

- 10 sources ([61, Ericsson], [68, Huawei], [26, Qualcomm], [56, vivo], [60, ZTE], [64, OPPO], [2, 55, Lenovo],  [25, NTT DOCOMO], [12, Intel], [7, InterDigital]) evaluated large delay spread (i.e. 40 ns in TDL-A and/or 50ns in CDL) with CPE compensation based on the existing Rel-15 NR PTRS structure with normal CP. Among 10 sources, 5 sources ([14, Ericsson], [68, Huawei], [5, 56, vivo], [2, 55, Lenovo], [25, NTT DOCOMO]) also evaluated extended CP at least for 960 kHz SCS with CPE compensation based on the existing Rel-15 NR PTRS structure.

  - 9 out 10 sources observed that for high MCS (64QAM) with normal CP, larger SCS (480 and 960 kHz) performs better than smaller SCS (120 and 240 kHz) when only CPE compensation based on the existing Rel-15 NR PTRS structure is used. The other source ([25, NTT DOCOMO]) reported better performance of smaller SCS.

  - 5 out 5 sources observed the performance of 960 kHz SCS with extended CP is significantly improved compared to with normal CP for large delay spread case when compared on the basis of equal MCS (code rate).

  - 4 sources ([14, Ericsson], [68, Huawei], [5, vivo], [2, 55, Lenovo]) compared throughput of normal CP and extended CP at least for 960 kHz SCS with CPE compensation based on the existing Rel-15 NR PTRS structure. They all reported worse throughput of extended CP.

Agreement:

Capture the following observations in the TR (updates to references and other editorial modifications can be made for inclusion in the TR):

For CP-OFDM, the following are observed with respect to phase noise compensation and PTRS.

- Compared to no phase noise compensation, CPE compensation shows little gain at low and medium MCSs for all the evaluated SCS values; while significant gain is observed for high MCS (64QAM) for all the evaluated SCS values.

  - Two sources ([57, InterDigital], [11, Mitsubishi])) reported that increased PTRS density in frequency domain based on Rel-15 configuration does not provide significant performance benefits.

- For a given SCS, the complexity of ICI compensation increases as the number of ICI filter tap increases

- For MCS 22 evaluation of the same SCS, performance gain of ICI compensation with additional complexity of multi-tap filtering compared to CPE-only compensation is observed when there is sufficient number of PTRS in the frequency domain for 120, 240 and 480 kHz SCS.

  - Note: the following references are used when derive the observations.

  - One source ([61, Ericsson]) showed performance gain of ICI compensation compared to CPE-only compensation for all evaluated SCS

  - One source ([68, Huawei]) evaluated ICI compensation and compared with CPE-only compensation. It reported performance gain for all evaluated SCS.

  - One source ([26, Qualcomm]) compared the performance of CPE and ICI compensation for 120 kHz SCS reported performance gain of ICI compensation.

  - One source ([64, OPPO]) compared the performance of CPE and ICI compensation for all SCS. It reported performance gain of ICI compensation for 240 kHz and 480 kHz SCS. It reported performance gain of ICI compensation in CDL-B but a performance loss in TDL-A for 960 kHz SCS. It also reported that 120 kHz SCS still cannot meet 10% BLER target with ICI compensation.

  - One source ([10, Nokia]) reported performance gain of ICI compensation for 120, 240 and 480 kHz SCS. It also reported performance gain of ICI compensation for 960 kHz SCS at 2GHz bandwidth and a performance loss of ICI compensation for 960 kHz SCS at 400MHz bandwidth.

  - One source ([65, Apple]) evaluated ICI compensation for different SCS with a new PTRS pattern. It reported improvement of ICI compensation compared to CPE-only compensation.

  - One source ([18, Samsung]) evaluated 120 kHz and 240 kHz SCS performance with ICI compensation based on some new PTRS pattern and reported performance improvement.

  - One source ([1, Futurewei]) compared ICI performance among SCS. It reported performance gain of multi-tap ICI filter over CPE compensation for 120, 240 and 480 kHz SCS

  - One source ([12, Intel]) evaluated performance of de-ICI method for MCS 22 with small RB allocations for 240, 480 and 960 KHz SCS. It is observed that the de-ICI method do not work when there isn’t sufficient number of PTRS tones in the frequency domain.

- For MCS 22 with normal CP when delay spread is not large, it is observed that ICI compensation of multi-tap filtering is required for 120, 240 and/or 480 kHz SCS to achieve comparable performance (< 1 dB difference) to that of 960 kHz SCS with CPE-only compensation for 10% BLER target

  - Note: the following references are used when derive the observations.

  - 2 sources ([61, Ericsson], [10, Nokia]) reported comparable performance of 480 kHz SCS with ICI compensation and 960 kHz SCS with CPE compensation in 1600 MHz bandwidth

  - 2 sources ([64, OPPO], [10, Nokia]) reported comparable performance of 480 kHz SCS with ICI compensation and 960 kHz SCS with CPE compensation in 400 MHz bandwidth

  - One source ([68, Huawei]) reported comparable performance of 240 kHz SCS with ICI compensation and 960 kHz SCS with CPE compensation in 400 MHz bandwidth

  - One source ([26, Qualcomm]) evaluated and compared 120 KHz SCS with ICI compensation to larger SCS with CPE compensation. It reported that at MCSs 22 and 24, 120 kHz SCS with ICI compensation performs almost equal to 960 kHz SCS with CPE-only compensation in 400 MHz bandwidth.

  - One source ([1, Futurewei]) reported comparable performance of 480 kHz SCS with ICI compensation and 960 kHz SCS with CPE compensation in TDL-A 5 and 10ns as well as in CDL-D 30ns in 400 MHz bandwidth.

- At very high MCS (e.g., MCS 26 or MCS 28), three sources ([12, Intel], [26, Qualcomm], [69, Huawei]) compared ICI and CPE compensation using the Rel-15 PTRS.

  - Note: the following references are used when derive the observations.

  - One source ([12, Intel]) evaluated the phase noise compensation performance with MCS 28 when delay spread is not large. It is observed that de-ICI technique with 3-taps filter for smaller subcarrier spacing (240 kHz) fails even though there are sufficient number of PTRS tones available for ICI covariance construction.

  - One source ([26, Qualcomm]) compared the performance of CPE and ICI compensation and reported for  MCS 26, 120kHz SCS with ICI compensation suffers from residual ICI and is outperformed by 960kHz SCS with CPE-only compensation when delay spread is not large.

  - One source ([68, Huawei]) showed that for MCS 28, de-ICI technique with large number of taps (11, 9 and 7 taps for 120, 240 and 480 kHz SCS respectively) outperforms 960 kHz with CPE compensation only when delay spread is not large. For normal CP, it also reported that 960 kHz with 3-tap ICI compensation has comparable performance to other SCS with larger number of taps (11, 9 and 7 taps for 120, 240 and 480 kHz SCS respectively) for MCS 28 when delay spread is not large. It also reported that with large delay spread (50ns in CDL), ECP and ICI compensation with at least 3 taps filter are needed for 960 kHz SCS to reach 1% BLER target for MCS 26.

- For high MCS (64QAM) with normal CP when delay spread is large (TDL-A with 40 ns and/or CDL-B with 50ns), 4 sources compared performance of smaller SCS (120, 240 and/or 480 kHz) with ICI compensation to that of 960 kHz SCS with CPE compensation and reported worse performance of 960 kHz SCS with CPE compensation for 10% BLER target.

  - Note: the following are references used when derive the observations.

  - One source ([61, Ericsson]) reported a performance gain of 5 dB in TDL-A 40ns and 0.3 dB in CDL-B 50ns for 480 kHz SCS with ICI compensation compared to 960 kHz SCS with CPE compensation in 1600 MHz bandwidth

  - One source ([68, Huawei]) reported a performance gain of 2.6 dB (for 240 kHz SCS) and 1.6 dB (for 120 kHz SCS) in CDL-B 50ns with ICI compensation compared to 960 kHz SCS with CPE compensation

  - One source ([64, OPPO]) reported a performance gain of 1 dB in CDL-B 50ns for 480 kHz SCS with ICI compensation compared to 960 kHz SCS with CPE compensation. It also reported the performance of 120 kHz with ICI compensation cannot meet the 10% BLER target.

  - One source ([1, Futurewei]) reported the performance of 960 kHz SCS with CPE compensation cannot meet the 10% BLER target. It also reported that the performance of 480 kHz SCS with ICI compensation cannot meet the 10% BLER target in TDL-A 40ns. With ICI compensation, it also reported comparable performance of 120, 240 and 480 kHz SCS in CDL-B 50ns and comparable performance of 120 and 240 kHz SCS in TDL-A 40ns.

- Multiple sources evaluated and compared ICI compensation schemes using the existing Rel-15 NR distributed PTRS structure and/or new PTRS patterns. The results from different sources are not aligned on whether new PTRS patterns perform better than existing Rel-15 PTRS structure when ICI compensation is used.

  - Note: the following are reference used when derive the observations.

  - One source ([11, Mitsubishi]) evaluated with 120 and 240 kHz SCS and reported that the PN compensation with block-based PTRS and cyclic sequence significantly outperforms in spectral efficiency both CPE compensation and de-ICI Wiener filtering with distributed PTRS, even when the density of the scattered pattern is increased above the Rel.15 defined density.

  - One source ([14, Ericsson]) reported that 3-tap direct de-ICI compensation with Rel-15 PTRS outperforms ICI filter approximation approach with clustered PTRS. 3-tap direct de-ICI compensation with a clustered PTRS structure does not offer any performance advantage over the existing Rel-15 NR distributed PTRS structure.

  - One source ([23, MediaTek]) reported that with a 3-tap BLS ICI equalizer, a clustered PTRS structure does not offer any performance advantage over the existing Rel-15 NR distributed PTRS structure.

  - One source ([62, LG]) reported that the performance of clustered PTRS allocation is worse than that of Rel-15 PTRS based ICI compensation scheme and further showed that the performance of subcarrier nulling allocation is similar or superior (up to 2 dB gain especially in the scenarios with low PTRS overhead, K=4) to that of Rel-15 PTRS based ICI compensation scheme.

  - Two sources ([18, Samsung], [65, Apple]) evaluated the performance with some new PTRS patterns (e.g. chunk based PTRS pattern to allow adjacent PTRS symbols in frequency) and reported that the performance with ICI compensation based on new PTRS patterns is better than the Rel-15 pattern with CPE compensation only.

  - One source ([26, Qualcomm]) reported that for the same ICI compensation algorithm, the legacy PTRS pattern outperforms the block PTRS pattern. It showed that for ICI compensation (direct de-ICI filtering) with the legacy PTRS pattern, the performance improves with the increasing number of de-ICI filter taps (3 to 5 taps). It also observed that with a fixed transport block size, the performance improves as the PTRS overhead decreases (the performance loss due to increased effective code rate is more pronounced at higher MCSs) and with a fixed effective code rate, the performance slightly improves as the PTRS overhead increases.

- For high MCS (64QAM) with normal CP, 2 sources ([61, Ericsson], [10, Nokia]) compared performance of 480 and 960 kHz SCS in 1600 MHz bandwidth when ICI compensation is used based on Rel-15 PTRS.

  - When delay spread is not large, both sources reported a smaller than 1 dB performance gain of 960 kHz SCS for both 10% and 1% BLER target in TDL-A. One source ([61, Ericsson]) reported that for CDL-B, there is up to 1.1 dB gain at 1% BLER target for 960 kHz SCS.

  - When delay spread is large (TDL-A with 40 ns DS), one source ([61, Ericsson]) reported 480 kHz SCS performed 3.6 dB better than 960 kHz SCS at 10% BLER target and 960 kHz SCS cannot meet the 1% BLER target.

Agreement:

Capture the following observations in the TR (updates to references and other editorial modifications can be made for inclusion in the TR):

For CP-OFDM, two sources ([14, 61, Ericsson], [68, Huawei]) evaluated PDSCH BLER performance with optional PN models in addition to PN model in Table A.1-1 of TR 38.808. Note that such optional PN models are not confirmed and/or recommended by RAN4 at the time of RAN1#103-e (These observations can be updated if RAN4 input is available).

- When CPE-only compensation is used with an optional PN model at the UE or at BS and UE, it is observed by both sources that there is significantly less dependence of BLER performance on SCS compared to the PN model in Table A.1-1 of TR 38.808. For all test cases, no error floor is observed for smaller SCS with TDL-A or CDL-B/CDL-D for 1% BLER target. There is around 1 to 2 dB performance difference between consecutive SCSs for 1% BLER target.

- However, multiple sources expressed concerns on the validity of such optional PN models given no confirmation and/or recommendation from RAN4. In consequence, there’s a concern on whether and how the observations based on such optional PN models can be used given no RAN4 input on these optional PN models.

Agreement:

- Update BS Antenna Pattern in Table A.2-1 of TR 38.808 as follows.

| BS Antenna Pattern | For outdoor scenarios: - Antenna power pattern given in Table 7.3-1 of TR38.901 (with exception of antenna element gain) For indoor/factory scenarios: - Antenna power pattern given in Table A.2.1-7 of TR38.802 for ceiling mount (with exception of antenna element gain) For factory scenarios: Companies to provide information on the antenna orientation and pattern used. |
| --- | --- |

Agreement:

- Update the indoor A description as follows:

Office box 120m x 50 m, 12 BS per operator, 2 operator, BS height at 3m (ceiling), UE height 1m, x-axis ISD = 20m and y-axis ISD = 25m, where ISD is define by the distance between two adjacent 10m x 10m virtual box, BS randomly deployed within 10m x 10m virtual box,  minimum distance between BS of different operators is 2m.”

[103-e-NR-52-71GHz-Eval_results] Email discussion/approval on collection of simulation results until 11/6 – Huaming/Jing (Vivo/Qualcomm)

R1-2009356	Collection of evaluation results on supporting NR from 52.6 GHz to 71 GHz	Moderator (vivo, Qualcomm)

Agreement:

Section 2 of R1-2009356 is endorsed for inclusion in the TR (formatting and other minor errors can be corrected when including in the TR).

## Enhanced Industrial Internet of Things (IoT) and URLLC

Please refer to RP-201310 for detailed scope of the WI

### Necessity and Support of Physical Layer feedback enhancements

#### UE feedback enhancements for HARQ-ACK

[103-e-NR-IIoT-URLLC-01] Email discussion/approval for UE feedback enhancements for HARQ-ACK – Klaus (Nokia)

- 1st check point: 11/5

- 2nd check point: 11/10

- 3rd check point: 11/12

R1-2007707	HARQ-ACK Enhancements for IIoT/URLLC	Ericsson

R1-2009257	HARQ-ACK enhancement for IOT and URLLC	Qualcomm Incorporated

Agreements: To address the issue of SPS HARQ-ACK dropping for TDD systems, focus on the following two options:

- Option 1: Deferring HARQ-ACK until a next (e.g., first) available PUCCH

  - FFS: Details including the definition of a next (e.g, first) available PUCCH, CB construction / multiplexing

- Option 2: Dynamic triggering of a one-shot / Type-3 CB type of re-transmission

  - FFS: Details on triggering and/or CB construction (incl. potential Type-3 CB optimizations) / multiplexing

Agreements: In the studies on PUCCH carrier switching for HARQ-ACK, PUCCH carrier switching for different cells operated is considered only for cells that are part of the active UL CA configuration.

Agreements: For the studies on SPS HARQ skipping for skipped SPS PDSCH, the further discussions should focus on the following reduced sets methods:

- ‘NACK skipping’ for (skipped) SPS PDSCH (Alt. 1)

  - FFS: details including at least when to skip the HARQ-ACK as well as NACK skipping configuration details (per SPS or group of SPS configurations etc.)

  - Note: this alternative assumes inherently no identification of a skipped SPS PDSCH by the UE

- Dynamic indication of skipped SPS PDSCH occasions (Alt. 3)

  - FFS: details including dynamic indication methods such as e.g. DCI, MAC CE, specific DM-RS instead of SPS DM-RS, …

Agreements: For the studies on SPS HARQ payload size reduction (of non-skipped SPS PDSCH), the further discussions should focus on the following reduced sets of methods:

- ACK skipping (NACK-only) (Alt. 1)

  - FFS: Details

- NACK skipping (ACK-only) (Alt. 2)

  - FFS: Details

- HARQ bundling / compression (Alt. 3)

  - FFS: Details including HARQ bundling / compression window, bundling / compression technique

- HARQ-ACK disabling /skipping for certain SPS configurations (Alt. 4)

  - The skipping / disabling is higher-layer configured per SPS configuration

  - FFS: HARQ-ACK skipping behaviour for Type 1 CB

R1-2007565	UE feedback enhancements for HARQ-ACK	Huawei, HiSilicon

R1-2007655	HARQ-ACK enahncements for Rel-17 URLLC	vivo

R1-2007789	UE feedback enhancements for HARQ-ACK	TCL Communication Ltd.

R1-2007849	UE feedback enhancements for HARQ-ACK	CATT

R1-2007900	HARQ feedback enhancement for URLLC	Beijing Xiaomi Software Tech

R1-2008007	Discussion on UE feedback enhancements for HARQ-ACK	CMCC

R1-2008057	Discussion on UE feedback enhancement for HARQ-ACK	LG Electronics

R1-2008159	HARQ-ACK feedback enhancements for Rel-17 URLLC/IIoT	Samsung

R1-2008279	HARQ-ACK enhancements for Rel-17 URLLC/IIoT	OPPO

R1-2008355	Considerations in HARQ-ACK enhancements for URLLC	Sony

R1-2008460	Discussion on UE feedback enhancements for HARQ-ACK	Apple

R1-2008821	Discussion on HARQ-ACK enhancements for eURLLC	ZTE

R1-2008842	HARQ-ACK Feedback Enhancements for URLLC/IIoT	Nokia, Nokia Shanghai Bell

R1-2008941	UE feedback enhancements for HARQ-ACK	NEC Corporation

R1-2008952	Discussion on UE feedback enhancements for HARQ-ACK	Panasonic Corporation

R1-2008984	Discussion on prioritized UE HARQ feedback enhancements for URLLC/IIoT	Intel Corporation

R1-2009011	UE feedback enhancements for HARQ-ACK	ETRI

R1-2009053	Discussion on UE feedback enhancements for HARQ-ACK	Asia Pacific Telecom co. Ltd

R1-2009063	On UE feedback enhancements for HARQ-ACK	MediaTek Inc.

R1-2009083	HARQ-ACK enhancements for DL SPS 	InterDigital, Inc.

R1-2009101	HARQ-ACK feedback enhancement for IIoT/URLLC	Lenovo, Motorola Mobility

R1-2009133	UE feedback enhancements for HARQ-ACK	Sharp

R1-2009140	UE feedback enhancements for HARQ-ACK	China Unicom

R1-2009148	Discussion on necessity and support of Physical Layer feedback enhancements	Spreadtrum Communications

R1-2009182	Discussion on HARQ-ACK feedback enhancements for Rel.17 URLLC	NTT DOCOMO, INC.

R1-2009246	Discussion on HARQ-ACK enhancement for URLLC/IIoT	WILUS Inc.

#### CSI feedback enhancements

[103-e-NR-IIoT-URLLC-02] Email discussion/approval for CSI feedback enhancements – Moonil (IDC)

- 1st check point: 11/5

- 2nd check point: 11/10

- 3rd check point: 11/12

R1-2008160	CSI feedback enhancements for URLLC	Samsung

Agreements

- No change of CSI processing time relative to Rel-16 CSI in this WI

- CSI processing time specific to a new CSI reporting quantity/type (if supported) can be studied

Agreement:

- For Case-2 new reporting, continue studying with focus on the new reporting type based on PDSCH decoding for OLLA performance enhancement for initial and re-transmissions of PDSCH.

Agreements:

For Case-1 New reporting, the following candidate schemes have been identified to address the fast interference change over time. Continue studying with focus on the identified schemes below for further study and evaluation.

- Scheme 1a: New reporting quantity based on CQI/SINR statistics, e.g.,

  - CQI/SINR statistics (e.g., mean, variance, etc.)

  - CSI prediction

- Scheme 1b: New reporting quantity of interference statistics (e.g., mean, variance, interference covariance matrix, etc.)

- Scheme 1c: New reporting quantity based on modifying existing reporting format, e.g.,

  - CQI reporting considering the worst subbands

  - Subband CQI granularity enhancement

- Scheme 1d: New reporting quantity related to CSI expiration time

- Scheme 1e: New reporting quantity with partial information update, e.g.,

  - CSI reporting with interference update only

Companies are encouraged to investigate the above schemes, aiming for down-selection in RAN1#104-e

Email summary in R1-2009775

R1-2007539	CSI feedback enhancements for URLLC	FUTUREWEI

R1-2007566	CSI feedback enhancements	Huawei, HiSilicon

R1-2007656	CSI feedback enhancements for Rel-17 URLLC	vivo

R1-2007708	CSI Feedback Enhancements for IIoT/URLLC	Ericsson

R1-2007850	CSI feedback enhancements	CATT

R1-2008008	Discussion on CSI feedback enhancements	CMCC

R1-2008058	Discussion on CSI feedback enhancements for URLLC	LG Electronics

R1-2008107	Discussion on CSI feedback enhancements	Spreadtrum Communications

R1-2008280	CSI feedback enhancements for URLLC	OPPO

R1-2008356	Considerations in CSI feedback enhancements	Sony

R1-2008461	Discussion on CSI feedback enhancements for URLLC/IIoT	Apple

R1-2008495	CSI Feedback Enhancements for IIoT/URLLC	III

R1-2008822	Discussion on CSI feedback enhancements for eURLLC	ZTE

R1-2008847	CSI feedback enhancement	NEC

R1-2008862	CSI feedback enhancements for URLLC/IIoT use cases	Nokia, Nokia Shanghai Bell

R1-2008936	CSI feedback enhancements for enhanced URLLC/IIoT	InterDigital, Inc.

R1-2008953	Discussion on CSI feedback enhancements	Panasonic Corporation

R1-2008985	Discussion on prioritized CSI feedback enhancements for URLLC/IIoT	Intel Corporation

R1-2009064	CSI feedback enhancements for URLLC	MediaTek Inc.

R1-2009102	CSI feedback enhancements for URLLC/IIoT	Lenovo, Motorola Mobility

R1-2009134	CSI feedback enhancements for eURLLC	Sharp, NICT

R1-2009183	Discussion on CSI feedback enhancements for Rel.17 URLLC	NTT DOCOMO, INC.

R1-2009258	CSI enhancement for IOT and URLLC	Qualcomm Incorporated

### Enhancements for unlicensed band URLLC/IIoT

R1-2008357	Considerations in unlicensed URLLC	Sony

[103-e-NR-IIoT-URLLC-03] Email discussion/approval for enhancements for unlicensed band URLLC/IIoT– Sorour (Ericsson)

- 1st check point: 11/5

- 2nd check point: 11/10

- 3rd check point: 11/12

R1-2009492	Summary#1 on Enhancements for URLLC/IIoT on Unlicensed Band	Moderator (Ericsson)

R1-2009601	Summary#2 on Enhancements for URLLC/IIoT on Unlicensed Band	Moderator (Ericsson)

R1-2009710	Summary#3 on Enhancements for URLLC/IIoT on Unlicensed Band	Moderator (Ericsson)

R1-2009781	Summary#4 on Enhancements for URLLC/IIoT on Unlicensed Band	Moderator (Ericsson)

Agreements:

- In semi-static channel access mode, a single FFP (periodicity and offset) is associated to an initiating device (gNB or UE) at a given time which can be used for the purpose of channel occupancy. The FFP configuration that is used for initiating channel occupancy purposes, is such that it shall not be changed for at least 200ms

Conclusion:

- For operation on unlicensed channels and irrespective of the adopted LBT mechanism (LBE or FBE), all transmissions in DL and UL are controlled by gNB similarly to licensed channels, and potential collisions or blocking are controlled/mitigated by gNB.

Agreements:

  - UE-to-gNB COT sharing in semi-static channel access mode with a gap > 16us is supported

Conclusion:

If a device X at a given time is initiating a COT, the applicable FFP for the device X is the FFP associated with X.

If a device X at a given time is sharing a COT initiated by a device Y, the applicable FFP for the device X  is the FFP associated with Y.

Note 1: One of the devices X and Y is a UE and the other is its serving gNB.

Note 2: Whether or not there is additional restriction on idle period is still FFS.

Agreements:

Down-select one of the following options (target RAN1#104-e):

- Option 1: Both “CG-UCI based procedures” and “CG-DFI based procedures” are enabled or disabled for unlicensed using one RRC parameter i.e. cg-RetransmissionTimer-r16.

- Option 2-a: “CG-UCI based procedures” and “CG-DFI based procedures” are independently enabled or disabled for unlicensed using respective RRC parameter, i.e. new parameter X and cg-RetransmissionTimer-r16, respectively.

- Option 2-b: “CG-UCI based procedures” and “CG-DFI based procedures” are independently enabled or disabled for unlicensed using respective RRC parameter, i.e. new parameter X and new parameter Y, respectively, where X and Y are different from cg-RetransmissionTimer-r16.

- Option 3: CG-UCI based procedures are supported for unlicensed. CG-DFI based procedures are enabled or disabled for unlicensed using one RRC parameter i.e. cg-RetransmissionTimer-r16

- Note: Procedures based on CG-UCI rely on UE including CG-UCI in CG PUSCH at least as in Rel-16 where the values of the respective fields of CG-UCI are decided by UE.

- Note: Procedures based on CG-DFI rely on automatic re-transmission on CG configuration and reception of CG downlink feedback information (DFI) in DCI for re-transmissions.

Update on 11/11

Agreements:

- The gNB configures a UE to initiate semi-static CO in an unlicensed channel(s) only if the gNB configures the UE also with the higher layer parameters of the gNB’s initiating semi-static CO in the same channel(s).

  - Note: UE initiated FBE configuration is configured per serving cell

Agreements:

In semi-static channel access mode,  FFP Period for UE-initiated COT is separately provided from FFP period for gNB-initiated COT.

o    Note: Any value for the period, shall be at least 1ms and at most 10ms.

o    Note: Aim for low complexity operation to handle gNB and UE COT interactions

Agreements:

In semi-static channel access mode, a UE should be able to determine whether a scheduled UL transmission should be transmitted according to shared gNB COT or UE-initiated COT.

- UE determines the initiator of a COT based on at least one of the following alternatives:

  - Alt 1: Introduce additional bit field in the scheduling DCI

  - Alt 2: Based on ChannelAccess-CPext field in DCI

  - Alt. 3: Based on a predetermined rule(s)

  - Alt. 4: Based on RRC signalling

  - Alt. 5: Based on MAC CE

  - FFS other alternatives

- FFS on overriding possibility and/or the assumption

- Note: A scheduled UL transmission cannot be transmitted according to both shared gNB COT and UE-initiated COT.

Agreements:

In semi-static channel access mode:

- When a configured UL transmission is aligned with a UE FFP boundary and ends before the idle period of that UE FFP associated to the UE, down-select one of the following:

  - Alt-a: If the transmission is confined within a gNB FFP before the idle period of that gNB FFP, and the UE has already determined that gNB is initiated that gNB FFP, UE assumes that the configured UL transmission corresponds to gNB-initiated COT. Otherwise, UE assumes that the configured UL transmission corresponds to UE-initiated COT

  - Alt-b: The UE assumes that the configured UL transmission corresponds to UE-initiated COT.

  - Alt-c: The UE assumption on whether the configured UL transmission is allowed to correspond to UE-initiated COT is based on gNB configuration.

- When a configured UL transmission starts after a UE FFP boundary and ends before the idle period of that UE FFP associated to the UE:

  - If the UE has already initiated the UE FFP, then UE assumes that the configured UL transmission corresponds to UE-initiated COT

  - Otherwise, If the transmission is confined within a gNB FFP before the idle period of that gNB FFP, and if the UE has already determined that gNB has initiated that gNB FFP, then UE assumes that the configured UL transmission corresponds to gNB-initiated COT.

- FFS on other conditions for determining the corresponding UE or gNB initiated COT

- Note: A configured UL transmission cannot be transmitted according to both shared gNB COT and UE-initiated COT.

R1-2007551	UE initiated COT for FFP	FUTUREWEI

R1-2007568	Uplink enhancements for URLLC in unlicensed controlled environments	Huawei, HiSilicon

R1-2007657	Enhancements for unlicensed band URLLC/IIoT	vivo

R1-2007709	Enhancements for IIoT/URLLC on Unlicensed Band	Ericsson

R1-2007851	Enhancements for unlicensed band URLLC/IIoT	CATT

R1-2007885	Enhancements for unlicensed band URLLC/IIoT	TCL Communication Ltd.

R1-2007902	Enhancement for unlicensed band URLLC IIoT	Beijing Xiaomi Software Tech

R1-2008059	Discussion on unlicensed band URLLC/IIOT	LG Electronics

R1-2008108	Discussion on enhancements for unlicensed band URLLCIIoT	Spreadtrum Communications

R1-2008161	Enhancements for unlicensed band URLLC/IIoT	Samsung

R1-2008281	Enhancements for unlicensed band URLLC/IIoT	OPPO

R1-2008462	URLLC uplink enhancements for unlicensed spectrum	Apple

R1-2008568	UL enhancements for IIoT/URLLC in unlicensed controlled environment	Nokia, Nokia Shanghai Bell

R1-2008823	Discussion on unlicensed band URLLC/IIoT	ZTE

R1-2008834	Unlicensed aspects for IIoT	Charter Communications

R1-2008891	Enhancements for unlicensed band URLLC/IIoT	NEC

R1-2008954	Enhancements for unlicensed band URLLC/IIoT	Panasonic Corporation

R1-2008986	Enhancements to Enable URLLC/IIoT in Unlicensed Band	Intel Corporation

R1-2009012	Enhancements for unlicensed band URLLC/IIoT	ETRI

R1-2009065	On the enhancements for unlicensed band URLLC/IIoT	MediaTek Inc.

R1-2009084	Enhancements for unlicensed band URLLC/IIoT	InterDigital, Inc.

R1-2009103	Enhancements for unlicensed band URLLC/IIoT	Lenovo, Motorola Mobility

R1-2009135	Enhancements for unlicensed band URLLC/IIoT	Sharp

R1-2009184	Discussion on enhancements for unlicensed band URLLC	NTT DOCOMO, INC.

R1-2009247	Discussion on enhancement for unlicensed URLLC/IIoT	WILUS Inc.

R1-2009259	Uplink enhancements for URLLC in unlicensed controlled environments	Qualcomm Incorporated

### Intra-UE Multiplexing/Prioritization

R1-2009045	Summary#1 on Intra-UE Multiplexing/Prioritization for R17	OPPO

R1-2009066	Methods for intra-UE multiplexing and prioritization	MediaTek Inc.

[103-e-NR-IIoT-URLLC-04] Email discussion/approval for intra-UE multiplexing/prioritization– Jia (OPPO)

- 1st check point: 11/5

- 2nd check point: 11/10

- 3rd check point: 11/12

Agreements:

For multiplexing UCIs of different priorities in a PUCCH in R17,

- Support of multiplexing between different resources not confined within a sub-slot if conditions are met

  - FFS: Details

- Support multiplexing in case a PUCCH overlaps with more than one PUCCH if conditions are met

  - FFS details

Agreements:

For multiplexing a high-priority (HP) HARQ-ACK and a low-priority (LP) HARQ-ACK into a PUCCH in R17, when the total number of LP and HP HARQ-ACK bits are more than 2 bits, down-select from the following options in RAN1#104-e:

l Option 1: Support joint coding.

l Option 2: Support separate coding.

l Option 3: Combination of Option1 and 2.

l FFS the details

For multiplexing a high-priority (HP) HARQ-ACK and a low-priority (LP) HARQ-ACK into a PUCCH in R17, when the total number of LP and HP HARQ-ACK bits is 2 bits, provide design details for decision for the following cases in RAN1#104-e:

·        Multiplexing on a PUCCH format 0

·        Multiplexing on a PUCCH format 1

Agreements:

For multiplexing a high-priority (HP) HARQ-ACK and a low-priority (LP) HARQ-ACK into a PUCCH in R17, support a mechanism for gNB to enable/disable the multiplexing.

- FFS the type of the mechanism, e.g. DCI indication and/or RRC configuration

- FFS: Interaction between the enable/disable mechanism and other multiplexing conditions

- FFS for other types of UCI.

Agreements:

For HARQ-ACK multiplexing on PUSCH of different priority in R17, support a mechanism for gNB to enable/disable the multiplexing.

- FFS the type of the mechanism, e.g. DCI indication and/or RRC configuration, beta_offset=0

- FFS: Interaction between the enable/disable mechanism and other multiplexing conditions

- FFS for other types of UCI.

Agreements:

Support PHY prioritization of overlapping high-priority dynamic grant PUSCH and low-priority configured grant PUSCH on a BWP of a serving cell in R17.

- FFS the related cancelation behavior for the PUSCH of lower PHY priority and other details.

  - First clarify what is the scope of this feature, e.g. if overlapping between more than 2 channels is considered.

- FFS the timeline requirements.

  - First clarify what is the behavior of Rel-16 UE in case of DG/CG/UCI overlapping, with and without uplink skipping enabled.

- FFS UE capability for this feature.

- Note: The main bullet has been agreed in the WID by RAN Plenary.

R1-2007567	Intra-UE multiplexing enhancements	Huawei, HiSilicon

R1-2007658	Intra-UE Multiplexing/Prioritization for Rel-17 URLLC	vivo

R1-2007710	Intra-UE Multiplexing/Prioritization Enhancements for IIoT/URLLC	Ericsson

R1-2007852	Intra-UE multiplexing and prioritization	CATT

R1-2007901	Intra-UE multiplexing prioritization	Beijing Xiaomi Software Tech

R1-2008009	Discussion on intra-UE multiplexing/prioritization	CMCC

R1-2008060	Discussion on Intra-UE multiplexing/prioritization	LG Electronics

R1-2008162	Uplink intra-UE multiplexing and prioritization	Samsung

R1-2008282	Enhancements on intra-UE multiplexing/prioritization	OPPO

R1-2008358	Considerations in intra-UE UL multiplexing	Sony

R1-2008463	Discussion on Intra-UE Multiplexing/Prioritization	Apple

R1-2008824	Discussion on enhanced intra-UE multiplexing	ZTE

R1-2008843	On UL intra-UE prioritization and multiplexing enhancements	Nokia, Nokia Shanghai Bell

R1-2008848	Discussion on Intra-UE prioritization and multiplexing	NEC

R1-2008937	Intra-UE multiplexing and prioritization	InterDigital, Inc.

R1-2008955	Discussion on Intra-UE multiplexing and prioritization of different priority	Panasonic Corporation

R1-2008987	On Intra-UE Multiplexing and Prioritization for Release 17 URLLC/IIoT	Intel Corporation

R1-2009013	Intra-UE Multiplexing/Prioritization	ETRI

R1-2009104	Intra-UE multiplexing enhancement for IIoT/URLLC	Lenovo, Motorola Mobility

R1-2009136	Enhancements on intra-UE UCI multiplexing and PUSCH prioritization	Sharp

R1-2009149	Discussion on intra-UE multiplexing/prioritization	Spreadtrum Communications

R1-2009185	Discussion on intra-UE multiplexing/prioritization for Rel.17 URLLC	NTT DOCOMO, INC.

R1-2009214	Discussion on intra-UE multiplexing	ITRI

R1-2009248	Discussion on Intra-UE multiplexing/prioritization for URLLC/IIoT	WILUS Inc.

R1-2009260	Intra-UE multiplexing and prioritization for IOT and URLLC	Qualcomm Incorporated

### Others

Including any RAN1 involvement in propagation delay compensation enhancements (including mobility issues, if any).

[103-e-NR-IIoT-URLLC-05] Email discussion/approval for 8.3.4 – Chengyan (Huawei)

- 1st check point: 11/5

- 2nd check point: 11/10

- 3rd check point: 11/12

Update on 11/5:

Agreements:

- Take 65 ns as the assumption of transmit timing error for evaluation of the overall time synchronization error for control-to-control.

- Asymmetry between downlink and uplink channel for smart grid scenario is not considered.

- TA adjustment accuracy is not considered for the evaluation of time synchronization error.

- errorBS,DL,TX is included in the equation for calculating the overall time synchronization error.

Agreements:

TA adjustment accuracy is not considered for the evaluation of time synchronization error.

Agreements:

For evaluation of the overall time synchronization error for smart grid, companies can take one of the following two options as the assumption for BS transmit timing error:

- Option 1: 200 ns

- Option 2: 65 ns

R1-2008318	Enhancements for support of time synchronization	Huawei, HiSilicon

R1-2007659	Discussion on propagation delay compensation enhancements	vivo

R1-2007711	Propagation Delay Compensation Enhancements for Time Synchronization	Ericsson

R1-2007853	Discussion on propagation delay compensation enhancements	CATT

R1-2008061	Discussion on propagation delay compensation enhancements	LG Electronics

R1-2008163	Discussion for propagation delay compensation enhancements	Samsung

R1-2008283	Enhancements for Propagation Delay Compensation	OPPO

R1-2008464	Discussion on Orphan symbol handling for unlicensed spectrum	Apple

R1-2008825	Discussion on propagation delay compensation enhancements	ZTE

R1-2008844	Discussion on enhancements for propagation delay compensation	Nokia, Nokia Shanghai Bell

R1-2008988	On propagation delay compensation for enhanced timing synchronization	Intel Corporation

R1-2009014	Processing time for COT sharing in FBE	ETRI

R1-2009261	Enhancements for support of time synchronization for enhanced IIoT and URLLC	Qualcomm Incorporated

## Solutions for NR to support non-terrestrial networks (NTN)

Please refer to RP-201256 for detailed scope of the WI

R1-2009051	NR_NTN_solutions work plan	THALES

R1-2009061	Support of smart phones in NTN 	THALES

### Timing relationship enhancements

R1-2009057	Timing relationship enhancements in NTN	Asia Pacific Telecom co. Ltd

R1-2007569	Discussion on timing relationship enhancements for NTN	Huawei, HiSilicon

R1-2007660	Discussion on timing relationship enhancements for NR-NTN	vivo

R1-2007854	Timing relationship discussion for NTN	CATT

R1-2007991	Discussion on timing relationship enhancements for NR NTN	China Telecom

R1-2008010	Discussion on timing relationship enhancements for NTN	CMCC

R1-2008164	Timing relationship enhancements for NTN	Samsung

R1-2008253	Discusson on timing relationship enhancement	OPPO

R1-2008359	Calculation of timing relationship offsets	Sony

R1-2008410	Discussions on timing relationship enhancements in NTN	LG Electronics

R1-2008465	Timing Relationship Enhancement in NTN	Apple

R1-2008722	Discussion on timing relationship enhancements for NTN	Fraunhofer IIS, Fraunhofer HHI

R1-2008808	Timing relationship enhancements for NR-NTN	MediaTek Inc., Eutelsat

R1-2008850	Discussion on timing relationship for NTN	ZTE

R1-2008922	Discussion on NTN timing relationship	Lenovo, Motorola Mobility

R1-2008989	On timing relationship enhancements for NTN	Intel Corporation

R1-2009015	Discussion on timing relationship enhancement for NTN	ETRI

R1-2009032	Discussion on the timing relationship enhancement for NTN	Xiaomi

R1-2009049	Timing relationship enhancement for NTN	Panasonic Corporation

R1-2009076	Timing relationship enhancements to support NTN	CAICT

R1-2009091	On timing relationship enhancements for NTN	Ericsson

R1-2009116	On timing relationship for NTN	InterDigital, Inc.

R1-2009152	Consideration on timing relationship enhancements	Spreadtrum Communications

R1-2009186	Discussion on timing relationship enhancements for NTN	NTT DOCOMO, INC.

R1-2009242	Views on DL-UL timing relationship for NTN operation	Nokia, Nokia Shanghai Bell

R1-2009262	Enhancements on Timing Relationship for NTN	Qualcomm Incorporated

[103-e-NR-NTN-Timing-Rel-Enh] Email discussion/approval on timing relationship enhancements with checkpoints for agreements on 11/5, 11/10, 11/12 – Xingqin (Ericsson)

R1-2009373	Feature lead summary#1 on timing relationship enhancements	Moderator (Ericsson)

R1-2009498	Feature lead summary#2 on timing relationship enhancements	Moderator (Ericsson)

R1-2009603	Feature lead summary#3 on timing relationship enhancements	Moderator (Ericsson)

R1-2009733	Feature lead summary#4 on timing relationship enhancements	Moderator (Ericsson)

Agreement:

Introduce K_offset (may or may not be the same as the K_offset value in other timing relationships) to enhance the timing relationship of HARQ-ACK on PUCCH to MsgB.

Agreement:

- For K_offset configured in system information and used in initial access, at least a cell specific K_offset configuration, which is used in all beams of a cell, should be supported.

- FFS: Beam specific K_offset configured in system information and used in initial access.

Working Assumption:

K_offset can be applied to indicate the first transmission opportunity of PUSCH in Configured Grant Type 2 in the same way as K_offset is applied to the transmission timing of DCI scheduled PUSCH.

Conclusion:

The agreement made at RAN1#102-e about introducing K_offset in the transmission timing of RAR grant scheduled PUSCH is also applicable to fallbackRAR scheduled PUSCH.

Agreement:

Denote by K_mac a scheduling offset other than K_offset:

- If downlink and uplink frame timing are aligned at gNB:

  - For UE action and assumption on downlink configuration indicated by a MAC-CE command in PDSCH, K_mac is not needed.

  - For UE action and assumption on uplink configuration indicated by a MAC-CE command in PDSCH, K_mac is not needed.

- If downlink and uplink frame timing are not aligned at gNB:

  - For UE action and assumption on downlink configuration indicated by a MAC-CE command in PDSCH, K_mac is needed.

  - For UE action and assumption on uplink configuration indicated by a MAC-CE command in PDSCH, K_mac is not needed.

- Note: This does not preclude identifying exceptional MAC CE timing relationship(s) that may or may not require K_mac.

### Enhancements on UL time and frequency synchronization

R1-2008809	UL Time and Frequency Synchronisation for NR-NTN	MediaTek Inc., Eutelsat

R1-2007570	Discussion on UL time and frequency synchronization enhancement for NTN	Huawei, HiSilicon

R1-2007661	Discussion on UL time and frequency synchronization  enhancements for NR-NTN	vivo

R1-2007855	UL time and frequency compensation for NTN	CATT

R1-2008011	Enhancements on uplink timing advance for NTN	CMCC

R1-2008165	Enhancements on UL time and frequency synchronization for NTN	Samsung

R1-2008254	Discussion on UL time and frequency synchronization	OPPO

R1-2008360	Enhancement for UL time synchronization	Sony

R1-2008411	Discussions on UL time and frequency synchronization enhancements in NTN	LG Electronics

R1-2008466	Uplink Time and Frequency Synchronization for NTN	Apple

R1-2008851	Discussion on UL synchronization for NTN	ZTE

R1-2008867	Satellite Position Accuracy	Eutelsat S.A.

R1-2008923	Discussion on NTN TA indication	Lenovo, Motorola Mobility

R1-2008990	On UL time and frequency synchronization for NTN	Intel Corporation

R1-2009016	Discussion on UL timing synchronization for NTN	ETRI

R1-2009033	Discussion on UL time and frequency synchronization for NTN	Xiaomi

R1-2009058	UL time and frequency synchronization in NTN	Asia Pacific Telecom co. Ltd

R1-2009068	Satellite Position Accuracy	Eutelsat S.A.

Withdrawn

R1-2009075	Discussion on UL time synchronization acquisition	Mitsubishi Electric RCE

R1-2009077	Considerations on Enhancements on UL Time Synchronization in NTN	CAICT

R1-2009092	On UL time and frequency synchronization enhancements for NTN	Ericsson

R1-2009097	NTN UL time frequency	PANASONIC R&D Center Germany

R1-2009117	On UL time/frequency synchronization for NTN	InterDigital, Inc.

R1-2009153	Consideration on enhancements on UL time and frequency synchronization	Spreadtrum Communications

R1-2009243	Discussion on time and frequency synchronization for NTN systems	Nokia, Nokia Shanghai Bell

R1-2009263	UL time and frequency synchronization for NTN	Qualcomm Incorporated

R1-2009292	UL time synchronization for NTN systems	CEWiT

R1-2009298	Considerations on UL timing and frequency synchronization	THALES

[103-e-NR-NTN-UL-Time-Freq-Sync-Enh] Email discussion/approval on UL time and frequency synchronization with checkpoints for agreements on 11/5, 11/10, 11/12 – Baptiste (Thales)

R1-2009556	FL Summary on enhancements on UL time and frequency synchronization for NR NTN	Moderator (Thales)

R1-2009697	FL Summary on enhancements on UL time and frequency synchronization for NR NTN	Moderator (Thales)

Agreement:

An NTN UE in RRC_IDLE and RRC_INACTIVE states is required to at least support UE specific TA calculation based at least on its GNSS-acquired position and the serving satellite ephemeris.

Agreement:

An NR NTN UE in RRC_IDLE and RRC_INACTIVE states shall be capable of at least using its acquired GNSS position and satellite ephemeris to calculate frequency pre-compensation to counter shift the Doppler experienced on the service link.

Agreement:

- In NTN, the network may broadcast

- A common timing offset value

  - FFS details of the common timing offset

- FFS: A common timing drift rate

- Before Msg1/MsgA transmission, the NR NTN UE in idle/inactive mode calculates its TA as follows:

Where:

is derived from the User specific TA self-estimation

is derived at least from the common timing offset value if broadcasted by the network. The granularity of  and whether  is indicated as a Timing Advance or as a Timing Offset value [unit] are FFS. Upon resolving the FFS, one of the X in the equation will be removed.

- depends on band and LTE/NR coexistence and is specified in TS 38.213 section 4.2.

- is specified in TS 38.211 section 4.1.

- Note: UE will not assume that the RTT between UE and gNB is equal to the calculated TA for Msg1/Msg A.

Working assumption:

It is assumed that the requirement on UL time pre-compensation for Msg1/MsgA transmission of an NR NTN UE in idle/inactive mode will be defined such that the existing TAC 12-bit field in msg2 (or msgB) can be reused without any extension.

Agreement:

An NR NTN UE in RRC_CONNECTED states shall be capable of at least using its acquired GNSS position and satellite ephemeris to perform frequency pre-compensation to counter shift the Doppler experienced on the service link.

### Enhancements on HARQ

R1-2007856	HARQ operation enhancement for NTN	CATT

R1-2007571	Discussion on HARQ enhancement for NTN	Huawei, HiSilicon

R1-2007662	Discussion on HARQ enhancements for NR-NTN	vivo

R1-2008012	Enhancements on HARQ for NTN	CMCC

R1-2008166	Enhancements on HARQ for NTN	Samsung

R1-2008255	Discussion on HARQ enhancement	OPPO

R1-2008256	Discussion on other aspects	OPPO

R1-2008361	Enhancements on HARQ for NTN	Sony

R1-2008412	Discussions on HARQ enhancements in NTN	LG Electronics

R1-2008467	HARQ Enhancements for NTN	Apple

R1-2008802	INTELLIGENT PACKET REPETITION IN MOBILE SATELLITE SERVICE (MSS) LINKS TO  OVERCOME CHANNEL BLOCKAGES	Ligado Networks

R1-2008810	HARQ in NR-NTN	MediaTek Inc.

R1-2008852	Discussion on HARQ for NTN	ZTE

R1-2008881	Discussion on HARQ for NTN	Nomor Research GmbH, Thales

R1-2008924	Enhancements on HARQ for NTN	Lenovo, Motorola Mobility

R1-2008991	On HARQ enhancements for NTN	Intel Corporation

R1-2009017	Discussion on HARQ Enhancements for NTN	ETRI

R1-2009034	Discussion on the HARQ enhancement for NTN	Xiaomi

R1-2009050	HARQ enhancement for NTN	Panasonic Corporation

R1-2009059	Enhancements on HARQ in NTN	Asia Pacific Telecom co. Ltd

R1-2009078	HARQ enhancements to support NTN	CAICT

R1-2009093	On HARQ enhancements for NTN	Ericsson

R1-2009118	On HARQ enhancement for NTN	InterDigital, Inc.

R1-2009154	Consideration on enhancements on HARQ	Spreadtrum Communications

R1-2009244	Further discussion on HARQ operation for NTN systems	Nokia, Nokia Shanghai Bell

R1-2009264	Enhancements on HARQ for NTN	Qualcomm Incorporated

[103-e-NR-NTN-HARQ-Enh] Email discussion/approval on HARQ enhancements with checkpoints for agreements on 11/5, 11/10, 11/12 – Nan (ZTE)

R1-2009420	Summary of AI 8.4.3 for HARQ for NTN	Moderator (ZTE)

R1-2009657	Summary #2 of AI 8.4.3 for HARQ for NTN	Moderator (ZTE)

Agreement:

For a DL HARQ process with disabled HARQ feedback, the UE is not expected to receive another PDSCH or set of slot-aggregated PDSCH scheduled for the given HARQ process that starts until [X] after the end of the reception of the last PDSCH or slot-aggregated PDSCH for that HARQ process.

- FFS: value of X and units in which it is defined.

- FFS: Whether TB of the two PDSCHs needs to be different

Agreement:

- Enhanced HARQ process ID indication is supported for DCI 0-2/1-2 and DCI 0-1/1-1 by at least one of following:

  - Option 1: Slot index as the MSB

  - Option 1-a:Slot index as the LSB

  - Option 2: Reusing one bit from other bit field

  - Option 3: Extending the HARQ process ID field up to 5 bits

- FFS: DCI 0-0/1-0

- Note: 32 is taken as maximal supported HARQ processes number for both UL and DL

Agreement:

HARQ codebook enhancement is supported as:

- For Type-2 HARQ codebook:

  - Option-1: Reduce codebook size with:

    - HARQ-ACK codebook only includes HARQ-ACK of PDSCH with feedback-enabled HARQ processes

      - FFS: the details of C-DAI and T-DAI counting for DCI of PDSCH with feedback-enable/disabled HARQ processes

    - FFS: at least DCI for SPS release/SPS PDSCH

  - Option-2: No enhancement

  - Other options are not precluded.

- For Type-1 HARQ codebook, further discuss is needed with down selection among following options:

  - Option-1: No enhancement;

  - Option-2: Report NACK on disabled process

  - Option-3: Reduce codebook size with criteria

- FFS: Enhancements for Type-3 HARQ codebook

### Others

Including the list of topics to specify if beneficial and needed

R1-2008812	Summary #1 of 8.4.4 Other Aspects of NR-NTN	MediaTek Inc.

R1-2007663	Discussion on other aspects for NR-NTN	vivo

R1-2007857	Other Aspects of  NR-NTN	CATT

R1-2008013	Other Aspects for NTN	CMCC

R1-2008167	Remaining issues for NTN	Samsung

R1-2008319	Discussion on other design aspects for NTN	Huawei, HiSilicon

R1-2008362	Discussion on beam management and polarization for NTN	Sony

R1-2008413	Discussions on other aspects of NTN	LG Electronics

R1-2008468	Other Aspects of NR NTN	Apple

R1-2008803	Discussion on the applicability of  DFT-S-OFDM for NTN	CAICT

Withdrawn

R1-2008804	Discussion on the applicability of DFT-S-OFDM for NTN	CAICT

R1-2008807	Considerations on Earth-fixed Beams and Earth-moving beams	CAICT

Withdrawn

R1-2008811	Other Aspects of  NR-NTN	MediaTek Inc.

R1-2008853	Discussion on additional enhancement for NTN	ZTE

R1-2008885	Study Cases and Parameters for System-Level Simulations in NTN WI	Nomor Research GmbH, Thales

R1-2008925	Discussion on NTN beam management	Lenovo, Motorola Mobility

R1-2009026	SSB, beam management and polarization signaling for NTN	Panasonic

R1-2009035	Discussion on the beam management for NTN	Xiaomi

R1-2009094	On other enhancements for NTN	Ericsson

R1-2009119	On feeder link switch and beam management for NTN	InterDigital, Inc.

R1-2009237	Discussion on the applicability of  DFT-S-OFDM for NTN	CAICT

R1-2009245	Additional aspects for NR over NTN	Nokia, Nokia Shanghai Bell

R1-2009249	Discussion on beam management of NTN	CAICT

R1-2009265	BWP operation and other issues for NTN	Qualcomm Incorporated

[103-e-NR-NTN-Other-Enh] Email discussion/approval on other enhancements including list of topics to specify if beneficial and needed with checkpoints for agreements on 11/5, 11/10, 11/12 – Gilles (MediaTek)

R1-2009487	Summary #2 of 8.4.4 Other Aspects of NR-NTN	MediaTek Inc.

R1-2009736	Summary #4 of 8.4.4 Other Aspects of NR-NTN	MediaTek Inc.

Agreement:

Indication of polarization information for DL and UL by the network is supported.

- FFS: Signaling details

## Study on NR Positioning Enhancements

Please refer to RP-202094 for detailed scope of the SI

R1-2008762	Update of TR 38.857	Ericsson

R1-2009418	Update of TR 38.857	Ericsson

TR draft in R1-2009418 endorsed with the following modifications:

- Remove all instances of “can be investigated in Rel-17”

- Add agreement on DL PRS aggregation into Section 7

R1-2009430	Study on NR Positioning Enhancements, v0.1.0	Ericsson

R1-2009431	Update of TR 38.857, v0.1.0	Ericsson

R1-2009431 is endorsed as v0.1.1

R1-2009665	Update of TR 38.857	Ericsson

R1-2009665 is endorsed as v0.1.2 with the following modifications:

- Change “For the issues related to LOS/NLOS issues in positioning” to “For the issues related to mitigating effects of multipath/NLOS for positioning” in Section 8.4

- Remove company names

- Move tables in Section 8.4 to an Appendix

- Remove “(as per the optional model)” in “For issues related to gNB/UE TX/RX timing errors (as per the optional model)” in Section 8.4

R1-2009670	3GPP TR 38.857 Study on NR Positioning Enhancements, v0.2.0	Ericsson

R1-2009743 	3GPP TR 38.857 Study on NR Positioning Enhancements, v0.3.0	Ericsson

Endorse TR draft in R1-2009743 in principle with the following changes:

- Merge 10.8, 10.9 and 10.10 as per Qualcomm’s comments

- Remove individual company observations as per Nokia’s comments

- Update per agreements made in the last GTW

### Additional scenarios for evaluation

R1-2007575	Scenarios and evaluation assumption for IIoT positioning	Huawei, HiSilicon

R1-2007664	Discussion on additional scenarios for NR positioning evaluation	vivo

R1-2007753	Remaining issues on evaluation assumptions	ZTE

R1-2007858	Remaining issues on additional scenarios for evaluation of NR Positioning Enhancements	CATT

R1-2007907	Remaining Issues on Evaluation Assumptions	FUTUREWEI

R1-2007944	NR positioning scenarios	Intel Corporation

R1-2008014	Remaining issues on target performance requirement of IIoT use case	CMCC

R1-2008224	Discussion on Additional Scenarios for Evaluation of NR Positioning	OPPO

R1-2008299	Additional scenarios for evaluation of NR positioning	Nokia, Nokia Shanghai Bell

R1-2008363	Considerations on Additional Scenarios for Evaluation	Sony

R1-2008415	Discussions on additional scenarios for evaluation for NR positioning	LG Electronics

R1-2008488	Discussion on evaluation assumptions for latency	InterDigital, Inc.

R1-2008617	Considerations on Additional Scenarios for Evaluation	Qualcomm Incorporated

R1-2008763	Remaining details on additional scenarios for evaluation	Ericsson

[103-e-NR-ePos-01] Email discussion/approval on additional scenarios for evaluation and TR updates until 11/4; address any remaining aspects by 11/12 – Florent (Ericsson)

Additional checkpoint where agreements can be declared: 11/9

R1-2009343	FL summary for additional scenarios for evaluation	Moderator (Ericsson)

R1-2009364	Output#1 of [103-e-NR-ePos-01] Email discussion on additional scenarios for evaluation	Moderator (Ericsson)

R1-2009503	Output #2 of [103-e-NR-ePos-01] Email discussion on additional scenarios for evaluation	Moderator (Ericsson)

Conclusion:

When reporting network efficiency, the methodology is left up to each company and details should be provided.

Conclusion:

For SRS bandwidth stitching and PRS bandwidth stitching, when modelling the effect of phase offset between two PFLs (for 2 CCs in the UL), a uniformly distributed phase offset is applied to the 2nd PFL (2nd CC in the UL) with respect to the 1st PFL (1st CC in the UL).

- If phase offset modeled, companies to provide how the phase offset is modeled in their evaluations.

Agreement:

- In Rel-17 target positioning requirements for commercial use cases are defined as follows:

  - Horizontal position accuracy (< 1 m) for 90% of UEs

  - Vertical position accuracy (< 3 m) for 90% of UEs

  - End-to-end latency for position estimation of UE (< 100 ms)

  - Physical layer latency for position estimation of UE (< 10 ms)

- In Rel-17 target positioning requirements for IIoT use cases are defined as follows:

  - Horizontal position accuracy (< 0.2 m) for 90% of UEs

  - Vertical position accuracy (< 1 m) for 90% of UEs

  - End-to-end latency for position estimation of UE (< 100ms, in the order of 10 ms is desired)

  - Physical layer latency for position estimation of UE (<10ms)

- Note 1: Target positioning requirements may not necessarily be reached for all scenarios and deployments

- Note 2: For some scenarios the requirement for Horizontal position accuracy can be relaxed to < 0.5 m in IIoT use cases.

- Note 3: All positioning techniques may not achieve the target positioning requirements over all scenarios

### Evaluation of achievable positioning accuracy and latency

R1-2007576	Evaluation results for Rel-16 positioning and Rel-17 enhancement	Huawei, HiSilicon

R1-2007665	Evaluation of NR positioning performance	vivo

R1-2007720	Evaluation of achievable positioning accuracy	BUPT

R1-2007754	Evaluation of achievable accuracy and latency	ZTE

R1-2007859	Discussion of evaluation of NR positioning performance	CATT

R1-2007908	NLOS Identification and Mitigation	FUTUREWEI

R1-2007945	NR positioning evaluation	Intel Corporation

R1-2007997	NR Positioning Latency Evaluations	Lenovo, Motorola Mobility

R1-2008225	Evaluation of NR positioning in IIOT scenario	OPPO

R1-2008300	Results on evaluation of achievable positioning accuracy and latency	Nokia, Nokia Shanghai Bell

R1-2008364	Discussion on Performance evaluation of Rel-17 positioning	Sony

R1-2008416	Discussions on evaluation of achievable positioning accuracy and latency for NR positioning	LG Electronics

R1-2008489	Evaluation of achievable positioning latency	InterDigital, Inc.

R1-2008618	Evaluation of achievable Positioning Accuracy & Latency	Qualcomm Incorporated

R1-2008709	Evaluation of positioning enhancements	Fraunhofer IIS, Fraunhofer HHI

R1-2008720	Positioning evaluation results on potential enhancements for additional use cases	CEWiT

R1-2008764	Evaluation of achievable positioning accuracy and latency	Ericsson

[103-e-NR-ePos-02] Email discussion/approval on achievable positioning accuracy and latency including observations/conclusions based on evaluation results until 11/3; address any remaining aspects by 11/11 – Alexey (Intel)

Additional checkpoint where agreements can be declared: 11/9

R1-2009345	Feature lead summary for evaluation of NR Positioning enhancements - AI 8.5.2		Moderator (Intel Corporation)

R1-2009375	Summary #1 of RAN WG1 E-mail Discussion [103e-NR-ePos-02] - AI 8.5.2	Moderator (Intel Corporation)

R1-2009376	Summary #2 of RAN WG1 E-mail Discussion [103e-NR-ePos-02]- AI 8.5.2	Moderator (Intel Corporation)

R1-2009406	Summary #3 of RAN WG1 E-mail Discussion [103e-NR-ePos-02]- AI 8.5.2	Moderator (Intel Corporation)

R1-2009407	Summary #4 of RAN WG1 E-mail Discussion [103e-NR-ePos-02] - AI 8.5.2	Moderator (Intel Corporation)

R1-2009606	Summary #5 of RAN WG1 E-mail Discussion [103e-NR-ePos-02] - AI 8.5.2	Moderator (Intel Corporation)

R1-2009607	Summary #6 of RAN WG1 E-mail Discussion [103e-NR-ePos-02] - AI 8.5.2	Moderator (Intel Corporation)

R1-2009762	Discussion on DL PRS and SRS for Positioning Measurement Time ([103-e-NR-ePos-02])			Moderator (Intel Corporation)

Agreement:

Capture the following observations (On horizontal positioning accuracy in InF-SH) in the TR (editorial modifications and addition of references for the sources can be added when incorporating into the TR):

For the case without modeling synchronization and gNB/UE TX/RX timing errors in the InF-SH scenario.

- Based on the results provided by a majority of sources, sub-meter level @ 90% of horizontal positioning accuracy is achieved by Rel.16 solutions.

- Results were provided by [12] out of [17] sources for FR1 and by [9] sources out of [17] for FR2

  - For NR positioning evaluations in FR1 band, the following is observed with respect to horizontal positioning accuracy:

    - Accuracy of ≤ 0.2m @ 90% is achieved in contributions from [3] sources  and is not achieved in contributions from [9] sources

    - Accuracy of ≤ 0.5m @ 90% is achieved in contributions from [7] sources and is not achieved in contributions from [5] sources

  - For NR positioning evaluations in FR2 band, the following is observed with respect to horizontal positioning accuracy:

    - Accuracy of ≤ 0.2m @ 90% is achieved in contributions from [6] sources and is not achieved in contributions from [3] sources

    - Accuracy of ≤ 0.5m @ 90% is achieved in contributions from [8] sources and is not achieved in contributions from [1] sources

Agreement:

Capture the following observations on horizontal positioning accuracy in InF-DH (Editorial modifications and updates to references to be made when capturing in the TR):

For the case without modeling synchronization and gNB/UE TX/RX timing errors in the baseline InF-DH scenario.

- Based on the results provided by a majority of sources, sub-meter level @ 90% of horizontal positioning accuracy is not achieved by Rel.16 based solutions.

- Results were provided by [13] sources (Huawei R1-2007576, BUPT R1-2007720, ZTE R1-2007754, CATT R1-2007859, FUTUREWEI R1-2007908, OPPO R1-2008225, Nokia R1- 2008300, Sony R1-2008364, CEWiT R1-2008720, Ericsson R1-2008764, QC R1-2008618, vivo R1-2007665, Intel R1-2007945) out of [17] for FR1 and by [9] sources (Huawei R1-2007576, BUPT R1-2007720, ZTE R1-2007754, CATT R1-2007859, Sony R1-2008364, Ericsson R1-2008764, QC R1-2008618, vivo R1-2007665, Intel R1-2007945) out of [17] for FR2

  - For NR positioning evaluations in FR1 band, the following is observed with respect to horizontal positioning accuracy:

    - Accuracy of ≤ 0.2m @ 90% is achieved in contribution from [1] source (CATT R1-2007859) and is not achieved in contributions from [12] sources (Huawei R1-2007576, BUPT R1-2007720, ZTE R1-2007754, FUTUREWEI R1-2007908, OPPO R1-2008225, Nokia R1- 2008300, Sony R1-2008364, CEWiT R1-2008720, Ericsson R1-2008764, QC R1-2008618, vivo R1-2007665, Intel R1-2007945)

    - Accuracy of ≤ 0.5m @ 90% is achieved in contributions from [4] sources (BUPT R1-2007720, CATT R1-2007859, QC R1-2009361, vivo R1-2007665) and is not achieved in contributions from [9] sources (Huawei R1-2007576, ZTE R1-2007754, FUTUREWEI R1-2007908, OPPO R1-2008225, Nokia R1- 2008300, Sony R1-2008364, CEWiT R1-2008720, Ericsson R1-2008764, Intel R1-2007945)

  - For NR positioning evaluations in FR2 band, the following is observed with respect to horizontal positioning accuracy:

    - Accuracy of ≤ 0.2m @ 90% is achieved in contributions from [3] sources (BUPT R1-2007720, QC R1-2008618, vivo R1-2007665) and is not achieved in contributions from [6] sources (Huawei R1-2007576, ZTE R1-2007754, CATT R1-2007859, Sony R1-2008364, Ericsson R1-2008764, Intel R1-2007945)

    - Accuracy of ≤ 0.5m @ 90% is achieved in contributions from [3] sources (BUPT R1-2007720, QC R1-2008618, vivo R1-2007665) and is not achieved in contributions from [6] sources (Huawei R1-2007576, ZTE R1-2007754, CATT R1-2007859, Sony R1-2008364, Ericsson R1-2008764, Intel R1-2007945)

Agreement:

Capture the following observations on vertical positioning accuracy in InF-SH (Editorial modifications and updates to references to be made when capturing in the TR):

For the case without modeling synchronization and gNB/UE TX/RX timing errors in the InF-SH scenario.

- Results were provided by [4] sources (ZTE R1-2007754, CATT R1-2007859, vivo R1-2007665, Intel R1-2007945) out of [17] for FR1 and by [4] sources (ZTE R1-2007754, CATT R1-2007859, QC R1-2008618, Intel R1-2007945) out of [17] for FR2 band

  - For NR positioning evaluations in FR1 band, the following is observed with respect to vertical positioning accuracy:

    - Accuracy of ≤ 1m @ 90% is achieved in contribution from [2] sources (ZTE R1-2007754, vivo R1-2007665) and is not achieved from [2] sources (CATT R1-2007859, Intel R1-2007945)

  - For NR positioning evaluations in FR2 band, the following is observed with respect to vertical positioning accuracy:

    - Accuracy of ≤ 1m @ 90% is achieved in contribution from [4] sources (ZTE R1-2007754, CATT R1-2007859, QC R1-2008618, Intel R1-2009390) [and is not achieved by [0] sources]

Agreement:

Capture the following observations on vertical positioning accuracy in InF-DH (Editorial modifications and updates to references to be made when capturing in the TR):

For the case without modeling synchronization and gNB/UE TX/RX timing errors in the InF-DH scenario (including evaluations with variable gNB/UE heights).

- Results were provided by [5] sources (ZTE R1-2007754, CATT R1-2007859, vivo R1-2007665, Intel R1-2007945, Huawei R1-2007576) out of [17] for FR1 and by [4] sources (ZTE R1-2007754, CATT R1-2007859, Intel R1-2007945, Huawei R1-2007576) out of [17] for FR2 band

  - For NR positioning evaluations in FR1 band, the following is observed with respect to vertical positioning accuracy:

    - Accuracy of ≤ 1m @ 90% is achieved in contribution from [2] sources (CATT R1-2007859, vivo R1-2007665) and is not achieved from [3] sources (ZTE R1-2007754, Intel R1-2007945, Huawei R1-2007576)

  - For NR positioning evaluations in FR2 band, the following is observed with respect to vertical positioning accuracy:

    - Accuracy of ≤ 1m @ 90% is achieved in contribution from [1] source (Huawei R1-2007576) and is not achieved from [3] sources (ZTE R1-2007754, CATT R1-2007859, Intel R1-2007945)

Agreement:

Capture the following in the TR:

- Evaluation results for LOS/NLOS identification, outlier rejection, NLOS mitigation based on triangle inequality algorithms in indoor factory scenarios were provided by [12] sources (OPPO, Futurewei, vivo, Intel, Qualcomm, ZTE, Huawei, CeWiT, Nokia, Sony, Fraunhofer, Ericsson) out of [17] sources

- NR positioning utilizing LOS/NLOS identification, outlier rejection, NLOS mitigation based on triangle inequality algorithms improve performance of positioning accuracy with respect to solutions that do not apply these techniques

- From the evaluations,

  - [9] sources (Futurewei, Intel, ZTE, Huawei, CeWiT, Nokia, Sony, Fraunhofer, Ericsson) evaluated LOS/NLOS identification with additional specification changes relative to Rel.16 solutions

  - [2] sources (vivo, Qualcomm) evaluated outlier rejection algorithm (implementation-based algorithm that can be applied for Rel.16 solutions without specification changes)

  - [1] source (OPPO) evaluated NLOS mitigation using triangle-based inequality algorithm (implementation-based algorithm that can be applied for Rel.16 solutions without specification changes)

- Comparative analysis of LOS/NLOS identification with specification changes vs implementation based methods (outlier rejection algorithms) was done by 6 sources (Intel, Huawei, vivo, Qualcomm, ZTE, Oppo)

  - Three sources (Intel, Huawei, ZTE) observe that NR positioning based on LOS/NLOS identification outperforms NR positioning utilizing outlier rejection

  - Three sources (vivo, Qualcomm, Oppo) observe that NR positioning utilizing outlier rejection outperforms NR positioning utilizing LOS/NLOS identification

Agreement:

- Capture the following in the TR:

  - Evaluation results of gNB/UE TX/RX timing errors (as per the optional model) are provided by [7] sources (Huawei, ZTE, Qualcomm, Intel, CATT, Ericsson, vivo) out of [17] sources)

- Capture summary of results provided in Tables in Section 2.1.2.3 of R1-2009606 in TR with the following modifications

  - Removal of the numbers in brackets after “YES”/”No”.

  - Change “barely” to “NO”

Agreement:

Capture the following in the TR:

- Evaluation results for aggregation of DL positioning frequency layers were provided by [5] sources (Intel, Qualcomm, Huawei, vivo, Ericsson) out of [17].

- Aggregation of NR positioning frequency layers improves positioning accuracy under certain scenarios, configurations, and assumptions on modelled impairments such as: bandwidth and spacing of aggregated layers, timing offset and frequency offset over frequency layers, phase discontinuity and possible amplitude imbalance.

  - One source (Huawei) observes that aggregation with phase continuity can help to improve the positioning accuracy, and discontinuous aggregation can approach the performance of contiguous aggregation with the same frequency span

  - One source (Intel) has shown that  aggregation of frequency layers (without modeling impairements) improves the positioning accuracy for intra-band contiguous configuration and that further study is needed for other cases including impairments

  - One source (Ericsson) has observed that PRS aggregation shows potential gains without modeling phase error, but these gains are lost when the phase error between CCs becomes too large

  - One source (Qualcomm) has analyzed aggregation of 2 and 4 frequency layers for different channel spacings, time and phase offset across frequency layers

  - One source (vivo R1-2007666) has analyzed aggregation of 2 frequency layers for different time offset values and observed that:

    - For the case without impairements modeling, aggregation of multiple DL positioning frequency layers 50MHz+50MHz, performance target [0.2m @ 90%] cannot be achieved in both InF-SH and InF-DH.

    - For the case without impairements modeling, aggregation of multiple DL positioning frequency layers 50MHz+50MHz, the performance is worse than 100MHz but better than 50MHz.

    - The performance of aggregation of frequency layers degrades if timing offset is increased

Agreement:

Capture the following observations (Editorial modifications and updates to references to be made when capturing in the TR):

- Summary table on physical layer latency for Rel.16 DL-TDOA/DL-AOD UE-Assisted NR positioning from discussion round #1 in the TR in Section 3.1.1 of R1-2009606

- Summary of physical layer latency for Rel.16 DL-TDOA/DL-AOD UE-assisted NR positioning in FR1 was provided by [11] sources

- Summary of physical layer latency for Rel.16 DL-TDOA/DL-AOD UE-assisted NR positioning in FR2 was provided by [4] sources

- For evaluation in FR1,

  - results from [11] sources out of [11] sources (Qualcomm, Huawei, ZTE, vivo, Lenovo, LGE, CATT, Nokia, OPPO, Interdigital, Intel) show that minimum estimated physical layer latency for Rel.16 DL-TDOA/DL-AOD UE-assisted NR positioning exceeds 10ms

  - results from [2] (ZTE, Intel) sources out of [11] sources (Qualcomm, Huawei, ZTE, vivo, Lenovo, LGE, CATT, Nokia, OPPO, Interdigital, Intel) show that minimum estimated physical layer latency for Rel.16 DL-TDOA/DL-AOD UE-assisted NR positioning exceeds 100ms

- For evaluation in FR2,

  - results from [4] sources out of [4] sources (ZTE, vivo, Lenovo, OPPO) show that minimum estimated physical layer latency for Rel.16 DL-TDOA/DL-AOD UE-assisted NR positioning exceeds 10ms

  - results from [2] (ZTE, vivo) sources out of [4] sources (ZTE, vivo, Lenovo, OPPO) show that minimum estimated physical layer latency for Rel.16 DL-TDOA/DL-AOD UE-assisted NR positioning exceeds 100ms

- The following list provides the major physical layer latency components for Rel.16 DL TDOA/DL-AOD UE-assisted NR Positioning

  - DL PRS alignment, transmission, measurement (including processing time) and report delay

  - Measurement gap request, configuration and alignment time

  - UE/gNB higher layer (LPP/RRC) processing times

Agreement:

Capture the following observations (Editorial modifications and updates to references to be made when capturing in the TR):

- Capture summary table in Section 3.2.1 of R1-2009606 on physical layer latency for Rel.16 UL-TDOA/UL-AOA NR positioning from discussion round #1 in the TR

- Summary of physical layer latency for Rel.16 UL-TDOA/UL-AOA NR positioning in FR1 was provided by [8] sources (Huawei, vivo, LGE, CATT, Nokia, OPPO, Interdigital, Intel)

- Summary of physical layer latency for Rel.16 UL-TDOA/UL-AOA NR positioning in FR2 was provided by [2] sources (vivo, OPPO)

- For evaluation in FR1,

  - results from [3] sources (Huawei, CATT, Nokia) out of [8] sources (Huawei, vivo, LGE, CATT, Nokia, OPPO, Interdigital, Intel) show that minimum estimated physical layer latency for Rel.16 UL-TDOA/UL-AOA NR positioning does not exceed 10ms

  - results from [8] sources out of [8] sources (Huawei, vivo, LGE, CATT, Nokia, OPPO, Interdigital, Intel) show that minimum estimated physical layer latency for Rel.16 UL-TDOA/UL-AOA NR positioning does not exceed 100ms

- For evaluation in FR2,

  - results from [2] sources out of [2] sources (vivo, OPPO) show that minimum estimated physical layer latency for Rel.16 UL-TDOA/UL-AOA NR positioning exceeds 10ms

  - results from [1] (OPPO) sources out of [2] sources (vivo, OPPO) show that minimum estimated physical layer latency for Rel.16 UL-TDOA/UL-AOA NR positioning does not exceed 100ms

- The following list provides the major physical layer latency components for Rel.16 UL-TDOA/UL-AOA NR Positioning

  - SRS for positioning processing time

  - SRS for positioning alignment time (depends on periodic or aperiodic SRS for positioning)

  - gNB higher layer processing delays (RRC/ NRPPa processing times)

Agreement:

Capture the following observations (Editorial modifications and updates to references to be made when capturing in the TR):

- Capture summary table on physical layer latency for Rel.16 Multi-RTT UE-assisted NR positioning from discussion round #1 in the TR

- Summary of physical layer latency for Rel.16 Multi-RTT UE-assisted NR positioning in FR1 was provided by [6] sources (Qualcomm, Huawei, vivo, LGE, Interdigital, Intel)

- Summary of physical layer latency for Rel.16 Multi-RTT UE-assisted NR positioning in FR2 was provided by [0] sources

- For evaluation in FR1,

  - results from [6] sources (Qualcomm, Huawei, vivo, LGE, Interdigital, Intel) out of [6] sources (Qualcomm, Huawei, vivo, LGE, Interdigital, Intel) show that minimum estimated physical layer latency for Rel.16 Multi-RTT UE-assisted NR positioning exceeds 10ms

  - results from [4] sources (Qualcomm, Huawei, vivo, Interdigital) out of [6] sources (Qualcomm, Huawei, vivo, LGE, Interdigital, Intel) show that minimum estimated physical layer latency for Rel.16 Multi-RTT UE-assisted NR positioning does not exceed 100ms

- The following list provides the major physical layer latency components for Rel.16 Multi-RTT UE-assisted NR positioning

  - DL PRS alignment, transmission, measurement time and report delay

  - Measurement gap request, configuration, alignment time

  - SRS for positioning processing time

  - SRS for positioning alignment time (depends on periodic or aperiodic SRS for positioning)

  - UE/gNB higher layer (LPP/RRC/NRPPa) processing times

Agreement:

Capture the following observations (Editorial modifications and updates to references to be made when capturing in the TR):

- Capture summary table on physical layer latency for Rel.16 E-CID NR positioning from discussion round #1 in the TR

- Summary of physical layer latency for Rel.16 E-CID NR positioning in FR1 was provided by [3] sources (Huawei, ZTE, LGE)

- Summary of physical layer latency for Rel.16 E-CID NR positioning in FR2 was provided by [0] sources

- For evaluation in FR1,

  - results from [2] sources (ZTE, LGE) out of [3] sources (Huawei, ZTE, LGE) show that minimum estimated physical layer latency for Rel.16 E-CID NR positioning exceeds 10ms

  - results from [3] sources (Huawei, ZTE, LGE) out of [3] sources (Huawei, ZTE, LGE) show that minimum estimated physical layer latency for Rel.16 E-CID NR positioning does not exceed 100ms

- The following list provides the major physical layer latency components for Rel.16 E-CID NR positioning

  - Higher layer signaling processing

Agreement:

Capture the following observations (Editorial modifications and updates to references to be made when capturing in the TR):

- Capture summary table on physical layer latency for Rel.16 DL-only UE-based NR positioning from discussion round #1 in the TR

- Summary of physical layer latency for Rel.16 DL-only UE-based NR positioning in FR1 was provided by [6] sources (Qualcomm, Huawei, vivo, Lenovo, OPPO, Interdigital)

- Summary of physical layer latency for Rel.16 DL-only UE-based NR positioning in FR2 was provided by [2] sources (vivo, Lenovo)

- For evaluation in FR1,

  - results from [4] sources (Huawei, vivo, OPPO, Interdigital) out of [6] sources (Qualcomm, Huawei, vivo, Lenovo, OPPO, Interdigital) show that minimum estimated physical layer latency for Rel.16 DL-only UE-based NR positioning exceeds 10ms

  - results from [6] sources out of [6] sources (Qualcomm, Huawei, vivo, Lenovo, OPPO, Interdigital) show that minimum estimated physical layer latency for Rel.16 DL-only UE-based NR positioning does not exceed 100ms

- For evaluation in FR2,

  - results from [2] sources out of [2] sources (vivo, Lenovo) show that minimum estimated physical layer latency for Rel.16 DL-only UE-based NR positioning exceeds 10ms

  - results from [1] (vivo) sources out of [2] sources (vivo, Lenovo) show that minimum estimated physical layer latency for Rel.16 DL-only UE-based NR positioning exceeds 100ms

- The following list provides the major physical layer latency components for Rel.16 DL-only UE-based NR positioning

  - DL PRS alignment, transmission, measurement time and, if requested, report delay

  - Measurement gap request, configuration, alignment time

  - Higher layer (LPP/RRC) processing times

Agreement:

Capture the following observations (Editorial modifications and updates to references to be made when capturing in the TR):

For the case without modeling synchronization and gNB/UE TX/RX timing errors in the UMa scenario

- Based on the results provided, 10 m level @ 90% of horizontal positioning accuracy is achieved by Rel.16 in UMa scenario

- Results were provided by [2] sources (Ericsson R1-2008764, QC R1-2008618) out of [17] for FR1 band

  - For NR positioning evaluations for UMa scenario in FR1 band, the following is observed with respect to horizontal positioning accuracy:

    - Accuracy of ≤ 1m @ 80% is achieved for the outdoor UEs in contributions from [1] source (QC, R1-2008618) out of [2] sources (Ericsson R1-2008764, QC R1-2008618) in the scenario without absolute time of arrival modelling. Zero sources met an accuracy of ≤ 1m @ 90%.

    - Accuracy of ≤ 10m @ 90% is achieved for the outdoor UEs in contributions from [2] sources (Ericsson R1-2008764, QC R1-2008618) out of [2] sources in the scenario without absolute time of arrival modelling

    - Accuracy of ≤ 10m @ 90% is achieved for the indoor UEs in contributions from [1] source (Ericsson R1-2008764) out of [2] sources in the scenario without absolute time of arrival modelling

Agreement:

Capture the following observations (Editorial modifications and updates to references to be made when capturing in the TR):

For the case without modeling synchronization and gNB/UE TX/RX timing errors in the UMi scenario

- Results were provided by [4] sources (Nokia R1- 2008300, Ericsson R1-2008764, QC R1-2008618, Fraunhofer R1-2009428) out of [17] for FR1 band

  - For NR positioning evaluations for UMi scenario in FR1 band, the following is observed with respect to horizontal positioning accuracy:

    - Accuracy of ≤ 1m @ 90% is achieved in contributions from [2] sources (Ericsson R1-2008764, QC R1-2008618) and is not achieved from [2] sources (Nokia R1- 2008300, Fraunhofer R1-2009428) in the scenario without absolute time of arrival modelling

    - Accuracy of ≤ 1m @ 90% is not achieved from [2] sources (QC R1-2008618, Fraunhofer) in a scenario with absolute time of arrival modelling

Agreement:

Capture the following observations (Editorial modifications and updates to references to be made when capturing in the TR):

For the case without modeling synchronization and gNB/UE TX/RX timing errors in the IOO scenario

- Based on the results provided by a majority of the sources, 1 m level @ 90% of horizontal positioning accuracy is achieved by Rel.16 in IOO scenario

- Results were provided by [5] sources (CATT R1-2007859, Nokia R1- 2008300, Sony R1-2008364, Ericsson R1-2008764, vivo R1-2007665) out of [17] for FR1 and [5] sources (CATT R1-2007859, Sony R1-2008364, Ericsson R1-2008764, QC R1-2008618, vivo R1-2007665) out of [17] for FR2 band

  - For NR positioning evaluations for IOO scenario in FR1 band, the following is observed with respect to horizontal positioning accuracy:

    - Accuracy of ≤ 1m @ 90% is achieved in contributions from [4] sources (CATT R1-2007859, Sony R1-2008364, Ericsson R1-2008764, vivo R1-2007665) and is not achieved from [1] source (Nokia R1- 2008300) in the scenario without absolute time of arrival modelling

    - Accuracy of ≤ 1m @ 90% is achieved from [1] source (vivo R1-2005380) in a scenario with absolute time of arrival modelling

  - For NR positioning evaluations for IOO scenario in FR2 band, the following is observed with respect to horizontal positioning accuracy:

    - Accuracy of ≤ 1m @ 90% is achieved in contributions from [5] sources (CATT R1-2007859, Sony R1-2008364, Ericsson R1-2008764, QC R1-2008618, vivo R1-2007665) in the scenario without absolute time of arrival modelling

    - Accuracy of ≤ 1m @ 90% is achieved from [1] source (vivo R1-2005380) in a scenario with absolute time of arrival modelling

Agreement:

Capture the following observations (Editorial modifications and updates to references to be made when capturing in the TR):

- The results for the UE efficiency (power saving) in the RRC_IDLE/RRC_INACTIVE states were analyzed by 2 sources (Huawei/HiSi, vivo) out of 17 sources (assumptions may be different between the different sources)

- In one source (Huawei/HiSi), the following observations were made:

  - RRC_IDLE/RRC_INACTIVE state positioning can save about 7%-40% power consumption compared to C-DRX configuration

- In one source (vivo), the following observations were made:

  - Positioning report in the RRC_IDLE state can provide 44.32 % of power saving gain compared to the report in the RRC_CONNECTED state

  - Positioning measurement and report in the RRC_IDLE state can provide at least 48.38 % of power saving gain compared to the measurement and report in the RRC_CONNECTED state

Conclusion:

Estimated minimum DL PRS measurement time in Rel.16 can be 88.5ms depending on DL PRS configuration settings

- Note: The following assumptions are made

  - One DL PRS frequency layer in FR1

  - CSSF = 1

  - NRxBeam, i = 1,

  - Nsample = 4 (DL PRS RSTD measurements are done across 4 DL PRS periods)

  - Both DL PRS periodicity and MGRP are equal to 20ms

  - Configured DL PRS resources are within UE DL PRS processing capacity (N,T) = (0.5ms, 8ms)

Conclusion:

- SRS for positioning measurement time of 12 ms can be achieved under certain SRS for positioning configuration settings depending on the frame configuration

  - Note: The following assumptions are made

    - SRS for positioning alignement time 0.5 ms

    - SRS for positioning transmission time 0.5ms

    - SRS for positioning processing time 5 ms

    - 30 kHz SCS in FR1

    - Single SRS resource set with single SRS resource

    - Four SRS instances

- Note: Considering UL link budget and interference on SRS for positioning signals, the longer transmission time may be needed that will further increase SRS for positioning measurement time

Agreement:

Capture the following observations in the TR (Editorial modifications and updates to references to be made when capturing in the TR):

  - The results for the PRS resource utilization were analyzed by 3 sources (Huawei/HiSi, vivo, CATT) out of 17 sources

  - In one source (Huawei), the PRS resource utilization was evaluated for the case of 160 ms DL PRS periodicity, 30 kHz subcarrier spacing, and 12, 4, and 1 symbol per PRS resource:

    - PRS with 12, 4, and 1 symbol has positioning resource utilization of 2.14 %, 0.714 %, and 0.179 %, respectively

  - In one source (vivo), the PRS resource utilization was evaluated:

    - In FR1, for 20 ms DL PRS periodicity and MG periodicity, 3ms MGL, 30 kHz subcarrier spacing, comb 6 and 6 symbols per PRS resource, 18 positioning sites and 1 beams per site, PRS resource utilization is 3.21% while the MGL/MGRP (UE overhead) is 15%.

    - In FR2, for 20 ms DL PRS periodicity, 20ms for MGL and MGRP, 120 kHz subcarrier spacing, comb 6 and 6 symbols per PRS resource, 18 positioning sites and 64 beams per site, PRS resource utilization is 51.42% while the MGL/MGRP (UE overhead) is 100%

    - It was observed by the source that the network and device efficiency can be improved by on-demand PRS (assuming the same latency) compared to periodic PRS

  - In one source (CATT), the PRS resource utilization was evaluated for the case of 20 ms DL PRS periodicity, 30 kHz subcarrier spacing, and 12 symbols per PRS resource:

    - PRS with 12 symbols has positioning resource utilization of 2.1 %.

[103-e-NR-ePos-04] Email discussion/approval on collection of simulation results until 11/6 – Ren Da(CATT)/Alexey (Intel)/Florent (Ericsson)

### Potential positioning enhancements

Including positioning techniques, DL/UL positioning reference signals, signalling and procedures for improved accuracy, reduced latency, network efficiency (scalability, RS overhead, etc.), and device efficiency (power consumption, complexity, etc.).

R1-2007552	Positioning Enhancements	FUTUREWEI

R1-2007577	Positioning enhancement in Rel-17	Huawei, HiSilicon

R1-2007666	Discussion on potential positioning enhancements	vivo

R1-2007721	Potential positioning enhancements	BUPT

R1-2007755	Discussion on potential NR positioning enhancements	ZTE

R1-2007860	Discussion of NR positioning enhancements	CATT

R1-2007886	Potential positioning enhancements	TCL Communication Ltd.

R1-2007946	NR positioning enhancements	Intel Corporation

R1-2007998	Potential NR Positioning Enhancements	Lenovo, Motorola Mobility

R1-2008015	Discussion on potential positioning enhancements	CMCC

R1-2008083	Potential positioning enhancements	Xiaomi

R1-2008168	Potential positioning enhancements	Samsung

R1-2008226	Discussions on NR Positioning Enhancements	OPPO

R1-2008301	Views on potential positioning enhancements	Nokia, Nokia Shanghai Bell

R1-2008365	Considerations on potential positioning enhancements	Sony

R1-2008417	Discussions on potential enhancements for NR positioning	LG Electronics

R1-2008491	Discussion on potential positioning enhancements	InterDigital, Inc.

R1-2008519	Views on positioning enhancement for Rel-17	MediaTek Inc.

R1-2008550	Discussion on potential techniques for NR Positioning Enhancements	NTT DOCOMO, INC.

R1-2008619	Potential Positioning Enhancements for NR Rel-17 Positioning	Qualcomm Incorporated

R1-2008710	Potential positioning enhancements	Fraunhofer IIS

R1-2008718	Discussion on positioning enhancements for Release 17	CEWiT

R1-2008765	Potential positioning enhancements	Ericsson

[103-e-NR-ePos-03] Email discussion/approval on potential positioning enhancements until 11/02; address any remaining aspects by 11/10 – Ren Da (CATT)

Additional checkpoint where agreements can be declared: 11/5

R1-2009314	FL Summary for Potential Positioning Enhancements		Moderator (CATT)

R1-2009366	FL Summary #2 for Potential Positioning Enhancements	Moderator (CATT)

R1-2009395	FL Summary #3 for Potential Positioning Enhancements	Moderator (CATT)

R1-2009396	FL Summary #4 for Potential Positioning Enhancements	Moderator (CATT)

R1-2009397	FL Summary #5 for Potential Positioning Enhancements	Moderator (CATT)

R1-2009398	FL Summary #6 for Potential Positioning Enhancements	Moderator (CATT)

R1-2009399	FL Summary #7 for Potential Positioning Enhancements	Moderator (CATT)

R1-2009673	FL Summary #8 for Potential Positioning Enhancements	Moderator (CATT)

R1-2009678	FL Summary #9 for Potential Positioning Enhancements	Moderator (CATT)

R1-2009679	FL Summary #10 for Potential Positioning Enhancements	Moderator (CATT)

Agreement:

- NR positioning for UEs in RRC_INACTIVE state is recommended for normative work, including

  - DL, UL and DL+UL positioning methods

  - UE-based and UE-assisted positioning solutions

  - Support of UE positioning measurements for UEs in RRC_inactive state

    - Options that can be considered include DL-PRS or DL-PRS and SSB

  - Support of gNB positioning measurements for UEs in RRC_inactive state

- The details of how to enable the UE positioning in RRC_ INACTIVE state can be further discussed during normative work. These details may include, but are not limited to the following aspects:

  - UL reference signals (e.g., SRS for positioning, PRACH preambles) for UL measurements

  - Signalling and procedures for support the assistance data delivery, DL-PRS configuration, UL reference signals for positioning resource configuration, measurement reporting), which may be developed based on the enhancements of existing signalling and procedures (e.g., existing 2-step and/or 4-step PRACH procedures, paging procedure, small data transmission).

Agreement:

Capture the following in the TR:

From a physical layer perspective, on-demand transmission and reception of DL PRS, which includes at least the following is recommended

  - UE-initiated request of on-demand DL PRS transmission

  - LMF (network)-initiated request of on-demand DL PRS transmission

- Above enhancements are recommended for both DL and DL+UL positioning methods and both UE-based and UE-assisted positioning solutions.

Agreement:

Capture the following in the TR:

Simultaneous transmission by the gNB and reception by the UE of intra-band one or more contiguous carriers in one or more contiguous PFLs can be studied further and if needed, specified during normative work

- From both gNB and UE perspective, the applicability and feasibility of this enhancement for different scenarios, configurations, bands and RF architectures, can be further studied

Agreement:

Capture the following in the TR:

Simultaneous transmission by the UE and aggregated reception by the gNB of the SRS for positioning in multiple contiguous intra-band carriers can be studied further and if needed, specified during normative work.

- From both gNB and UE perspective, the applicability and feasibility of this enhancement for different scenarios, configurations, particular bands and RF architectures, can be further studied.

Agreement:

Capture the following in the TR:

- The methods, measurements, signaling, and procedures for improving positioning accuracy in the presence of the UE Rx/Tx timing delays, and/or and gNB Rx/Tx timing delays are recommended for normative work, including

  - DL, UL and DL+UL positioning methods

  - UE-based and UE-assisted positioning solutions

- Note: The details of the solutions are left for further discussion in normative work.

Agreement:

Capture the following in the TR:

The enhancements of the procedure, measurements, reporting, and signalling for improving the accuracy of

- UL AoA is recommended for normative work for network-based positioning solutions.

- DL-AoD is recommended for normative work for UE-based and network-based (including UE-assisted) positioning solutions.

Agreement:

Capture the following in the TR:

Enhancements of information reporting from UE and gNB for supporting multipath/NLOS mitigation can be studied further, and if needed, specified during normative work for improving positioning accuracy.

- Note: The details of the enhancements of reporting are left for further discussion in normative work, which may include, but are not limited to the following information associated with multi-path, e.g., LOS/NLOS identification, time of arrival of the multi-path components, signal power and/or relative power, power delay profile, angle, and/or polarization information, coherence bandwidth, etc.

Agreement:

Capture the following in the TR:

Aperiodic reception of DL PRS from the TRPs of the serving gNB and aperiodic reception of DL PRS from the TRPs of the neighbouring gNBs can be studied further and if needed, specified during normative work.

- Note: Aperiodic reception in the above corresponds to DCI-triggered reception

Agreement:

Capture the following in the TR:

- Semi-persistent reception of DL PRS from the TRPs of the serving gNB and Semi-persistent reception of DL PRS from the TRPs of the neighbouring gNBs can be studied further and if needed, specified during normative work.

- Note: Semi-persistent reception in the above corresponds to MAC-CE activated reception

Agreement:

Capture the following in the TR:

- The enhancements of signaling & procedures for reducing NR positioning latency are recommended for normative work, including DL and DL+UL positioning methods

  - The details of the solutions are left for further discussion in normative work, which may include the following aspects:

    - Latency reduction related to the measurement gap

    - Latency reduction related to the reporting and request of the measurements (e.g., via RRC signaling, MAC-CE and/or physical layer procedure, and/or priority rules)

    - Latency reduction related to measurement time

- The following enhancements of signaling & procedures for reducing NR positioning latency can be studied and specified, if needed

  - Latency reduction related to the request and response of positioning assistance data (e.g., via RRC signaling, MAC-CE and/or physical layer procedure)

  - Latency reduction related to the reception of DL PRS (e.g., priority rules for the reception of DL PRS)

- No assumptions are made on whether the LCS architecture specified in TS 23.273 is enhanced or not.

Agreement:

Capture the following in the TR:

From a physical layer perspective, it is feasible for a UE to perform DL positioning measurement in RRC_IDLE state.

- Note: This does not imply that measurements have to be reported in RRC_IDLE state.

Conclusion:

It is up to RAN2 to decide whether to support the enhancements of NR positioning reporting of DL positioning measurements and/or positioning estimates for RRC_IDLE UEs.

Agreement:

Capture the following for the Conclusions section of the TR (Modifications can be made based on further discussion as necessary):

------------------------------------------------------- Begin Text ------------------------------------------------------

This study focused on the analysis of potential enhancements and solutions necessary to support the high accuracy, low latency, high network efficiency and device efficiency to NR positioning targeting both general commercial and IIOT use cases.

In the study item, Rel-17 target positioning requirements for RAT dependent solutions were discussed and defined for general commercial use cases and IIoT use cases, including horizontal and vertical positioning accuracy, and physical layer and end-to-end positioning latency (see Section 5). Additional scenarios and channel models for evaluating Rel-17 NR positioning enhancements were developed for the evaluation of the achievable positioning performance of the enhancements (see Section 6).

The potential positioning enhancements for improving positioning accuracy, reducing latency, and improving network and device efficiency of NR positioning were studied. The potential positioning enhancements, which were investigated rigorously in this study, as outlined in Section 7.X.

NR positioning accuracy with Rel.16 positioning solutions were evaluated under the condition that gNB time synchronization error and gNB/UE TX/RX timing errors are not modelled for InF-SH scenario and InF-DH scenario for both FR1 and FR2 bands. The evaluation results show:

- For horizontal positioning accuracy,

  - in the InF-SH scenario, based on the results provided [by a majority of sources], sub-meter level @ 90% is achieved in both FR1 and FR2 bands.

  - in the InF-DH scenario, based on the results provided [by a majority of sources], sub-meter level @ 90% is not achieved in both FR1 and FR2 bands.

- For vertical positioning accuracy

  - in the InF-SH scenario,

    - sub-meter level @ 90% is achieved by some sources but not achieved by some other sources in FR1 band

    - sub-meter level @ 90% is achieved by all sources in FR2 band;

  - in the InF-DH scenario,

    - sub-meter level @ 90% is achieved by some sources and is not achieved by some other sources in both FR1 and FR2 bands

For the case without modeling synchronization and gNB/UE TX/RX timing errors in the IOO scenario

- Based on the results provided by a majority of the sources, 1 m level @ 90% of horizontal positioning accuracy is achieved by Rel.16 in IOO scenario

For the case without modeling synchronization and gNB/UE TX/RX timing errors in the UMa scenario

- Based on the results provided, 10 m level @ 90% of horizontal positioning accuracy is achieved by Rel.16 in UMa scenario

For the case without modeling synchronization and gNB/UE TX/RX timing errors in the UMi scenario

- Based on the results provided by some of the companies, 1 m level @ 90% of horizontal positioning accuracy is achieved by Rel.16 in UMi scenario

The impact of NLOS/multipath on NR positioning accuracy and the resolutions for NLOS/multipath mitigation were investigated. NR positioning utilizing LOS/NLOS identification, outlier rejection, NLOS mitigation based on triangle inequality algorithms improve performance of positioning accuracy with respect to solutions that do not apply these techniques.

The impact of gNB/UE TX/RX timing errors on NR positioning accuracy were investigated. Evaluation results show the gNB/UE TX/RX timing errors have significant impact on positioning accuracy.

Aggregation of NR positioning frequency layers for improving positioning accuracy were investigated. Evaluation results show that aggregation of NR positioning frequency layers improves positioning accuracy under certain scenarios, configurations, and assumptions on modelled impairments as outlined in Section [8.4].

Physical layer latency for Rel.16 DL-TDOA/DL-AOD UE-Assisted, UL-TDOA/UL-AOA, Multi-RTT, E-CID and DL-only UE-based NR positioning were investigated, and the major physical layer latency components for these NR positioning techniques were also identified as shown in Section [8.4].

The UE efficiency (power saving) in the RRC_IDLE/RRC_INACTIVE states were also analysed, and power saving gains are observed with detailed observations related to power savings are outlined in Section [8.4].

The network efficiency in terms of resource utilization was analyzed and benefits of potential positioning enhancements observed are outlined in Section [8.4].

The potential positioning enhancements for improving positioning accuracy, reducing latency, and improving network and device efficiency of NR positioning were studied.

The following enhancements have been recommended for normative work

- NR positioning for UEs in RRC_INACTIVE state, including

  - DL, UL and DL+UL positioning methods

  - UE-based and UE-assisted positioning solutions

  - Support of UE positioning measurements for UEs in RRC_inactive state

    - Options that can be considered include DL-PRS or DL-PRS and SSB

  - Support of gNB positioning measurements for UEs in RRC_inactive state

- On-demand transmission and reception of DL PRS, which includes at least

  - UE-initiated request of on-demand DL PRS transmission

  - LMF (network)-initiated request of on-demand DL PRS transmission

  - Above enhancements are recommended for both DL and DL+UL positioning methods and both UE-based and UE-assisted positioning solutions.

- The methods, measurements, signaling, and procedures for improving positioning accuracy in the presence of the UE Rx/Tx timing delays, and/or and gNB Rx/Tx timing delays, including

  - DL, UL and DL+UL positioning methods

  - UE-based and UE-assisted positioning solutions

- The enhancements of the procedure, measurements, reporting, and signalling for improving the accuracy of

  - UL AoA for network-based positioning solutions.

  - DL-AoD for UE-based and network-based (including UE-assisted) positioning solutions

- The enhancements of signaling & procedures for reducing NR positioning latency related to, including DL and DL+UL positioning methods:

  - the measurement gap

  - the measurement request and reporting (e.g., via RRC signaling, MAC-CE and/or physical layer procedure, and/or priority rules)

  - the measurement time

The following enhancements are considered beneficial for the purpose of improving positioning accuracy, reducing latency, improving network and/or device efficiency and are being recommended to be studied further and if needed, specified during normative work

- Simultaneous transmission by the gNB and aggregated reception by the UE of intra-band one or more contiguous carriers in one or more contiguous PFLs

- Simultaneous transmission by the UE and aggregated reception by the gNB of the SRS for positioning in multiple contiguous intra-band carriers

- Enhancements of information reporting from UE and gNB for supporting multipath/NLOS mitigation

- Aperiodic reception of DL PRS from the TRPs of the serving gNB and aperiodic reception of DL PRS from the TRPs of the neighbouring gNBs

- Semi-persistent reception of DL PRS from the TRPs of the serving gNB and Semi-persistent reception of DL PRS from the TRPs of the neighbouring gNBs

- Enhancements of signaling & procedures for reducing NR positioning latency related to

  - the request and response of positioning assistance data (e.g., via RRC signaling, MAC-CE and/or physical layer procedure)

  - the reception of DL PRS (e.g., priority rules for the reception of DL PRS

From a physical layer perspective, it is feasible for a UE to perform DL positioning measurement in RRC_IDLE state. This does not imply that measurements have to be reported in RRC_IDLE state.

It is recommended to proceed with a normative work to support NR positioning enhancements.

------------------------------------------------------- End Text ------------------------------------------------------

### Others

R1-2007667	Discussion on new techniques for NR positioning	vivo

R1-2007756	Channel state estimation based on prior channel information	ZTE

R1-2007861	Discussion of NLOS channel modelling and network time synchronization for NR positioning			CATT

R1-2007910	Polarization-based LOS Detection	FUTUREWEI

R1-2008169	Uplink Transmission Based Relative Positioning	Samsung

R1-2008227	Analysis of NR Positioning Requirements	OPPO

R1-2008302	Additional considerations for positioning enhancements	Nokia, Nokia Shanghai Bell

R1-2008321	Evaluations on additional channel model features	Huawei, HiSilicon

R1-2008492	Discussion on positioning failure and mobility modeling	InterDigital, Inc.

R1-2008766	PRS with cyclic shifts	Ericsson

## Study on Support of Reduced Capability NR Devices

Please refer to RP-201677 for detailed scope of the SI

R1-2007528	TR38.875 skeleton updates for Study on support of reduced capability NR devices	Rapporteur (Ericsson)

The draft TR update in x7528 is endorsed, except the update in Section 3.1.

[103-e-NR-RedCap-01] Eamil discussion for TR38.875 update – Johan (Ericsson)

- 1st check point: 10/29 (particularly related to any previous agreements)

- 2nd check point: 11/12 further update based on the progress during this e-meeting.

Update on 10/29:

- Suggested a definition for RedCap UE – check on 11/2 – update on 11/2, the suggested definition is endorsed, see the further endorsed TR update in R1-2009490.

Further update? Till 11/12.

R1-2007482	FL summary on initial collection of RedCap evaluation results	Moderator (Ericsson, Apple, Qualcomm)

### Potential UE complexity reduction features

R1-2008869	FL summary #1 for Potential UE complexity reduction features for RedCap Moderator (Ericsson)

R1-2009391	FL summary #2 for Potential UE complexity reduction features for RedCap Moderator (Ericsson)

R1-2009393	FL summary #3 for Potential UE complexity reduction features for RedCap Moderator (Ericsson)

R1-2009394	FL summary #4 for Potential UE complexity reduction features for RedCap Moderator (Ericsson)

R1-2009795

[103-e-NR-RedCap-02] Email discussion for potential UE complexity reduction features– Johan (Ericsson)

- 1st check point: 10/29

- 2nd check point: 11/4

- 3rd check point: 11/10

- Last check point 11/12

Agreements:

For evaluating complexity reduction, to come up with a set of combinations of techniques:

- For each case (FR1 FDD, FR1 TDD, & FR2), target up to 6 to 8 combinations

  - Detailed combinations are FFS

Update on 11/4

Agreements:

- Adopt the TP in R1-2009393 as baseline text for TR clause 7.2.1.

- Adopt the TP in R1-2009393 for TR clause 7.3.1.

- Adopt the TP in R1-2009393 as baseline text for TR clause 7.3.2.

  - Companies are invited to double-check their entries in the cost reduction spreadsheet with respect to the above comments (and to catch potential typos).

  - The table will be further updated with potential updated cost estimates.

- Capture the recommendation that maximum bandwidth of an FR1 RedCap UE is 20 MHz during and after initial access.

  - FFS: Whether an FR1 RedCap UE can optionally support a maximum bandwidth larger than 20 MHz after initial access

- Adopt the TP in R1-2009393 as baseline text for TR clause 7.4.1.

- Adopt the updated TP in R1-2009393 as baseline text for TR clause 7.6.1

- Adopt the updated TP in R1-2009393 as baseline text for TR clause 7.6.2.

- Adopt the TP in R1-2009393 as baseline text for TR clause 7.7.2.

  - Companies are invited to double-check their entries in the cost reduction spreadsheet with respect to the above comments (and to catch potential typos).

  - The table will be further updated with potential updated cost estimates.

Other proposals? 11/5

Agreements:

Working assumption: Support that the maximum bandwidth of an FR2 RedCap UE is 100 MHz during initial access and 100MHz after initial access.

Agreements:

For TR section 7.2.2 (on reduced number of Rx antennas), the following combinations of complexity reduction techniques are evaluated.

- FR1 FDD: 1 layer, 1 Rx

- FR1 TDD: 1 layer, 1 Rx

- FR1 TDD: 2 layers, 2 Rx

- FR2: 1 layer, 1 Rx

Agreements: For FR1 FDD, the following combinations of complexity reduction techniques are evaluated:

- 1 layer, 1 Rx, 20 MHz

- 1 layer, 1 Rx, 20 MHz, HD-FDD type A

- 1 layer, 1 Rx, 20 MHz, relaxed modulations for DL & UL

- 1 layer, 1 Rx, 20 MHz, doubled processing time for N1 & N2 only

- 1 layer, 1 Rx, 20 MHz, relaxed modulations for DL & UL, doubled processing time for N1 & N2 only

- 1 layer, 1 Rx, 20 MHz, relaxed modulations for DL & UL, HD-FDD type A, doubled processing time for N1 & N2 only

- 2 layers, 2 Rx, 20 MHz, HD-FDD type A

- 2 layers, 2 Rx, 20 MHz, doubled processing time for N1 & N2 only

Agreements: For FR1 TDD, the following combinations of complexity reduction techniques are evaluated:

- 1 layer, 1 Rx, 20 MHz

- 1 layer, 1 Rx, 20 MHz, relaxed modulations for DL & UL

- 1 layer, 1 Rx, 20 MHz, doubled processing time for N1 & N2 only

- 1 layer, 1 Rx, 20 MHz, relaxed modulations for DL & UL, doubled processing time for N1 & N2 only

- 2 layers, 2 Rx, 20 MHz

- 2 layers, 2 Rx, 20 MHz, relaxed modulations for DL & UL

- 2 layers, 2 Rx, 20 MHz, doubled processing time for N1 & N2 only

- 2 layers, 2 Rx, 20 MHz, relaxed modulations for DL & UL, doubled processing time for N1 & N2 only

Agreements: For FR2, the following combinations of complexity reduction techniques are evaluated:

- 1 layer, 1 Rx, 100 MHz

- 1 layer, 1 Rx, 100 MHz, relaxed modulations DL & UL

- 1 layer, 1 Rx, 100 MHz, doubled processing time for N1 & N2 only

- 1 layer, 1 Rx, 100 MHz, relaxed modulations DL & UL, doubled processing time for N1 & N2 only

- 2 layers, 2 Rx, 100 MHz, relaxed modulations DL & UL

- 2 layers, 2 Rx, 100 MHz, doubled processing time for N1 & N2 only

- 2 layers, 2 Rx, 100 MHz, relaxed modulations DL & UL, doubled processing time for N1 & N2 only

Update on 11/9:

Agreements:

- Adopt the updated TP in x9394 for TR clause 7.7.1

Agreements: For averaging of cost estimates, take the average of all values

Update on 11/11

Agreements (see R1-2009651 for the TPs)

- Adopt the updated TP above for TR clause 6.1.

- Adopt the above description of the benefit of reduced number of UE Rx branches in terms of reducing the device size in FR1 as a baseline text for TR 38.875.

- Adopt the above description of the benefit of reduced number of UE Rx branches in terms of reducing the device size in FR2 as a baseline text for TR 38.875.

- Adopt the TP above as baseline text for TR clause 7.4.2.

- Adopt the above description of the benefit of HD-FDD operation in terms of reducing the device size in FR1 FDD as a baseline text for TR 38.875.

- Adopt the TP above as baseline text for TR clause 7.5.1.

- Adopt the TP above as baseline text regarding relaxed CSI computation, either in TR clause 7.5.1 or in a TR (sub)clause on relaxed CSI computation.

- Adopt the TP above as baseline text for TR clause 7.5.2.

- Confirm the working assumption: Support that the maximum bandwidth of an FR2 RedCap UE is 100 MHz during initial access and 100MHz after initial access.

- Adopt the TPs corresponding to Questions 7.2.3-2/3a/4a/5a/7a in R1-2009651

- Adopt the TPs corresponding to Questions 7.3.3-2/3a/5a/7a in R1-2009651

- Adopt the TPs corresponding to Questions 7.4.3-2a/3a/6/7a in R1-2009651

- Adopt the TP corresponding to Question 7.5.3-3a in R1-2009651

- Adopt the TPs corresponding to Questions 7.6.3-2/3a/4a/5a in R1-2009651

- Adopt the TPs corresponding to Questions 7.7.3-2/4a/5/6a in R1-2009651

Agreements:

- For FR1 FDD bands where a non-RedCap UE is required to be equipped with a minimum of 2 Rx branches,

  - The minimum number of Rx branches supported by specification for a RedCap UE is 1.

  - Specification also supports of 2 Rx branches for a RedCap UE.

Agreements:

- For FR1 TDD bands where a non-RedCap UE is required to be equipped with a minimum of 4 Rx branches, the minimum number of Rx branches supported by specification for a RedCap UE is N. To be down-selected during the WI phase or at RAN plenary:

  - Alt 1: N=2

  - Alt 2: N=1, where N=2 is also supported

Agreements:

- For FR1 FDD bands where a non-RedCap UE is required to be equipped with a minimum of 2 Rx branches,

  - For a RedCap UE with 1 Rx branch, the maximum number of DL MIMO layers is 1.

  - For a RedCap UE with 2 Rx branches, the maximum number of DL MIMO layers is M. Down-select between the following during the WI phase or at RAN plenary

    - Option 1: M=1, where M=2 is also supported

    - Option 2: M=2

Agreements:

- For FR1 TDD bands where a non-RedCap UE is required to be equipped with a minimum of 4 Rx branches,

  - For a RedCap UE with 1 Rx branch (if supported), the maximum number of DL MIMO layers is 1.

  - For a RedCap UE with 2 Rx branches, the maximum number of DL MIMO layers is M. Down-select between the following options during the WI phase or at RAN plenary

    - Option 1: M=1, where M=2 is also supported

    - Option 2: M=2

Agreements:

- For FR2 bands where a non-RedCap UE is required to be equipped with a minimum of 2 Rx branches,

  - For a RedCap UE with 1 Rx branch (if supported), the maximum number of DL MIMO layers is 1.

  - For a RedCap UE with 2 Rx branches (if supported), the maximum number of DL MIMO layers is M. Down-select between the following options during the WI phase or at RAN plenary:

    - Option 1: M=1, where M=2 is also supported

    - Option 2: M=2

Agreements:

- Recommend that HD-FDD type B is not supported for RedCap FR1 FDD UEs in Rel-17.

- Decide at RAN plenary whether to have support FD-FDD or HD-FDD type A or both by specification for an FR1 FDD RedCap UE

Agreements: Decide at RAN plenary whether to support relaxed UE processing time in terms of N1/N2 by specification for a RedCap UE.

Agreements:

- Recommend that support of 256QAM in DL is optional (instead of mandatory) for a FR1 RedCap UE.

- Recommend that relaxed maximum mandatory UL modulation (from 64QAM to 16QAM) is not supported by specification for an FR1 RedCap UE.

- Recommend that relaxed maximum mandatory DL modulation (from 64QAM to 16QAM) is not supported by specification for an FR2 RedCap UE.

- Recommend that relaxed maximum mandatory UL modulation (from 64QAM to 16QAM) is not supported by specification for an FR2 RedCap UE.

Agreements:

- For FR2 bands where a non-RedCap UE is required to be equipped with a minimum of 2 Rx branches,

  - The minimum number of Rx branches supported by specification for a RedCap UE is 1.

  - Specification also supports of 2 Rx branches for a RedCap UE.

- Agree the following TPs in R1-2009652 as baseline for TR 38.875:

  - TP on introduction to UE complexity reduction features in Question 7.1-1

  - TP for TR clause 7.2.2 in Question 7.2.2-1d

  - TP on observations of the impact on coverage for UE with relaxed UE processing time in Question 7.5.3-2a

  - TP on observation of the coexistence impacts for UE with relaxed maximum number of MIMO layers in Question 7.6.4-2

  - TP on observation of specification impacts for UE with relaxed maximum number of MIMO layers in Question 7.6.5-2

  - TP on observations of the impact on network capacity and spectral efficiency for UE with relaxed maximum modulation orders in Question 7.7.3-3a

  - TP on observation of coexistence impacts for UE with relaxed maximum modulation orders in Question 7.7.4-2

  - TP on description of combinations of UE complexity reduction techniques in Question 7.8.1-1

  - TP for TR clause 7.8.2 in Proposal 7.8.2-1a

  - TP on performance impacts for combinations of UE complexity reduction techniques in Question 7.8.3-2

  - TP on coexistence impacts for combinations of UE complexity reduction techniques in Question 7.8.4-1

  - TP on specification impacts for combinations of UE complexity reduction techniques in Question 7.8.5-1

Agreements:

- Agree the following TPs in R1-2009795 as baseline for TR 38.875:

  - TP on observations of specification impacts of UE bandwidth reduction in Question 7.3.5-2a

  - TP on observations of the impact on latency and reliability for UE with relaxed UE processing time in Question 7.5.3-5b

  - TP on observations of specification impacts for UE with relaxed maximum modulation orders in Question 7.7.5-2a

  - TP on peak data rate impacts for combinations of UE complexity reduction techniques in Question 7.8.3-1a

R1-2008837	Potential UE complexity reduction features for RedCap	Ericsson

Revision of R1-2007529

R1-2007534	Complexity reduction features for RedCap UEs	FUTUREWEI

R1-2009318	Potential UE complexity reduction features	Huawei, HiSilicon

Revision of R1-2007596

R1-2007668	Complexity reduction for Reduced Capability NR devices	vivo, Guangdong Genius

R1-2007715	Potential UE complexity reduction features	ZTE

R1-2007862	Discussion on UE complexity reduction features	CATT

R1-2007887	Potential UE complexity reduction features	TCL Communication Ltd.

R1-2007947	On potential UE complexity reduction features for RedCap	Intel Corporation

R1-2008016	Discussion on  UE complexity reduction features	CMCC

R1-2008048	Discussion on potential UE complexity reduction features	LG Electronics

R1-2008068	UE complexity reduction features	Nokia, Nokia Shanghai Bell

R1-2008084	Discussion on the complexity reduction for reduced capability device	Xiaomi

R1-2008100	Discussion on potential UE complexity reduction features	Spreadtrum Communications

R1-2008114	Discussion on bandwidth related features for RedCap devices	NEC

R1-2008170	UE complexity reduction	Samsung

R1-2008260	Discussion on UE complexity reduction	OPPO

R1-2008294	UE complexity reduction features for RedCap	Lenovo, Motorola Mobility

R1-2008315	Reduced Capability UE Complexity Reduction Features	Sierra Wireless, S.A.

R1-2008366	On potential complexity reduction techniques for NR devices	Sony

R1-2008382	Discussion on potential UE complexity reduction features	Panasonic Corporation

R1-2008394	Discussion on Potential UE complexity reduction features	Sharp

R1-2008469	Potential UE complexity reduction features for RedCap	Apple

R1-2008510	On complexity reduction features for NR RedCap UEs	MediaTek Inc.

R1-2008551	Discussion on potential UE complexity reduction features for RedCap	NTT DOCOMO, INC.

R1-2008581	Discussion on potential UE complexity reduction features	ASUSTeK

R1-2008620	Complexity Reduction for RedCap Devices	Qualcomm Incorporated

R1-2008684	UE complexity reduction features for reduced capability NR devices	InterDigital, Inc.

R1-2008738	Complexity reduction features for RedCap UE	Sequans Communications

### Reduced PDCCH monitoring

R1-2008471	Feature lead summary #1 on reduced PDCCH monitoring	Moderator (Apple)

[103-e-NR-RedCap-03] Email discussion for reduced PDCCH monitoring– Hong (Apple)

- 1st check point: 10/29

- 2nd check point: 11/4

- 3rd check point: 11/10

- Last check point 11/12

R1-2009370

Agreements:

- To include description of the evaluated schemes #1/#2/#3 as in R1-2009370 to the TR

  - Further discussion the detailed text proposal for these schemes

  - Note: the description for scheme #1 is taken as a higher priority than #2/#3

Agreements:

- Determine the Xx (smallest power saving gain)-Yy (largest power saving gain) value based on the smallest and largest values reported by each company at least considering:

  - Separate observations with corresponding Xx-Yy values are captured at least for cross-slot and same slot scheduling cases.

  - Separate observations for FR1 & FR2

  - Additonal cases for separate observations

- Capture average/mean value of Xx-Yy excluding the smallest and the largest values among companies.

- Explicitly mention the result/observations if it was provided by a few source companies e.g. 1 or 2 with special setup or assumptions.

- Highlighting the gain is compared to the UE with configuring the maximum blind decoding for PDCCH monitoring defined in Rel-15/Rel-16

Agreements:

Incorporate the revised Table 2A/2B and Table 3A/3B in R1-2009493 into Redcap TR 38.875 as baseline.

- It is up to TR editor to use a separate excel sheet to include these Tables or directly capture these tables for inclusion in the TR.

- The table will be further updated with potential updated power saving gain results.

Agreements:

For FR1, capture the following observations in the TR (editorial modifications by TR editor can be made for inclusion in the TR)

- 11 sources ([vivo], [Ericsson], [Qualcomm], [CATT], [Spreadtrum], [OPPO], [Huawei, HiSilicon], [Apple], [Futurewei],[Intel], [ZTE]) reported the evaluation results of power saving gain for FR1 with same-slot scheduling for the 1 Rx antenna case.

The following is observed for 1 Rx antenna case:

  - For the instant message traffic model, with reducing maximum PDCCH blind decoding (i.e. 36) by 25% and 50%, the power saving gains are in the range of approximately [0.7%~5.7%] and [1.3%~11.4%], respectively.  With excluding the smallest and the largest values among sources, the mean value of power saving gain with reducing maximum PDCCH blind decoding (i.e. 36) by 25% and 50% are approximately 2.84% and 5.91%, respectively.

  - For the heartbeat traffic model with 200ms inactivity timer configuration, with reducing maximum PDCCH blind decoding (i.e. 36) by 25% and 50%, the power saving gains are in the range of approximately [0.01%~3.40%] and [0.02%~6.80%], respectively. With excluding the smallest and the largest values among sources, the mean value of power saving gain by reducing maximum PDCCH blind decoding (i.e. 36) by 25% and 50% are approximately 1.59% and 3.33%, respectively.

  - For the heartbeat traffic model with 80ms inactivity timer configuration, with reducing maximum PDCCH blind decoding (i.e. 36) by 25% and 50%, the power saving gains are in the range of approximately [0.01%~3.20%] and [0.02%~6.40%], respectively.  With excluding the smallest and the largest values among sources, the mean value of power saving gain with reducing maximum PDCCH blind decoding (i.e. 36) by 25% and 50% are approximately 1.41% and 3.06%, respectively.

  - For the VoIP traffic model, with reducing maximum PDCCH blind decoding (i.e. 36) by 25% and 50%, the power saving gains are in the range of approximately [0.90%~3.88%] and [1.82%~6.48%], respectively. With excluding the smallest and the largest values among sources, the mean value of power saving gain with reducing maximum PDCCH blind decoding (i.e. 36) by 25% and 50% are approximately 2.59% and 4.74%, respectively.

- 13 sources ([vivo], [Ericsson], [Qualcomm], [Nokia], [CATT], [Spreadtrum], [OPPO], [Huawei, HiSilicon], [Apple], [Futurewei], [Intel], [ZTE], [InterDigital]) reported the evaluation results of power saving gain for FR1 with same-slot scheduling for 2 Rx antennas cases.

The following is observed for 2 Rx antennas case:

- For the instant message traffic model, with reducing maximum PDCCH blind decoding (i.e. 36) by 25% and 50%, the power saving gains are in the range of approximately [0.64%~6.20%] and [1.55%~12.30%], respectively.  With excluding the smallest and the largest values among sources, the mean value of power saving gain with reducing maximum PDCCH blind decoding (i.e. 36) by 25% and 50% are approximately 3.20% and 6.85%.

- For the heartbeat traffic model with 200ms inactivity timer configuration, with reducing maximum PDCCH blind decoding (i.e. 36) by 25% and 50%, the power saving gains are in the range of approximately [0.01%~4.10%] and [0.02%~8.20%], respectively.  With excluding the smallest and the largest values among sources, the mean value of power saving gain with reducing maximum PDCCH blind decoding (i.e. 36) by 25% and 50% are approximately 1.65% and 3.92%, respectively.

- For the heartbeat traffic model with 80ms inactivity timer configuration maximum PDCCH blind decoding (i.e. 36) by 25% and 50%, the power saving gains are in the range of approximately [0.01%~3.90%] and [0.02%~7.80%], respectively.  With excluding the smallest and the largest values among sources, the mean value of power saving gain with reducing maximum PDCCH blind decoding (i.e. 36) by 25% and 50% are approximately 1.49% and 3.62%, respectively.

- For the VoIP traffic model, with reducing maximum PDCCH blind decoding (i.e. 36) by 25% and 50%, the power saving gains are in the range of approximately [??%-??%] and [??%~??%].  With excluding the smallest and the largest values among sources, the mean value of power saving gain with reducing maximum PDCCH blind decoding (i.e. 36) by 25% and 50% are approximately 2.85% and 5.66%, respectively.

Agreements:

For FR1, capture the following observations in the TR (editorial modifications by TR editor can be made for inclusion in the TR)

- 8 sources ([vivo], [Ericsson], [Samsung], [Qualcomm], [OPPO], [Apple], [ZTE], [MediaTek]) reported the evaluation results of power saving gain for FR1 with cross-slot scheduling for the 1 Rx antenna and 2 Rx antennas cases.

The following is observed for 1 Rx antenna case:

  - For the instant message traffic model, with reducing maximum PDCCH blind decoding (i.e. 36) by 25% and 50%, the power saving gains are in the range of approximately [0.66%~4.5%] and [0.81%~9%], respectively. With excluding the smallest and the largest values among sources, the mean value of power saving gain with reducing maximum PDCCH blind decoding (i.e. 36) by 25% and 50% are approximately 2.79% and 4.64%, respectively.

  - For the heartbeat traffic model with 200ms inactivity timer configuration, with reducing maximum PDCCH blind decoding (i.e. 36) by 25% and 50%, the power saving gains are in the range of approximately [0.01%~2.7%] and [0.01%~5.5%], respectively  With excluding the smallest and the largest values among sources, the mean value of power saving gain with reducing 36 PDCCH blind decoding by 25% and 50% are approximately 1.81% and 3.26%, respectively.

  - For the heartbeat traffic model with 80ms inactivity timer configuration, with reducing maximum PDCCH blind decoding (i.e. 36) by 25% and 50%, the power saving gains are in the range of approximately [0.01%~2.6%] and [0.01%~5.1%], respectively.  With excluding the smallest and the largest values among sources, the mean value of power saving gain with reducing maximum PDCCH blind decoding (i.e. 36) by 25% and 50% are approximately 1.8% and 3.35%, respectively.

  - For the VoIP traffic model, with reducing maximum PDCCH blind decoding (i.e. 36) by 25% and 50%, the power saving gains are in the range of approximately [0.87%~4.5%] and [1.39%~7%], respectively.  With excluding the smallest and the largest values among sources, the mean value of power saving gain with reducing maximum PDCCH blind decoding (i.e. 36) by 25% and 50% are approximately 2.29% and 3.20%, respectively.

The following is observed for 2 Rx antennas case:

- For the instant message traffic model, with reducing maximum PDCCH blind decoding (i.e. 36) by 25% and 50%, the power saving gains are in the range of approximately [0.77%~4.69%] and [1.44%~9.38%], respectively. With excluding the smallest and the largest values among sources, the mean value of power saving gain with reducing maximum PDCCH blind decoding (i.e. 36) by 25% and 50% are approximately 3.31% and 6.13%, respectively.

- For the heartbeat traffic model with 200ms inactivity timer configuration, with reducing maximum PDCCH blind decoding (i.e. 36) by 25% and 50%, the power saving gains are in the range of approximately [0.01%~2.9%] and [0.02%~5.7%], respectively  With excluding the smallest and the largest values among sources, the mean value of power saving gain with reducing maximum PDCCH blind decoding (i.e. 36) by 25% and 50% are approximately 1.95% and 3.51%, respectively.

- For the heartbeat traffic model with 80ms inactivity timer configuration, with reducing maximum PDCCH blind decoding (i.e. 36) by 25% and 50%, the power saving gains are in the range of approximately [0.01%~2.5%] and [0.02%~4.94%], respectively.  With excluding the smallest and the largest values among sources, the mean value of power saving gain with reducing maximum PDCCH blind decoding (i.e. 36) by 25% and 50% are approximately 1.69% and 3.21%, respectively.

- For the VoIP traffic model, with reducing maximum PDCCH blind decoding (i.e. 36) by 25% and 50%, the power saving gains are in the range of approximately [0.83%~3.5%] and [1.65%~6.07%], respectively.  With excluding the smallest and the largest values among sources, the mean value of power saving gain with reducing maximum PDCCH blind decoding (i.e. 36) by 25% and 50% are approximately 2.28% and 4.45%, respectively.

- In general, it is expected that the power saving gain by BD reduction for cross-slot scheduling is less than that of the same-slot scheduling.

- In general, it is expected that the power saving gain by BD reduction for 1 Rx case is less than that of the 2 Rx case.

Agreements:

Incorporate the revised Table 4A/4B and Table 5A/5B in R1-2009493 into Redcap TR 38.875.

- It is up to TR editor to use a separate excel sheet to include these Tables or directly capture these tables for inclusion in the TR.

- The table will be further updated with potential updated power saving gain results.

- Note for Tables 4A & 5A, with the following update

1 packet requires 1 PDSCH for Heartbeat traffic model; 1 packet requires 24 16 PDSCHs for IM model, assuming cell center UE.

Agreements:

Fo FR2, capture the following observations in the TR (editorial modifications by TR editor can be made for inclusion in the TR)

- 6 sources ([Ericsson], [CATT], [Spreadtrum], [Futurewei], [Intel], [ZTE]) reported the evaluation results of power saving gain for FR2 with same-slot scheduling for the 1 Rx antenna and 2 Rx antennas cases.

The following is observed for 1 Rx antenna case:

  - For the instant message traffic model, with reducing maximum PDCCH blind decoding (i.e. 20) by 25% and 50%, the power saving gains are in the range of approximately [1.94%~6.6%] and [3.59%~13.1%], respectively.  With excluding the smallest and the largest values among sources, the mean value of power saving gain with reducing maximum PDCCH blind decoding (i.e. 20) by 25% and 50% are approximately 4.77% and 9.60%, respectively.

  - For the heartbeat traffic model with 200ms inactivity timer configuration, with reducing maximum PDCCH blind decoding (i.e. 20) by 25% and 50%, the power saving gains are in the range of approximately [0.03%~4.30%] and [0.07%~8.60%], respectively. With excluding the smallest and the largest values among sources, the mean value of power saving gain by reducing maximum PDCCH blind decoding (i.e. 20) by 25% and 50% are approximately 2.14% and 4.41%, respectively.

  - For the heartbeat traffic model with 80ms inactivity timer configuration, with reducing maximum PDCCH blind decoding (i.e. 20) by 25% and 50%, the power saving gains are in the range of approximately [0.03%~4%] and [0.06%~7.9%], respectively.  With excluding the smallest and the largest values among sources, the mean value of power saving gain with reducing maximum PDCCH blind decoding (i.e. 20) by 25% and 50% are approximately 1.60% and 3.21%, respectively.

  - For the VoIP traffic model, with reducing maximum PDCCH blind decoding (i.e. 20) by 25% and 50%, the power saving gains are in the range of approximately [2.52%~5%] and [4.66%~9.4%], respectively. With excluding the smallest and the largest values among sources, the mean value of power saving gain with reducing maximum PDCCH blind decoding (i.e. 20) by 25% and 50% are approximately 3.81% and 7.43%, respectively.

The following is observed for 2 Rx antennas case:

  - For the instant message traffic model, with reducing maximum PDCCH blind decoding (i.e. 20) by 25% and 50%, the power saving gains are in the range of approximately [2.45%~6.8%] and [4.54%~13.6%], respectively.  With excluding the smallest and the largest values among sources, the mean value of power saving gain with reducing maximum PDCCH blind decoding (i.e. 20) by 25% and 50% are approximately 4.94% and 9.87%, respectively.

  - For the heartbeat traffic model with 200ms inactivity timer configuration, with reducing maximum PDCCH blind decoding (i.e. 20) by 25% and 50%, the power saving gains are in the range of approximately [0.04%~4.90%] and [0.10%~11.90%], respectively. With excluding the smallest and the largest values among sources, the mean value of power saving gain by reducing maximum PDCCH blind decoding (i.e. 20) by 25% and 50% are approximately 2.55% and 4.95%, respectively.

  - For the heartbeat traffic model with 80ms inactivity timer configuration, with reducing maximum PDCCH blind decoding (i.e. 20) by 25% and 50%, the power saving gains are in the range of approximately [0.04%~4.6%] and [0.09%~9.2%], respectively.  With excluding the smallest and the largest values among sources, the mean value of power saving gain with reducing maximum PDCCH blind decoding (i.e. 20) by 25% and 50% are approximately 2.38% and 4.64%, respectively.

  - For the VoIP traffic model, with reducing maximum PDCCH blind decoding (i.e. 20) by 25% and 50%, the power saving gains are in the range of approximately [3.10%~5.5%] and [5.74%~10.5%], respectively. With excluding the smallest and the largest values among sources, the mean value of power saving gain with reducing maximum PDCCH blind decoding (i.e. 20) by 25% and 50% are approximately 4.27% and 8.27%, respectively.

Agreements:

For FR2, capture the following observations in the TR (editorial modifications by TR editor can be made for inclusion in the TR)

- 4 sources ([Ericsson], [Samsung], [ZTE], [MediaTek]) reported the evaluation results of power saving gain for FR2 with cross-slot scheduling for the 1 Rx antenna and 2 Rx antennas cases.

The following is observed for 1 Rx antenna case:

  - For the instant message traffic model, with reducing maximum PDCCH blind decoding (i.e. 20) by 25% and 50%, the power saving gains are in the range of approximately [1.40%~6.30%] and [2.70%~12.7%], respectively.  With excluding the smallest and the largest values among sources, the mean value of power saving gain with reducing maximum PDCCH blind decoding (i.e. 20) by 25% and 50% are approximately 3.64% and 7.04%, respectively.

  - For the heartbeat traffic model with 200ms inactivity timer configuration, with reducing maximum PDCCH blind decoding (i.e. 20) by 25% and 50%, the power saving gains are in the range of approximately [0.02%~4.20%] and [0.04%~8.30%], respectively. With excluding the smallest and the largest values among sources, the mean value of power saving gain by reducing maximum PDCCH blind decoding (i.e. 20) by 25% and 50% are approximately 1.30% and 2.60%, respectively.

  - For the heartbeat traffic model with 80ms inactivity timer configuration, with reducing maximum PDCCH blind decoding (i.e. 20) by 25% and 50%, the power saving gains are in the range of approximately [0.02%~3.9%] and [0.04%~7.6%], respectively.  With excluding the smallest and the largest values among sources, the mean value of power saving gain with reducing maximum PDCCH blind decoding (i.e. 20) by 25% and 50% are approximately 1.24% and 2.48%, respectively.

  - For the VoIP traffic model, with reducing maximum PDCCH blind decoding (i.e. 20) by 25% and 50%, the power saving gains are in the range of approximately [1.94%~6.5%] and [3.6%~13.1%], respectively. With excluding the smallest and the largest values among sources, the mean value of power saving gain with reducing maximum PDCCH blind decoding (i.e. 20) by 25% and 50% are approximately 3.27% and 6.33%, respectively.

The following is observed for 2 Rx antennas case:

  - For the instant message traffic model, with reducing maximum PDCCH blind decoding (i.e. 20) by 25% and 50%, the power saving gains are in the range of approximately [1.89%~6.6%] and [3.50%~13.20%], respectively.  With excluding the smallest and the largest values among sources, the mean value of power saving gain with reducing maximum PDCCH blind decoding (i.e. 20) by 25% and 50% are approximately 3.81% and 7.37%, respectively.

  - For the heartbeat traffic model with 200ms inactivity timer configuration, with reducing maximum PDCCH blind decoding (i.e. 20) by 25% and 50%, the power saving gains are in the range of approximately [0.03%~4.90%] and [0.07%~9.60%], respectively. With excluding the smallest and the largest values among sources, the mean value of power saving gain by reducing maximum PDCCH blind decoding (i.e. 20) by 25% and 50% are approximately 1.56% and 3.13%, respectively.

  - For the heartbeat traffic model with 80ms inactivity timer configuration, with reducing maximum PDCCH blind decoding (i.e. 20) by 25% and 50%, the power saving gains are in the range of approximately [0.03%~4.6%] and [0.06%~8.9%], respectively.  With excluding the smallest and the largest values among sources, the mean value of power saving gain with reducing maximum PDCCH blind decoding (i.e. 20) by 25% and 50% are approximately 1.37% and 2.74%, respectively.

  - For the VoIP traffic model, with reducing maximum PDCCH blind decoding (i.e. 20) by 25% and 50%, the power saving gains are in the range of approximately [1.97%~6.8%] and [3.95%~13.7%], respectively. With excluding the smallest and the largest values among sources, the mean value of power saving gain with reducing maximum PDCCH blind decoding (i.e. 20) by 25% and 50% are approximately 3.38% and 6.52%, respectively.

- In general, it is expected that the power saving gain by BD reduction for cross-slot scheduling is less than that of the same-slot scheduling.

- In general, it is expected that the power saving gain by BD reduction for 1 Rx case is less than that of the 2 Rx case.

R1-2009571

Agreements: Using both absolute increase and relative increase (as summarized in R1-2009571) to capture the observations for PDCCH blocking rate increase into TR 38.875.

Agreements: Separate the following observations to capture the PDCCH blocking rate increase into TR 38.875:

  - Separate observations for Aggregation Level (AL) distributions for AL [1,2,4,8,16] i.e. C1/C2/C3/Others.

  - Separate observations for number of simultaneously scheduled UEs X.

  - Separate observations for 25% and 50% reduction in BD limit.

  - FFS separate observations for baseline parameters and optional parameters, including comparison between baseline parameters and optional parameters.

R1-2009659

Agreements:

- For each of the simultaneously scheduled UE numbers denoting as ‘N’ (1<N<=10)

  - Step 1: Determine a single average/mean value Average_a_N(i) based on values reported by each company ‘i’ with existing Rel-15/16 schemes for DCI transmission

  - for company ‘j’. M represents the number of configurations that are simulated by company ‘j’ for ‘N’ simultaneously scheduled UEs in a slot.

  - Step 2: Determine a single average/mean value Average_a_N by averaging the values from different companies for a sperate observation, excluding the smallest and the largest values of Average_a(i)_N among companies if number of source companies > 3.

    - where ‘K’ denotes the number of source companies that simulated a same observation configuration (e.g. ‘N=2’ in Table 10A) after excluding the smallest and largest value.

  - Step-3: Reuse the same approach to derive the Average_b_N for Case 2 and Case 3 with approximately 25% and 50% BD reduction.

  - Step-4: Determine the absolute increase and relative increase as follows:

    - X_N% = [Average_b_N - Average_a_N].

    - Y_N% = [(Average_b_N - Average_a_N)/ Average_a_N ]

  - Step-5: Capture the following into TR for PDCCH blocking rate impact based on the template in Q 8.2.3.1-1

For FR1 with AL distribution configuration A1 in Table 8 with ‘N’ simultaneously scheduled UE in a slot, it was observed that the PDCCH blocking rate is increased X_N% from [Average_a_N] which corresponds to Y_N% increase relative to [Average_a_N]

Agreements: Capturing the following formulation for PDCCH blocking rate impact observations decoding into TR 38.875 section 8.2.3.1.

-        The observation for PDCCH blocking rate impact is formulated using the vector format: <N, A%,  z1%, x1%,y1%,z2%,x2%,y2%>, which represents the following:

§       With N simultaneously scheduled UEs in a slot and z1% reduction in maximum PDCCH blind decoding, the PDCCH blocking rate is increased approximately x1% from A%, which corresponds to y1% increase relative to A%. With N simultaneously scheduled UEs in a slot and z2% reduction in maximum PDCCH blind decoding, the PDCCH blocking rate is increased approximately x2% from A%, which corresponds to y2% increase relative to A%.

Agreements: To include evaluation results for PDCCH AL distributions of AL configurations A1~A7 of Table 8 in R1-2009659 to the TR 38.875.

Agreements: To include evaluation results for number of PDCCH Candidates for AL [1,2,4,8,16] of Table 9 in R1-2009659 to the TR 38.875.

R1-2009720

Agreements: Adopt the proposal 8.2.3.1 in R1-2009720 for TR 38.875 clause 8.2.3.1.

- Note: the results for A2/A3 may not represent a typical case, e.g., because of the assumptions of unfavorable channel conditions.

Ageements: Capture the following note into TR 38.875 clause 8.2.3:

For the cases where the number of PDCCH candidates per AL is more than 8, the following configuration should be assumed, i.e., multiple overlapping search space sets are allowed.

Agreements: For FR1, capture the following updated observations in the TR (editorial modifications by TR editor can be made for inclusion in the TR) for same-slot scheduling with 2 Rx antennas:

§  For the VoIP traffic model, with reducing maximum PDCCH blind decoding (i.e. 36) by 25% and 50%, the power saving gains are in the range of approximately [1.16%~4.60%] and [2.32%~7.20%].  With excluding the smallest and the largest values among sources, the mean value of power saving gain with reducing maximum PDCCH blind decoding (i.e. 36) by 25% and 50% are approximately 2.85% and 5.66%, respectively.

R1-2009766

Agreements:

Captured the following into TR 38.875 for section 8.2.4

- The potential impacts on legacy UEs, in terms of PDCCH blocking probability, when coexisting with RedCap UEs in a shared CORESET depend on the scheduling strategy and system parameters. Depending on the network implementation, if legacy UEs are prioritized over RedCap UEs, there is no any coexistence impact on the legacy UEs at the cost of increased latency at the Redcap device side.

Agreements:

Capture the following feature description for Scheme #3 in the TR:

Scheme #3: Dynamic adaptation of PDCCH Blind Decoding (BD) parameters in connected mode In Rel-15/16, the parameters of PDCCH monitoring is configured by RRC signaling on a per search space set basis. Scheme #3 is to dynamically adapt PDCCH BD parameters e.g. maximum number of PDCCH candidates per PDCCH monitoring occasion and minimum time separation between two consecutive PDCCH monitoring occasions. For example, to address real-time traffic variations on a cell or for a UE while accounting for blocking, a gNB can indicate reduced/full PDCCH BD on the cell to the UE when traffic is low/high.

Agreements:

Capture the following feature description for Scheme #1 in the TR:

Scheme #1: Reduced maximum number of Blind Decoding (BD) per slot in connected mode In Rel-15 and Rel-16 NR, the limits on maximum number of BDs per slot is configurable up to the limits are defined for different SCS configurations, as summarized in Error: Reference source not found. Scheme #1 reduces the maximum number of BDs in a slot. In Rel-15 and Rel-16 specifications, the total number of different DCI sizes configured to monitor is up to 4 with up to 3 different DCI sizes with C-RNTI. Two alternatives were studied under Scheme #1, which includes reduced maximum number of BDs per slot with additionally reduced DCI size budget (Alt.1a) and reduced maximum number of BDs per slot without reduced DCI size budget (Alt.1b). Table 1: Blind decoding limits in NR. SCS [kHz]; 15; 30; 60; 120. Max # BD per slot (in NR); 44; 36; 22; 20.

Agreements: Capture the following feature description for Scheme #2 in the TR:

In Rel-15/16 NR, the range of PDCCH monitoring periodicity is configurable, which is in a range of a few symbol (s) to 2560 slots subject to UE capability. Scheme#2 is to extend the minimum separation between two consecutive PDCCH monitoring occasions, spans or slots with configured PDCCH candidates to be X slots, where X>1.

Agreements: Capturing the following into TR 38.875 for latency impact:

The latency impact due to BD reduction may largely depend on PDCCH blocking rate performance impact. If the PDCCH blocking rate is increased by BD reduction, the latency performance is expected to be increased; Otherwise, BD reduction has no impact on the latency.

R1-2007530	Reduced PDCCH monitoring for RedCap	Ericsson

R1-2007535	Power savings for RedCap UEs	FUTUREWEI

R1-2007597	Power saving for reduced capability devices	Huawei, HiSilicon

R1-2007625	Discussion on PDCCH monitoring reduction for RedCap UEs	Panasonic

R1-2007669	Reduced PDCCH monitoring for Reduced Capability NR devices	vivo, Guangdong Genius

R1-2007716	Consideration on reduced PDCCH monitoring	ZTE

R1-2007863	Discussion on PDCCH monitoring reduction	CATT

R1-2007888	Reduced PDCCH monitoring	TCL Communication Ltd.

R1-2007948	On reduced PDCCH monitoring for RedCap UEs	Intel Corporation

R1-2008017	Discussion on PDCCH monitoring reduction	CMCC

R1-2008049	Discussion on PDCCH monitoring for reduced capability NR devices	LG Electronics

R1-2008069	Reduced PDCCH monitoring	Nokia, Nokia Shanghai Bell

R1-2008085	Discussion on reduced PDCCH monitoring for reduced capability device	Xiaomi

R1-2008105	Discussion on reduced PDCCH monitoring	Spreadtrum Communications

R1-2008115	Reduced PDCCH monitoring for REDCAP NR devices	NEC

R1-2008171	Reduced PDCCH monitoring	Samsung

R1-2008261	Solutions of reduced PDCCH monitoring	OPPO

R1-2008336	PDCCH monitoring at reduced capability UE	Lenovo, Motorola Mobility

R1-2008395	Reduced PDCCH Monitoring for RedCap Devices	Sharp

R1-2008470	Reduced PDCCH Monitoring for RedCap Devices	Apple

R1-2008511	Discussion on reduced PDCCH monitoring for NR RedCap UEs	MediaTek Inc.

R1-2008552	Discussion on reduced PDCCH monitoring for RedCap	NTT DOCOMO, INC.

R1-2008621	PDCCH Monitoring Reduction and Power Saving for RedCap Devices	Qualcomm Incorporated

R1-2008685	Reduced PDCCH monitoring for reduced capability NR devices	InterDigital, Inc.

R1-2008712	Reduced PDCCH Monitoring for RedCap UEs	Fraunhofer HHI, Fraunhofer IIS

R1-2008727	Discussion on PDCCH monitoring for RedCap UE	WILUS Inc.

R1-2008739	Reduced PDCCH monitoring for RedCap UE	Sequans Communications

### Coverage recovery and capacity impact

Including aspects related to evaluations of the impact to coverage, network capacity and spectral efficiency

R1-2009311	FL summary #1 on Coverage Recovery and Capacity Impact for RedCap Moderator (Qualcomm Inc.)

R1-2009365	FL summary #2 on Coverage Recovery and Capacity Impact for RedCap Moderator (Qualcomm Inc.)

R1-2009479	FL summary #3 on Coverage Recovery and Capacity Impact for RedCap Moderator (Qualcomm Inc.)

R1-2009580	FL summary #4 on Coverage Recovery and Capacity Impact for RedCap Moderator (Qualcomm Inc.)

R1-2009721

R1-2009722

[103-e-NR-RedCap-04] Email discussion for coverage recovery and capacity impact– Chao (Qualcomm)

- 1st check point: 10/29

- 2nd check point: 11/4

- 3rd check point: 11/10

- Last check point 11/12

Agreements:

- If coverage recovery target performance requirement is based on Option 1

  - Maximum pathloss loss (MPL) is used as the coverage evaluation metric

- If coverage recovery target performance requirement is based on Option 3

  - Maximum isotropic loss (MIL) is used as the coverage evaluation metric

Agreements:

- For Option 3, down-selection on the following alternatives for coverage recovery

  - Alt 1: A single coverage recovery target based on the same bottleneck channel is used for initial access channels and non-initial access channels of RedCap UE

  - Alt 2: Identify 2 coverage recovery targets for the RedCap UE initial access channels and non-initial access channels, respectively:

  - The 1st target is based on the bottleneck channel among the initial access channels of the reference NR UE

  - The 2nd target is based on the bottleneck channel among all the channels of the reference NR UE

  - Note: The initial access channels include at least PBCH, PRACH, Msg2, Msg3, Msg4 and PDCCH CSS

Agreements:

- Agree in principle using Option 3 for determining the coverage recovery target

  - Option 3: The coverage recovery target for each channel of RedCap UE corresponds to the link budget of the bottleneck channel(s) for the reference NR UE within the same deployment scenario

  - Note: The reference UE is a Rel-15/16 NR UE with mandatory features only

- FFS For Option 3, companies report their individual observations of the amount of compensation for each channel by comparing the link budget with that of the bottleneck channel for the reference NR UE (i.e. the LB of the channel for RedCap UE – the LB of the bottleneck channel for the reference UE)

  - A representative value of the amount of compensation is derived by taking the mean value (in dB domain) from all the compensation values including both negative and non-negative values

    - Excluding the highest & the lowest values when the number of samples is more than 3

    - If the number of samples used to compute a representative value is less than 4 for each scenario, this representative value is not used for bottleneck identification

    - In this case, observations may still be drawn

  - The representative value of a channel is used for identifying whether the channel needs coverage recovery

    - Coverage recovery is not needed if the representative value of a channel is larger than or equal to zero

Agreements:

- For Option 3, companies report their individual observations of the amount of coverage loss for each channel by comparing the link budget with that of the bottleneck channel for the reference NR UE (i.e. the LB of the channel for RedCap UE – the LB of the bottleneck channel for the reference UE)

  - A representative value of the amount of coverage loss is derived by taking the mean value (in dB domain) from all the compensation values including both negative and non-negative values

    - Excluding the highest & the lowest values when the number of samples is more than 3

    - If the number of samples used to compute a representative value is less than 4 for each scenario, this representative value is not used for bottleneck identification

    - In this case, observations may still be drawn

  - The representative value of a channel is used for identifying whether the channel needs coverage recovery

    - Coverage recovery is not needed if the representative value of a channel is larger than or equal to zero

    - The amount of coverage recovery to recommend will depend on further discussion of the techniques, scenarios, etc

Agreements:

- Capture the following to the TR 38.875

  - Coverage recovery for Msg2 PDSCH was studied from several aspects, including TBS scaling [and Msg2 PDSCH repetition]

  - It is noted that TBS scaling is an existing technique mandatory for Rel-15 UE

  - Potential specification impacts of Msg2 PDSCH repetition (if supported) include

    - Msg2 PDSCH repetition configuration

    - Mechanism to differentiate enhanced UE and legacy UE, e.g., separate PRACH configurations (e.g, separate PRACH occasions or preambles)

Agreements:

- Capture the following to the TR 38.875

  - Coverage recovery for Msg4 PDSCH was studied from several aspects, including scaling factor for TBS determination, PDSCH repetition and the use of the lower-MCS table.

  - Some techniques, such as scaling factor for TBS determination and PDSCH repetition have been studied also in the Rel-17 coverage enhancement SI

  - Potential specification impacts of using the lower-MCS table for Msg4 PDSCH include

    - Related signaling design

Update on 11/11

Agreements:

  - Capture the link budget evaluation results (Urban 2.6 GHz) in Table 3.1-1 to Table 3.1-3 in R1-2009660 to the Appendix of TR 38.875

    - The tables will be further updated with potential updated evaluation results (to catch potential typos) and a clarification of assumption for Msg2 and PRACH.

    - MPL results to be included also. Up to editor to use the same or different tables

Agreements:

Adopted the updated TP in section 3.1 of R1-2009660 as baseline text for TR clause 9.1

  - Remove “and coverage recovery is needed” from the TP

Agreements:

  - Capture the link budget evaluation results (rural 0.7 GHz) in Table 3.2-1 to Table 3.2-3 in R1-2009660 to the Appendix of TR 38.875

    - The tables will be further updated with potential updated evaluation results (to catch potential typos) and a clarification of assumption for Msg2 and PRACH.

    - MPL results to be included also. Up to editor to use the same or different tables.

Agreements:

Adopted the updated TP in section 3.2 of R1-2009660 as baseline text for TR clause 9.1

  - Remove “and coverage recovery is needed” from the TP

Agreements:

- Capture the link budget evaluation results (Urban 4 GHz) in Table 3.3-1 to Table 3.3-3 in R1-2009660 to the Appendix of TR 38.875

  - The tables will be further updated with potential updated evaluation results (to catch potential typos) and a clarification of assumption for Msg2, PRACH and DL PSD.

  - MPL results to be included also. Up to editor to use the same or different tables.

Agreements:

Adopted the updated TP in section 3.3 of R1-2009660 as baseline text for TR clause 9.1

- Remove “and coverage recovery is needed” from the TP

- Add the following sentence to the last paragraph of the TP

  - It should be noted that for DL PSD 24 dBm/MHz and 1 Rx RedCap UE case Msg2 results are based on no TBS scaling

Agreements:

- For FR2 indoor scenario, the representative value is derived based on results for max TRP 12 dBm. The aggregated value for UL channels has then been obtained by considering

  - Results presented by companies assuming max TRP 12 dBm; and

  - Results presented by companies assuming max TRP 23 dBm, where corresponding MIL values have been reduced by 11 dB, and each company is counted only once (no double value is considered, if any).

Agreements:

- Adopt the updated TP as baseline text for TR clause 10

The SLS evaluations for the impacts of UE complexity reduction and antenna inefficiency to network capacity and spectrum efficiency are summarized in Table 4-1 to 4-25. Burst traffic model and optional full buffer traffic are considered.

The impact from potential coverage recovery techniques is reflected in some of the SLS results in the sense that we allow the PDSCH/PUSCH spectral efficiency to go lower due to, e.g. repetitions and/or HARQ transmissions (i.e. trading data rate for coverage).

For burst traffic evaluation, FTP model 3 is assumed for eMBB users. The assumption of traffic model for RedCap users varies across the sourcing companies. The instant message (IM) traffic model which in average generates an offered load of 400 kbps/s (0.1 MB payload every 2 s) is assumed for RedCap users by some sourcing companies. Compared to the assumed traffic model for the eMBB users which have an offered load of 20 Mbps (0.5 MB payload every 200 ms), the RedCap users will produce a very low data volume even with a 50-50 split of eMBB and RedCap users. The use of IM traffic for downlink capacity evaluation corresponds to video surveillance and industrial wireless sensor use cases for which traffic pattern is dominated by UL transmissions. In addition, the IM traffic may also be possible for some low data rate wearable use cases.

Some companies have considered to reuse the same FTP model 3 for RedCap users by assuming wearable use cases have DL heavy traffic and the traffic pattern is the same for RedCap users and eMBB users. It should be noted that among the companies assuming FTP3 traffic model for RedCap, there may be differences in the average traffic volume assumption. Such a difference may contribute to different conclusion.

For burst traffic evaluation with IM traffic model for RedCap users:

- 3 sources observed that the RedCap users have minor or no impact on spectral efficiency and capacity, and little impact to the performance of co-existing eMBB users in the system

- It is further noted that the 1 Rx RedCap users do not make an appreciable change on the user throughput performance of the eMBB users compared to the 2 Rx RedCap users

For burst traffic evaluation with FTP model 3 for RedCap users:

- One source with the respective simulation assumptions including the schedulable bandwidth reported the user throughput performance of the eMBB users is not degraded with the presence of the RedCap users in the system.

- One source with the respective simulation assumptions including the schedulable bandwidth reported the impact on spectral efficiency will be substantial. It is further observed substantial cell spectral efficiency loss about 30% due to UE Rx antenna reduced from four to two and DL modulation order restriction from 256QAM to 64QAM in FR1 and about 50% spectral efficiency reduction due to UE Rx antenna reduced from four to one and DL modulation order restriction from 256QAM to 64QAM in FR1.

For optional full buffer traffic evaluation:

- One source with the respective simulation assumptions including the schedulable bandwidth reported a minor degradation of the spectral efficiency for the eMBB users and the degree of spectral efficiency loss is irrespective of the number of Rx antennas for RedCap users.

- One source with the respective simulation assumptions including the schedulable bandwidth reported the impact on spectral efficiency will be substantial. It is further observed substantial cell spectral efficiency loss about 54% due to UE Rx antenna reduced from four to two and DL modulation order restriction from 256QAM to 64QAM in FR1 and about 70% spectral efficiency reduction due to UE Rx antenna reduced from four to one and DL modulation order restriction from 256QAM to 64QAM in FR1.

Agreements:

- Capture the following observations for FR1 coverage recovery to the TR 38.875

  - For FR1, under the consideration of potential reduced antenna efficiency due to device size limitations, the MIL(s) of PUSCH and/or Msg3 are worse than that of the bottleneck channel for the reference NR UE and coverage recovery is needed. The amount of coverage recovery is up to 3 dB. For other UL channels, coverage recovery may be not needed.

  - For FR1 including both FDD and TDD bands and RedCap UE with 2 Rx and reduced antenna efficiency, the MIL(s) of all the downlink channels are better than that of the bottleneck channel for the reference NR UE and coverage recovery is not needed.

  - For RedCap UE with 1 Rx and reduced antenna efficiency, dependent on frequency bands and the assumption of DL PSD, the need for coverage recovery can be different

    - For carrier frequency of 4 GHz with DL PSD 24 dBm/MHz, coverage recovery may be needed for the downlink channels of Msg2, Msg4 and PDCCH CSS. A small or moderate compensation can be considered:

      - [1 dB] for PDCCH CSS

      - [2-3 dB] for Msg4

      - [5-6 dB] for Msg2 without TBS scaling. It is noted that coverage loss for Msg2 can be compensated by using the existing TBS scaling technique.

    - For other carrier frequencies or DL PSD other than 24 dBm/MHz, coverage recovery is not needed for the downlink channels if the target for coverage recovery is based on the MIL of the bottleneck channel for the reference NR UE

    - It is noted that in the methodology for RedCap UE coverage recovery target determination, absolute ISD/MPL targets are not considered

    - The determination of which channels require coverage recovery and the amount of coverage recovery depend on the choice of the target for coverage recovery

Agreements:

- Capture the following observations for FR2 coverage recovery to the TR 38.875

  - For FR2, there is no assumption of reduced antenna efficiency for RedCap UE and the MIL of the UL channels is the same as the reference NR UE and coverage recovery for UL channels is not needed.

  - [For RedCap UE with 100 MHz BW and 1Rx, although there is performance loss from reducing the number of Rx branches to 1, the MIL(s) of all the DL channels is better that that of the bottleneck channel for the reference NR UE and coverage recovery for DL channels is not needed. ]

  - For RedCap UE with 50MHz BW and 1Rx, coverage recovery may be needed for PDSCH when the same target data rate as the reference NR UE is assumed, and the amount of coverage recovery to be considered is approximately [2-3 dB]

    - The tradeoff between data rate and coverage can be considered and the amount of coverage recovery may depend on this choice.

  - The determination of which channels require coverage recovery and the amount of coverage recovery depend on the choice of the target for coverage recovery

    - E.g. coverage recovery may not be needed for FR2 indoor scenario when the target is based on an MPL value from a target ISD of 20m

    - E.g. a large amount of coverage recovery may be needed for the initial access channels if the target is to achieve the same coverage for the initial access channels between RedCap UE and the reference NR UE

Agreements:

- Capture the following to the TR 38.875

  - Coverage recovery for broadcast PDCCH (PDCCH monitored in a Type0/0A/1/2/3-PDCCH CSS) was studied from several aspects, including PDCCH repetition, compact DCI, new AL [of 12, 24 or 32], PDCCH transmission via CORESET or search space bundling, PDCCH-less mechanism for SIB1 and/or SI message

  - If PDCCH repetition is supported, the potential specification impacts include

    - Repetition configuration (e.g. intra-slot or inter-slot)

    - DMRS design among PDCCH repetitions

    - Search space design for PDCCH repetition

  - If compact DCI is supported, the potential specification impacts include

    - DCI format with a small payload size

    - Reuse existing format by fixing some DCI bits

  - If new AL is supported, the potential specification impacts include

    - Mechanism for codeword generation and mapping to CCEs

    - CORESET duration extension

    - Related signaling design

  - If PDCCH transmission via CORESET bundling is supported, the potential specification impacts include

    - CORESET bundling configuration

    - DMRS design among CORESET bundling

  - If PDCCH-less is supported, the potential specification impacts include

    - Mechanism or resource allocation for indicating scheduling information for SIB1 and/or SI message in L1 signals(s)/channels(s) other than PDCCH

  - It is noted that some of the techniques may have compatibility issue if RedCap and normal UEs share the same initial DL BWP

Agreements:

- Capture the following to the TR 38.875

  - Coverage recovery for PUSCH was studied from several aspects, including cross-slot or cross-repetition channel estimation, lower DM-RS density in time domain, enhancements on PUSCH repetition Type A and/or Type B, frequency hopping or BWP switching across a larger system bandwidth

  - Some techniques, such as cross-slot or cross-repetition channel estimation, lower DM-RS density in time domain, enhancements on PUSCH repetition Type A and/or Type B have been studied also in the Rel-17 coverage enhancement SI

  - Potential specification impacts of frequency hopping or BWP switching across a larger system bandwidth include:

    - Frequency domain hopping offsets/positions

    - Faster switching/RF retuning time.

      - Note this aspect requires RAN4 involvement, where the corresponding study in RAN4 is not performed yet.

    - Transmission/reception interruption during RF retuning time

Agreements:

- Capture the following to the TR 38.875

  - Coverage recovery for Msg3 was studied including repetition for Msg3 PUSCH initial and/or retransmission

  - It is noted that enhancements on Msg3 PUSCH repetition have been studied also in the Rel-17 coverage enhancement SI

Agreements:

- Capture the following to the TR 38.875

  - Coverage recovery for PDSCH was studied from several aspects, including the use of the lower-MCS table, larger aggregation factor for PDSCH reception, cross-slot or cross-repetition channel estimation, increasing the granularity of PRB bundling, frequency hopping or BWP switching across a larger system bandwidth.

  - Some techniques, such as the lower-MCS table and larger aggregation factor for PDSCH reception are existing techniques with optional UE capability signaling

  - If cross-slot or cross-repetition channel estimation for PDSCH is supported, potential specification impacts include:

    - Time-domain precoder cycling and DM-RS configuration

  - If hopping or BWP switching across a larger system bandwidth is supported, potential specification impacts include

    - PDSCH hopping configuration

    - Faster switching/RF retuning time

      - Note this aspect requires RAN4 involvement, where the corresponding study in RAN4 is not performed yet.

    - Transmission/reception interruption during RF retuning time

  - Potential specification impacts of increasing the granularity of PRB bundling include

    - Related signaling design

Agreements:

- Capture the link budget evaluation results (indoor 28 GHz) in Table 3.4-1 to Table 3.4-3 in R1-2009660 to the Appendix of TR 38.875

  - The tables will be further updated with potential updated evaluation results (to catch potential typos) and a clarification of assumption for Msg2, PRACH and UE maximum Tx power.

  - MPL results to be included also. Up to editor to use the same or different tables

Agreements:

- Capture the SLS evaluation results in Table 4-1 to Table 4-25 in R1-2009722 to TR 38.875

  - The tables will be further updated with potential updated evaluation results (to catch potential typos) and a clarification of evaluation assumption

R1-2008865	Coverage recovery and capacity impact for RedCap	Ericsson

Revision of R1-2007531

R1-2007536	Coverage recovery for RedCap	FUTUREWEI

R1-2008813	Functionality for coverage recovery	Huawei, HiSilicon

Revision of R1-2007598

R1-2007670	Discussion on coverage recovery, capacity and spectrum efficiency impact	vivo, Guangdong Genius

R1-2007717	Discussion on coverage recovery for RedCap UE	ZTE

R1-2007864	Coverage recovery for reduced capability NR devices	CATT

R1-2007889	Coverage recovery and capacity impact	TCL Communication Ltd.

R1-2007949	On coverage recovery for RedCap UEs	Intel Corporation

R1-2007990	Coverage Recovery and Capacity Impact	Panasonic Corporation

R1-2008018	Discussion on coverage recovery for RedCap UEs	CMCC

R1-2008050	Discussion on the coverage recovery of reduced capability NR devices	LG Electronics

R1-2008070	Functionality for coverage recovery	Nokia, Nokia Shanghai Bell

R1-2008086	Discussion on coverage recovery for reduced capability device	Xiaomi

R1-2008102	Discussion on coverage recovery and capacity impact	Spreadtrum Communications

R1-2008172	Coverage recovery for low capability device	Samsung

R1-2008262	Discussion on coverage recovery issues and evaluation	OPPO

R1-2009173	Coverage recovery for RedCap	Lenovo, Motorola Mobility

Revision of R1-2008295

R1-2008367	Coverage recovery for Redcap devices	Sony

R1-2008396	Coverage recovery for reduced capability UEs	Sharp

R1-2008472	Functionality for Coverage Recovery for RedCap	Apple

R1-2008512	Discussion on coverage recovery for NR RedCap UEs	MediaTek Inc.

R1-2008518	On coverage recovery for reduced capability UEs 	Convida Wireless

R1-2008553	Discussion on coverage recovery for RedCap	NTT DOCOMO, INC.

R1-2009310	Coverage Recovery for RedCap Devices	Qualcomm Incorporated

Revision of R1-2008622

R1-2008686	Coverage recovery for reduced capability NR devices	InterDigital, Inc.

R1-2008728	Discussion on Coverage Recovery for RedCap UE	WILUS Inc.

R1-2008740	Coverage recovery for RedCap UE	Sequans Communications

### Framework and Principles for Reduced Capability

R1-2008555	Summary on Framework and Principles for Reduced Capability	Moderator (NTT DOCOMO, INC.)

R1-2009534 	Summary on Framework and Principles for Reduced Capability	Moderator (NTT DOCOMO, INC.)

[103-e-NR-RedCap-05] Email discussion for framework and principles for RedCap – Shinya (DCM)

- 1st check point: 10/29

- 2nd check point: 11/5

- Last check point 11/12

Update on 10/29:

Conclusion:

- Defer to RAN2 on the framework how to indicate the capabilities of RedCap UEin connected mode

- Note: Possible early identification is used for UEs in idle mode and is discussed in AI8.6.5

- Note: RAN1 continues the discussion on the exact composition of the set of L1 capabilities of the RedCap UE type

Conclusion:

- Following coexistence issuesare not studied in Rel.17 RedCap SI

  - Efficient Beam-based operation in FR2

  - Efficient resource usage in FR2

  - How to mitigate the PRACH collision in FR2

Other proposals? 11/5

Agreements:

- At least for RedCap UE identification, explicit definition of RedCap UE type(s) is needed. Ppending conclusions on the reduced complexity features in AI8.6.1 and RedCap UE identification in AI8.6.5, the definition of the RedCap UE types can be based on one of:

  - Option 1: All the reduced capabilities recommended at the end of the RedCap study

  - Option 2: Only include the reduced capabilities that the network needs to know during initial access, if any

  - Option 3: All the recommended reduced capabilities as well as recommended power saving features

  - Option 4: The corresponding minimum set of the reduced capabilities that one RedCap UE type shall mandatorily support

  - FFS for other usages

Agreements:

·        If early identification during initial access is supported, at least maximum supported UE BW during initial access is included in the set of L1 capabilities of the device type for RedCap early identification

§  Note: 20 MHz for FR1 and 100 MHz for FR2

§  Identification of UEs optionally supporting bandwidths larger than 20 MHz in FR1 or larger than 100 MHz in FR2 after initial access, if supported, is not supported by early identification during initial access

§  FFS other L1 capabilities

§  Note: This does not preclude the case where the early indication only indicates whether it is a Redcap UE or which type of the Redcap UEs if multiple UE types are defined

Email summary in R1-2009732

R1-2007532	Framework and principles for RedCap	Ericsson

R1-2007537	Framework for RedCap UEs	FUTUREWEI

R1-2007599	Framework and principles for reduced capability devices	Huawei, HiSilicon

R1-2007671	Framework and Principles for Reduced Capability	vivo, Guangdong Genius

R1-2007718	Views on Framework and Principles for Reduced Capability	ZTE

R1-2007865	Framework and principles for reduced capability NR devices	CATT

R1-2007950	Framework and principles for introduction of RedCap UEs	Intel Corporation

R1-2008019	Discussion on design principles and definition for RedCap device type	CMCC

R1-2008051	Consideration on the framework to support reduced capability NR devices	LG Electronics

R1-2008071	Framework and Principles for Reduced Capability UE	Nokia, Nokia Shanghai Bell

R1-2008087	Framework and Principles for Reduced Capability	Xiaomi

R1-2008101	Discussion on Framework and Principles for Reduced Capability	Spreadtrum Communications

R1-2008173	Framework and Principles for Reduced Capability	Samsung

R1-2008263	Further considerations on reduced UE capability	OPPO

R1-2008290	Discussion on Framework and Principles for Reduced Capability	Panasonic

R1-2008296	Framework and Principles for RedCap	Lenovo, Motorola Mobility

R1-2008473	Framework and principles for RedCap	Apple

R1-2008513	On the framework for RedCap UEs	MediaTek Inc.

R1-2008554	Discussion on framework and principles for RedCap	NTT DOCOMO, INC.

R1-2008623	Standardization Framework and Design Principles for RedCap Devices	Qualcomm Incorporated

R1-2008687	Framework and Principles for Reduced Capability	InterDigital, Inc.

R1-2008741	Framework and principles for RedCap UE	Sequans Communications

### Others

Including RAN2-led aspect on allowing devices with reduced capabilities to be explicitly identifiable to networks and network operators, and allow operators to restrict their access, if desired

R1-2009317	Moderator summary on RedCap - Others	Moderator (Intel)

R1-2009404	Moderator summary #2 on RedCap – Others	Moderator (Intel)

R1-2009608 	Moderator summary #3 on RedCap – Others	Moderator (Intel)

[103-e-NR-RedCap-06] Email discussion for 8.6.5 – Debdeep (Intel)

- 1st check point: 10/29

- 2nd check point: 11/5

- Last check point 11/12

Update on 10/29:

Conclusion:

- For access control for RedCap UEs, detailed signaling options associated with system information are postponed to the WI phase.

Other proposals? 11/3

Agreements:

- As a next step, for the study on the options for RedCap UE identification during RAN1 #103-e meeting, RAN1 to focus on establishing feasibility, necessity, and identifying pros and cons for the following schemes:

  - Opt. 1: During Msg1 transmission, e.g., via separate initial UL BWP, separate PRACH resource, or PRACH preamble partitioning.

  - Opt. 2: During Msg3 transmission.

  - Opt. 3: Post Msg4 acknowledgment.

    - E.g., during Msg5 transmission or part of UE capability reporting.

  - Opt. 4: During MsgA transmission.

Update on 11/9:

Agreements:

- Considerations on Option 4 (during MsgA transmission) are deprioritized until further progress is made on Options 1 and 2 for 4-step RACH procedure.

Agreements:

- Observation: Identification of RedCap UE type(s) during transmission of Msg1 could be feasible from the perspective of RAN1, at least for the following solutions:

  - Separation of PRACH resources (e.g., occasions and/or formats) or PRACH preambles between RedCap and non-RedCap UEs

  - Separation of initial UL BWP for RedCap and non-RedCap UEs

- Note: The appropriateness of each solution, considering the number of UE type(s) to be indicated, etc. needs further considerations.

Agreements:

- Observation: Identification of RedCap UE type(s) during transmission of Msg5 or as part of UE capability reporting are feasible options from the perspective of RAN1

Agreements:

- Observation: If early identification of RedCap UE type(s) via Options 1, 2, or 4 are not supported, then RedCap UE type(s) need to be identified either during transmission of Msg5 or as part of UE capability reporting.

Agreements:

- Observation: Early identification of RedCap UE type(s) during transmission of Msg1 may be necessary for:

  - coverage recovery (including link adaptation) for one or more of: Msg2 PDCCH/PDSCH, Msg3 PUSCH and PDCCH scheduling Msg3 reTx, Msg4 PDCCH/PDSCH or PUCCH in response to Msg4, Msg5 PUSCH and associated PDCCH, if it is determined that coverage recovery for RedCap UEs is necessary for one of more of these channels;

  - identifying UE minimum processing times capabilities for PDSCH processing and PUSCH preparation, if relaxations to UE min processing times are defined for N1 and N2;

  - identifying UE capability for UL modulation order for Msg3 and Msg5 scheduling, if relaxations to max UL modulation order (i.e., UL modulation order restricted to lower than 64QAM) are introduced;

  - identifying UE max bandwidth capability for Msg3 and Msg5 scheduling and PUCCH in response to Msg4.

- Note: Exact necessity depends on outcome of studies on UE cost/complexity reduction in AI 8.6.1 and Coverage Recovery in AI 8.6.3, and the SI on Coverage Enhancements.

Agreements:

- Observation: The following pros and cons are identified for identification of RedCap UE type(s) during transmission of Msg1:

| Pros | Cons |
| --- | --- |
| Enables efficient handling of different UE minimum processing times between RedCap and non-RedCap UEs for: minimum timing between PDSCH carrying RAR and start of Msg3 PUSCH; minimum timing between PDSCH carrying Msg4 and the corresponding HARQ-ACK feedback; minimum timing between PDCCH with the reTx grant and the corresponding Msg3 PUSCH retransmission, if relaxed UE min processing times are introduced for RedCap UEs. | Potential reduction in PRACH user capacity (for the options based on separation of PRACH preambles), impacting both RedCap and non-RedCap UEs respectively, e.g., if the total PRACH resources in the cell is not increased. The exact impact depends on numbers of device type(s)/sub-types/capabilities to be identified and exact details of PRACH preamble partitioning schemes. |
| Enables coverage recovery, including link adaptation, for any one or more of: broadcast PDCCH, PDSCH associated with Msg2, PDSCH associated with Msg4, and PUSCH associated with Msg3, if coverage recovery is needed for these channels. | Potential increase in UL OH from PRACH (for the options based on separation of PRACH resources), impacting both RedCap and non-RedCap UEs. |
| The option of configuring separate initial UL BWPs, in addition to the above pros, enables address congestion (if congestion may occur) in the initial UL BWP that may otherwise need to be restricted to the mandatory required BW for RedCap UEs in the band/FR. | Potential increase in UL OH and complexity in configuration and maintenance of multiple initial UL BWP for the gNodeB, for the option of configuring separate initial UL BWPs. |
|  | The indication mechanisms in this category may be limiting in terms of the number of further sub-types/capabilities within RedCap device type that may be distinguished, if such sub-types/capability indication are introduced. |
|  | Higher impact to RAN1 and RAN2 specifications as well as increased SIB signaling OH compared to other options. |

Agreements:

- Observation: The following pros and cons are identified for identification of RedCap UE type(s) during transmission of Msg5 or in UE capability report:

| Pros | Cons |
| --- | --- |
| This option of UE capability reporting offers a simple option for indication of RedCap UE type, including possibility of indicating further RedCap sub-types/capabilities if introduced. | Cannot facilitate additional coverage recovery (if needed) or separate link adaptation for broadcast PDCCH and/or Msg2 and/or Msg4 PDSCH, and/or Msg3 PUSCH for RedCap UEs. Too conservative scheduling and link adaptation for all UEs imply increased system OH for initial access in the initial DL and UL BWPs. |
| Limited or no impact to RAN1 specifications. | If UE minimum processing times are relaxed, cannot facilitate scheduling with separate minimum timing relationships for RedCap UEs between PDSCH carrying RAR and start of Msg3 PUSCH; minimum timing between PDSCH carrying Msg4 and the corresponding HARQ-ACK feedback; minimum timing between PDCCH with the reTx grant and the corresponding Msg3 PUSCH retransmission. This could result in increased initial access latency for non-RedCap UEs. |
|  | Cannot address the issue where Msg3 or PUCCH in response to Msg4 or Msg5 is scheduled with a bandwidth/hopping range larger than the maximum RedCap UE bandwidth in the UL initial BWP. |

Agreements:

- Observation: Identification of RedCap UE type(s) during transmission of Msg3 may be feasible from the perspective of RAN1, at least for the following solutions:

  - Using the spare bit in existing Msg3 definition

  - Extending the Msg3 size to carry additional one or more bits, indicating RedCap UE type(s)

- Note: The appropriateness and feasibility of each solution, considering the number of UE type(s) to be indicated, coverage performance for Msg3, etc. need further considerations from RAN2 and RAN1.

Agreements:

- Observation: If early identification of RedCap UE type(s) via Option 1 is not supported, identification of RedCap UE type(s) during transmission of Msg3 may be necessary for coverage recovery (including link adaptation) for one or more of: Msg4 PDCCH/PDSCH, Msg5 PUSCH and associated PDCCH

- Note: Exact necessity depends on outcome of studies on Coverage Recovery in AI 8.6.3

Agreements:

- Observation: The following pros and cons are identified for identification of RedCap UE type(s) during transmission of Msg3:

| Pros | Cons |
| --- | --- |
| Enables coverage recovery (if needed) and/or appropriate link adaptation for PDSCH (and associated PDCCH and PUCCH) for Msg4, and scheduling of Msg5. | If only the spare bit in Msg3 is used, it would consume the single spare bit currently available in Msg3 payload, and this may not be desirable. |
| Limited impact to RAN1 specifications if only the spare bit in Msg3 payload is utilized. | If extended Msg3 size is introduced, mechanisms to enable detection between use of legacy Msg3 and extended Msg3 definitions necessary. |
| The option of extending Msg3 size may offer good scalability in the number of bits for such UE identification; e.g., if sub-types of RedCap device types (if defined) are to be indicated in Msg3. | The option of only using the spare bit in Msg3 scales poorly – limiting to a single-bit indication may not be sufficient if intending to distinguish between further sub-types/capabilities within RedCap device type, if RedCap UE sub-types/capabilities are defined in the context of RedCap UE identification. |
|  | Cannot facilitate additional coverage recovery (including separate link adaptation) for broadcast PDCCH and/or Msg2 PDSCH, and/or Msg3 PUSCH (and associated PDCCH) for RedCap UEs. |
|  | If UE minimum processing times are relaxed, cannot facilitate scheduling with separate minimum timing relationships for RedCap UEs (compared to non-RedCap UEs) between PDSCH carrying RAR and start of Msg3 PUSCH; minimum timing between PDCCH with the reTx grant and the corresponding Msg3 PUSCH retransmission. This could result in increased initial access latency for non-RedCap UEs. |
|  | May degrade reliability/coverage of Msg3 in case of increased Msg3 payload size. |
|  | Cannot address the issue where Msg3 is scheduled with a bandwidth/hopping range larger than the maximum RedCap UE bandwidth in the UL initial BWP. |

Conclusion: The option of carrying RedCap UE type(s) identification as part of UCI multiplexed in Msg3 PUSCH is not considered during the Rel-17 RedCap SI.

Email summary in R1-2009780.

R1-2007533	UE identification and access restriction for RedCap	Ericsson

R1-2007538	Identification for RedCap UEs	FUTUREWEI

R1-2007672	RRM relaxation for Reduced Capability NR devices	vivo, Guangdong Genius

R1-2007719	Access control and identification for Reduced Capability NR devices	ZTE

R1-2007866	Identification and access restriction for reduced capability NR devices	CATT

R1-2007951	On identification of and access control for RedCap UEs	Intel Corporation

R1-2008020	Discussion on identification and access control for Reduced Capability NR devices	CMCC

R1-2008052	Other aspects for reduced capability NR devices	LG Electronics

R1-2008072	Initial access for RedCap UEs	Nokia, Nokia Shanghai Bell

R1-2008075	Procedure of identification for Reduced Capability NR devices	TCL Communication Ltd.

R1-2008088	Discussion on the access control and configuration for reduced capability device	Xiaomi

R1-2008106	Consideration on power saving for reduced capability NR devices	Spreadtrum Communications

R1-2008174	UE identification and access barring	Samsung

R1-2008264	Other considerations for reduced UE capability	OPPO

R1-2008291	On RedCap device identification	Panasonic

R1-2008329	Other aspects for reduced capability devices	Huawei, HiSilicon

R1-2008337	Narrowband operation and identification of RedCap UE	Lenovo, Motorola Mobility

R1-2008397	Identification and access restriction for reduced capability UEs	Sharp

R1-2008556	Discussion on UE identification for RedCap	NTT DOCOMO, INC.

R1-2008585	Discussion on identification of reduced capability UE	ASUSTeK

R1-2008688	Device identification and access restriction for RedCap	InterDigital, Inc.

## UE Power Saving Enhancements

Please refer to RP-200938 for detailed scope of the WI

Note: although for now this WI has its own separate agenda, it is expected to be managed jointly under a single agenda with 8.6 when RedCap becomes a WI.

### Enhancements for idle/inactive-mode UE power saving

#### Potential paging enhancements

R1-2008964	Paging enhancements for idle/inactive-mode UE power saving	MediaTek Inc.

[103-e-NR-PowSav-Enh-01] Email discussion/approval for potential paging enhancements– Weide (MTK)

- 1st check point: 11/5

- 2nd check point: 11/10

- 3rd check point: 11/12

Update on 11/5  proposals 4/5/6 11/9

Update on 11/10:

Agreements:

Observation: For NR idle/inactive-mode UEs, UE sub-grouping indication within a PO can provide the following power saving gains w.r.t. Rel-16:

- If the original group paging rate is 10%:

  - [0.3%] - [1.1%] where the baseline assumes 1 SS burst for synchronization before PO reception

  - [0.4%] - [0.8%] where the baseline assumes 2 SS bursts for synchronization before PO reception

  - [0.3%] - [1.0%] where the baseline assumes 3 SS bursts for synchronization before PO reception

- Some sources also evaluated performance if the original group paging rate is in the range between 20% and 80% and showed following results:

  - [0.7%] - [7.6%] where the baseline assumes 1 SS burst for synchronization before PO reception

  - [0.8%] - [3.0%] where the baseline assumes 2 SS bursts for synchronization before PO reception

  - [0.5%] - [4.7%] where the baseline assumes 3 SS bursts for synchronization before PO reception

The number of UE sub-groups evaluated ranges from 2 to 16.

Some companies show concern on assuming group paging rate larger than 60%.

Note: It is FFS in RAN1 another group paging rate > 10% for the evaluation of Rel-17 paging enhancement.

Agreements:

Observation: For NR idle/inactive-mode UEs, UE sub-grouping indication carried in paging early indication can provide the following power saving gains w.r.t Rel-16:

- If the original group paging rate is 10%:

  - [10.6%] –[19.1%] where the baseline assumes 1 SS burst for synchronization before PO reception

  - [16.0%] –[36.0%] where the baseline assumes 2 SS bursts for synchronization before PO reception

  - [14.3%] –[46.0%] where the baseline assumes 3 SS bursts for synchronization before PO reception

- Some sources also evaluated performance if the original group paging rate is in the range between 20% and 60% and showed following results:

  - [8.0%] –[19.1%] where the baseline assumes 1 SS burst for synchronization before PO reception

  - [18.1%] –[34.0%] where the baseline assumes 2 SS bursts for synchronization before PO reception

  - [20.6%] –[42.0%] where the baseline assumes 3 SS bursts for synchronization before PO reception

The additional power saving gains w.r.t. paging early indication without UE sub-grouping are given as follows:

- If the original group paging rate is 10%:

  - [0.6%] –[2.7%] where the baseline assumes 1 SS burst for synchronization before PO reception

  - [0.6%] –[4.0%] where the baseline assumes 2 SS bursts for synchronization before PO reception

  - [0.6%] –[4.7%] where the baseline assumes 3 SS bursts for synchronization before PO reception

- Some sources also evaluated performance if the original group paging rate is in the range between 20% and 60% and showed following results:

  - [1.3%] –[8.0%] where the baseline assumes 1 SS burst for synchronization before PO reception

  - [2.1%] –[13.0%] where the baseline assumes 2 SS bursts for synchronization before PO reception

  - [3.3%] –[16.1%] where the baseline assumes 3 SS bursts for synchronization before PO reception

The number of UE sub-groups evaluated ranges from 2 to 16.

The power saving gains are dependent on the assumptions about placement of PEI and PO relative to SSB.

Note: It is FFS in RAN1 another group paging rate > 10% for the evaluation of Rel-17 paging enhancement.

Note: Not all sources providing results for paging early indication without UE sub-grouping also provide results for paging early indication with UE sub-grouping.

Agreements:

Observation: For NR idle/inactive-mode UEs with 10% group paging rate, cross-slot scheduling with K0 = 1, which can be supported by Rel-15/Rel-16 for Type 2 CSS, can provide the following power saving gains w.r.t. same-slot scheduling (K0 = 0):

- [<1%] –[2.5%] where the baseline assumes 1 SS burst for synchronization before PO reception

- [<1%] -[1.6%] where the baseline assumes 2 SS bursts for synchronization before PO reception

- [<1%] -[1.44%] where the baseline assumes 3 SS bursts for synchronization before PO reception

One source shows that cross-slot scheduling with K0 = 32, which cannot be supported by Rel-15/Rel-16 for Type 2 CSS, can provide the following power saving gains w.r.t. same-slot scheduling (K0 = 0):

- [0%] where the baseline assumes 1 SS burst for synchronization before PO reception

- [6.3%] where the baseline assumes 3 SS bursts for synchronization before PO reception

The power saving gain will become lower with higher group paging rate.

Agreements: For NR idle/inactive-mode paging enhancement, paging early indication before paging occasion is supported from RAN1 perspective

- FFS: Physical layer design based on DCI, SSS or TRS/CSI-RS

- Send LS to inform RAN2 and kindly ask RAN2 to inform RAN1 if there is anything that RAN1 should take into consideration in the physical layer design for this feature, including any other progress RAN2 has made in this WI which may has RAN1 impact

Agreements:

Observation: For NR idle/inactive-mode UEs with 10% group paging rate, paging early indication without UE sub-grouping can achieve the following power saving gains w.r.t. Rel-16:

- [0%] - [22.8%] where the baseline assumes 1 SS burst for synchronization before PO reception

  - Note: [0%] means UE can apply the baseline behavior if the time offset between the utilized SS burst and PO is small.

- [5.0%] - [32.0%]  where the baseline assumes 2 SS bursts for synchronization before PO reception

- [10.2%] - [67.7%]  where the baseline assumes 3 SS bursts for synchronization before PO reception

The power saving gains will become lower for higher group paging rate.

The power saving gains are dependent on the assumptions about placement of PEI and PO relative to SSB.

The power saving gains may vary with different paging early indication design.

Draft LS in R1-2009754 is approved. Final LS in R1-200xxxx.

R1-2007600	Paging enhancement(s) for UE power saving in IDLE/inactive mode	Huawei, HiSilicon

R1-2007673	Paging enhancements for idle/inactive mode UE power saving	vivo

R1-2007867	Paging enhancement for UE power saving	CATT

R1-2007890	Potential paging enhancements	TCL Communication Ltd.

R1-2007898	Paging enhancement for power saving	Beijing Xiaomi Software Tech

R1-2007971	Discussion on power saving enhancements for paging	ZTE

R1-2008021	Discussion on paging enhancement	CMCC

R1-2008053	Discussion on potential paging enhancements	LG Electronics

R1-2008103	Discussion on potential paging enhancements	Spreadtrum Communications

R1-2008175	Discussion on paging enhancements	Samsung

R1-2008265	Further discussion on Paging enhancements for power saving	OPPO

R1-2008287	Potential paging enhancements for idle/inactive-mode UE power saving	Panasonic

R1-2008368	Paging enhancement for idle/inactive mode UE	Sony

R1-2008474	Paging enhancements for idle/inactive-mode UE power saving	Apple

R1-2008689	Paging enhancements for UE power saving	InterDigital, Inc.

R1-2008933	On paging enhancements for UE power saving	Nokia, Nokia Shanghai Bell

R1-2008972	Paging enhancements for idle/inactive-mode UE power saving	MediaTek Inc.

Withdrawn

R1-2008992	On paging enhancements for UE power saving	Intel Corporation

R1-2009105	Paging enhancement for UE power saving	Lenovo, Motorola Mobility

R1-2009187	Discussion on paging enhancements	NTT DOCOMO, INC.

R1-2009200	Discussion on potential paging enhancements for UE power savings	Ericsson

R1-2009266	Paging enhancements and evaluation methodology	Qualcomm Incorporated

#### TRS/CSI-RS occasion(s) for idle/inactive UEs

R1-2008178	Moderator summary for TRS/CSI-RS occasion(s) for idle/inactive UEs	Moderator (Samsung)

[103-e-NR-PowSav-Enh-02] Email discussion/approval for TRS/CSI-RS occasion(s) for idle/inactive UEs– Taehyoung (Samsung)

- 1st check point: 11/5

- 2nd check point: 11/10

- 3rd check point: 11/12

R1-2008934	On RS information to IDLE/Inactive mode Ues	Nokia, Nokia Shanghai Bell

Agreements:

- Functionality of RRM measurement for neighbour cell is not supported for TRS/CSI-RS for idle/inactive UE(s).

Agreements:

- SIB signalling provides the configuration of TRS/CSI-RS occasion(s) for idle/inactive UE(s).

  - Up to RAN2 to decide which SIB is to be used.

  - Whether or not to additionally support other high-layer signalling methods (e.g., dedicated RRC, RRC release message, etc.) is up to RAN2

Send an LS to RAN2 informing the above agreements, and

- To further add that RAN1 is working on the detailed physical layer design

Draft LS is endorsed, with final LS in R1-2009791.

Agreements:

- Aperiodic TRS and semi-persistent/aperiodic CSI-RS are not used as TRS/CSI-RS occasion(s) for idle/inactive UEs.

Agreements:

- Target sending an LS to RAN2 and RAN4 to ask whether it is feasible to allow a UE to use the potential TRS/CSI-RS occasion to enhance the SSB based IDLE/Inactive mode evaluations of the serving cell. (to also include agreements from last meeting)

* Further discussion whether any additional information needs to be included in the LS or not, including potential re-wording of the leading sentence

Agreements:

- Discuss further based on the following alternatives and down-select at RAN1#104-e:

  - Alt 1: The availability of TRS/CSI-RS at the configured occasion(s) is NOT informed to the UE.

  - Alt 2: The availability of TRS/CSI-RS at the configured occasion(s) is informed to the UE.

  - Alt 3. The conditional availability of TRS/CSI-RS at the configured occasion(s) is informed to the UE.

    - The condition can be, e.g., existence of paging.

  - Alt 4. Combination of the above alternatives.

  - FFS for details

  - FFS for UE behavior when the availability is not informed.

  - Other techniques are not precluded.

  - Companies encourage to provide sufficient information for the proposal, e.g.,

    - how to achieve power saving gain

    - how to minimize impact on NW

how to minimize extra UE implementation complexity

    - feasibility check on sharing the TRS/CSI-RS between connected UEs and idle/inactive UEs

  - Proposals should be consistent with the WID objective.

Conclusion:

- TRS/CSI-RS based PEI is discussed in AI 8.7.1.1.

- PEI functionality is not further discussed under AI 8.7.1.2.

- Note: This does not prevent to potentially use PEI to carry the indication for TRS/CSI-RS presence.

R1-2007601	Assistance RS occasions for IDLE/inactive mode	Huawei, HiSilicon

R1-2007674	TRS/CSI-RS occasion(s) for idle/inactive UEs	vivo

R1-2007868	Configuration of TRS/CSI-RS for paging enhancement	CATT

R1-2007891	TRS/CSI-RS occasion(s) for idle/inactive UEs	TCL Communication Ltd.

R1-2007899	TRS CSI-RS occasion(s) for idle inactive UEs	Beijing Xiaomi Software Tech

R1-2007972	Reference signal for RRC idle and inactive UEs	ZTE

R1-2008022	Discussion on TRS/CSI-RS occasion(s) for idle/inactive-mode UEs	CMCC

R1-2008054	Discussion on TRS/CSI-RS occasion(s) for idle/inactive UEs	LG Electronics

R1-2008176	Discussion on TRS/CSI-RS occasion(s) for idle/inactive Ues	Samsung

R1-2008177	Discussion on DCI-based power saving techniques	Samsung

R1-2008266	Further discussion on RS occasion for idle/inactive UEs	OPPO

R1-2008288	Potential enhancements for TRS/CSI-RS occasion(s) for idle/inactive Ues	Panasonic

R1-2008369	Considerations on TRS/CSI-RS occasion(s) for idle/inactive mode UE	Sony

R1-2008475	Indication of TRS/CSI-RS for idle/inactive-mode UE power saving	Apple

R1-2008690	Discussion on TRS/CSI-RS occasion(s) for idle/inactive UEs	InterDigital, Inc.

R1-2008965	On TRS/CSI-RS occasion(s) for idle/inactive mode UE power saving	MediaTek Inc.

R1-2008973	On TRS/CSI-RS occasion(s) for idle/inactive mode UE power saving	MediaTek Inc.

Withdrawn

R1-2008993	Discussion on TRS/CSI-RS reception during idle/inactive mode	Intel Corporation

R1-2009106	Provision of TRS/CSI-RS for idle/inactive UEs	Lenovo, Motorola Mobility

R1-2009151	Consideration on TRS/CSI-RS occasion(s) for idle/inactive UEs	Spreadtrum Communications

R1-2009188	Discussion on TRS/CSI-RS occasion for idle/inactive UEs	NTT DOCOMO, INC.

R1-2009201	Provisioning of potential TRS/CSI-RS occasion(s) for Idle/Inactive UEs	Ericsson

R1-2009267	TRS CSI-RS for idle and inactive UE power saving	Qualcomm Incorporated

#### Others

R1-2007675	Discussion on paging grouping	vivo

R1-2007869	Link level performance of IDLE UE for UE power saving	CATT

R1-2007973	Additional simulation results of UE power consumption in RRC idle and inactive state	ZTE

R1-2008335	Analysis on power consumption for IDLE mode UE	Huawei, HiSilicon

R1-2009202	High-level UE energy consumption profiling	Ericsson

R1-2009300	On network power consumption model	Nokia, Nokia Shanghai Bell

### Potential extension(s) to Rel-16 DCI-based power saving adaptation during DRX ActiveTime

R1-2008267	Discussion on DCI-based power saving adaptation	OPPO

[103-e-NR-PowSav-Enh-03] Email discussion/approval for potential extension(s) to Rel-16 DCI-based power saving adaptation during DRX ActiveTime – Xiaodong (vivo)

- 1st check point: 11/5

- 2nd check point: 11/10

- 3rd check point: 11/12

Agreements:

Observation:

- Each of the following schemes is individually shown to be beneficial for UE power saving compared to the baseline.

  - Dynamically switching search space set

  - Dynamically skipping PDCCH monitoring for a certain duration or until next DRX ON

- At least the following Rel-15 and/or Rel-16 power saving solutions have been utilized for baseline,

  - For eMBB traffic,

    - DRX setting(including using short DRX or long DRX with a short IAT or long IAT), Wake-up signal, Cross-slot scheduling, CA/Scell dormancy, MAC-CE skipping, BWP switching

  - For VoIP traffic,

    - DRX setting(only long DRX cycle with a short IAT), Wake-up signal,  Cross-slot scheduling, MAC-CE skipping

  - For IM traffic,

    - DRX setting(long DRX cycle [with a short IAT]), Wake-up signal

  - For intensive eMBB traffic,

    - DRX setting(including using short DRX or long DRX with a short IAT), Wake-up signal, Cross-slot scheduling, [CA/Scell dormancy], MAC-CE skipping, BWP switching

    - Note: intensive eMBB traffic is optional and companies may use FTP model 3 with different packet size and mean data arrival time, e.g., 15ms, 30ms, 50ms or 100ms.

- Note 1: For Search space switching, switching from 1slot monitoring to 2, 4, 8, 10, 16 or 32 slot with 30kHz SCS (FR1) and 120kHz (FR2) is utilized.

- Note 2: For PDCCH skipping , skipping 2ms, 4ms, 5ms, 8ms, 15ms, 16ms, 32ms,  64ms or to next DRX cycle is utilized

- Note 3: the baseline assumed may vary across companies

Agreements:

- Specify at least one of the following options for Rel-17 dynamic PDCCH adaptation in time-domain for active time,

  - Option 1: Search space set group switching,e.g., potential adjustments/enhancements forincluding explicit and implicit search spaceset group switching specified in R16 for NR-U

  - Option 2: PDCCH skipping for a certain duration / DRX cycle

- FFS: which option(s)(e.g. taking into account additional gain of option 1 over option 2, or vice-versa)

- Candidate DCI formats for dynamic PDCCH adaptation include DCI formats 1_1(including scheduling and non-scheduling DCI), 0_1, 1_2, 0_2, 2_0, 2_6.

- Note:

  - Companies are encouraged to provide analysis on specification impact, power saving benefit and system impact (e.g., packet latency, system overhead)

- FFS: other schemes are not precluded for further study

R1-2007602	Extension(s) to Rel-16 DCI-based power saving adaptation for an active BWP	Huawei, HiSilicon

R1-2007676	Discussion on DCI-based power saving adaptation in connected mode	vivo

R1-2007701	Extension to Rel-16 DCI-based power sabing adaptation during DRX Active Time	GDCNI

R1-2007870	PDCCH monitoring adaptation	CATT

R1-2007974	Extension to Rel-16 DCI-based power saving adaptation during DRX Active Time	ZTE

R1-2008023	Discussion on PDCCH monitoring reduction during DRX active time	CMCC

R1-2008055	Discussion on DCI-based power saving adaptation during DRX ActiveTime	LG Electronics

R1-2008289	Potential extension(s) to Rel-16 DCI-based power saving adaptation during DRX ActiveTime			Panasonic

R1-2008476	Enhanced DCI-based power saving adapation	Apple

R1-2008691	PDCCH-based power saving signal design considerations	InterDigital, Inc.

R1-2008711	DCI-based Power Saving Enhancements	Fraunhofer HHI, Fraunhofer IIS

R1-2008714	Power saving adaptation during Active Time	ASUSTeK

R1-2008935	UE power saving enhancements for Active Time	Nokia, Nokia Shanghai Bell

R1-2008966	Discussion on DCI-based power saving adaptation during DRX active time	MediaTek Inc.

R1-2008974	Discussion on DCI-based power saving adaptation during DRX active time	MediaTek Inc.

Withdrawn

R1-2008994	On PDCCH monitoring reduction techniques during active time	Intel Corporation

R1-2009056	Discussion on extension(s) to Rel-16 DCI-based power saving adaptation	Asia Pacific Telecom co. Ltd

R1-2009107	Enhanced DCI based power saving adaptation	Lenovo, Motorola Mobility

R1-2009150	Discussion on power saving techniques for connected-mode UE	Spreadtrum Communications

R1-2009189	Discussion on extension to DCI-based power saving adaptation	NTT DOCOMO, INC.

R1-2009203	Discussion on potential enhancements for power savings during active time	Ericsson

R1-2009268	DCI-based power saving adaptation during DRX ActiveTime	Qualcomm Incorporated

R1-2009299	On power saving adaptation during the DRX active time	Sony

### Others

R1-2007677	Discussion on RAN2 and RAN4 related power saving relaxation	vivo

R1-2007871	Feature interaction between Secondary DRX group and DCP/SCell dormancy	CATT

R1-2007975	Further discussion on potential power saving schemes for RRC connected UEs	ZTE

R1-2008268	Discussion on RLM relaxation	OPPO

R1-2008330	Other considerations on power saving in Rel-17	Huawei, HiSilicon

R1-2008477	Power saving evaluation for RLM/BFD relaxation	Apple

R1-2009204	Evaluation of additional UE power saving schemes	Ericsson

## Study on NR coverage enhancement

Please refer to RP-200861for detailed scope of the SI

R1-2007992	TR 38.830 v0.0.3 Study on NR coverage enhancements	China Telecom

[103-e-NR-CovEnh-01] Eamil discussion for TR38.830 update – Jianchi (CT)

- 1st check point: 10/29 (particularly to endorse x7992 if possible)

- 2nd check point: 11/12 further update based on the progress during this e-meeting.

Update on 10/29:

- The latest update TR is endorsed as in R1-2009461.

Next update? Till 11/12

R1-2007484	[102-e-Post-NR-CovEnh-02] Summary on email discussion/approval of initial collection of simulation results for baseline	Moderator (SoftBank, Nokia)

R1-2007483	[102-e-Post-NR-CovEnh-02] Phase 3: initial collection of simulation results for enhancements			Moderator (China Telecom)

### Baseline coverage performance using LLS

#### FR1

R1-2009341	Summary #1 on A.I. 8.8.1.1 baseline coverage performance using LLS for FR1 Moderator (SoftBank)

[103-e-NR-CovEnh-02] Eamil discussion for FR1 coverage performance – Yusuke (Softbank)

- 1st check point: 10/29

- 2nd check point: 11/4

- 3rd check point: 11/10

- Last check point 11/12

Agreements:

- Representative values of the absolute values of [MCL, MIL and MPL] are used for bottleneck identification

  - Further down-selection one or more of MCL/MIL/MPL may be performed depending on the decision of target performance metric(s)

  - Companies can also report their individual observations of the bottleneck based on individual simulation results

  - How to use the respresentive values is FFS

  - A representative value is derived by taking the mean value (in dB domain) from companies’ evaluation results

    - Excluding the highest & the lowest values when the number of samples is more than 3

    - If the number of samples used to compute a representative value is less than 4 for each scenario, this representative value is not used for bottleneck identification

      - In this case, observations may still be drawn

Agreements:

- Rephrase the following terminologies, which is used in e.g. link budget template:

  - “PDCCH of Msg.2” refers to as “broadcast PDCCH (PDCCH of Msg.2)”

  - “PDCCH” refers to as “unicast PDCCH”

- Agreements:

- Representative values are computed for the following channels or signals for FR1, and they are used to draw observation and to identify bottlenecks (if any) if the number of samples for each scenario is more than Y (Y=3)

  - PUSCH for eMBB (FDD, & TDD with DDDSU and DDDSUDDSUU for 4GHz, DDDDDDDSUU for 2.6GHz)

  - PUSCH for VoIP (FDD, & TDD with DDDSU and DDDSUDDSUU for 4GHz, DDDDDDDSUU for 2.6GHz)

  - PUCCH Format 1 with 2bits

  - PUCCH Format 3 with 11bits

  - PUCCH Format 3 with 22bits

  - SSB

  - PRACH format 0

  - PRACH format B4

  - Broadcast PDCCH (PDCCH of Msg.2)

  - PDSCH for Msg.2

  - PUSCH of Msg.3

  - PDSCH of Msg.4

  - Unicast PDCCH

  - PDSCH for eMBB (FDD, & TDD with DDDSU and DDDSUDDSUU for 4GHz, DDDDDDDSUU for 2.6GHz)

  - PUCCH with HARQ-ACK for Msg.4

Agreements:

- The following scenarios are used for drawing observations and bottleneck identification for FR1

  - 1st priority

    - Urban 4GHz TDD

    - Urban 2.6GHz TDD

    - Rural 4GHz TDD NLOS O2I

    - Rural 700MHz FDD NLOS O2I

    - Rural 2GHz FDD NLOS O2I

    - Rural 2.6GHz TDD NLOS O2I

  - 2nd priority

    - Rural 700MHz with long distance FDD LOS O2O

    - Rural 4GHz with long distance TDD LOS O2O

  - Note: the difference between 1st priority and 2nd priority is as follows

    - RAN1 discussion will focus on 1st priority scenarios for drawing observation and bottleneck identification, while less time will be spent on 2nd priority scenarios

    - If RAN1 cannot reach consensus on 2nd priority scenarios, the scenario(s) will still be captured in the Appendix of the TR for completeness, but no observation/conclusion will be made for them.

- Agreements

- No categorization by other simulation parameters (such as UE speed, antenna gain correction factors) will be introduced for FR1 for deriving representative values

  - The amount of available results for DL channels in FR1 4GHz scenario should be considered as given by the total number of available results for both 33 dBm/MHz and 24 dBm/MHz, given that they can be derived one from the other by simple subtraction, and where each company is counted only once.

  - In order to address the misalignment issue of the companies’ evaluation results due to no categorization and/or different simulation assumptions,

    - Number of samples and standard deviation is shown together with a representative value, and

    - A description on the potential fluctuation due to no categorization and/or different simulation assumptions can be added in the observation

- The evaluation results, which are used for neither representative value derivation nor coverage bottleneck identification due to the lack of number of samples etc., can be used to make additional observations to be captured in the TR.

  - This discussion will be held in RAN1#103-e on a low priority basis.

Agreements:

  - For, Scenario dependent targets, e.g., ISD/MPL

      - The following formula is used to convert an ISD value to a target MPL value (to add the reference when capturing into TR):

        - For urban scenarios,

        - For rural scenarios,

        - For rural with long distance scenarios (working assumption)

Agreements:

- All the parameters/values/configurations related to FR1 modelling for which an agreement has not been reached among companies prior to RAN1 #103-e, will be henceforth treated according to the “reported by companies” principle. RAN1 will not spend further time during RAN1 #103-e on the resolution of these issues.

Agreements:

  - For, Scenario dependent targets, e.g., ISD/MPL

    - For each scenario, multiple target ISD values can be used to draw observations, and a single target ISD value can be used to identify bottlenecks (if applicable)

    - Target ISD values for each scenario are as follows:

        - Urban 4GHz TDD –400, 500m for observation and 400m for bottleneck identification

        - Urban 2.6GHz TDD –400, 500m for observation and 400m for bottleneck identification

        - Rural 4GHz TDD NLOS O2I – 1732 and 3000 m for observation and 1732m with 33dBm/MHz BS transmit power for bottleneck identification

        - Rural 2.6 GHz TDD NLOS O2I – 1732m for observation and bottleneck identification

        - Rural 2 GHz FDD NLOS O2I – 1732m for observation and bottleneck identification

        - Rural 700MHz FDD NLOS O2I –3000m, 4000m for observation and 4000m for bottleneck identification

Agreements:

  - For Service dependent targets for VoIP

    - MCL of 139.2dB is used for Rural 700MHz scenario for drawing observation and can be used to identify bottlenecks (if applicable)

      - Captured the following table, which shows how the target value (i.e. 139.2dB) is derived, in Annex of the TR.

| Transmitter |  |
| --- | --- |
| (2a) Number of transmit chains modelled in LLS | 1 |
| (3) Total transmit power (dBm) Note: total transmit power for system bandwidth | 23 |
| (3a) System bandwidth for downlink, or occupied bandwidth for uplink (Hz) | 3840000 |
| (3b) Power Spectrum Density = (3) - 10 log( (3a) / 1000000 ) (dBm/MHz) Note: For FR1 downlink, (3b) should satisfy the following: For 4GHz frequency, 24 and 33 For 2.6 GHz frequency, 33 For 700MH and 2GHz frequency, 36 Note: For FR2 downlink, the following should be satisfied: 40 dBm for 100 MHz Urban scenario, 23 dBm for 100 MHz Indoor scenario. Note: no PSD constraint for uplink | 17.16 |
| (3c) bandwidth used for the evaluated channel (Hz) Note: (3c) is identical to the number of PRBs assigned to the channel evaluated. for uplink, (3a) = (3c) | 3840000 |
| (3bis) Total transmit power for occupied bandwidth = (3b) + 10 log ( (3c) / 1000000 ) (dBm) | 23.00 |
| (5) total antenna gain at antenna gain component 2 of transmitter = (5a) - (5b) (dB) Note: zero for uplink | 0 |
| (5a) antenna gain at antenna gain component 2 of transmitter = 10 log( (2)/(2a)) (dB) Note: zero for uplink | 0 |
| (5b) antena gain correction factor at antenna gain component 2 of transmitter (dB) Note: zero for uplink | 0 |
| Receiver |  |
| (10a) Number of [receive TxRUs] Note: this row is void (empty) for downlink | 2 |
| (10b) Number of receive chains modelled in LLS | 2 |
| (11bis) total antenna gain at antenna gain component 2 of receiver = (11bis-a) - (11bis-b) (dB) Note: zero for downlink | 0 |
| (11bis-a) antenna gain at antenna gain component 2 of receiver = 10 log( (10a)/(10b)) (dB) Note: zero for donwlink | 0 |
| (11bis-b) antena gain correction factor at antenna gain component 2 of receiver (dB) Note: zero for downlink | 0 |
| (13) Receiver noise figure (dB) | 5 |
| (14) Thermal noise density (dBm/Hz) | -174 |
| (15) Receiver interference density (dBm/Hz) | -165.7 |
| (16) Total noise plus interference density = 10 log (10^(( (13) + (14))/10) + 10^((15)/10)) (dBm/Hz) | -164.0 |
| (18) Effective noise power = (16) + 10 log((3c)) (dBm) | -98.2 |
| (19) Required SNR (dB) | 5 |
| (20) Receiver implementation margin (dB) | 2 |
| (21) H-ARQ gain (dB) or Process gain for UMTS Note: Only applicable if HARQ is not considered in LLS, | 25 |
| (22) Receiver sensitivity = (18) + (19) + (20) – (21) (dBm) | -116.2 |
| (22bis) MCL = (3bis) - (22) + (5) + (11bis) (dB) | 139.2 |

    - The following channels/signals are used:

      - PUSCH for VoIP

      - PUCCH format 1

      - PUCCH format 3 with 11bits

      - SSB

      - PRACH format 0

      - PDCCH of Msg.2

      - PDSCH for Msg.2

      - PUSCH of Msg.3

      - PDSCH of Msg.4

      - PDCCH

- Agreements:

- Capture the following table (working assumption) showing the result of bottleneck identification by using absolute metrics

- Agreements:

- For FR1, the potential bottleneck channels identified by absolute metrics can be further filtered by using relative difference between channels in MIL

  - FFS details, including the possibility of applying the relative difference for a limited set of scenario(s)

Agreements:

- For Rural with long distance 4GHz TDD LOS O2O scenario, 12km is applied for observation and bottleneck identification

Agreements:

- The following channels are identified as the potential bottleneck channels derived from the absolute metrics (i.e. service dependent metric and scenario dependent metrics) and the relative metric (i.e. relative difference between channels)

  - 1st priority

    - PUSCH for eMBB (for FDD and TDD with DDDSU, DDDSUDDSUU and DDDDDDDSUU)

    - PUSCH for VoIP (for FDD and TDD with DDDSU, DDDSUDDSUU)

  - 2nd priority

    - PRACH format B4

    - PUSCH of Msg.3

    - PUCCH format 1

    - PUCCH format 3 with 11bit

    - PUCCH format 3 with 22bit

    - Broadcast PDCCH

R1-2007581	Evaluation on the baseline performance for FR1	Huawei, HiSilicon

R1-2007678	Evaluation on NR coverage performance for FR1	vivo

R1-2007741	Discussion on baseline coverage performance for FR1	ZTE

R1-2007872	Baseline coverage performance for FR1	CATT

R1-2007904	Baseline coverage performance for uplink in FR1	Indian Institute of Tech (H)

R1-2007931	FR1 PUSCH Baseline Coverage Performance	Sierra Wireless, S.A.

R1-2007952	On baseline coverage performance for FR1	Intel Corporation

R1-2007993	Updated baseline performance for NR coverage enhancements for FR1	China Telecom

R1-2008024	Discussion on the baseline performance in FR1	CMCC

R1-2008089	Baseline coverage performance for FR1	Xiaomi

R1-2008179	Baseline coverage performance using LLS for FR1	Samsung

R1-2008269	Evaluation on NR coverage performance for FR1	OPPO

R1-2008343	Link and System Evaluation of Coverage for FR1	Ericsson LM

R1-2008377	Baseline coverage performance analysis in FR1	Panasonic Corporation

R1-2008380	Target value for FR1 voice coverage enhancements	SoftBank Corp.

R1-2008398	Link budget analysis for FR1	Sharp

R1-2008478	Evaluation on FR1 coverage performance	Apple

R1-2008481	FR1 baseline coverage performance using LLS	InterDigital, Inc.

R1-2008515	Discussion on scenarios for FR1 baseline performance evaluation	MediaTek Inc.

R1-2008557	Baseline coverage performance for FR1	NTT DOCOMO, INC.

R1-2008624	Baseline FR1 coverage performance	Qualcomm Incorporated

R1-2008701	Baseline coverage evaluation of UL and DL channels – FR1	Nokia, Nokia Shanghai Bell

#### FR2

R1-2009342	Summary on A.I. 8.8.1.2 baseline coverage performance using LLS for FR2 Moderator (Nokia/NSB)

[103-e-NR-CovEnh-03] Eamil discussion for FR2 coverage performance – Marco (Nokia)

- 1st check point: 10/29

- 2nd check point: 11/4

- 3rd check point: 11/10

- Last check point 11/12

Conclusion:

- All the parameters/values/configurations related to FR2 modelling for which an agreement has not been reached among companies prior to RAN1 #103-e, will be henceforth treated according to the “reported by companies” principle. RAN1 will not spend further time during RAN1 #103-e on the resolution of these issues.

Agreements:

The amount of available results for UL channels in FR2 should be considered as given by the total number of available results for both 23 dBm and 12 dBm, given that they can be derived one from the other by simple subtraction, and where each company is counted only once.

Agreements :

- For FR2, representative values are computed according to agreements made for FR1 related on representative value calculation method;

- For FR2, classification of scenarios/channels/frame structures into 1st priority and 2nd priority as follows:

  - 1st priority has enough available results, i.e., larger than 2;

  - 2nd priority has less than 3 available results.

- No categorization by other simulation parameters (such as UE speed, antenna array gain correction factors, UE Tx power) will be introduced for FR2.

- At least for FR2

  - RAN1 discussion will focus on 1st priority scenarios/channels/frame structures for drawing observations and bottleneck identification.

  - RAN1 discussion will focus on 2nd priority scenarios/channels/frame structures on a low priority basis, i.e., after discussion on 1st priority scenarios/channels/frame structures.

  - If results presented for 2nd priority scenarios/channels/frame structures are used by RAN1 for neither representative value derivation nor coverage bottleneck identification, they

    - will still be captured in the Appendix of the TR for completeness;

    - can be used to make additional observations to be captured in the TR.

    - cannot be used to draw conclusions to be captured in the TR.

Agreements :

If absolute ISD/MPL targets are agreed to be used for coverage bottleneck identification then the following targets are considered for FR2:

  - Dense Urban: ISD = 200m; MPL = [123.1] dB;

  - Indoor: ISD = [20]m; MPL = [94.03] dB

FFS: If MIL targets are also considered for control and data channels.

Agreements:

Scenarios, with corresponding frame structures, are classified as follows:

- 1st priority

  - Urban 28 GHz O2I, DDDSU

  - Urban 28 GHz, O2O DDDSU

  - Urban 28 GHz, O2I, DDSU [Only PUSCH VoIP, PUSCH and PDSCH]

  - Indoor 28 GHz, DDDSU

- 2nd priority

  - Indoor 28 GHz, DDSU

  - Urban 28 GHz, O2I, DDSU [only PDSCH of msg2]

  - Urban 28 GHz, O2O DDSU

  - Suburban 28 GHz, O2I DDDSU

  - Suburban 28 GHz, O2O DDSU

Channels are classified as follows:

- 1st priority

  - PUSCH for eMBB (12 dBm and 23 dBm) [DDDSU and DDSU]

  - PUSCH for VoIP (12 dBm and 23 dBm) [DDDSU and DDSU (Only for Urban)]

  - PUCCH Format 1 with 2bits (12 dBm and 23 dBm) [DDDSU]

  - PUCCH Format 3 with 11bits (12 dBm and 23 dBm) [DDDSU]

  - PUCCH Format 3 with 22bits (12 dBm and 23 dBm) [DDDSU]

  - SSB [DDDSU]

  - PRACH format B4 (12 dBm and 23 dBm) [DDDSU]

  - PDCCH for Msg.2 [DDDSU]

  - PUSCH for Msg.3 (12 dBm and 23 dBm) [DDDSU]

  - PDSCH for Msg.4 [DDDSU]

  - PDCCH [DDDSU]

  - PDSCH for eMBB [DDDSU and DDSU (Only for Urban)]

- 2nd priority

  - PUSCH for CSI with 11 bits (12 dBm and 23 dBm)

  - PUSCH for CSI with 22 bits (12 dBm and 23 dBm)

  - PUCCH with 3-HARQ-ACK bits + SR (12 dBm and 23 dBm)

  - PUCCH with HARQ-ACK for Msg.4 (12 dBm and 23 dBm)

  - PRACH format C2 (12 dBm and 23 dBm)

  - PDSCH of Msg.2

  - PUCCH Format 0 (12 dBm and 23 dBm)

  - PUCCH Format 2 (12 dBm and 23 dBm)

Agreements:

If absolute ISD/MPL targets are agreed to be used for coverage bottleneck identification then the following targets are considered for FR2:

  - Dense Urban: ISD = 200m; MPL = 123.1 dB;

  - Indoor: ISD = 20m; MPL = 94.03 dB;

Where MPL values are calculated from ISD targets using the following equations (add reference when capturing into the TR)

FFS d3D with respect to ISD

FFS: If absolute MIL targets are also considered for coverage bottleneck identification including possible different targets for data and control channels.

Update on 11/11:

Agreements:

For target MPL calculation associated to agreed ISD targets,  is equal to the target range calculated by ISD/.

Agreements:

PUSCH for SIP invite is added to the list of 2nd priority channels for FR2.

Agreements:

- Performance targets for FR2 are calculated, for all scenarios, as follows:

- [Absolute] ISD targets are used to find corresponding absolute target MPL values;

- [Relative] Relative differential MIL value of a target channel is calculated considering PUCCH F1 as reference channel, as follows:

  - (MIL of the target channel) – (MIL of PUCCH F1)

- Coverage bottleneck identification for FR2 is performed using at least absolute MPL and relative differential MIL targets, as follows:

- Absolute MPL targets are used to filter the channels/signals, i.e., the candidate bottleneck channels.

- Filtered channels/signals whose relative differential MIL value is negative are considered as a potential coverage bottleneck.

- The necessary link budget increase for each bottleneck channel/signal, expressed in the form of MPL increase (in dB), that enhancements for that channel/signal that is to be targeted, are obtained by changing the sign of the relative differential MIL value of the channel/signal.

- Other options to draw additional observations from collected results are not precluded.

Agreements:

- The following channels are identified as the potential bottleneck channels for 28 GHz scenario

    - PUSCH eMBB (DDDSU and DDSU)

    - PUSCH VoIP (DDDSU and DDSU)

    - PUCCH F3 11bits

    - PUCCH F3 22bits

    - PRACH B4

    - PUSCH of Msg3

    - PUCCH F1

- No evident coverage bottleneck is identified for Indoor scenario for FR2

Agreements :

- PRACH B4 is reference to assess how many additional dBs over the baseline PRACH enhancements may target.

- PUCCH F3 with either 11 bits or 22 bits or both is reference to assess how many additional dBs over the baseline PUCCH F3 enhancements may target

Agreements:

For FR2, coverage bottleneck identification and discussion on enhancements will not include aspects related to the deprioritized Suburban scenario.

R1-2007582	Evaluation on the baseline performance for FR2	Huawei, HiSilicon

R1-2007679	Evaluation on NR coverage performance for FR2	vivo

R1-2007742	Discussion on baseline coverage performance for FR2	ZTE

R1-2007873	Baseline coverage performance for FR2	CATT

R1-2007953	On baseline coverage performance for FR2	Intel Corporation

R1-2008025	Discussion on the baseline performance in FR2	CMCC

R1-2008090	Evaluation on NR coverage performance for FR2	Xiaomi

R1-2008180	Baseline coverage performance using LLS for FR2	Samsung

R1-2008270	Evaluation on NR coverage performance for FR2	OPPO

R1-2008344	Link and System Evaluation of Coverage for FR2	Ericsson LM

R1-2008482	FR2 baseline coverage performance using LLS	InterDigital, Inc.

R1-2008558	Baseline coverage performance for FR2	NTT DOCOMO, INC.

R1-2008625	Baseline FR2 coverage performance	Qualcomm Incorporated

R1-2008702	Baseline coverage evaluation of UL and DL channels – FR2	Nokia, Nokia Shanghai Bell

### Potential techniques for coverage enhancements

#### PUSCH coverage enhancement

R1-2009320	FL summary of PUSCH coverage enhancements	Moderator (China Telecom)

[103-e-NR-CovEnh-04] Eamil discussion for PUSCH coverage enhancement – Jianchi (CT)

- 1st check point: 10/29

- 2nd check point: 11/4

- 3rd check point: 11/10

- Last check point 11/12

Agreements:

For the agreement made in RAN1 #102-e:

Agreements: - Study following power domain based solution for PUSCH enhancements - Waveform design to optimize MPR/A-MPR - [FDD high power UE] - Power boosting for pi/2 BPSK Note: if a LS to RAN4 (for the last two bullets) is deemed necessary, target sending the LS in the 1st week of RAN1#103-e.

RAN1 targets to make a decision whether to further study on power boosting for pi/2 BPSK during this e-meeting.

- Agreements: Capture the followings into the TR

- Enhancements on inter-slot frequency hopping were studied from several aspects, including:

  - More frequency offsets, e.g. 4 for BWP less than 50 PRBs, 8 for BWP greater than 50 PRBs.

  - More frequency hopping positions, e.g. 4.

- Potential specification impacts of enhancements on inter-slot frequency hopping include:

  - Frequency domain hopping offsets/positions.

- Agreements: Capture the followings into the TR

- Enhancements on DM-RS density were studied from several aspects, including lower DM-RS density in time domain, DM-RS sharing among multiple PUSCH transmissions in the time domain, lower DMRS density in frequency domain, 1-comb DM-RS, e.g., DM-RS with single port spans to occupy the whole DM-RS symbol, and additional DM-RS symbol position in a slot.

- Potential specification impacts of lower DM-RS density in time domain, and DM-RS sharing among multiple PUSCH transmissions include:

  - DM-RS pattern and configuration, power consistency, phase continuity, and TBS determination.

- Potential specification impacts of lower DMRS density in the frequency domain include:

  - DM-RS design, DM-RS pattern and configuration.

- Potential specification impacts of 1-comb DM-RS include:

  - DM-RS design, and TBS determination.

- Potential specification impacts of additional DM-RS symbol position in a slot include:

  - DM-RS position.

- Agreements: Capture the followings into the TR

- Adaptive DM-RS configuration was studied. Potential specification impacts include:

  - Related signaling design.

- Agreements: Capture the followings into the TR

- DM-RS balancing among frequency hops was studied. Potential specification impacts include:

  - Related signaling design, DMRS configuration and pattern.

- Update on 11/4

- Agreements: Capture the followings into the TR

- Enhancements on PUSCH repetition type A were studied from several aspects, including increasing the maximum number of repetitions, the number of repetitions counted on the basis of available UL slots and flexible symbol resource allocation in different slots.

- Potential specification impacts of enhancements on increasing the maximum number of repetitions include:

  - TDRA (Time-Domain Resource Allocation).

- Potential specification impacts of enhancements on the number of repetitions counted on the basis of available UL slots include:

  - TDRA (Time-Domain Resource Allocation).

  - Mechanism to determine transmission occasion of actual repetition.

  - Mechanism to determine whether flexible special slot can be determined as an available UL slot.

- Potential specification impacts of enhancements on flexible symbol resource allocation in different slots include:

  - TDRA (Time-Domain Resource Allocation).

- Mechanism to determine UL symbols for each slot.

Agreements: Capture the followings into the TR

- TB processing over multi-slot PUSCH was studied from several aspects, including TBS determined based on single slot and transmitted in parts over multiple slots, TBS determined based on multiple slots and transmitted over multiple slots.

- Potential specification impacts of TB processing over multi-slot PUSCH include:

  - TDRA (Time-Domain Resource Allocation), TBS determination, RV determination.

- Note that power consistency, phase continuity and enhancements for DM-RS configurations may or may not be required depending on factors such as cross-slot channel estimation, etc.

Agreements: Capture the followings into the TR

- Joint channel estimation or DM-RS bundling with/without optimization of DMRS location/granularity was studied from several aspects, including cross-slot channel estimation over consecutive slots, cross-slot channel estimation over non-consecutive slots, cross-repetition channel estimation within one slot, and inter-slot frequency hopping with inter-slot bundling to enable cross-slot channel estimation.

- Potential specification impacts of joint channel estimation or DM-RS bundling include:

  - Power consistency and phase continuity, DM-RS placement in special slot and DM-RS configuration.

  - Time domain hopping interval for inter-slot frequency hopping with inter-slot bundling

Agreements: Capture the followings into the TR

- Power boosting for pi/2 BPSK was studied, including beyond 26 dBm as a function of the UL duty cycle.

- Potential specification impacts include

- UE behavior for power boosting based on the UL time domain resource allocation, explicit or implicit signaling, RF requirement.

Agreements: Capture the followings into the TR

- SIP signal compression was studied for enhancement large payload PUSCH including SigComp used for application information compression and the compression efficiency.

- Potential specification impacts include

  - Using compression algorithm to compress the large SIP signaling message in higher layer.

Agreements: Capture the followings into the TR

- Dynamic PUSCH waveform adaptation was studied. Potential specification impacts include:

  - Related signaling design.

- Agreements: Capture the followings into the TR

- Enhancements on PUSCH repetition type B were studied from several aspects, including actual PUSCH transmission across the slot boundary/invalid symbols, the length of actual repetition larger than 14 symbols, and RV enhancement.

- Potential specification impacts of enhancements on PUSCH repetition type B include:

  - TBS determination, DM-RS pattern, TDRA (Time-Domain Resource Allocation), RV determination,

  - Note that power consistency and phase continuity may or may not be required depending on factors such as cross-slot channel estimation, etc.

- Agreements:

- Sub-PRB transmission for VoIP was studied from several aspects, including number of tones, sub-PRB transmission with single slot and sub-PRB transmission with multi-slot aggregation.

- Potential specification impacts of sub-PRB transmission with single slot include:

  - Frequency domain resource allocation, TBS determination, DM-RS pattern, hopping pattern within/between the PRBs, PUSCH signal generation for DFT-s-OFDM waveform, RF requirement.

- Potential specification impacts of sub-PRB transmission with multi-slot aggregation include:

  - Frequency domain resource allocation, time domain resource allocation, TBS determination, DM-RS pattern, RV determination, hopping pattern within/between the PRBs, PUSCH signal generation for DFT-s-OFDM waveform, RF requirement.

- Agreements: Capture the followings into the TR

- Enhancements on intra-slot frequency hopping were studied from several aspects, including:

  - More frequency offsets, e.g. 4 for BWP less than 50 PRBs, 8 for BWP greater than 50 PRBs.

  - More frequency hopping positions, e.g. 3.

  - More time-domain hop positions within a slot, e.g. 3.

  - DM-RS sharing among multiple PUSCH transmissions with the same frequency position between two consecutive slots [add a reference to the section of DM-RS enhancements when capturing in the TR].

- Potential specification impacts of enhancements on intra-slot frequency hopping include:

  - Frequency domain hopping offsets, DM-RS pattern, TBS determination.

  - Power consistency and phase continuity for DM-RS sharing among multiple PUSCH transmissions

- Agreements: Capture the followings into the TR

- UE transmit waveform design to reduce MPR was studied from several aspects, including tone reservation, FDSS (Frequency Domain Spectral Shaping) without spectral extension for pi/2 BPSK, and FDSS with and without spectral extension for QPSK.

- Potential specification impacts include

  - Related signalling, design for spectral extension, RF requirements.

- Note: For tone reservation, a fraction of tones allocated to a UE are reserved for the UE to shape its waveform; no data is transmitted on these tones.

- Agreements: Capture the followings into the TR

- Spatial domain based solutions were studies from several aspects, including multiple layer PUSCH transmission with DFT-S-OFDM and Open-loop Tx diversity.

- Potential specification impacts include

  - Mechanism to indicate the support of multiple layer PUSCH transmission with DFT-S-OFDM and to determine the precoder, e.g. reuse a subset of the R15 codebooks.

  - Signalling related to support of Tx diversity for PUSCH with DFT-s-OFDM, and different PUSCH spatial filter parameters and different antenna ports for different PUSCH transmissions

- Agreements:

- Capture the following observation into the TR.

Observation:

- Fifteen sources ([China Telecom, R1-2008874], [ZTE, R1-2007743], [Intel, R1-2007954], [Qualcomm, R1-2008626], [Sharp, R1-2008399], [Panasonic, R1-2008378], [DOCOMO, R1-2008559], [Samsung, R1-2009647], [CMCC, R1-2008026], [vivo, R1-2007680], [Ericsson, R1-2008419], [Nokia, R1-2008703], [Apple, R1-2008479], [InterDigital, R1-2009168], [Huawei, R1-2007583]) evaluate the performance of joint channel estimation.

  - Eleven sources ([China Telecom, R1-2008874], [ZTE, R1-2007743], [Qualcomm, R1-2008626], [Sharp, R1-2008399], [Panasonic, R1-2008378], [CMCC, R1-2008026], [vivo, R1-2007680], [Ericsson, R1-2008419], [Nokia, R1-2008703], [Apple, R1-2008479], [Huawei, R1-2007583]) show 0.2~2.1 dB  SNR gain for joint channel estimation over multiple slots for eMBB at 10% iBLER depending on the number of slots for FR1, compared to Rel-16 PUSCH transmission without joint channel estimation.

  - One source ([Intel, R1-2007954]) shows 2 dB  SNR gain for joint channel estimation over multiple non-consecutive slots with inter-slot frequency hopping for eMBB at 10% iBLER, compared to Rel-16 inter-slot frequency hopping without joint channel estimation.

  - Three sources ([Samsung, R1-2009647], [NTT DOCOMO, R1-2008559]) show 0.9~1.3 dB  SNR gain for joint channel estimation over multiple slots for VoIP at 2% rBLER depending on the number of slots for FR1, compared to Rel-16 PUSCH transmission without joint channel estimation.

  - One source ([InterDigital, R1-2009168]) shows 0.3 dB SNR gain for joint channel estimation over multiple slots with DMRS in a special slot for VoIP at 2% rBLER, compared to Rel-16 PUSCH transmission without joint channel estimation.

  - One source shows ([Samsung, R1-2009647]) 0.85~1.1 dB SNR gain for joint channel estimation over multiple slots for VoIP at 2% rBLER depending on the number of slots for FR2, compared to Rel-16 PUSCH transmission without joint channel estimation.

  - One source ([vivo, R1-2007680]) shows 0.8 dB SNR gain for joint channel estimation over multiple repetition within a slot, compared to Rel-16 PUSCH transmission without joint channel estimation.

  - Two sources ([CMCC, R1-2008026], [vivo, R1-2007680]) show 1.0~1.22 dB required SNR gain for lower DM-RS density in time domain with joint channel estimation and using multi-slot PUSCH with 4 symbols in the special slot over multiple slots for eMBB at 10% iBLER for FR1, compared to Rel-16 DM-RS density without joint channel estimation.

  - One source ([vivo, R1-2007680]) shows 1.0 dB required SNR gain for lower DM-RS density in time domain with joint channel estimation over multiple repetition within a slot for eMBB at 10% iBLER for FR1, compared to Rel-16 PUSCH transmission without joint channel estimation.

- Five sources ([China Telecom, R1-2008874], [ZTE, R1-2007743], [Intel, R1-2007954], [Samsung, R1-2009647], [Ericsson, R1-2008419]) evaluate the performance of inter-slot frequency hopping with inter-slot bundling and joint channel estimation.

  - Two sources ([China Telecom, R1-2008874], [Samsung, R1-2009647]) show 0.5~2.5 dB  SNR gain for inter-slot frequency hopping with inter-slot bundling for VoIP at 2% rBLER depending on bundle size, DM-RS configurations for FR1, compared to Rel-16 inter-slot frequency hopping.

  - One source ([Samsung, R1-2009647]) shows 1.0~1.55 dB  SNR gain for inter-slot frequency hopping with inter-slot bundling for VoIP at 2% rBLER depending on bundle size, DM-RS configurations for FR2, compared to Rel-16 inter-slot frequency hopping.

  - Three sources ([ZTE, R1-2007743], [Intel, R1-2007954], [Ericsson, R1-2008419]) show 0.5~3 dB  SNR gain for inter-slot frequency hopping with inter-slot bundling for eMBB at 10% iBLER depending on bundle size for FR1, compared to Rel-16 inter-slot frequency hopping.

  - One source ([Intel, R1-2007954]) shows 1 dB  SNR gain for inter-slot frequency hopping with inter-slot bundling and joint channel estimation over multiple slot for eMBB at 10% iBLER for FR1, compared to Rel-16 inter-slot frequency hopping with joint channel estimation over multiple non-consecutive slots.

Agreements:

- Capture the following observation into the TR.

Observation:

- Two sources ([ZTE, R1-2007743], [Intel, R1-2007954]) evaluate the performance of lower DM-RS density.

  - One source ([ZTE, R1-2007743]) shows around 1.0 dB SNR gain for lower DM-RS density in frequency domain for eMBB at 10% iBLER for FR1, compared to Rel-16 DM-RS density. The results is based on the assumption with only 1 RX antenna, single front-loaded DMRS symbol and without frequency hopping.

  - One source ([Intel, R1-2007954]) shows around 0.2 dB SNR loss for lower DM-RS density in time domain for eMBB at 10% iBLER for FR1, compared to Rel-16 DM-RS density.

Agreements:

- Capture the following observation into the TR.

Observation:

- Three sources ([China Telecom, R1-2008874], [Intel, R1-2007954], [DOCOMO, R1-2008559]) evaluate the performance of higher DM-RS density.

  - One source ([China Telecom, R1-2008874]) shows 0.5~1.5 dB SNR gain for 1-comb DM-RS for eMBB at 10% iBLER for FR1, compared to Rel-16 DM-RS density without power boosting.

  - One source ([DOCOMO, R1-2008559]) shows around 1.0 dB SNR gain for additional DM-RS symbol position for VoIP at 2% rBLER for FR1, compared to Rel-16 DM-RS density with only single DMRS symbol.

  - One source ([Intel, R1-2007954]) shows around 0.05 dB SNR loss for higher DM-RS density in time domain for eMBB 10% iBLER for FR1, compared to Rel-16 DM-RS density.

Agreements:

- Capture the following observation into the TR.

Observation:

- One source ([Qualcomm, R1-2008626]) evaluates the performance of adaptive DM-RS configuration and shows 1.7 dB  SNR gain for eMBB at 10% iBLER for FR1, compared to Rel-16 semi-static DM-RS configuration. For low SNR such as -10 to -12 dB, it shows that adaptive DM-RS configuration can bring 10-100% increase in throughput compared to an ill-suited DMRS configuration depending on factors such as UE speed, DMRS bundling, and PUSCH repetition.

- One source ([China Telecom, R1-2008874]) evaluates the performance of enhanced intra-slot frequency hopping with more frequency offsets/ more frequency hopping positions and shows around 1.8 dB SNR gain for VoIP at 2% rBLER and 0.4 dB SNR gain for eMBB at 2% rBLER for FR1, compared to Rel-16 intra-slot frequency hopping.

- One source (IITH, R1-2007905)) evaluates the performance of power boosting for pi/2 BPSK and shows around 3 dB gain for UL duty cycle less than 50% and around 6 dB gain for UL duty cycle less than 25%.

- One source ([Qualcomm, R1-2008626]) evaluates the performance of dynamic switching between DFT-S-OFDM and CP-OFDM and shows 2~3 dB gain, compared to semi-static switching between DFT-S-OFDM and CP-OFDM when using QPSK modulation.

- One source ([Qualcomm, R1-2008626]) evaluates the performance of UE transmit waveform design to reduce MPR and shows 1 ~1.5 dB gain, compared to Rel-16 DFT-S-OFDM and CP-OFDM.

- One source ([Panasonic, R1-2008378]) evaluates the performance of symbol level repetition and shows around 0.4 dB SNR gain for UE speed 3km/h and around 0.3dB SNR loss for UE speed 120km/h, respectively, for eMBB at 10% iBLER for FR1, compared to Rel-16 PUSCH repetition type A.

- One source ([Mitsubishi, R1-2008743]) evaluates the performance of Alamouti-based transmit diversity and shows 2-2.7dB SNR gain for FR1, and 2-3dB SNR gain with QPSK and up to 8.5dB SNR gain with 16QAM for FR2.

- One source ([Ericsson, R1-2008419]) evaluates the performance of multiple layer PUSCH transmission with DFT-S-OFDM and shows around 3 dB cubic metric gain, compared to multiple layer PUSCH transmission with CP-OFDM.

Agreements: Capture the following observation into the TR.

Observation:

- Five sources ([China Telecom, R1-2008874], [ZTE, R1-2007743], [vivo, R1-2007680], [InterDigital, R1-2009168], [Samsung, R1-2009647]) evaluate the performance of enhancements on PUSCH repetition type B.

  - Three sources ([China Telecom, R1-2008874], [InterDigital, R1-2009168], [Samsung, R1-2009647]) show 0.2~2.0 dB SNR gain when the actual PUSCH transmission can cross the slot boundary, cross-slot channel estimation is used, and the length of actual repetition can be larger than 14 symbols for VoIP at 2% rBLER for FR1 TDD, compared to Rel-16 PUSCH repetition type B without joint channel estimation. HARQ is not used.

  - One source ([ZTE, R1-2007743]) show 0.8 dB required SNR gain when the actual PUSCH transmission can across the slot boundary or the length of actual repetition can be larger than 14 symbols and cross-slot channel estimation is used for VoIP at 2% rBLER for FR1 TDD, compared to Rel-16 PUSCH repetition type B without joint channel estimation. HARQ is not used. Same TB size is used for both baseline and enhancement.

  - One source ([Samsung, R1-2009647]) shows around 1.4 dB SNR gain when the actual PUSCH transmission can cross the slot boundary, cross-slot channel estimation is used, and the length of actual repetition can be larger than 14 symbols for VoIP at 2% rBLER for FR2 TDD, compared to Rel-16 PUSCH repetition type B without joint channel estimation.

  - One source ([InterDigital, R1-2009168]) shows 0.33~1.3 dB SNR gain when the actual PUSCH transmission can cross the slot boundary, cross-slot channel estimation is used, and the length of actual repetition can be larger than 14 symbols for eMBB at 10% iBLER for FR1 TDD, compared to Rel-16 PUSCH repetition type B with 14-symbol actual repetition without joint channel estimation. HARQ is not used.

  - One source ([InterDigital, R1-2009168]) shows the number of RBs can be reduced from 38 to 33, when the actual PUSCH transmission can cross the slot boundary, cross-slot channel estimation is used, and the length of actual repetition can be larger than 14 symbols for VoIP at 2% rBLER for FR1 TDD, compared to Rel-16 PUSCH repetition type B with 14-symbol actual repetition without joint channel estimation. HARQ is not used.

  - One source ([InterDigital, R1-2009168]) shows the number of RBs can be reduced from 30 to 26, when the actual PUSCH transmission can cross the slot boundary, cross-slot channel estimation is used, and the length of actual repetition can be larger than 14 symbols for eMBB at 10% iBLER for FR2 TDD, compared to Rel-16 PUSCH repetition type B with 14-symbol actual repetition without joint channel estimation. HARQ is not used.

  - One source ([vivo, R1-2007680]) shows around 2.0 dB SNR gain for RV enhancement for eMBB at 10% iBLER for FR 1 TDD, compared to Rel-16 PUSCH repetition type B without joint channel estimation.

Agreements: Capture the following observation into the TR.

Observation:

- Six sources ([ZTE, R1-2007743], [Intel, R1-2007954], [Qualcomm, R1-2009729], [vivo, R1-2007680], [Ericsson, R1-2008419], [Huawei, R1-2007583]) evaluate the performance of inter-slot frequency hopping with more frequency offsets/ more frequency hopping positions.

  - Five sources ([ZTE, R1-2007743], [Intel, R1-2007954], [vivo, R1-2007680], [Ericsson, R1-2008419], [Huawei, R1-2007583]) show 0.3~1.7 dB  SNR gain for inter-slot frequency hopping with more frequency offsets/ more frequency hopping positions for eMBB at 10% iBLER for FR1, compared to Rel-16 inter frequency hopping. Cross-slot channel estimation is not used, 2 RX is assumed for gains higher than 0.3dB, [300ns] is assumed for gains higher than 0.3dB.

  - One source ([Qualcomm, R1-2009729]) shows no gain for inter-slot frequency hopping with more frequency offsets/ more frequency hopping positions for eMBB at 10% iBLER for FR1, compared to Rel-16 inter frequency hopping.

  - One source ([Ericsson, R1-2008419]) shows no gain for inter-slot frequency hopping with more frequency offsets/ more frequency hopping positions and joint channel estimation over multiple slots is implemented for eMBB at 10% iBLER for FR1, compared to Rel-16 inter frequency hopping with joint channel estimation over multiple slots.

Agreements: Capture the following observation into the TR.

- Enhancements on PUSCH repetition type A is beneficial for PUSCH coverage enhancements for TDD. It is recommended to support enhancements on PUSCH repetition type A in Rel-17, including the following two options (potential down-selection during the WI phase):

  - Option 1: Increasing the maximum number of repetitions, e.g., up to 32.

  - Option 2: The number of repetitions counted on the basis of available UL slots.

Agreements: Capture the following observation into the TR.

TB processing over multi-slot PUSCH is beneficial for PUSCH coverage enhancements. It is recommended to support TB processing over multi-slot PUSCH in Rel-17, including:

- TBS determined based on multiple slots and transmitted over multiple integer slots.

Agreements: Capture the following observation into the TR.

Joint channel estimation is beneficial for PUSCH coverage enhancements. It is recommended to support Joint channel estimation or DM-RS bundling for PUSCH in Rel-17, including:

- Joint channel estimation over consecutive PUSCH transmissions

- Inter-slot frequency hopping with inter-slot bundling

R1-2007583	Potential solutions for PUSCH coverage enhancement	Huawei, HiSilicon

R1-2007640	PUSCH coverage enhancement	Beijing Xiaomi Mobile Software

R1-2007680	Discussion on Solutions for PUSCH coverage enhancement	vivo

R1-2007743	Discussion on potential techniques for PUSCH coverage enhancements	ZTE

R1-2007874	Discussion on potential techniques for PUSCH coverage enhancement	CATT

R1-2007905	PUSCH coverage enhancements	Indian Institute of Tech (H)

R1-2007930	Potential techniques for NR coverage enhancements	Sierra Wireless, S.A.

R1-2007954	On potential techniques for PUSCH coverage enhancement	Intel Corporation

R1-2007989	PUSCH coverage enhancement	ETRI

R1-2007994	Discussion on PUSCH coverage enhancements	China Telecom

R1-2008026	Discussion on the PUSCH coverage enhancement	CMCC

R1-2008078	Discussion on PUSCH coverage enhancement	NEC

R1-2008092	Potential solutions for PUSCH coverage enhancement	Spreadtrum Communications

R1-2008181	PUSCH coverage enhancement	Samsung

R1-2008271	Consideration on PUSCH coverage enhancement	OPPO

R1-2008370	On PUSCH coverage enhancement techniques	Sony

R1-2008378	Discussion on PUSCH coverage enhancements	Panasonic Corporation

R1-2008399	PUSCH coverage enhancement	Sharp

R1-2008403	Discussions on PUSCH coverage enhancement	LG Electronics

R1-2008419	PUSCH coverage enhancement	Ericsson

R1-2008479	On potential techniques for PUSCH coverage enhancement	Apple

R1-2009168	PUSCH coverage enhancements	InterDigital, Inc.

Revisin of R1-2008483

R1-2008559	Potential techniques for PUSCH coverage enhancements	NTT DOCOMO, INC.

R1-2008626	Potential coverage enhancement techniques for PUSCH	Qualcomm Incorporated

R1-2008700	On the use of Tx diversity in DFT-s-OFDM for PUSCH coverage enhancement	NICT

R1-2008703	Discussion on approaches and solutions for NR PUSCH coverage enhancement	Nokia, Nokia Shanghai Bell

R1-2008729	Discussion on potential techniques for PUSCH coverage enhancement	WILUS Inc.

R1-2008743	On transmit diversity techniques for PUSCH coverage enhancement	Mitsubishi Electric RCE

#### PUCCH coverage enhancement

R1-2009321 	FL summary of PUCCH coverage enhancement Moderator (Qualcomm)

[103-e-NR-CovEnh-05] Eamil discussion for PUCCH coverage enhancement – Yi (Qualcomm)

- 1st check point: 10/29

- 2nd check point: 11/4

- 3rd check point: 11/10

- Last check point 11/12

Agreements:

(Working assumption): For coverage enhancement study for PUCCH with >2 bits UCI, in addition to the 1% BLER performance metric agreed in RAN1 101e, the following performance metric can be considered:

- For UCI with HARQ-ACK payload (with or without CSI/SR payload), the performance metric for HARQ-ACK is 1% DTX to ACK error rate, 1% ACK miss detection (including ACK to NACK and ACK to DTX) error rate, and 0.1% NACK to ACK error rate.

  - The payload size is 3 and 11 bits for HARQ-ACK. Other payload sizes can be evaluated and if so, reported by each individual company

- For UCI with HARQ-ACK and CSI/SR payload, the performance metric for CSI/SR is 1% false alarm rate, 1% BLER [or 10% BLER], 5% undetectable error rate for <=11 bits, and 2% undetectable error rate for >11 bits

  - The payload size is 11 bits or 22 bits, where 4 and 8 bits for HARQ-ACK, respectively. Other payload sizes can be evaluated and if so, reported by each individual company

Note 1: In addition to the results already submitted to RAN1 103e which does not consider DTX detection, for any PUCCH coverage enhancement scheme especially the four prioritized schemes, companies are encouraged to submit more simulation results by 11/10/2020 with DTX detection, considering the above performance metric. Both results with and without DTX detection will be captured in the TR.

Note 2: false alarm rate is the probability that DTX is detected as a correct payload.

Note 3: undetectable error rate = # instances that a UCI payload is declared as correct when the UCI payload is in error / Total # instances that UCI payloads are in error, where a UCI payload is declared as correct if it passes the error detection check (with details up to each company, and to be reported)

Update on 11/11:

Agreements:

For DMRS-less PUCCH, capture the following in the TR

Potential Spec impact:

- A new PUCCH format would need needs to be specified, including the power control of the new PUCCH format. The new PUCCH format is would be an addition to existing PUCCH formats.

- Two approaches to generate sequence for DMRS-less PUCCH (i.e., reuse Rel-15/16 CGS/ZC/Gold/m-sequence or design new sequences) were studied. The potential spec impacts include:

  - If reusing Rel-15/16 CGS/ZC/Gold/m-sequence of the same length being supported by the current Rel-15/16 specification, no new sequences need to be specified.

  - If new sequences (including new sequence type or same type as in Rel-15/16 but with different length) or sequences based on modification of NR Rel-15/16 UCI encoding scheme are adopted, the new sequences or the modification of NR Rel-15/16 UCI encoding scheme need to be specified.

- UCI to sequence mapping and sequence to RE mapping need to be specified

- [UCI info bits size (X) needs to be specified]

- [New RAN4 MPR requirement needs to be defined, if new sequences other than Rel-15/16 CGS/ZC/Gold/m-sequences are adopted]

- [CSI and HARQ-ACK UCI multiplexing for this new PUCCH format need to be specified]

Agreements:

For PUSCH repetition type-B like PUCCH repetition, capture the following in the TR

Impact to receiver:

- gNB needs to process more than one PUCCH repetitions in a slot

- gNB needs to combine multiple repetitions with different code rates/time length

Agreements:

For dynamic PUCCH repetition factor indication, capture the following in the TR

Potential Spec impact:

- a new PUCCH repetition signalling mechanism needs to be specified

Impact to receiver: None

Impact to UE implementation:

- Need implement transmissions of the PUCCH repetitions based on the dynamic indicator

[Impact to system]

- [FFS the impact to system]

Agreements:

For DMRS bundling cross PUCCH repetitions, capture the following in the TR

Potential Spec impact:

- Restrictions to guarantee phase coherency cross repetitions need to be specified

- UE behaviour needs to be defined if the phase coherency of PUCCH repetition is impacted by other procedures

- DMRS bundling with inter-slot frequency hopping pattern enhancement need to be specified, if the frequency hopping enhancement is agreed.

Agreements:

For DMRS bundling cross PUCCH or PUSCH repetitions, send an LS to RAN4 to ask the following

- Under what conditions UE can keep phase continuity cross PUCCH or PUSCH repetitions

  - Whether back-to-back PUCCH or PUSCH repetitions is one of the conditions required to keep phase continuity cross the repetitions

- Power control tolerance level cross PUCCH or PUSCH repetitions

Agreements: For DMRS-less PUCCH, capture Table 1, Table 2, and Table 3 in the TR.

Table 1: Performance (SNR) gain observed for DMRS-less PUCCH over Rel-15/16 baseline

| Simulated scenario | Performance metric | Observed SNR gains | Source |
| --- | --- | --- | --- |
| Scenario 1: 2 bits UCI Baseline: PF1 Enhancement: DMRS-less PUCCH | 1% FA, 1% ACK miss detection, 0.1% NACK->ACK error | 3dB | QC |
|  |  | 3dB | OPPO |
|  |  | 3~4dB | Huawei |
| Scenario 2: 3/4/6 bits UCI Baseline: PF3 Enhancement: DMRS-less PUCCH Note: Intel simulated 3-7 bits UCI | 1% BLER | 3dB | QC |
|  |  | 3dB | Sharp |
|  |  | 1.5 ~ 2.1dB | Eurecom |
|  | 1% FA, 1% BLER | 0dB | Intel |
|  |  | 0.3~0.5dB | VIVO |
|  | 1% FA, 1% ACK miss detection, and 0.1% NACK to ACK | 1~2dB | VIVO |
|  |  | 2.8dB | QC |
|  |  | 0dB | Ericsson |
| Scenario 3: 11 bits UCI Baseline: PF3 Enhancement: DMRS-less PUCCH Note: Intel simulated 8-11 bits UCI | 1% BLER | 3~4dB | QC |
|  |  | 3~4dB | HW |
|  |  | 2~3dB | ZTE |
|  |  | 1.5~2.1dB | Eurecom |
|  |  | 0 ~ 0.2dB | Ericsson |
|  |  | 1 ~ 2.7dB | CMCC |
|  | 1% FA, 1% BER | 0.3dB | Intel |
|  |  | 2.1dB | QC |
|  | 1% FA, 1% ACK miss detection, and 0.1% NACK to ACK error | 4dB | VIVO |
|  |  | 3.8dB | ZTE |
|  |  | 4dB | QC |
|  |  | 0dB | Ericsson |
|  | 1% FA, 1% BLER, and 5% undetectable error rate | 4dB | QC |
| Scenario 3: 22/24 bits UCI Baseline: PF3 Enhancement: DMRS-less PUCCH | 1% BLER | -2dB | Eurecom |
|  |  | 1dB | QC |

Table 2: Performance (PAPR/CM) gain observed for DMRS-less PUCCH over Rel-15/16 baseline

| Modulation order | Observed PAPR/CM gain | Source |
| --- | --- | --- |
| QPSK | 3.5dB PARR gain 1dB CM gain | QC |
|  | 6.3dB PAPR gain | Eurecom |
|  | 4.5dB PAPR gain | Huawei |
| Pi/2 BPSK | 0.5dB PAPR gain 0.6dB CM gain | QC |
|  | 4.8 dB PAPR gain | Eurecom |

Table 3: Key simulation assumptions for DMRS-less PUCCH study over Rel-15/16 baseline

| Company | Key simulation assumptions |
| --- | --- |
| ZTE | Receiver for Rel-15/16 PUCCH: ML coherent receiver Receiver for sequence based PUCCH: ML noncoherent sequence detector |
| Intel | Receiver for Rel-15/16 PUCCH: ML coherent receiver (MMSE channel estimator and equalizer) and non-coherent receiver Receiver for sequence based PUCCH: ML noncoherent sequence detector/correlator |
| Qualcomm | Receiver for Rel-15/16 PUCCH: ML coherent receiver Receiver for sequence based PUCCH: ML noncoherent receiver (correlator with 2D-FFT or fast Hadamard transform) |
| Sharp | Receiver for Rel-15/16 PUCCH: MMSE channel estimation (with genie Doppler and delay spread) + ML coherent detection Receiver for sequence based PUCCH: ML noncoherent sequence detector/correlator |
| CMCC | Receiver for Rel-15/16 PUCCH: ML coherent receiver Receiver for sequence based PUCCH: ML noncoherent sequence detector/correlator |
| vivo | Receiver for Rel-15/16 PUCCH: ML coherent receiver Receiver for sequence based PUCCH: ML noncoherent sequence detector/correlator Ideal noise power estimation is used for both receiver for both legacy PUCCH and new sequence based PUCCH, and the noise power is used only in DTX detection. |
| Ericsson | Receiver for Rel-15/16 PUCCH: conventional and ML noncoherent receiver Receiver for sequence based PUCCH: ML noncoherent receiver |
| EURECOM | Receiver for Rel-15/16 PUCCH: advanced receivers for <=11 bits(non-coherent ML), conventional receiver for 22 bits (LS channel estimation + MMSE/MRC) Receiver for sequence based PUCCH: ML noncoherent sequence detector/correlator for 4/11 bit case; non-coherent LLR unit adapted to 3GPP polar code for 22-bit case. Also simulated low-complexity receiver for 11-bit UCI case. |
| Huawei, HiSi | Receiver for Rel-15/16 PUCCH: 2D-Wiener filter based channel estimation + MMSE equalization Receiver for sequence based PUCCH: CHIRRUP algorithm based sequence detection |
| OPPO | Receiver for Rel-15/16 PUCCH: LMMSE-IRC receiver. Receiver for sequence based PUCCH: ML correlation. |

Agreements: For DMRS-less PUCCH, capture the following in the TR

- Receiver needs to implement a non-coherent sequence detector/correlator for reception of the new PUCCH format.

- [For reception of the new PUCCH format, channel and noise covariance matrix estimation is not required. ]

- Computation efficient implementations of the receiver for the new PUCCH format have been studied. Their complexity can be lower or higher than the decoder for existing NR PUCCH coherent receiver depending on the adopted sequence, on the UCI payload size and on the implementation of the considered coherent receiver.

- [Receiver that uses PUCCH DM-RS for channel parameters estimation, channel tracking, and/or interference estimation must instead use other signals.]

Agreements: For DMRS-less PUCCH, capture the following in the TR

- Receiver implementation for the new PUCCH format is an extension of the PUCCH format 0 receiver with similarity that both are noncoherent sequence detectors, while the new receiver needs to perform correlation over a larger sequence pool. The size of the sequence pool over which the receiver for the new PUCCH format needs to perform correlation increases exponentially with the number of UCI bits.

Agreements: For DMRS-less PUCCH, capture the following in the TR

- UE needs to implement a UCI to sequence mapping and sequence to RE mapping for the new PUCCH format

- Four potential approaches to implement the sequences for DMRS-less PUCCH were studied.

  - Approach 1: Reuse Rel-15/16 CGS/ZC/Gold/m-sequences generation with the same sequence length being supported in Rel-15/16

  - Approach 2: Reuse Rel-15/16 CGS/ZC/Gold/m-sequences generation with a different sequence length being supported in Rel-15/16

  - Approach 3: Modification of NR Rel-15/16 UCI encoding scheme to generate the sequences

  - Approach 4: implement a new sequence generation which is not covered by above, if the new sequence is adopted in spec.

Agreements: For PUSCH repetition type-B like PUCCH repetition, capture the following in the TR

Restriction of the scheme:

- Only applicable to UCI <=11 bits

Agreemetns: For PUSCH repetition type-B like PUCCH repetition, captured Table 4 in the TR.

Table 4: Performance gain observed for PUSCH repetition Type-B like PUCCH repetition

| Company | Observed performance gain | Key simulation assumptions |
| --- | --- | --- |
| VIVO | 0.5dB (w/o DMRS bundling) 1~1.5dB (w DMRS bundling) Note: the 1~1.5 gain observed is a combination of DMRS bundling gain and type-B PUSCH repetition. | 11 bits UCI, w/ DTX detection, 1% BLER Receiver for Rel-15/16 PUCCH: coherent detection, DTX is performed based on union of DMRS and UCI symbols. Receiver for PUCCH enhancement scheme: with and without joint channel estimation for the consecutive PUCCH repetitions, in addition to receiver for Rel-15 and Rel-16 UEs. Note: Ideal noise power estimation is used for above receivers, and the noise power is used only in DTX detection. |

Agreements: For PUSCH repetition type-B like PUCCH repetition, capture the following in the TR

Potential Spec impact:

- Nominal repetition, actual repetition, segmentation for type B PUCCH repetition, and flexible time domain resource allocation in each slot need to be specified

- Procedure to handle postpone/cancel PUCCH repetitions (including interaction with dynamic SFI) needs to be specified

- [Upper bound on UCI info bits size needs to be specified]

- [PUSCH type B repetition specification can be leveraged]

- [Procedure to transmit actual repetition in DFT-S-OFDM waveform with 1/2/3 OFDM symbols needs to be specified, if 1/2/3 OFDM symbol actual type B PUCCH repetition is supported]

  - [Potentially new DMRS patterns need to be specified]

- The issue of whether supporting type B PUCCH repetitions with different PUCCH formats was studied and three options were identified to resolve this issue:

  - Option 1:  Restrict type B PUSCCH repetition applicable to actual repetitions with the same PUCCH format.

  - Option 2: Allow type B PUCCH repetition with different PUCCH formats. The procedure to handle format switch between repetitions needs to be specified.

  - Option 3: Introduce and specify PUCCH format 3/4 of length 1/2/3 OFDM symbols to support type B PUCCH repetition.

- [Procedure and RAN4 requirements to handle different PUCCH formats (with potential switching between different waveforms of OFDM and DFT-S-OFDM) cross actual repetitions needs to be specified, if option 2 is adopted]

- Power control for actual repetitions needs to be specified

- [CSI and HARQ-ACK multiplexing with type B PUCCH repetition need to be specified]

Agreements: For dynamic PUCCH repetition factor indication, capture Table 5 in the TR.

Table 5: Performance gain observed for Dynamic PUCCH repetition factor indication

| Company | Observed performance gain | Key simulation assumptions |
| --- | --- | --- |
| Ericsson | 5 dB (with repetition factor 8) | 11 bits CSI, w/o DTX detection, 10% BLER Receiver for Rel-15/16 PUCCH: conventional DMRS based receiver Receiver for PUCCH enhancement scheme: conventional DMRS based receiver (without cross slot channel estimation). |
| ZTE | Reducing the number of PUCCH repetitions for more than 70% cases. | 11 bits UCI, w/o DTX detection, 1% BLER |

Agreements: For DMRS bundling cross PUCCH repetitions, capture the following in the TR

Restriction of the scheme:

- Phase coherency cross PUCCH repetitions is required

- The same frequency resource allocation cross PUCCH repetitions is required

- The same power cross PUCCH repetitions is required

Agreements:: For DMRS bundling cross PUCCH repetitions, capture Table 6 in the TR

Table 6: Performance gain observed for DMRS bundling cross PUCCH repetitions over Rel-15/16 baseline

| Company | Observed performance gain | Key simulation assumptions |
| --- | --- | --- |
| ZTE | 1 dB | 22 bits UCI, w/o DTX detection, 1% BLER, 4 PUCCH repetitions Receiver for Rel-15/16 PUCCH: ML coherent receiver, w/o cross-slot channel estimation Receiver for PUCCH enhancement scheme: ML coherent receiver, w/ cross-slot channel estimation |
| Intel | ~1.2 dB | 22 bits UCI, w/o DTX detection, 1% BLER, 8 PUCCH repetitions Receiver for Rel-15/16 PUCCH: coherent receiver, w/o cross-slot channel estimation Receiver for PUCCH enhancement scheme: coherent receiver, w/ cross-slot channel estimation |
| VIVO | 0.85 ~ 1.3 dB | 11 bits UCI, w/ DTX detection, 1% BLER, 2 PUCCH repetitions Receiver for Rel-15/16 PUCCH: Coherent detection, DTX is performed based on union of DMRS and UCI symbols. Channel estimation is performed individually for each repetition. Receiver for PUCCH enhancement scheme: Joint channel estimation is used for PUCCH repetitions in consecutive slots, in addition to receiver for Rel-15 and Rel-16 UEs. Note: Ideal noise power estimation is used for both receivers, and the noise power is used only in DTX detection. |

Agreements: For DMRS bundling cross PUCCH repetitions, capture the following in the TR

- New channel estimator needs to be implemented at receiver to process DMRS across multiple repetitions

- Same phase and transmission power need to be maintained at UE cross PUCCH repetitions

- [Maintaining phase coherence across slots requires UE to alter how slot boundaries events (such as timing or power adjustments) are handled]

R1-2007584	Potential solutions for PUCCH coverage enhancement	Huawei, HiSilicon

R1-2008942	Discussion on Solutions for PUCCH coverage enhancement	vivo

Revision of R1-2007681

R1-2007744	Discussion on potential techniques for PUCCH coverage enhancements	ZTE

R1-2007875	Discussion on potential techniques for PUCCH coverage enhancement	CATT

R1-2007955	On potential techniques for PUCCH coverage enhancement	Intel Corporation

R1-2007995	Discussion on PUCCH coverage enhancements	China Telecom

R1-2008027	Discussion on PUCCH coverage enhancement	CMCC

R1-2008079	Discussion on PUCCH coverage enhancement	NEC

R1-2008182	PUCCH coverage enhancement	Samsung

R1-2008272	PUCCH coverage enhancement schemes	OPPO

R1-2008371	On PUCCH coverage enhancement techniques	Sony

Withdrawn

R1-2008379	Discussion on PUCCH coverage enhancements	Panasonic Corporation

R1-2008400	PUCCH coverage enhancement	Sharp

R1-2008404	Discussions on PUCCH coverage enhancement	LG Electronics

R1-2008420	PUCCH coverage enhancement	Ericsson

R1-2008484	PUCCH coverage enhancements	InterDigital, Inc.

R1-2008560	Potential techniques for PUCCH coverage enhancements	NTT DOCOMO, INC.

R1-2009315	Potential coverage enhancement techniques for PUCCH	Qualcomm Incorporated

Revision of R1-2008627

R1-2008704	Discussion on approaches and solutions for NR PUCCH coverage enhancement	Nokia, Nokia Shanghai Bell

R1-2008730	Discussion on potential techniques for PUCCH coverage enhancement	WILUS Inc.

R1-2008756	PUCCH coverage enhancements	Indian Institute of Tech (H)

R1-2008759	Low-PAPR Sequence-Based Approaches for PUCCH Coverage Enhancement	EURECOM

#### Coverage enhancement for channels other than PUSCH and PUCCH

R1-2009322	Feature lead summary on coverage enhancement for channels other than PUSCH and PUCCH Moderator (ZTE Corporation)

[103-e-NR-CovEnh-06] Eamil discussion for coverage enhancement for channels other PUSCH and PUCCH – Xianghui (ZTE)

- 1st check point: 10/29

- 2nd check point: 11/4

- 3rd check point: 11/10

- Last check point 11/12

Update on 10/29: to check right after the end of the silence period for proposals 9 and 11.

Update on 11/2:

Agreements: Capture the followings into the TR.

- PUCCH repetition carrying HARQ-ACK for Msg4 was studied. Potential specification impacts include related signaling design, differentiation between enhanced UE and legacy UE.

Agreements: Capture the followings into the TR

- Beam reporting during initial/random access procedure was studied from several aspects, including the best SSB, alternative SSB beam and early CSI report in Msg3 PUSCH. Potential specification impacts include signaling design in Msg3 PUSCH, CSI-RS resources configured during initial access, beam indication for the following steps for RACH procedure.

Update on 11/4:

- Agreements: Capture the followings into the TR

- PRACH enhancements were studied from several aspects, including multiple PRACH transmissions with the same beam, multiple PRACH transmissions with different beams, and PRACH enhancements with finer beam.

- Potential specification impacts of multiple PRACH transmissions include:

- For multiple PRACH transmissions with the same transmission beam and multiple PRACH transmissions with different transmission beams,

  - mechanism on triggering/initiating multiple PRACH transmissions,

  - determination of number of transmissions and transmission pattern,

  - differentiation between enhanced UE and legacy UE and

  - possible collision handling between PRACH transmission with and without multiple PRACH transmissions.

- Only for multiple PRACH transmissions with different transmission beams,

  - transmission beam to be used for each initial transmission and

  - beam determination for the following steps in RACH procedure.

- Potential specification impacts of PRACH enhancements with finer beam include finer beam for PRACH based on CSI-RS resources configured during initial access.

- Agreements: Capture the followings into the TR

Broadcast PDCCH repetition was studied. Potential specification impacts include PDCCH repetition configuration.

Agreements: Capture the followings into the TR

- Msg4 PDSCH enhancements were studied from several aspects, including introducing early CSI on Msg3 PUSCH for early link adaptation , scaling factor for TBS determination and PDSCH repetition.

- Potential specification impacts of early CSI on Msg3 PUSCH for early link adaptation include:

  - CSI-RS resources configured during initial access.

- Potential specification impacts of scaling factor for TBS determination include:

  - TBS determination.

- Potential specification impacts of PDSCH repetition include:

  - PDSCH repetition configuration, DMRS design among PDSCH repetitions.

- Agreements: Capture the followings into the TR

Enhancements on Msg3 PUSCH repetition were studied from several aspects, including the indication of the number of repetitions for Msg3 initial transmission and re-transmission, the repetition type, the feasibility and applicability of enhancements studied for PUSCH in RRC_CONNECTED state for Msg3 PUSCH initial and re-transmission, inter-slot frequency hopping and differentiation between enhanced UE and legacy UE.

- Potential specification impacts of indication of the number of repetitions for Msg3 initial transmission include:

  - Explicit indication mechanism, e.g., indicated by RAR UL grant, DCI format 1_0 with CRC scrambled by RA-RNTI or SIB1.

  - Implicit indication mechanism, e.g., determined by PRACH configuration or information carried by RAR.

- Potential specification impacts of indication of the number of repetitions for Msg3 re-transmission include:

  - Explicit indication mechanism, e.g., indicated by DCI format 0_0 with CRC scrambled by TC-RNTI.

  - Implicit indication mechanism, e.g., determined by Msg3 initial transmission.

- Potential specification impacts of the repetition type include:

  - Introducing PUSCH repetition Type A.

- Potential specification impacts of the feasibility and applicability of enhancements studied for PUSCH in RRC_CONNECTED state for Msg3 PUSCH initial and re-transmission include:

  - The potential specification impacts for the solutions studied in Section 6.1.

- Potential specification impacts of inter-slot frequency hopping include:

  - Inter-slot frequency hopping configuration and frequency hopping pattern.

- Potential specification impacts of differentiation between enhanced UE and legacy UE include:

  - Mechanism to differentiate enhanced UE and legacy UE, e.g., separate PRACH configurations (e.g, separate PRACH occasions or preambles) and separate Msg3 configurations (e.g., separate DMRS ports).

Agreements: Capture the followings into the TR

Power domain-based solutions were studied for Msg3 PUSCH, including pi/2 BPSK waveform using DFT-s-OFDM and power control enhancements.

- Potential specification impacts of pi/2 BPSK waveform using DFT-s-OFDM include defining the usage of pi/2 BPSK modulation for Msg3 and either explicit or implicit power boosting based on the Msg3 time domain resource allocation.

- Potential specification impacts of power control enhancements include configuration of multiple sets of power control parameters.

Update on 11/11:

Agreements: Capture the followings into the TR

CSI repetition on PUSCH was studied. Potential specification impacts include mechanism to determine A-CSI repetitions on PUSCH, e.g. A-CSI request and/or repetition factor in UL DCI, one A-CSI in each PUSCH repetition, and PUSCH repetition type A.

Agreements: Capture the followings into the TR

Spatial domain based solutions were studied from several aspects for Msg3 PUSCH, including spatial filter setting between PRACH transmission and corresponding Msg3 PUSCH transmission and open-loop transmission diversity.

- Potential specification impacts of spatial filter setting between PRACH transmission and corresponding Msg3 PUSCH transmission include specifying the same spatial filter between PRACH transmission and corresponding Msg3 PUSCH transmission, mechanism to differentiate enhanced UE and legacy UE.

- Potential specification impacts of open-loop transmission diversity include, mechanism to indicate support of transmission diversity for Msg3 PUSCH with DFT-s-OFDM, mechanism to differentiate enhanced UE and legacy UE, mechanism to determine the precoder cycling pattern during random access procedure, e.g. on different Msg3 PUSCH repetitions.

Agreements: Capture the followings into the TR

Compact DCI and PDCCH-less for broadcast PDCCH were studied for broadcast PDCCH.

- Potential specification impacts of compact DCI include mechanism for DCI bit field design for fallback DCI.

- Potential specification impacts of PDCCH-less include the mechanism to indicate the scheduling information for broadcast PDSCH carrying SIB messages.

Agreements: Capture the following observations into the TR

Observation 1:

Nine sources ([ZTE], [Intel], [NTT DOCOMO], [CMCC], [vivo], [Ericsson], [Nokia/NSB], [Huawei, HiSilicon], [Apple]) evaluated the performance of enhancements on Msg3 repetition.

- Eight sources show about 2 dB gain when the number of repetitions is doubled in FR1.

- One source shows 4.27 dB gain when the number of repetitions is increased to 8 in FR2.

- One source shows 1.1~1.75 dB gain when performing cross-slot channel estimation among 2 repetitions.

- One source shows 0.5~1.07 dB gain when performing cross-slot channel estimation among 4 repetitions.

- One source shows 3.8 dB gain with 2 repetitions and inter-slot hopping comparing with no repetition and no intra-slot hopping.

- One source shows 3.2 dB gain with 2 repetitions and inter-slot hopping comparing with no repetition and intra-slot hopping.

Observation 2:

One source ([IITH, IITM, CEWIT, Reliance Jio, Tejas Networks]) evaluated the performance of power boosting using pi/2 BPSK waveform for Msg3 and shows 3 dB gain for UL duty cycle lower than 50% and 6 dB gain for UL duty cycle lower than 25%.

Observation 3:

Three sources ([ZTE],[NTT DOCOMO], [Qualcomm]) evaluated the performance of enhancements on PDCCH repetition.

- Two sources show 2 dB gain and one source shows 2.8~3.1 dB gain when the number of repetitions is increased to 2.

- One source shows 4~5.8 dB gain and one source shows 4 dB gain when the number of repetitions is increased to 4.

- One source shows about 3dB and 6dB gain if DMRS bundling is considered for 2 and 4 repetitions respectively.

Observation 4:

One source [[NTT DOCOMO]] evaluated the performance of compact DCI and shows 1.5 dB gain if the number of DCI payload size is reduced from 40 bits to 20 bits.

Observation 5:

One source ([ZTE]) evaluated the performance of increasing the number of SSBs and shows 1.84 dB gain when the number of SSBs is increased from 4 to 8 at 700MHz in rural scenario.

Observation 6:

Two sources ([ZTE], [Nokia/NSB]) evaluated the performance of PRACH enhancements.

- One source ([ZTE]) shows 3.7 dB and 5.2 dB gain when performing 2 and 4 PRACH transmissions with the same transmission beam respectively at 4 GHz in urban scenario.

- One source ([ZTE]) shows 1.7 dB and 3.7 dB gain when performing 2 and 4 PRACH transmissions with the same transmission beam respectively at 28 GHz in urban scenario.

- One source ([Nokia/NSB])) shows 2 dB gain when performing 2 PRACH transmissions with different transmission beams at 2 GHz in rural scenario.

- One source ([Nokia/NSB]) shows 2 dB and 4.7 dB gain when performing 2 and 4 PRACH transmissions with different transmission beams respectively at 28 GHz in urban scenario.

Observation 7:

- One source ([ZTE]) evaluated the performance of PUCCH repetition with HARQ-ACK for Msg4 and shows 3 dB and 6 dB gain when the number repetitions is increased to 2 and 4 respectively at 2 GHz in rural scenario.

Observation 8:

- One source ([Ericsson]) evaluated the performance of A-CSI repetition on PUSCH and shows 4 dB gain for 8 repetitions with 11 bits CSI at 10% BLER target at 4GHz.

Agreements: Capture the followings into the TR

UE awareness of paired orthogonally polarized SSBs has been studied. Potential specification impacts of dual polarized SSBs with the same spatial filter setting include mechanisms to ensure UE awareness of polarization properties of SSBs, e.g., communication of paired SSB indices associated with the same spatial filtering and different polarizations, to the UE.

Agreements: Capture the followings into the TR

A-CSI on PUCCH to allow A-CSI repetition was studied. Potential specification impacts include

- mechanism to determine the repetition of A-CSI PUCCH, e.g. CSI request and/or repetition factor in the downlink DCI, configuration of repetition levels per PUCCH resource, and related timeline,

- mechanism for the PUCCH resource determination, e.g. based on existing PUCCH resource configuration framework in DL DCI (i.e., DCI format 1_0, 1_1, 1_2), existing PUCCH formats that can carry CSI.

- RS resource for CSI measurement (e.g. aperiodic CSI-RS, DMRS)

R1-2007585	Discussion on the potential coverage enhancement solutions for other channels	Huawei, HiSilicon

R1-2007682	Discussion on coverage enhancement for channels other than PUCCH and PUSCH	vivo

R1-2007745	Discussion on potential techniques for channels other than PUSCH and PUCCH	ZTE

R1-2007876	Discussion on coverage enhancement for channels other than PUSCH and PUCCH	CATT

R1-2007906	Coverage enhancement for Msg3	Indian Institute of Tech (H)

R1-2007956	On coverage enhancement for channels other than PUSCH and PUCCH	Intel Corporation

R1-2007996	Discussion on Msg3 PUSCH enhancements	China Telecom

R1-2008028	Discussion on coverage enhancement for channels other than PUSCH and PUCCH	CMCC

R1-2008080	Discussion on Msg3 coverage enhancement	NEC

R1-2008183	Coverage enhancement for channels other than PUSCH and PUCCH	Samsung

R1-2008273	Coverage enhancement on NR channels other than PUSCH and PUCCH	OPPO

R1-2008310	Coverage Enhancements for initial access	AT&T

R1-2008372	Coverage enhancement for initial access	Sony

R1-2008401	Coverage enhancement for channels other than PUCCH/PUSCH	Sharp

R1-2008405	Discussions on coverage enhancement for channels other than PUSCH and PUCCH	LG Electronics

R1-2008421	Coverage enhancement for channels other than PUSCH and PUCCH	Ericsson

R1-2008480	Discussion on Msg3 coverage enhancement	Apple

R1-2008485	Coverage enhancements for initial access	InterDigital, Inc.

R1-2008561	Potential techniques for coverage enhancement for channels other than PUSCH and PUCCH			NTT DOCOMO, INC.

R1-2009309	Potential coverage enhancement techniques for other channels	Qualcomm Incorporated

Revision of R1-2008628

R1-2008705	Discussion on approaches and solutions for NR coverage enhancement: other channels than PUSCH and PUCCH	Nokia, Nokia Shanghai Bell

R1-2008716	Potential techniques for Msg3 PUSCH coverage enhancement	Potevio

R1-2008731	Discussion on potential techniques for coverage enhancement for channels other than PUSCH and PUCCH	WILUS Inc.

### Others

R1-2007683	Considerations on Parameters for Coverage Evaluation	vivo

R1-2007746	Discussion on target performance for NR coverage enhancements	ZTE

R1-2007877	Discussion on remaining issues for coverage enhancement	CATT

R1-2007957	On simulation assumptions for NR coverage enhancement	Intel Corporation

R1-2008274	Functionality of Coverage Enhancement and other WI	OPPO

R1-2008422	Coverage Parameter Sensitivity and Network Enhancement	Ericsson

R1-2008487	Discussion on simulation assumptions for VoIP	InterDigital, Inc.

R1-2008629	Other coverage enhancement aspects	Qualcomm Incorporated

R1-2008706	Evaluation assumptions for NR coverage enhancement evaluation	Nokia, Nokia Shanghai Bell

## Rel-17 enhancements for NB-IoT and LTE-MTC

Please refer to RP-201306 for detailed scope of the WI

### Support of 16-QAM for unicast in UL and DL for NB-IoT

R1-2007618	Support of 16QAM for unicast in UL and DL in NB-IoT	Huawei, HiSilicon

R1-2008073	Support of 16-QAM for NB-IoT	Nokia, Nokia Shanghai Bell

R1-2008697	Discussion on UL and DL 16QAM for NB-IoT	ZTE

R1-2008920	Support 16QAM for NBIoT	Lenovo, Motorola Mobility

R1-2008930	Support of 16-QAM for unicast in UL and DL in NB-IoT	Ericsson

R1-2008969	Further considerations on support of 16QAM for NB-IOT	MediaTek Inc.

R1-2009112	Support of 16-QAM for NB-IoT	Qualcomm Incorporated

R1-2009125	Design considerations to support 16-QAM for NB-IOT	Sierra Wireless, S.A.

### Support additional PDSCH scheduling delay for introduction of 14-HARQ processes in DL for eMTC

R1-2007619	Support of 14-HARQ processes in DL for HD-FDD MTC UEs	Huawei, HiSilicon

R1-2008074	Support of 14-HARQ processes in DL for eMTC	Nokia, Nokia Shanghai Bell

R1-2008698	Support additional PDSCH scheduling delay for introduction of 14-HARQ processes in DL for eMTC		ZTE

R1-2008931	Support of 14 HARQ processes in DL in LTE-MTC	Ericsson, Telefónica, Verizon, SoftBank, AT&T, Telstra

R1-2009113	Support of 14 HARQ processes and scheduling delay	Qualcomm Incorporated

R1-2009124	Design considerations to support 14-HARQ Feature for LTE-M	Sierra Wireless, S.A.

### Others

R1-2007620	Channel quality reporting in NB-IoT to support 16QAM	Huawei, HiSilicon

R1-2008699	DL TBS increase for eMTC	ZTE

R1-2008932	On the support of a maximum DL TBS of 1736 bits in LTE-MTC	Ericsson

## Enhancements to Integrated Access and Backhaul

Please refer to RP-201293 for detailed scope of the WI

### Enhancements to resource multiplexing between child and parent links of an IAB node

R1-2007594	Resource multiplexing between backhaul and access for IAB duplexing enhancements	Huawei, HiSilicon

R1-2007684	Enhancement to resource multiplexing between child and parent links	vivo

R1-2008029	Discussion on resource multiplexing in the simultaneous operation	CMCC

R1-2008184	Enhancements to Resource Multiplexing for NR IAB	Samsung

R1-2008312	Enhancements for IAB resource multiplexing	AT&T

R1-2008406	Discussions on IAB resource multiplexing enhancements	LG Electronics

R1-2008858	Enhancements to the IAB resource multiplexing	ZTE, Sanechips

R1-2008863	Enhancements for resource multiplexing among IAB backhaul and access links	Nokia, Nokia Shanghai Bell

R1-2008995	Enhancements to Resource Multiplexing between Child and Parent Links of an IAB Node			Intel Corporation

R1-2009018	Enhancements to resource multiplexing between child and parent links of an IAB node	ETRI

Withdrawn

R1-2009108	Enhancements to resource multiplexing for IAB	Lenovo, Motorola Mobility

R1-2009190	Resource multiplexing between child and parent links of an IAB node	NTT DOCOMO, INC.

R1-2009220	Discussion on IAB resource multiplexing enhancements	ETRI

R1-2009221	Discussions on enhancements to resource multiplexing between child and parent links of an IAB node		CEWiT

R1-2009269	Resource management for enhanced duplexing	Qualcomm Incorporated

R1-2009301	Resource multiplexing and DC in enhanced IAB	Ericsson

### Other enhancements for simultaneous operation of IAB-node’s child and parent links

Including IAB-node timing mode(s), extensions for DL/UL power control, and CLI and interference measurements of BH links, as needed

R1-2007595	Enhancements for simultaneous operation of MT and DU	Huawei, HiSilicon

R1-2007685	Other enhancements for simultaneous operation of child and parent links	vivo

R1-2007786	Other enhancements for simultaneous operation of IAB-node’s child and parent links	Fujitsu

R1-2008030	Discussion on the enhancement for simultaneous operation	CMCC

R1-2008185	Enhancements to Timing, Power Control and CLI for NR IAB	Samsung

R1-2008313	Enhancements for IAB interference management	AT&T

R1-2008407	Discussions on other enhancements for simultaneous operation	LG Electronics

R1-2008816	Discussion on simultaneous operation of IAB-node’s child and parent links	CEWiT

R1-2008859	Enhancements for simultaneous operation of child and parent links	ZTE, Sanechips

R1-2008864	Other enhancements for simultaneous operation of IAB Parent and Child	Nokia, Nokia Shanghai Bell

R1-2008996	Power Control Enhancements for Simultaneous Operations	Intel Corporation

R1-2009019	Discussion on simultaneous operation enhancements	ETRI

R1-2009109	Enhancements for simultaneous operation in IAB systems	Lenovo, Motorola Mobility

R1-2009137	Power Control: Subsequent Contribution	Sharp

R1-2009191	Other enhancements for simultaneous operation of IAB-node’s child and parent links	NTT DOCOMO, INC.

R1-2009270	On enhancements for simultaneous operation of IAB-node's child and parent links	Qualcomm Incorporated

R1-2009302	Timing, CLI and power control in enhanced IAB	Ericsson

### Others

R1-2007686	Other enhancements to Rel-17 IAB nodes	vivo

R1-2008314	Performance of simultaneous operation of IAB-node’s child and parent links	AT&T

R1-2008327	Other enhancements for IAB	Huawei, HiSilicon

R1-2008408	Discussion on RACH procedure for IAB enhancement	LG Electronics

R1-2008860	Discussion on multiplexing capability for IAB	ZTE, Sanechips

R1-2009020	Other enhancements to IAB	ETRI

R1-2009303	Refined beam calibration in enhanced IAB	Ericsson

## NR Sidelink enhancement

Please refer to RP-201516 for detailed scope of the WI

R1-2008186	On Sidelink Enhacement Work Item	Samsung

### Remaining issues for sidelink evaluation methodology update for power saving

[103-e-NR-Sidelink-Enh-01] Email discussion/approval for remaining issues for sidelink evaluation methodology update for power saving – Seungmin (LGE)

- 1st check point: 11/5

- 2nd check point: 11/10

- 3rd check point: 11/12

Agreements:

Confirm the following agreement with red changes:

- (Working assumption) Scaling of SL BWP size adaptation in RX perspective

  - X MHz is by (0.4 +0.6*(X-20)/80), where X is in MHz *100 MHz

- (Working assumption) For “PSCCH/PSSCH RX”,

  - In non-PSFCH-slot (i.e., the number of PSCCH/PSSCH symbols is 13),

    - the power consumption level is the same as that of “PDCCH+PDSCH”

- (Working assumption) For power consumption level of “PSFCH RX”,

  - the power consumption level is power consumption level of “PDCCH-only” for cross-slot scheduling

    - Its minimum value is 50

Agreements:

Remove the square brackets in the following agreements with red-colored clarification.

- Agreements made in RAN1#102-e meeting:

  - For power consumption level of “1st SCI/2nd SCI RX”,

    - the power consumption level is [0.7]* power consumption level of “PSCCH/PSSCH RX”

  - For power consumption level of “PSFCH TX”,

    - the power consumption level is [0.35]*power consumption level of “UL” for long PUCCH or PUSCH

  - For power consumption level of “S-SSB RX”,

    - the power consumption level is [1.5]*power consumption level of “Uu SSB-processing”

Agreements:

Support following three states for V2P/P2V links.

- LOS

  - A link is in LOS state if two UEs are in the same street and the LOS path is not blocked by vehicles

- NLOS (i.e., LOS path blocked by buildings)

  - A link is in NLOS state if the two UEs are in different streets.

- NLOSv (i.e., LOS path blocked by vehicles)

  - A link is in NLOSv state if the two UEs are in the same street and the LOS path is blocked by vehicles

Agreements:

For two UEs are in the same street in V2P/P2V links, reuse the probability of LOS and NLOSv states for Urban case specified in TR 37.885 (see below)

Agreements

For V2P/P2V links, reuse “additional vehicle blockage loss” specified in TR 37.885 (see below).

When a link is in NLOSv, additional vehicle blockage loss is added as follows: - The blocker height is the vehicle height which is randomly selected out of the three vehicle types according to the portion of the vehicle types in the simulated scenario. - The additional blockage loss is max {0 dB, a log-normal random variable}. - Case 1: Minimum antenna height value of TX and RX > Blocker height - No additional blockage loss - Case 2: Maximum antenna height value of TX and RX < Blocker height - Mean: 9 + max(0, 15*log10(d)-41) dB, standard deviation: 4.5 dB - Case 3: Otherwise - Mean: 5 dB + max(0, 15*log10(d)-41), standard deviation: 4 dB

Agreements:

For V2P/P2V links, reuse the fast fading parameters of V2V link specified in TR 37.885.

- Note: this does not imply that a Ped UE is required to use the same antenna configuration of a Veh UE

Agreements:

For the public safety and commercial use cases, reuse the parameters of “Reference system deployments” specified in Section A.2.1.1 of TR 36.843 with following modification:

- Carrier frequency:

  - Include 3.5 GHz for commercial use case (optional)

- System bandwidth:

  - Include 40 MHz for commercial use case (optional) and 20 MHz dedicated spectrum for out-of-coverage scenarios (optional)

- “eNB” is replaced by “gNB”

- FFS any refinement/variation is necessary, e.g., 19 vs. 7 sites, etc.

Agreements:

For the public safety and commercial use cases, reuse the parameters of “Channel models” specified in Section A.2.1.2 of TR 36.843 with following modification:

- Each component of channel model reuses what is specified in TR 38.901.

Update on 11/12/20

Agreements:

      - For the layout for public safety and commercial use cases, support “7 macro sites with 3 cells per site in the layout”

Agreements:

      - For public safety use case, at least following layout option is supported:

- Option 5 of TR 36.843: Urban macro (1732m ISD)

  - UE dropping as in Table A.2.1.1-1

    - All UEs are outdoors UEs

    - Mix of outdoor and indoor UEs

Agreements:

- For public safety and commercial use cases, at least following option is supported for UE RF parameters:

  - Reuse the number of TX AP, the number of RX AP, antenna gain for P-UE specified in TR 37.885.

Agreements:

- For public safety and commercial use cases, one OFDM symbol of NR SL slot is used for AGC

Agreements:

For public safety and commercial use cases, at least performance metrics for communication specified in A2.1.4.2 of TR 36.843 are reused with following modification:

- “FTP2 traffic model” is replaced with “FTP traffic model or periodic traffic model”

- Power consumption model agreed in R-17 NR sidelink enhancement WI is used

- the metrics for latency and WAN are not needed

Agreements:

- For public safety and commercial use cases, reuse in-band emission model used for NR V2X specified in section 6.4E.2.4 in TS 38.101

Agreements:

- For the channel model for P2P link,

- Option 2: LOS, NLOS, NLOSv are supported.

  - Option 2-2: Reuse definition of NLOS state, the probability of LOS/NLOSv, and additional vehicle blockage loss for V2V/V2P/P2V, and modify the definition of LOS/NLOSv states as follow

    - LOS

      - A link is in LOS state if two UEs are in the same sidewalk in the same street and the LOS path is not blocked by vehicles

      - A link is in LOS state if two UEs are in the different sidewalk in the same street and the LOS path is not blocked by vehicles

    - NLOS (i.e., LOS path blocked by buildings)

      - A link is in NLOS state if the two UEs are in different streets.

    - NLOSv (i.e., LOS path blocked by vehicles)

      - A link is in NLOSv state if the two UEs are in the different sidewalk in the same street and the LOS path is blocked by vehicles

- Note that the intention of channel model above is at least for modeling the interference generation in P2P link. The modeling P2P link is not applied to the scenario of V2P only, optionally applied or not to the scenario of P2V only, but applied to the scenario of combination of V2P and P2V.

Agreements:

- For the fast fading parameters for P2P link, reuse fast fading parameters of V2V/V2P/P2V links.

- Pedestrian UE speed is 3 km/h

- Location update is not modelled for pedestrian UE

- Note that the intention of channel model above is at least for modeling the interference generation in P2P link. The modeling P2P link is not applied to the scenario of V2P only, optionally applied or not to the scenario of P2V only, but applied to the scenario of combination of V2P and P2V.

Agreements:

- For P2V link, at least following traffic model is supported:

- Option 1: Traffic model for P-UE’s transmission specified in TS 36.885

  - The message size is fixed at 300 bytes and transmission frequency is 1 Hz

  - ‘100ms’ latency requirement

- Option 4: Aperiodic Model 1 specified in TR37.885 with following changes:

  - Inter-packet arrival time: 250 ms + an exponential random variable with the mean of 250 ms

  - Packet size: Uniformly random in the range between 200 bytes and 800 bytes with the quantization step of 200 bytes

  - Latency requirement: 250 ms or 100 ms

Agreements:

- For commercial use case, at least following option is supported for traffic model:

- Option 7: Periodic traffic model 3 specified in TR 37.885

Agreements:

- For the pedestrian UE dropping in V2X evaluation, reuse those specified in TR 36.885.

- Support that total number of pedestrian UEs is 1000 as optional

Agreements:

- For V2P link, V2V traffic model and the following options for traffic model are supported. Companies declare which traffic model is used for their V2P evaluation.

- Option 7: Periodic Model 2 specified in TR 37.885 with following change:

  - Inter-packet arrival time: 500ms

  - Latency requirement: 500 ms or 100 ms

- Option 8: Aperiodic Model 1 specified in TR 37.885 with following change:

  - Inter-packet arrival time: 250 ms + an exponential random variable with the mean of 250 ms

  - Packet size: Uniformly random in the range between 200 bytes and 800 bytes with the quantization step of 200 bytes

  - Latency requirement: 250 ms or 100 ms

R1-2007614	Sidelink evaluation methodology update for power saving	Huawei, HiSilicon

R1-2007621	Discussion of remaining issues for sidelink evaluation methodology update for power saving			Nokia, Nokia Shanghai Bell

R1-2007687	Discussion on sidelink evaluation methodology	vivo

R1-2007832	Discussion on sidelink evaluation methodology for power saving	CATT

R1-2007894	Discussion on remaining aspects of sidelink evaluation methodology update for power saving	LG Electronics

R1-2008187	On Sidelink Evaluation Methodology Updates for Power Saving	Samsung

R1-2008238	Discussion on evaluation methodology for SL power saving	OPPO

R1-2008445	Discussion on Sidelink Power Consumption Model	Apple

R1-2008877	Analysis on remaining issues for sidelink evaluation methodology	ZTE Corporation, Sanechips

R1-2008916	Discussion on sidelink evaluation methodology update for power saving	Lenovo, Motorola Mobility

R1-2008970	Remaining issues for sidelink evaluation methodology	MediaTek Inc.

R1-2008997	Remaining issues for sidelink evaluation methodology update for power saving	Intel Corporation

R1-2009036	Discussion on remaining issues of sidelink evaluation methodology update	Xiaomi

R1-2009071	Remaining evaluation assumptions and methodology for power saving	Ericsson

R1-2009120	V2X evaluation methodology and channel model updates	InterDigital, Inc.

R1-2009192	Remainig issues on sidelink evaluation methodology for power saving	NTT DOCOMO, INC.

R1-2009271	Evaluation Methodology for Power Saving in Sidelink	Qualcomm Incorporated

### Resource allocation enhancement

R1-2007878	Discussion on enhancement for NR V2X Mode 2	ITRI

R1-2008188	On Resource Allocation Enhacements	Samsung

#### Resource allocation for power saving

R1-2007615	Sidelink resource allocation to reduce power consumption	Huawei, HiSilicon

R1-2007688	Resource allocation for sidelink power saving	vivo

R1-2009193	Discussion on sidelink resource allocation for power saving	NTT DOCOMO, INC.

[103-e-NR-Sidelink-Enh-02] Email discussion/approval for resource allocation for power saving – Kevin (OPPO)

- 1st check point: 11/5

- 2nd check point: 11/10

- 3rd check point: 11/12

Update on 11/5  11/9

Conclusion

- SL reception Type A and Type D should be used as the reference for evaluation and designing of SL power saving features in R17.

  - Type A: UE is not capable of performing reception of any SL signals and channels, FFS with exception of performing PSFCH and S-SSB reception (aim to conclude in RAN1#104-e)

  - Type D: UE is capable of performing reception of all SL signals and channels defined in R16. It does not preclude UE to perform reception of a subset of SL signals/channels

  - If there are evaluations with assumptions other than the above reference, the detailed assumptions need to be reported

  - Note: the types and the associated capability defined here are not intended to be defined as Rel-17 UE features as is.

Agreements:

- Partial sensing based RA is supported as a power saving RA scheme

  - FFS details

- Random resource selection is supported as a power saving RA scheme

  - FFS any changes or enhancement

  - FFS on conditions to apply random resource selection

Agreements:

- In R17, a SL Mode 2 Tx resource pool can be (pre-)configured to enable full sensing only, partial sensing only, random resource selection only, or any combination(s) thereof

  - FFS details, including usage, potential restrictions, whether/how any enhancement or condition is needed for the coexistence of full sensing and power saving RA scheme(s) in a same resource pool, etc.

Agreements:

- Re-evaluation and pre-emption checking are not supported by UEs that do not perform any sensing (i.e. PSCCH reception)

- Re-evaluation and pre-emption checking are supported by UEs that perform sensing

  - FFS details and any conditions(s) in which re-evaluation and pre-emption can be performed

- FFS whether/how re-evaluation and pre-emption can be supported by UEs performing random resource selection that do perform sensing

- Note: details about sensing in this context, including when it is performed, are not decided yet.

Agreements:

- Further study congestion control based on CBR and CR for power saving RA schemes

  - Identify necessary changes from R16 CBR/CR (if any), including transmission resource selection and transmission parameters that can be adjusted and applicable to power savings RA schemes

  - Note: this is not intended to require all UEs to perform sensing for the purpose of CBR measurement

R1-2007553	Power consumption reduction for sidelink resource allocation	FUTUREWEI

R1-2007622	Discussion of resource allocation for power saving	Nokia, Nokia Shanghai Bell

R1-2007787	Considerations on partial sensing in NR V2X	Fujitsu

R1-2007833	Discussion on resource allocation for power saving	CATT

R1-2007879	Discussion on sidelink resource allocation for power saving	ITRI

R1-2007892	Resource allocation for power saving	TCL Communication Ltd.

R1-2007895	Discussion on resource allocation for power saving	LG Electronics

R1-2008031	Discussion on resource allocation for power saving	CMCC

R1-2008098	Discussion on sidelink resource allocation for power saving	Spreadtrum Communications

R1-2008189	On Resource Allocation for Power Saving	Samsung

R1-2008239	Power saving mechanism in NR sidelink	OPPO

R1-2008373	Discussion on sidelink resource allocation for power saving	Sony

R1-2008446	Discussion on Resource Allocation for Power Saving	Apple

R1-2008817	NR Sidelink Resource Allocation for UE Power Saving	Fraunhofer IIS, Fraunhofer HHI

R1-2008836	Discussion on Sidelink Resource Allocation for Power Saving	Panasonic Corporation

R1-2008878	Discussion on resource allocation for power saving	ZTE Corporation, Sanechips

R1-2008917	Sidelink resource allocation for Power saving	Lenovo, Motorola Mobility

R1-2008950	Discussion on resource allocation for power saving	NEC

R1-2008971	Resource allocation for sidelink power saving	MediaTek Inc.

R1-2008998	Potential sidelink enhancements for UE power saving	Intel Corporation

R1-2009021	Discussion on resource allocation for power saving	ETRI

R1-2009037	Discussion on sidelink resource allocation for power saving	Xiaomi

R1-2009072	Resource allocation mechanisms for power saving	Ericsson

R1-2009079	Considerations on partial sensing in NR V2X	CAICT

R1-2009121	Sidelink resource allocation for power saving	InterDigital, Inc.

R1-2009128	Sidelink resource allocation to reduce power consumption	ROBERT BOSCH GmbH

R1-2009138	Discussion on resource allocation for power saving	Sharp

R1-2009161	On Resource Allocation Power Saving Enhancement for NR Sidelink	Convida Wireless

R1-2009222	Resource allocation for power saving with partial sensing in NR sidelink enhancement	ITL, KRRI

R1-2009272	Power Savings for Sidelink	Qualcomm Incorporated

R1-2009287	Views on power saving for NR sidelink enhancement	KT Corp.

#### Feasibility and benefits for mode 2 enhancements

R1-2008879	Inter-UE coordination in mode-2	ZTE Corporation, Sanechips

R1-2008999	Analysis of potential sidelink enhancements targeting Mode 2 reliability and latency	Intel Corporation

R1-2009273	Reliability and Latency Enhancements for Mode 2	Qualcomm Incorporated

[103-e-NR-Sidelink-Enh-03] Email discussion/approval for feasibility and benefits for mode 2 enhancements– Seungmin (LGE)

- 1st check point: 11/5

- 2nd check point: 11/10

- 3rd check point: 11/12

Conclusion:

  - The schemes of inter-UE coordination in Mode 2 are categorized as being based on the following types of “A set of resources” sent by UE-A to UE-B:

    - UE-A sends to UE-B the set of resources preferred for UE-B’s transmission

      - e.g., based on its sensing result

    - UE-A sends to UE-B the set of resources not preferred for UE-B’s transmission

      - e.g., based on its sensing result and/or expected/potential resource conflict

    - UE-A sends to UE-B the set of resource where the resource conflict is detected

    - FFS: details of resource conflict, e.g., including type of resource conflict

    - FFS: details of sensing operation at UE-A side

    - FFS: which type(s) of resource set information is(are) beneficial/feasible to which cast type(s)

    - Note: these different types may be used in combination with each other

  - From RAN1 perspective, further study on the feasibility/benefit of inter-UE coordination is required

  - Send an LS to RAN plenary

Conclusion:

  - For the schemes of inter-UE coordination identified as feasible/beneficial, at least the following aspects are further discussed.

    - How/when UE-A determines the contents of ”A set of resources”, including consideration of UL scheduling

    - When UE-A sends ”A set of resources” to UE-B, including which UE(s) sends it

    - How UE-A and UE-B are determined

    - How UE-A sends ”A set of resources” to UE-B, including container used for carrying it, implicitly or explicitly or both

    - How/when/whether UE-B receives “A set of resources” and takes it into account in the resource selection for its own transmission

    - How/whether to define the relationship between support/signaling of inter-UE coordination and cast type

Is there an LS?

R1-2007554	Views on resource allocation enhancements for sidelink communication	FUTUREWEI

R1-2007616	Inter-UE coordination in sidelink resource allocation	Huawei, HiSilicon

R1-2007623	Discussion of feasibility and benefits for mode 2 enhancements	Nokia, Nokia Shanghai Bell

R1-2007689	Discussion on mode-2 enhancements	vivo

R1-2007771	Inter-UE Coordination Mode 2	Kyocera Corporation

R1-2007788	Considerations on inter-UE coordination for mode 2 enhancements	Fujitsu

R1-2007834	Discussion on feasibility and benefits for mode 2 enhancements	CATT

R1-2007880	Enhancement of Mode 2 Latency Performance	ITRI

R1-2007893	Feasibility and benefits for mode 2 enhancements	TCL Communication Ltd.

R1-2007896	Discussion on feasibility and benefits for mode 2 enhancements	LG Electronics

R1-2008032	Discussion on reliability and latency enhancements for mode-2 resource  allocation	CMCC

R1-2008099	Discussion on feasibility and benefit of mode 2 enhancements	Spreadtrum Communications

R1-2008190	On Feasibility and Benefits for Mode2 Enhancements	Samsung

R1-2009319	Inter-UE coordination in mode 2 of NR sidelink	OPPO

Revision of R1-2008240

R1-2008374	Discussion on reliability and latency enhancements for mode 2	Sony

R1-2008447	Discussion on Inter-UE Coordination for Mode 2 Resource Allocation	Apple

R1-2008499	Discussion on V2X mode 2 enhancements	ASUSTeK

R1-2008757	Resource Allocation Enhancements for Mode 2	Fraunhofer HHI, Fraunhofer IIS

R1-2008820	Views on inter-UE coordination for mode 2 enhancements	Zhejiang Lab

R1-2008861	Inter-UE coordination for enhanced resource allocation	Mitsubishi Electric RCE

R1-2008892	Inter-UE coordination for mode 2 enhancement	ITL

R1-2008918	Sidelink resource allocation for Reliability enhancement	Lenovo, Motorola Mobility

R1-2008951	Discussion on feasibility and benefits for mode 2 enhancements	NEC

R1-2008975	Discussion on Mode 2 enhancements	MediaTek Inc.

R1-2009022	Discussion on feasibility and benefits for mode 2 enhancements	ETRI

R1-2009038	Considerations on Mode 2 enhancement for enhanced reliability and reduced latency	Xiaomi

R1-2009073	Feasibility and benefits of mode 2 enhancements for inter-UE coordination	Ericsson

R1-2009122	NR SL Mode 2 enhancement for reliability improvement	InterDigital, Inc.

R1-2009126	Mode 2 enhancements in sidelink	Panasonic Corporation

R1-2009127	Discussion on sidelink mode-2 resource allocation enhancements	ROBERT BOSCH GmbH

R1-2009139	Enhancements of resource allocation Mode 2 for NR sidelink	Sharp

R1-2009162	On Resource Allocation Mode 2 Enhancement for NR Sidelink	Convida Wireless

R1-2009194	Resource allocation for reliability and latency enhancements	NTT DOCOMO, INC.

R1-2009291	Discussion on feasibility and benefits for NR Sidelink mode 2 enhancements	CEWiT

R1-2009297	Views on feasibility and benefits for mode 2 enhancements	KT Corp.

### Others

R1-2007690	Discussion on sidelink DRX	vivo

R1-2007881	Discussion on NR sidelink enhancements	ITRI

R1-2007897	Discussion on physical layer design considering sidelink DRX operation	LG Electronics

R1-2008191	On Sidelink Issues and RAN1 Impacts	Samsung

R1-2008241	The effect of DRX on resource selection	OPPO

R1-2008332	Physical layer impacts of sidelink DRX	Huawei, HiSilicon

R1-2008448	Discussion on Sidelink DRX	Apple

R1-2008500	Discussion on SL DRX impact on physical layer	ASUSTeK

R1-2008758	Other Sidelink Aspects Impacting RAN1	Fraunhofer HHI, Fraunhofer IIS

R1-2008880	Potential impact of DRX enhancement to RAN1 discussion	ZTE Corporation, Sanechips

R1-2008919	Discussion on potential sidelink DRX impacts in RAN1	Lenovo, Motorola Mobility

R1-2009039	Discussion on power saving and congestion control	Xiaomi

R1-2009074	On the relationship between partial sensing and DRX	Ericsson

R1-2009123	On multi-carrier and DRX operation for SL	InterDigital, Inc.

## NR Multicast and Broadcast Services

Please refer to RP-201038 for detailed scope of the WI

R1-2008033	Updated NR MBS work plan	CMCC

### Mechanisms to support group scheduling for RRC_CONNECTED UEs

R1-2008034	Discussion on group scheduling mechanisms	CMCC

R1-2008940	Summary#1 on mechanisms to support group scheduling for RRC_CONNECTED UEs for NR MBS			Moderator (CMCC)

[103-e-NR-MBS-01] Email discussion/approval for mechanisms to support group scheduling for RRC_CONNECTED UEs– Fei (CMCC)

- 1st check point: 11/5

- 2nd check point: 11/10

- 3rd check point: 11/12

Update on 11/5  11/10

R1-2009504

Agreements: For convenience of discussion, consider the following clarification as RAN1 common understanding.

- PTP transmission: For RRC_CONNECTED UEs, use UE-specific PDCCH with CRC scrambled by UE-specific RNTI (e.g., C-RNTI) to schedule UE-specific PDSCH which is scrambled with the same UE-specific RNTI.

- PTM transmission scheme 1: For RRC_CONNECTED UEs in the same MBS group, use group-common PDCCH with CRC scrambled by group-common RNTI to schedule group-common PDSCH which is scrambled with the same group-common RNTI. This scheme can also be called group-common PDCCH based group scheduling scheme.

- PTM transmission scheme 2: For RRC_CONNECTED UEs in the same MBS group, use UE-specific PDCCH with CRC scrambled by UE-specific RNTI (e.g., C-RNTI) to schedule group-common PDSCH which is scrambled with group-common RNTI. This scheme can also be called UE-specific PDCCH based group scheduling scheme.

- Note: The ‘UE-specific PDCCH / PDSCH’ here means the PDCCH / PDSCH can only be identified by the target UE but cannot be identified by the other UEs in the same MBS group with the target UE.

- Note: The ‘group-common PDCCH / PDSCH’ here means the PDCCH / PDSCH are transmitted in the same time/frequency resources and can be identified by all the UEs in the same MBS group.

- FFS whether or not to have additional definition of transmission scheme(s)

Agreements: For RRC_CONNECTED UEs, if initial transmission for multicast is based on PTM transmission scheme 1, at least support retransmission(s) can use PTM transmission scheme 1.

- FFS: whether to support PTP transmission for retransmission(s).

- FFS: whether to support PTM transmission scheme 2 for retransmission(s).

- FFS: How to indicate the association between PTM scheme 1 and PTP transmitting the same TB.

- FFS: If multiple retransmission schemes are supported, then can different retransmission schemes be supported simultaneously for different UEs in the same group?

R1-2009629	Summary#1 on mechanisms to support group scheduling for RRC_CONNECTED UEs for NR MBS Moderator (CMCC)

Working assumption:

For multicast of RRC-CONNECTED UEs, a common frequency resource for group-common PDCCH / PDSCH is confined within the frequency resource of a dedicated unicast BWP to support simultaneous reception of unicast and multicast in the same slot

- Down select from the two options for the common frequency resource for group-common PDCCH/ PDSCH

  - Option 2A: The common frequency resource is defined as an MBS specific BWP, which is associated with the dedicated unicast BWP and using the same numerology (SCS and CP)

    - FFS BWP switching is needed between the multicast reception in the MBS specific BWP and unicast reception in its associated dedicated BWP

  - Option 2B: The common frequency resource is defined as an ‘MBS frequency region’ with a number of contiguous PRBs, which is configured within the dedicated unicast BWP.

    - FFS: How to indicate the starting PRB and the length of PRBs of the MBS frequency region

- FFS whether UE can be configured with no unicast reception in the common frequency resource

- FFS on details of the group-common PDCCH / PDSCH configuration

- FFS whether to support more than one common frequency resources per UE / per dedicated unicast BWP subjected to UE capabilities

Agreements: Support TDM between one unicast PDSCH and one group-common PDSCH in a slot based on UE capability for RRC_CONNECTED UEs.

Agreements: Support SPS group-common PDSCH for MBS for RRC_CONNECTED UEs

- FFS: use group-common PDCCH or UE-specific PDCCH for SPS group-common PDSCH activation/deactivation

- FFS: whether to support more than one SPS group-common PDSCH configuration per UE

- FFS: whether and how uplink feedback could be configured

- FFS: retransmission of SPS group-common PDSCH

Agreements: For PTM transmission scheme 1, the CORESET for group-common PDCCH is configured within the common frequency resource for group-common PDSCH.

- FFS: number of CORESET(s) for group-common PDCCH within the common frequency resource for group-common PDSCH

Agreements: For search space set of group-common PDCCH of PTM scheme 1 for multicast in RRC_CONNECTED state, the CCE indexes are common for different UEs in the same MBS group.

Agreements: Down select from the two options for BDs/CCEs limit for Rel-17 MBS

- Option 1: the maximum number of monitored PDCCH candidates and non-overlapped CCEs per slot per serving cell defined in Rel-15 is kept unchanged for Rel-17 MBS.

- Option 2: For UEs supporting CA capability, the budget of BDs/CCEs of an unused CC can be used for group-common PDCCH to count the number of BDs/CCEs, which is similar to the method used for multi-DCI based multi-TRP in Rel-16.

Agreements:For RRC_CONNECTED UEs, support inter-slot TDM between unicast PDSCH and group-common PDSCH in different slots (mandatory for the UE supporting MBS).

Agreements:Further study the following cases for simultaneous reception of unicast PDSCH and group-common PDSCH in a slot based on UE capability for RRC_CONNECTED UEs.

- Case 1: support TDM between multiple TDMed unicast PDSCHs and one group-common PDSCH in a slot

- Case 2: support TDM among multiple group-common PDSCHs in a slot

- Case 3: support TDM between multiple TDMed unicast PDSCHs and multiple TDMed group-common PDSCHs in a slot

- Case 4: support FDM between multiple TDMed unicast PDSCHs and multiple TDMed group-common PDSCHs in a slot

- Case 5: support FDM among multiple group-common PDSCHs in a slot

- FFS: maximum number of PDSCHs in a slot simultaneous received per UE

Agreements:For search space set of group-common PDCCH of PTM scheme 1 for multicast in RRC_CONNECTED state, further study the following options.

- Option 1: Define a new search space type specific for multicast

- Option 2: Reuse the existing CSS type(s) in Rel-15/16

  - FFS: whether modifications are needed for multicast

- Option 3: Reuse the existing USS in Rel-15/16 with necessary modifications for MBS

  - FFS: detailed modifications

Agreements:No specification enhancement in Rel-17 to support SDM between unicast PDSCH and group-common PDSCH in a slot for RRC_CONNECTED UEs.

Agreements: For PTM transmission scheme 1, if Option 2A or Option 2B for common frequency resource for group-common PDCCH/PDSCH is agreed, the FDRA field of group-common PDCCH is interpreted based on the common frequency resource.

Agreements: For search space set of group-common PDCCH of PTM scheme 1 for multicast in RRC_CONNECTED state, further study the following options for the monitoring priority of search space set

- Option 1: The monitoring priority of search space set for multicast is the same as existing Rel-15/16 CSS

- Option 2: The monitoring priority of search space set for multicast is the same as existing Rel-15/16 USS

- Other options are not precluded

- The monitoring priority is used at least for PDCCH overbooking case

  - FFS for other cases (e.g., to prune PDCCH in terms of whether it’s unicast or multicast, etc.)

R1-2007556	Group scheduling for MC/BC services	FUTUREWEI

R1-2007562	Resource configuration and group scheduling for RRC_CONNECTED UEs	Huawei, HiSilicon

R1-2007637	Group scheduling for RRC_CONNECTED UEs	CHENGDU TD TECH LTD.

R1-2007691	Discussion on mechanisms to support group scheduling for RRC_CONNECTED UEs	vivo

R1-2007835	Discussion on group scheduling mechanism for RRC_CONNECTED UEs in MBS	CATT

R1-2008064	Support of group scheduling for RRC_CONNECTED UEs	LG Electronics

R1-2008192	On mechanisms to support group scheduling for RRC_CONNECTED UEs	Samsung

R1-2008242	Group scheduling for NR Multicast and Broadcast Services	OPPO

R1-2008375	Considerations on MBMS group scheduling for RRC_CONNECTED UEs	Sony

R1-2008449	Discussion on group scheduling mechanism for RRC_connected UEs	Apple

R1-2008826	Mechanisms to Support Group Scheduling for RRC_CONNECTED UEs	ZTE

R1-2008833	Discussion on mechanisms to support group scheduling for RRC_CONNECTED UEs	ETRI

R1-2008882	Group Scheduling Mechanisms to Support 5G Multicast / Broadcast Services for RRC_CONNECTED UEs	Nokia, Nokia Shanghai Bell

R1-2008926	Discussion on group scheduling mechanism for NR MBS	Lenovo, Motorola Mobility

R1-2008961	Discussion on NR MBS group scheduling for RRC_CONNECTED UEs	MediaTek Inc.

R1-2009000	Group Scheduling for NR-MBS	Intel Corporation

R1-2009055	Discussion on mechanisms to support group scheduling for RRC_CONNECTED UEs	Asia Pacific Telecom co. Ltd

R1-2009163	Mechanisms to support group scheduling for RRC_CONNECTED UEs	Convida Wireless

Withdrawn

R1-2009165	On group scheduling mechanism for NR multicast and broadcast	Convida Wireless

R1-2009238	On Optimal Multiplexing for Simultaneous Operation of Broadcast/Multicast and Unicast Services		BBC

R1-2009274	Views on group scheduling for Multicast RRC_CONNECTED UEs	Qualcomm Incorporated

R1-2009305	Mechanisms to support group scheduling for RRC_CONNECTED Ues	Ericsson

### Mechanisms to improve reliability for RRC_CONNECTED UEs

R1-2009275	Views on reliability enhancement for Multicast RRC_CONNECTED UEs	Qualcomm Incorporated

[103-e-NR-MBS-02] Email discussion/approval for mechanisms to improve reliability for RRC_CONNECTED UEs– Jinhuan (Huawei)

- 1st check point: 11/5

- 2nd check point: 11/10

- 3rd check point: 11/12

Update on 11/5 – 11/10

R1-2009654

Agreements:

For RRC_CONNECTED UEs receiving multicast, at least for PTM scheme 1, support at least one of the following:

- ACK/NACK based HARQ-ACK feedback for multicast,

  - From per UE perspective, UE feedback ACK or NACK.

  - From UEs within the group perspective,

    - FFS: PUCCH resource configuration for ACK/NACK feedback e.g., shared or separate PUCCH resources.

  - FFS details including conditions for it to be used

- NACK-only based HARQ-ACK feedback for multicast,

  - From per UE perspective, UE only feedback NACK.

  - From UEs within the group perspective, further down-select between:

    - FFS: PUCCH resource configuration for NACK only feedback.

  - FFS details including conditions for it to be used

- To decide in RAN1#104-e whether or not to support only one or both of the above schemes

  - If both are supported, FFS configuration/selection of ACK/NACK-based and NACK-only based HARQ-ACK feedback

Agreements:

For RRC_CONNECTED UEs receiving multicast, for ACK/NACK based HARQ-ACK feedback if supported for group-common PDCCH scheduling, PUCCH resource configuration for HARQ-ACK feedback from per UE perspective is, down-select one of the following options:

- Option 1: shared with PUCCH resource configuration for HARQ-ACK feedback for unicast

- Option 2: separate from PUCCH resource configuration for HARQ-ACK feedback for unicast

- Option 3: Option 1 or option 2 based on configuration

Agreements:

For RRC_CONNECTED UEs receiving multicast, for NACK-only based HARQ-ACK feedback if supported for group-common PDCCH scheduling, PUCCH resource configuration for HARQ-ACK feedback from per UE perspective is separate from PUCCH resource configuration for HARQ-ACK feedback for unicast.

- FFS PUCCH format

Agreements:

Enabling/disabling HARQ-ACK feedback for MBS is supported, further down-select between:

- Option 1: DCI

- Option 2: RRC configures enabling/disabling

- Option 3: RRC configures the enabling/ disabling function and DCI indicates enabling /disabling

- FFS: Option 4: MAC-CE indicates enabling/disabling

- FFS: Option 5: RRC configures the enabling/ disabling function and MAC-CE indicates enabling /disabling

Agreements:

For slot-level repetition for group-common PDSCH of RRC_CONNECTED UEs, for indicating the repetition number, further down-select among:

- Opt 1: by DCI

- Opt 2: by RRC

- Opt 3: by RRC+DCI

- FFS: Opt 4: by MAC-CE

- FFS: Opt 5: by RRC+MAC-CE

- FFS details for each option.

- FFS further enhancements for configuration of slot-level repetition

Agreements:

From the perspective of RRC_CONNECTED UEs receiving multicast, at least for PTM scheme 1 initial transmission, retransmission supports, for the purpose of down-selection, options are:

- Option 1: group-common PDCCH scheduled group-common PDSCH

- Option 2: UE-specific PDCCH scheduled PDSCH

  - Alt 1: PDSCH is UE-specific PDSCH

  - Alt 2: PDSCH is group-common PDSCH

- Option 3: both option 1 and option 2

- FFS other options

- FFS CBG based retransmission

Agreements:

FFS whether CSI feedback enhancement is needed for MBS, including but not limited:

- New CQI measurement

- New CSI report formats

- Targeted BLER

- CSI-RS configuration

- A-CSI-RS transmission triggering

- SRS configuration

Agreements:

For ACK/NACK based HARQ-ACK feedback if supported, both Type-1 and Type-2 HARQ-ACK codebook are supported for RRC_CONNECTED UEs receiving multicast,

·        FFS details of HARQ-ACK codebook design.

·         FFS whether enhanced Type-2 and/or Type-3 HARQ-ACK codebook is supported or not.

R1-2007557	Improving reliability for MC/BC services	FUTUREWEI

R1-2007563	Mechanisms to improve reliablity for RRC_CONNECTED UEs	Huawei, HiSilicon

R1-2007638	Study on the reliability for RRC_CONNNECTED UEs	CHENGDU TD TECH LTD.

R1-2007692	Discussion on mechanisms to improve reliability for RRC_CONNECTED UEs	vivo

R1-2007836	Discussion on reliability improvement mechanism for RRC_CONNECTED UEs in MBS	CATT

R1-2008035	Discussion on reliability improvement	CMCC

R1-2008065	Mechanisms to improve reliability of Broadcast/Multicast service	LG Electronics

R1-2008193	On mechanisms to improve reliability for RRC_CONNECTED UEs	Samsung

R1-2008243	UL feedback for RRC-CONNECTED UEs in MBMS	OPPO

R1-2008376	Considerations on MBMS reliability for RRC_CONNECTED UEs	Sony

Withdrawn

R1-2008450	Discussion on MBS reliability improvement for RRC_connected UEs	Apple

R1-2008715	Reliability improvement for RRC_CONNECTED UEs in MBS	Potevio

R1-2008827	Mechanisms to Improve Reliability for RRC_CONNECTED UEs	ZTE

R1-2008883	Reliability Improvements for RRC_CONNECTED UEs	Nokia, Nokia Shanghai Bell

R1-2008893	Views on improving reliability for RRC_CONNECTED UEs in MBS	Google Inc.

R1-2008927	Discussion on reliability improvement for RRC-CONNECTED UEs	Lenovo, Motorola Mobility

R1-2008962	Discussion on HARQ operation for NR MBS reliable transmission	MediaTek Inc.

R1-2009001	Mechanisms to Improve Reliability for NR-MBS	Intel Corporation

R1-2009164	Mechanisms to improve reliability for RRC_CONNECTED UEs	Convida Wireless

Withdrawn

R1-2009166	On reliability enhancement for NR multicast and broadcast	Convida Wireless

R1-2009306	Discussion on reliability mechanisms for NR MBS	Ericsson

### Basic functions for broadcast/multicast for RRC_IDLE/RRC_INACTIVE UEs

R1-2007837	Discussion on basic functions for broadcast/multicast for RRC_IDLE/RRC_INACTIVE UEs			CATT, CBN

[103-e-NR-MBS-03] Email discussion/approval for basic functions for broadcast/multicast for RRC_IDLE/RRC_INACTIVE UEs– David (BBC)

- 1st check point: 11/5

- 2nd check point: 11/10

- 3rd check point: 11/12

Agreements: For RRC_IDLE/RRC_INACTIVE UEs, support group-common PDCCH with CRC scrambled by a common RNTI to schedule a group-common PDSCH, where the scrambling of the group-common PDSCH is based on the same common RNTI.

- FFS details

Update on 11/11

Agreements:

- For RRC_IDLE/RRC_INACTIVE Ues, beam sweeping is supported for group-common PDCCH/PDSCH.

  - FFS: Details for support of beam sweeping for group-common PDCCH/PDSCH.

Agreements: For RRC_IDLE/RRC_INACTIVE UEs, define/configure common frequency resource(s) for group-common PDCCH/PDSCH.

- the UE may assume the initial BWP as the default common frequency resource for group-common PDCCH/PDSCH, if a specific common frequency resource is not configured.

- FFS: the relation of the common frequency resource(s) (if configured) and initial BWP.

- FFS: whether to configure one/more common frequency resources

- FFS: configuration and definition details of the common frequency resource

Agreements: From physical layer perspective, for broadcast reception, the same group-common PDCCH and the corresponding scheduled group-common PDSCH can be received by both RRC_IDLE/RRC_INACTIVE UEs and RRC_CONNECTED UEs.

- FFS details.

Agreements: For RRC_IDLE/RRC_INACTIVE UEs, CSS is supported for group-common PDCCH.

- FFS: reuse current CSS type, define a new CSS type, etc.

- FFS other details.

Agreements: For RRC_IDLE/RRC_INACTIVE UEs, a CORESET can be configured within the common frequency resource for group-common PDCCH/PDSCH. CORESET0 is used by default if the common frequency resource for group-common PDCCH/PDSCH is the initial BWP and the CORESET is not configured.

- FFS: configuration details of the CORESET for group-common PDCCH/PDSCH

Email summary in R1-2009554.

R1-2007564	Discussion on multicast support for IDLE/INACTIVE UEs	Huawei, HiSilicon

R1-2007639	Basic functions for MBS for RRC_IDLE/RRC_INACTIVE UEs	CHENGDU TD TECH LTD.

R1-2007693	Discussion on basic functions for broadcast/multicast for RRC_IDLE/RRC_INACTIVE Ues			vivo

R1-2008036	Discussion on NR MBS in RRC_IDLE/RRC_INACTIVE states	CMCC

R1-2008066	Basic function for broadcast/multicast	LG Electronics

R1-2008194	On basic functions for broadcast/multicast for RRC_IDLE/RRC_INACTIVE UEs	Samsung

R1-2008244	Discussion on enhancements for IDLE and INACTIVE state UEs	OPPO

R1-2008828	Basic Functions for Broadcast or Multicast for RRC_IDLE or RRC_INACTIVE UEs	ZTE

R1-2008884	Basic Functions for Broadcast / Multicast for  RRC_IDLE / RRC_INACTIVE UEs	Nokia, Nokia Shanghai Bell

R1-2008928	Basic functions for broadcast/multicast in idle/inactive states	Lenovo, Motorola Mobility

R1-2009002	NR-MBS for RRC_IDLE/INACTIVE UEs	Intel Corporation

R1-2009167	On NR multicast and broadcast for RRC_IDLE/RRC_INACTIVE UEs	Convida Wireless

R1-2009276	Discussion on broadcast/multicast for RRC_IDLE/RRC_INACTIVE UEs	Qualcomm Incorporated

R1-2009307	Support for NR multicast reception in RRC Inactive/Idle	Ericsson

### Others

R1-2007641	Effects of NR MBS on PDSCH and PDCCH	CHENGDU TD TECH LTD.

R1-2007694	Other issues for Rel-17 MBS	vivo

R1-2007838	Discussion on search space type definition for group scheduling	CATT

R1-2008067	Other aspects for MBS	LG Electronics

R1-2008245	PUCCH resource allocation for UL feedback in MBMS	OPPO

R1-2008317	Resource for receiving MBS	Huawei, HiSilicon

R1-2008829	Preliminary Simulation Results of Rel-17 MBS	ZTE

R1-2009308	Assumptions for Performance Evaluations of NR-MBS	Ericsson

## NR Dynamic spectrum sharing (DSS)

Please refer to RP-193260 for detailed scope of the WI

Also include RAN1 impact from RP-201040 (LTE_NR_DC_enh2)

R1-2009205	Work plan for Rel17 WI on NR Dynamic spectrum sharing (DSS)	Ericsson

### Cross-carrier scheduling (from Scell to Pcell)

[103-e-NR-DSS-01] Email discussion/approval for CCS – Ravi (Ericsson)

- 1st check point: 11/5

- 2nd check point: 11/10

- 3rd check point: 11/12

Update on 11/5  11/9

R1-2009046	Cross-carrier scheduling from SCell to Pcell	Nokia, Nokia Shanghai Bell

R1-2009486

Agreements:

- When CCS from an SCell (sSCell) to PCell/PSCell is configured, UE monitors Type 0/0A/1/2 CSS sets (for the DCI formats associated with those SS sets) only on the PCell/PSCell and not on the sSCell

  - Note: UE monitors Type 0/0A/2 CSS only on PCell while Type 1 CSS can be monitored on PCell/PSCell

Conclusion

- When CCS from sSCell to PCell/PSCell is configured, the configuration of Type 3 CSS set for DCI formats 2_0, 2_1, 2_2, 2_3, 2_4 and applicability of the information in the DCI formats are the same as in Rel-15/Rel-16

  - FFS: DCI format 2_5 and DCI Format 2_6 handling

- Note: The SCell configured with CCS to Pcell/PSCell is referred to as ‘sSCell’

Conclusion

- When the PCell/PSCell and sSCell use different numerologies, the PDSCH reception preparation time between the PDCCH on the sSCell and the PDSCH on the PCell/PSCell is applied (i.e., as specified in TS38.214 Section 5.5).

Agreements:

- Discuss in RAN1#104-e how to handle ‘DCI formats 0_1,1_1,0_2,1_2 scheduling PDSCH/PUSCH on PCell/PSCell’ from USS set(s), when CCS from sSCell to PCell/PSCell is configured.. Below alternatives can be considered in the discussion (other alternatives are not precluded)

- Below alternatives can be considered in the discussion (other alternatives are not precluded)

  - Alt 1: When CCS from sSCell to PCell/PSCell is configured, UE cannot be configured to monitor DCI formats 0_1,1_1,0_2,1_2 on PCell/PSCell USS set(s), and can be configured to monitor them only on the sSCell USS set(s)

  - Alt 2: When CCS from sSCell to PCell/PSCell is configured, UE can be configured to monitor DCI formats 0_1/1_1/0_2/1_2 on PCell/PSCell USS set(s), and/or on sSCell USS set(s). The PDCCH monitoring is based on following alternatives (other alternatives are not precluded)

    - Alt 2-1:

      - UE can monitor DCI formats 0_1,1_1,0_2,1_2 on both PCell USS set(s) and sSCell USS sets simultaneously

        - FFS activation/deactivation of scheduling from sSCell to PCell/PSCell

    - Alt 2-2:

      - Dynamic switching of PDCCH monitoring of DCI formats 0_1,1_1,0_2,1_2 between monitoring on PCell/PSCell USS sets and monitoring on sSCell USS sets is supported

        - FFS: Details of switching mechanism (e.g. based on SS group switching, based on BWP switching,…)

      - UE does not monitor DCI formats 0_1,1_1,0_2,1_2 on both PCell USS set(s) and sSCell USS sets simultaneously

    - Alt 2-3:

      - UE does not monitor the same DCI format on both PCell USS set(s) and sSCell USS sets simultaneously. UE can monitor some DCI formats on sSCell USS sets and other DCI formats on PCell/PSCell USS sets simultaneously

    - Alt 2-4:

      - The USS set(s) on PSCell/PCell and the USS set(s) on sSCell are configured such that UE does not monitor DCI formats 0_1,1_1,0_2,1_2 on both PCell USS set(s) and sSCell USS set(s) simultaneously

- FFS following aspects

  - Impact of sSCell activation/deactivation and sSCell dormancy

  - Impact on BD/CCE limit handling including considering PDCCH monitoring on CSS sets and PDCCH monitoring of ‘DCI formats 0_0, 1_0 scheduling PUSCH/PDSCH on PCell/PSCell’

  - Whether PDCCH overbooking on sSCell is supported or not supported and impact (if any) on overbooking handling on PCell/PSCell

  - Impact from different numerologies between PDCCH on the PCell/PSCell and that on the sSCell

  - Whether or not to have mechanism for activation/deactivation of scheduling from sSCell to PCell/PSCell

  - USS configuration details (e.g. handling of USS type (self-scheduling, cross carrier scheduling) for a configured USS set configured for scheduling of in PCell/PSCell)

R1-2007579	Discussion on SCell PDCCH scheduling P(S)Cell PDSCH or PUSCH	Huawei, HiSilicon

R1-2007695	Discussion on Scell scheduling Pcell	vivo

R1-2007839	Disucssion on cross-carrier scheduling from Scell to Pcell	CATT

R1-2008062	Discussion on cross-carrier scheduling from SCell to Pcell	LG Electronics

R1-2008110	Discussion on cross-carrier scheduling from SCell to Pcell	Spreadtrum Communications

R1-2008195	Cross carrier scheduling from SCell to Pcell	Samsung

R1-2008284	Discussion on cross-carrier scheduling from Scell to Pcell	OPPO

R1-2008451	Views on Rel-17 DSS SCell scheduling PCell	Apple

R1-2008695	Discussion on cross-carrier scheduling from SCell to PCell	ASUSTeK

R1-2008830	Discussion on Cross-Carrier Scheduling from SCell to PCell	ZTE

R1-2009003	On SCell scheduling PCell transmissions	Intel Corporation

R1-2009023	Cross-carrier scheduling from SCell to Pcell	ETRI

R1-2009040	Discussion on Cross-carrier scheduling from SCell to Pcell	Xiaomi

R1-2009085	Search space monitoring to support SCell scheduling PCell 	InterDigital, Inc.

R1-2009110	Cross-carrier scheduling (from Scell to Pcell)	Lenovo, Motorola Mobility

R1-2009195	Discussion on cross-carrier scheduling enhancements for NR DSS	NTT DOCOMO, INC.

R1-2009206	Enhanced cross-carrier scheduling for DSS	Ericsson

R1-2009277	Views on cross-carrier scheduling from an SCell to the PCell/PSCell	Qualcomm Incorporated

### Multi-cell PDSCH scheduling via a single DCI

Focusing on study whether or not to support the feature first

[103-e-NR-DSS-02] Email discussion/approval for multi-cell PDSCH scheduling via a single DCI – Haipeng (Lenovo)

- 1st check point: 11/5

- 2nd check point: 11/10

- 3rd check point: 11/12

Update on 11/5  11/10

Agreements:

Further study multi-cell PDSCH scheduling via a single DCI with below simulation assumptions:

Table 1: Link level simulation assumptions

| Parameters | Values |
| --- | --- |
| Carrier frequency | Option 1: Inter-band CA (700MHz + 4GHz) Intra-band CA (2GHz) Option 2: Only 4GHz is considered |
| SCS | 15 kHz for 700MHz/2GHz 30 kHz for 4GHz |
| Bandwidth | Option 1: Baseline: PCell 10MHz + SCell 10/40MHz Optional: PCell 20MHz + SCell 20/40/100MHz Option 2: Baseline: Scheduling cell 100 MHz Optional: Scheduling cell 20 MHz |
| Channel model | TDL-C |
| Delay spread | 300 ns |
| Number of symbols for CORESET | [1], 2 or 3 |
| CORESET BW (contiguous PRB allocation) | 24/48/96 RBs depending on the bandwidth |
| CCE-to-REG mapping | Interleaved, [non-interleaved] |
| REG bundle size | 6 |
| Interleaver size | 2 |
| DCI payload size (excluding CRC) | Single PDSCH scheduling: 60 bits as baseline payload size Multi-cell PDSCH scheduling: 72/84/96/104 bits |
| BLER target for multi-cell scheduling DCI | Option 1: 1% Option 2: 0.5% |
| Number of BS antennas | 2 Tx for 700MHz/2GHz carrier frequency 4 Tx for 4GHz |
| Number of UE antennas | 2 Rx for 700MHz/2GHz carrier frequency 4 Rx for 4GHz carrier frequency |
| Modulation | QPSK |
| Channel coding | Polar code |
| UE speed | 3km/h |
| Aggregation level | 1/2/4/8/16 |
| Tx Diversity | One port precoder cycling |

Note 1: For two-cell scheduling via a single DCI, PDCCH transmitted on SCell schedules one PDSCH on the SCell and another PDSCH on PCell.

Note 2: For comparison, for single-cell scheduling, one PDCCH transmitted on SCell schedules one PDSCH on the SCell via self-scheduling and another PDCCH transmitted on the SCell schedules another PDSCH on PCell via cross-carrier scheduling.

Further discussion which rows are applicable to the scheduling cell/the scheduled cell for PDCCH

Agreements:

Further study with below simulation assumptions:

Simulation scenarios:

- For two-cell scheduling via a single DCI, PDCCH transmitted on a first cell schedules one PDSCH on the first cell and another PDSCH on a second cell.

- For single-cell scheduling (baseline), one PDCCH transmitted on a first cell schedules one PDSCH on the first cell via self-scheduling and another PDCCH transmitted on the first cell schedules another PDSCH on a second cell via cross-carrier scheduling.

  - Companies can optionally compare to the case of PDCCH transmitted on each of the two cells via self-scheduling. In this case, company should provide details on how to calculate the PDCCH blocking rate.

Simulation assumptions on carrier frequency, SCS, antenna configuration, carrier bandwidth as well as CORESET configuration

- Combination 1: 2 GHz, 15 kHz SCS, 2 Tx, 2 Rx, 20 MHz carrier BW, 2-symbol CORESET with 96RBs

- Combination 2: 4 GHz, 30 kHz SCS, 4 Tx, 4 Rx, 100 MHz carrier BW, 1-symbol CORESET with 270RBs

- [Combination 3: 700MHz, 15 kHz SCS, 2 Tx, 2 Rx, 10 MHz carrier BW, 3-symbol CORESET with 48RBs]

- [Combination 4: 4GHz, 30 kHz SCS, 4 Tx, 4 Rx, 40 MHz carrier BW, 2-symbol CORESET with 96RBs]

Payload size of two-cell scheduling DCI (excluding CRC):

- 60 for single-cell scheduling DCI (baseline).

- 72/84/96/108 for two-cell scheduling DCI.

- Companies are encouraged to report how the values are obtained, e.g., via separate or shared fields in DCI format.

Target BLER for two-cell scheduling DCI: 1% (baseline), 0.5%(optional)

- Option 1: 1%.

- Supported by OPPO, vivo, Nokia, Qualcomm, CATT, Ericsson, Huawei, Lenovo, Intel, MediaTek

- Option 2: 0.5%.

- Supported by Samsung, LG

Regarding the CCE-to-REG mapping, based on the agreed interleaved CCE-to-REG mapping, whether to adopt non-interleaved CCE-to-REG mapping is up to the proponent.

Agreements:

- Further study with below simulation assumptions:

Table 2: System level simulation assumptions

| Parameters | Values |
| --- | --- |
| Carrier frequency | For scheduling cell, follow agreed link level simulation assumptions For scheduled cell, consider 700MHz/2GHz with 10/20MHz BW (LTE overhead on DSS carrier can be optionally provided, up to proponent) |
| SCS |  |
| Simulation bandwidth |  |
| BS antenna height | 25 m |
| UE height | 1.5m |
| TRP transmit power | 46 dBm for 10MHz |
| Scenario | Urban Macro |
| ISD | 500m |
| TRP antenna configuration | (M,N,P,Mg,Ng;Mp,Np)= (1,2,2,1,1;1,1) for 700MHz (M,N,P,Mg,Ng;Mp,Np)= (2,8,2,1,1;1,1) for 2GHz (M,N,P,Mg,Ng;Mp,Np)= (8,4,2,1,1;1,1) for 4GHz |
| UE antenna configuration | (M,N,P,Mg,Ng;Mp,Np)= (1,1,2,1,1;1,1) for 700MHz/2GHz (M,N,P,Mg,Ng;Mp,Np)= (1,2,2,1,1;1,1) for 4GHz |
| Device deployment | 80% indoor, 20% outdoor |
| UE speeds of interest | Indoor users: 3km/h |
|  | Outdoor users (in-car): 30 km/h |
| BS noise figure | 5 dB |
| BS antenna element gain | 8 dBi |
| UE noise figure | 9 dB |
| Thermal noise level | -174 dBm/Hz |
| Traffic | Full Buffer(baseline), FTP model 1 or 3 up to company |
| Macro sites | 19 |
| Number of UEs per cell | 10/15/20 UEs |
| Downtilt | 102° |
| Minimum BS to UE distance | 35m |

R1-2008929	Discussion on multi-cell PDSCH scheduling via a single DCI	Lenovo, Motorola Mobility

R1-2007580	Discussion on multi-carrier scheduling using single PDCCH	Huawei, HiSilicon

R1-2007696	Discussion on joint scheduling	vivo

R1-2007840	Discussion on multi-cell PDSCH scheduling via a single DCI	CATT

R1-2008063	Discussion on multi-cell PDSCH scheduling via a single DCI	LG Electronics

R1-2008111	Discussion on multi-cell PDSCH scheduling via a single DCI	Spreadtrum Communications

R1-2008196	On the use of one DCI format for scheduling on two cells	Samsung

R1-2008285	Discussion on multi-cell PDSCH scheduling via a single DCI	OPPO

R1-2008452	Views on Rel-17 DSS Multi-cell PDSCH scheduling via a single DCI	Apple

R1-2008696	Discussion on multi-cell PDSCH scheduling via a single DCI	ASUSTeK

R1-2008831	Discussion on Multi-cell PDSCH Scheduling via a Single DCI	ZTE

R1-2008835	Multi-cell scheduling and dormancy	Charter Communications

R1-2008963	Evaluation on On Multi-cell PDSCH Scheduling via Single DCI	MediaTek Inc.

R1-2009004	On 2-cell scheduling via single DCI	Intel Corporation

R1-2009024	Discussion on multi-cell PDSCH scheduling via a single DCI	ETRI

R1-2009047	On support of Single DCI scheduling two cells	Nokia, Nokia Shanghai Bell

R1-2009086	Discussion on the support of single DCI scheduling multi-cell 	InterDigital, Inc.

R1-2009196	Discussion on multi-cell PDSCH scheduling via a single DCI for NR DSS	NTT DOCOMO, INC.

R1-2009207	Study on single DCI scheduling PDSCH on multiple cells	Ericsson

R1-2009278	Views on multi-cell PDSCH scheduling via a single DCI	Qualcomm Incorporated

### Support efficient activation/de-activation mechanism for SCells in NR CA

R1-2008832	Discussion on Support Efficient Activation De-activation Mechanism for SCells in NR CA			ZTE

[103-e-NR-DSS-03] Email discussion/approval for efficient activation/de-activation mechanism for SCells in NR CA – Frank (Huawei)

- 1st check point: 11/5

- 2nd check point: 11/10

- 3rd check point: 11/12

Update on 11/5  11/9

Agreements:

As working assumption, with respect to efficient SCell activation, reuse existing Rel-15/16 TRS structure for temporary RS

- FFS: how many burst/symbols are required for both AGC settling and Time/Frequency tracking for different cases, e.g. FR1 and FR2, known and unknown SCell

  - A burst of temporary RS is notated as in S5.1.6.1.1 of TS 38.214

    - “2-slot with four CSI-RSs resources (4 samples)” for FR1

    - either “1-slot with two CSI-RSs resources (2 samples)” or “2-slot with four CSI-RSs resources (4 samples)” for FR2

- The working assumption can be confirmed after RAN4 check. (A LS for such request is planned).

Agreements:

For efficient SCell activation, discuss and agree from the following alternatives at RAN1#104-e

- Alt 1:  the trigger of temporary RS is integrated into a single triggering signaling with the trigger of SCell activation transmitted on an activated cell.

  - FFS detailed design of this integrated triggering signaling.

  - Potential examples of single triggering signaling for further discussions

  - A PDSCH TB, e.g. containing two respective MAC-CEs for both triggers, one MAC-CE for both triggers

  - A DCI for both triggers

  - A PDSCH TB and its scheduling DL grant, e.g. MAC-CE for activation and DL grant for temporary RS

  - A DL grant and a UL grant received in the same slot/OFDM symbols of PDCCH where the DL grant is scheduling a MAC-CE for SCell activation and the UL grant is triggering the RS.

  - Rel-15/16 SCell activation MAC-CE and a specific configuration of temporary RS being implicitly triggered as well

- Alt2: Triggering of temporary RS separately from SCell activation command is not precluded and both ‘separate’ triggers (examples below) and ‘integrated’ triggers (examples in Alt 1) are considered for SCell activation

  - FFS detailed design of separate triggering signaling.

  - Potential examples of separate triggering signaling for further discussions

  - Rel-15/16 SCell activation MAC-CE and Rel 15/16 DCI triggering

  - Rel-15/16 SCell activation MAC-CE and new DCI triggering for temporary RS

- Note: temporary RS should be triggered by DCI or MAC-CE.

- Note: the final mechanism of trigger signaling targets at applicability to one or more SCell activation.

- FFS handling of  SCell activation by existing Rel15/16 CA activation command when temporary RS is configured and triggered/not triggered

Draft LS (R1-2009786) to RAN4 is approved. Final LS in R1-2009798.

R1-2007548	Support efficient activation/de-activation mechanism for Scells	FUTUREWEI

R1-2007697	Discussion on efficient activation/de-activation mechanism for Scells	vivo

R1-2007841	Disucssion on efficient activation/de-activation mechanism for Scell in NR CA	CATT

R1-2008112	Discussion on efficient activationde-activation mechanism for SCells in NR CA	Spreadtrum Communications

R1-2008197	On efficient activation/de-activation mechanism for Scells	Samsung

R1-2008286	Discussion on efficient activation/de-activation for Scell	OPPO

R1-2008322	Discussion on efficient activation/de-activation mechanism for SCells	Huawei, HiSilicon

R1-2008453	On efficient SCell Activation/Deactivation	Apple

R1-2008713	Efficient activation/deactivation of SCell	ASUSTeK

R1-2008849	Discussion on efficient activation mechanism for SCells	NEC

R1-2008968	On supporting efficient activation mechanism for SCells in NR CA	MediaTek Inc.

R1-2009005	On efficient activation/de-activation for SCells	Intel Corporation

R1-2009048	On low latency Scell activation	Nokia, Nokia Shanghai Bell

R1-2009197	Discussion on efficient activation/deactivation mechanism for SCells	NTT DOCOMO, INC.

R1-2009208	Reduced Latency SCell Activation	Ericsson

R1-2009279	Views on efficient activation/de-activation mechanism for SCells in NR CA	Qualcomm Incorporated

R1-2009283	On work plan for SI on XR Evaluations for NR	Qualcomm Incorporated

## Study on XR Evaluations for NR (RAN1)

Please refer to RP-201145 for detailed scope of the WI

NOTE: SA4 has ongoing work in the XR area, RAN1 will not address these SA4 aspects but will wait for SA4’s outcome

R1-2009281	TR skeleton for Study on XR Evaluations for NR	Qualcomm Incorporated

[103-e-NR-XR-01] Email discussion/approval for XR TR skeleton till 11/5 – Eddy (Qualcomm)

Update on 11/5: 11/9  11/12 morning  the latest draft is endorsed, as in R1-2009707.

### Applications, Traffic Model and Evaluation Methodology

Focus on applications and evaluation methodology including identification of KPIs of interest for relevant deployment scenarios

[103-e-NR-XR-02] Email discussion/approval for applications, traffic model and evaluation methodology– Eddy (Qualcomm) and Xiaohang (vivo)

- 1st check point: 11/5

- 2nd check point: 11/10

- 3rd check point: 11/12

Update on 11/5: 11/9 11/11 11/13

Agreement:

XR applications

RAN1 confirms that diverse applications of VR1/2, AR1/2, (XR conference FFS), CG are of interest for study. Potential prioritization/down selection of these applications for evaluation is to be discussed after detailed traffic models and relevant evaluation assumptions are stable.

- FFS: other applications, e.g., XR conferencing

Agreement:

Traffic model

Traffic model for DL and UL should reflect various aspects, e.g., various bit rates, variable frame/packet (definition of frame/packet to be clarified with traffic model as necessary) size, and periodicity (how to model jitter is FFS).  RAN1 will strive to conclude on detailed traffic models in the next RAN1 meeting (104-e) where SA4 outcome on traffic model is expected to be available.

- Statistical model is preferred.

- It is preferred traffic model for both UL and DL have a certain degree of variability so thatand the total number of traffic models can be reduced.

- Note: Taking into account the fact that the decision on traffic models may hold many other crucial decisions, discussion on traffic model in the next RAN1 meeting is prioritized from the beginning.

Agreement:

Adopt the following deployment for XR/CG evaluations

- Indoor hotspot: FR1 and FR2

  - Detailed definition of Indoor hotspot refers to TR 38.913.

  - Channel model: InH. Detailed definition of InH refers to TR 38.901.

- Dense urban: FR1 and FR2

  - Detailed deployment refers to TR 38.913, where single layer with Marco layer is assumed.

  - Channel model: UMi. Detailed definition of UMi refers to TR 38.901.

FFS: Whether to prioritize FR1 for evaluation.

Note 1: When selecting the deployment and evaluation assumptions for XR/CG evaluations, it is up to company to evaluate FR1 or FR2 or both for the frequency range.

Note 2: It does not mean that all applications are evaluated for all the deployment scenarios.

Agreement:

Urban Macro can be optionally reported for XR/CG evaluations only for FR1.

- FFS: whether Uma is optional or not

- Following parameters can be assumed.

| Parameter | Proposed value |
| --- | --- |
|  | Urban Macro (FR1) |
| Layout | 21cells with wraparound ISD = 500 m |
| BS Tx power | FR1: 49 dBm/20 MHz |

Agreement:

It is to be further discussed how to prioritize the combinations of deployment scenarios and applications after traffic models for each application are stable.

Agreement:

System capacity is defined as the maximum number of users per cell with at least X % of UEs being satisfied.

- X=90 (baseline) or 95 (optional)

- Other values of X can also be evaluated optionally

Note: The exact ‘satisfied’ requirements will be discussed separately

FFS: how to calculate the percentage of satisfied users across multiple drops of simulations

Agreement:

Adopt the simulation assumptions in table 1 as below

Table 1: Simulation assumptions for XR evaluation (Part 1) (updated)

| Parameter | Proposed value |  |
| --- | --- | --- |
|  | Indoor hotspot FR1/FR2 | Dense urban FR1/FR2 |
| Layout | 120m x 50m ISD: 20m TRP numbers: 12 | 21cells with wraparound ISD: 200m |
| Carrier frequency | FR1: 4 GHz FR2: 30 GHz |  |
| Subcarrier spacing | FR1: 30 kHz FR2: 120 kHz |  |
| BS height | 3m | 25m |
| UE height | hUT=1.5 m |  |
| BS noise figure | FR1: 5 dB FR2: 7 dB |  |
| UE noise figure | FR1: 9 dB FR2: 13 dB |  |
| BS receiver | MMSE-IRC |  |
| UE receiver | MMSE-IRC |  |
| Channel estimation | Realistic FFS:Ideal(optional) |  |
| UE speed | 3 km/h |  |
| MCS | Up to 256QAM |  |
| BS antenna pattern | Ceiling-mount antenna radiation pattern, 5 dBi | 3-sector antenna radiation pattern, 8 dBi |
| UE antenna pattern | FR1: Omni-directional, 0 dBi, FR2: UE antenna radiation pattern model 1, 5dBi |  |

Agreement:

Adopt the following UE distribution for XR/CG evaluation for outdoor scenario

- For outdoor scenario:

  - FR1: 80% indoor, 20% outdoor

  - FR2: 100% outdoor

Other UE distribution can be evaluated optionally.

Agreement:

Adopt the following TDD configuration for XR/CG evaluation

- FR1:

  - Option 1: DDDSU

  - Option 2: DDDUU

- FR2:

  - Option 1: DDDSU

FFS detailed S slot format

Note: Other TDD configuration or FDD can be optionally evaluated.

Agreement:

Adopt the following BS antenna parameters for indoor scenario for XR/CG evaluation

- FR1:

  - 32 TxRU, (M, N, P, Mg, Ng; Mp, Np) = (4,4,2,1,1;4,4)

  - (dH, dV) = (0.5, 0.5)λ

- FR2:

  - Option 2: 2 TxRU, (M, N, P, Mg, Ng; Mp, Np) = (16, 8, 2,1,1;1,1)

  - (dH, dV) = (0.5, 0.5)λ

Other BS antenna parameters can be optionally evaluated

Agreement:

For XR/CG evaluation, adopt the following assumptions for downtilt

·         For XR/CG evaluation, adopt the following assumptions for downtilt

- Dense Urban

  - FFS: 6 or 12 degree

  - Other downtilt can be optionally evaluated.

- Indoor hotspot

  - 90° (pointing to the ground)

Other downtilt can be optionally evaluated

Agreement:

Adopt the simulation assumptions in table 3 as below

Table 3: Simulation assumptions for XR evaluation (Part 3)

| Power control parameter | Companies should report |
| --- | --- |
| Transmission scheme | Companies should report, such as Type I/II codebook, rank assumption |
| Scheduler | SU/MU-MIMO PF scheduler (company to report SU or MU), other scheduler (e.g., delay aware scheduler) is up to companies report |
| CSI acquisition | Realistic Both CSI feedback and SRS are considered Companies should report • CSI feedback delay, CSI report periodicity, whether using CSI quantization, CSI error model or not, • Assumptions on SRS: periodicity, processing gain, processing delay, etc • and etc. |
| PHY processing delay | Baseline: UE PDSCH processing Capability #1 Optional: UE PDSCH processing Capability #2 Companies should report gNB processing delay, e.g. DL NACK to retransmission delay, UL previous transmission to current transmission delay and etc. |
| PDCCH overhead | Companies should report |
| DMRS overhead | Companies should report |
| Target BLER | Companies should report |
| Max HARQ transmission | Companies should report |

Agreement:

The following aspects are to be discussed after traffic model is stable.

- For the system capacity definition, how to determine whether a UE is satisfied or not is to be deferred until the exact traffic model along with how to measure E2E user experience is available.  Additional metrics to be collected will be further discussed after traffic model is stable.

- Various options for traffic arrival offset among UEs per cell were proposed by companies, e.g., even offset, random offset, no offset. It will be discussed after traffic model is determined.

Agreement:

System bandwidth for XR/CG evaluations are as follows.

- For FR1,

  - Baseline: 100 MHz

  - Optional: 20/40 MHz (FFS: 200 MHz)

- FFS FR2

Agreement:

For outdoor scenarios, the baseline BS antenna parameters are as follows.

- FFS FR1,

  - Option 1: 64 TxRU, (M, N, P, Mg, Ng; Mp, Np) = (8,8,2,1,1;4,8)

  - Option 2: 32 TxRU, (M, N, P, Mg, Ng; Mp, Np) = (8,2,2,1,1,8,2)

  - Option 3: 32TxRUs (M, N, P, Mg, Ng; Mp, Np) = (4,4,2,1,1,4,4)

(dH, dV) = (0.5λ, 0.85λ)

- FR2:

  - 2 TxRU, (M, N, P, Mg, Ng; Mp, Np) = (4,8,2,2,2;1,1)

(dH, dV) = (0.5λ, 0.5λ)

Other configurations can be optionally evaluated.

Agreement:

UE antenna parameters for XR/CG evaluations are as follows

- FR1:

  - Baseline: 2T/4R, (M, N, P, Mg, Ng; Mp, Np) = (1,2,2,1,1;1,2), (dH, dV) = (0.5, N/A)λ

  - Optional: 4T/4R, 1T/2R, 2T2R

- FFS FR2: down-selection between the next two options. Please indicate if you have preference.

  - Option 1 (Follow Rel-17 evaluation methodology for FeMIMO in R1-2007151)

    - (M, N, P)=(1, 4, 2), 3 panels (left, right, top)

    - (Mp, Np) is up to company. Need to be reported with simulation result.

  - Option 2 (from TR 38.802 – developed in Rel-14)

    - 4Tx/4Rx: (M, N, P, Mg, Ng; Mp, Np) = (2,4,2,1,2;1,2), (dH,dV) = (0.5, 0.5)λ, the polarization angles are 0° and 90°

Agreement:

BS Tx power for XR/CG evaluations are as follows

- For Indoor hotspot:

  - FR1:

    - 24 dBm per 20 MHz

  - FR2:

    - 23 dBm per 80 MHz. EIRP should not exceed 58 dBm

- For Dense urban:

  - FR1:

    - 44 dBm per 20 MHz

  - FR2:

    - 40 dBm per 80 MHz. EIRP should not exceed 73 dBm

For system BW larger than above, Tx power scales up accordingly.

Agreement:

UE max Tx power for XR/CG evaluations are as follows

- FR1: 23 dBm

- FR2: 23 dBm, maximum EIRP 43 dBm

Agreement:

Baseline power evaluation methodology

If UE power consumption is agreed as a KPI for evaluation of XR performance over NR,TR38.840 is the baseline methodology potentially with some modifications if necessary.  RAN1 aim to minimize modeling effort. For example, the following aspects can be considered for further discussion but not limited to.

·        FFS whether/how to model UE power consumption for UE tx power other than 0dBm and 23dBm,

·        FFS whether/how to model UE power consumption for UL slots that are not defined in TR38.840

·        FFS whether/how to model UE power consumption for ‘S’ slot

·        FFS whether/how to model UE power consumption for 400MHz in FR2 including scaling rule for FR2 BWP adaption.

·        FFS whether/how to model UE consumption for the corresponding number of Tx antennas

·        FFS whether/how to model the UE power consumption for UE tx power under FR2

Agreement:

- RAN1 continues to discuss evaluation methodologies for UE power consumption and system capacity.

- RAN1 is to discuss whether/how to study/evaluate mobility and coverage at a later stage, e.g., starting from Q1 2021.

R1-2007555	XR applications and scenarios	FUTUREWEI

R1-2007561	Discussion on applications, traffic model, and evaluation methodology for XR and Cloud Gaming			Huawei, HiSilicon

R1-2007698	Discussion on XR applications, traffic model and evaluation methodologies	vivo

R1-2007843	XR use cases, evaluation methodologies and traffic model	CATT

R1-2007976	Discussion on applications, traffic model and evaluation methodology for XR	ZTE

R1-2008037	Discussion on XR evaluation and Challenges for NR	CMCC

R1-2008198	Applications, Evaluation Methodology, and KPIs for XR	Samsung

R1-2008311	XR evaluations for NR: Applications and Evaluation Methodology	AT&T

R1-2008454	XR Applications, Traffic Model and Evaluation Methodology	Apple

R1-2008818	Discussion on traffic models and evaluation assumptions for XR	InterDigital, Inc.

R1-2008896	Applications, Traffic Model and Evaluation Methodology for XR evaluations for NR	Nokia, Nokia Shanghai Bell

R1-2008939	Discussion for study in XR evaluation for NR	LG Electronics

R1-2008967	On Applications, Traffic Model, and Evaluation Methodology for XR and CG	MediaTek Inc.

R1-2009006	Scenarios, Traffic Model and EVM for XR	Intel Corporation

R1-2009041	Discussion on XR application and evaluation methodology	Xiaomi

R1-2009087	XR use cases, traffic modelling and performance measure	Ericsson

R1-2009198	Discussion on study on XR evaluations for NR	NTT DOCOMO, INC.

R1-2009280	Evaluation Methodology for XR	Qualcomm Incorporated

### Others

R1-2007699	Performance evaluation of XR traffic	vivo

R1-2007842	Potential area of NR enhancement for the support of XR services	CATT

R1-2008316	Initial evaluation results for XR and Cloud Gaming	Huawei, HiSilicon

R1-2008455	Views on enhancements for XR	Apple

R1-2008819	Discussion on potential enhancements for XR	InterDigital, Inc.

R1-2009289	Initial XR performance evaluations	Ericsson

## Study on NB-IoT/eMTC support for Non-Terrestrial Network

Please refer to RP-193235 for detailed scope of the WI

R1-2007882	NB-IoT Waveform Tests over LEO Satellite	OQ Technology

R1-2009096	Rel-17 IoT NTN Work Plan	MediaTek Inc., Eutelsat

### Scenarios applicable to NB-IoT/eMTC

R1-2008868	Email summary discussion on Scenarios applicable to NB-IoT/eMTC	Eutelsat S.A.

R1-2007572	Application scenarios of IoT in NTN	Huawei, HiSilicon

R1-2007844	Application scenarios discussion on NB-IoT/eMTC	CATT

R1-2008038	Discussion on scenarios for IoT NTN	CMCC

R1-2008199	On Scenarios applicable to NB-IoT/eMTC	Samsung

R1-2008257	Discussion on scenarios applicable to NB-IoT/eMTC	OPPO

R1-2008815	Reference Link-Budget parameters for IoT NTN	Eutelsat S.A.

R1-2008854	Preliminary views on the scenarios and assumption for IoT-NTN	ZTE

R1-2009007	On scenarios for NB-IoT and eMTC NTN	Intel Corporation

R1-2009042	Discussion on the scenarios for NB-IoT/eMTC over NTN	Xiaomi

R1-2009088	On scenarios and evaluations for eMTC and NB-IoT based NTN	Ericsson

R1-2009098	Discussion on scenarios applicable to NB-IoT over NTN	Sateliot

R1-2009114	Scenarios applicable to NB-IoT/eMTC 	Qualcomm Incorporated

R1-2009215	Observations on NB-IoT/eMTC for NTN scenarios	Nokia, Nokia Shanghai Bell

R1-2009235	Scenarios for support of NB-IoT and eMTC over NTN	Sony

R1-2009304	Discussion on IoT NTN scenarios - link budget	MediaTek Inc., Eutelsat

[103-e-NR-NB_IoT_eMTC_NTN]	Email discussion/approval on scenarios applicable to NB-IoT/eMTC for NTN with checkpoints for agreements on 11/5, 11/10, 11/12 - Sanaa (Eutelsat)

R1-2008868	Email summary discussion in scenario applicable to NB-IoT/eMTC	Moderator (Eutelsat)

Agreement:

IoT NTN scenarios A, B, and C are included in the study as shown below:

| NTN Configurations | Transparent satellite |
| --- | --- |
| GEO based non-terrestrial access network | Scenario A |
| LEO based non-terrestrial access network generating steerable beams (altitude 1200 km and 600km) | Scenario B |
| LEO based non-terrestrial access network generating fixed beams whose footprints move with the satellite (altitude 1200 km and 600km) | Scenario C |

Agreement:

The following IoT NTN reference scenario parameters are agreed:

| Scenarios | GEO based non-terrestrial access network - scenario A | LEO based non-terrestrial access network -Scenario B & C |
| --- | --- | --- |
| Orbit type | station keeping a nominally fixed position in terms of elevation/azimuth with respect to a given earth point | circular orbiting at low altitude around the earth |
| Altitude | 35,786 km | 600 km 1,200 km |
| Frequency Range (service link) | < 6 GHz (e.g. 2 GHz in S band) |  |
| Device channel Bandwidth (service link) (NOTE 7) | - NB-IoT 180 kHz (DL), Up to 180 kHz with all permissible smaller resource allocations 12*15 kHz, 6*15 kHz, 3*15 kHz, 1*15 kHz, 1*3.75 kHz - eMTC: 1080 kHz (DL), Up to 1080 kHz with all permissible smaller resource allocations , including 2*180 kHz, 180 kHz, 2*15 kHz or 3*15 kHz or 6*15 kHz (UL) |  |
| Payload | Transparent type | Transparent Type |
| Earth-fixed beams | Yes | Scenario B: Yes (steerable beams), see NOTE 1 Scenario C: No (the beams move with the satellite) |
| Max beam foot print size (edge to edge) regardless of the elevation angle | 3500 km (NOTE 3) | 1000 km (NOTE 2) |
| Min Elevation angle for both sat-gateway and C-IoT device | 10° for service link and 10° for feeder link | 10° for service link and 10° for feeder link |
| Max distance between satellite and C-IoT device at min elevation angle | 40,581 km | 1,932 km (600 km altitude) 3,131 km (1,200 km altitude) |
| Max Round Trip Delay (propagation delay only) | 541.46ms (service and feeder links) | 25.77 ms (600km) (service and feeder links) 41.77 ms (1200km) (service and feeder links) |
| Max differential delay within a cell | 10.3 ms | 3.12 ms and 3.18 ms for respectively 600km and 1200km |
| Max Doppler shift (earth fixed user equipment) (NOTE 6) | 0.93 ppm | 24 ppm (600km) 21ppm(1200km) |
| Max Doppler shift variation (earth fixed user equipment) (NOTE 6) | 0.000 045 ppm/s | 0.27 ppm/s (600km) 0.13 ppm/s (1200km) |
| C-IoT device motion on the earth | Min 0 km/s (stationary device), max 120 km/h | Min 0 km/s (stationary device), max 120 km/h |
| C-IoT device antenna types | Omnidirectional antenna with 0 dBi TX antenna gain and 0 dBi RX antenna gain (NOTE 4) |  |
| C-IoT device max Tx power | UE power class 3 with up to 200 mW (23dBm), UE power class 5 with up to 100 mW (20 dBm) |  |
| C-IoT device Noise Figure | Omnidirectional antenna: 7 dB or 9 dB (NOTE 5) |  |
| Service link | 3GPP defined Narrow Band IoT and eMTC |  |

NOTE 1:    Each satellite has the capability to steer beams towards fixed points on earth using beamforming techniques. This is applicable for a period of time corresponding to the visibility time of the satellite.

NOTE 2:   This beam size refers to the Nadir pointing of the satellite.

NOTE 3: The Maximum beam foot print size for GEO is based on current state of the art GEO High Throughput systems, assuming either spot beams at the edge of coverage (low elevation) or a single wide-beam.

NOTE 4: The use of a Circular polarized antenna is optional.

NOTE 5: Same Noise Figure of 7 dB as in Release 16 TR 38.821 or 9 dB as in Release 12 TR 36.888  for device can be assumed for link budget. The noise figure is device vendor implementation specific.

NOTE 6: Max Doppler shift and Max Doppler shift variation in the absence of any device pre-compensation of satellite Doppler shift on the service link.

NOTE 7: System bandwidth is FFS

### Necessary changes to support NB-IoT and eMTC over satellite

A placeholder only: contributions may be submitted but will not be formally handled

R1-2007573	Solutions to support IoT in NTN	Huawei, HiSilicon

R1-2007845	Potential enhancements to support NB-IoT and eMTC over satellite	CATT

R1-2008039	Discussion on enhancements for IoT NTN	CMCC

R1-2008200	On Necessary changes to support NB-IoT and eMTC over satellite	Samsung

R1-2008258	Discussion on necessary changes to support NB-IoT/eMTC over NTN	OPPO

R1-2008456	Potential Enhancement for NB-IoT/eMTC over Satellite	Apple

R1-2008855	Discussion on enhancements for IoT-NTN	ZTE

R1-2008921	Enhancement to Support NBIoT and eMTC on NTN	Lenovo, Motorola Mobility

R1-2009008	On enhancements for NB-IoT and eMTC NTN	Intel Corporation

R1-2009043	Necessary changes to support NB-IoT and eMTC over satellite	Xiaomi

R1-2009089	An overview of technical aspects in IoT NTN	Ericsson

R1-2009095	Discussion on RAN1 Aspects of  IoT NTN	MediaTek Inc., Eutelsat

R1-2009115	Necessary changes to support NB-IoT and eMTC over satellite	Qualcomm Incorporated

R1-2009199	On necessary changes to support IoT devices in NTN	InterDigital, Inc.

R1-2009216	Necessary changes to support NB-IoT and eMTC over satellite	Nokia, Nokia Shanghai Bell

R1-2009236	Considerations for support of NB-IoT and eMTC over NTN	Sony

### Others

R1-2008259	Discussion on other aspects	OPPO

R1-2008320	Other aspects to support IoT in NTN	Huawei, HiSilicon

R1-2008856	Discussion on power consumption and NPRACH capacity for NTN	ZTE

R1-2009090	On evaluation assumptions for eMTC and NB-IoT based NTN	Ericsson

## Introduction of DL 1024QAM for NR FR1

Please refer to RP-202044 for detailed scope of the RAN4-led WI, particularly the details to specify high order modulation for PDSCH.

[103-e-NR-1024QAM-01] Email discussion/approval for DL 1024QAM – Ajit (Ericsson)

- 1st check point: 11/5

- 2nd check point: 11/10

- 3rd check point: 11/12

Update on 11/5:

Agreements:

- Introduce new RRC signaling to indicate use of 1024-QAM CQI table.

- For supporting 1024-QAM in NR downlink, adopt the LTE 1024-QAM constellation.

- 1024-QAM MCS table can be used only with DCI format with CRC scrambled by C-RNTI or CS-RNTI.

Other proposals 11/10

Update on 11/11:

Agreements:

- Introduce new RRC signaling to indicate use of 1024-QAM MCS table for at least DCI format 1_1

  - FFS : support of 1024-QAM MCS table for DCI format 1_2

    - Note: If 1024-QAM MCS table for DCI format 1_2 is supported, separate RRC signaling is used for each of the two DCI formats 1_1 and 1_2, respectively

  - FFS : whether the RRC signaling is only introduced in PDSCH-Config or it can also be separately configured in SPS-Config

Agreements:

- RRC signaling (mcs-Table-r17) to indicate use of 1024-QAM MCS table for DCI format 1_1 is present only in PDSCH-config

- When UE is configured with mcs-Table-r17 set to ‘qam1024’ in PDSCH-Config,

  - UE uses 1024-QAM MCS table for PDSCH scheduled with a DCI format 1_1 with CRC scrambled by C-RNTI,

  - UE uses 1024-QAM MCS table for PDSCH scheduled with the DCI format 1_1 with CRC scrambled by CS-RNTI if the UE is not configured with mcs-Table in SPS-Config

- Note: If 1024-QAM MCS table for DCI format 1_2 is supported, similar approach is used for 1024-QAM MCS table usage with DCI format 1_2

Agreements:

- Adopt following TP for in 38.212, subclause 5.4.2.1 for TBS_LBRM determination.

Agreements:

- Adopt following text proposal for TS 38.201, subclause 4.2.2.

-------------------------------------------------------- Omitted ----------- 4.2.2 Physical channels and modulation -------------------------------------------------------- Omitted --------------- The modulation schemes supported are - in the downlink, QPSK, 16QAM, 64QAM, and 256QAM and 1024 QAM - in the uplink, QPSK, 16QAM, 64QAM and 256QAM for OFDM with a CP and π/2-BPSK, QPSK, 16QAM, 64QAM and 256QAM for DFT-s-OFDM with a CP -------------------------------------------------------- Omitted ------

Agreements:

- Adopt following TP for 38.211 to reflect the agreed 1024-QAM constellation.

Agreements:

- Adopt following TP to 38.211, subclause 7.3.1.2, to reflect 1024-QAM support for PDSCH

Section 7.3.1.2

Table 7.3.1.2-1: Supported modulation schemes.

| Modulation scheme | Modulation order |
| --- | --- |
| QPSK | 2 |
| 16QAM | 4 |
| 64QAM | 6 |
| 256QAM | 8 |
| 1024QAM | 10 |

Agreements:

- Adopt following TP for 38.214, subclause 5.2.2.1, reflecting the 1024-QAM CQI table usage based on corresponding RRC parameter as follows.

  - Note : RAN1 to further align with the RAN2 signaling design

5.2.2.1   Channel quality indicator (CQI)

-------------------------------------------------------- Omitted ----------------------------------------------------------------------------------------------------------------------

The CQI indices and their interpretations are given in Table 5.2.2.1-2 or Table 5.2.2.1-4 for reporting CQI based on QPSK, 16QAM and 64QAM. The CQI indices and their interpretations are given in Table 5.2.2.1-3 for reporting CQI based on QPSK, 16QAM, 64QAM and 256QAM. The CQI indices and their interpretations are given in Table 5.2.2.1-5 for reporting CQI based on QPSK, 16QAM, 64QAM, 256QAM and 1024 QAM.

Based on an unrestricted observation interval in time unless specified otherwise in this Clause, and an unrestricted observation interval in frequency, the UE shall derive for each CQI value reported in uplink slot n the highest CQI index which satisfies the following condition:

-    A single PDSCH transport block with a combination of modulation scheme, target code rate and transport block size corresponding to the CQI index, and occupying a group of downlink physical resource blocks termed the CSI reference resource, could be received with a transport block error probability not exceeding:

-    0.1, if the higher layer parameter cqi-Table in CSI-ReportConfig configures ‘table1’ (corresponding to Table 5.2.2.1-2), or ‘table2’ (corresponding to Table 5.2.2.1-3), or if the higher layer parameter cqi-Table-r17 in CSI-ReportConfig configures ‘table4’ (corresponding to Table 5.2.2.1-5)

-    0.00001, if the higher layer parameter cqi-Table in CSI-ReportConfig configures ‘table3’ (corresponding to Table 5.2.2.1-4).

-------------------------------------------------------- Omitted -------------------------------------------------------------------------------------------------------------------------

Agreements:

- Companies are encouraged to use below link-level simulation assumptions for assessing at least transition point between 256-QAM and 1024-QAM.

| Parameter | Value |
| --- | --- |
| Carrier frequency, SCS, System BW | 3.5GHz, 30kHz, 100 MHz |
| Channel model | AWGN, CDL-B or CDL-C in TR 38.901 with up to 30ns delay spread |
| UE speed | 3km/h, 0km/h |
| Number of UE antennas | 1T4R, 2T4R or 4T4R |
| Number of gNB antennas | 32T32R or 64T64R or 2T or 8T |
| Tx EVM | 0, 2% |
| Rx EVM | 0, 3% |
| MCS | 256 QAM, 1024 QAM Coding Rate*: 0.70, 0.75, 0.80, 0.85, 0.90, 0.925 Other coding rates are not precluded and, if simulated, to be reported by each company |
| DMRS type | DM-RS type 1 |
| Number of DMRS symbols | 1 |
| Number of scheduled RBs | 273 |
| PDSCH mapping | Type A, Start symbol 2, Duration 12 |
| Rank | Rank1, Rank 2, |
| Channel estimation | Realistic channel estimation |
| Metric | Crossover SNR at transition points between 256-QAM and 1024-QAM |
| Note*: Coding rates are used for 1024QAM, while coding rates for 256QAM are selected from TS38.214 MCS table 2 |  |

R1-2007617	On support of DL 1024QAM for NR FR1	Huawei, HiSilicon

R1-2007700	On supporting DL 1024QAM for NR FR1	vivo

R1-2007846	DL 1024QAM for NR FR1	CATT

R1-2007977	Discussion on DL 1024QAM for NR FR1	ZTE

R1-2008201	Discussion on DL 1024QAM for NR FR1	Samsung

R1-2009009	Support of 1024QAM	Intel Corporation

R1-2009171	Work plan on supporting 1024 QAM	Rapporteurs (Nokia, Ericsson)

R1-2009172	Considerations for NR DL 1024 QAM in FR1	Nokia, Nokia Shanghai Bell

R1-2009209	1024QAM for NR DL	Ericsson

R1-2009282	Introduction of 1024-QAM modulation for NR PDSCH	Qualcomm Incorporated

## Others

# Closing of the meeting (Day 13: UTC 5:00PM at the latest)