## Study on supporting NR from 52.6GHz to 71 GHz

Please refer to RP-201838 for detailed scope of the SI

R1-2007958	Draft TR 38.808 v002: Study on supporting NR from 52.6 GHz to 71 GHz	Intel Corporation

Agreement:

R1-2007958 is endorsed with the “smallest of Z_min” modifed to “smallest value of Z_max” and setting Z_min equal to 0 in Section A.3. Modifications to fix errors will be made as part of upcoming updates.

### Required changes to NR using existing DL/UL NR waveform

R1-2007549	"Further discussion on B52 numerology"	FUTUREWEI

R1-2007558	Discussion on physical layer impacts for NR beyond 52.6 GHz	Lenovo, Motorola Mobility

R1-2007604	PHY design in 52.6-71 GHz using NR waveform	Huawei, HiSilicon

R1-2007642	Physical layer design for NR 52.6-71GHz	Beijing Xiaomi Software Tech

R1-2007652	Discussion on requried changes to NR using existing DL/UL NR waveform	vivo

R1-2007785	Consideration on required changes to NR using existing NR waveform	Fujitsu

R1-2007790	Consideration on supporting above 52.6GHz in NR	InterDigital, Inc.

R1-2007847	System Analysis of NR opration in 52.6 to 71 GHz	CATT

R1-2007883	Required changes to NR using existing DL/UL NR waveform	TCL Communication Ltd.

R1-2007926	Required changes to NR using existing DL/UL NR waveform	Nokia, Nokia Shanghai Bell

R1-2007929	On phase noise compensation for NR from 52.6GHz to 71GHz	Mitsubishi Electric RCE

R1-2007941	Discussion on Required Changes to NR in 52.6 – 71 GHz	Intel Corporation

R1-2007965	On the required changes to NR for above 52.6GHz	ZTE, Sanechips

R1-2007982	On NR operations in 52.6 to 71 GHz	Ericsson

R1-2008045	Consideration on required physical layer changes to support NR above 52.6 GHz	LG Electronics

R1-2008076	Discussion on required changes to NR using existing DL/UL NR waveform in 52.6GHz ~ 71GHz			CMCC

R1-2008082	Study on the numerology to support 52.6 GHz to 71GHz	NEC

R1-2008156	Design aspects for extending NR to up to 71 GHz	Samsung

R1-2008250	Discusson on required changes to NR using DL/UL NR waveform	OPPO

R1-2008353	Considerations on required changes to NR from 52.6 GHz to 71 GHz	Sony

R1-2008457	A Discussion on Physical Layer Design for NR above 52.6GHz	Apple

R1-2008493	Discussions on required changes on supporting NR from 52.6GHz to 71 GHz	CAICT

R1-2008501	On required changes to NR using existing DL/UL NR waveform for operation in 60GHz band			MediaTek Inc.

R1-2008516	On NR operation between 52.6 GHz and 71 GHz	Convida Wireless

R1-2008547	Evaluation Methodology and Required Changes on NR from 52.6 to 71 GHz	NTT DOCOMO, INC.

R1-2008615	NR using existing DL-UL NR waveform to support operation between 52p6 GHz and 71 GHz			Qualcomm Incorporated

R1-2008726	Discussion on physical layer aspects for NR beyond 52.6GHz	WILUS Inc.

R1-2008769	Waveform considerations for NR above 52.6 GHz	Charter Communications

[103-e-NR-52-71-Waveform-Changes] Email discussion/approval on required changes to NR using existing DL/UL NR waveform and any TR updates until 11/2; address any remaining aspects by 11/10 – Daewon (Intel)

Additional checkpoints where agreements can be declared: 11/5 and 11/12 (11/12 for approval of final TR)

R1-2009313	Issue Summary for physical layer changes for supporting NR from 52.6 GHz to 71 GHz	Moderator (Intel Corporation)

Agreement:

Numerologies below 120 kHz or above 960 kHz are not supported for any signal or channel.

R1-2009352	[103-e-NR-52-71-Waveform-Changes] Discussions Summary #1	Moderator (Intel Corporation)

Agreement:

For operation in 52-71 GHz:

- 120 kHz should be supported

- Up to two additional SCS may be considered and at least one should be supported

- FFS: Applicability of additional SCS to particular signals and channels

### Channel access mechanism

R1-2007550	On channel access modes in 60GHz	FUTUREWEI

R1-2007559	Discussion on channel access for NR beyond 52.6 GHz	Lenovo, Motorola Mobility

R1-2007605	Channel access mechanism for 60 GHz unlicensed operation	Huawei, HiSilicon

R1-2007643	Channel access mechanism for NR on 52.6-71 GHz	Beijing Xiaomi Software Tech

R1-2007653	Discussion on channel access mechanism	vivo

R1-2007791	On Channel access mechanisms	InterDigital, Inc.

R1-2007848	Channel Access Mechanism in support of NR operation in 52.6 to 71 GHz	CATT

R1-2007884	Channel access mechanism	TCL Communication Ltd.

R1-2007918	Channel access mechanisms for NR from 52.6-71GHz	AT&T

R1-2007927	Design of NR channel access mechanisms for 60 GHz unlicensed band	Nokia, Nokia Shanghai Bell

R1-2007942	Channel Access Procedure for NR in 52.6 - 71 GHz	Intel Corporation

R1-2007966	On the channel access mechanism for above 52.6GHz	ZTE, Sanechips

R1-2007983	Channel Access Mechanism	Ericsson

R1-2008046	Considerations on channel access mechanism to support NR above 52.6 GHz	LG Electronics

R1-2008091	Discussion on channel access mechanism for above 52.6GHz	Spreadtrum Communications

R1-2008157	Channel access mechanism for 60 GHz unlicensed spectrum	Samsung

R1-2008251	Discussion on channel access	OPPO

R1-2008354	Channel access mechanism for 60 GHz unlicensed spectrum	Sony

R1-2008458	Views on Channel Access Mechanisms  for Unlicensed Access above 52.6 GHz	Apple

R1-2008494	Discussions on channel access mechanism on supporting NR from 52.6GHz to 71 GHz	CAICT

R1-2008517	On Channel Access Mechanism and Interference Handling for Supporting NR from 52.6 GHz to 71 GHz	Convida Wireless

R1-2008548	Channel Access Mechanism for NR in 60 GHz unlicensed spectrum	NTT DOCOMO, INC.

R1-2008563	Discussion on channel access mechanism	ITRI

R1-2008630	Channel access mechanism for NR in 52p6 to 71GHz band	Qualcomm Incorporated

Revision of R1-2008616

R1-2008717	Discussion on channel access mechanism for 52.6 to 71GHz unlicensed band	Potevio

R1-2008770	Further aspects of channel access mechanisms	Charter Communications

[103-e-NR-52-71-Channel-Access] Email discussion/approval on channel access mechanisms including aspects related to system level simulations until 11/3; address any remaining aspects by 11/11 – Jing (Qualcomm)

Additional checkpoint where agreements can be declared: 11/9

R1-2009344	FL summary for channel access mechanism for 52.6GHz-71GHz band	Moderator (Qualcomm Incorporated)

R1-2009363	FL summary for channel access mechanism for 52.6GHz-71GHz band	Moderator (Qualcomm Incorporated)

R1-2009368	FL summary for channel access mechanism for 52.6GHz-71GHz band	Moderator (Qualcomm Incorporated)

R1-2009408

R1-2009521	FL summary for channel access mechanism for 52.6GHz-71GHz band	Moderator (Qualcomm)

Conclusion:

It is assumed that at least one transmission of a signal/channel that meets the OCB requirement defined in BRAN should be supported.

Alternate Conclusion:

- It is assumed that nominal bandwidths for the purpose of OCB requirements at the UE are the channel BWs for transmission supported by the UE from the set of channel BWs (carrier BWs) to be defined in 38.101.

- It is assumed that nominal bandwidths for the purpose of OCB requirements at the gNB are the channel BWs for transmission supported by the gNB from the set of channel BWs (carrier BWs) to be defined in 38.104.

Agreement:

At least when operating with LBT, MCOT is 5ms, including all the gaps inside.

Note: Discussions related to further reductions in MCOT due to potential definition of CAPC will be handled separately.

Agreement:

Use the CCA check procedure in EN 302 567 (per RAN1 understanding as from RAN1 #102-e) as the baseline for channel access for 60GHz band when LBT is applied. The following can be discussed further during normative work.

- Whether CAPC and contention window adjustment mechanisms are introduced

- Whether ED threshold change is needed, e.g., due to changes in bandwidth, beamforming gain etc.

- Whether contention window range needs to be adjusted

Conclusion:

There is no maximum channel occupancy time defined when gNB and all UEs in the cell are operating without LBT.

Conclusion:

If a COT is defined for the case where no LBT is used by a gNB/UE, there is no COT sharing when a channel occupancy is not initiated with LBT

Conclusion:

A COT can only be shared if it is initiated after LBT.

Proposal:

It is RAN1 understanding that nominal bandwidth at the UE or gNB is equivalent to channel bandwidth supported by UE or gNB respectively as defined if 38.101.

Agreement:

Capture the following observations in the TR. Editorial modifications and changes to references can be made when capturing the observations in the TR.

- Comparison of No-LBT (NLBT) and Tx Side ED based Omnidirectional Sensing (TxED-Omni) for Indoor Scenerio A: 6 Companies have compared No-LBT with Tx Side ED based Omni sensing LBT

  - Vivo, show tail and median benefits of using TxED-Omni LBT on DL, at high loading. In other cases, including all loads for UL and other loads for DL, TdxED-Omni LBT scheme shows losses. All results are at ED threshold -47.

  - Intel shows gains for 5%ile DL throughput at high loads with TxED-Omni LBT. In other cases including all loads for UL and other loads for DL, TdxED-Omni LBT scheme shows losses. All results are at ED threshold -47.

  - Ericsson, HW, Nokia, Qualcomm and Samsung show loss for TxED-Omni LBT with an EDT of -47 or -48 dB for all cases.

Agreement:

Capture the following in the TR:

On the LBT bandwidth (bandwidth over which a single contiguous LBT is performed) relative to channel bandwidth (as defined in RAN4), the following alternatives have been discussed. Further down-selection of one or more of these alternatives (if needed) should be further discussed when specifications are developed.

- Alt 1: LBT bandwidth equals channel bandwidth

- Alt 2: LBT bandwidth equals the minimum of channel bandwidth and the transmission bandwidth (number of RBs for a given transmission)

- Alt 3: LBT bandwidth can be wider than channel bandwidth

- Alt 4: LBT bandwidth can be narrower than the channel bandwidth, with multiple LBT subband within a channel

- Alt 5: LBT bandwidth equals with minimum supported channel bandwidth or multiples of the minimum supported channel bandwidth

Agreement:

Capture the following in the TR:

For operation where LBT is not required, it can be further discussed when specifications are developed

- If RAN1 should introduce additional conditions/mechanisms for no-LBT to be used, or leave it for gNB implementation

- When no-LBT mode is used, if RAN1 should introduce additional restrictions, such as DFS needs to be applied, ATPC needs to be applied, long term sensing needs to be applied, certain duty cycle limitation, certain transmit power limitation, MCOT limits, etc, or leave the restriction for gNB implementation

- When no-LBT mode is used, if RAN1 should introduce mechanism for the system to fallback to LBT mode, or leave it for gNB implementation

### Others

R1-2007560	Additional evaluations for NR beyond 52.6GHz	Lenovo, Motorola Mobility

R1-2007654	Evaluation on different numerologies for NR using existing DL/UL NR waveform	vivo

R1-2007792	Evaluation results for above 52.6 GHz	InterDigital, Inc.

R1-2007928	Simulation Results for NR from 52.6 GHz to 71 GHz	Nokia, Nokia Shanghai Bell

Late submission

R1-2007943	Considerations on performance evaluation for NR in 52.6-71GHz	Intel Corporation

R1-2007967	Simulation results for NR above 52.6GHz	ZTE, Sanechips

R1-2007984	Evaluation results for NR in 52.6 - 71 GHz	Ericsson

R1-2008047	Considerations on phase noise compensation to support NR above 52.6 GHz	LG Electronics

R1-2008158	Evaluaton results for extending NR to up to 71 GHz	Samsung

R1-2008252	Discussion on other aspects	OPPO

R1-2008459	Evaluation results for Physical Layer Design for NR above 52.6GHz	Apple

R1-2008549	Potential Enhancements for NR on 52.6 to 71 GHz	NTT DOCOMO, INC.

R1-2008771	Performance evaluations for NR above 52.6 GHz	Charter Communications

R1-2008779	Link level and System level evaluation for NR system operating in 52.6GHz to 71GHz	Huawei, HiSilicon

[103-e-NR-52-71-Evaluations] Email discussion/approval on aspects related to link level evaluations until 11/4; address any remaining aspects by 11/12 – Huaming (Vivo)

Additional checkpoint where agreements can be declared: 11/9

R1-2009111	Summary of link level evaluation results and related issues on supporting NR from 52.6 GHz to 71 GHz	Moderator (Vivo)

R1-2009355	Discussion summary #1 for [103-e-NR-52-71-Evaluations]	Moderator (Vivo)

R1-2009377	Discussion summary #2 for [103-e-NR-52-71-Evaluations]	Moderator (Vivo)

R1-2009392	Discussion summary #3 for [103-e-NR-52-71-Evaluations]	Moderator (vivo)

Agreement:

Capture the following observations in the TR (updates to references and other editorial modifications can be made for inclusion in the TR):

7 sources ([61, Ericsson], [26, Qualcomm], [56, vivo], [64, OPPO], [21, Apple], [25, NTT DOCOMO], [12, Intel]) reported evaluation results of PSS/SSS detection performance in terms of SINR in dB achieving cell ID detection probability of 90% by one-shot detection from PSS/SSS. 4 sources ([61, Ericsson], [26, Qualcomm], [56, vivo], [21, Apple]) reported PBCH performance in terms of SINR in dB achieving PBCH BLER target of 10%. 2 sources ([5, vivo], [14, 61, Ericsson]) compared link budget of SSB for different SCS.

- For PSS and SSS detection performance, all evaluated candidate SCSs (120, 240, 480 and 960 kHz) show comparable performances with the non-optional (non-optional to be replaced by references to channel model in Tables to be added when capturing in TR) channel models and delay spread values.

  - The performance degrades as the increase of SCS.

  - Note: The following references are used to derive the observations.

  - 6 out of 7 sources reported minor performance difference (< or ~ 1 dB) between adjacent SCS for all evaluated candidate SCSs (120, 240, 480 and 960 kHz). The other source ([21, Apple]) reported more than 3 dB performance gap of 960 kHz SCS compared to other 120, 240 and 480 kHz SCS. It also reported that the gap of 960 kHz increases as the delay spread increases.

- For PBCH BLER performance, all evaluated candidate SCSs (120, 240, 480 and 960 KHz) show comparable performances with the non-optional (non-optional to be replaced by references to channel model in Tables to be added when capturing in TR) channel models and delay spread.

  - The performance degrades as the increase of SCS.

  - All 4 sources reported minor performance difference (< or ~ 1 dB) between adjacent SCS for all evaluated candidate SCSs (120, 240, 480 and 960 KHz).

  - The performance gap between 120 and 960 kHz is up to ~ 1.8 dB.

- In terms of SSB link budget, smaller SCS have better coverage than larger SCS

  - The MCL and MIL difference between 120 kHz SCS and 480 kHz SCS is about 5 dB. The MCL and MIL difference between 120 kHz SCS and 960 KHz SCS is about 8 dB.

Agreement:

Capture the following observations in the TR (updates to references and other editorial modifications can be made for inclusion in the TR):

8 sources ([61, Ericsson], [68, Huawei], [26, Qualcomm], [56, vivo], [60, ZTE], [64, OPPO], [25, NTT DOCOMO], [12, Intel]) reported evaluation results of PRACH preamble detection performance in terms of SINR in dB achieving PRACH preamble misdetection probability of 1% with evaluation assumptions and parameters as in Table A.1-1 of TR 38.808.  Two sources ([14, 61, Ericsson], [19, OPPO]) compared link budget of PRACH for different SCS.

The following are observed.

- For PRACH preamble detection performances for the same PRACH format, all evaluated candidate SCSs (120, 240, 480 and 960 kHz) show comparable performances

  - Note: The following references were used to derive the observations.

  - 7 out of 8 sources reported minor performance difference (< or ~ 1 dB) between adjacent SCS for all evaluated candidate SCSs (120, 240, 480 and 960 kHz). The other source ([64, OPPO]) reported minor performances difference among all SCS for TDL-A with 5 and 10ns DS. It reported infinite SINR for 960 kHz SCS and comparable SINR for 120, 240 and 480 kHz SCS in TDL-A with 20ns DS using the metrics of preamble miss detection probability of 1% and the estimated timing error is within [-Tcp/2, Tcp/2].

- For PRACH link budget of the same PRACH format and the same sequence length, maximum isotropic loss (MIL) and maximum coupling loss (MCL) degrade as the subcarrier spacing is increased, negatively impacting coverage.

  - Two sources ([14, 61, Ericsson], [19, OPPO]) reported that with UE power limitation of 25 dBm EIRP, the MCL/MIL difference between 120 KHz SCS and 480 KHz SCS is about 4 to 5 dB; the MCL/MIL difference between 120 KHz SCS and 960 KHz SCS is about 8 dB.

  - One source ([14, 61, Ericsson]) reported that without UE power limitation of 25 dBm EIRP (but still under regulatory limits), the MCL difference between 120 kHz SCS and 480 kHz SCS is less than 2.5 dB; the MCL difference between 120 kHz SCS and 960 kHz SCS is less than 1 dB.

  - One source ([14, 61, Ericsson]) reported that without UE power limitation of 25 dBm EIRPs (but still under regulatory limits), compared to short PRACH sequence length, longer PRACH sequence length improve MCL/MIL significantly for 120 kHz SCS due to wider bandwidth for a given SCS.

Agreement:

Capture the following observations in the TR (updates to references and other editorial modifications can be made for inclusion in the TR):

For CP-OFDM, the following are observed regarding the impact of DMRS to BLER performance.

- One source ([57, InterDigital]) reported performance improvement with increased number of DMRS symbols or increased DMRS density especially for higher modulation order for 960 kHz SCS in TDL-A (5 ns and 10 ns delay spread).

- One source ([14, Ericsson]) reported for 480 kHz SCS and below with large delay spread (TDL-A with 40 ns delay spread), the room for performance improvement with a change to the Rel-15 DMRS design is very limited.

- One source ([12, Intel]) reported a performance drop when frequency domain OCC is enabled especially for higher order modulation such as 64 QAM (MCS 22) for 960 kHz SCS in TDL-A (10ns and 20 ns delay spread) and 480 kHz SCS (20 ns delay spread). The performance gap increases when channel delay spread increases.

- One source ([26, Qualcomm]) reported performance improvement with a new DMRS pattern featured by high frequency density (i.e., every RE) and 2-FD-OCC across adjacent REs for 960 kHz SCS in TDL-A (20 ns and 40 ns delay spread)..

- One source ([10, Nokia]) reported that with Rel-15 DMRS type-1, different delay spread values (10ns and 20ns) have a negligible impact to the demodulation performance of PDSCH for a high SCS (such as 960 kHz).

Agreement:

Capture the following observations in the TR (updates to references and other editorial modifications can be made for inclusion in the TR):

7 sources ([61, Ericsson], [68, Huawei], [26, Qualcomm], [56, vivo], [64, OPPO], [10, Nokia], [21, Apple]) evaluated DFT-S-OFDM PUSCH BLER performance with different SCS.

- Compared to CP-OFDM when CPE-only compensation is enabled, DFT-s-OFDM is more robust under phase noise.

- For low and medium MCSs (QPSK and 16QAM), there’s minor performance difference among evaluated SCSs up to 960 kHz.

- With normal CP, for high MCS (64QAM), the performance improves as the increase of SCS, 120 kHz SCS shows up to ~2.0dB loss compared to other larger SCS.

  - Note: the following are references when derive the observations.

  - One source ([61, Ericsson]) reported a performance gap of 1.4~1.8 dB between 120 and 960 kHz SCS

  - One source ([68, Huawei]) reported a performance gap of 1.3~2.5 dB between 120 and 960 kHz SCS

  - One source ([26, Qualcomm]) reported a performance gap of 1.2~1.7 dB between 120 and 960 kHz SCS

  - One source ([56, vivo]) reported a performance gap of ~1.4 dB between 120 and 960 kHz SCS

  - One source ([10, Nokia]) did not report numerical SINR results in table but provided figures showing approximately similar performance difference (~ 2 dB) between 120 and 960 kHz SCS.

  - One source ([21, Apple]) reported a performance gap of more than 7 dB performance gap between 120 kHz SCS and other SCS (240, 480 and 960 kHz) at TDL-A 5 ns DS. It also reported 120 kHz SCS cannot meet the BLER target of 10% at TDL-A 10ns DS and 960 kHz SCS cannot meet the BLER target of 10% at TDL-A 20ns DS.

  - Another source ([64, OPPO]) reported 120 and 240 kHz SCS cannot meet the BLER target of 10% for all evaluated DS values.

- For high MCS (64QAM) at large delay spread (TDL-A 40ns or CDL-B 50ns DS), there’s error floor for 960 KHz SCS at least for BLER target 1%.

  - Note: the following are reference when derive the observations.

  - One source ([26, Qualcomm]) reported an error floor for 960 kHz SCS for BLER target 1%.

  - One source ([56, vivo]) reported an error floor for 960 kHz SCS for BLER target 10%

  - One source ([64, OPPO]) reported no error floor of 960 kHz SCS for the BLER target of 10% and 1% for CDL-B 50ns but an error floor for 960 kHz SCS at TDL-A 20ns for BLER target 1%

Agreement:

Capture the following observations in the TR (updates to references and other editorial modifications can be made for inclusion in the TR):

For CP-OFDM, with evaluation assumptions and parameters as in Table A.1-1 of TR 38.808, the following are observed when CPE-only compensation based on the existing Rel-15 NR PTRS structure is used for normal CP when delay spread is not large. The performance is measured in terms of SINR in dB achieving BLER target of 10% or 1%.

- For low MCS (QPSK) and medium MCS (16QAM), there is minor performance difference between different SCS values up to 960 kHz.

- For high MCS (64QAM), the performance improves in general as the increase of SCS

- For high MCS (64QAM), 13 sources ([61, Ericsson], [68, Huawei], [26, Qualcomm], [56, vivo], [60, ZTE], [64, OPPO], [10, Nokia], [2, 55, Lenovo], [21, Apple], [18, Samsung], [25, NTT DOCOMO], [12, Intel], [7, InterDigital]) compared performance of 120 and 240 kHz SCS in 400 MHz bandwidth

  - for 10% BLER target, there is a performance gap between 120kHz and 240kHz SCS where 240 kHz SCS performs better.

    - Note: the following references are used when derive the observations.

    - One source ([61, Ericsson]) reported better performance of 240 kHz SCS in CDL-D. It also reported both SCS cannot meet 10% BLER target for other evaluated channel model.

    - 3 sources ([68, Huawei], [64, OPPO], [10, Nokia]) reported  both SCS cannot meet 10% BLER target

    - 4 sources ([56, vivo], [60, ZTE], [21, Apple], [7, InterDigital]) reported  120 kHz SCS cannot meet 10% BLER target while 240 kHz SCS can

    - One source ([2, 55, Lenovo]) reported better performance of 240 kHz SCS at TDL-A 5 and 10ns. It also reported that both SCS cannot meet 10% BLER target for other evaluated cases.

    - One source ([12, Intel]) reported better performance of 240 kHz SCS in CDL-D. It also reported that both SCS cannot meet 10% BLER target for other evaluated cases.

    - 2 sources ([26, Qualcomm], [18, Samsung]) reported better performance of 240 kHz SCS

    - One source ([25, NTT DOCOMO]) reported comparable performance for both SCS in CDL-D. It also reported better performance of 120 kHz SCS for other evaluated channel model.

- For high MCS (64QAM), 13 sources ([61, Ericsson], [26, Qualcomm], [56, vivo], [60, ZTE], [64, OPPO], [10, Nokia], [2, 55, Lenovo], [21, Apple], [18, Samsung], [25, NTT DOCOMO], [12, Intel], [67, Charter], [7, InterDigital]) compared performance of 240 and 480 kHz SCS in 400 MHz bandwidth

  - for 10% BLER target, there is a performance gap between 240kHz and 480kHz SCS where 480 kHz SCS performs better.

    - Note: the following references are used when derive the observations.

    - One source ([61, Ericsson]) reported better performance for 480 kHz SCS in CDL-D. It also reported 240 kHz SCS cannot meet 10% BLER target for other evaluated channel model.

    - 3 sources ([64, OPPO], [10, Nokia], [67, Charter]) reported  240 kHz SCS cannot meet 10% BLER target while 480 kHz SCS can

    - One source ([2, 55, Lenovo]) reported better performance of 480 kHz SCS at TDL-A 5 and 10ns. It also reported 240 kHz SCS cannot meet 10% BLER target for other evaluated cases.

    - One source ([12, Intel]) reported better performance of 480 kHz SCS in CDL-D. It also reported 240 kHz SCS cannot meet 10% BLER target for other evaluated cases.

    - 6 sources ([26, Qualcomm], [56, vivo], [60, ZTE], [21, Apple], [18, Samsung], [7, InterDigital]) reported better performance of 480 kHz SCS

    - One source ([25, NTT DOCOMO]) reported comparable performance for both SCS in CDL-D. It also reported better performance of 240 kHz SCS for other evaluated channel model.

- For high MCS (64QAM), 14 sources ([61, Ericsson], [68, Huawei], [26, Qualcomm], [56, vivo], [60, ZTE], [64, OPPO], [10, Nokia], [2, 55, Lenovo], [21, Apple], [18, Samsung], [25, NTT DOCOMO], [12, Intel], [67, Charter], [7, InterDigital]) compared performance of 480 and 960 kHz SCS in 400 MHz bandwidth

  - for 10% BLER target, there is a performance gap between 480kHz and 960kHz SCS where 960 KHz SCS performs better.

    - Note: the following references are used when derive the observations.

    - 7 sources ([61, Ericsson], [60, ZTE], [64, OPPO], [10, Nokia], [2, 55, Lenovo], [67, Charter], [7, InterDigital]) reported  a greater than 1 dB gain of 960 kHz SCS

    - 3 sources ([26, Qualcomm], [56, vivo], [18, Samsung]) reported a smaller than 1 dB performance gain of 960 kHz SCS

    - One source ([68, Huawei]) reported better performance of 480 kHz SCS for CDL-B 50ns and better performance of 960 kHz SCS for other evaluated cases. In all comparison, the difference is greater than 1 dB.

    - Two sources ([21, Apple], [12, Intel]) reported a better performance of 480 kHz SCS than 960 kHz SCS at 20ns DS in TDL-A where 960 kHz SCS cannot meet 10% BLER target and comparable performance for both SCS in all other evaluated cases

    - One source ([25, NTT DOCOMO]) reported comparable performance for both SCS in CDL-D. It also reported better performance of 480 kHz SCS in TDL-A 5ns and better performance of 960 kHz SCS in CDL-B 20ns.

  - for 1% BLER target, the performance for 960kHz SCS is better than 480kHz SCS.

    - Among sources reported SINR values when both SCS can meet 1% BLER target, the absolute value of the performance gap between 480 kHz and 960 kHz SCS is larger than that for 10% BLER target.

- For high MCS (64QAM), 4 sources ([61, Ericsson], [56, vivo], [10, Nokia], [18, Samsung]) compared performance of 480 and 960 kHz SCS in 1600 or 2000 MHz bandwidth. 4 out of 4 sources reported performance gain around 4 ~ 5 dB of 960 kHz SCS for 10% BLER target. All 4 sources also reported that 480 kHz SCS cannot meet 1% BLER target.

Agreement:

Capture the following observations in the TR (updates to references and other editorial modifications can be made for inclusion in the TR):

For CP-OFDM, with evaluation assumptions and parameters as in Table A.1-1 of TR 38.808 (including optional delay spread value), the following are observed when CPE-only compensation based on the existing Rel-15 NR PTRS structure is used with respect to CP type and large delay spread.

- When delay spread is not large (< 40 ns in TDL-A), there is minor performance difference between normal and extended CP for SCS values up to 960 kHz when compared on the basis of equal MCS (code rate). If comparing on the basis of equal TBS (equal throughput), the performance of ECP is degraded due to higher overhead of ECP.

- Among 11 sources ([61, Ericsson], [68, Huawei], [26, Qualcomm], [56, vivo], [60, ZTE], [64, OPPO], [2, 55, Lenovo], [1, Futurewei], [25, NTT DOCOMO], [12, Intel], [7, InterDigital]) evaluated with large delay spread (i.e. 40 ns in TDL-A and/or 50ns in CDL) based on the existing Rel-15 NR PTRS structure for normal CP, 10 sources observed that for low MCS (QPSK) and medium MCS (16QAM), there is minor performance difference between different SCS values up to 960kHz for 10% BLER target

  - The other source ([1, Futurewei]) evaluated SCS 960 kHz with CPE compensation at MCS16 with normal CP in TDL-A channel with 40ns DS. It reported that the BLER for SCS 960 kHz, MCS16, and Normal CP is not acceptable (cannot meet 10% BLER target) for 40ns DS.

- 10 sources ([61, Ericsson], [68, Huawei], [26, Qualcomm], [56, vivo], [60, ZTE], [64, OPPO], [2, 55, Lenovo],  [25, NTT DOCOMO], [12, Intel], [7, InterDigital]) evaluated large delay spread (i.e. 40 ns in TDL-A and/or 50ns in CDL) with CPE compensation based on the existing Rel-15 NR PTRS structure with normal CP. Among 10 sources, 5 sources ([14, Ericsson], [68, Huawei], [5, 56, vivo], [2, 55, Lenovo], [25, NTT DOCOMO]) also evaluated extended CP at least for 960 kHz SCS with CPE compensation based on the existing Rel-15 NR PTRS structure.

  - 9 out 10 sources observed that for high MCS (64QAM) with normal CP, larger SCS (480 and 960 kHz) performs better than smaller SCS (120 and 240 kHz) when only CPE compensation based on the existing Rel-15 NR PTRS structure is used. The other source ([25, NTT DOCOMO]) reported better performance of smaller SCS.

  - 5 out 5 sources observed the performance of 960 kHz SCS with extended CP is significantly improved compared to with normal CP for large delay spread case when compared on the basis of equal MCS (code rate).

  - 4 sources ([14, Ericsson], [68, Huawei], [5, vivo], [2, 55, Lenovo]) compared throughput of normal CP and extended CP at least for 960 kHz SCS with CPE compensation based on the existing Rel-15 NR PTRS structure. They all reported worse throughput of extended CP.

[103-e-NR-52-71GHz-Eval_results] Email discussion/approval on collection of simulation results until 11/6 – Huaming/Jing (Vivo/Qualcomm)