[103-e-NR-Rel-16-V2X-01] Email discussion/approval for remaining issues for PT-RS design

- Issue PS-1-1: For PT-RS sequence generation, when r(m) is defined, which DM-RS symbol is used for reference is not clear.

- Issue PS-1-2: Removal of duplication for PT-RS mapping

till 10/29, with a potential CR by 11/4 – Jeongho (Samsung)

Issue PS-1-1. For PT-RS sequence generation, when r(m) is defined, which DM-RS symbol is used for reference is not clear

Four contributions [Huawei, HiSilicon], [LG], [Sharp], [Apple] discuss on this topic, which is related to the yellow-highlighted part below in TS38.211.

#### 8.4.1.2 Phase-tracking reference signals for PSSCH ##### 8.4.1.2.1 Sequence generation The precoded sidelink phase-tracking reference signal for subcarrier k on layer j is given by r(pj)m=rmif j=j' or j=j"0otherwise where - antenna ports pj' or pj',pj'' associated with PT-RS transmission are given by clause 8.2.3 of [6, TS 38.214]; - rm is given by clause 8.4.1.1.1 at the position of a DM-RS symbol.

The proposed ways are that, r(m) is given at

- Alt 1. the position of the DM-RS symbol transmitted in the first symbol of the 2nd-stage SCI [Huawei, HiSilicon]

- Alt 2. the last position on a DM-RS symbol [LG]

- Alt 3. The position of the first actual transmitted DM-RS symbol [Sharp], [Apple]

- Alt 4. Other(s)

Please provide your views and reason.

| Company | Views |
| --- | --- |

Issue PS-1-2: Removal of duplication for PT-RS mapping

One contribution discusses on PT-RS mapping with the following TP.

##### 8.4.1.2.2 Mapping to physical resources The UE shall transmit phase-tracking reference signals only in the resource blocks used for the PSSCH, and only if the procedure in [6, TS 38.214] indicates that phase-tracking reference signals are being used. The PSSCH PT-RS shall be mapped to resource elements according to ak,lpo,μ⋮ak,lpρ-1,μ=βDMRSPSSCHWr(p0)(2n+k')⋮r(pυ-1)(2n+k') k=4n+2k'+Δ when all the following conditions are fulfilled - l is within the OFDM symbols allocated for the PSSCH transmission; - resource element k,l is not used for sidelink CSI-RS, PSCCH, nor DM-RS associated with PSSCH; - k' and Δ correspond to p0, …, pυ-1 The precoding matrix W is given by clause 8.3.1.4. =====omitted===== PSSCH PT-RS shall not be mapped to resource elements containing PSCCH or PSCCH DMRS by puncturing PSSCH PT-RS. A UE is not expected to receive sidelink CSI-RS and PSSCH PT-RS on the same resource elements. =====omitted=====

The contribution refers the following agreements in RAN1#101-e.

Agreements: - SL-PT-RS is not mapped to the resources for PSCCH by puncturing SL-PT-RS (i.e., sequence & resource mapping). Agreements: - A TX UE is not expected to transmit a SL CSI-RS in a same symbol with the 2nd SCI or PSSCH DMRS - Note: this implies that it’s an error case if there is such a collision Working assumption: - The 2nd SCI is rate-matched around SL-PT-RS Working assumption: - The frequency-domain OCC length for PSCCH is {2, 3, 4} - The same LTE requirement and procedure for UE blind decoding (w.r.t. to OCC vs. LTE’s cyclic shifts) for PSCCH applies Agreements: A TX UE is not expected to transmit a SL CSI-RS and a SL PT-RS which overlap - Note: this implies that it’s an error case if there is such a collision (no puncturing, and the collision is avoided by the Tx UE) This imples a Rx UE is not expected to receive a SL CSI-RS and a SL PT-RS which overlap

Please share your views including whether this change is needed or not.

| Company | Views |
| --- | --- |

# Reference

- R1-2007610	Correction on sidelink PT-RS sequence generation	Huawei, HiSilicon

- R1-2007772	Discussion on essential corrections in physical layer structure	LG Electronics

- R1-2007809	Remaining issues on physical layer structure for NR sidelink	CATT

- R1-2007921	Remaining issues of NR sidelink physical layer structure	ZTE, Sanechips

- R1-2007934	Remaining opens of sidelink physical structure for NR V2X design		Intel Corporation

- R1-2008129	Text Proposals on Physical Layer Structures for NR Sidelink	Samsung

- R1-2008230	Draft TP on physical structure for NR sidelink	OPPO

- R1-2008381	Remaining issue on physical layer structure and procedure for sidelink in NR V2X	Panasonic Corporation

- R1-2008387	Remaining issues on physical layer structure for NR sidelink	Sharp

- R1-2008429	Remaining Issue of Sidelink Physical Layer Structure		Apple

- R1-2008496	Maintenance for PSFCH and PSCCH symbol on NR sidelink	ASUSTeK

- R1-2008529	Maintenance for sidelink physical layer structure		NTT DOCOMO, INC.

- R1-2008604	Remaining Issues in Physical Layer Structure		Qualcomm Incorporated

- R1-2008665	Remaining issues on physical layer structure for NR sidelink	vivo

- R1-2008750	Discussion paper on the remaining issues in Rel. 16 for NR V2X	Ericsson