3GPP TSG RAN WG1 #103-e			R1-200xxxx

e-Meeting, October 26th – November 13th, 2020

Title:	[Draft] LS reply on RAN1 agreement on pre-emption

Response to:	R2-2010930 (R1-2009508)

Release:	Rel-16

Work Item:	5G_V2X_NRSL-Core

Source:	Intel Corporation, [RAN WG1]

To:	RAN WG2

Cc:

Contact Person:

#### Name:	Sergey Panteleev

E-mail Address:	<sergey.panteleev@intel.com>

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	none

1. Overall Description:

RAN1 received LS in R1-2009508 (R2-2010930) from RAN2 with questions about the previous RAN1 agreement to be captured in MAC specification. RAN1 discussed the LS and agreed on the following replies:

Q1: Does the TB in the above agreement correspond to only MAC PDU created for transmission?

RAN1 reply to Q1:

- RAN1 thinks that decision whether a MAC PDU has been created for a TB lies outside of RAN1 scope.

- To the best of RAN1 understanding of the issue behind the question and taking into account the questions and replies for Q2 and Q3 RAN1 considers that it is important to highlight that pre-emption checking is performed for resources within a resource selection of current transport block, i.e. within one period as per other RAN1 agreements.

Q2: Is it correct understanding that whenever a MAC PDU is created, UE checks whether there is(are) pre-empted resource(s) among all remaining periodic reserved resources assuming that the periodic reserved resources are used for transmissions of potential MAC PDUs which have the same priority with the created MAC PDU?

RAN1 reply to Q2:

- For the cited RAN1 agreement, RAN1 assumes that a UE checks whether there is(are) pre-empted resources(s) among resources within a selection window for current TB

Q3: Does RAN1 assume that only retransmission resources in periodic reservation are checked for pre-emption?

RAN1 reply to Q3:

- RAN1 assumes any reserved resource(s), regardless whether it is for an initial transmission or for a retransmission, are checked for pre-emption subject to the conditions that it fits to the resource selection window of current TB

2. Actions:

To RAN WG2: RAN1 kindly asks RAN2 to take the above replies into account.

3. Date of Next TSG-RAN WG1 Meetings:

TSG RAN WG1 Meeting #104-e	25 Jan – 2 Feb 2021				e-Meeting

TSG RAN WG1 Meeting #104-bis-e	12 Apr – 20 Apr 2021				e-Meeting